| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0884-v5 |
| --- | --- |

| Title: | Simplified GEO without multiplication and minimum blending mask storage (harmonization of JVET-P0107, JVET-P0264 and JVET-P0304) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Gao, Semih Esenlik, Elena Alshina, Anand Meher Kotra, Biao Wang (Huawei), Max Bläser, Johannes Sauer (RWTH Aachen) | Tel: Email: | han.gao@huawei.com semih.esenlik@huawei.com elena.alshina@huawei.com blaeser@ient.rwth-aachen.de |
| Source: | Huawei Technologies Co., LTD, RWTH Aachen University |  |  |

_____________________________

# Abstract

In this contribution the CE test CE4-1.14 is used as a basis GEO implementation and 2 simplifications are included according to JVET-P0107, JVET-P0264 and JVET-P0304.

- First modification: Quantize GEO angles to replace per-sample multiplications with shift. Same in JVET-P0107, JVET-P0264.

- Second modification: enable storing precalculated blending masks with minimum total storage requirement. As in JVET-P0304.

The number of partition modes supported are 80. The simulations results show 0.32% and 0.65% BD-rate reduction in RA and LDB configurations. This contribution is identical to JVET-P0885.

Following modifications are applied on top of proposed method as discussed in the JVET group:

Turn off TPM mode

Add TPM angles into GEO (82 modes in total)

With the modifications, simulations results shows 0.54% and 1.19% BD-rate reduction in RA and LDB compared with VTM6.0 without TPM, and 0.20% and 0.49% BD-rate reduction in RA and LDB compared with VTM 6.0.

# Introduction

Geometric merge mode is proposed in [1], which using the same predictors blending concept as TPM and extended the blending masks.

This proposal jointed the proposal jointed the proposed simplification of JVET-P0107 slope based version 2, JVET-P0304 and JVET-P0246 test 1 on top of CE4-1.14.

## JVET-P0107 slope based version 2

Based on the slope based GEO version 2, The Dis[.] look up table is illustrated in Table 3

Table 3 2 bits Dis[.] look up table for slope based GEO

| idx | 0 | 1 | 2 | 4 | 6 | 7 | 8 | 9 | 10 | 12 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dis[idx] | 4 | 4 | 4 | 4 | 2 | 1 | 0 | -1 | -2 | -4 | -4 | -4 |
| idx | 16 | 17 | 18 | 20 | 22 | 23 | 24 | 25 | 26 | 28 | 30 | 31 |
| Dis[idx] | -4 | -4 | -4 | -4 | -2 | -1 | 0 | 1 | 2 | 4 | 4 | 4 |

With the slope based GEO version 2, the computation complexity of geo blending mask derivation is considered as multiplication (up to 2 bits shift) and addition. There is no different partitions compared to TPM. Furthermore, the rounding operation of distFromLine is removed in order to easier store the blending mask. This bugfix guarantees that sample weights are repeated in each row or column in a shifted fashion.

For example:

TPM:

… The variable wIdx is derived as follows: - If cIdx is equal to 0 and triangleDir is equal to 0, the following applies: wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 8, ( x / nCbR − y ) + 4 ) ) (8-842) : ( Clip3( 0, 8, ( x − y / nCbR ) + 4 ) )s - Otherwise, if cIdx is equal to 0 and triangleDir is equal to 1, the following applies: wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 8, ( nCbH − 1 − x / nCbR − y ) + 4 ) ) (8-843) ( Clip3( 0, 8, ( nCbW − 1 − x − y / nCbR ) + 4 ) ) - Otherwise, if cIdx is greater than 0 and triangleDir is equal to 0, the following applies: wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 4, ( x / nCbR − y ) + 2 ) ) (8-844) : ( Clip3( 0, 4, ( x − y / nCbR ) + 2 ) ) - Otherwise (if cIdx is greater than 0 and triangleDir is equal to 1), the following applies: wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 4, ( nCbH − 1 − x / nCbR − y ) + 2 ) ) (8-845) ( Clip3( 0, 4, ( nCbW − 1 − x − y / nCbR ) + 2 ) ) …

GEO

Angle idx is 0: ((x<<1) + 1)*(4 <<1) – ((y<<1) + 1))*(1<<1) –rho

Angle idx is 4: ((x<<1) + 1)*(4 <<1) – ((y<<1) + 1))*(4<<1) –rho

Angle idx is 6: ((x<<1) + 1)*(2 <<1) – ((y<<1) + 1))*(4<<1) –rho

## JVET-P0264 Test 1

In JVET-P0264, the angles in GEO is replaced with the angles which have powers of 2 as tangent. Since the tangent of the proposed angles is a power-of-2 number, most of multiplications can be replaced by bit-shifting. Besides, the weight values for these angles can be implemented by repeating row-by-row or column-by-column with phase shift. With the proposed angles, one row or column is needed to store per block size and per partition mode.

- Proposed angles for GEO along with their corresponding width:height ratio.

## JVET-P0304

In JVET-P0304, it is proposed to derive the weights and the mask for motion field storage for all blocks and partition modes from two sets of pre-defined masks, one for the blending weights derivation and the other for the masks of motion field storage. There are totally 16 masks in each set. Each mask per angle is calculated using the same equations in GEO with block width and block height set to 256 and displacement set to 0. For a block of size W×H with angle φ and distance ρ, the blending weights for the luma samples are directly cropped from the pre-defined masks with offsets calculated as follows:

- Variables offsetX and offsetY are calculated as follows:

offsetX=(M-W)≫1φ % 16 = 8 or (φ % 16 ≠ 0 and H ≥ W)M-W≫1+φ<16 ?ρ×W≫3 :-(ρ×W≫3)otherwise

offsetY=M-H≫1+φ<16 ?ρ×H≫3 :-(ρ×H≫3)φ % 16 = 8 or (φ % 16 ≠ 0 and H ≥ W)M-H≫1otherwise

- sampleWeightLxy=g_sampleWeightL[φ % 16][x+offsetX][y+offsetY].

, where g_sampleWeightL[] is the pre-defined masks for the blending weights.

# Proposal

## Proposed method

In the proposed method 2 modifications on top of CE4-1.14 are descripted as:

- In the jointed contribution, the geo angles are defined slope (tangle power of 2) same as in JVET-P0107 and JVET-P0264.The slope used in this proposal is (1, ½, ¼, 4, 2). In this case, the multiplications are replaced by shift operation if the blending mask is calculated on the fly.

- The rho calculation is replaced by offset X and offset Y as descripted in JVET-P304. In this case, only the 24 blending masks need to be stored in case of not calculate the blending mask on the fly

## Computational complexity analysis

The Table 1 shows the computational complexity (worst case) to generate the blending mask of the proposal method and CE4-1.14

Table 1 Computational complexity analysing of proposed methods

| Operations (per pixel) | TPM | CE4-1.14 | Proposed |
| --- | --- | --- | --- |
| Mults | 0 | 2 | 0 |
| Adds | 1 | 1 | 1 |
| Shifts | 2 | 3 | 4 |

## Motion storage requirements analysis

Since the blending maks can be created by shifting and offsetting. only partial of one line each angle need to be storied, one example of blending mask storage is

blendWeight[24][14] = {

{ 8, 8, 8, 7, 5, 3, 1, 0, 0, 0, 0, 0, 0, 0 },

{ 0, 1, 1, 2, 3, 3, 4, 4, 5, 5, 6, 7, 7, 8 },

{ 0, 0, 0, 1, 2, 3, 5, 6, 7, 8, 8, 8, 8, 8 },

{ 8, 8, 8, 8, 6, 4, 2, 0, 0, 0, 0, 0, 0, 0 },

{ 8, 8, 8, 7, 6, 5, 3, 2, 1, 0, 0, 0, 0, 0 },

{ 8, 7, 7, 6, 5, 5, 4, 4, 3, 3, 2, 1, 1, 0 },

{ 0, 0, 0, 1, 3, 5, 7, 8, 8, 8, 8, 8, 8, 8 },

{ 0, 1, 1, 2, 3, 3, 4, 4, 5, 5, 6, 7, 7, 8 },

{ 0, 0, 0, 0, 0, 1, 2, 3, 5, 6, 7, 8, 8, 8 },

{ 0, 0, 0, 0, 0, 0, 0, 2, 4, 6, 8, 8, 8, 8 },

{ 8, 8, 8, 8, 8, 7, 6, 5, 3, 2, 1, 0, 0, 0 },

{ 8, 7, 7, 6, 5, 5, 4, 4, 3, 3, 2, 1, 1, 0 },

{ 8, 8, 8, 7, 5, 3, 1, 0, 0, 0, 0, 0, 0, 0 },

{ 0, 1, 1, 2, 3, 3, 4, 4, 5, 5, 6, 7, 7, 8 },

{ 0, 0, 0, 1, 2, 3, 5, 6, 7, 8, 8, 8, 8, 8 },

{ 8, 8, 8, 8, 6, 4, 2, 0, 0, 0, 0, 0, 0, 0 },

{ 8, 8, 8, 7, 6, 5, 3, 2, 1, 0, 0, 0, 0, 0 },

{ 8, 7, 7, 6, 5, 5, 4, 4, 3, 3, 2, 1, 1, 0 },

{ 0, 0, 0, 1, 3, 5, 7, 8, 8, 8, 8, 8, 8, 8 },

{ 0, 1, 1, 2, 3, 3, 4, 4, 5, 5, 6, 7, 7, 8 },

{ 0, 0, 0, 0, 0, 1, 2, 3, 5, 6, 7, 8, 8, 8 },

{ 8, 8, 8, 8, 8, 8, 8, 6, 4, 2, 0, 0, 0, 0 },

{ 8, 8, 8, 8, 8, 7, 6, 5, 3, 2, 1, 0, 0, 0 },

{ 8, 7, 7, 6, 5, 5, 4, 4, 3, 3, 2, 1, 1, 0 },

};

The total memory requirement for blending weights is 32 * 24 * 4 = 1344 bits (168 bytes)

In addition, the motion storage mask can be store as:

maskValue[24][6] = {

{ 0, 1, 1, 1, 1, 1 },

{ 1, 2, 2, 2, 2, 0 },

{ 1, 2, 2, 0, 0, 0 },

{ 0, 2, 1, 1, 1, 1 },

{ 0, 2, 2, 1, 1, 1 },

{ 0, 2, 2, 2, 2, 1 },

{ 1, 0, 0, 0, 0, 0 },

{ 1, 2, 2, 2, 2, 0 },

{ 1, 1, 1, 2, 2, 0 },

{ 1, 1, 1, 1, 2, 0 },

{ 0, 0, 0, 2, 2, 1 },

{ 0, 2, 2, 2, 2, 1 },

{ 0, 1, 1, 1, 1, 1 },

{ 1, 2, 2, 2, 2, 0 },

{ 1, 2, 2, 0, 0, 0 },

{ 0, 2, 1, 1, 1, 1 },

{ 0, 2, 2, 1, 1, 1 },

{ 0, 2, 2, 2, 2, 1 },

{ 1, 0, 0, 0, 0, 0 },

{ 1, 2, 2, 2, 2, 0 },

{ 1, 1, 1, 2, 2, 0 },

{ 0, 0, 0, 0, 2, 1 },

{ 0, 0, 0, 2, 2, 1 },

{ 0, 2, 2, 2, 2, 1 },

}

The total memory requirement for the masks of motion field storage is 24 * 6 * 2 =288 (36 bytes). Also,

Overall memory requirement for the proposed method is 204 bytes.

# Simulation results

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.16% | -0.26% | 102% | 99% |
| Class A2 | -0.20% | -0.19% | -0.23% | 101% | 99% |
| Class B | -0.20% | -0.17% | -0.23% | 102% | 98% |
| Class C | -0.68% | -1.00% | -1.01% | 103% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.32% | -0.39% | -0.44% | 102% | 99% |
| Class D | -0.45% | -0.78% | -0.58% | 104% | 104% |
| Class F (optional) | -0.28% | -0.31% | -0.31% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.38% | -0.53% | -0.81% | 103% | 97% |
| Class C | -0.82% | -0.89% | -0.88% | 103% | 97% |
| Class E | -0.87% | -0.57% | -0.37% | 101% | 97% |
| Overall | -0.65% | -0.66% | -0.73% | 102% | 97% |
| Class D | -0.78% | -1.20% | -0.10% | 104% | 98% |
| Class F (optional) | -0.42% | -0.53% | -0.04% | 101% | 98% |

# Software Implementation

# Software encoder

# Based on VTM6.0 (TPM removed), the encoder stores 7x224x224 masks for blending weights and 7x56x56 masks for motion storage value in order to simplify the SIMD implementation. By using the mirroring features of the blending weights, only angle indices in the first quadrant need to be pre-stored (i.e. angleIdx 0~6). The offsetX and offsetY for all possible block sizes are pre-calculated. During encoding, the addresses of the corresponding weights can be fetched from pre-stored weights.

# In this case the total encoder side storage is calculated as 224x224x7x4 + (224x224/16)x7x2 (i.e. ~177KB)

# On-the-fly decoder

# In the attached software, an on-the-fly decoder is implemented in the demon purpose. In case the macro ON_THE_FLY_DEC is turned on, decoder side will calculate the blending mask on the fly based on the provided equations in the WD without any pre-stored or pre-calculated information. With ON_THE_FLY_DEC on or off, the decoding simulation results are identical.

# Bugs fixing

# Following bugs are fixed under macro P0884_P0885_BUGFIX, the software will provide bit exact results compared with JVET_P0884(Vtm6.0TpmOff_vs_P0884_AddTpmAng_TpmOff).xlsm if this macro is turned off .

# Offsets calculation bug

# SPS flag and MaxNumGeoCand is set as 0 bug

# MaxNumGeoCand equal to 2 case bug

# Encoder SAD mask re-use bug.

# Conclusion

In this contribution, a combination method of JVET-P017, JVET-P0264 and JVET-P0304 is proposed to simplify the weights derivation process for GEO. With this simplification method, the multiplications are removed and the memory requirement is 204 bytes. Besides, majority of the coding gain of the original GEO design can be obtained. Considering the trade-off between complexity and coding performance, we recommend to adopt this contribution to VVC.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

RWTH Aachen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, would be prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form), or make them available free of charge (per box 1 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form). We are however aware that also other entities may have current or pending patent rights relating to the technology described in this contribution.