| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0900-v2 |
| --- | --- |

| Title: | Non-CE8: Simplification of chroma BDPCM Syntax for single-tree |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chi-Chang Kuo, Ching-Chieh Lin1, Chun-Lung Lin2 | Tel: Email: | +886-3-5915758 JackLin@itri.com1 chunlung@itri.com2 |
| Source: | ITRI International |  |  |

_____________________________

- Abstract

Chroma BDPCM can provide benefit on coding efficiency especially for low QP and lossless test conditions. This contribution proposed two modified syntax designs for chroma BDPCM when single-tree partition is used. In the first method, the signalling of chroma BDPCM depends on the luma BDPCM flag. In addition, the chroma BDPCM prediction direction is the same as luma BDPCM. For the second method, chroma BDPCM fully reuses the mode and prediction direction form luma BDPCM.

Both methods were implemented on top of CE8-4.1 source code and compared over the results of CE8-4.1. For common QP condition, average BD-rate (Y, U, V) for the first method are (-0.40%, -0.12%, -0.09%) and (-0.23%, 0.06 %, 0.03%) with 6% and 3% encoding time reduction for AI and RA respectively. For the second method, average BD-rate are (-0.33%, 0.37%, 0.24%) and (-0.11%, 0.29%, 0.10%) with 9% and 4% encoding time reduction. When test under low QP condition (QP=2, 7, 12, 17), average BD-rate for the first method are (-0.10%, 0.17%, 0.02%) and (-0.25%, 0.84 %, 0.96%) with 5% and 4% encoding time reduction and (0.30%, 0.72%, 0.58%) and (0.35%, 1.12%, 1.20%) average BD-rate for the second method with 11% and 6% encoding runt time reduction.

Introduction

In current design of CE8-4.1 [2], BDPCM is available to be applied on the chroma components with a syntax sps_bdpcm_chroma_enable_flag to indicate whether BDPCM is enabled for the chroma components. When chroma BDPCM is enabled, an intra_bdpcm_chroma_flag is sent to indicate whether BDPCM is used on the chroma components. When it is on, an additional intra_bdpcm_dir_chroma flag is sent to indicate which prediction direction is selected.

The syntax of chroma BDPCM in CE8-4.1 is shown in Table 1-1.

Table 1-1: Syntax of chroma BDPCM in CE8-4.1

| intra_bdpcm_chroma_flag[ x0 ][ y0 ] | ae(v) |
| --- | --- |
| if(intra_bdpcm_chroma_flag[ x0 ][ y0 ] ) |  |
| intra_bdpcm_chroma_dir_flag[ x0 ][ y0 ] | ae(v) |

For VVC intra prediction, the coding performance is improved when the chroma component directly reuses the coding mode of the corresponding luma component. The chroma BDCPM of CE8-4.1 uses independent syntaxes which may result in redundancy. Therefore, this contribution examined the modified syntax for chroma BDPCM by inferring luma BDPCM to explore the correlation between the luma and chroma components.

Proposed Methods

There are 2 methods proposed in this contribution:

- Method 1:

  - If tree type is equal to SINGLE_TREE, the modifications relative to CE8-4.1 are described as follows:

    - Syntax intra_bdpcm_chroma_flag[ x0 ][ y0 ] is sent only when luma BDPCM syntax intra_bdpcm_flag[ x0 ][ y0 ] is not equal to 0.

    - Syntax intra_bdpcm_chroma_dir_flag[ x0 ][ y0 ] is not sent and it is inferred to be equal to luma BDPCM syntax intra_bdpcm_dir_flag[ x0 ][ y0 ].

  - Otherwise, the behaviour in CE8-4.1 is unchanged.

Table 2-1: Syntax of proposed method 1

| if ( treeType = = SINGLE_TREE ) { |  |
| --- | --- |
| if( intra_bdpcm_flag[ x0 ][ y0 ] ) // it refers to luma BDPCM |  |
| intra_bdpcm_chroma_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| intra_bdpcm_chroma_flag[ x0 ][ y0 ] | ae(v) |
| if(intra_bdpcm_chroma_flag[ x0 ][ y0 ] ) |  |
| intra_bdpcm_chroma_dir_flag[ x0 ][ y0 ] | ae(v) |
| } |  |

- Method 2:

  - If tree type is equal to SINGLE_TREE, the modifications relative to CE8-4.1 are described as follows:

    - No chroma BDPCM syntax is sent. Syntaxes intra_bdpcm_chroma_flag[ x0 ][ y0 ] and intra_bdpcm_chroma_dir_flag[ x0 ][ y0 ] are inferred to be equal to luma BDPCM syntaxes intra_bdpcm_flag[ x0 ][ y0 ] and intra_bdpcm_dir_flag[ x0 ][ y0 ], respectively.

  - Otherwise, the behaviour in CE8-4.1 is unchanged.

Table 2-2: Syntax of proposed method 2

| if ( treeType = = SINGLE_TREE ) { |  |
| --- | --- |
| // no syntax is sent, chroma BDPCM is inferred from luma BDPCM |  |
| } else { |  |
| intra_bdpcm_chroma_flag[ x0 ][ y0 ] | ae(v) |
| if(intra_bdpcm_chroma_flag[ x0 ][ y0 ] ) |  |
| intra_bdpcm_chroma_dir_flag[ x0 ][ y0 ] | ae(v) |
| } |  |

Simulation Results

The implementation was on top of the CE8-4.1 source code. The tests were conducted under CE8 test condition [3]. There are 2 tests for each proposed method described as follows:

  - Test 1: DualITreeOff under common QP condition

  - Test 2: DualITreeOff under low QP condition (QP=2,7,12,17)

Note that the merge request #871, the bug fixes for tickets #512 and #521 were applied to both anchor (VTM 6.0) and test.

Tables 3-1 and 3-2 summarise results of Method 1 for Test 1 and Test 2, respectively.

Tables 3-3 and 3-4 summarise results of Method 2 for Test 1 and Test 2, respectively.

Table 3-5 and Table 3-6 shows the results of Method 1 when use CE8-4.1 as anchor.

Table 3-7 and Table 3-8 shows the results of method 2 when use CE8-4.1 as anchor.

Table 3-9 shows the overall results for CE8-4.1, Method 1 and Method 2 for comparison.

Table 3-1: Results of DualITreeOff under common QP condition (Test 1) for Method 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.47% | -1.94% | -2.03% | 116% | 101% |
| TGM 720p | 0.92% | -1.84% | -1.26% | 111% | 100% |
| Animation | 0.21% | -0.50% | -0.12% | 109% | 100% |
| Mixed content | 0.01% | -4.31% | -4.52% | 111% | 99% |
| Camera-Captured content | 0.04% | 0.38% | 0.20% | 107% | 100% |
| Overall | 0.39% | -1.80% | -1.67% | 111% | 100% |

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.41% | -3.37% | -3.38% | 109% | 100% |
| TGM 720p | 0.22% | -2.57% | -1.84% | 106% | 99% |
| Animation | 0.29% | -1.25% | -0.59% | 106% | 100% |
| Mixed content | -0.88% | -6.07% | -5.98% | 104% | 100% |
| Camera-Captured content | 0.04% | 0.44% | 0.33% | 103% | 100% |
| Overall | -0.15% | -2.80% | -2.49% | 106% | 100% |

Table 3-2: Results of DualITreeOff under low QP condition (Test 2) for Method 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0, Low QPs (2, 7, 12, 17) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -4.94% | -5.83% | -6.22% | 119% | 101% |
| TGM 720p | -1.74% | -3.67% | -3.52% | 115% | 100% |
| Animation | -0.28% | -2.76% | -2.21% | 112% | 100% |
| Mixed content | -3.66% | -7.42% | -8.34% | 116% | 99% |
| Camera-Captured content | 0.02% | -0.02% | -0.01% | 106% | 100% |
| Overall | -2.40% | -4.29% | -4.41% | 114% | 100% |

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0, Low QPs (2, 7, 12, 17) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -6.50% | -9.34% | -9.87% | 110% | 99% |
| TGM 720p | -1.60% | -8.10% | -8.48% | 104% | 100% |
| Animation | -2.47% | -8.51% | -7.43% | 110% | 98% |
| Mixed content | -1.89% | -15.48% | -16.44% | 108% | 98% |
| Camera-Captured content | -0.01% | -0.08% | -0.38% | 105% | 100% |
| Overall | -2.84% | -8.87% | -9.11% | 107% | 99% |

Table 3-3: Results of DualITreeOff under common QP condition (Test 1) for Method 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.52% | -1.84% | -1.86% | 108% | 101% |
| TGM 720p | 0.91% | -1.22% | -0.96% | 108% | 99% |
| Animation | 0.37% | 0.10% | 0.24% | 108% | 100% |
| Mixed content | 0.15% | -3.24% | -3.76% | 108% | 99% |
| Camera-Captured content | 0.07% | 0.37% | 0.20% | 108% | 100% |
| Overall | 0.46% | -1.31% | -1.34% | 108% | 100% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.26% | -3.05% | -3.26% | 107% | 100% |
| TGM 720p | 0.42% | -2.35% | -1.60% | 104% | 100% |
| Animation | 0.50% | -1.01% | -0.68% | 106% | 100% |
| Mixed content | -0.87% | -5.74% | -5.84% | 104% | 100% |
| Camera-Captured content | 0.00% | 0.31% | 0.08% | 103% | 100% |
| Overall | -0.03% | -2.58% | -2.43% | 105% | 100% |

Table 3-4: Results of DualITreeOff under low QP condition (Test 2) for Method 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0, Low QPs (2, 7, 12, 17) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -4.55% | -5.46% | -5.74% | 108% | 103% |
| TGM 720p | -1.29% | -3.27% | -3.07% | 107% | 101% |
| Animation | 0.36% | -1.71% | -1.36% | 108% | 100% |
| Mixed content | -3.29% | -6.76% | -7.60% | 111% | 98% |
| Camera-Captured content | 0.03% | -0.01% | -0.01% | 107% | 100% |
| Overall | -2.01% | -3.77% | -3.89% | 108% | 101% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0, Low QPs (2, 7, 12, 17) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -6.09% | -9.20% | -9.75% | 105% | 99% |
| TGM 720p | -0.89% | -7.97% | -8.54% | 102% | 99% |
| Animation | -1.85% | -7.73% | -6.70% | 108% | 98% |
| Mixed content | -0.84% | -15.33% | -16.25% | 105% | 98% |
| Camera-Captured content | -0.01% | -0.09% | -0.41% | 104% | 100% |
| Overall | -2.25% | -8.63% | -8.93% | 105% | 99% |

Table 3-5: Results of DualITreeOff under common QP condition (Test 1) for Method 1 (CE8-4.1 as anchor)

|  |  | All Intra Main10 |  |
| --- | --- | --- | --- |
|  | Over CE8-4.1 |  |  |
|  | Y | U | V |
| TGM 1080p | -0.24% | -0.08% | -0.08% |
| TGM 720p | -0.58% | -0.31% | -0.19% |
| Animation | -0.25% | 0.07% | 0.12% |
| Mixed content | -0.65% | -0.25% | -0.29% |
| Camera-Captured content | -0.20% | 0.07% | 0.06% |
| Overall | -0.40% | -0.12% | -0.09% |
|  | Random access Main10 |  |  |
|  | Over CE8-4.1 |  |  |
|  | Y | U | V |
| TGM 1080p | -0.19% | -0.16% | -0.17% |
| TGM 720p | -0.43% | 0.24% | -0.09% |
| Animation | -0.02% | 0.11% | 0.09% |
| Mixed content | -0.36% | 0.01% | 0.23% |
| Camera-Captured content | -0.04% | 0.17% | 0.31% |
| Overall | -0.23% | 0.06% | 0.03% |

Table 3-6: Results of DualITreeOff under low QP condition (Test 2) for Method 1 (CE8-4.1 as anchor)

|  |  | All Intra Main10 |  |
| --- | --- | --- | --- |
|  | Over CE8-4.1, Low QPs |  |  |
|  | Y | U | V |
| TGM 1080p | -0.18% | -0.17% | -0.14% |
| TGM 720p | -0.13% | -0.01% | -0.03% |
| Animation | -0.03% | 1.15% | 0.28% |
| Mixed content | -0.10% | 0.01% | 0.07% |
| Camera-Captured content | -0.01% | 0.00% | 0.01% |
| Overall | -0.10% | 0.17% | 0.02% |
|  | Random access Main10 |  |  |
|  | Over CE8-4.1, Low QPs |  |  |
|  | Y | U | V |
| TGM 1080p | -0.10% | 0.03% | -0.02% |
| TGM 720p | -0.45% | 1.47% | 1.82% |
| Animation | -0.08% | -0.17% | 0.27% |
| Mixed content | -0.51% | 2.63% | 2.43% |
| Camera-Captured content | -0.01% | 0.03% | 0.05% |
| Overall | -0.25% | 0.84% | 0.96% |

Table 3-7: Results of DualITreeOff under common QP condition (Test 1) for Method 2 (CE8-4.1 as anchor)

|  |  | All Intra Main10 |  |
| --- | --- | --- | --- |
|  | Over CE8-4.1 |  |  |
|  | Y | U | V |
| TGM 1080p | -0.19% | 0.04% | 0.11% |
| TGM 720p | -0.59% | 0.30% | 0.11% |
| Animation | -0.09% | 0.67% | 0.49% |
| Mixed content | -0.52% | 0.81% | 0.46% |
| Camera-Captured content | -0.17% | 0.06% | 0.05% |
| Overall | -0.33% | 0.37% | 0.24% |
|  | Random access Main10 |  |  |
|  | Over CE8-4.1 |  |  |
|  | Y | U | V |
| TGM 1080p | -0.04% | 0.16% | -0.05% |
| TGM 720p | -0.24% | 0.45% | 0.12% |
| Animation | 0.18% | 0.36% | 0.01% |
| Mixed content | -0.35% | 0.33% | 0.37% |
| Camera-Captured content | -0.08% | 0.05% | 0.06% |
| Overall | -0.11% | 0.29% | 0.10% |

Table 3-8: Results of DualITreeOff under low QP condition (Test 2) for Method 2 (CE8-4.1 as anchor)

|  |  | All Intra Main10 |  |
| --- | --- | --- | --- |
|  | Over CE8-4.1, Low QPs |  |  |
|  | Y | U | V |
| TGM 1080p | 0.22% | 0.23% | 0.35% |
| TGM 720p | 0.32% | 0.42% | 0.45% |
| Animation | 0.61% | 2.24% | 1.17% |
| Mixed content | 0.29% | 0.74% | 0.87% |
| Camera-Captured content | 0.00% | 0.01% | 0.01% |
| Overall | 0.30% | 0.72% | 0.58% |
|  | Random access Main10 |  |  |
|  | Over CE8-4.1, Low QPs |  |  |
|  | Y | U | V |
| TGM 1080p | 0.32% | 0.20% | 0.11% |
| TGM 720p | 0.28% | 1.63% | 1.79% |
| Animation | 0.55% | 0.63% | 1.04% |
| Mixed content | 0.53% | 2.88% | 2.80% |
| Camera-Captured content | -0.01% | 0.02% | 0.02% |
| Overall | 0.35% | 1.12% | 1.20% |

Table 3-5: Comparison of CE 8-4.1, Method 1 and Method 2

|  | Over VTM-6.0, CE8 Anchor, DualITreeOff, AI |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| CE8-4.1, Test1 [2] | 0.80% | -1.67% |  | -1.57% |  | 117% |  | 99% |  |
| Method 1, Test 1 | 0.39% | -1.80% |  | -1.67% |  | 111% |  | 100% |  |
| Method 2, Test 1 | 0.46% | -1.31% |  | -1.34% |  | 108% |  | 100% |  |
|  | Over VTM-6.0, CE8 Anchor, DualITreeOff, RA |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| CE8-4.1, Test1 [2] | 0.08% | -2.86% |  | -2.52% |  | 109% |  | 100% |  |
| Method 1, Test 1 | -0.15% | -2.80% |  | -2.49% |  | 106% |  | 100% |  |
| Method 2, Test 1 | -0.03% | -2.58% |  | -2.43% |  | 105% |  | 100% |  |
|  | Over VTM-6.0, CE8 Anchor, DualITreeOff, LowQP, AI |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| CE8-4.1, Test2 [2] | -2.31% | -4.44% |  | -4.43% |  | 119% |  | 99% |  |
| Method 1, Test 2 | -2.40% | -4.29% |  | -4.41% |  | 114% |  | 100% |  |
| Method 2, Test 2 | -2.01% | -3.77% |  | -3.89% |  | 108% |  | 101% |  |
|  | Over VTM-6.0, CE8 Anchor, DualITreeOff, LowQP, RA |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| CE8-4.1, Test2 [2] | -2.59% |  | -9.54% |  | -9.89% |  | 111% |  | 97% |
| Method 1, Test 2 | -2.84% |  | -8.87% |  | -9.11% |  | 107% |  | 99% |
| Method 2, Test 2 | -2.25% |  | -8.63% |  | -8.93% |  | 105% |  | 99% |

Conclusions

This contribution proposed two methods to simplify chroma BDPCM syntax for single-tree structure. It is suggested to further study in a CE.

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, July 2019.

- G. Clare, F. Henry, T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki, “CE8-4.1: BDPCM and Transform skip for Chroma,” JVET-P0059, October 2019.

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” JVET-O2028, July 2019.

Patent rights declaration(s)

ITRI International may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).