| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0902 |
| --- | --- |

| Title: | Cross-check of JVET-P0592 on Phase shifts for resampling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jiancong Luo | Tel: Email: | jiancong.luo@alibaba-inc.com |
| Source: | Alibaba Group Inc. |  |  |

_____________________________

# Abstract

This contribution reports cross-check results for JVET-P0592, which signal luma and chroma phase shifts in PPS that are used for reference picture resampling and scalability by applying the phase shifts during motion compensation when scaling ratio is different from 1x. This cross-check report confirms the test results and specification changes provided in JVET-P0592.

# Introduction

The simulation was carried out under the CE1 common test condition [1]. Proponent provided the source code of their proposal based on the CE1 software and results. The tests are not completed yet. Crosscheck results match those provided by the proponent.

Results for test 1:

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | -0.11% | -0.26% | 100% | 101% |
| Class C | -0.04% | -0.30% | -0.08% | 100% | 101% |
| Class E | -0.24% | -0.16% | 0.69% | 100% | 101% |
| Overall | -0.09% | -0.18% | 0.04% | 100% | 101% |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

1.5x PSNR1

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.46% | -0.56% | -0.88% | 100% | 101% |
| Class C | -1.06% | 0.30% | 0.78% | 100% | 101% |
| Class E | -3.50% | -2.39% | -1.31% | 100% | 101% |
| Overall | -1.42% | -0.73% | -0.44% | 100% | 101% |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

1.5x PSNR2

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 1.06% | 0.61% | 100% | 101% |
| Class C | -0.08% | 0.37% | 0.15% | 100% | 101% |
| Class E | -0.07% | 1.15% | 0.73% | 100% | 101% |
| Overall | -0.04% | 0.85% | 0.49% | 100% | 101% |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

2.0x PSNR1

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.18% | -0.54% | 100% | 101% |
| Class C | -0.04% | 0.68% | -0.31% | 100% | 101% |
| Class E | 0.05% | -2.56% | 0.17% | 100% | 101% |
| Overall | -0.02% | -0.49% | -0.29% | 100% | 101% |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

2.0x PSNR2

Results for test 2:

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.47% | 0.34% | 101% | 101% |
| Class C | 0.03% | 0.42% | 0.36% | 100% | 101% |
| Class E | -0.08% | 0.62% | 2.06% | 101% | 102% |
| Overall | 0.00% | 0.49% | 0.78% | 101% | 101% |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

1.5x PSNR1

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 27.64% | -0.09% | -0.12% | 101% | 101% |
| Class C | 11.87% | 0.24% | 0.32% | 100% | 101% |
| Class E | 43.26% | 1.38% | 2.52% | 101% | 102% |
| Overall | 26.29% | 0.39% | 0.69% | 101% | 101% |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

1.5x PSNR2

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 1.67% | 1.61% | 101% | 101% |
| Class C | -0.07% | 0.96% | 0.83% | 101% | 101% |
| Class E | 0.06% | 3.17% | 4.33% | 102% | 102% |
| Overall | 0.00% | 1.81% | 2.03% | 101% | 101% |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

2.0x PSNR1

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 79.93% | 2.39% | 3.16% | 101% | 101% |
| Class C | 22.19% | 2.39% | 1.81% | 101% | 101% |
| Class E | 79.48% | 0.04% | 3.65% | 102% | 102% |
| Overall | 60.57% | 1.80% | 2.83% | 101% | 101% |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

2.0x PSNR2