| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 Oct. 2019 | Document: JVET-P_Notes_d2 |
| --- | --- |

| Title: | Meeting Report of the 16th Meeting of the Joint Video Experts Team (JVET), Geneva, CH, 1–11 Oct. 2019 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from the chairs of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chairs of JVET |  |  |

_____________________________

# Summary

The Joint Video Experts Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its sixteenth meeting during 1–11 October 2019 at the ITU premises in Geneva, CH. The JVET meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 2.13 of this document. It is further noted that the unabbreviated name of JVET was formerly known as “Joint Video Exploration Team”, but the parent bodies modified it when entering the phase of formal development of a new standard. The name Versatile Video Coding (VVC) was chosen in April 2018 as the informal nickname for the new standard.

The JVET meeting began at approximately 1400 hours on Tuesday 1 October 2019. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Friday 11 October 2019. On the first two days of the meeting, only aspects related to high level syntax were on the agenda. Approximately XXX people attended the JVET meeting, and approximately XXX input documents, 8 CE summary reports, and 18 AHG reports were discussed. The meeting took place in a collocated fashion with a meeting of SG16 – one of the two parent bodies of the JVET. The subject matter of the JVET meeting activities consisted of developing video coding technology with a compression capability that significantly exceeds that of the current HEVC standard, or otherwise gives better support regarding the requirements of future application domains of video coding. As a primary goal, the JVET meeting reviewed the work that was performed in the interim period since the fourteenth JVET meeting in producing a fifth draft of the VVC standard and the fifth version of the associated VVC test model (VTM). Further important goals were reviewing the results of 8 Core Experiments (CE), reviewing other technical input on novel aspects of video coding technology, producing the next versions of the VVC draft text and VTM, and plan next steps for further investigation of candidate technology towards the formal standard development.

The JVET produced XX output documents from the meeting (update):

- JVET-O2001 Versatile Video Coding specification text (Draft 6), also issued as ISO/IEC CD 23090-3 Versatile Video Coding

- JVET-O2002 Algorithm description for Versatile Video Coding and Test Model 6 (VTM 6)

- JVET-O2007 Supplemental enhancement information messages for coded video bitstreams

- JVET-O2011, JVET common test conditions and software reference configurations for HDR/WCG video

- JVET-O2021 through JVET-O2028, Description of Core Experiments 1 through 8

For the organization and planning of its future work, the JVET established XX “ad hoc groups” (AHGs) to progress the work on particular subject areas. At this meeting, X Core Experiments (CE) were defined. The next four JVET meetings were planned for 8–17 January 2020 under WG 11 auspices in Brussels, BE, during 15–24 April 2020 under WG 11 auspices in Alpbach, AT, during 23 June – 01 July 2020 under ITU-T SG16 auspices in Geneva, CH, and during 7–16 October 2020 under WG 11 auspices in Rennes, FR.

The document distribution site http://phenix.it-sudparis.eu/jvet/ was used for distribution of all documents.

The reflector to be used for discussions by the JVET and all its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/.

# Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Video Experts Team (JVET) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/SC 29/WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JVET are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Video Experts Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its sixteenth meeting during 1–11 October 2019 at the ITU premises in Geneva, CH. The JVET meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany).

It is further noted that the unabbreviated name of JVET was formerly known as “Joint Video Exploration Team”, but the parent bodies modified it when entering the phase of formal development of a new standard. The name Versatile Video Coding (VVC) was chosen in April 2018 as the informal nickname for the new standard.

## Meeting logistics

Information regarding logistics arrangements for the meeting had been provided via the email reflector jvet@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jvet-site/2019_10_P_Geneva/.

## Primary goals

As a primary goal, the JVET meeting reviewed the work that was performed in the interim period since the fifteenth JVET meeting in producing a sixth draft of the VVC standard and the sixth version of the associated VVC test model (VTM). Further important goals were reviewing the results of 8 Core Experiments (CE), reviewing other technical input on novel aspects of video coding technology, producing the next versions of draft text and VTM, and planning next steps for further investigation of candidate technology towards the formal standard development.

## Documents and document handling considerations

### General

The documents of the JVET meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jvet/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report is practised as follows:

- Decisions made by the group that might affect the normative content of a future standard are identified in this report by prefixing the description of the decision with the string “Decision:”.

- Decisions that affect the VTM software but have no normative effect are marked by the string “Decision (SW):”.

- Decisions that fix a “bug” in the VTM description (an error, oversight, or messiness) or in the software are marked by the string “Decision (BF):”.

This meeting report is based primarily on notes taken by the JVET chairs. The preliminary notes were also circulated publicly by ftp and http during the meeting on a daily basis. It should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Tuesday, 24 September 2019. Any documents uploaded after 1159 hours Paris/Geneva time on Wednesday 25 September were considered “officially late”, giving a grace period of 12 hours to accommodate those living in different time zones of the world. The deadline does not apply to AHG reports, and other such reports which can only be produced after the availability of other input documents.

As agreed by the fifteenth meeting as permanent rule, contributions related to CE proposals (including draft text) were to be uploaded 1 week ahead of the above mentioned deadline, such that more thourough study was possible, and in particular the CE summary reports could be provided in time by the regular deadline. Consequently, CE proposal documents which were uploaded after 1159 hours Paris/Geneva time on Wednesday 18 September were considered “officially late”.

It was suggested to have CE description documents include a description of how the results are planned to be reported – e.g., the form of the tables to be used for the results data. Complexity analysis characterizations were suggested to be a particular issue where this applies.

All contribution documents with registration numbers higher than JVET-P0618 were registered after the “officially late” deadline (and therefore were also uploaded late). Likewise, CE proposal documents with registration numbers higher than JVET-P0081 were registered late. However, some documents in the “late” range might include break-out activity reports that were generated during the meeting, and are therefore better considered as report documents rather than as late contributions. Also, all cross-check reports were uploaded late.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following technical design proposal contributions were registered and/or uploaded late:qq

- JVET-P0XXX (a proposal on …), uploaded XX-XX.

- …

It may be observed that some of the above-listed contributions were submissions made in response to issues that arose in discussions during the meeting or from the study of other contributions, and thus could not have been submitted by the ordinary deadline. For example, some of them were proposing combinations or simplifications of other proposals.

The following other document not proposing normative technical content, but with some need for consideration, were registered and/or uploaded late:

- JVET-P0XXX (a document on …), uploaded XX-XX.

- …

All cross-verification reports at this meeting (except for JVET-P0594) were registered late and all were uploaded late. In the interest of brevity, these are not specifically identified here. Initial upload times for each document are recorded in Annex A of this report.

The following (X) contribution registrations were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JVET-P0XXX (withdrawn), JVET-P0XXX (missing by the end of meeting), … .

The following crosschecks had not been uploaded yet by the end of the meeting, but were provided later (check if they become available – otherwise withdraw them):

…

“Placeholder” contribution documents that were basically empty of content, or lacking any results showing benefit for the proposed technology, and obviously uploaded with an intent to provide a more complete submission as a revision, had been agreed to be considered unacceptable and to be rejected in the document management system until a more complete version was available (which would then typically be counted as a late contribution). At the current meeting, this situation applied to the initial uploads of documents JVET-P0487, JVET-P0596, JVET-P0620, and … .

Contributions that had significant problems with uploaded versions included the following:

- JVET-P0XXX (…)

- …

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when there was a consensus to consider them and there was sufficient time available for their review. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload can also be a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This can especially be a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the “comments” field on the web site can be used to indicate what is different in a revision although participants tend to seldom notice what is recorded there.

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the responsible coordinators).

Some other errors were noticed in other initial document uploads (wrong document numbers or meeting dates or meeting locations in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Outputs of the preceding meeting

All output documents of the previous meeting, particularly the meeting report JVET-O2000, the Versatile Video Coding specification text (Draft 6) JVET-O2001, the Algorithm description for Versatile Video Coding and Test Model 6 (VTM 6) JVET-O2002, the Methodology and reporting template for coding tool testing JVET-O2005, the Supplemental enhancement information messages for coded video bitstreams JVET-O2007, the JVET common test conditions and software reference configurations for HDR/WCG video JVET-N2011, and the Description of Core Experiments 1 through 8 (JVET-O2021 through JVET-O2028), had been completed and were approved. The software implementation of VTM (versions 6.0 and 6.1) was also approved.

The group was initially asked to review the meeting report of the previous meeting for finalization. The meeting report was later approved without modification.

The available output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

## Attendance

The list of participants in the JVET meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

## Agenda

The agenda for the meeting was as follows:

- Opening remarks and review of meeting logistics and communication practices

- IPR policy reminder and declarations

- Contribution document allocation

- Review of results of the previous meeting

- Reports of ad hoc group (AHG) activities

- Reports of core experiments planned at the previous meeting

- Consideration of contributions and communications on project guidance

- Consideration of additional video coding technology contributions

- Consideration of information contributions

- Coordination activities

- Approval of output documents and associated editing periods

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, other planning issues

- Other business as appropriate for consideration

- On the first two days of the meeting (October 1 and 2), only aspects related to high level syntax (including AHG8, AHG12, AHG14, and AHG17 reports) were on the agenda. In the morning of October 3, the meeting was continued with general status review and administrative matters, and then proceeded with reports of ad hoc group activities, reports of core experiments, and other matters.

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JVET and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JVET necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jvet-site (JVET contribution templates)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/‌SC 29 Procedures)

It is noted that the ITU TSB director’s AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

“TSB has reported to the TSB Director’s IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur’s group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation.”

The responsible coordinators invited participants to make any necessary verbal reports of previously-unreported IPR in technology that might be considered as prospective candidate for inclusion in future standards, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that the VTM software implementation package uses the same software copyright license header as the HEVC reference software, where the latter had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time. This license header language is based on the BSD license with a preceding sentence declaring that other contributor or third party rights, including patent rights, are not granted by the license, as recorded in N 10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the VTM software, and for evaluating proposals for technology to be potentially included in the design. This software or parts thereof might be published by ITU-T and ISO/IEC as an example implementation of a future video coding standard and for use as the basis of products to promote adoption of such technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

These considerations apply to the 360Lib video conversion software and HDRTools as well.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jvet/.

It was reminded to send a notice to the chairs in cases of changes to document titles, authors etc.

JVET email lists are managed through the site https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/, and to send email to the reflector, the email address is jvet@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JVET participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work. The current number of subscribers was 1221.

For distribution of test sequences, a password-protected ftp site had been set up at RWTH Aachen University, with a mirror site at FhG-HHI. Accredited members of JVET may contact the responsible JVET coordinators to obtain the password information (but the site is not open for use by others).

## Terminology

Some terminology used in this report is explained below:

(check for completeness with JVET-N0013, and draft text)

- ACT: Adaptive colour transform.

- AFF: Affine.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component).

- AMVP: Adaptive motion vector prediction.

- AMT or MTS: Adaptive multi-core transform, or multiple transform selection.

- AMVR: (Locally) adaptive motion vector resolution.

- APS: Adaptation parameter set.

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR).

- ARSS: Adaptive reference sample smoothing.

- ATMVP or “subblock-based temporal merging candidates”: Alternative temporal motion vector prediction.

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BC: See CPR or IBC.

- BCW: Biprediction with CU based weighting

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BDOF: Bi-directional optical flow (formerly known as BIO).

- BDPCM: Block-wise DPCM.

- BL: Base layer.

- BMS: Benchmark set (no longer used), a former preliminary compilation of coding tools on top of VTM, which provide somewhat better compression performance, but are not deemed mature for standardzation.

- BoG: Break-out group.

- BR: Bit rate.

- BV: Block vector (used for intra BC prediction).

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CC: May refer to context-coded, common (test) conditions, or cross-component.

- CCLM: Cross-component linear model.

- CCP: Cross-component prediction.

- CE: Core Experiment – a coordinated experiment conducted toward assessment of coding technology.

- CG: Coefficient group.

- CGS: Colour gamut scalability (historically, coarse-grained scalability).

- CIIP: Combined inter/intra prediction.

- CL-RAS: Cross-layer random-access skip.

- CPMV: Control-point motion vector.

- CPMVP: Control-point motion vector prediction (used in affine motion model).

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- CST: Chroma separate tree.

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DF: Deblocking filter.

- DMVR: Decoder-side motion vector refinement.

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR).

- DT: Decoding time.

- ECS: Entropy coding synchronization (typically synonymous with WPP).

- EMT: Explicit multiple-core transform.

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- ECV: Extended Colour Volume (up to WCG).

- EL: Enhancement layer.

- ET: Encoding time.

- FRUC: Frame rate up conversion (pattern matched motion vector derivation).

- GRA: Gradual random access

- HDR: High dynamic range.

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized by ITU-T as Rec. ITU-T H.265 and by ISO/IEC as ISO/IEC 23008-2.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- HMVP: History based motion vector prediction.

- HRD: Hypothetical reference decoder.

- HyGT: Hyper-cube Givens transform (a type of NSST).

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (ordinarily 12 bits per sample).

- IBF: Intra boundary filtering.

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC).

- ISP: Intra subblock partitioning

- JCCR: Joint coding of chroma residuals

- JEM: Joint exploration model – the software codebase for future video coding exploration.

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- KLT: Karhunen-Loève transform.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP.

- LFNST: Low-frequency non-separable transform

- LIC: Local illumination compensation.

- LM: Linear model.

- LMCS: Luma mapping with chroma scaling (formerly sometimes called “in-loop reshaping”)

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference pictures.

- MC: Motion compensation.

- MCP: Motion compensated prediction.

- MDNSST: Mode dependent non-separable secondary transform.

- MIP: Matrix-based intra prediction

- MMLM: Multi-model (cross component) linear mode.

- MMVD: Merge with MVD.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JVET).

- MPM: Most probable mode (in intra prediction).

- MRL: Multiple reference line intra prediction.

- MV: Motion vector.

- MVD: Motion vector difference.

- NAL: Network abstraction layer (as in AVC and HEVC).

- NSQT: Non-square quadtree.

- NSST: Non-separable secondary transform.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC and HEVC).

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F).

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value.

- OLS: Output layer set.

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g. l,ight input to a camera) to output light (e.g., light emitted by a display).

- operation point: A temporal subset of an OLS.

- PDPC: Position dependent (intra) prediction combination.

- PMMVD: Pattern-matched motion vector derivation.

- POC: Picture order count.

- PoR: Plan of record.

- PROF: Prediction refinement with optical flow

- PPS: Picture parameter set (as in AVC and HEVC).

- PTL: Profile/tier/level combination.

- QM: Quantization matrix (as in AVC and HEVC).

- QP: Quantization parameter (as in AVC and HEVC, sometimes confused with quantization step size).

- QT: Quadtree.

- BT: Binary tree.

- TT: Ternary tree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD).

- RADL: Random-access decodable leading.

- RASL: Random-access skipped leading.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- RDPCM: Residual DPCM

- ROT: Rotation operation for low-frequency transform coefficients.

- RPLM: Reference picture list modification.

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC.

- RPS: Reference picture set.

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SBT: Subblock transform.

- SbTMVP: Subblock based temporal motion vector prediction.

- SD: Slice data; alternatively, standard-definition.

- SDT: Signal-dependent transform.

- SEI: Supplemental enhancement information (as in AVC and HEVC).

- SH: Slice header.

- SHM: Scalable HM.

- SHVC: Scalable high efficiency video coding.

- SIMD: Single instruction, multiple data.

- SMVD: Symmetric MVD.

- SPS: Sequence parameter set (as in AVC and HEVC).

- STMVP: Spatial-temporal motion vector prediction.

- TBA/TBD/TBP: To be announced/determined/presented.

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content.

- TPM: Triangular partitioning mode

- UCBDS: Unrestricted center-biased diamond search.

- UWP: Unequal weight prediction.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JVET).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- VTM: VVC Test Model.

- VVC: Versatile Video Coding, the standardization project developed by JVET.

- WAIP: Wide-angle intra prediction

- WCG: Wide colour gamut.

- WG: Working group, a group of technical experts (usually used to refer to WG 11, a.k.a. MPEG).

- WPP: Wavefront parallel processing (usually synonymous with ECS).

- Block and unit names in HEVC:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU.

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component.

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma.

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC.

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntax within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU).

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size.

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component.

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32.

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions).

- Block and unit names in VVC:

  - CTB: Coding tree block (luma or chroma) – there are three CTBs per CTU in a P or B slice or in an I slice that uses a single tree, and one CTB per luma CTU and two CTBs per chroma CTU in an I slice that uses separate trees.

  - CTU: Coding tree unit (synonymous with LCU, containing both luma and chroma in a P or B slice or in an I slice that uses a single tree, containing only luma or only chroma in an I slice that uses separate trees), with a size of 16x16, 32x32, 64x64, or 128x128 for the luma component.

  - CB: Coding block, a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), a leaf node of a QTBT. It’s the level at which the prediction process and residual transform are performed in JEM. A CU can be square or rectangle shape.

  - PB: Prediction block, a luma or chroma block of a PU.

  - PU: Prediction unit, has the same size as a CU in the VVC context.

  - TB: Transform block, a luma or chroma block of a TU.

  - TU: Transform unit, has the same size as a CU in the VVC context.

## Opening remarks

Remarks during the opening session of the meeting 1430 Tuesday 1 October (chaired by GJS) were as follows.

- The first two days are dedicated to high-level syntax

- Workshop on "The Future of Media" planned for October 8

- 0900 start time generally

- Balloting and approval timeline: "H.VVC" and ISO/IEC 23090-3

  - CD in July 2019 – Results: m49979

  - DIS as output of this meeting, with ballot period between the January and April meetings.

    - Post-meeting editing

  - FDIS and Consent in July

- The meeting logistics, agenda, working practices, policies, and document allocation were reviewed.

  - The meeting host is the ITU

  - Having text available is crucial (and not just arriving at the end of the meeting).

  - There were no objections voiced in the opening plenary to the consideration of late contributions.

- The results of the previous meeting were reviewed.

- On placeholders – there were a number of cases where there was some description of a concept but no test results (see section 2.4.2).

- The primary goals of the meeting were to review the results of CEs, identify promising technology directions, and adopt proposed technology into the VVC draft text and VTM.

- Due to the high number of input contributions, parallelization and breakout work were planned to be used at the meeting.

- Planning of viewing & equipment setup is needed

- Principles of standards development were discussed.

## Scheduling of discussions

Scheduling: Generally meeting time was scheduled during 0900–2100+ hours, with coffee and lunch breaks as convenient. Ongoing scheduling refinements were announced on the group email reflector as needed. Some particular scheduling notes are shown below, although not necessarily 100% accurate or complete:

- Tue. 1 Oct., 1st day

  - 1430–1530 Opening remarks, review of practices, agenda, IPR reminder

  - 1530-1600 Reports of AHGs 8, 12, 17

  - 1630 High-level functional discussion:

    - Scalability

      - 1630 OLSs – section 6.20.1.1

      - 1750 RAPs – section 6.20.1.2

      - 1830 PTL / conformance – section 6.20.1.3

      - 1910 ROI scalability – section 6.20.1.4

      - 1950 VPS and single-layer decoders – section 6.20.1.5

      - 2005 External independent layers – section 6.20.1.6

      - 2015 Layered coding and subpictures – section 6.20.1.7

      - 2120 Diagonal inter-layer referencing – section 6.20.1.8

    - Subpictures

      - 2140-2215 JVET-P0693 summarizes some categories of the HLS proposals on subpictures – section 6.19.1.1

- Tue. 2 Oct., 2nd day

    - 0900 Cross-RAP referencing / external decoder refresh section 6.18.3 [closed]

    - Subpictures

      - 1100-1645 JVET-P0693 summarizes some categories of the HLS proposals on subpictures – section 6.19.1.1

    - Slices, tiles, bricks

      - 1720-2000 JVET-P0686 summarizes HLS proposal on slices, tiles and bricks

    - 2030 JVET-P0687 summarizes HLS proposals on access unit delimiter, picture header, and slice header parameters signalling

  - …

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized as follows (note that the noted document counts do not include crosschecks, and may not be completely accurate):

- AHG reports (18) (section 3) (Plenary)

- Project development (X) (section 4) (Plenary)

  - Text and software development (1)

  - Test conditions (1)

  - Performance assessment (3)

  - Coding studies on specific use cases (0)

  - Test Material (0)

  - Conformance (5)

  - Implementation (2)

- Core Experiments (xx) (section 5) with subtopics

  - CE1: Reference picture resampling filters (4) (section 5.1) (Track B)

  - CE2: Gradual decoding refresh (3) (section 5.2) (Track B)

  - CE3: Intra prediction and mode coding (4) (section 5.3) (Track A)

  - CE4: Inter prediction (8) (section 5.4) (Track B)

  - CE5: Loop filtering (8) (section 5.5) (Track A)

  - CE6: Transforms and transform signalling (6) (section 5.6) (Track A)

  - CE7: Quantization and coefficient coding (9) (section 5.7) (Track A)

  - CE8: Screen content coding tools (6) (section 5.8) (Track A)

- Non-CE technology proposals (xx) (section 6) with subtopics

  - CE1 related – Reference picture resampling filters (9) (section 6.1) (Track B)

  - CE2 related – Gradual decoding refresh (0) (section 6.2) (Track B)

  - CE3 related – Intra prediction and mode coding (48) (section 6.3) (Track A)

  - CE4 related – Inter prediction (104) (section 6.4) (Track B)

  - CE5 related – Loop filtering (72) (section 6.5) (Track A)

  - CE6 related – Transforms and transform signalling (39) (section 6.6) (Track A)

  - CE7 related – Quantization and coefficient coding (26) (section 6.7) (Track A)

  - CE8 related – Screen content coding tools (35) (section 6.8) (Track A)

  - Quantization control (21) (section 6.9) (Track A)

  - Entropy coding 4) (section 6.10) (Track A)

  - Partitioning (14) (section 6.11) (Track B)

  - Chroma sampling and chroma formats (1) (section 6.12) (Track B)

  - Lossless and near lossless coding (24) (section 6.13) (Track A)

  - Miscellaneous coding tools (6) (section 6.14) (Track B)

  - Neural networks (1) (section 6.15) (Track A)

  - 360 degree video (2) (section 6.16) (Track B)

  - High level tool control (5) (section 6.17) (Track B)

  - AHG17: General high-level syntax (47) (section 6.18) (Track B)

  - AHG12: High-level parallelism and coded picture regions (46) (section 6.19) (Track B)

  - AHG8: Layered coding and resolution adaptation (36) (section 6.20) (Track B)

- Complexity analysis and reduction (2) (section 7) (Track X)

- Encoder optimization (1) (section 8) (Track X)

- Metrics and evaluation criteria (1) (section 9) (Track X)

- Withdrawn (13) (section 10) (Track none)

- Joint meetings, plenary discussions, BoG reports, Summary of actions (section 11)

- Project planning (section 12)

- Establishment of AHGs (section 13)

- Output documents (section 14)

- Future meeting plans and concluding remarks (section 15)

The document counts above do not include cross-checks and CE summary reports.

Track A (294) was generally chaired by JRO, and Track B (281) by GJS.

# AHG reports (18)

These reports were discussed Thursday 3 Oct. 0900–XXXX (chaired by GJS and JRO), except otherwise noted.

JVET-P0001 JVET AHG report: Project management (AHG1) [J.-R. Ohm, G. J. Sullivan]

JVET-P0002 JVET AHG report: Draft text and test model algorithm description editing (AHG2) [B. Bross, J. Chen, J. Boyce, S. Kim, S. Liu, Y.-K. Wang, Y. Ye]

JVET-P0003 JVET AHG report: Test model software development (AHG3) [F. Bossen, X. Li, K. Sühring]

JVET-P0004 JVET AHG report: Test material and visual assessment (AHG4) [T. Suzuki, M. Wien, V. Baroncini, R. Chernyak, A. Norkin, J. Ye]

JVET-P0005 JVET AHG report: Memory bandwidth consumption of coding tools (AHG5) [R. Hashimoto, T. Ikai, X. Li, D. Luo, H. Yang, M. Zhou]

JVET-P0006 JVET AHG Report: 360° video coding tools, software and test conditions (AHG6) [Y. He, J. Boyce, K. Choi, J.-L. Lin, Y. Ye]

JVET-P0007 JVET AHG report: Coding of HDR/WCG material (AHG7) [A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy]

JVET-P0008 JVET AHG report: Layered coding and resolution adaptivity (AHG8) [S. Wenger, A. Segall, M.M. Hannuksela, Hendry, S. McCarthy, Y.-C. Sun, P. Topiwala, M Zhou]

Discussed Tuesday 1550 (GJS)

AHG8 was established by the 15th meeting (Gothenburg) to study layered coding and resolution adaptivity. Except for the kickoff message, no emails were exchanged on the reflector. Approximately 44 documents related to the subject matter of this ad hoc group were identified.

Approximately 43 documents were identified by the submitting organizations as related to AHG8’s mandates. An additional 5 documents marked as relating to AHG17 were also identified as relating to AHG8. Of these 48 total documents, 27 were allocated to scalability; 8 were allocated to reference picture resampling (RPR); and 13 were allocated to other parts of the meeting agenda (mostly CE1, AHG12, and AHG17)

The AHG recommended reviewing the related input contributions and to study layered coding and resolution adaptivity aspects.

JVET-P0009 JVET AHG report: Neural Networks in Video Coding (AHG9) [S. Liu, Y.M. Li, B. Choi, K. Kawamura, Y. Li, L. Wang, P. Wu, H. Yang]

JVET-P0010 JVET AHG report: Encoding algorithm optimization (AHG10) A. Duenas, A. Tourapis, S. Ikonin, A. Norkin, R. Sjöberg, J. Le Tanou, J.-M. Thiesse]

JVET-P0011 JVET AHG report: Screen content coding (AHG11) [S. Liu, J. Boyce, A. Filippov, Y.-C. Sun, J. Xu, H. Yang]

JVET-P0012 JVET AHG report: High-level parallelism and coded picture regions (AHG12) [S. Deshpande, M. M. Hannuksela, R. Sjöberg, R. Skupin, W. Wan, Y.-K. Wang, S. Wenger]

Discussed Tuesday 1530 (GJS)

The document summarizes activities of AHG on high-level parallelism and coded picture regions between the 15th and the 16th JVET meetings.

Relevant input documents (total 52) related to AHG12 were listed in the AHG report. An email conversation was conducted on the reflector based on a slide deck submitted by a member company regarding analysis of subpicture grip approach. A total of 9 emails were exchanged on this topic on the JVET email reflector between 2019-08-26 and 2019-09-06.

Contributions that summarize input contributions were noted:

- JVET-P0686 summarizes HLS proposal on slices, tiles and bricks

- JVET-P0693 summarizes some categories of the HLS proposals on subpictures

The AHG recommended:

- To review the related contributions

- To continue to study VVC high-level parallelism and coded picture regions aspects.

JVET-P0013 JVET AHG report: Tool reporting procedure (AHG13) [W.-J. Chien, J. Boyce, W. Chen, Y.-W. Chen, R. Chernyak, K. Choi, R. Hashimoto, Y.-W. Huang, H. Jang, R.-L. Liao, S. Liu]

JVET-P0014 JVET AHG report: Operation modes for low latency support (AHG14) [J.-M. Thiesse, S. Deshpande, A. Duenas, Hendry, K. Kazui, R. Sjöberg, A. Tourapis]

JVET-P0015 JVET AHG report: Quantization control (AHG15) [R. Chernyak, E. François, C. Helmrich, S. McCarthy, A. Segall]

JVET-P0016 JVET AHG report: Implementation studies (AHG16) [M. Zhou, J. An, E. Chai, K. Choi, S. Sethuraman, T. Hsieh, X. Xiu]

JVET-P0017 JVET AHG report: High-level syntax (AHG17) [R. Sjöberg, J. Boyce, S. Deshpande, M. Hannuksela, R. Skupin, A. Tourapis, Y.-K. Wang, W. Wan, S. Wenger]

Discussion Tuesday 1545 (GJS)

This AHG report summarizes the activities of the AHG on High-level syntax (HLS) between the 15th JVET meeting in Gothenburg, SE (3–12 July 2019).and the 16th JVET meeting in Geneva, CH (1–11 October 2019).

It was reported that the estimated number of input contributions related to high-level syntax has increased from 123 at the 15th JVET meeting to 137 at this 16th meeting (10%).

The number of related contributions in particular categories was summarized as:

| Category | HLS inputs (AHG estimation) |
| --- | --- |
| Layered coding and resolution adaptivity (AHG8) | 36 |
| High-level parallelism and coded picture regions (AHG12) | 46 |
| Gradual random access (CE2) | 3 |
| High level tool control | 5 |
| SEI messages | 6 |
| Other general high-level syntax (AHG17) | 41 |
| Total | 137 |

It is noted that the first two days of the 16th meeting in Geneva were previously announced to be devoted to high-level syntax related topics.

An email conversation was conducted on the reflector labeled AHG12/AHG16/AHG17 based on a slide deck submitted by a member company regarding analysis of subpicture grid approach.

The AHG noted the following summary contribution

- JVET-P0687 contains a summary of HLS proposals on access unit delimiter, picture header, and slice header parameters signalling

The AHG recommended that this JVET meeting is planned such that there is enough time allocated to review high-level syntax related contributions.

JVET-P0018 JVET AHG report: Lossless and near-lossless coding tools (AHG18) [T. Nguyen, T.-C. Ma, M. Ikeda, S. Iwamura, H. Jang, X. Zhao]

# Project development (X)

Contributions in this category were discussed XXday X Oct. XXXX–XXXX (chaired by XXX)

## Text and software development (1)

JVET-P0113 AHG2: Editorial input on VVC draft text [Y.-K. Wang (Futurewei), B. Bross (HHI), J. Chen (Futurewei), G. J. Sullivan (Microsoft)] [late]

## Test conditions (1)

JVET-P0345 Low-Delay B encoder configuration proposal [F. Le Léannec, R. Jullian, E. François (InterDigital)]

## Performance assessment (3)

JVET-P0328 AHG10: Performance of the GOP-based temporal filter in VTM-6.1 [J. Enhorn, R. Sjöberg, J. Ström, P. Wennersten (Ericsson)]

JVET-P0616 AHG13: Compression performance analysis for 8K HLG sequences [S. Nemoto, S. Iwamura, A. Ichigaya (NHK), K. Kazui (Fujitsu)] [late] [miss]

JVET-P0622 AHG13: Low Delay results for Affine, ALF and DBF (Class A included) [M. Sychev (Huawei)] [late]

## Coding studies on specific use cases (0)

## Test material (0)

## Conformance (5)

JVET-P0099 Level limits for number of luma samples, tiles, and subpictures [M. M. Hannuksela (Nokia)]

JVET-P0133 AHG17: DPB size analysis of VTM6.0 using RA configuration in current CTC [X. Ma, H. Yang (Huawei)]

JVET-P0295 Level restrictions on maximum tile width for line buffer reduction [M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)]

JVET-P0388 On VVC Conformance [J. Boyce, I. Moccagatta (Intel), L. Litwic (Ericsson), A. Stein (Technicolor), S. McCarthy, W. Husak, B. Lee (Dolby)]

JVET-P0389 Proposal of preliminary list of VVC conformance bit streams [I. Moccagatta, J. Boyce (Intel)]

## Implementation (2)

JVET-P0207 Mobile based heterogeneous multi-core for a VVC Decoder [W. Liu, Y. B. Cho (??)]

JVET-P0307 Early Implementation of VVC software player and Demonstration on Mobile devices [J. R. Arumugam, S. Kotecha, S. Ramamurthy, A. Chelawat, Jayasanker J., A. K. Bedgujar, N. M. Thomas, S. Agrawal, Vijayakumar G. R., K. Patankar (Ittiam)]

# Core Experiments

## CE1: Reference picture resampling filters (4)

Contributions in this category were discussed XXday X Oct. XXXX–XXXX in Track X (chaired by XXX).

JVET-P0021 CE1: Summary report on reference picture resampling filters [J. Luo, V. Seregin, W. Wan]

JVET-P0083 CE1-3: Reference picture resampling filters [J. Luo, M. Sarwer, Y. Ye, (Alibaba)] [late]

JVET-P0088 CE1-1: RPR downsampling filter [J. Samuelsson, S. Deshpande, A. Segall (Sharp Labs of America)] [late]

JVET-P0563 Crosscheck of JVET-P0088: CE1-1: RPR downsampling filter [W. Wan, M. Zhou, P. Chen (Broadcom)] [late]

JVET-P0628 CE1-2: RPR Resampling Filters [P. Topiwala, M Krishnan, W. Dai (FastVDO)] [late]

## CE2: Gradual decoding refresh (3)

Contributions in this category were discussed XXday X July XXXX–XXXX in Track X (chaired by XXX).

JVET-P0022 CE2: Summary Report on Gradual Decoding Refresh [K. Kazui, J.-M. Thiesse, Hendry, L. Wang, K. Kawamura]

JVET-P0112 CE2-3: Wavefront-Based GRA [L. Wang, S. Hong, K. Panusopone (Nokia)] [late]

JVET-P0193 CE2: Gradual Random Access (GRA) using encoder and normative restrictions (Tests 2.1.a, 2.1.b and 2.1.c) [D. Gommelet, J.-M. Thiesse, D. Nicholson (VITEC)] [late]

JVET-P0842 Crosscheck of JVET-P0193: Gradual Decoding Refresh (GDR) using encoder and normative restrictions (Test 2.1.b) [L. Wang, S. Hong (Nokia)]

## CE3: Intra prediction and mode coding (4)

Contributions in this category were discussed XXday X July XXXX–XXXX in Track X (chaired by XXX).

JVET-P0023 CE3: Summary Report on Intra Prediction and Mode Coding [G. Van der Auwera, L. Li, A. Filippov]

JVET-P0054 CE3-3: Up-sampling with a fixed order in MIP [H. Liu, L. Zhang, K. Zhang, Z. Deng, J. Xu, Y. Wang (Bytedance)]

JVET-P0056 CE3-2: On rounding shift of MIP [K. Kondo (Sony), J.-Y. Huo (Xidian Univ.)]

JVET-P0064 CE3-1: MIP downsampling process – align positions of reduced boundary samples with positions of reduced prediction samples [Z. Zhang, K. Andersson, D. Saffar, R. Sjöberg, J. Ström, R Yu (Ericsson)]

## CE4: Inter prediction (8)

Contributions in this category were discussed XXday X July XXXX–XXXX in Track X (chaired by XXX).

JVET-P0024 CE4: Summary report on inter prediction [C.-C. Chen, H. Yang, X. Xiu]

JVET-P0057 CE4: Harmonization of BDOF and PROF (Test 4-2.1 and 4-2.2) [X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai), H. Huang, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm), C.-Y. Lai, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek), Y. He, W. Chen (InterDigital)

JVET-P0068 CE4: CE4-1.1, CE4-1.2 and CE4-1.14: Geometric Merge Mode (GEO) [H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang (Huawei), M. Bläser, J. Sauer (RWTH Aachen Univ.)]

JVET-P0069 CE4: CE4-1.7, CE4-1.8: GEO and TPM Blending Off for SCC [H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang (Huawei), M. Bläser, J. Sauer (RWTH Aachen Univ.)]

JVET-P0070 CE4-1.10: CIIP with triangular partitions + GEO [S. Blasi, A. Seixas Dias, G. Kulupana (BBC), H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang (Huawei)]

JVET-P0071 CE4-1.9: CIIP with triangular partitions [S. Blasi, A. Seixas Dias, G. Kulupana (BBC)]

JVET-P0074 CE4: Geometric merge mode (GEO) with partial transform (tests CE4-1.5 and CE4-1.6) [A. Filippov, V. Rufitskiy (Huawei)]

JVET-P0075 CE4: CIIP with triangular partitions and partial transform (tests CE4-1.12 and CE4-1.13)	 [A. Filippov, V. Rufitskiy (Huawei), S. Blasi, A. Seixas Dias, G. Kulupana (BBC)]

JVET-P0808 Cross-check of CE4-1.1 and CE4-1.2: Geometric Merge Mode (GEO) (JVET-P0068), and CE4-1.2 restricted to 64 modes (JVET-P0107) [A. Wieckowski (HHI)]

## CE5: Loop filtering (8)

Contributions in this category were discussed XXday X Oct. XXXX–XXXX in Track X (chaired by XXX).

JVET-P0025 CE5: Summary Report on Loop filtering [C.-Y. Chen, A. Norkin]

JVET-P0045 CE5-3.3: Signalling EO signs in SAO [C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0639 CE5: Crosscheck of JVET-P0045 (CE5-3.3: Signalling EO signs in SAO) [E. Alshina, A.M. Kotra (Huawei)] [late]

JVET-P0061 CE5-1.2: Consistent chroma deblocking [J. Xu, J. Wang, W. Zhu, L. Zhang, K. Zhang, H. Liu, Z. Deng (Bytedance)]

JVET-P0062 CE5-1.1.1: DMVR deblocking, CE5-1.1.2: DMVR restricted deblocking and CE5-1.1.3: DMVR off [K. Andersson, J. Enhorn (Ericsson)]

JVET-P0073 CE5-3.1: Combination of bilateral filter and SAO [J. Ström, P. Wennersten, J. Enhorn, D. Liu, K. Andersson, L. Litwic, D. Saffar, C. Hollmann, R. Yu, R. Sjöberg (Ericsson)]

JVET-P0078 CE5-3: Combination of Hadamard filter and SAO (CE5-3.2, CE5-3.4) [S. Ikonin, V. Stepin, A. Karabutov, S. Nikolaeva (Huawei)]

JVET-P0080 CE5-2.1, CE5-2.2: Cross Component Adaptive Loop Filter [K. Misra, F. Bossen, A. Segall (Sharp Labs of America)]

JVET-P0081 CE5-1.3: Unified design for longer tap deblocking line buffer reduction [A. M. Kotra, S. Esenlik, B. Wang, H. Gao, E. Alshina (Huawei)]

JVET-P0819 Cross-check of CE5.3.1, CE5.3.2, CE5.3.3, and CE5.3.4 [K. Choi, Y. Piao, W. Choi, K. P. Choi (Samsung)]

## CE6: Transforms and transform signalling (6)

Contributions in this category were discussed XXday X Oct. XXXX–XXXX in Track X (chaired by XXX).

JVET-P0026 CE6: Summary Report on Transforms and Transform Signalling [X. Zhao, H. E. Egilmez, M. Salehifar]

JVET-P0041 CE6-1: LFNST with one mode [M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek), J. Lainema (Nokia)]

JVET-P0044 CE6-2.1: LFNST with 3 transform sets [Y. Zhao, H. Yang (Huawei)]

JVET-P0051 CE6-2.2a/c: LFNST with unified kernels [X. Zhao, X. Li, S. Liu (Tencent)]

JVET-P0052 CE6-2.2d: Combination of CE6-2.2a and CE6-2.3 [X. Zhao, X. Li, S. Liu (Tencent), T. Zhou, T. Hashimoto, T. Ikai (Sharp)]

JVET-P0065 CE6-2.3a: Simplification with new LFNST transform basis [T. Zhou, T. Hashimoto, T. Ikai (Sharp)] [late]

JVET-P0594 Crosscheck of CE6-2.2a and CE6-2.2d: An Orthogonality Analysis [H. E. Egilmez]

JVET-P0647 Crosscheck of CE6 Proposals, Under non-CTC Configuration [M. Koo (LGE)]

## CE7: Quantization and coefficient coding (9)

Contributions in this category were discussed XXday X Oct. XXXX–XXXX in Track X (chaired by XXX).

JVET-P0027 CE7: Summary report on quantization and coefficient coding [H. Schwarz, M. Coban, C. Auyeung]

JVET-P0046 CE7-1.1 and CE7-2.1: Improving transform skip residual coding [S.-T. Hsiang, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)

JVET-P0047 CE7-3.1: Context reduction for sig_coeff_flag [S.-T. Hsiang, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0049 CE7-4.1: Coding of chroma cbf flags [S. Esenlik, B. Wang, A. M. Kotra, H. Gao, E. Alshina (Huawei)]

JVET-P0066 CE7-3.2: Context reduction by reduced usage of local neighbourhood [Y. Chen, F. Le Léannec, T. Poirier, F. Galpin (InterDigital)]

JVET-P0067 CE7-3.3: Context reduction for sig_coeff_flag, par_level_flag, abs_level_gtx_flag [Y. Chen, F. Le Léannec, T. Poirier, F. Galpin (Intedigital), S.-T. Hsiang, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0072 CE7-1.3: Simplification of transform-skip residual coding [M. G. Sarwer, R. -L. Liao, J. Luo, Y. Ye (Alibaba)]

JVET-P0076 CE7-2.2: QP dependent binarization in TS residual coding [M. Karczewicz, H. Wang, Y.-H. Chao, M. Coban (Qualcomm)]

JVET-P0079 CE7: Modifications to transform-skip residual coding (CE7-1.2) [M. Karczewicz, H. Wang, Y.-H. Chao, M. Coban (Qualcomm)]

## CE8: Screen content coding tools (6)

Contributions in this category were discussed XXday X Oct. XXXX–XXXX in Track X (chaired by XXX).

JVET-P0028 CE8: Summary Report on 4:4:4 Screen Content Coding Tools [X. Xu, Y.-C. Chao, Y.-C. Sun, J. Xu]

JVET-P0055 CE8-1.2: Compound Palette Mode [W. Zhu, J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)]

JVET-P0058 CE8-2.1: Transform Skip for Chroma with limiting maximum number of context-coded bin in TS residual coding [T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)]

JVET-P0059 CE8-4.1: BDPCM and Transform skip for Chroma [G. Clare (bcom), F. Henry (Orange), T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)]

JVET-P0060 CE8-1.1: Palette mode with neighboring pixel copy [Y.-C. Sun, T.-S. Chang, J. Lou (Alibaba)]

JVET-P0077 CE8-1.3: Line-based CG Palette Mode [Y.-H. Chao, C.-H. Hung, W.-J. Chien, T. Hsieh, M. Karczewicz (Qualcomm)]

# Non-CE Technology proposals

## CE1 related – Reference picture resampling filters (9)

Contributions in this category were discussed XXday X Oct. XXXX–XXXX in Track X (chaired by XXX).

JVET-P0119 AHG8: On reference picture resampling (RPR) [J. Chen (Futurewei)	]

JVET-P0274 Non-CE1: Enabling TMVP in RPR [T.-S. Chang, Y.-C. Sun, L. Zhu, J. Lou (Alibaba)]

JVET-P0723 Crosscheck of JVET-P0274 (Non-CE1: Enabling TMVP in RPR) [Y.-W. Chen (Kwai Inc.)]

JVET-P0353 CE1-related: Reference picture resampling filters [J. Luo, Y. Ye, M. Sarwer (Alibaba)]

JVET-P0836 Cross-check of JVET-P0353 on reference picture resampling filters [V. Seregin (Qualcomm)]

JVET-P0381 Non-CE1: A fix on reference sample offset for RPR [K. Zhang, L. Zhang, H. Liu, Z. Deng, J. Xu, Y. Wang (Bytedance)]

JVET-P0382 Non-CE1: Simplified motion compensation in RPR [K. Zhang, L. Zhang, H. Liu, Z. Deng, J. Xu, N. Zhang (Bytedance)]

JVET-P0661 Crosscheck of JVET-P0382 (Non-CE1: Simplified motion compensation in RPR) [J. Chen (Alibaba)]

JVET-P0390 CE1-related: Anti-aliasing motion compensation interpolation downsampling filters for reference picture resampling [A. Alshin, J. Boyce, P. Frolov, V. Aristarkhov (Intel)] [late]

JVET-P0409 CE1-related: Enable PROF for RPR [J. Chen, R.-L. Liao, J. Luo, Y. Ye (Alibaba)]

JVET-P0655 Crosscheck of JVET-P0409 (CE1-related: Enable PROF for RPR) [K. Zhang (Bytedance)]

JVET-P0592 AHG8/Non-CE1: Phase shifts for resampling [V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-P0593 AHG8/CE1-related: Signalling filter coefficients for resampling [V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

## CE2 related – Gradual decoding refresh (0)

Contributions in this category were discussed XXday X Oct. XXXX–XXXX in Track X (chaired by XXX).

## CE3 related – Intra prediction and mode coding (48)

Contributions in this category were discussed XXday X Oct. XXXX–XXXX in Track X (chaired by XXX).

JVET-P0111 CE3-related: Chroma intra prediction mode mapping for 4:2:2 format [B. Wang, S. Esenlik, A. M. Kotra, H. Gao, E. Alshina (Huawei)]

JVET-P0136 Non-CE3: MIP simplification [J.-Y. Huo, H.-X. Wang, Y. Sun, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.), S. Wan (NPU), Y.-F. Yu, Y. Liu (OPPO)]

JVET-P0150 CE3-related: Simplification of reference luma intra prediction mode derivation [Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0768 Crosscheck of JVET-P0150 (CE3-related: Simplification of reference luma intra prediction mode derivation) [Y.-W. Chen (Kwai)]

JVET-P0177 Non-CE3: On Constraint of CCLM and CST [C.-W. Kuo, J. Li, C.S. Lim (Panasonic)]

JVET-P0649 Crosscheck of JVET-P0177 (Non-CE3: On Constraint of CCLM and CST) [T.-S. Chang (Alibaba)]

JVET-P0194 CE3-related: Simplified MIP with reduced memory footprint [T. Biatek, A.K. Ramasubramonian, G. Van Der Auwera, M. Karczewicz (Qualcomm)]

JVET-P0198 Non-CE3: Enable MIP prediction for 64×N or N×64 blocks at maximum transform size 32 [Z. Zhang, K. Andersson, D. Saffar, R. Sjöberg, J. Ström, R. Yu (Ericsson)]

JVET-P0199 Non-CE3: Align MIP matrix multiplication process [Z. Zhang, K. Andersson, D. Saffar, R. Sjöberg, J. Ström, R. Yu (Ericsson)]

JVET-P0749 Crosscheck of JVET-P0199 (Non-CE3: Align MIP matrix multiplication process) [Y. Yasugi (Sharp)]

JVET-P0208 Non CE3: Intra mode coding of non-MPM modes [S. Blasi, A. Seixas Dias, G. Kulupana (BBC)]

JVET-P0265 CE3-related: CCLM with unified filter shape [D.-Y. Kim (Chips&Media), S.-C. Lim, J. Lee, J. Kang, H. Lee (ETRI)]

JVET-P0812 Crosscheck of JVET-P0265 (CE3-related: CCLM with unified filter shape) [T. Toma, K. Abe (Panasonic)]

JVET-P0277 Non-CE3: LFNST restriction based on MIP [A. Kumar, S. Shrestha, B. Lee (Chosun Univ.), Y. Lee, J. Park (Humax)]

JVET-P0289 Non-CE3: MIP simplification [Y. Yasugi, T. Ikai (Sharp)]

JVET-P0294 Non-CE3: Simplification of MPM derivation [D. Gwon, H. Han, H. Choi (HNU), H. Lee, J. Kang (ETRI)]

JVET-P0822 Cross-check of JVET-P0294 (Non-CE3: Simplification of MPM derivation) [K. Choi (Samsung)]

JVET-O0297 Non-CE3: On CCLM neighboring sample [M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)]

JVET-P0763 Cross-check of JVET-P0297 (Non-CE3: On CCLM neighboring sample) [S.-C. Lim, H. Lee, J. Lee, J. Kang (ETRI)]

JVET-P0302 Non-CE3: PDPC without clipping [A. Filippov, V. Rufitskiy (Huawei)]

JVET-P0782 Crosscheck of JVET-P0302 (Non-CE3: PDPC without clipping) [Y. Morigami, M. Ikeda (Sony)]

JVET-P0303 Non-CE3: Removal of leaving out operation for 4×16 and 16×4 MIP blocks [X. Li, R.-L. Liao, Y. Chen, J. Chen, J. Luo, Y. Ye (Alibaba)]

JVET-P0751 Crosscheck of JVET-P0303 (Non-CE3: Removal of leaving out operation for 4×16 and 16×4 MIP blocks) [Y. Yasugi (Sharp)]

JVET-P0318 Non-CE3: Context reduction for intra_luma_not_planar_flag [D. Park, Y.-U. Yoon, J. Do, J.-G. Kim (KAU), J. Lee, J. Kang (ETRI)]

JVET-P0780 Crosscheck of JVET-P0318 (Non-CE3: Context reduction for intra_luma_not_planar_flag) [M. Ikeda (Sony)]

JVET-P0329 CE3-related: Simplification of Planar intra prediction [B. Wang, S. Esenlik, A. M. Kotra, H. Gao, E. Alshina (Huawei)]

JVET-P0840 Cross-check of JVET-P0329 (CE3-related: Simplification of Planar intra prediction) [M. G. Sarwer (Alibaba)]

JVET-P0352 Non-CE6 / Non-CE3: MIP UP TO 64x64 CU’s [K. Naser, T. Poirier, F. Le Léannec, F. Galpin (InterDigital)]

JVET-P0831 Cross-check result of JVET-P0352: Non-CE6 / Non-CE3: MIP UP TO 64x64 CU’s [Z. Zhang (Ericsson)]

JVET-P0355 Non CE3: Simplified coding of chroma intra modes [S. Blasi, A. Seixas Dias, G. Kulupana (BBC)]

JVET-P0633 Crosscheck of JVET-P0355 (Non CE3: Simplified coding of chroma intra modes) [?? (??)]

JVET-P0358 Non-CE3: Improved intra mode coding simplified [G. Rath, F. Galpin, F. Urban (InterDigital)]

JVET-P0777 Crosscheck of JVET-P0358 (Non-CE3: Improved intra mode coding simplified) [J. Choi (LGE)]

JVET-P0369 Non-CE3: MPM flag signaling with MRL [G. Rath, F. Galpin, F. Le Léannec (InterDigital)]

JVET-P0685	Crosscheck of JVET-P0369 (Non-CE3: MPM flag signaling with MRL) [J. Yao (Fujitsu)]

JVET-P0374 Non-CE3: One directional Intra planar mode [H. Yang, Y. He (InterDigital)]

JVET-P0716 Crosscheck of JVET-P0374: Non-CE3: One directional Intra planar mode [Y.-H. Chao (Qualcomm)]

JVET-P0398 Non-CE3: MIP Cleanup [J. Pfaff, B. Stallenberger, M. Schäfer, P. Merkle, T. Hinz, P. Helle, H. Schwarz, D. Marpe, T. Wiegand]

JVET-P0832 Cross-check result of JVET-P0398: Non-CE3: MIP cleanup [Z. Zhang (Ericsson)]

JVET-P0401 CE3-related: MIP downsampling process with re-trained MIP matrix [Z. Zhang, K. Andersson, D. Saffar, R. Sjöberg, J. Ström, R. Yu (Ericsson)]

JVET-P0793 Crosscheck of JVET-P0401 (CE3-related: MIP downsampling process with re-trained MIP matrix) [J. Pfaff (HHI)]

JVET-P0406 Non-CE3: Generalization of SCIPU for different YUV formats [L. Pham Van, G. Van der Auwera, V. Seregin, H. Huang, A. K. Ramasubramonian, M. Karczewicz (Qualcomm)]

JVET-P0701 Crosscheck of JVET-P0406 (Non-CE3: Generalization of SCIPU for different YUV formats) [Z. Deng (Bytedance)]

JVET-P0418 Non-CE3: Cleanup of MRLP Line Storage [T. Hellman, M. Zhou, B. Heng (Broadcom)]

JVET-P0484 Non-CE3: Signalling unification of prediction mode and SCIPU mode [L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, H. Huang, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-P0500 Non-CE3: Multiple reference sample set for CCLM [P.-H. Lin, C.-Y. Teng (Foxconn)]

JVET-P0718 Crosscheck of JVET-P0500 (Non-CE3: Multiple reference sample set for CCLM) [H.-J. Jhu (Kwai Inc.)]

JVET-P0503 Non-CE3: Simplification of harmonization for LFNST and MIP [S. Shrestha, A. Kumar, B. Lee (Chosun Univ.), Y. Lee, J. Park (Humax)]

JVET-P0507 Non-CE3: ISP simplifications [L. Zhao, X. Li, X. Zhao, S. Liu (Tencent)]

JVET-P0725 Crosscheck of JVET-P0507 (Non-CE3: ISP simplifications) [Y.-W. Chen (Kwai Inc.)]

JVET-P0509 CE3-related: Simplification for MIP matrix multiplication [L. Zhao, X. Zhao, X. Li, S. Liu (Tencent)]

JVET-P0726 Crosscheck of JVET-P0509 (CE3-related: Simplification for MIP matrix multiplication) [Y.-W. Chen (Kwai Inc.)]

JVET-P0510 Non-CE3/AHG8: On picture size restriction [L. Zhao, X. Zhao, X. Li, S. Liu (Tencent)]

JVET-P0511 Non-CE3: ISP extension to avoid small Chroma intra blocks [L. Zhao, X. Li, X. Zhao, S. Liu (Tencent)]

JVET-P0520 Non-CE3: Spec fix for the smallest chroma intra prediction unit (SCIPU) [Y.-W. Chen, X. Xiu, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)]

JVET-P0531 Non-CE3: Removal of 2×N chroma intra blocks [Z. Deng, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)]

JVET-P0805 Crosscheck of JVET-P0531 (Non-CE3: Removal of 2×N chroma intra blocks) [L. Pham Van, G. Van der Auwera (Qualcomm)]

JVET-P0535 Non-CE3: Removal of block size restriction in MIP [J. Choi, J. Heo, J. Lim, S. Kim (LGE)]

JVET-P0791 Crosscheck of JVET-P0535 (Non-CE3: Removal of block size restriction in MIP) [J. Pfaff (HHI)]

JVET-P0536 Non-CE3: MRL with non-MPM intra modes [J. Choi, J. Heo, J. Lim, S. Kim (LGE)]

JVET-P0787 Crosscheck of JVET-P0536 (Non-CE3: MRL with non-MPM intra modes) [F. Urban (InterDigital)]

JVET-P0537 Non-CE3: Cleanups on local dual tree for non-4:2:0 chroma formats [Z. Deng, J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)]

JVET-P0548 Non-CE3: Removal of MRL restriction [J. Heo, H. Jang, J. Choi, J. Lim, S. Kim (LGE)]

JVET-P0549 Non-CE3: Interpolation filter selection for chroma intra prediction [J. Heo, J. Choi, H. Jang, J. Lim, S. Kim (LGE)]

JVET-P0772 Crosscheck of JVET-P0549 (Non-CE3 : Interpolation filter selection for chroma intra prediction) [Y.-W. Chen (Kwai)]

JVET-P0550 Non-CE3: Cleanup of intra reference sample filter selection [J. Heo, J. Choi, J. Nam, H. Jang, J. Lim, S. Kim (LGE)]

JVET-P0773 Crosscheck of JVET-P0550 (Non-CE3 : Cleanup of intra reference sample filter selection) [Y.-W. Chen (Kwai)]

JVET-P0560 Non-CE3: Simplification on MIP and MPM context model [H.-J. Jhu, T.-C. Ma, X. Xiu, Y.-W. Chen, X. Wang (Kwai Inc.)]

JVET-P0694 Cross-check of JVET-P0560: Non-CE3: Simplification on MIP and MPM context model [H. Sun, J. Li (Panasonic)]

JVET-P0596 Non-CE3: Removal of chroma 2xN blocks in CIIP mode [L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, T Hsieh, V. Seregin, M. Karczewicz (Qualcomm)] [late]

Initial version rejected as “placeholder”

JVET-P0599 Non-CE3: Cleanup of interpolation filtering for intra prediction [A. Filippov, V. Rufitskiy (Huawei)]

JVET-P0788 Crosscheck of JVET-P0599 (Non-CE3: Cleanup of interpolation filtering for intra prediction) [S. Blasi, (BBC)]

JVET-P0615 Non-CE3: Intra chroma mode coding cleanup [B. Ray, G. Van der Auwera, M. Karczewicz (Qualcomm)]

JVET-P0790 Crosscheck of JVET-P0615 (Non-CE3: Intra chroma mode coding cleanup) [S. Blasi (BBC)]

JVET-P0625 Non-CE3: simplified MIP with power-of-two offsets [T. Biatek, A.K. Ramasubramonian, G. Van Der Auwera, M. Karczewicz (Qualcomm)] [late]

JVET-P0835 Cross-check result of JVET-P0625: Non-CE3: simplified MIP with power-of-two offset [Z. Zhang (Ericsson)]

JVET-P0626 Non-CE3: Cleanup of reference sample padding for intra prediction [A. Filippov, V. Rufitskiy (Huawei)] [late]

JVET-P0781 Crosscheck of JVET-P0626 (Non-CE3: Cleanup of reference sample padding for intra prediction) [Y. Morigami, M. Ikeda (Sony)]

JVET-P0638 CE3-related: Adaptive coding subset for intra mode [M. Bhat, D. Gommelet, J.-M. Thiesse, D. Nicholson (VITEC)] [late] [miss]

JVET-P0641 Non-CE3: Combination of JVET-P0596 and JVET-P0531 on removal of 2xN chroma intra blocks [L. Pham Van, G. Van der Auwera, T. Hsieh, A. K. Ramasubramonian, V. Seregin, M. Karczewicz (Qualcomm), Z. Deng, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)] [late]

JVET-P0778 Crosscheck of JVET-P0641 (Non-CE3: Combination of JVET-P0596 and JVET-P0531 on removal of 2xN chroma intra blocks) [J. Choi (LGE)]

JVET-P0802 Non-CE3: PDPC spec and software mismatch [H. Yang, Y. He, H. Li (InterDigital)] [late]

JVET-P0803 Non-CE3: Combined cleanup of MIP [J. Pfaff, B. Stallenberger, M. Schäfer, P. Merkle, T. Hinz, P. Helle, H. Schwarz, D. Marpe, T. Wiegand (HHI), T. Biatek, A. K. Ramasubramonian, G. Van der Auwera, M. Karczewicz (Qualcomm), H. Liu, L. Zhang, K. Zhang, Z. Deng, J. Xu (Bytedance), J. Choi, J. Heo, J. Lim, S. Kim (LGE), K. Naser, T. Poirier, F. Le Leannec, F. Galpin (Inter Digital)] [late]

## CE4 related – Inter prediction (104)

Contributions in this category were discussed XXday X Oct. XXXX–XXXX in Track X (chaired by XXX).

JVET-P0089 Non-CE4: DMVR control by reference picture type [H. Huang, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-P0090 Non-CE4: 32 bits coding for abs_mvd_minus2 [H. Huang, M. Coban, H. Wang, V. Seregin, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-P0673 Crosscheck of JVET-P0090 (Non-CE4: 32 bits coding for abs_mvd_minus2) [C.-Y. Lai (MediaTek)]

JVET-P0091 CE4-related: Simplification of PROF and BDOF [H. Huang, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-P0094 AHG16: Fix on overflow issue in PROF [H. Chen, H. Yang (Huawei)]

JVET-P0107 CE4-Related: Geometric Merge Mode (GEO) Simplifications [H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang (Huawei), M. Bläser, J. Sauer (RWTH Aachen Uni.)]

JVET-P0677 Crosscheck of JVET-P0107 (CE4-related: Geometric merge mode simplifications) [Y.-L. Hsiao (MediaTek)]

JVET-P0728 Crosscheck of JVET-P0107 on reducing number of GEO modes and removing flipping [R.-L. Liao (Alibaba)]

JVET-P0137 Non-CE4: On coding of merge triangle index [J.-Y. Huo, H.-X. Wang, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.), S. Wan (NPU), Y.-F. Yu, Y. Liu (OPPO)]

JVET-P0151 CE4-related: Simplification of half-pel switchable interpolation filter [Y.-L. Hsiao, C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0152 CE4-related: On maximum number of subblock-based merging candidates [O. Chubach, C.-C. Chen, C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0153 CE4-related: Overlapped block optical flow [Y.-L. Hsiao, Y.-C. Lin, C.-C. Chen, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0784 Crosscheck of JVET-P0153 (CE4-related: Overlapped block optical flow) [H. Lee, S.-C. Lim, J. Lee, J. Kang (ETRI)]

JVET-P0154 CE4-related: PROF prediction sample range reduction [T.-D. Chuang, Z.-Y. Lin, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0769 Crosscheck of JVET-P0154 TEST1, TEST3 (CE4-related: PROF prediction sample range reduction) [Y.-W. Chen (Kwai)]

JVET-P0742 Crosscheck of JVET-P0254 (Issue of simplified luma mapping of LMCS) [T. Chujoh (Sharp)]

JVET-P0155 CE4-related: An encoder switch for forcing temporal MV to zero in SbTMVP [Y.-L. Hsiao, C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0172 AHG16/Non-CE4: Prediction sample value clipping for PROF [J. Li, C. S. Lim (Panasonic)]

JVET-P0174 Non-CE4: Modifications of TPM and GEO [J. Li, C. S. Lim (Panasonic)]

JVET-P0175 AHG16/Non-CE4: A clean-up to merge list generation by removing shared merge list [J. Li, C. S. Lim (Panasonic)]

JVET-P0191 Enabling BDOF and DMVR according to reference picture types [N. Zhang, H. Liu, L. Zhang, K. Zhang, Y. Wang (Bytedance)]

JVET-P0200 Non-CE4: Modifications on BDOF intermediate parameter derivation [D. Liu, R.Yu (Ericsson)]

JVET-P0678 Crosscheck of JVET-P0200 (Non-CE4: Modifications on BDOF intermediate parameter derivation-Test 1) [F. Chen (Hikvision)]

JVET-P0201 Non-CE4: Simplification on BDOF offset calculation [D. Liu, R. Yu (Ericsson)]

JVET-P0734 Crosscheck of JVET-P0201 on simplification on BDOF offset calculation [X. Xiu (Kwai Inc.)]

JVET-P0209 Non-CE4: On the affine AMVR [H. Huang, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-P0698 Cross-check of JVET-P0209: Non-CE4: On the affine AMVR [H. Sun, J. Li (Panasonic)]

JVET-P0214 CE4-related: Simplification of the CIIP chroma Intra Prediction Mode [J. Park, B. Jeon (SKKU)]

JVET-P0234 AHG-2/Non-CE4: On clarification of weightedPredFlag [T. Chujoh, T. Ikai (Sharp)]

JVET-P0236 Non-CE4: On a simplification for triangle merge mode [T. Chujoh, E. Sasaki, T. Ikai (Sharp)]

JVET-P0690 Crosscheck of JVET-P0236 (Non-CE4: On a simplification for triangle merge mode) [P. Bordes (InterDigital)]

JVET-P0811 Crosscheck of JVET-P0236 (Non-CE4: On a simplification for triangle merge mode) [K. Abe (Panasonic)]

JVET-P0237 Non-CE4: On conditions for PROF [T. Chujoh, E. Sasaki, T. Ikai (Sharp)]

JVET-P0833 Crosscheck of JVET-P0237 (Non-CE4: On conditions for PROF) [H. Liu (Bytedance)]

JVET-P0238 Non-CE4: On definition of motion vector range for PROF and BDOF [T. Chujoh, E. Sasaki, T. Ikai (Sharp)]

JVET-P0248 CE4-related: Modification of GEO mode [L. Xu, X. Cao, Y. Sun, F. Chen, L. Wang (Hikvision)]

JVET-P0729 Crosscheck of JVET-P0248 (CE4-related: On Modifications of GEO) [R.-L. Liao (Alibaba)]

JVET-P0249 CE4-related: On simplification for CIIP with triangular partitions [Y. Sun, F. Chen, L. Wang (Hikvision)]

JVET-P0789 Crosscheck of JVET-P0249 (CE4-related: On simplification for CIIP with triangular partitions) [S. Blasi, (BBC)]

JVET-P0250 CE4-related: On simplification for GEO weight derivation [Y. Sun, F. Chen, L. Wang (Hikvision)]

JVET-P0253 Non-CE4: Simplify the division process of DMVR [K. Abe, Y. Kato, T. Toma (Panasonic)]

JVET-P0741 Crosscheck of JVET-P0253 (Non-CE4: Simplify the division process of DMVR) [H. Tsurusaki, K. Unno (KDDI)]

JVET-P0260 CE4-related: On inheritance of half-pel interpolation filter in merge mode [S. H. Wang (PKU)]

JVET-P0695 Cross-check of JVET-P0260: CE4-related: On inheritance of half-pel interpolation filter in merge mode [H. Sun, J. Li (Panasonic)]

JVET-P0261 Non-CE4: On Affine Motion Vector Restriction [X. W. Meng (PKU)]

JVET-P0263 Non-CE4: On PROF conditions [X. W. Meng (PKU)]

JVET-P0679 Crosscheck of JVET-P0263 (Non-CE4: On PROF conditions) [F. Chen (Hikvision)]

JVET-P0264 CE4-related: Simplification of GEO using angles with power-of-two tangents [K. Reuzé, C.-C Chen, H. Huang, W.-J Chien, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-P0722 Crosscheck of JVET-P0264 (CE4-related: Simplification of GEO using angles with power-of-two tangents) [H.-J. Jhu (Kwai Inc.)]

JVET-P0278 Non-CE4: HMVP buffer update for TPM block [N. Park, J. Nam, H. Jang, J. Lim, S. Kim (LGE)]

JVET-P0660 Crosscheck of JVET-P0278 (Non-CE4: HMVP buffer update for TPM block) [N. Zhang (Bytedance)]

JVET-P0275 Non-CE4/AHG17: On slice-level syntax for TPM and GEO [A. Filippov, V. Rufitskiy (Huawei)]

Move to non-CE4.

JVET-P0753 Crosscheck of JVET-P0275 (Non-CE4/AHG17: On slice-level syntax for TPM and GEO) and JVET-P0320 (Non-CE4/AHG17: On slice-level syntax for BDOF and DMVR) [K. Unno (KDDI)]

Move to non-CE4.

JVET-P0829 Cross-check result of JVET-P0275 (Non-CE4/AHG17: On slice-level syntax for TPM and GEO) [K. Kawamura, S. Naito (KDDI)]

JVET-P0320 Non-CE4/AHG17: On slice-level syntax for BDOF and DMVR [A. Filippov, V. Rufitskiy (Huawei)]

Move to non-CE4.

JVET-P0830 Cross-check result of JVET-P0320 (Non-CE4/AHG17: On slice-level syntax for BDOF and DMVR) [K. Kawamura, S. Naito (KDDI)]

JVET-P0279 Non-CE4: Interaction between PROF and other tools [N. Park, J. Nam, H. Jang, J. Lim, S. Kim (LGE)]

JVET-P0280 Non-CE4: Fix the behavior between BCW and WP [N. Park, J. Nam, H. Jang, J. Lim, S. Kim (LGE)]

JVET-P0281 Non-CE4: Corrections on parameter calculation for PROF and BDOF [N. Park, J. Nam, H. Jang, J. Lim, S. Kim (LGE)]

JVET-P0282 Non-CE4: Block size alignment between PROF and BDOF [N. Park, J. Nam, H. Jang, J. Lim, S. Kim (LGE)]

JVET-P0285 Non-CE4: Intra prediction mode of CIIP selection using left adjacent coded block [D. Jiang, J. Lin, F. Zeng, C. Fang (Dahua)]

JVET-P0286 Non-CE4: Intra prediction mode of CIIP selection using left and above adjacent coded block [D. Jiang, J. Lin, F. Zeng, C. Fang (Dahua)]

JVET-P0287 Non-CE4: Modified MVD derivation method for bidirectional MMVD using the magnitude of MV [D. Jiang, J. Lin, F. Zeng, C. Fang (Dahua)]

JVET-P0288 Non-CE4: Modified MVD derivation method for bidirectional MMVD using the magnitude of MV and POC distance [D. Jiang, J. Lin, F. Zeng, C. Fang (Dahua)]

JVET-P0291 AHG17: MMVD and CIIP parameters in PPS or slice header [Y. Yasugi, E. Sasaki, T. Ikai (Sharp)] [late]

This proposes moving some control from the SPS to lower levels.

JVET-P0304 CE4-related: Simplification of blending weights and motion field storage in geometric merge mode [R.-L. Liao, J. Chen, Y. Ye, J. Luo (Alibaba)]

JVET-P0305 CE4-related: Unification of triangle partition mode and geometric merge mode [R.-L. Liao, J. Chen, Y. Ye, J. Luo (Alibaba)]

JVET-P0309 Non-CE4: Improved signaling method for merge modes [F. Chen, L. Wang (Hikvision)]

JVET-P0310 Non-CE4: On simplification of PROF and BDOF [F. Chen, L. Wang (Hikvision)

JVET-P0311 Non-CE4: On enabling condition of BDOF and DMVR [F. Chen, L. Wang (Hikvision)]

JVET-P0750 Crosscheck of JVET-P0311 (Non-CE4: On Enabling Condition of BDOF and DMVR) [D. Liu, R. Yu (Ericsson)]

JVET-P0312 Non-CE4: On condition of SMVD [F. Chen, L. Wang (Hikvision)]

JVET-P0752 Crosscheck of JVET-P0312 (Non-CE4: On Condition of SMVD) [D. Liu, R. Yu (Ericsson)]

JVET-P0314 Non-CE4: Harmonization of PROF, BDOF and DMVR syntax [X.W. Meng (PKU), X. Zheng (DJI), S.S. Wang, S.W. Ma (PKU)]

JVET-P0317 Non-CE4: BCW clean-up for weight signaling [D. Park, Y.-U. Yoon, J. Do, J.-G. Kim (KAU), J. Lee, J. Kang (ETRI)]

JVET-P0675 Crosscheck of JVET-P0317 (Non-CE4: BCW cleanup for weight signalling) [Y.-L. Hsiao (MediaTek)]

JVET-P0322 Non-CE4: CIIP size restriction [G. Ko, D. Kim, J. Jung, J. Son, J. Kwak (WILUS)]

JVET-P0325 Non-CE4: Construction of spatial merge candidates [Z. Wang, Y. Yan, J. Luo (Alibaba)]

JVET-P0340 Non-CE4: Unification of merge interpolation filter (triangle and pairwise-average) [P. Bordes, A. Robert, F. Le Léannec, F. Galpin (InterDigital)]

JVET-P0658 Crosscheck of JVET-P0340 (Non-CE4: Unification of merge interpolation filter (triangle and pairwise-average)) [Y. Wang (Bytedance)]

JVET-P0341 Non-CE4: Coding of interpolation filter index in non-merge inter mode [P. Bordes, F. Galpin, F. Le Léannec, E. François (InterDigital)]

JVET-P0744 Crosscheck of JVET-P0341 (Non-CE4: Coding of interpolation filter index in non-merge inter mode) [T. Chujoh (Sharp)]

JVET-P0384 Non-CE4: Flexible MMVD candidates [F. Galpin, F. Le Léannec, A. Robert (InterDigital)] [late]

JVET-P0385 Non-CE4: MV rounding in sbTMVP [K. Zhang, L. Zhang, H. Liu, Z. Deng, N. Zhang, Y. Wang (Bytedance)]

JVET-P0387 Non-CE4: Flexible CIIP mode [F. Galpin, F. Le Léannec, A. Robert, K. Naser (InterDigital)] [late]

JVET-P0408 CE4-related: On PROF on/off control [J. Chen, R.-L. Liao, J. Luo, Y. Ye (Alibaba)]

JVET-P0413 CE4-related: Clipping for PROF [W. Chen, Y. He (InterDigital)]

JVET-P0697 Cross-check of JVET-P0413: CE4-related: Clipping for PROF [H. Sun, J. Li (Panasonic)]

JVET-P0414 Non-CE4: Simplification of SIF [W. Chen, Y. He (InterDigital)]

JVET-P0415 Non-CE4: Unification of DMVR and BDOF enabling conditions [W. Chen, Y. He (InterDigital)]

JVET-P0817 Crosscheck of JVET-P0415 (Non-CE4: Unification of DMVR and BDOF enabling conditions) [N. Park (LGE)]

JVET-P0416 Non-CE4: Using integer MV in CIIP [W. Chen, Y. He (InterDigital)]

JVET-P0755 Crosscheck of JVET-P0416 on using integer MV in CIIP [X. Xiu (Kwai Inc.)]

JVET-P0434 Non-CE4: Enabling conditions of PROF [Z. Deng, L. Zhang, K. Zhang, H. Liu, N. Zhang, K. Fan (Bytedance)]

JVET-P0439 Non-CE4: Motion pruning with alternative half-pel interpolation filter flag [G. Li, X. Li, X. Xu, S. Liu (Tencent)]

JVET-P0828 Crosscheck of JVET-P0439 (Non-CE4 Motion pruning with alternative half-pel interpolation filter flag) [A. Robert (InterDigital)]

JVET-P0440 Non-CE4: On inheritance of alternative half-pel interpolation filter flag in MMVD [G. Li, X. Li, X. Xu, S. Liu (Tencent)]

JVET-P0646 Cross-check of JVET-P0440 (Non-CE4: On inheritance of alternative half-pel interpolation filter flag in MMVD) [K. Zhang (Bytedance)]

JVET-P0442 Non-CE4: Unified AMVR signalling [G. Li, X. Li, X. Xu, S. Liu (Tencent)]

JVET-P0443 Non-CE4: On DMVR early termination [G. Li, X. Li, X. Xu, S. Liu (Tencent)]

JVET-P0735 Crosscheck of JVET-P0443 on DMVR early termination [X. Xiu (Kwai Inc.)]

JVET-P0444 Non-CE4: On DMVR without padding [G. Li, X. Li, X. Xu, S. Liu (Tencent)]

JVET-P0736 Crosscheck of JVET-P0444 on DMVR without padding [X. Xiu (Kwai Inc.)]

JVET-P0445 Non-CE4: Encoder optimization for subblock-based merge candidate search [G. Li, X. Li, X. Xu, S. Liu (Tencent)]

JVET-P0714 Cross-check of JVET-P0445: Non-CE4: Encoder optimization for subblock-based merge candidate search [J. Zhao (LGE)]

JVET-P0449 CE4-related: Thin Partition Mode [K. Panusopone, S. Hong, L. Wang (Nokia)]

JVET-P0779 Cross-check of JVET-P0449 (CE-4 related: Thin Partition Mode) [M. Zhou (Broadcom)]

JVET-P0461 Non-CE4: Usage of half-pel switchable interpolation filter for pairwise candidate [N. Zhang, H. Liu, L. Zhang, K. Zhang, Y. Wang (Bytedance)]

JVET-P0490 Non-CE4: Cleanup of half-pel switchable interpolation filter [H. Liu, N. Zhang, L. Zhang, K. Zhang, Z. Deng, J. Xu (Bytedance)]

JVET-P0491 CE4-related: On MVD derivation in PROF [H. Liu, L. Zhang, K. Zhang, Y. Wang, J. Xu, Y. Wang (Bytedance)]

JVET-P0745 Crosscheck of JVET-P0491 (CE4-related: On MVD derivation in PROF) [T. Chujoh (Sharp)]

JVET-P0496 Non-CE4: Switchable interpolation filter in TPM [Y. Wang, L. Zhang, H. Liu, K. Zhang, N. Zhang, Y. Wang (Bytedance)]

JVET-P0689 Crosscheck of JVET-P0496 (Non-CE4: Switchable interpolation filter for TPM) [P. Bordes (InterDigital)]

JVET-P0498 Non-CE4: On weight derivation process in CIIP [N. Zhang, H. Liu, L. Zhang, K. Zhang, Y. Wang (Bytedance)

JVET-P0818 Crosscheck of JVET-P0498 (Non-CE4: On weight derivation process in CIIP) [N. Park (LGE)]

JVET-P0499 Non-CE4: Removal of MVD scaling process in MMVD [N. Zhang, H. Liu, L. Zhang, K. Zhang, Y. Wang (Bytedance)]

JVET-P0645 Cross-check of JVET-P0499: Non-CE4: Removal of MVD scaling process in MMVD [J. Zhao (LGE)]

JVET-P0512 Non-CE4: SIMD support for motion compensated prediction at high internal bit-depth [X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)]

JVET-P0513 Non-CE4: Extending switchable interpolation filter to affine mode [X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)]

JVET-P0518 AHG16/Non-CE4: Addressing 16-bit multiplication overflow issue of the PROF [X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)]

JVET-P0519 Non-CE4: On SAD threshold for BDOF early termination [X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)]

JVET-P0524 On high-level control flags of BDOF and PROF [X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)]

JVET-P0530 Non-CE4: Alignment of luma and chroma weight calculation for TPM blending [Z. Deng, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)]

JVET-P0532 CE4-related: Modified hpelIfIdx derivation for half-pel interpolation filter [Y. Kidani, K. Kawamura, K. Unno, S. Naito (KDDI)]

JVET-P0533 CE4-related: Motion vector rounding in enabled hpelIfIdx [Y. Kidani, K. Kawamura, K. Unno, S. Naito (KDDI)]

JVET-P0541 Non-CE4: Context modeling for inter prediction mode [J. Nam, H. Jang, N. Park, J. Lim, S. Kim (LGE)]

JVET-P0542 Non-CE4: Cleanup on alternative half-pel interpolation filter [J. Nam, H. Jang, N. Park, J. Lim, S. Kim (LGE)]

JVET-P0544 Non-CE4: A slice header flag disabling PROF [K. Unno, K. Kawamura, S. Naito (KDDI)]

JVET-P0546 Non-CE4: An applying condition of BDOF [K. Unno, K. Kawamura, S. Naito (KDDI)]

JVET-P0810 Crosscheck of JVET-P0546 (Non-CE4: An applying condition of BDOF) [K. Abe (Panasonic)]

JVET-P0572 AHG17/Non-CE4: Restrict integer-pel fetch for BDOF and RPOF for subpicture [H. Jang, J. Nam, N. Park, S. Paluri, S. Kim, J. Lim (LGE)]

JVET-P0583 CE4-related: On the SBT in CE4-1.1 [H. Huang, T. Hsieh, V. Seregin, K. Reuze, C.-C. Chen, H.E. Egilmez, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-P0595 Non-CE4: Neighboring locations for CIIP [G. Ko, D. Kim, J. Jung, J. Son, J. Kwak (WILUS)]

JVET-P0598 CE4-related: Harmonized CU-level Condition Check in BDOF and DMVR [C.-C. Chen, H. Huang, K. Reuzé, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-P0600 CE4-related: Support large rotation and flipping in affine and PROF [A. Aminlou (Nokia)]

JVET-P0664 Cross-check of JVET-P0600: CE4-related: Support large rotation and flipping in affine and PROF [Y. He (InterDigital)]

JVET-P0601 CE4-related: Support slice level disabling for PROF [K. Kondo, M. Ikeda (Sony)] [late]

JVET-P0604 Non-CE4: Motion field storage optimization for the line buffer [T. Solovyev, S. Ikonin, A. Karabutov, R. Chernyak (Huawei)]

JVET-P0605 Non-CE4: On triangular merge list size signalling [T. Solovyev, S. Ikonin, A. Karabutov, R. Chernyak (Huawei)]

JVET-P0617 Non-CE4: On TPM merge mode in the presence of weighted prediction [A. Filippov, V. Rufitskiy (Huawei)] [late] [miss]

JVET-P0620 Non-CE4: Flexible SBT mode [F. Galpin, F. Le Léannec, A. Robert, K. Naser, E. François (InterDigital)] [late]

Initial version rejected as “placeholder”

JVET-P0621 Non-CE4: On motion information comparison [A. Robert, F. Le Léannec, F. Galpin, T. Poirier (InterDigital)] [late]

JVET-P0629 CE4-related: CIIP Simplification [J. Y. Lee (Sejong Univ.), W. Lim, G. Bang (ETRI)] [late]

JVET-P0821 Cross-check of JVET-P0629 (CE4-related: CIIP Simplification) [K. Choi (Samsung)]

JVET-P0653 Non-CE4: On BDOF and PROF parameter derivation [X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai)] [late]

JVET-P0654 Non-CE4: Unified BDOF and DMVR early termination threshold [X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai)] [late]

JVET-P0663 CE4-1.14 related: Block size limitation of enabling TPM and GEO [Z. Deng, L. Zhang, H. Liu, K. Zhang, Y. Wang (Bytedance)] [late]

JVET-P0785 Crosscheck of JVET-P0663 (CE4-1.14 related: Block size limitation of enabling TPM and GEO) [T.-H. Li, C.-Y. Teng (Foxconn)]

JVET-P0746 CE4-related: Combination of JVET-P0264 and JVET-P0304 on GEO simplification [K. Reuzé, C.-C. Chen, H. Huang, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm), R.-L. Liao, J. Chen, Y. Ye, J. Luo (Alibaba)] [late]

## CE5 related – Loop filtering (71)

Contributions in this category were discussed XXday X Oct. XXXX–XXXX in Track X (chaired by XXX).

JVET-P0043 AHG16/Non-CE5: A cleanup for de-blocking in the affine and TPM mode [B. Heng, M. Zhou (Broadcom)]

JVET-P0656 Crosscheck of JVET-P0043 (AHG16/Non-CE5: A cleanup for de-blocking in the affine and TPM mode) [J. Xu (Bytedance)]

JVET-P0053 AHG16/Non-CE5: A cleanup for the ALF sample padding [M. Zhou (Broadcom)]

JVET-P0086 AHG16/Non-CE5: Deblocking boundary strength fix for Affine and TPM [A. M. Kotra, S. Esenlik, H. Gao, B. Wang, E. Alshina (Huawei)]

JVET-P0844 Crosscheck of JVET-P0086 (AHG16/Non-CE5: Deblocking boundary strength fix for Affine and TPM) [C.-M. Tsai (MediaTek)]

JVET-P0087 Non-CE5: Deblocking boundary strength modification for triangle and affine mode [K. Misra, F. Bossen, A. Segall (Sharp Labs of America)]

JVET-P0845 Crosscheck of JVET-P0087 (Non-CE5: Deblocking boundary strength modification for triangle and affine mode) [C.-M. Tsai (MediaTek)]

JVET-P0105 Non-CE5: Modified Chroma QP derivation for deblocking filter [A. M. Kotra, S. Esenlik, B. Wang, H. Gao, E. Alshina (Huawei)]

JVET-P0846 Crosscheck of JVET-P0105 (Non-CE5: Modified Chroma QP derivation for deblocking filter) [C.-M. Tsai (MediaTek)]

JVET-P0106 AHG16/CE5-Related: Simplifications for Cross Component Adaptive Loop Filter [A. M. Kotra, S. Esenlik, B. Wang, H. Gao, E. Alshina (Huawei)]

JVET-P0847 Crosscheck of JVET-P0106 (AHG16/CE5-related: Simplifications for cross component adaptive loop filter) [C.-M. Tsai (MediaTek)]

JVET-P0109 AHG18/Non-CE5: Boundary strength fix for coding units using BCW [A. M. Kotra, S. Esenlik, B. Wang, H. Gao, E. Alshina (Huawei)]

JVET-P0848 Crosscheck of JVET-P0109 (AHG18/Non-CE5: Boundary strength fix for coding units using BCW) [C.-M. Tsai (MediaTek)]

JVET-P0121 On adaptive loop filter [J. Chen, Hendry (Futurewei)]

JVET-P0156 CE5-related: ALF padding process when raster scan slices are used [C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek), A. M. Kotra, S. Esenlik, H. Gao, B. Wang, E. Alshina (Huawei)]

JVET-P0157 CE5-related: Align ALF padding processes at picture and subpicture boundaries [C.-Y. Lai, O. Chubach, L. Chen, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0158 CE5-related: Align ALF virtual boundary processes in the bottom CTU rows of one picture and one subpicture [C.-Y. Lai, O. Chubach, L. Chen, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0159 CE5-related: DMVR deblocking by inheriting neighbouring boundary strength values [C.-M. Tsai, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0160 CE5-related: Unification of deblocking processes for transform block and prediction block boundaries [C.-M. Tsai, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0792 Crosscheck of JVET-P0160 (CE5-related: Unification of deblocking processes for transform block and prediction block boundaries) [Y. Han, N. Hu (Qualcomm)	]

JVET-P0161 CE5-related: Deblocking considering prediction weights in BCW and TPM [C.-M. Tsai, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0162 CE5-related: Simplified ALF syntax with removal of alf_ctb_use_first_aps_flag [Y.-L. Hsiao, C.-C. Chen, O. Chubach, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0748 Cross-check of JVET-P0162 (CE5-related: Simplified ALF syntax with removal of alf_ctb_use_first_aps_flag) [K. Andersson (Ericsson)]

JVET-P0163 CE5-related: SAO encoder-only improvements [C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, O. Chubach, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0820 Cross-check of JVET-P0163 (CE5-related: SAO encoder-only improvements) [K. Choi, W. Choi, K. P. Choi (Samsung)]

JVET-P0164 CE5-related: Simplified ALF syntax regarding to alf_luma_coeff_signalled_flag and alf_luma_coeff_flag [C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, O. Chubach, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0165 CE5-related: Simplified CCALF [O. Chubach, C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0173 AHG16/Non-CE5: Cross component ALF simplification [J. Li, C. S. Lim (Panasonic)]

JVET-P0178 CE5-related: Unification of CCALF and ALF [C.-W. Kuo, J. Li, C.S. Lim (Panasonic)]

JVET-P0717 Crosscheck of JVET-P0178 (CE5-related: Unification of CCALF and ALF) [H.-J. Jhu (Kwai Inc.)]

JVET-P0179 Non-CE5: ALF as a post-filter [H.-B. Teo, H.-W. Sun, C.-S. Lim (Panasonic)]

JVET-P0180 Non-CE5: Cleanup of ALF syntax elements [H. Liu, L. Zhang, K. Zhang, N. Zhang, Y. Wang, Y. Wang (Bytedance)]

JVET-P0672 Crosscheck of JVET-P0180 (Non-CE5: Cleanup of ALF syntax elements) [C.-Y. Lai (MediaTek)]

JVET-P0192 CE5-related: Reducing the number of luma filters in ALF [O. Chubach, C.-Y. Chen, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0767 Crosscheck of JVET-P0192 (CE5-related: Reducing the number of luma filters in ALF) [Y.-W. Chen (Kwai)]

JVET-P0195 Non-CE5: Unified strong deblocking filter for luma and chroma [Y. Ahn, D. Sim (Digital Insights)]

JVET-P0760 Cross-check of JVET-P0195 (Non-CE5: Unified strong deblocking filter for luma and chroma) [S.-C. Lim, H. Lee, J. Lee, J. Kang (ETRI)]

JVET-P0246 AHG17/Non-CE5: on loop filter processing for subpicture treated as a picture

See 6.19.1.1 for the disposition of the HLS aspects of this contribution.

TBP for the low-level signal processing aspects.

JVET-P0247 Non-CE5: Intra deblocking coefficients for weak filters [L.-H. Xu, J.-Q. Zhu, K. Kazui (Fujitsu)]

JVET-P0251 CE5-related: Simplified CCALF [Y. Zhao, H. Yang (Huawei)]

JVET-P0849 Crosscheck of JVET-P0251 (CE5-related: Simplified CCALF with 6 filter coefficients) [C.-M. Tsai (MediaTek)]

JVET-P0255 CE5-related: Bilateral filter with SAO band offset [K. Abe, T. Toma (Panasonic)]

JVET-P0262 AHG16/Non-CE5: Deblocking boundary design cleanup for affine and TPM mode [X. W. Meng (PKU)]

JVET-P0269 AHG18 / non-CE5: Deblocking for TPM and BCW

JVET-P0290 CE5-related: Clipping of intermediate value in CC-ALF [Y. Yasugi, T. Ikai (Sharp)]

JVET-P0668 Crosscheck of JVET-P0290 (CE5-related: Clipping of intermediate value in CC-ALF) [A. Nalci (Qualcomm)]

JVET-P0299 CE5-related: Interaction between Bilateral Filter and Cross-Component Adaptive Loop Filter [C. Hollmann, J. Ström (Ericsson)]

JVET-P0710 Cross check report of JVET-P0299 [K. Misra (Sharp Labs of America)]

JVET-P0306 Non CE5: DMVR internal edge deblocking without using refined MVs [J. R. Arumugam, S. Kotecha, S. Ramamurthy (Ittiam)]

JVET-P0743 Crosscheck of JVET-P0306 (Non CE5: DMVR internal edge deblocking without using refined MVs) [T. Chujoh (Sharp)]

JVET-P0330 Non-CE5: CC-ALF filtering simplification [J. Taquet, P. Onno, C. Gisquet, G. Laroche (Canon)]

JVET-P0331 Non-CE5: CC-ALF design simplification [J. Taquet, P. Onno, C. Gisquet, G. Laroche (Canon)]

JVET-P0801 Crosscheck of JVET-P0332: AHG17/Non-CE5: On parsability of alternative ALF Chroma filter signaling [S. Esenlik (Huawei)]

JVET-P0333 On SAO performance [P. Onno, G. Laroche (Canon)]

JVET-P0372 CE5-related: Joint chroma cross-component adaptive loop filtering [H. Yang, Y. He, H. Li (InterDigital)]

JVET-P0704	Crosscheck of JVET-P0372 on joint chroma cross-component adaptive loop filtering [X. Li (Tencent)]

JVET-P0386 Non-CE8/Non-CE5: Deblocking for IBC blocks [H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang (Huawei)]

JVET-P0411 Non-CE5: Fixes for long luma deblocking filter decision [K. Andersson, J. Enhorn (Ericsson)]

JVET-P0412 CE5-related: On further modification of Hadamard filter from CE5-3.2 [S. Ikonin, V. Stepin, A. Karabutov, S. Nikolaeva (Huawei)]

JVET-P0423 Non-CE5: Modified boundary strength derivation [W. Zhu, L. Zhang, J. Xu (Bytedance)]

JVET-P0635 Crosscheck of JVET-P0423 (Non-CE5: Modified boundary strength derivation) [T.-H. Li, C.-Y. Teng (Foxconn)]

JVET-P0441 Non-CE5: On deblocking on affine internal prediction sub-block edges [G. Li, X. Li, X. Xu, S. Liu (Tencent)]

JVET-P0452 Non-CE5/AHG17: Low-delay ALF syntax [K. Sühring, R. Skupin, Y. Sanchez, T. Schierl (HHI)]

JVET-P0468 CE5-Related: Reducing multiplier count in CC-ALF [K. Misra, F. Bossen, A. Segall (Sharp Labs of America)]

JVET-P0470 Non-CE5: Simplification of Cross Component Adaptive Loop Filter [X. Li, X. Zhao, L. Zhao, Y. Du, S. Liu (Tencent)]

JVET-P0738 Crosscheck report of JVET-P0470 [K. Misra (Sharp Labs of America)]

JVET-P0492 Non-CE5: Fixes of ALF sample padding [H. Liu, L. Zhang, K. Zhang, Y. Wang, J. Xu, Y. Wang (Bytedance)]

JVET-P0504 CE5-related: Harmonization of Hadamard filter, SAO and ALF [V. Stepin, S. Ikonin, A. Karabutov (Huawei)]

JVET-P0505 Non-CE5: On non-linear ALF clipping values [Y. Du, X. Zhao, X. Li, S. Liu (Tencent)]

JVET-P0708 Crosscheck of JVET-P0505 (Non-CE5: On non-linear ALF clipping values) [H. Yang (InterDigital)]

JVET-P0534 AhG16/Non-CE5: On deblocking at ALF virtual boundaries [Z. Deng, L. Zhang, K. Zhang, H. Liu, H.-C. Chuang (Bytedance)]

JVET-P0727 Crosscheck of JVET-P0534 (AhG16/Non-CE5: On deblocking at ALF virtual boundaries) [Y.-W. Chen (Kwai Inc.)]

JVET-P0539 Non-CE5: A chroma deblocking clean-up [J. Xu, L. Zhang, W. Zhu, K. Zhang, H. Liu (Bytedance)]

JVET-P0543 CE5-related: Cleanup on cross-component adaptive loop filter [J. Nam, J. Choi, J. Lim, S. Kim (LGE)]

JVET-P0547 Non-CE5: Deblocking filter at PROF sub-block boundary [K. Unno, K. Kawamura, S. Naito (KDDI)]

JVET-P0551 AHG16/Non-CE5: On ALF boundary padding [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-P0552 AHG12/Non-CE5: Extending slice boundary processing for adaptive loop filter for raster scanned slices [N. Hu, V. Seregin, M. Coban, A. K. Ramasubramonian, M. Karczewicz (Qualcomm)]

JVET-P0553 Non-CE5: Using truncated binary codes for ALF filter indices [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-P0554 Non-CE5: Clean up of coefficient coding of adaptive loop filter [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-P0555 CE5-related: Dynamic range reduction for coefficients of cross component adaptive loop filter [N. Hu, J. Dong, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-P0850 Crosscheck of JVET-P0555 (CE5-related: Dynamic range reduction for coefficients of cross component adaptive loop filter) [C.-M. Tsai (MediaTek)]

JVET-P0556 CE5-related: Temporal buffer removal for cross component adaptive loop filter [N. Hu, J. Dong, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-P0557 CE5-related: Multiplication removal for cross component adaptive loop filter [N. Hu, J. Dong, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-P0703 Crosscheck of JVET-P0557 on Multiplication removal for cross component adaptive loop filter [X. Li (Tencent)]

JVET-P0558 CE5-related: Reduced filter shape for cross component adaptive loop filter [N. Hu, J. Dong, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-P0674 Crosscheck of JVET-P0558 (CE5-related: Reduced filter shape for cross component adaptive loop filter) [C.-Y. Lai (MediaTek)]

JVET-P0570 AHG10/Non-CE5: Performance of encoder-side deblocking optimization in VTM-6.0 [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-P0571 CE5/CE8: Deblocking Filter for BDPCM coded block [H. Jang, J. Nam, S. Kim, J. Lim (LGE)]

JVET-P0851 Crosscheck of JVET-P0571 (CE5/CE8: Deblocking filter for BDPCM coded block) [C.-M. Tsai (MediaTek)]

JVET-P0586 [AHG18][Non-CE5] Proposed cleanup of deblocking filter process [S. Iwamura, S. Nemoto, A. Ichigaya (NHK), K. Andersson, R. Yu, J. Enhorn (Ericsson)]

JVET-P0700 Crosscheck of JVET-P0586 Aspect #1 ([AHG18][Non-CE5] Proposed cleanup of deblocking filter process) [Z. Deng (Bytedance)]

JVET-P0783 Crosscheck of JVET-P0586 (Aspect 3 of [AHG18][Non-CE5] Proposed cleanup of deblocking filter process) [K. Kondo, M. Ikeda (Sony)]

JVET-P0815 Crosscheck of JVET-P0586 (Non-CE5: Proposed cleanup of deblocking filter process) [K. Kazui (Fujitsu)]

JVET-P0602 [AHG18][non-CE5] Boundary strength derivation for CUs with TPM [S. Iwamura, S. Nemoto, A. Ichigaya (NHK)]

JVET-P0611 Non-CE5/AHG-11: Boundary strength harmonization for BDPCM, TS, Palette and IBC [B. Ray, G. Van der Auwera, M. Karczewicz (Qualcomm)]

JVET-P0613 Non-CE5: On the average calculation for luma adaptive deblocking filter [B. Ray, D. Rusanovskyy, G. Van der Auwera, M. Karczewicz (Qualcomm)]

JVET-P0614 Non-CE5: On the gridsize for virtual boundaries [B. Ray, G. Van der Auwera, M. Karczewicz (Qualcomm)]

JVET-P0665 Non-CE5: Spec fix for ALF filter and transpose index calculation [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)] [late]

JVET-P0666 Non-CE5: Combination of JVET-P0470 and JVET-P0557 on simplification of CC-ALF [X. Li., X. Zhao, L. Zhao, X. Du, S. Liu (Tencent), N. Hu, J. Dong, V. Seregin, M. Karczewicz (Qualcomm)] [late]

JVET-P0709 Crosscheck of JVET-P0666 (Non-CE5: Combination of JVET-P0470 and JVET-P0557 on simplification of CC-ALF) [H. Yang (InterDigital)]

JVET-P0688 Non-CE5: Intra deblocking coefficients for weak filters [L. Xu, J. Zhu, K. Kazui (Fujitsu)] [late]

JVET-P0837 Crosscheck of JVET-0688 on Intra deblocking coefficients for weak filters [S. Iwamura (NHK)]

JVET-P0739 CE5-related: Combination of CCALF methods in JVET-P0165, JVET-P0556, and JVET-P0557 [O. Chubach, C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek), N. Hu, J. Dong, V. Seregin, M. Karczewicz (Qualcomm)] [late] [miss]

JVET-P0740 CE5-related: Combination of JVET-P0086 and JVET-P0161 for TPM deblocking [C.-M. Tsai, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek), A. M. Kotra, S. Esenlik, H. Gao, B. Wang, E. Alshina (Huawei)] [late] [miss]

## CE6 related – Transforms and transform signalling (39)

Contributions in this category were discussed XXday X Oct. XXXX–XXXX in Track X (chaired by XXX).

JVET-P0166 CE6-related: Transform selection with intra prediction mode for implicit MTS [M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0196 CE6-related: Latency reduction for LFNST signalling [J. Lainema (Nokia)]

JVET-P0197 CE6-related: LFNST transform mapping [J. Lainema (Nokia) , M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0215 CE6 related: Harmonization of CE6-1.1 and two-mode LFNST [X. Zhao, X. Li, S. Liu (Tencent)]

JVET-P0216 Non-CE6: Simplified SBT mode coding [J. Park, B. Jeon (SKKU)]

JVET-P0806 Crosscheck of JVET-P0216 (Non-CE6: Simplified SBT mode coding) [L. Pham Van, G. Van der Auwera (Qualcomm)]

JVET-P0259 Non-CE6: On LFNST reduced kernels [J. Gan, C. Rosewarne (Canon)]

JVET-P0266 Non-CE6: LFNST signalling [C. Rosewarne, J. Gan (Canon)]

JVET-P0271 Non-CE6: Context modelling for LFNST index coding [T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)]

JVET-P0771 Crosscheck of JVETP0271 (Non-CE6: Context modeling for LFNST index coding) [Y.-W. Chen (Kwai)]

JVET-P0273 Non-CE6: On MTSIntraMaxCand [C. Hollmann, D. Saffar, J. Ström (Ericsson)]

JVET-P0276 Non-CE6: On Supporting 64x64 Chroma Transform Unit with Composite VPDU [W. Cai, J. Zhu, J. Yao, K. Kazui (??)]

JVET-P0786 Crosscheck of JVET-P0276 (Non-CE6:On Supporting 64x64 Chroma Transform Unit with Composite VPDU) [F. Urban (InterDigital)]

JVET-P0301 Non-CE6: Decoding process of implicit TU partitioning [G. Ko, D. Kim, J. Jung, J. Son, J. Kwak (WILUS)]

JVET-P0313 Non-CE6: Simplification of LFNST LUT [S. Shrestha, A. Kumar, B. Lee (Chosun Univ.), Y. Lee, J. Park (Humax)]

JVET-P0342 CE6-related: LFNST applied to ISP mode [F. Le Léannec, K. Naser, F. Galpin (InterDigital)]

JVET-P0346 Non-CE6: SPS Clean-up of SBT [K. Naser, T. Poirier, F. Le Léannec, F. Galpin (InterDigital)]

JVET-P0349 CE6 Related: MIP Adaptation On Top of CE6-1.1d [K. Naser, F. Galpin, T. Poirier, Y. Chen (InterDigital)]

JVET-P0350 CE6 Related: Modified LFNST Index Coding For Fast Encoder Implementation [K. Naser, F. Galpin, T. Poirier, Y. Chen (InterDigital)]

JVET-P0354 Non-CE6: LFNST Up to 64x64 CU’s [K. Naser, T. Poirier, F. Le Léannec, F. Galpin (InterDigital)]

JVET-P0376 Non-CE6: Refined LFNST restriction with MIP [H. Yang, Y. He (InterDigital)]

JVET-P0682 Crosscheck of JVET-P0376: Refined LFNST restriction with MIP [X. Zhao (Tencent)]

JVET-P0379 Non-CE6: Unified zero-out range for 4x4 LFNST [K. Fan, L. Zhang, K. Zhang, Y. Wang (Bytedance)]

JVET-P0696 Cross-check of JVET-P0379: Non-CE6: A unified zero-out range for 4x4 LFNST [H. Sun, J. Li (Panasonic)]

JVET-P0391 Non-CE6: A clean-up of SPS maximum SBT size signaling [M. G. Sarwer, R. L. Liao, J. Luo, Y. Ye (Alibaba)]

JVET-P0392 Non-CE6: Combination of ISP and LFNST [S. De-Luxán-Hernández, V. George, G. Venugopal, J. Brandenburg, B. Bross, H. Schwarz, D. Marpe, T. Wiegand (HHI), M. Koo, M. Salehifar, J. Lim, S. Kim (LGE)]

JVET-P0405 Non-CE6: Cleanups of maximum transform size related syntax elements [Z. Deng, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)]

JVET-P0430 AHG17: High level syntax cleanup on the syntax elements of transform skip [Y.-J. Chang, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-P0493 CE6-related: Further simplification with new LFNST transform basis [?? (??)] [late]

JVET-P0495 Non-CE6: On implicit MTS [Y. Wang, H. Liu, L. Zhang, K. Zhang, Z. Deng, Y. Wang (Bytedance)]

JVET-P0691 Crosscheck of JVET-P0495: Non-CE6: On implicit MTS [C. Hollmann (Ericsson)]

JVET-P0501 Non-CE6: Cleanup on MTS related HLS [X. Zhao, X. Li, S. Liu (Tencent)]

JVET-P0527 Non-CE6: On context model for coding last NZ position [T.-C. Ma, Y.-W. Chen, X. Xiu, H.-J. Jhu, X. Wang (Kwai Inc.)]

JVET-P0758 Crosscheck of JVET-P0527 (Non-CE6: On context model for coding last NZ position) [T. Tsukuba (Sony)]

JVET-P0538 Non-CE6: Transform skip with fixed transform shift [J. Xu, L. Zhang, W. Zhu, K. Zhang, H. Liu, Z. Deng (Bytedance)]

JVET-P0759 Crosscheck of JVET-P0538 (Non-CE6: Transform skip with fixed transform shift) [T. Tsukuba (Sony)]

JVET-P0540 Non-CE6: Disabling MTS for 64xN and Nx64 CUs [J. Jung, D. Kim, G. Ko, J. Son, J. Kwak (WILUS)]

JVET-P0680 Crosscheck of JVET-P0540: Disabling MTS for 64xN and Nx64 CUs [X. Zhao (Tencent)]

JVET-P0545 CE6-2.3-related: further reduced 8x8 LFNST matrices [K. Fan, L. Zhang, K. Zhang, Y. Wang (Bytedance)]

JVET-P0566 CE6-related: On LFNST Support Patterns [H. E. Egilmez, A. Said, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-P0567 CE6-related: Optimized LFNST matrices for CE6-2.1 [H. E. Egilmez, A. Said, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-P0568 CE6-related: An LFNST Index Signaling with Bin Prediction [H. E. Egilmez, V. Seregin, A. Nalci, A. Said, M. Karczewicz (Qualcomm)]

JVET-P0569 AHG17/Non-CE6: High-level syntax for MTS and Implicit Transform Derivations [H. E. Egilmez, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-P0634 Non-CE6: Removal of LFNST for 4x4, 4xN and Nx4 blocks [H. E. Egilmez, T. Hsieh, V. Seregin, M. Karczewicz (Qualcomm)] [late]

JVET-P0637 CE6-related: On measuring reconstruction error of LFNST matrices [X. Zhao (Tencent)] [late]

JVET-P0640 Non-CE6: High Level Syntax Flag for SBT Transform Selection [K. Naser, F. Le Léannec, M. Kerdranvat, T. Poirier (InterDigital)] [late]

JVET-P0699 Non-CE6 / Non-CE3: On ISP and Maximum Transform Size [K. Naser, T. Poirier, T. Urban, G. Rath (InterDigital)] [late]

JVET-P0702 Non-CE6 / Non-CE3: Combined Test of JVET-P0352 and JVET-P0354 [K. Naser, T. Poirier, F. Le Léannec, F. Galpin (InterDigital)] [late]

JVET-P0747 Crosscheck of JVET-P0702 (Non-CE6 / Non-CE3: Combined Test of JVET-P0352 and JVET-P0354) [T. Biatek (Qualcomm)]

JVET-P0814 Non-CE6: On LFNST kernels [M. Koo, M. Salehifar, J. Lim, S. Kim (LGE)] [late]

## CE7 related – Quantization and coefficient coding (26)

Contributions in this category were discussed XXday X Oct. XXXX–XXXX in Track X (chaired by XXX).

JVET-P0042 Non-CE7: A cleanup for inter_pred_idc coding [B. Heng, M. Zhou (Broadcom)]

JVET-P0713 Crosscheck of JVET-P0042 (Non-CE7: A cleanup for inter_pred_idc coding) [K. Panusopone (Nokia)]

JVET-P0050 AHG16/Non-CE7: A study of bin to bit ratio for VTM6.0 [M. Zhou (Broadcom)]

JVET-P0167 CE7-related: Improved coding of user defined quantization matrices [O. Chubach, C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0813 Crosscheck of JVET-P0167 (CE7-related: Improved coding of user defined quantization matrices) [T. Toma, K. Abe (Panasonic)]

JVET-P0168 CE7-related: Removing 2x2 chroma quantization matrices [O. Chubach, C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek), H. Jang, J. Nam, S. Paluri, S. Kim, J. Lim (LGE)]

JVET-P0169 CE7-related: Further context reduction for sig_coeff_flag [S.-T. Hsiang, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0170 CE7-related: Simplification of coding transform coefficient levels [S.-T. Hsiang, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0270 Non-CE7: Modified signalling method of cu_cbf and tu_cbf_luma [D. -J. Won, J. -M. Ha, J. -H. Moon (Sejong university)]

JVET-P0762 Cross-check of JVET-P0270 (Non-CE7: Modified signaling method of cu_cbf and tu_cbf_luma) [S.-C. Lim, H. Lee, J. Lee, J. Kang (ETRI)]

JVET-P0272 On Scaling of Transform Skip [T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)]

JVET-P0809 Crosscheck of JVET-P0272 (On Scaling for Transform Skip) [J. Xu (Bytedance)]

JVET-P0298 CE7-related: Unification of CCB check method and bypass coding between two residual coding modes [Y. Kato, K. Abe, T. Toma (Panasonic)]

JVET-P0705 Crosscheck of JVET-P0298 (CE7-related: Unification of CCB check method and bypass coding between two residual coding modes) [H. Yang (InterDigital)]

JVET-P0319 CE7-related: Re-position GT3 flag into the first coding pass in TS residual coding [Y. Chen, F. Le Léannec, T. Poirier, F. Galpin (InterDigital)]

JVET-P0631 Crosscheck of JVET-P0319: Re-position GT3 flag into the first coding pass in TS residual coding [Y.-H. Chao (Qualcomm)]

JVET-P0360 On chroma CBFs and transform units [A. K. Ramasubramonian, G. Van der Auwera, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-P0370 Non-CE7: Signaling of coded_sub_block_flag [M. G. Sarwer, R. L. Liao, J. Luo, Y. Ye (Alibaba)]

JVET-P0373 CE7-related: Decouple level mapping from transform skip residual coding [M. G. Sarwer, R. L. Liao, J. Luo, Y. Ye (Alibaba)]

JVET-P0756 Crosscheck of JVET-P0373 (CE7-related: Decouple level mapping from transform skip residual coding) [T. Tsukuba (Sony)]

JVET-P0397 CE7-related: Simplified two-pass transform-skip residual coding [M. G. Sarwer, R. L. Liao, J. Luo, Y. Ye (Alibaba)]

JVET-P0692 Crosscheck of JVET-P0397 (CE7-related: Simplified two-pass transform-skip residual coding) [S.-T. Hsiang (MediaTek)]

JVET-P0402 CE7-related: Unification of CCB count method between transform residual and transform skip residual coding [Y. Chen, F. Le Léannec, T. Poirier, K. Naser(InterDigital)]

JVET-P0632 Crosscheck of JVET-P0402: Unification of CCB count method between transform residual and transform skip residual coding [Y.-H. Chao (Qualcomm)]

JVET-P0422 Non-CE7: Unified neighbouring block selection in context and MV/mode derivation [W. Zhu, L. Zhang, J. Xu (Bytedance)]

JVET-P0648 Crosscheck of JVET-P0422 (Non-CE7: Unified neighbouring block selection in context and MV/mode derivation) [H. Dou, L. Xu (Intel)]

JVET-P0435 CE7-related: Modifications to transform skip significant flag coding [C. Auyeung, X. Li, X. Zhao, S. Liu (Tencent)]

JVET-P0437 CE7-related: Modifications to transform skip coefficient sign flag coding [C. Auyeung, X. Li, X. Zhao, S. Liu (Tencent)]

JVET-P0447 CE7-related: Rice parameter range extension in residual coding with transform skip [C. Auyeung, X. Li, X. Zhao, S. Liu (Tencent)]

JVET-P0451 CE7-related: Combination of JVET-P0435, JVET-P0437, and JVET-P0447 for residual coding at 1.75 bins/coefficient [C. Auyeung, X. Li, X. Zhao, S. Liu (Tencent)]

JVET-P0823 Crosscheck of JVET-P0451, JVET-P0435, JVET-P0437, and JVET-P0447: Modifications to the context modelling for residual coding [T. Nguyen (HHI)]

JVET-P0465 Non-CE7: Rice parameter derivation for coefficient level coding [J. Choi, J. Heo, S. Yoo, J. Choi, J. Lim, S. Kim (LGE)]

JVET-P0488 Non-CE7: On updating a variable for context model selection [S. Yoo, J. Choi, J. Lim, S. Kim (LGE)]

JVET-P0508 CE7-related: unification for joint chroma residue coding modes [L. Zhao, X. Li, X. Zhao, S. Liu (Tencent)]

JVET-P0585 CE7-related: Alignment of number of context coded bins per coefficients for TS residual coding and transform coefficient coding [M. Coban, M. Karczewicz (Qualcomm)]

JVET-P0587 CE7-related: Simplifying the derivation process of ctxInc for residual coding [S.-T. Hsiang, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0619 Non-CE7: State-dependent Binarization for Transform Coefficient Levels [H. Schwarz, A. Henkel, D. Marpe, T. Wiegand] [late]

## CE8 related – Screen content coding tools (35)

Contributions in this category were discussed XXday X Oct. XXXX–XXXX in Track X (chaired by XXX).

JVET-P0210 Non-CE8: Transform skip for chroma block in the single tree [J. Park, B. Jeon (SKKU)]

JVET-P0681 Crosscheck of JVET-P0210: Transform skip for chroma block in the single tree [X. Zhao (Tencent)]

JVET-P0230 CE8-1.2-related: Compound palette mode with signalled merge index [W. Zhu, J. Xu, L. Zhang (Bytedance)]

JVET-P0283 Non-CE8: Modified DM mode selection under IBC mode using left block [D. Jiang, J. Lin, F. Zeng, C. Fang (Dahua)] [late]

Initial version rejected: requested authors not referring to CTC as other test set is used.

JVET-P0284 Non-CE8: Modified DM mode selection under IBC mode using left and above block [D. Jiang, J. Lin, F. Zeng, C. Fang (Dahua)] [late]

Initial version rejected: requested authors not referring to CTC as other test set is used.

JVET-P0375 Non-CE8: Palette mode CU size restriction [H. Yang, Y. He (InterDigital)]

JVET-P0797	Crosscheck of JVET-P0375: Non-CE8: Palette mode CU size restriction [W. Zhu (Bytedance)]

JVET-P0399 Non-CE8: On escape sample coding [W. Zhu, J. Xu, L. Zhang (Bytedance)]

JVET-P0706 Crosscheck of JVET-P0399 (Non-CE8: On escape sample coding) [H. Yang (InterDigital)]

JVET-P0400 AhG16/Non-CE8: Removal of shared merge list [Y. Wang, L. Zhang, H. Liu, K. Zhang, J. Xu, Y. Wang (Bytedance)]

JVET-P0662 Crosscheck of JVET-P0400 (AhG16/Non-CE8: Removal of shared merge list) [J. Chen (Alibaba)]

JVET-P0421 Non-CE8: Palette mode with 8-bit entries [W. Zhu, J. Xu, L. Zhang, H.-C. Chuang (Bytedance)]

JVET-P0644 Cross-check of JVET-P0421: Non-CE8: Palette mode with 8-bit entries [J. Zhao (LGE)]

JVET-P0424 Non-CE8: Resetting predictor palette at CTU row [W. Zhu, L. Zhang, J. Xu (Bytedance)]

JVET-P0425 CE8-related: On index adjustment in palette mode [W. Zhu, J. Xu, L. Zhang (Bytedance)]

JVET-P0732 Crosscheck of JVET-P0425 on index adjustment in palette mode [X. Xiu (Kwai Inc.)]

JVET-P0454 Non-CE8: Constraints on IBC reference block locations [X. Xu, X. Li, S. Liu (Tencent)]

JVET-P0733 Crosscheck of JVET-P0454 on constraints on IBC reference block locations [X. Xiu (Kwai Inc.)]

JVET-P0455 Non-CE8: IBC slice level on/off [X. Xu, X. Li, S. Liu (Tencent)]

JVET-P0456 Non-CE8: IBC chroma mode with dual-tree [X. Xu, X. Li, S. Liu (Tencent)]

JVET-P0838 Cross-check of JVET-P0456 (Non-CE8: IBC chroma mode with dual-tree) [M. G. Sarwer (Alibaba)]

JVET-P0457 Non-CE8: On IBC merge list size signaling [X. Xu, X. Li, S. Liu (Tencent)]

JVET-P0676 Crosscheck of JVET-P0457 (Non-CE8: On IBC merge list size signalling) [Y.-L. Hsiao (MediaTek)]

JVET-P0460 Non-CE8: Minimum QP for Palette Escape Coding [J. Zhao, S. Paluri, S. Kim (LGE)]

JVET-P0798 Crosscheck of JVET-P0460: Non-CE8: Minimum QP for Palette Escape Coding [W. Zhu (Bytedance)]

JVET-P0472 Non-CE8: Palette Coding Encoder Improvement [J. Zhao, S. Yoo, S. Kim (LGE)]

JVET-P0719 Crosscheck of JVET-P0472 (Non-CE8: Palette Coding Encoder Improvement) [H.-J. Jhu (Kwai Inc.)]

JVET-P0473 Non-CE8: Implicit block partitioning in palette mode [Y.-H. Chao, T. Hsieh, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-P0724 Crosscheck of JVET-P0473 (Non-CE8: Implicit block partitioning in palette mode) [Y.-W. Chen (Kwai Inc.)]

JVET-P0474 Non-CE8: Minimum QP for escape mode in palette [Y.-H. Chao, H. Wang, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-P0795 Cross-check of JVET-P0474: Non-CE8: Minimum QP for escape mode in palette [J. Zhao (LGE)]

JVET-P0475 Non-CE8: Simplification of palette predictor update for small CUs [Y.-H. Chao, T. Hsieh, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-P0707 Crosscheck of JVET-P0475 (Non-CE8: Simplification of palette predictor update for small CUs) [H. Yang (InterDigital)]

JVET-P0476 Non-CE8: Palette mode and prediction mode signaling [Y.-H. Chao, C.-H. Hung, W.-J Chien, M. Karczewicz (Qualcomm)]

JVET-P0843	Crosscheck of JVET-P0476 (Non-CE8: Palette mode and prediction mode signalling) [Y.-W. Chen (Kwai Inc.)]

JVET-P0479 CE8-related: Restriction on context coded bins in palette [C.-H. Hung, Y.-H. Chao, M. Karczewicz (Qualcomm)]

JVET-P0799 Crosscheck of JVET-P0479: CE8-related: Restriction on context coded bins in palette [W. Zhu (Bytedance)]

JVET-P0483 Non-CE8: Quantization in palette escape mode [M. Karczewicz, H. Wang, Y.-H. Chao, M. Coban (Qualcomm)]

JVET-P0720 Crosscheck of JVET-P0483 (Non-CE8: Quantization in palette escape mode) [H.-J. Jhu (Kwai Inc.)]

JVET-P0486 CE8-related: Alignment of maximum transform-skip size with maximum transform block size [M. G. Sarwer, R. L. Liao, J. Luo, Y. Ye (Alibaba)]

JVET-P0487 Non-CE8: Binarization of palette escape value [S. Yoo, J. Zhao, J. Nam, J. Choi, S. Kim, J. Lim (LGE)] [late]

Initial version rejected as “placeholder”.

JVET-P0515 Non-CE8: Quantization unification for palette escape and transform skip [H.-J. Jhu, X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)]

JVET-P0715 Crosscheck of JVET-P0515: Non-CE8: Quantization unification for palette escape and transform skip [Y.-H. Chao (Qualcomm)]

JVET-P0516 Non-CE8: On palette mode signaling binarization [Y.-W. Chen, X. Xiu, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)]

JVET-P0630 Crosscheck of JVET-P0516: On palette mode signalling binarization [Y.-H. Chao (Qualcomm)]

JVET-P0522 Non-CE8: On palette mode syntax [Y.-W. Chen, X. Xiu, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)]

JVET-P0650 Crosscheck of JVET-P0522 (Non-CE8: On palette mode syntax) [T.-S. Chang (Alibaba)]

JVET-P0523 Non-CE8: Context coded bin constraint for palette mode signaling [Y.-W. Chen, X. Xiu, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)]

JVET-P0651 Crosscheck of JVET-P0523 (Non-CE8: Context coded bin constraint for palette mode signaling) [T.-S. Chang (Alibaba)]

JVET-P0526 Non-CE8: Palette encoder improvements [H.-J. Jhu, Y.-W. Chen, X. Xiu, T.-C. Ma, X. Wang (Kwai Inc.)]

JVET-P0796 Cross-check of JVET-P0526: Non-CE8: Palette encoder improvements [J. Zhao (LGE)]

JVET-P0529 Non-CE8: QP dependent binarization for palette escape [H.-J. Jhu, Y.-W. Chen, X. Xiu, T.-C. Ma, X. Wang (Kwai Inc.)]

JVET-P0652 Crosscheck of JVET-P0529 (Non-CE8: QP dependent binarization for palette escape) [T.-S. Chang (Alibaba)]

JVET-P0562 Non-CE8: Transform skip residual coding simplification [Y.-W. Chen, X. Xiu, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)]

JVET-P0642 Crosscheck of JVET-P0562 (Non-CE8: Transform skip residual coding simplification) [L. Zhao (Tencent)]

JVET-P0565 CE8-related: Cross-component residual prediction for 4:4:4 format [K. Kawamura, S. Naito (KDDI)]

JVET-P0573 AHG17/Non-CE8: Support parallel decoding based on initialization for palette predictor at CTU row [H. Jang, N. Park, J. Nam, S. Kim, J. Lim (LGE)]

JVET-P0643 Crosscheck of JVET-P0573: AHG17/NonCE8: Support parallel decoding based on initialization for palette predictor at CTU row [W. Zhu (Bytedance)]

JVET-P0577 AHG17/Non-CE8: APS support for palette predictor entry initialization [H. Jang, S. Paluri, S. Yoo, J. Nam, S. Kim, J. Lim (LGE)]

JVET-P0670 Non-CE8: On num_palette_indices_minus1 [W. Zhu, L. Zhang, J. Xu (Bytedance)] [late]

JVET-P0731 Crosscheck of JVET-P0670 (Non-CE8: On num_palette_indices_minus1) [H. Yang (InterDigital)]

## Quantization control (21)

Contributions in this category were discussed XXday X Oct. XXXX–XXXX in Track X (chaired by XXX).

JVET-P0093 Fix on value ranges of cu_qp_delta_subdiv and cu_chroma_qp_offset_subdiv [E. Sasaki, T. Chujoh, T. Ikai (Sharp)]

JVET-P0110 AHG15: Quantization matrices with single identifier and enhanced prediction [P. de Lagrange, F. Leléannec, E. François, K. Naser (InterDigital)]

JVET-P0754 Crosscheck of JVET-P0110 (AHG15: Quantization matrices with single identifier and enhanced prediction) [J. Le Tanou (MediaKind)]

JVET-P0256 AHG15: 16x16 base scaling matrix for 64x64 TU [T. Toma, K. Abe (Panasonic)]

JVET-P0257 AHG15: VTM decoder speed-up for handling scaling matrices [T. Toma, K. Abe (Panasonic)]

JVET-P0761 Cross-check of JVET-P0257 (AHG15: VTM decoder speed-up for handling scaling matrices) [S.-C. Lim, H. Lee, J. Lee, J. Kang (ETRI)]

JVET-P0267 Quantization group subdivision level signalling [C. Rosewarne, J. Gan (Canon)]

JVET-P0292 AHG15: Scaling process for LFNST case [T. Hashimoto, E. Sasaki, T. Ikai (Sharp)]

JVET-P0657 Crosscheck of JVET-P0292 (AHG15: Scaling process for LFNST case) [Y. Wang (Bytedance)]

JVET-P0293 AHG15: Signalling scaling matrix for LFNST case [T. Hashimoto, E. Sasaki, T. Ikai (Sharp)]

JVET-P0321 AHG15: Signaling of chroma QP mapping table [Z. Wang, Y. Ye, J. Luo (Alibaba)]

JVET-P0324 AHG15: Quantization matrix selection based on block area [P. de Lagrange, P. Bordes, K. Naser, E. François (InterDigital)]

JVET-P0335 AHG15: Chroma QP mapping table for HDR [T. Lu, F. Pu, P. Yin, M. Sean, W. Husak, T. Chen (Dolby)]

JVET-P0623 Crosscheck report of JVET-P0335 on Chroma QP mapping table for HDR [E. François (InterDigital)]

JVET-P0339 AHG15: Signalling of CU level chroma QP offset [Z. Wang, Y. Ye, J. Luo (Alibaba)

JVET-P0361 On chroma quantization groups [A. K. Ramasubramonian, G. Van der Auwera, W.-J. Chien, H. Huang, Y. Han, B. Ray, M. Karczewicz (Qualcomm)]

JVET-P0365 AHG15: Scaling matrices for LFNST-coded blocks [A. K. Ramasubramonian, G. Van der Auwera, V. Seregin, H. E. Egilmez, M. Karczewicz (Qualcomm)]

JVET-P0371 AHG7/AHG15: signalling of corrective values for chroma residual scaling [E. François, F. Galpin, K. Naser, P. de Lagrange (InterDigital)]

JVET-P0711 Crosscheck of JVET-P0371 (AHG7/AHG15: signalling of corrective values for chroma residual scaling) [T. Lu (Dolby)]

JVET-P0407 AHG15: cu_qp_delta_subdiv and cu_chroma_qp_offset_subdiv syntax dependency removal [J. Chen, R.-L. Liao, J. Luo, Y. Ye (Alibaba)]

JVET-P0410 AHG15: chroma QP mapping table syntax variant with less bits [P. de Lagrange, P. Bordes, K. Naser, E. François (InterDigital)]

JVET-P0426 AHG17/AHG15: On quantization control parameters signalling [R. Chernyak (Huawei)]

JVET-P0667 AHG17/AHG15: Comments on aspect 1 of JVET-P0426 [B. Ray, A. K. Ramasubramonian, G. Van der Auwera, M. Karczewicz (Qualcomm)] [late]

JVET-P0436 AHG15: On CU Adaptive Chroma QP Offset Signalling [J. Zhao, S. Paluri, S. Kim (LGE)]

JVET-P0608 [AHG15] On scaling list for JCCR [S. Iwamura, S. Nemoto, A. Ichigaya (NHK), P. de Lagrange, F. Le Leannec, E. François (InterDigital)]

JVET-P0469 AHG15: efficient coding of qp_out_val [F. Bossen, A. Segall (Sharp)]

JVET-P0730 AHG15: default quantization matrix [P. de Lagrange, E. François (InterDigital)] [late]

## Entropy coding (4)

Contributions in this category were discussed XXday X Oct. XXXX–XXXX in Track X (chaired by XXX).

JVET-P0300 High throughput CABAC mode for VVC [H. Kirchhoffer, D. Marpe, B. Bross, T. Nguyen, C. Rudat, H. Schwarz, T. Wiegand (HHI)]

JVET-P0395 CABAC zero word thresholds [A. Browne, S. Keating, K. Sharman (Sony)]

JVET-P0396 Switching between CABAC context coded bins and bypass coded bins [A. Browne, S. Keating, K. Sharman (Sony)]

JVET-P0603 CABAC zero word signalling [M. Coban, A. Ramasubramonian, V. Seregin, M. Karczewicz (Qualcomm)]

## Partitioning (14)

Contributions in this category were discussed XXday X Oct. XXXX–XXXX in Track X (chaired by XXX).

JVET-P0063 AHG16: Fix on local dual tree [Y. Zhao, H. Yang (Huawei), Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0766 Crosscheck of JVET-P0063 (AHG16: Fix on local dual tree) [C. Rosewarne (Canon)]

JVET-P0188 On MinCU constraint [T. Suzuki (Sony)]

JVET-P0343 Flexible maximum MTT hierarchy depth [F. Le Léannec, F. Galpin, T. Poirier (InterDigital)]

JVET-P0839 Cross-check of JVET-P0343 (AHG17: Flexible maximum MTT hierarchy depth) [M. G. Sarwer (Alibaba)]

JVET-P0344 Overriding qtbtt_dual_tree_flag [F. Le Léannec, K. Naser, F. Galpin, T. Poirier (InterDigital)]

JVET-P0347 Comments on maximum MTT Depth and MinCbSize [T. Poirier, F. Le Léannec, F. Urban, F. Galpin (InterDigital)]

JVET-P0348 On BT/TT flag signalling [T. Poirier, F. Le Léannec, F. Urban, F. Galpin (InterDigital)]

JVET-P0419 AHG16/AHG17: Simplification of CU Splitting Controls [T. Hellman, W. Wan, M. Zhou, B. Heng (Broadcom)]

JVET-P0429 AHG17: On log2_min_luma_coding_block_size_minus2 [Y.-J. Chang, V. Seregin, M. Coban, A. Ramasubramonian, M. Karczewicz (Qualcomm)]

JVET-P0506 On max QT/BT/TT size signaling [Y. Du, X. Li, X. Zhao, B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-P0764 Crosscheck of JVET-P0506: AHG17: On max QT/BT/TT size signaling [J. Jung, D. Kim, G. Ko, J. Son, J. Kwak (WILUS)]

JVET-P0575 Non-CE: to support SCIPU with local dual tree for various color format [H. Jang, J. Nam, S. Kim, J. Lim (LGE)]

JVET-P0578 Interaction between dual tree and minimum CU size [T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0580 On signaling CTU size in SPS [Y. Du, B. Choi, X. Li, X. Zhao, S. Wenger, S. Liu (Tencent)]

JVET-P0582 AhG16: On dynamic internal bit depth [Y. Du, X. Zhao, X. Li, L. Zhao, S. Liu (Tencent)]

JVET-P0624 AHG16: Bugfix for local dual tree [L. Zhao, X. Li, X. Zhao, S. Liu (Tencent)] [late]

JVET-P0737 Crosscheck of JVET-P0624 on Bugfix for local dual tree [X. Xiu (Kwai Inc.)]

## Chroma sampling and chroma formats (1)

Contributions in this category were discussed XXday X Oct. XXXX–XXXX in Track X (chaired by XXX).

JVET-P0517 Support of adaptive color transform for 444 video coding in VVC [X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)]

JVET-P0683 Crosscheck of JVET-P0517: Support of adaptive color transform for 444 video coding in VVC [X. Zhao (Tencent)]

## Lossless and near lossless coding (24)

Contributions in this category were discussed XXday X Oct. XXXX–XXXX in Track X (chaired by XXX).

JVET-P0082 AHG18: BDPCM for Lossless [A. Nalci, H. Wang, H. E. Egilmez, M. Coban, M. Karczewicz (Qualcomm)]

JVET-P0807 Crosscheck of JVET-P0082 Proposal#4 and #5 (AHG18: BDPCM for Lossless) [T. Tsukuba (Sony)]

JVET-P0147 AHG18: BDPCM in lossless coding [Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0775 Crosscheck of JVET-P0147, TEST1, TEST2 (AHG18: BDPCM in lossless coding) [Y.-W. Chen (Kwai)]

JVET-P0148 AHG18: Disabling dependent quantization in lossless coding [Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek), T.-C. Ma, Y.-W. Chen, X. Xiu, H.-j. Jhu, X. Wang (Kwai)]

JVET-P0659 Crosscheck of JVET-P0148 (AHG18: Disabling dependent quantization in lossless coding) [N. Zhang (Bytedance)]

JVET-P0149 AHG18: Disabling LMCS for lossless coding [Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0176 AHG18: Lossless coding support related to cross-component ALF [J. Li, C. S. Lim (Panasonic)]

JVET-P0258 AHG18: Disabling Dependent Quantization for Lossless [A. Nalci, H. Wang, H. E. Egilmez, M. Coban, M. Karczewicz (Qualcomm)]

JVET-P0827 Crosscheck of JVET-P0258: AHG18: Disabling Dependent Quantization for Lossless [T.Poirier (Interdigital)]

JVET-P0327 AHG18: Configurable Maximum Transform Size for Mixed Lossy and Lossless Coding [A. Nalci, H.E. Egilmez, H. Wang, Y.-H. Chao, M. Coban, M. Karczewicz (Qualcomm)]

JVET-P0816 Cross-check of JVET-P0327: AHG18: Configurable Maximum Transform Size for Mixed Lossy and Lossless Coding [H. Sun, J. Li (Panasonic)]

JVET-P0463 AHG18: Residual coding method for lossless mode [M. G. Sarwer, R. L. Liao, J. Luo, Y. Ye (Alibaba)]

JVET-P0466 AHG18: Residual coding for lossless video coding [J. Choi, H. Jang, J. Heo, S. Yoo, J. Lim, S. Kim (LGE)]

JVET-P0824	Crosscheck of JVET-P0466: Test #3, #4, and #5 [T. Nguyen (HHI)]

JVET-P0467 AHG18: Rice parameter extension [J. Choi, H. Jang, J. Heo, S. Yoo, J. Lim, S. Kim (LGE)]

JVET-P0774 Crosscheck of JVET-P467, TEST2, TEST4 (AHG18: Rice parameter extension) [Y.-W. Chen (Kwai)]

JVET-P0477 AHG18: On Last Position Signaling for Lossless [A. Nalci, H. E. Egilmez, M. Coban, M. Karczewicz (Qualcomm)]

JVET-P0826 Crosscheck of JVET-P0477: Last coding for lossless [T. Nguyen (HHI)]

JVET-P0485 AHG18: Performance of CE7-1.2d and CE7-2.2 on lossless coding [Y.-H. Chao, A. Nalci, H. Wang, M. Coban, M. Karczewicz (Qualcomm)]

JVET-P0502 AHG18: Interaction between lossless coding and max transform size [X. Zhao, X. Li, S. Liu (Tencent)]

JVET-P0514 AHG18: Enable lossless coding for VVC [T.-C. Ma, Y.-W. Chen, X. Xiu, H.-J. Jhu, X. Wang (Kwai Inc.)]

JVET-P0521 AHG18: Disabling DMVR and BDOF for lossless coding [T.-C. Ma, Y.-W. Chen, X. Xiu, H.-J. Jhu, X. Wang (Kwai Inc.)]

JVET-P0525 AHG18: Residual coding selection signaling for lossless VVC [T.-C. Ma, Y.-W. Chen, X. Xiu, H.-J. Jhu, X. Wang (Kwai Inc.), Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S. Lei (MediaTek)]

JVET-P0757 Crosscheck of JVET-P0525 Test2 (AHG18: Residual coding selection signaling for lossless VVC) [T. Tsukuba (Sony)]

JVET-P0804 Crosscheck of JVET-P0525 (AHG18: Residual coding selection signaling for lossless VVC) [J. Heo (LGE)]

JVET-P0528 AHG18: On residual scanning order for lossless coding [T.-C. Ma, Y.-W. Chen, X. Xiu, H.-J. Jhu, X. Wang (Kwai Inc.)]

JVET-P0671 Crosscheck of JVET-P0528 (AHG18: On residual scanning order for lossless coding) [Z.-Y. Lin (MediaTek)]

JVET-P0559 AHG18: Rice parameter derivation for coefficient level coding in lossless [M. Karczewicz, H. Wang, Y.-H. Chao, M. Coban (Qualcomm)]

JVET-P0825 Crosscheck of JVET-P0559: Rice parameter derivation for lossless [T. Nguyen (HHI)]

JVET-P0574 AHG18: BDPCM residual coding for lossless coding [H. Jang, J. Choi, J. Nam, S. Kim, J. Lim (LGE)]

JVET-P0841 Cross-check of JVET-P0574 (AHG18: BDPCM residual coding for lossless coding) [M. G. Sarwer (Alibaba)]

JVET-P0576 AHG18: on low level coding for lossless [H. Jang, J. Choi, J. Nam, S. Kim, J. Lim (LGE)]

JVET-P0776 Crosscheck of JVET-P0576 (AHG18: on low level coding for lossless) [Y.-W. Chen (Kwai)]

JVET-P0834 Cross-check result of JVET-P0576: AHG18: on low level coding tool for lossless coding (mipTuTiling) [Z. Zhang (Ericsson)]

JVET-P0584 AHG18: BDPCM for luma and chroma in lossless coding [T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)]

JVET-P0770 Crosscheck of JVET-P0584 (AHG18: BDPCM for luma and chroma in lossless coding) [Y.-W. Chen (Kwai)]

JVET-P0606 AHG18: Enabling lossless coding with minimal impact on VVC design [B. Bross, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand (HHI), M. Karczewicz, Y.-H. Chao, H. Wang, M. Coban (Qualcomm)]

JVET-P0684 Crosscheck of JVET-P0606: Enabling lossless coding with minimal impact on VVC design [L. Zhao, X. Zhao (Tencent)]

JVET-P0607 AHG18: Two Stage Residual Coding for Lossless [B. Bross, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand (HHI)] [late]

JVET-P0721 Crosscheck of JVET-P0607: AHG18: Two Stage Residual Coding for Lossless [Y.-H. Chao (Qualcomm)]

JVET-P0612 AHG18: Non-Lossless Coding Tools in VVC [?? (??)] [late] [miss]

## Miscellaneous coding tools (6)

Contributions in this category were discussed XXday X Oct. XXXX–XXXX in Track X (chaired by XXX).

JVET-P0254 Issue of simplified luma mapping of LMCS [K. Abe, T. Toma (Panasonic)]

JVET-P0383 Non-CE: JCCR and LMCS interaction [K. Fan, L. Zhang, K. Zhang, Y. Wang (Bytedance)]

JVET-P0712 Crosscheck of JVET-P0383 (Non-CE3: JCCR and LMCS interaction) [T. Lu (Dolby)]

JVET-P0394 AHG13: Luma Clipping instead of LMCS [S. Keating, K. Sharman, A. Browne (Sony)]

JVET-P0417 AHG13: Removal of ISP [T. Hellman, B. Heng, W. Wan (Broadcom)]

JVET-P0561 AHG16: Context restriction on CTU boundary for line buffer reduction [H.-J. Jhu, Y.-W. Chen, X. Xiu, T.-C. Ma, X. Wang (Kwai Inc.)]

JVET-P0636 Crosscheck of JVET-P0561 (AHG16: Context restriction on CTU boundary for line buffer reduction) [T.-H. Li, C.-Y. Teng (Foxconn)]

## Neural networks (1)

JVET-P0489 AHG9: Multiple Convolution Neural Networks for Sequence-Independent Processing [H. Yin, R. Yang, X. Fang, Z. Gao, R. Yang (Intel)] [late]

## 360 degree video (2)

Contributions in this category were discussed XXday X Oct. XXXX–XXXX in Track X (chaired by XXX).

JVET-P0315 Modified 360Lib for more flexible cube face arrangements [J. Sauer, M. Bläser (RWTH Aachen)

JVET-P0669 AHG6: Wrap-around motion vector prediction at the picture boundary [M. Lee, J. Lee, J. Park, D. Sim, S.-J. Oh (KWU), W. Lim, G. Bang (ETRI)] [late]

## High level tool control (0)

## AHG17: General high-level syntax (47)

### NAL unit header (3)

JVET-P0362 AHG17: NAL unit header extension to extend the number of layers [R. Sjöberg, M. Pettersson, M. Damghanian (Ericsson)]

Discussed 2040 Thursday (YKW)

Proposes a 2- or 3-byte NUH, to enable up to 2104 more layers.

It was agreed at the previous meeting that the reserved bit in the NAL unit can be used for extending the NUH when needed.

It was agreed to reserve layer IDs 56 to 63, inclusive, in VVCv1.

JVET-P0363 AHG17: On NAL unit type table [M. Pettersson, R. Sjöberg, M. Damghanian (Ericsson)]

A nice clean up. Thanks!

Agreed.

JVET-P0187 AHG8/AHG17: Comments on HLS of VVC [S. Deshpande (Sharp)]

Aspects 3 and 4 belong to this category.

See notes under P0363.

### PTL definition and signalling (2)

JVET-P0217 On Profile, Tier, Level Syntax Structure [J. Samuelsson, S. Deshpande, A. Segall (Sharp)]

Discussed 2030 Thursday (YKW)

1. Move general constraints and general_level_idc before sub-profiles. Agreed.

2. Allow zero sub-profile. Agreed.

3. Add inferrence of sub-layer level. Agreed.

JVET-P0478 AHG17: On (sub)profiles signalling in the DPS [R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)] [late]

Discussed 2120 Thursday (YKW)

Proposes to allow more than PTL syntax structures in the DPS.

For byte alignment, it'd be better to reserve 5 bits instead of 1, and use u(4) for the number of PTL syntax structures.

It was commented that, if VVCv1 does not end up with multiple profiles that are not onion-shelled, then in VVCv1 the current syntax is good enough, while in the future if needed the DPS extension mechanims can be used to allow adding more PTL syntax structures.

It was commented that if we allow more than one PTL now, then even if future needs arise, no extension is needed herein as well as for systems specificaitons e.g. MPEG-2 TS.

Agreed, with the following changes: reserve 5 bits instead of 1, and use u(4) for the number of PTL syntax structures.

### Cross-RAP referencing and external reference pictures (2)

JVET-P0114 AHG17: On external decoding refresh (EDR) [Y.-K. Wang, Hendry, J. Chen (Futurewei), H. Yu, L. Yu (ZJU)]

Discussed 0900 Wednesday (GJS & JRO)

This contribution proposes a design for support of random accessing a bitstream from inter-coded pictures without a recovery period. This technique is referred to as external decoding refresh (EDR) or cross RAP referencing (CRR). Using this technique, random access point (RAP) pictures can be coded as inter-predicted pictures typically using only one or two earlier pictures in the bitstream for inter prediction reference(s), thus reportedly achieving higher coding efficiency than coding of the RAP pictures as IRAP pictures.

When random accessing from such inter-coded RAP pictures, the reference pictures would not be available in the bitstream starting from the RAP, but need to be provided through an external means, e.g., in a separate file format track or a separate dynamic adaptive streaming over HTTP (DASH) representation. The externally-provided pictures are referred to as external pictures, and such RAP pictures are referred to as external decoding refresh (EDR) pictures.

The proposed design is summarized as follows:

- A new NAL unit type (EDR_NUT) is defined for EDR pictures.

- An EDR picture may start a CVS (hence may also start a bitstream).

- External pictures are provided in the form of coded pictures, and there is no change to the RPL syntax, semantics, and derivation process.

- The general decoding process is updated to specify the decoding process of a bitstream starting from an EDR picture (i.e., when random accessing from an EDR picture) with externally-provided reference pictures.

- Two alternative options regarding POC handling are proposed.

  - In the first option, POC signalling and derivation are kept unchanged, while POC values are restricted as follows: For each EDR picture, let listOfPictures be the list of pictures that consists of all the pictures referred to by all entries in RefPicList[ 0 ] and all entries in RefPicList[ 1 ] of the EDR picture and the EDR picture itself, listed in increasing decoding order of these pictures. It is a requirement of bitstream conformance that the difference between the PicOrderCntVal values of any two of these pictures that are consecutive in the list shall be greater than −MaxPicOrderCntLsb / 2 and less than MaxPicOrderCntLsb / 2.

  - In the second option, the POC MSB values of the external pictures except for the first external picture as well as the POC MSB value of the EDR picture itself are signalled in the slice headers, while the above constraint on POC values are not imposed.

The proposed detailed spec text changes are attached.

The "external" pictures are provided as coded pictures, and the decoder decodes them when performing random access.

It was asked why the decoder would need awareness of what happened. It was suggested that the system could handle the construction of a bitstream that appears to a decoder as an ordinary bitstream but was constructed by putting the coded AUs together in the right order before the inter-coded "RAP" AU. In this case, the encoder would need to be responsible for making sure that the POC MSBs have the necessary relationship to retain the MSBs tracking correctly and satisfy our constraint on the relationship of the POC of the current picture relative to its reference pictures. This seemed adequate without a need for special syntax or other handling in the decoding process spec. (We are not responsible for the format of the data within a server before the construction of the bitstream that is fed to a decoder.)

So no action seemed necessary.

JVET-P0211 AHG17: On cross-RAP referencing [M. M. Hannuksela (Nokia)]

Discussed 0900 Wednesday (GJS & JRO)

JVET-P0212 proposes enabling the use of a single decoder instance and the single-layer VVC decoding process for decoding multiple independent layers. The decoding is performed as if all NAL units resided in a single layer only.

This contribution is asserted to achieve the functionality of cross-RAP referencing using the multiple independent layers functionality as proposed in the following ingredients of JVET-P0212:

- A VPS flag to indicate that there is one and only one picture per each POC value among all independent layers and that the independent layers can be decoded as if there were in the same layer.

- "Collective" HRD parameters can be signalled for the bitstream that contains multiple independent layers.

- Possibility to control by external means that the entire bitstream is decoded as if it were a single-layer bitstream. Otherwise, the current layer-wise operation of VVC is carried out.

It is proposed to support the cross-RAP referencing functionality in VVC by adopting the above-listed aspects 1 to 3 of JVET-P0212.

The proponent indicated that this seems conceptually aligned with what had been agreed in discussion of P0114. As with P0114, POC relationships would need to be handled appropriately in the "encoder".

No further change seemed needed for the PTL signalling relative to the plan agreed earlier in the meeting (i.e., PTL indicates total capability needed for an operation point).

The remaining element of the proposal to consider was a high-level flag (associated with an OLS) that would indicate that every AU in the OLS contains only one picture.

A participant brought up the idea of combining this with multiple layers of a different type, e.g., having two spatial layers in which each spatial layer acts like the proposed scheme. The proponent suggested adjusting the semantics of the flag to indicate that every AU in the OLS contains only one output picture.

Another case brought up is multiview. In such a case, more than one view is output for an AU, but each view could be coded separately in the described manner.

Underlying the described case is the notion of using layers to deliver a functionality that can be provided with single-layer coding. The spirit of what is experienced by a decoder is basically single-layer operation.

Further study is encouraged to determine whether the single layer approach can be fully adequate for the envisioned functionality, or could be made adequate with some small adjustment, or if such a layering approach is needed instead.

JVET-P0326 AHG17: External reference picture [P. Bordes, D. Doyen, F. Galpin, M. Kerdranvat (InterDigital)]

It is proposed to signal in the reference-picture-list-struct whether some reference pictures are provided by an external means not specified in the Specification. It is also proposed to signal in the reference-picture-list-struct whether the motion information associated with one reference picture is zero (default) or not.

Two distinctions between this proposal and P0114 are:

- The externally provide picture is provided in decoded form

- That picture may not come from a VVC bitstream, and thus may not have the syntax elements and motion data and other properties (e.g., a POC, a layer ID, a temporal ID, a NUT) that we ordinarily have available for a referenc picture.

The proposal includes proposal of a flag for whether motion data is available for the externally provided reference picture or not.

It was commented that SHVC conformance testing data was never provided for the similar functionality there.

If provided, such a capability would need to be optional in some way – e.g., made an optional feature of any profile or given a different profile.

A participant commented that this could provide an opportunity for externally specified operations (e.g., disparity compensation) to be applied to pictures that are then fed back into the ordinary VVC decoding process.

It was agreed that this functionality should be deferred for future study for consideration as part of some possible v2 (or v3) extension project (but not included in v1).

### Reference picture list signalling and constraints (8)

JVET-P0123 AHG17: RPL constraints for RASL and RADL pictures [Hendry, Y.-K. Wang (Futurewei)]

Discussed Friday 10/4, chaired by M. M. Hannuksela in the HLS BoG.

Comment: "When the current picture is a RADL picture, there shall be no picture referred to by an entry" should be "When the current picture is a RADL picture, there shall be no picture referred to by an active entry" as proposed in JVET-0356. Confirmed.

See notes for P0356 on conclusions.

JVET-P0356 AHG17: Bitstream constraints on RPL and GDR [R. Sjöberg, M. Pettersson, M. Damghanian (Ericsson)]

Discussed Friday 10/4, chaired by M. M. Hannuksela in the HLS BoG.

Constraint 1: It was commented that the constraint is already enforced in VVC Draft 6 (with different phrasing). No action.

Constraint 2: Covered also in P0123. BoG recommends adoption.

Constraint 3: It was agreed that the more detailed corresponding constraint in P0123 is preferred. Offline work to check that the GDR aspect is correctly handled.

Constraint 4: Agreed with the following change: "no entry" instead of "no active entry", "in output order" instead of "in decoding order".

Constraint 5: It was agreed that the more detailed corresponding constraint in P0123 is preferred.

Constraint 6: Agreed. (See notes for P0123.)

Constraint 7: It was commented that HRD tests the conformance starting from each CVS start picture (including GDR), hence the constraints are enforced by the HRD tests. Thus, the proposed constraints should be expressed as informative notes only: agreed.

It was agreed that it should be allowed to have an IRAP picture that appears in a GDR refresh period, e.g. for live encoding that detected a scene cut. Offline work to check.

It was commented that it should be verified that the constraints are valid also in multi-layer bitstreams.

Offline work to merge P0123 and P0356 with agreements as above. Revisit the merged contribution.

JVET-P0134 AHG17: On long-term reference picture signalling [X. Ma, H. Yang (Huawei)]

Discussed Friday 10/4, chaired by Y.-K. Wang in the HLS BoG.

1. Suggests to editorially improve the semantics of long_term_ref_pics_flag. Delegated to the editors.

2. Proposes allowing reordering of LTRPs by adding syntax into the slice header, which can help reduce the number of RPL candidates in the SPS.

It was commented that this is like reference picture reordering in AVC. Why only do it for LTRPs, not for STRPs as well? Proponet said that was becasue STRPs are typically used in a pattern, thus more suitable to be included in the RPL candidates in the SPS.

It was commented that if many pictures need such reordering, the number of bits needed for reordering can be more than having one more RPL candidate in the SPS.

Proponent said one reason is that the number of RPL candidates is limited. The current limit is 64.

It was commented tha the reordering pseduocode seems long.

No action was taken.

JVET-P0135 AHG8/AHG17: On inter-layer reference picture signalling [X. Ma, H. Yang (Huawei)]

Discussed Friday 10/4, chaired by Y.-K. Wang in the HLS BoG.

Proposal 1: On VPS signalling of layer dependency. See section 6.20.1.5.

Proposal 2: Add a syntax condition for the ILRP present flag in the SPS.

This introduces parsing dependency of SPS on VPS.

Should we as the group allow parsing dependency of SPS/PPS/APS etc. on VPS?

It seems that we should keep independent parsing among parameters, considering that this has been the agreed principle and considering that VPS does not need to be present in single-layer bitstreams.

No action was taken.

Proposal 3: Add a syntax condition for ilrp_idc in the RPL syntax structure.

This introduces parsing dependency of SPS on VPS.

No action was taken.

JVET-P0182 AHG8/AHG17: On Reference Picture List and Inter-Layer Prediction [S. Deshpande (Sharp)]

Discussed Friday 10/4, chaired by Y.-K. Wang in the HLS BoG.

Proposal 1: A syntax modification is proposed for reference picture list signalling. The proposed modification needs two syntax elements instead of three syntax elements as needed by VVC WD 6 to specify whether an entry is ILRP/ LTRP/ STRP with positive delta POC/ STRP with negative delta POC. Thus it is asserted to have a lower complexity.

It was commented that the current syntax and semantics are more straightforward. The proponent said otherwise.

It was commented that the proposed syntax has long syntax condition.

The proposal needs two syntax elements as compared to three in the current syntax.

It was claimed that 1 bit per RPL entry can be saved for each RPL candiate when both the ILRP and LTRP flags are equal to 1.

No action was taken.

Proposal 2: To conditionally signal inter layer reference picture flag (inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ]) such that it is signalled only when necessary. When not signalled the value for inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] is inferred in this case.

Some options have parsing dependency issues.

Agreed to add a constraint that, when vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, the value of inter_layer_ref_pics_present_flag shall be equal to 0.

This constraint is also proposed in JVET-P0221.

Proposal 3: A bug fix is proposed to the reference picture list derivation of delta POC values for the short-term reference picture entries.

Revisit after offline checking to confirm the bug, and if confirmed, the fix.

JVET-P0221 AHG8: On signaling interlayer reference picture list [B. Choi, S. Wenger, S. Liu (Tencent)]

Discussed Friday 10/4, chaired by Y.-K. Wang in the HLS BoG.

- Modification of ilrp_idc[ listIdx ][ rplsIdx ][ i ] semantics

Agreed. Also proposed JVET-P0589. JVET-P0185 has an aspect that is also related to this.

- Modification of inter_layer_ref_pics_present_flag semantics

No further action needed for this item, some aspects covered by earlier agreements.

- Bug fix of the derivation process of the variable DirectDependentLayerIdx[ i ][ j ]

Agreed. JVET-P0187 proposal 1 is related.

JVET-P0235 AHG17: A bugfix of SPS flags and reference picture list structure [T. Chujoh, E. Sasaki, T. Ikai (Sharp)]

Discussed Friday 10/4, chaired by Y.-K. Wang in the HLS BoG.

The text bug has already been fixed.

The software bug still needs to be noted.

JVET-P0356 AHG17: Bitstream constraints on RPL and GDR [R. Sjöberg, M. Pettersson, M. Damghanian (Ericsson)]

JVET-P0589 AHG8: On inter-layer reference picture index range [V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

Discussed Friday 10/4, chaired by Y.-K. Wang in the HLS BoG.

See notes elsewhere.

### AUD, picture header, slice header parameters signalling (10)

JVET-P0687 AHG17: A summary of HLS proposals on access unit delimiter, picture header, and slice header parameters signalling [Hendry (Futurewei)] [late]

Discussed Wednesday 2030 (GJS & JRO)

This contribution provides a summary of high-level syntax proposals submitted to this JVET meeting related to access unit delimiter, picture header, and slice header parameters signalling. A list of design questions is provided to help structure the review of the proposals.

The contributions in this category can be classified into two sub-categories:

- AUD and Picture Header

- Modifications to constant slice header paramater signalling (related to constant_slice_header_params_enabled_flag)

General questions:

- Do we want to avoid signalling syntax elements in slice header that are constrained to have same values for all slices of a picture?

If yes. Where to signal them? AUD? New NAL Picture Header? Use the constant slice header params signalling in PPS?

- For constant slice header parameters, should some of the syntax elements be in PPS? or in AUD? or in Picture Header, if exists?

On AUD and Picture Header.

- Do we want to have Picture Header?

Proposing yes. JVET-P0095, JVET-P0120, JVET-P0239

Why? JVET-P0092 & JVET-P0120: AUD is AU specific. An AU may contain multiple pictures.

JVET-P0120: Order of AUD is supposed to the first NAL unit in an AU. If AUD preceeds SPS & PPS, some picture-level syntax elements in SH cannot be moved to AUD. Having PH is asserted would solve this problem.

Decision: Yes, after considering the quantity of syntax elements that would go there, we should have a picture header.

- If yes, what to signal in PH?

  - Move some signalling from PPS to PH

JVET-P0095: Subpicture Id mapping, if it is not signalled in the SPS and not signaled in the PPS, to avoid needing to send a PPS for this

Decision: Allow the mapping in the PH when it is not in the SPS. If present, it overrides what is in the PPS. If not in the PPS and not in the SPS, then is shall be in the PH. As detailed below:

SPS syntax:

fancy_weird_stuff_allowed_flag

if( fancy_weird_stuff_allowed_flag ) {

sending_details_in_sps_flag /* infer 0 if not present */

if ( sending_details_in_sps_flag )

Details

}

PPS syntax:

sending_details_in_pps_flag

if (sending_details_in_pps_flag)

Details

constraint: If fancy_weird_stuff_allowed_flag is 0 or sending_details_in_sps_flag is 1, sending_details_in_pps_flag shall be equal to 0.

PH syntax:

if ( fancy_weird_stuff_allowed_flag && !sending_details_in_sps_flag ) {

sending_details_in_ph_flag

if(sending_details_in_ph_flag)

Details

}

  - Signal syntax element both in PPS and PH. Present in PH if not present in PPS

JVET-P0095: Tile/Brick/Rect Slice signalling

Deferred to after considering GDR needs.

  - Move some signalling from SH to PH

Preliminary decision:

    - JVET-P0095, P0120: no_output_of_prior_pics_flag

    - JVET-P0120, P0239: slice_pic_parameter_set_id, non_reference_picture_flag, colour_plane_id, slice_pic_order_cnt_lsb, recovery_poc_cnt, pic_output_flag, slice_temporal_mvp_enabled_flag.

Comment: Think more about slice_pic_order_cnt_lsb as a loss detection mechanism

    - JVET-P0239: slice_lmcs_aps_id and slice_scaling_list_aps_id

    - JVET-P0239 move some syntax elements that are currently not constrained to be the same for all slices of a picture, but asserted having no / minimal benefit and coding loss to transmitting them in every slice header as their anticipated usage would change at the picture level. 18 such syntax elements are identified in the contribution. (In the CTC, about half of these don't even change across different pictures.)

It was commented that this could affect the ability to merge slices from different encoders. However, others commented that encoding for such applications would already be customized to this use case (and possibly only coming from one common encoder or encoders from only one company).

      - six_minus_max_num_merge_cand

      - five_minus_max_num_subblock_merge_cand

      - slice_fpel_mmvd_enabled_flag

      - slice_disable_bdof_dmvr_flag

      - max_num_merge_cand_minus_max_num_triangle_cand

      - slice_six_minus_max_num_ibc_merge_cand

      - partition_constraints_override_flag

      - slice_log2_diff_min_qt_min_cb_luma

      - slice_max_mtt_hierarchy_depth_luma

      - slice_log2_diff_max_bt_min_qt_luma

      - slice_log2_diff_max_tt_min_qt_luma

      - slice_log2_diff_min_qt_min_cb_chroma (only needed for I slices)

      - slice_max_mtt_hierarchy_depth_chroma (only needed for I slices)

      - slice_log2_diff_max_bt_min_qt_chroma (only needed for I slices)

      - slice_log2_diff_max_tt_min_qt_chroma (only needed for I slices)

      - mvd_l1_zero_flag (only needed for B slices)

      - dep_quant_enabled_flag

      - sign_data_hiding_enabled_flag

  - Signal some syntax element that are not constrained to be the same for all slices of a picture both in PH and SH. Present in SH if not present in PH

    - JVET-P0120: RPL, joint cb/cr sign flag, SAO, ALF, LMCS, Scaling list

Preliminary decision:

joint cb/cr sign flag should be in the PH and never be in the SH

RPL, SAO, ALF, LMCS, deblocking (4 syntax elements in July output doc) should possible to put in SH (as override if also in higher layer), also possible in PH and in SPS (for each of these five categories, if it's in the PH, then it's not in the SH)

Scaling list control should never be in the SH

  - New syntax elements:

    - JVET-P0095: A flag to specify whether associated picture contains mixed NAL unit types (i.e, mixed_nalu_types_in_pic_flag). See JVET-P0124, JVET-P0146, and JVET-P0222.

Revisit after section 6.19.1.3.

    - JVET-P0120: Picture type (whether picture is IDR, CRA, GDR, contains only I-slices, only P- and I- slices, any type of slices)

Revisit after determining handling/need of AUD.

- If yes, what is property of PH?

  - JVET-P0095, JVET-P0120, JVET-P0239: Mandating one PH one picture

  - JVET-P0120: Specifies layer Id, temporal Id, order of PH in access unit, etc.

  - JVET-P0095, JVET-P0120: PH is a non-VCL NAL unit.

  - JVET-P0239: PH is a VCL NAL unit.

- Do we want to keep AUD?

Proposing no: JVET-P0095, JVET-P0120

- Do we want to keep mandating the presence of AUD?

Proposing no:

JVET-P0218. Signal single_slice_in_pic_flag in SPS. When single_slice_in_pic_flag is equal to 1, AUD is not mandated to be present. single_slice_in_pic_flag can be used to condition the presence of other syntax elements such as num_bricks_in_slice_minus1.

JVET-P0367: Signal a flag aud_in_next_access_unit_present_flag in AUD to specify if AUD is present for next AU.

JVET-P0480: AUD is mandated only when there are more than 1 subpicture. Signal first_slice_in_sub_pic_flag in SH. This flag can be used for detecting first slice of each picture when there is only one subpicture in a picture.

- Do we want to signal more syntax elements in AUD?

  - JVET-P0218: Optionally moves PPS Id, non_reference_picture_flag, colour_plane_id, slice_pic_order_cnt_lsb, and pic_output_flag to AUD, if present. Create a syntax structure pic_header(). NOTE: pic_header is not in its own NAL unit.

  - JVET-P0146: Signal syntax elements which specifies types of subpictures of a picture (non-IRAP, IDR, CRA). This would allow a picture to contain mixed subpicture types.

  - JVET-P0222: Signal rap_type and au_order_cnt. Syntax element rap_type specifies which specifies whether the coded pictures in the access unit contain non-IRAP NAL unit only, IRAP NAL unit only, or both IRAP and non-IRAP.

Comments: JVET-P0146 and first aspect of JVET-P0222 suggest to allow mixed of IRAP and non-IRAP in a picture. They should be discussed together with JVET-P0095 and JVET-P0124.

- AUD properties:

  - JVET-P0218: The AUD shall have nuh_layer_id value equal to vps_layer_id[ 0 ].

  - JVET-P0380: The AUD shall have nuh_layer_id value equal to 0.

  - JVET-P0218: The AUD shall have TemporalId equal to the TemporalId of the access unit containing the NAL unit

On signalling constant slice header params.

- Some bugs are asserted to be present in the signalling of syntax element when constant_slice_header_params_enabled_flag?

  - JVET-P0152:

    - Update semantics of pps_five-minus_max_mun_subblock_merge_cand_plus1 to incorporate the value of sps_affine_enabled_flag.

Comment: The proposed changes seem to be more appropriate to be expressed as constraints.

    - Update the inference of the value five_minus_max_num_subblock_merge_cand in SH when it is not present taking into account the proposed change above.

  - JVET-P0427 proposal 1: Condition the signalling of pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 on the value of pps_six_minus_max_num_merge_cand_plus1

  - JVET-P0427 proposal 2: Update the range value of pps_max_num_merge_cand_minus_max_triangle_cand_plus1 to not depend on the value of MaxNumMergeCand

  - JVET-P0427 proposal 3: MaxNumTriangleMergeCand is inferred to be equal to 2 in slice header if MaxNumMergeCand is equal to 2 without signaling in either PPS or slice headers

  - JVET-P0605: proposed to modify pps_max_num_merge_cand_minus_max_num_triangle_cand_minus1 to pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1

This has been addressed by editor between last meeting and this meeting. The update is reflected in JVET-P0113

- Adding more syntax element to be signalled in PPS when constant_slice_header_params_enabled_flag is equal to 1:

  - JVET-P0334 proposal / modification 1

  - JVET-P0368

  - JVET-P0428 proposal 1

- Improvement for signalling mechanism of constant slice header params:

  - JVET-P0334 proposal 2 & 3:

    - Instead of signalling 1 set of constant slice header params in PPS, allow signalling up to 7 sets of constant slice header params in PPS. In SH, signal index of the set to be used.

    - When the number of constant slice header params set in PPS is equal to 0, signal the syntax elements in SH

  - JVET-P0428 proposal 2: signal conformance_constant_params_enabled_flag in SH to specify if the slice uses the constant syntax elements signalled in PPS or not. This allow constant slice params to be present in the PPS but also present in SH (i.e., when the value of conformance_constant_params_enabled_flag is equal to 0)

JVET-P0480 AHG17: On simplification of subpicture design [R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)] [late]

The first_slice_in_sub_pic_flag part of this contribution belongs to this category. For other aspects see section 6.19.1.1.

JVET-P0095 AHG12/AHG17: On signalling of picture-specific syntax elements in access unit delimiter [M. M. Hannuksela (Nokia)]

JVET-P0120 AHG17: On access unit delimiter [Hendry, Y.-K. Wang (Futurewei)]

JVET-P0218 AHG8/AHG17: On Access Unit Delimiter and Picture Detection [J. Samuelsson, S. Deshpande, A. Segall (Sharp)]

JVET-P0222 AHG17: On signaling parameters in AUD [B. Choi, S. Wenger, S. Liu (Tencent)]

The aspect of rap_type was disccussed in section 6.19.1.3.

JVET-P0239 AHG17: HLS Cleanup [W. Wan, T. Hellman, B. Heng (Broadcom)]

JVET-P0334 AHG17: On constant slice header parameter set in PPS [B. Wang, S. Esenlik, A. M. Kotra, H. Gao, E. Alshina (Huawei)]

JVET-P0367 AHG17: On AUD signaling [M. Pettersson, R. Sjöberg, M. Damghanian (Ericsson)]

JVET-P0368 AHG17: On selectively signal slice header parameters in PPS [M. Pettersson, R. Sjöberg, M. Damghanian (Ericsson)]

JVET-P0427 AHG17: Bugfixes on constant slice header parameters [Y.-J. Chang, V. Seregin, M. Coban, A. Ramasubramonian, M. Karczewicz (Qualcomm)]

JVET-P0428 AHG17: Improvement on signaling of constant slice header parameters [Y.-J. Chang, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

### Miscellaneous HLS topics (15)

JVET-P0122 AHG17: On adaptation parameter set [Hendry, J. Chen, Y.-K. Wang (Futurewei)]

Discussed Friday 10/4, chaired by S. Deshpande in the HLS BoG.

Agreed to take option 1.

JVET-P0359 AHG17: An STSA_NUT restriction on PPS and APS availability [R. Sjöberg, M. Pettersson, M. Damghanian (Ericsson)]

Discussed Friday 10/4, chaired by Y.-K. Wang in the HLS BoG.

The sub-layer switching behavior that the proposal tries to ensuure is desirable. On the other hand, just putting such constraints in the spec seems having some issues with out-of-band transmission of PPSs and APSs.

It was agreed to add a NOTE expressing that, if PPSs and APSs are in-band, it'd better that the TemporalId is set equal to 0 for them, to be more friendly for sub-layer switching to work properfly. If such an SEI message is adopted, then we shoud not have the above NOTE.

It was suggested that specifying an SEI message that includes a flag, the value of which equal to 1 indicates that the proposed constraints apply. Revisit when syntax and semantics for such an SEI message is available. Consider inclusion of some of fields in the HEVC active parameter sets SEI message in the same SEI message.

JVET-P0184 AHG8/AHG17: On Decoding Process for Unavailable Reference Pictures and Reference Picture List Construction [S. Deshpande (Sharp)]

Discussed Friday 10/4, chaired by Y.-K. Wang in the HLS BoG.

Proposal 1: Modifications are proposed to the general decoding process for unavailable reference pictures and decoding process for reference picture list construction. It is asserted that these are necessary for correct handling of inter-layer reference pictures signalling in reference picture lists.

No action was taken.

Proposal 2: A conformance constraint in decoding process for reference picture list construction related to set of unique pictures referred by referenc picture lists 0 and 1 is proposed to be modified as a bug-fix.

Agreed (editorial bug fix).

JVET-P0243 AHG17: Unequal bit depths [W. Wan, T. Hellman, B. Heng (Broadcom)]

Discussed Friday 10/4, chaired by Y.-K. Wang in the HLS BoG.

The current VVC draft text allows the bit depth of the luma samples and the chroma samples to be coded independently, enabling more combinations than if the bit depths were constrained to be identical. This contribution claims that the cost of this flexibility, specifically the additional burden to test and verify these combinations, exceeds the practical benefit seen in product use so far as well as anticipated usage in the future. It is proposed to code a single bit depth to be used for both luma and chroma. This would significantly reduce the number of combinations while still maintaining some flexibility at the encoder to adjust bit depths, if it deems necessary.

It was commented that the YCgCo format, for lossless, the Y would be 8 bits and the other two formats would be 9 bits. It was suggested that for these type of signal, it'd be better to use larger bit depth, such as 10 bits, for all the components.

Agreed.

JVET-P0244 AHG17: Miscellaneous HLS corrections [W. Wan, T. Hellman (Broadcom)]

Discussed Friday 10/4, chaired by Y.-K. Wang in the HLS BoG.

This contribution proposes five changes to syntax elements in the SPS. The first three proposed changes are to change the parsing of syntax elements from Exp-Golomb coding to fixed length coding. The remaining two proposed changes suggest specifying valid conformance ranges for syntax elements that currently do not have an explicitly bounded range.

1. It is proposed to code sps_seq_parameter_set_id with u(4).

Agreed to use (4) for SPS IDs in the SPS syntax as well as in other places for referring to SPS.

2. It is proposed to code chroma_format_idc with u(2).

Agreed to code chroma_format_idc with u(3).

3. It is proposed to code log2_max_pic_order_cnt_lsb_minus4 with u(4).

Agreed.

4 It is proposed to specify the conformant range of this syntax element to be 0 to 48, inclusive.

Agreed.

5. It is proposed to specify the value range of log2_min_luma_coding_block_size_minus2 to be 0 to log2_ctu_size_minus5 + 3, inclusive.

JVET-P0429 contains the same proposal.

Agreed.

JVET-P0316 Coding of 360° video in non-compact cube layout using uncoded areas [J. Sauer, M. Bläser (RWTH Aachen)]

Discussed Friday 10/4, chaired by Y.-K. Wang in the HLS BoG.

Proposes to allow uncoded area.

This was asserted to allow more contious edges in CMP faces layout (at the cost of dubling the picture size), and can be useful in other applications as well.

The uncode areas are proposed to be padded with constant values.

The reported coding again agaisnt CMP 3 by 2 layout was up to 0.65% for Class S2.

It was commented that this could be done without a normative change.

It was commented that signalling of some related metadata in SEI messages that do not affect conformance could be an interesting area to look at.

Further study encouraged to have a generic mechanism to signal of metadata that indicates padded/non-coded regions that would apply to both 360 video and other applications.

JVET-P0366 AHG17: Signalling absence of NAL unit types in DPS [M. Pettersson, R. Sjöberg, M. Damghanian (Ericsson)]

Discussed Friday 10/4, chaired by Y.-K. Wang in the HLS BoG.

This contribution proposes to add flags to signal the absence of some of the NAL unit types in the bitstream in the general_constraint_info() struct in DPS and VPS. In a first option the following constraint flags are proposed to be added to the general_constraint_info():

- no_trail_constraint_flag

- no_stsa_constraint_flag

- no_rasl_constraint_flag

- no_radl_constraint_flag

- no_idr_constraint_flag

- no_cra_constraint_flag

- no_gdr_constraint_flag

- no_aps_constraint_flag

In a second option, constraint flags are only proposed for the NAL unit and picture types the proponents assess are the most useful. Another difference compared to the first option is that the no_idr_constraint_flag and the no_cra_constraint_flag are grouped into a no_irap_constraint_flag. The following constraint flags are proposed to be added to the general_constraint_info() in the second option:

- no_stsa_constraint_flag

- no_irap_constraint_flag

- no_gdr_constraint_flag

- no_aps_constraint_flag

It is proposed to add option 1 or option 2 to the VVC specification, or a mixture of the two.

It was commetend that it might be good to use one flag to group some features that sort of always go together.

Agreed on option 1. Further study enouraged on such grouping as mentioned above.

JVET-P0420 AHG8/AHG17: Reduction of overlapping between POC LSB and Temporal ID [M. Sychev (Huawei)]

JVET-P0438 AHG17: On Constraints for ALF APS [J. Chen, Hendry (Futurewei)]

Discussed Friday 10/4, chaired by S. Deshpande in the HLS BoG.

It was mentioned that if spliting of ALF APS into luma and chroma ALF APSs, then some of these would not be needed and reworded.

Agreed.

JVET-P0497 AHG17: On LMCS Signalling [Y.-C. Yang, C.-Y. Teng, P.-H. Lin (Foxconn)]

Discussed Friday 10/4, chaired by Y.-K. Wang in the HLS BoG.

Proposal 1: There was some support. However, there was no consensus to take an action.

Proposal 2: This is for coding efficiency improvement. The coding again is up to 0.02% for all intra. No action.

JVET-P0800 Crosscheck of JVET-P0497: AHG17: On LMCS Signalling [W. Zhu (Bytedance)]

JVET-P0588 AHG17: APS for low latency ALF [V. Seregin, M. Coban, A. K. Ramasubramonian, N. Hu, M. Karczewicz (Qualcomm)]

JVET-P0452 Non-CE5/AHG17: Low-delay ALF syntax [K. Sühring, R. Skupin, Y. Sanchez, T. Schierl (HHI)]

JVET-P0610 AHG8/AHG17: Sub-layer picture rates [M. Sychev (Huawei)]

Discussed Friday 10/4, chaired by M. M. Hannuksela in the HLS BoG.

Agreed on the SEI message option.

JVET-P0108 AHG17: On parsing dependency between slice data and APS [S. Esenlik, A. M. Kotra, B. Wang, H. Gao, E. Alshina (Huawei)]

Discussed Friday 10/4, chaired by S. Deshpande in the HLS BoG.

It was mentioned that slice level parsing dependency on parameter sets is natural.

It was commented that it is unclear that this would actually be used for error resilience in practice.

No action was taken.

JVET-P0332 AHG17/Non-CE5: On parsability of alternative ALF Chroma filter signaling [J. Taquet, P. Onno, C. Gisquet, G. Laroche (Canon)]

Discussed Friday 10/4, chaired by S. Deshpande in the HLS BoG.

See notes under JVET-P0108.

### HRD (6)

JVET-P0181 AHG17: On HRD Information Signalling [S. Deshpande (Sharp)]

Discussed Friday 10/4, chaired by Y.-K. Wang in the HLS BoG.

Proposal 1: It is proposed to conditionally signal initial buffering delay information related syntax elements based on if either bp_nal_hrd_parameters_present_flag is equal to 1 or bp_vcl_hrd_parameters_present_flag is equal to 1.

Agreed to require that bp_nal_hrd_parameters_present_flag cannot be both equal to 0, and then remove the syntax condition of when either is equal to 1.

Proposal 2: Modifications are proposed to avoid syntax element parsing dependency between Buffering Period SEI message and Picture timing SEI message.

No action was taken.

Proposal 3: Missing inference rules are provided for the du_cpb_removal_delay_increment_length_minus1 and dpb_output_delay_du_length_minus1 syntax elements.

Conditionally agreed to add the inferrence rules if the syntax elements are used in the spec when not present.

Proposal 4: It is proposed to signal a flag in Buffering Period SEI message to specify if the initial CPB buffering delay values are signalled for each temporal sub-layer or only for the highest temporal sub-layer. Also some additional bug-fixes are proposed.

Agreed. There are also some editorial improvements, and those were delegated to the editors.

JVET-P0183 AHG17: On Picture Timing Information Signalling [S. Deshpande (Sharp)]

Discussed Friday 10/4, chaired by Y.-K. Wang in the HLS BoG.

Proposal 1: It is proposed to provide signalling to specify that each subpicture is a decoding unit.

This would introduce a parsing dependency of PT/DUI SEI messages on SPS.

No action was taken.

Proposal 2: It is proposed to conditionally signal cpb_removal_delay_delta_enabled_flag[ i ] for i in the range of TemporalId to pt_max_sub_layers_minus1−1, inclusive, based on cpb_removal_delay_deltas_present_flag.

Agreed.

Proposal 3: It is asserted that the syntax element cpb_removal_delay_delta_idx[ i ] is coded as u(v) however the number of bits used for coding it are not specified. It is proposed to specify the number of bits used to code cpb_removal_delay_delta_idx[ i ].

Agreed with option A.

JVET-P0189 AHG8/AHG17: On buffering period, picture timing, and decoding unit information SEI messages [Y.-K. Wang (Futurewei)]

Discussed Friday 10/4, chaired by M. M. Hannuksela in the HLS BoG.

It was commented that now the picture header includes PPS ID, and the SEI messages come after the picture header, thus the SPS information needed for parsing the SEI messages would be known when parsing the SEI messages.

See notes under JVET-P0202.

JVET-P0202 AHG17: Harmonized HRD parameters signalling for decoding units [V. Drugeon (Panasonic)]

Discussed Friday 10/4, chaired by M. M. Hannuksela in the HLS BoG.

Agreed, but remove pt_max_sub_layers_minus1 and dui_max_sub_layers_minus1, and use the corresponding syntax element in the BP SEI message.

JVET-P0189 contains a similar proposal.

JVET-P0203 AHG17: Parsing HRD related SEI messages independently from the SPS [V. Drugeon (Panasonic), K. Sühring (HHI)]

Discussed Friday 10/4, chaired by M. M. Hannuksela in the HLS BoG.

Agreed. The propsoal allows parsing dependency of PT/DUI SEI messages on BP SEI message.

JVET-P0446 AHG17: On HRD for VVC: Splicing, Open GOP and DRAP support [Y. Sanchez, R. Skupin, K. Sühring, T. Schierl (HHI)]

Discussed Friday 10/4, chaired by Y.-K. Wang in the HLS BoG.

Part 1: Bug report fix: Deferred.

Part 2: On seamless switching

What is seamless switching that is used in the proposed semantics?

Why having the flag in each and every picture timing SEI message regardless of whether the AU contains a random acessible picture?

It was commented that it should be optional such that encoders could avoid trying to figure out the value.

The proponent said that this applies only when the bitstream has constant picture rate. Does the current concatenation_flag make sense when the bitstream does not havev constant picture rate?

Revisit.

Part 3: Alternative HRD parameters and HRD operations for discarding of RASL pictures and "skipped" pictures when random accessing from a picture associated with a DRAP picture.

Revisit.

### VUI and SEI (6)

JVET-P0337 AHG17: On porting SEI messages specified in HEVC and AVC [M. Sean, T. Lu, F. Pu, P. Yin, W. Husak, T. Chen (Dolby)]

Discussed Friday 10/4, chaired by Y.-K. Wang in the HLS BoG.

Agreed exluding the deinterlace field information SEI message. The software implementation for these SEI messages would be easy; just remove some macros.

JVET-P0338 AHG17: Shutter interval information SEI message [M. Sean, T. Lu, F. Pu, P. Yin, W. Husak, T. Chen (Dolby)]

Discussed Friday 10/4, chaired by Y.-K. Wang in the HLS BoG.

It was mentioned that the variant 2 proposal was adopted into HEVC by the JCT-VC earlier today (10/4/2019), with a change suggested to add the number of sub-layers to avoid parsing dependnecy on SPS.

Agreed, plus adding of the number of sub-layers to avoid parsing dependnecy on SPS.

JVET-P0404 SEI message for MPEG-I Part 7 metadata [M. M. Hannuksela (Nokia)]

Discussed Friday 10/4, chaired by Y.-K. Wang in the HLS BoG.

The HLS BoG liked the idea of having a generic MPEG-I Part 7 SEI message as suggested by the contribbution.

It was commented that the SEI message persistency mechanism seems not good.

Would it be good enough to say in the H.SEI spec that the persistency scope of this SEI message depends on the content (i.e., controlled by the semantics of the content specified in MPEG-I Part 7), e.g., just the same wording as for the user data SEI message? This way, persistency mechanism is not included.

Would it be sufficient to just use the user data SEI message? In that case, there needs to have a registration.

Revisit in track/plenary.

JVET-P0459 AHG17: Mergeability identifier for subpictures [R. Skupin, K. Sühring, S. Sanchez, T. Schierl (HHI)]

JVET-P0462 AHG6: 360-degree video related SEI messages [R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)]

Discussed Friday 10/4, chaired by Y.-K. Wang in the HLS BoG.

It was commented that a generic mechanism as proposed in JVET-P0404 might be sufficient without adding these SEI messages into H.SEI.

It was commented that some implementors or applications (application specs) may want just to use SEI messages without paying attention to MPEG-I Part 7.

It was commented that one shortcoming of doing a generic mechanism only is the imcompatibilties (only a few) between what is in OMAF / MPEG-I Part 7 and the SEI messages in HEVC/AVC. While adding these SEI messages needs more mainteance efforts, editorial work to adjust the wording to be generic.

It was commented that allowing SEI message content out of control of JVET may have implicatios on referencablity of H.SEI.

It was suggested that the generic mechanism can also be used for other types of metadata in MPEG-I Part 7, now or in the future, and thus it makes sense to do both.

Agreed, excluding the CMP SEI message.

JVET-P0597 AHG6/AHG17: Generalized cubemap projection syntax for 360-degree videos [Y.-H. Lee, J.-L. Lin, Y.-J. Chen, C.-C. Ju (MediaTek), J. Boyce, M. Dmitrichenko (Intel)]

Discussed Friday 10/4, chaired by Y.-K. Wang in the HLS BoG.

Why to support all the 4 different CMP packing types?

One mentioned justification is that different kinds packing types are being tried/used, not converged.

It was questioned that the outside world has not converged seemed not a convincing justifiction to support multiple options.

Propoent said that the packing/layout functionality included in the proposed SEI message is a subset of what is supported by the region-wise packing (RWP) SEI message.

It was asserted that the proposed SEI message enables some functionalities that cannot be realized by using the HEVC CMP SEI message in combination with the RWP SEI message.

It was suggested to split the proposed SEI message into two SEI messages, one contains the proejction part, and the other contains the packing/layout part.

If so, then the functionality of the second SEI message would be true subset of the functionality of the RWP SEI message. It seemed that is a bit silly to specify two SEI messages. And it was commented that SEI messages are not required to be fully implemented and tested by decoders. It was counter argued that even full implementations and tests by decoders are not needed, partial implementations do affect interoperablity. And some times it could be difficult to implement only parts of the functionalities of a particular feature, e.g., the RWP SEI message.

It was commented that saying like "this is just an SEI message, optional, decoders can ignore, etc." and using that as an argument for asking for adopting of SEI messages seems not right, and it's proably better for the MPEG-I people to clearly communicate to JVET what SEI messages they want.

Agreed, but remove the byte alingment two lines of syntax.

JVET-P0459 AHG17: Mergeability identifier for subpictures [R. Skupin, K. Sühring, S. Sanchez, T. Schierl (HHI)]

Discussed Friday 10/4, chaired by Y.-K. Wang in the HLS BoG.

There was some interest on have some kind of mergability indication.

There were some issues pointed out during the discussion. For example, the types are onion-shelled, thus it does not makes sense to signal more than one.

Further study encoraged.

## AHG12: high-level parallelism and coded picture regions (46)

### Subpictures (35)

#### General aspects (3)

JVET-P0693 AHG12: Summary of HLS proposals on subpictures [M. M. Hannuksela (Nokia)] [late]

Discussed Tuesday 2140 (GJS)

This contribution provides a topic-wise summary of contributions related to:

- General aspects of subpictures

- Subpicture layout and ID signalling

- Subpicture boundary filtering

The purpose of the summary is to help structure and improve efficiency of the discussion of these contributions.

This version of the contribution does not cover contributions related to:

- subpicture-level random access and merging

- subpicture wraparound, padding, and cropping

- subpicture conformance definition and signalling

Discussed general aspects:

- A subpicture consists of one or more complete rectangular slices within a picture. Subpictures cannot be used with raster-scan-order slices. (several contributions). Confirmed.

- Tile rows and columns at the picture level apply across the entire picture. (P0245) Confirmed.

- SPS/PPS parameters must be the same for all the subpictures in a picture. (P0245) Confirmed.

  - Supposedly this means that the same slice_pic_parameter_set_id value is referenced by all the VCL NAL units of the same picture Confirmed.

- APS parameters must be the same for all the subpictures in a picture. (P0245). Not confirmed. Limits are established in the text.

- All of the subpictures in the aggregate picture must use a common DPB with common DPB management (P0245) Confirmed. The DPB operates in units of pictures.

- Properties shared across each subpicture (P0245):

  - Same frame rate. Confirmed (i.e., each picture must be complete).

  - Same reference picture lists. Not confirmed. Each slice has its own RPL, but has the same RPS.

  - DPB can only be flushed at the same picture. Confirmed. The DPB operates in units of pictures.

  - Same temporal sublayer. Confirmed (i.e., each picture must be complete and all VCL NAL units of the picture shall have the same TID).

  - Same reference picture resampling (RPR) scale factor. Proposed to be violated by P0403.

  - Same collocated reference picture. Confirmed (as a semantic constraint).

  - Delta-POCs to all reference pictures must be the same. Confirmed. POC operates in units of pictures.

- Can subpictures, if extractable, have a different random access properties from the whole picture? (P0139) Possibly, but not necessarily affecting the decoding process

Discussed Wednesday 1100 (GJS) & JRO)

  - Container structure for signalling subpicture layout

- SPS

  - When signalled in the SPS, the signalling cannot use slices, tiles, or bricks for indicating subpicture boundaries

- PPS

  - When signalled in the PPS, the signalling can use slices, tiles, or bricks for indicating subpicture boundaries

  - Note: There can be semantic constraints to keep the subpicture layout unchanged within a C(L)VS. This is discussed below.

Currently, we have the subpicture layout established at the SPS level. We have no proposals trying to change the subpicture layout within a CLVS.

It was agreed to keep the subpicture signalling in the SPS.

  - Subpicture layout signalling in SPS

- Using a subpicture grid approach

  - Modifications of the subpicture grid design on VVC Draft 6

    - Indicating the grid in units of 8 luma samples (P0142)

    - Indicating the grid in units of CTBs (P0142, P0377, P0432)

    - replace max_subpics_minus1 with max_subpics_minus2 (P0129, P0432)

    - single_subpic_per_grid_flag to avoid signaling grid index for each subpicture when the grid is at the subpicture granularity (P0129)

    - Asserted bug fix: P0464 proposes modifications to the existing semantic derivations of the subpicture height and width dimensions i.e., SubPicHeight and SubPicWidth. In the current working draft (WD 6) of the Versatile Video Coding (VVC), the semantic derivations for SubPicHeight and SubPicWidth result in incorrect values when the subpicture grid index (subpic_grid_idx) present in the last row (i.e., subpic_grid_idx[row][col])  is different from the grid index present directly above (i.e., subpic_grid_idx[row-1][col]) it and alternatively, when the subpicture grid index present in the last column (i.e., subpic_grid_idx[row][col]) is different from the subpicture grid index present in front of it (i.e., subpic_grid_idx[row][col-1]).

  - Using the grid in units of CTBs with the signalling of the bottom-right delta grid index (P0139, P0143)

  - Having uniform and non-uniform subpicture grid signalling modes (similar to the signalling modes for tiles) (P0471)

- Using units of CTBs, indicating top-left and bottom-right coordinates (P0171)

Decision: Units of CTBs, using top-left with width (minus1) and height (minus1). The proponent of P0171 agreed to provide text and to help develop software for this.

Constraints:

  - It is required that sub-pictures cannot be overlapped with each other, and all the sub-pictures must cover the whole picture. (P0377). This is already specified, and confirmed not to be changed.

  - Add a requirement of bitstream conformance that the following constraints apply: NumSubPicGridCols and NumSubPicGridRows cannot be concurrently equal to 1 if subpics_present_flag is equal to 1. NumSubPics shall be equal to 0 if subpics_present_flag is equal to 0. (P0432). This is not relevant to the approach taken.

  - Subpicture layout signalling in PPS

  - This section of the document is not relevant to the approach taken.

  - Constraints on subpicture layout

  - This section of the document is not relevant to the approach taken.

  - single_slice_per_subpic_flag

- In SPS as an indication only (not affecting decoding) (P0579).

- A constraint flag general_constraint_info( ) of the PTL structure: single_slice_subpic_only_constraint_flag (P0579)

- In PPS, used for inferring the derivation of subpicture_id when they are not explicitly present (P0579)

- In PPS, used for skipping subpicture boundary signalling (P0126, P0130) or slice boundary signalling (P0126)

As an initial assessment, it seems sensible to not need to signal slice layout in the PPS if this special case applies. However, other details should be considered for drafting the exact syntax and semantics before making a final conclusion on this. Revisit to finalize.

  - Subpicture ID in slice header

- Subpicture ID is included in the slice header (P0126, P0431, …)

  - Present, when:

    - subpics_present_flag equal to 1 (P0126, P0579, P0609)

    - subpics_present_flag equal to 1 and NumSubPics greater than 0 (P0431)

    - rect_slice_flag equal to 1 (P0480)

- subpic ID is proposed in NAL unit header or slice header. First, a flag use_nuh_layer_id_as_subpic_id_flag is signalled in SPS. When use_nuh_layer_id_as_subpic_id_flag is equal to 1, subpicID is set equal to nuh_layer_id signaled in NAL unit header. When use_nuh_layer_id_as_subpic_id_flag is equal to 0, subpicID is set equal to subpic_id, which may be signalled in slice header (P0224)

- Define a subpicture index for each slice. This could be a fixed length code (FLC) at the top of the slice header for easy bit stream manipulation. (P0242)

  - Details missing from the contribution.

Decision: When the subpics_present_flag is equal to 1 (in which case, raster scan slices are not supported), a subpic ID is sent in the SH, and the slice address is interpreted as an address within the subpicture.

P0224 proposes to have a mode in which the layer ID is instead a subpicture ID. This would have two advantages: 1) saving bits for subpicture ID, 2) exposing the subpicture ID at a more easily accessible location in the NAL unit. A participant commented that it seems undesirable to mix purposes for the layer ID, and that this would only be useful when the number of subpictures is less than 65. No action was taken on that aspect.

  - Semantics of slice address (when subpictures are in use)

- Slice_address is defined as the slice index within the subpicture. (P0126, P0609)

- The slice address is the subPic brick ID (P0480).

  - subPic brick ID is the brick index within the subpicture, where bricks are indexed in their decoding order – to be confirmed with the proponent

Decision: When subpictures are used, the slice address is the slice index within the subpicture.

  - Other syntax elements

- first_slice_in_sub_pic_flag (P0480)

This could be used for detection of subpicture and AU boundaries.

For this aspect see 6.18.5

  - Mapping of subpicture IDs to the subpicture layout – may subpicture IDs change within a CVS?

- Yes (e.g. P0139)

- Yes, but it can be signalled if they stay unchanged (P0126)

  - It is indicated with a flag (in PPS) whether the mapping of subpicture IDs to subpicture indices remains unchanged within a CLVS or may change within a CLVS.

- No (those contributions proposing to signal the mapping in SPS, e.g. P0579, P0609)

Viewport-dependent streaming is a use case where it is suggested for the IDs to be able to change.

Decision:

- flag for presence of the mapping at the PPS level

- flag for presence of the mapping at the SPS level

- the flags cannot both be equal to 1

- if both flags are equal to 0, the IDs are the same as the indexes

- The range of values used for the IDs can be bigger than for the indexes

  - Container structure for the mapping of subpicture IDs

- A mapping indicated in the picture header NAL unit (P0095)

To be considered with picture header proposals.

- A mapping of subpicture IDs to subpicture indices is signalled in the PPS

- Loop over the number of subpictures in a picture, each loop entry indicating the subpicture ID (P0126, P0141, P0431, P0579)

- Loop over slices, each loop entry listing the subpicture ID to which the slice belongs (P0144)

Decision: The first approach above is selected (regardless of whether it is as the PPS or SPS level) as a consequence of the conclusion in the previous category.

- When subpicture IDs do not change in the CVS, a mapping of subpicture IDs to subpicture indices is signalled in the SPS (P0141, P0609)

See above conclusions.

- A mapping of subpicture IDs to subpicture indices is signalled in the SPS, loop over subpictures, each loop entry indicating the subpicture ID (P0579)

See above conclusions.

  - On explicitly signalled slice IDs

- The explicitly signalled slice IDs are removed from the PPS and the slice. (P0126, P0609)

This is confirmed as a consequence of the above conclusions.

It was commented that the subpicture concept has been designed to support some particular use cases in which there may be many subpictures in a picture (e.g., 96). The subpicture concept is not adequate without some special slice support (e.g., explicitly signalled slice IDs) if there is a limit on the number of allowed subpictures that is too restrictive.

- On flags controlling the behaviour of subpicture boundaries

- subpic_treated_as_pic_flag[ i ] and loop_filter_across_subpic_enabled_flag[ i ] moved to PPS (P0126, P0480, …)

Conclusion: Keep these in the SPS level.

- P0145 and P0246: Disallow loop_filter_across_subpic_enabled_flag[ i ] equal to 1 when subpic_treated_as_pic_flag[ i ] is equal to 1, by either of the following:

  - Make the presence loop_filter_across_subpic_enabled_flag[ i ] conditional on subpic_treated_as_pic_flag[ i ] equal to 0. When not present, the value of loop_filter_across_subpic_enabled_pic_flag[ i ] is inferred to be equal to 0.

| if( !subpic_treated_as_pic_flag[ i ] ) |  |
| --- | --- |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |

  - Semantic constraint.

Conclusion: Keep these independently switchable, since they affect entirely different parts of the decoding process (and it seems plausible to want to switch them independently).

- May the value of the flags change within a C(L)VS?

  - For each subpicture index i, subpic_treated_as_pic_flag[ i ] and loop_filter_across_subpic_enabled_flag[ i ] are required to be unchanged within a CLVS (P0126) / CVS (P0480).

  - Related to subpicture-based adaptive resolution change: The constraints on having the same subpicture layout and the same values of subpic_treated_as_pic_flag[ i ] and loop_filter_across_subpic_enabled_flag[ i ] throughout a CLVS are not imposed. Instead, when a reference picture is used for prediction, it is required that it has a subpicture with the same ID as the subpicture ID of the current slice.

Conclusion: No, these are in the SPS, so they can't change within the CLVS.

- Miscellaneous

- max_subpics_minus1 is changed to max_subpics_minus2 and added in the VPS. Adding num_subpics_minus2 in the SPS. (P0140)

Conclusion: No action, since the extraction and merging functionality requires the ability to have subpics_present_flag equal to 1 even when max_subpics_minus1 is equal to 0.

- A bug fix (P0579): To change "<=" to "<" in the following.

Conclusion: Actually, this was just a bad variable name, not an error in the loop count. However, this part of the spec is being replaced anyway.

| for( i = 0; i <= NumSubPics; i++ ) { |  |
| --- | --- |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |

JVET-P0139 AHG17/AHG12: General questions on subpicture designs for discussion [L. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0242 AHG12/AHG16/AHG17: Subpicture Support [W. Wan, T. Hellman, M. Zhou, B. Heng, P. Chen (Broadcom)]

JVET-P0245 AHG12/AHG17: Subpicture Properties [W. Wan, T. Hellman, B. Heng (Broadcom)]

#### Subpicture layout and ID signalling (18)

See 6.19.1.1 for the disposition of topics in this category.

JVET-P0126 AHG12: Signalling of subpicture IDs and layout [M. M. Hannuksela (Nokia), Y.-K. Wang, Hendry (Futurewei)]

Some aspects of this are discussed in section 6.19.2.

JVET-P0129 AHG12: On subpicture grid syntax [Y. He, A. Hamza (InterDigital)]

JVET-P0130 AHG12: On subpicture ID [Y. He, A. Hamza (InterDigital)]

JVET-P0140 AHG17/AHG12: Some comments on the VVC text [L. Chen, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

Aspects 1 and 2 of JVET-P0140 belong to this category.

JVET-P0141 AHG17/AHG12: On signalling the subpicture IDs [L. Chen, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0142 AHG17/AHG12: Comments on the subpicture grid in the SPS [L. Chen, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0143 AHG17/AHG12: On signalling of subpicture structure in the SPS [L. Chen, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0144 AHG17/AHG12: On associating slices with a subpicture [L. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

For a bug fix aspect, see section 6.19.2.

JVET-P0171 AHG12: Modification for subpicture [J. Li, K. Abe, V. Drugeon (Panasonic)]

JVET-P0224 AHG12: On sub-picture layout and ID [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

JVET-P0377 AHG12: Cleanups on syntax design of sub-pictures [K. Zhang, L. Zhang, H. Liu, Z. Deng, J. Xu, N. Zhang (Bytedance)]

JVET-P0431 AHG12: On subpicture ID [Y.-J. Chang, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-P0432 AHG12: On signaling and grid size of subpictures [Y.-J. Chang, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-P0464 AHG12: Comments on Subpicture Semantic Derivations [S. Paluri, J. Zhao, S. Kim (LGE)]

JVET-P0471 AHG12 Explicit Signalling of Uniform and Non-Uniform Subpicture Grid in the SPS [S. Paluri, J. Zhao, S. Kim (LGE)

JVET-P0480 AHG17: On simplification of subpicture design [R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)] [late]

For the AU boundary detection aspect of this proposal, see section 6.18.5.

JVET-P0579 AHG12: On Subpicture Information Signalling [S. Deshpande, J. Samuelsson (Sharp)]

JVET-P0609 AHG12: On slice address signalling [R. Sjöberg, M. Damghanian, M. Pettersson (Ericsson)] [late]

#### Subpicture-level random access and merging (7)

# Overall discussion:

Thursday Oct. 3, Chaired by S. Deshpande in HLS BoG.

It was commented that the main goal of the first 3 proposals is inferring picture type from NAL unit type. It was commented that this functionality should be gated by a new general constraint flag.

It was commented that the mixed nal unit type is mainly solving merging use case. And that it was asserted by the commenter that the extraction use case may be more important.

A proponent commented that the problem these proposals (at least the first 3) are trying to solve is not having to rewrite NAL unit types and that there are existing implementations using HEVC where such NALU rewriting is being done without a big burden (since it is at a fixed location).

It was asked if the decoder will have to check every VCL NAL unit type of a picture to know if it is an IDR picture.

Proponents commented that only one VCL NAL unit needs to be checked to know if it is an IDR picture or not.

It was commented by proponent that it is a good goal to have a bitstream merging supported without having to change any VCL NAL units PH. And it was asserted by proponents that these proposals would only require PPS and SPS to be rewritten currently. It was commented that for example careful consideration is needed for new HLS designs such as PH.

One participant commented that the benefit that these proposals brings seems rather small and for a corner case.

# Support was expressed by non-proponents for JVET-P0124/ JVET-P0095.

Revisit after offline discussion.

JVET-P0095 AHG12/AHG17: On signallin	g of picture-specific syntax elements in access unit delimiter [M. M. Hannuksela (Nokia)]

One aspect (on allowing mixed IRAP and non-IRAP NUTs within a picture) of JVET-P0095 belongs to here.

Thursday Oct. 3, Chaired by S. Deshpande in HLS BoG.

See general discussion notes above and notes under JVET-P0124.

JVET-P0124 AHG17: On mixed NAL unit types within a picture [Y.-K. Wang, Hendry (Futurewei)]

Thursday Oct. 3, Chaired by S. Deshpande in HLS BoG.

See also general discussion notes above.

For describing the use case:

The encoding could be done like this:

The client selects which subpictures are received and merges the bitstream. The following figure provides one example.

A new mixed_nalu_types_flag is proposed.

It was commented that for this topic JVET-P0095 is subset of solution in JVET-P0124.

It was asked about the burden of parsing this compared to the one where NAL unit-based parsing is used.

It was answered that we should put all fixed length NAL unit part first.

It was pointed out that putting this in PH means you still have to rewrite this.

So, it was then proposed by proponents that mixed_nalu_types_flag should be in PPS.

It was commented that this would increase number of required PPSes.

It was asked how CRAs will work with this approach. It was commented that they operate similarly to IDR. A bug in a constraint regarding two reference picture lists was pointed out and proponent agreed that this needs to be fixed. For example, further mixing needs to be allowed for leading and trailing picture types.

JVET-P0458 AHG17: NAL unit type mapping [R. Skupin, K. Sühring, Y. Sanchez, T. Schierl (HHI)]

Thursday Oct. 3, Chaired by S. Deshpande in HLS BoG.

See also general discussion notes above.

# It defines a new MAP_NUT and defines mapped_nut nal unit type syntax element in SPS.

It was commented that in this case middle box can not rely on just looking at NAL unit type to know real nal unit type. It was answered that this was the case.

It was pointed out that one difference of this versus of JVET-P0095 and JVET-P0124 is that this one is at sequence level compared to those two proposals doing this at PPS level.

It was discussed and agreed that two NAL unit types would be needed if want to support both IDR_N_LP and IDR_W_RADL and up to 4 new NAL unit types would be needed to also support CRAs.

It was commented that JVET-P0480 needs to only touch one SPS regarding this aspect.

JVET-P0146 AHG17/AHG12: On AUD as a picture header [L. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

Thursday Oct. 3, Chaired by S. Deshpande in HLS BoG.

See also general discussion notes above.

The discussion was focused on au_subpic_type[i] syntax element only, for which there is a loop proposed over subpictures in the picture.

Proponent said that this syntax can move to PH.

Proponent commented that this proposal is mainly for extraction and not for merging.

It was asked that the PH would need to be rewritten for merging use case. This was agreed. And proponent commented that only one bit needs to be changed in the picture header for the merging case.

It was commented by proponent that the proposed solution here can still apply when we have a solution to handle mixed nal unit type problem.

JVET-P0222 AHG17: On signaling parameters in AUD [B. Choi, S. Wenger, S. Liu (Tencent)]

The aspect of rap_type was discussed in the category.

Thursday Oct. 3, Chaired by S. Deshpande in HLS BoG.

It is proposed to add syntax element to AUD for rap_type indication.

Proponent mentioned that now for this proposal, the rap_type is proposed to be signaled in PH instead of in AUD.

The discussion was only focused on rap_type field part of this proposal.

It was commented by proponent that this proposal builds on top of either JVET-P0124/JVET-P0095/ JVET-P0480.

It was asked what additional benefit this brings on top of those proposals and that this would need changing the value of rap_type in PH after merging use case.

It was commented by proponent that this information can also move to PPS and that would avoid the need for rewriting PH.

It was asked and confirmed that this is an indication only proposal and has no impact on decoding process.

# Further study.

JVET-P0351 [AHG12] On Subpictures merging [N. Ouedraogo, F. Denoual, F. Mazé (Canon)]

The proposed solution allows incomplete CTUs that are not at the picture boundaries. It was commented that pictures must be complete, i.e., cannot contain missing samples. This could be resloved by putting some sample values, e.g., padded.

The proposal relates to inferrence of the value of the value of the split_cu_flag, which is a coding tool feature.

It was commented that this might need more than one conformance window. This means that there is a need to compose stuff to form the output picture, which was asseretd to be an implementation burden.

It was commtented that there might be other aspects, like motion vectors, in addition to split_cu_flag, that also need to be handled. The proponent clarified that, by moving the subpicture boundary to be the boudary between the boundary of the missing blocks, those apsects are addressed.

It was commented that without doing anything, the subpicture design seems to be somewhat restrictive.

It was commented that adding special case low-level tools handling for allowing such flexibilities seemed a bad idea.

It was suggested that just taking the inferrence of the split_cu_flag plus padding the missing samples could be a complete solution. It was quested how much better is this comapred to padding to make the picture boundary CTUs complete.

Further study encouraged.

JVET-P0378 AHG12: Removal of dependency between sub-pictures [K. Zhang, L. Zhang, H. Liu, Z. Deng, J. Xu, Y. Wang (Bytedance)]

#### Subpicture wraparound, padding, and cropping (4)

JVET-P0127 AHG12: On subpicture wraparound motion compensation signalling [Y. He, Y. He, A. Hamza (InterDigital)]

This is only for higher coding efficency.

Further study on how much coding efficiency improvement this brings for typical use cases.

JVET-P0220 AHG12: On padding processing with sub-pictures [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

See notes under P0127.

JVET-P0494 AHG12: Sub-picture signaling [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

See notes under P0581.

JVET-P0581 AHG12: On Subpicture Cropping [S. Deshpande, J. Samuelsson, A. Segall (Sharp)]

Proposes enabling signalling a conformance window for each subpicture.

The composition of the output picture is not described.

It was commented that needing to compose stuff to form the output picture would be an implementation burden and the composition would need a lot of memory bandwidth.

It was commented that, when taking into account the rendering engine, in 360 video applications, the rendering engine needs to handle de-projection and possibly also region-wise packing, thus in that enirnment, this addition composition of the output pictures does not seem like a real burden.

Subpictures can only be used in other applications than 360 video.

Does this solution bring any technical benefit compared to having a big conformance windwow that includes the padded areas, and having SEI messages or other mechanisms indicate the padded areas?

It was commented that signalling of some subpicture related metadata in SEI messages that do not affect conformance could be an interesting area to look at.

Further study encouraged to have a generic mechanism to signal of metadata that indicates padded regions that would apply to both 360 video and other applications.

#### Subpicture conformance definition and signalling (2)

JVET-P0131 AHG12: On output sub_picture sets [Y. He, A. Hamza (InterDigital)]

Propnent indicated that there was no further need to review this proposal because of earlier decisions made at this JVET meeting.

JVET-P0448 AHG17: Sub-picture level info for extraction and merging [R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)]

The HLS BoG agreed that there should be some mechnism for signalling/inferrence of PTL information (in the strict conformance sense or not) for sets of subpicture sequences.

Revist after some offline discussion.

#### Subpicture boundary filtering (2)

JVET-P0145 AHG17/AHG12: On the flags for processing with subpictures [L. Chen, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

See 6.19.1.1 for the disposition of this contribution.

JVET-P0246 AHG17/Non-CE5: on loop filter processing for subpicture treated as a picture

See 6.19.1.1 for the disposition of the HLS aspects of this contribution.

The low-level signal processing aspect of this contribution, see section 6.5.

### Slices, tiles, and bricks (12)

JVET-P0144 AHG17/AHG12: On associating slices with a subpicture [L. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

For other aspects of this contribution, see section 6.19.1.1.

A bug fix: When single_tile_in_pic_flag is equal to 1, the value of num_slices_in_pic_minus1 is inferred to be equal to 0. (P0144). Decision: Adopt this aspect if still relevant and necessary after consideration of JVET-P0240.

JVET-P0686 AHG12: A summary of HLS proposals on slices, tiles, and bricks [Y.-K. Wang (Futurewei)] [late]

Discussed Wednesday 1720 (GJS & JRO).

This contribution provides a summary of high-level syntax proposals submitted to this JVET meeting related to slices, tiles, and bricks. A list of design questions is provided to help structure the review of the proposals.

- Have a single signalling mode instead of separate uniform and explicit tile/brick spacing modes (JVET-P0096)? Note that this proposal overlaps with a lot of aspects listed below. See notes below for JVET-P0096.

- Change brick signalling as in JVET-P0233? Not relevant after other actions.

- Remove brick signalling and signal rectanglar slices directly based on tiles? (JVET-P0240). See notes for that contribution.

- Remove num_tiles_in_pic_minus1 from the brick syntax? (JVET-P0096, JVET-P0140, JVET-P0308, JVET-P0357, JVET-P0433) Not relevant after other actions.

- Change " i  <=  num_tiles_in_pic_minus1 + 1" to "i < NumTilesInPic" in the loop for brick signalling? (JVET-P0096, JVET-P0140, JVET-P0186, JVET-P0308, JVET-P0357, JVET-P0433)? Resolved by action taken on JVET-P0096.

- Add CTU size in PPS? (JVET-P0308, JVET-P0357, JVET-P0433) Resolved by action taken on JVET-P0096.

- Add "i > 0" to the syntax condition for brick_idx_delta_sign_flag[ i ]? (JVET-P0126, JVET-P0140, JVET-P0232) Not relevant if action taken on JVET-P0240.

- Add "isLastTileColumn[ BottomRightBrickIdx[ i − 1 ] ]  = =  0" to the syntax condition for brick_idx_delta_sign_flag[ i ]? (JVET-P0232)? Not relevant if action taken on JVET-P0240.

- Add the missing value range of brick_row_height_minus1[ i ][ j ] to be 0..RowHeight[ i ] − 2? (JVET-P0186) Not relevant if action taken on JVET-P0240.

- Allow a picture that contains only a single tile to be split into bricks/slices? (JVET-P0186) Editorial action item: Change the name of the flag and clarify the semantics to indicate that the flag disallows any partitioning of the picture into tiles/slices/brick/subpictures. Decision: Also allow there to be only one tile in the picture when the flag is 0 (which may or may not contain multiple slices).

- Change/fix the equation for deriving slices based on bricks? (JVET-P0126, JVET-P0186) Not relevant if action taken on JVET-P0240.

- Change the inferred value of brick_height_minus1[ i ] from RowHeight[ i ] − 1 to RowHeight[ i ] − 2? (JVET-P0231) Not relevant if action taken on JVET-P0240.

- Add a constraint to explicitly express the requirement that when single_tile_in_pic_flag is equal to 1 there shall be no brick splitting? (JVET-P0231) See item 11 above.

- Add loop_filter_across_tiles_enabled_flag? (JVET-P0252) Decision: Adopt.

- Split uniform_tile_spacing_flag into uniform_tile_row_spacing_flag and uniform_tile_col_spacing_flag? (JVET-P0308) Resolved by action taken on JVET-P0096.

- Replace tile_row_height_minus1[ i ] and tile_column_width_minus1[ i ] with tile_row_height_delta[ i ] plus a sign flag and tile_column_width_delta[ i ] plus a sign flag? (JVET-P0308) Resolved by action taken on JVET-P0096.

- Allow signalling of multiple alternative tile configurations in the PPS and referring to one in the slice header? (JVET-P0364) It was commented that this may be related to how we support GDR. Revisit after offline study and considering GDR/CE2 contributions.

- Replace "if( RowHeight[ i ] > 1 )" and "if( RowHeight[ i ] > 2 )" in brick syntax with "if( TileHeight[ i ] > 1 )" and "if( TileHeight[ i ] > 2 )", respectively, and add that TileHeight[ i ] is derived to be equal to RowHeight[ i % ( num_tile_columns_minus1 + 1 ) ]? (JVET-P0433) Not relevant if action taken on JVET-P0240.

JVET-P0096 AHG12: On signalling of tile and brick partitioning [M. M. Hannuksela (Nokia)]

Discussed Wednesday 1820 (GJS & JRO).

This contribution proposes having a single signalling mode instead of separate uniform and explicit tile/brick spacing modes. The contribution is asserted to reduce:

- the number of syntax elements from 13 to 8,

- the number of syntax table rows from 28 to 17,

- the length of the semantics, and

- the length of the scanning process in clause 6.5.1.

The contribution is asserted to require a similar amount or fewer bits compared to VVC Draft 6 depending on the partitioning scheme.

Decision (Cleanup/simplification): Adopt.

JVET-P0126 AHG12: Signalling of subpicture IDs and layout [M. M. Hannuksela (Nokia), Y.-K. Wang, Hendry (Futurewei)]

Some aspects in section 6.19.1.2.

Two aspects of JVET-P0126 (adding "i > 0" to the syntax condition for brick_idx_delta_sign_flag[ i ], and changing/fixing the equation for deriving slices based on bricks) belong to this category.

JVET-P0140 AHG17/AHG12: Some comments on the VVC text [L. Chen, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

Aspects 3 and 4 of JVET-P0140 belong to this category.

JVET-P0186 AHG12: On Tiles, Bricks, Slices [S. Deshpande (Sharp)]

JVET-P0231 AHG12: On brick information signalling [B.-K. Lee (XRis), D. Jun (Kyungnam University)]

JVET-P0232 AHG12: On rectangular slice addressing information [B.-K. Lee (XRis), D. Jun (Kyungnam University)]

JVET-P0233 AHG12: Signaling of tile and brick information [B.-K. Lee (XRis), D. Jun (Kyungnam University)]

JVET-P0240 AHG17: Slices [T. Hellman, W. Wan, M. Zhou, B. Heng, P. Chen (Broadcom)]

Discussed Wednesday 1740 (GJS & JRO).

This contribution proposes that the dimensions and locations of rectangular slices be signalled directly by using tile information rather than bricks. The proposal removes the need to predefine and store brick information at the PPS level in an effort to reduce the computation and storage required to determine rectangular slice dimensions and locations. It is asserted that there is no change to the slice and brick partitioning options available to the encoder, as the proposed changes only affect the signalling while keeping the underlying partitioning concepts intact.

The proposal was missing signalling for the number of entry points.

Brick boundaries would be established by a stop bit after each CTU row.

The proponent indicated that if the encoder does not want to use bricks, this could be indicated, e.g. at the picture or sequence level and the stop bit would not need to be sent.

It was noted that if the height is checked, the stop bit would not be needed after the last CTU row of the tile, which seems to be a sensible optimization.

The proponent indicated that the problem with the current syntax is that it requires storing information at the PPS level and using indirection to use this information which is needed only for determining brick sizes, whereas this could be signalled in the "old-fashioned" way of using a stop bit within the slice data.

It was commented that there is a bug in this approach, in that the entry points are useless because the vertical starting position of the brick would be unknown without parsing the slice data of the previous brick.

It was suggested that an alternative signalling that would achieve the same goal would be to signal in the slice header the number of bricks and the brick heights; this approach would preserve the usefulness of entry points and the ability to have multi-brick slices and would eliminate the need for the stop bits and might be more bit efficient.

It was questioned whether we really need multi-brick slices within tiles.

Revisit after studying:

- Do we need multi-brick slices within tiles?

- Text for the approach with brick heights in header.

- Text for the approach without multi-brick slices within tiles.

JVET-P0252 AHG12: Loop filter control flag for tile boundary [K. Abe, T. Toma, V. Drugeon (Panasonic)]

JVET-P0308 AHG12: On signalling of tile partitioning [J. Do, D. Park, Y.-U. Yoon, J.-G. Kim (KAU)]

JVET-P0357 AHG12: Signal the CTU size in both SPS and PPS [R. Sjöberg, M. Pettersson, M. Damghanian (Ericsson)]

JVET-P0364 AHG12: Multiple tile partitions in PPS [M. Pettersson, R. Sjöberg, M. Damghanian (Ericsson)]

JVET-P0433 AHG12: On brick signalling [Y.-J. Chang, M. Coban, M. Karczewicz (Qualcomm)]

## AHG8: layered coding and resolution adaptivity (36)

### Scalability (27)

#### Output layer sets (3)

See also P0226 (open).

JVET-P0115 AHG8: Scalability - general and operation points [Y.-K. Wang (Futurewei)]

Discussed 1630 Tuesday (GJS).

It is asserted that the scalability design in VVC version 1 should be as simple as possible as long as main scalability use cases are addressed, while those fancy features that are not necessary for main scalability use cases, whenever they complicates the design and the specification, should not be included in VVC version 1.

This contribution proposes signalling of output layer sets.

It was asked whether we should define OLSs with inter-layer referencing. The current draft supports only identification of one operating point.

It was commented that this may partly be a matter of whether we consider the perspective of the server, which may have a more complete data set than what is served to a decoder. It was commented that conformance of an entire server-side bitstream may not need to be specified, versus only specifying what is sent to a decoder (which uses one operating point) and how to extract that.

At least as a starting point, the following was agreed:

A bitstream fed to a decoder would have only one specified conforming decoded output.

Metadata may be present in a bitstream to indicate how to convert the bitstream to a different bitstream with a different conforming decoded output.

There may be a larger set of data stored on a server for which there is no conformance point specified as the decoded output, without first applying a conversion/extraction/rewriting process to that set of data.

This is different from the current spec in regard to temporal sublayering , as the current draft has a concept of an indication to a decoder of the highest sublayer to decode. Similarly, the current spec has a concept of a decoder receiving an indication of a layer to decode within a bitstream that contains more than one layer (for independent simulcast layers). Decision: Agreed. This affects only subclause 8.1.1 of the draft text.

The extraction process is not conceptually part of the decoding process; it is something that happens before the creation of a bitstream to be consumed by a decoder. It becomes part of the specification of the semantics of the syntax used for the extraction.

It was noted that the current text does not even have level specifications yet (or even profile text).

JVET-P0204 AHG8: Clarifications on output layers mode [V. Drugeon (Panasonic)]

JVET-P0225 AHG8: On output layer set and PTL signalling [B. Choi, S. Wenger, S. Liu (Tencent)]

#### Random access and POC (3)

Discussed Tuesday 1750 (GJS).

Both proposals suggest to allow IRAP and non-IRAP pictures in the same AU (not new; also supported in SHVC and SVC). For example, this would allow spatial scalability with more frequent IRAPs in the base layer than in the enhancement layer. Viewport-dependent ROI enhancement is another example, where there could be more frequent IRAPs in the enhancement layer than in the base layer.

Delay and coding efficiency were mentioned as issues involved in this. There was some questioning of whether this functionality is really needed.

It was mentioned that the point cloud coding activity in MPEG is using different IRAP periods in different layers. Another participant said that if the layers are independent, a system could use multiple bitstreams.

It was suggested that this functionality should not be difficult to support – mostly a matter of POC bookkeeping.

This is left open for revisit.

JVET-P0100 AHG8: Unaligned IRAP pictures across layers and layer-wise start-up [M. M. Hannuksela (Nokia)]

JVET-P0101 AHG8: POC derivation for a multi-layer bitstream [M. M. Hannuksela (Nokia)]

JVET-P0116 AHG8: Scalability - random access [Y.-K. Wang (Futurewei)]

#### PTL, bitstream extraction, and conformance (6)

Discussed Tuesday 1830 (GJS).

SHVC supports PTL indication per layer in the context of each OLS.

It was discussed whether level should indicate the incremental resource needed to decode an individual layer or the total resource needed to decode an entire OLS. In SVC the latter approach was taken and in SHVC the former approach was taken. It was commented that the desire here is to use the same resource pool for scalable decoding as for single-layer decoding (e.g., with RPR being the same resource as spatial scalability upsampling and with a single DPB serving as a multilayer DPB), which would suggest the OLS approach. In SHVC the concept was more like using multiple single-layer decoders to implement a scalable decoder. Decision in principle: Level indicates total resource for an operating point (incl. CPB and DPB).

[An operation point is a temporal subset of an OLS.]

JVET-P0117 AHG8: Scalability - PTL and decoder capability [Y.-K. Wang (Futurewei)]

JVET-P0098 AHG8/AHG17: Handling of VPS and EOB NAL units in the sub-bitstream extraction process [M. M. Hannuksela (Nokia)]

JVET-P0380 AHG8/AHG17: On the role of nuh_layer_id in the layer design [E. Thomas (TNO), M. Hannuksela (Nokia), L. Chen (MediaTek), Yong.He@InterDigital.com, S. Wenger, B. Choi (Tencent)]

JVET-P0118 AHG8: Scalability - HRD [Y.-K. Wang (Futurewei)]

JVET-P0125 AHG17: Miscellaneous HLS topics [Y.-K. Wang (Futurewei)]

JVET-P0190 AHG8/AHG17: Scalable nesting SEI message [Y.-K. Wang (Futurewei)]

#### Region-wise scalability (1)

JVET-P0336 AHG8: Support of ROI (Region-Of-Interest) scalability [T. Lu, F. Pu, P. Yin, M. Sean, W. Husak, T. Chen (Dolby)]

In SHVC we have the proposed functionality, illustrated below. A basic functionality is to have controllable spatial offsets and scaling between the reference layer and the enhancement layer. The proposal might also involve having some different treatment based on location within enhancement layer. It might also involve changing the bounding box of picture extrapolation for motion compensation.

It was commented that P0219 and P0482 for RPR are related.

Further clarity on the details of what is proposed and consideration of such details is needed.

Revisit.

#### VPS and single-layer bitstreams/decoders (2)

Discussed Tuesday 1950 (GJS).

At the previous meeting, we had agreed in principle for VPS syntax to not be required to be present in a single-layer bitstream (and we actually had an ability to not have the VPS present, but did not properly specify what happened in that case).

JVET-P0097 AHG8/AHG17: Removing dependencies on VPS from the decoding process of a non-scalable bitstream [M. M. Hannuksela (Nokia)]

This was said to be a bug fix for places in the spec text that are not aligned with the above-described decision.

JVET-P0205 AHG17: Presence of Video Parameter Set in bitstreams [V. Drugeon (Panasonic)]

This was said to be a bug fix for places in the spec text that are not aligned with the above-described decision.

JVET-P0185 AHG8/AHG17: On Video Parameter Set and Highest Temporal Sub-layer [S. Deshpande (Sharp)]

This has a bug fix for the above-described issue and has some additional aspects.

JVET-P0135 AHG8/AHG17: On inter-layer reference picture signalling [X. Ma, H. Yang (Huawei)]

Proposal 1 (on VPS signalling of layer dependency) belongs to this category.

#### External independent layers (1)

Discussed Tuesday 2005 (GJS).

The basic question is whether to support external independent layers as was done in SHVC. This could be something for further study as an extension project, but does not seem like a priority to consider for v1.

JVET-P0213 AHG8: External independent layers [M. M. Hannuksela (Nokia)]

#### Multi-layer based single-layer decoding and subpicture support (3)

JVET-P0212 AHG8/AHG12: Decoding multiple independent layers with single-layer decoding process [M. M. Hannuksela (Nokia), S. Wenger, B. Choi (Tencent), E. Thomas (TNO), V. Seregin, M. Coban (Qualcomm), Y. He (InterDigital), L. Chen (MediaTek)]

Discussed Tuesday 2015 (GJS).

This proposes a method of using a single decoder instance and the single-layer VVC decoding process for decoding multiple independent layers. The decoding is performed as if all NAL units resided in a single layer only.

It is reported that the proposal can be used with any types of independent layers, such as "independent coded region" layers, and texture or depth layers, e.g. for 3DoF+. The type of a layer has no normative impact in the decoding process.

One use of this is to enable subpictures by decoding the rectangles separately and using a post-decoding compositor to assemble a combined picture. It was commented that this can enable non-CTU-aligned subpicture sizes. However, it was commented that the compositing step would be an additional processing compositing stage that would be needed and may or may not be outside the scope of the specified conforming decoding process.

In the contribution, the layout of the subpictures is signalled in an SEI message.

This was assuming aspects such as PTL per layer that are different from the design philosophy agreed to earlier in the meeting.

No action was taken on this.

JVET-P0226 AHG8/AHG12: On output layer set with sub-pictures [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

Discussed Tuesday 2035 (GJS).

Part of this relates to multi-layer-based subpicture support as with P0212; see notes for that contribution.

Another part proposes signalling of OLSs and should be considered with contributions in 6.20.1.1.

JVET-P0481 AHG17: On output layers and output layer sets [Y. Sanchez, R. Skupin, K. Sühring, T. Schierl (HHI)] [late]

Discussed Tuesday 2050 (GJS).

Part of this relates to multi-layer-based subpicture support as with P0212 and P0226; see notes for that contribution.

In this proposal, the subpictures would be CTU aligned.

This aspect can provide a way to indicate conformance of a set of subpictures, since the subset an OLS. However, a participant commented that the same thing can be done for the illustrated use case using subpictures without using layers.

That subpicture aspect is only one functionality of the proposed approach.

Another is layer switching where the decoder is instructed to treat different layer IDs as the same for adaptive stream switching. However, it was commented that the server could use just one layer ID for both tracks and keep track for itself of which track to put into the bitstream that is given to the decoder and when to switch between the tracks; this sort of thing is sometimes done today (and DASH has some track switching metadata support).

There was no clear need for action on this, although it can be kept in mind if some further study indicates a way this can support necessary functionality that is not otherwise provided.

#### Miscellaneous scalability HLS topics (7)

JVET-P0128 AHG8: Scalability - GDR [Y. He, A. Hamza (InterDigital)]

JVET-P0132 AHG8: On inter-layer motion vector prediction [Y. He, Y. He, A. Hamza (InterDigital)]

JVET-P0187 AHG8/AHG17: Comments on HLS of VVC [S. Deshpande (Sharp)]

Aspects 1 and 2 belong to this category.

JVET-P0223 AHG8: Signalling representation format [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

JVET-P0228 AHG17: General comments on HLS of VVC [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-P0453 AHG8: On diagonal referencing for layered VVC [Y. Sanchez, R. Skupin, K. Sühring, T. Schierl (HHI)]

Discussed Tuesday 2120 (GJS).

This contribution proposes to add support of diagonal referencing for layered VVC bitstream, i.e., inter-layer dependency for picture of different access units.

It was reported that SVC had a "medium granularity scalability" referencing of higher reference layers, but otherwise we have not supported diagonal referencing in prior scalability designs.

The proposal allows diagonal references to lower layers.

This would seem not especially difficult to support. However, no strong argument for supporting it was provided. Further study was encouraged to determine whether there is any significant advantage (e.g., in coding efficiency) for supporting this.

### Reference picture resampling (RPR) (8)

JVET-P0206 AHG17: Clean-up on disabling temporal motion vector prediction in case of reference picture resampling [T. Nishi, V. Drugeon (Panasonic)]

JVET-P0219 AHG8/AHG17: On signaling reference picture resampling [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-P0241 AHG17/CE1-related: RPR [T. Hellman, W. Wan, P. Chen (Broadcom)]

JVET-P0403 AHG8/AHG12: Subpicture-specific reference picture resampling [M. M. Hannuksela, A. Aminlou, K. Kammachi-Sreedhar (Nokia)]

JVET-P0450 AHG17: On Sample Aspect Ratio for RPR [R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)]

JVET-P0482 AHG8: On reference picture resampling [Y. Sanchez, R. Skupin, K. Sühring, T. Schierl (HHI)] [late]

JVET-P0590 AHG8: Scaling window for scaling ratio derivation [V. Seregin, A. K. Ramasubramonian, M. Coban, M. Karczewicz (Qualcomm)]

JVET-P0591 AHG8: Resampled output picture [V. Seregin, A. K. Ramasubramonian, M. Coban, M. Karczewicz (Qualcomm)]

# Complexity analysis and reduction (2)

Contributions in this category were discussed XXday X Oct. XXXX–XXXX in Track X (chaired by XXX).

JVET-P0084 Decoding Energy Assessment of VTM-6.0 [M. Kränzler, C. Herglotz, A. Kaup]

JVET-P0085 Bit Stream Analyzer for Coding Tool Statistics [C. Herglotz, M. Kränzler, A. Kaup]

# Encoder optimization (1)

Contributions in this category were discussed XXday X Oct. XXXX–XXXX in Track X (chaired by XXX).

JVET-P0092 AHG13: Encoder speed up for SMVD [H. Chen, H. Yang (Huawei)]

# Metrics and evaluation criteria (1)

Contributions in this category were discussed XXday X Oct. XXXX–XXXX in Track X (chaired by XXX).

JVET-P0393 On BD rate computation for tools affecting quantization [S. Keating, K. Sharman, A. Browne (Sony)]

# Withdrawn (14)

JVET-P0048 Withdrawn

JVET-P0102 Withdrawn

JVET-P0103 Withdrawn

JVET-P0104 Withdrawn

JVET-P0138 Withdrawn

JVET-P0227 Withdrawn

JVET-P0229 Withdrawn

JVET-P0268 Withdrawn

JVET-P0296 Withdrawn

JVET-P0323 Withdrawn

JVET-P0564 Withdrawn

JVET-P0618 Withdrawn

JVET-P0627 Withdrawn

JVET-P0765 Withdrawn

JVET-P0794	Withdrawn

# Plenary meetings, joint meetings, BoG reports, and summary of actions taken

## High-level syntax / systems relation meeting Saturday 5 Oct. 0900-XXXX

This session, held on XXXX at XXXX, was co-chaired by GJS and Youngkown Lim.

## Plenary meeting XXday X Oct. XXXX-XXXX

Reports of the tracks were presented as follows:

The general status of track A was presented and discussed, which particularly included the following aspects (not every small decision is noted):

…

The general status of track B was then presented and discussed, which particularly included the following aspects. Decisions recommended from track B were agreed and approved, unless otherwise noted:

…

## Closing Plenary meeting Friday 11 Oct.

… .

## Joint meeting XXday XX March XXXX-

JVET with … .

## BoGs (X)

## List of actions taken affecting the draft text of VVC, the VTM, and 360Lib

The following is a summary, in the form of a brief list, of the actions taken at the meeting that affect the text of the VVC draft text, VTM or 360Lib description. Both technical and editorial issues are included. This list is provided only as a summary – details of specific actions are noted elsewhere in this report and the list provided here may not be complete and correct. The listing of a document number only indicates that the document is related, not that it was adopted in whole or in part. The description given in the “Tool” column is a best effort for the sake of understanding but may not precisely reflect the functionality of the tool. It is also noted that in cases where several contributions proposed the same method, usually only one of the is listed as adoption below; refer to the meeting notes about the adoption to see which other contributions are related.

…

# Project planning

## Core experiment planning

…

## Drafting of specification text, encoder algorithm descriptions, and software

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in EEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without draft specification text

- VTM algorithm description text is strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be XXday XX Jan 2020.

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name.

## General issues for experiments

It was emphasized during the opening plenary on January 9 that those rules which had been set up or refined during the 12th meeting should be observed. In particular, for some CEs, results were available late, and some changes in the experimental setup (particularly in CE4) were not discussed on the JVET reflector.

Group coordinated experiments have been planned as follows:

- “Core experiments” (CEs) are the coordinated experiments on coding tools which are deemed to be interesting but require more investigation and could potentially become part of the draft standard by the next meeting.

- A CE is a test of a specific fully described technology in a specific agreed way. It is not a forum for thinking of new ideas (like an AHG). The CE coordinators are responsible for making sure tha the CE description is complete and correct and has adequate detail. Reflector discussions about CE description clarity and other aspects of CE plans are encouraged.

- A description of each experiment is to be approved at the meeting at which the experiment plan is established. This should include the issues that were raised by other experts when the tool was presented, e.g., interference with other tools, contribution of different elements that are part of a package, etc. The experiment description document should provide the names of individual people, not just company names.

- Software for tools investigated in a CE will be provided in one or more separate branches of the software repository. Each CE will have a “fork” of the software, and within the CE there may be multiple branches established by the CE coordinator. The software coordinator will help coordinate the creation of these forks and branches and their naming. All JVET members will have read access to the CE software branches (using shared read-only credentials; the method for members to obtain the credentials is TBA on the reflector).

- During the experiment, revisions of the experiment plans can be made, but not substantial changes to the proposed technology.

- The CE description must match the CE testing that is done. The CE description needs to be revised if there has been some change of plans.

- The CE summary report must describe any changes that were made in the process of finalizing the CE.

- By the next meeting it is expected that at least one independent cross-checker will report a detailed analysis of each proposed feature that has been tested and confirm that the implementation is correct. Commentary on the potential benefits and disadvantages of the proposed technology in cross-checking reports is highly encouraged. Having multiple cross-checking reports is also highly encouraged (especially if the cross-checking involves more than confirmation of correct test results). The reports of cross-checking activities may (and generally should) be integrated into the CE report rather than submitted as separate documents.

It is possible to define sub-experiments within particular CEs, for example designated as CEX.a, CEX.b, etc., where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the group test model software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments are described in the output document JVET-N1010.

Experiment descriptions should be written in a way such that it is understood as a JVET output document (written from an objective “third party perspective”, not a proponent perspective – e.g. not referring to methods as “improved”, “optimized”, etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions contain a basic description of the technology under test, but should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in documents that are referenced in the CE description that are also available in the JVET document archive.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

[Add info on software access.]

Some agreements relating to CE activities were established as follows:

- Only qualified JVET members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting. Participation is requested by contacting the CE coordinator.

- All software, results, and documents produced in the CE should be announced and made available to JVET in a timely manner.

- A JVET CE reflector will be established and announced on the main JVET reflector. Discussion of logistics arrangements, exchange of data, minor refinement of the test plans, and preparation of documents shall be conducted on the JVET CE reflector, with subject lines prefixed by “[CEx: ]”, where “x” is the number of the CE. All substantial communications about a CE other than such details shall take place on main JVET reflector. In the case that large amounts of data are to be distributed, it is recommended to send a link to the data rather than the data itself, or upload the data as an input contribution to the next meeting.

General timeline for CEs

Reviewed Thursday 11 July 2019 1900 (Y. Ye).

T1= 3 weeks after the JVET meeting: To revise the CE description and refine questions to be answered. Questions should be discussed and agreed on JVET reflector. Any changes of planned tests after this time need to be announced and discussed on the JVET reflector. Initially assigned description numbers shall not be changed later. If a test is skipped, it is to marked as “withdrawn”.

T2 = Test model software release + 2 weeks or X XX, whichever is earlier: Integration of all tools into a separate CE branch of the VTM is completed and announced to JVET reflector.

- Initial study by cross-checkers can begin.

- Proponents may continue to modify the software in this branch until T3

- 3rd parties are encouraged to study and make contributions to the next meeting with proposed changes

T3: 3 weeks before the next JVET meeting or T2 + 1 week, whichever is later: Any changes to the CE test branches of the software must be frozen, so the cross-checkers can know exactly what they are cross-checking. A software version tag should be created at this time. The name of the cross-checkers and list of specific tests for each tool under study in the CE plan description shall be documented in an updated CE description by this time.

T4: Regular document deadline – 1 week: CE contribution documents including specification text and complete test results shall be uploaded to the JVET document repository (particularly for proposals targeting to be promoted to the draft standard at the next meeting).

The CE summary reports shall be available by the regular deadline. This shall include documentation about crosscheck of software, matching of CE description and confirmation of the appropriateness of the text change, as well as sufficient crosscheck results to create evidence about correctness (crosscheckers must send this information to the CE coordinator at least 3 days ahead of the document deadline). Furthermore, any deviations from the timelines above shall be documented. The numbers used in the summary report shall not be changed relative to the description document.

CE reports may contain additional information about tests of straightforwared combinations of the identified technologies. Such supplemental testing needs to be clearly identified in the report if it was not part of the CE plan.

New branches may be created which combine two or more tools included in the CE document or the VTM (as applicable).

It is not necessary to formally name cross-checkers in the initial version of the CE description document. To adopt a proposed feature at the next meeting, we would like see comprehensive cross-checking done, with analysis that the description matches the software, and recommendation of value of the tool given tradeoffs.

The establishment of a CE does not indicate that a proposed technology is mature for adoption or that the testing conducted in the CE is fully adequate for assessing the merits of the technology, and a favourable outcome of CE does not indicate a need for adoption of the technology.

Availability of spec text is important to have a detailed understanding of the technology and also to judge what its impact on the complexity of the spec will be. There must also be sufficient time to study it in detail. CE contributions without sufficiently mature draft spec text in the CE input document should not be considered for adoption.

Plans for the CEs to be conducted were established Thursday 11 July (Y. Ye); CE plan documents were reviewed Thursday 11 July (Y. Ye).

Lists of participants in CE documents should be pruned to include only the active participants. Read access to software will be available to all members.

## Software development and anchor generation

The planned timeline for software releases was established as follows:

- VTM6.0 will be released by 2019-08-12 including all adoptions necessary for CTC and CE basis references. VTM6.1 with non-CTC adoptions will be released later. Further versions of VTM may be released for additional bug fixing, as appropriate.

- Preparation of the VTM software will include immediate removal of macros that were added in the previous meeting cycle. The software coordinator has the discretion to retain some such macros.

- No change of of 360lib or HDRTools was noted in response to meeting.

# Establishment of ad hoc groups

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups was agreed to be the main JVET reflector (jvet@lists.rwth-aachen.de).

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| Project Management (AHG1) (jvet@lists.rwth-aachen.de) - Coordinate overall JVET interim efforts. - Supervise CE and AHG studies. - Report on project status to JVET reflector. - Provide a report to next meeting on project coordination status. | J.-R. Ohm, G. J. Sullivan (co-chairs) | N |
| Draft text and test model algorithm description editing (AHG2) (jvet@lists.rwth-aachen.de) - Produce and finalize JVET-O2001 VVC text specification draft 6. - Produce and finalize JVET-O2002 VVC Test Model 6 (VTM 6) Algorithm and Encoder Description. - Gather and address comments for refinement of these documents. - Coordinate with test model software development AhG to address issues relating to mismatches between software and text. | B. Bross, J. Chen (co-chairs), J. Boyce, S. Kim, S. Liu, Y.-K. Wang, Y. Ye (vice-chairs) | N |
| Test model software development (AHG3) (jvet@lists.rwth-aachen.de) - Coordinate development of test model (VTM) software and associated configuration files. - Produce documentation of software usage for distribution with the software. - Discuss and make recommendations on the software development process. - Propose improvements to the guideline document for developments of the test model software. - Perform tests of VTM 6 behaviour relative to HEVC and VTM 5 using the VTM common test conditions. - Coordinate with AHG on Draft text and test model algorithm description editing (AHG2) to identify any mismatches between software and text, and make further updates and cleanups to the software as appropriate. - Coordinate with AHG6 for integration with 360lib software. | F. Bossen, X. Li, K. Sühring (co-chairs) | N |
| Test material and visual assessment (AHG4) (jvet@lists.rwth-aachen.de) - Maintain the video sequence test material database for development of the VVC standard. - Identify and recommend appropriate test materials for use in the development of the VVC standard. - Identify missing types of video material, solicit contributions, collect, and make available a variety of video sequence test material. - Evaluate new test sequences, particularly including the material recently submitted by the Blender Foundation / Blender Animation Studio and Twitch. - Propose a new structure for the test sequence repository. - Prepare availability of viewing equipment and facilities arrangements for the next meeting, and prepare testing upon consultation with CE coordinators. - Coordinate with AHG11 on test material for screen content coding | T. Suzuki, M. Wien (co-chairs), V. Baroncini, R. Chernyak, A. Norkin, J. Ye (vice-chairs) | N |
| Memory bandwidth consumption of coding tools (AHG5) (jvet@lists.rwth-aachen.de) - Develop improved software tools for measuring both average and worst case of memory bandwidth, and provide information for usage of these tools. - Study cache configurations for measuring decoder memory bandwidth consumption. - Identify coding tools in CEs and VTM with significant memory bandwidth impact. - Study the impact of memory bandwidth on specific application cases. | R. Hashimoto (chair), T. Ikai, X. Li, D. Luo, H. Yang, M. Zhou (vice-chairs) | N |
| 360° video coding tools, software and test conditions (AHG6) (jvet@lists.rwth-aachen.de) - Study the effect on compression and subjective quality of different projections formats, resolutions, and packing layouts. - Discuss refinements of common test conditions, test sequences, and evaluation criteria. - Solicit additional test sequences, and evaluate suitability of test sequences on head-mounted displays and normal 2D displays. - Study coding tools dedicated to 360° video, their impact on compression, and implications to the core codec design. - Study the effect of viewport resolution, field of view, and viewport speed/direction on visual comfort. - Study complexity of GPU rendering of projection formats - Study syntax for signalling of projection formats, cubeface layouts, spherical rotations - Prepare and deliver the 360Lib-9.1 software version and common test condition configuration files according to JVET-M1012. - Generate CTC (PHEC) anchors and PERP results for VTM according to JVET-M1012, and finalize the reporting template for the common test conditions. - Produce documentation of software usage for distribution with the software. | J. Boyce and Y. He (co-chairs), K. Choi, J.-L. Lin, Y. Ye (vice-chairs) | N |
| Coding of HDR/WCG material (AHG7) (jvet@lists.rwth-aachen.de) - Study and evaluate available HDR/WCG test content. - Study objective metrics for quality assessment of HDR/WCG material, including investigation of the correlation between subjective and objective results. - Compare the performance of the VTM and HM for HDR/WCG content. - Prepare for expert viewing of HDR content at the next JVET meeting if feasible. - Investigate the implications of chroma sampling location. - Coordinate implementation of HDR anchor aspects in the test model software with AHG3. - Study additional aspects of coding HDR/WCG content. | A. Segall (chair), E. François, W. Husak, S. Iwamura, D. Rusanovskyy (vice-chairs) | N |
| Layered coding and resolution adaptivity (AHG8) (jvet@lists.rwth-aachen.de) - Study adaptive-resolution coding approaches for real-time communication, adaptive streaming, and 360-degree viewport-dependent streaming, including reference picture management and related scope and signalling - Study approaches for temporal scalability to avoid temporal judder when temporal scalability sub-bitstream extraction is used for achieving lower frame rate, and consider whether this should have a normative impact. - Develop a software framework which allows testing various layered coding and resolution adaptivity modalities in the context of the VTM software - Study approaches for support of layered coding scalability including spatial, temporal, quality, and view scalability; and analyse their coding efficiency and complexity characteristics - Coordinate with CEx on resampling filters | S. Wenger and A. Segall (co-chairs), M. M. Hannuksela, Hendry, S. McCarthy, Y.-C. Sun, P. Topiwala, M. Zhou (vice-chairs) | N |
| Neural networks in video coding (AHG9) (jvet@lists.rwth-aachen.de) - Investigate the benefit of using neural networks in video compression such as CNN loop filter, intra prediction, resampling in adaptive resolution coding, and encoder-side partition mode decisions. - Investigate the complexity impact of using neural networks in video compression. - Investigate the complexity measurement of neural network coding tools. - Investigate benefit of universal versus selectable networks, both in terms of compression benefit and complexity. - Investigate how CNN parameters can be established for operation of the decoding process. - Investigate the impact of training materials on the performance of neural network coding tools. - Investigate the impact of the training process on performance and complexity. | S. Liu and Y. M. Li (co-chairs), B. Choi, K. Kawamura, Y. Li, L. Wang, P. Wu, H. Yang (vice-chairs) | N |
| Encoding algorithm optimization (AHG10) (jvet@lists.rwth-aachen.de) - Study the impact of using techniques such as GOP structures and perceptually optimized adaptive quantization for encoder optimization. - Study the impact of adaptive quantization on individual tools in the test model. - Study the quantization adaptation tool in the test model. - Investigate the feasibility of adding a CTC test category in which adaptive quantization is turned on. - Study quality metrics for measuring subjective quality using e.g. the CfP response MOS scores. - Investigate other methods of improving objective and/or subjective quality, including adaptive coding structures, adaptive quantization without signalling, and multi-pass encoding. - Study methods of rate control and their impact on performance, subjective and objective quality. | A. Duenas, A. Tourapis (co-chairs), S. Ikonin, A. Norkin, R. Sjöberg, J. Le Tanou, J.-M. Thiesse (vice-chairs) | N |
| Screen content coding (AHG11) (jvet@lists.rwth-aachen.de) - Investigate coding tools targeted at screen content in terms of compression benefit and implementation complexity. - Identify test materials, discuss testing conditions for screen content coding, and propose associated updated common test conditions. - Study the impact of loop filters on screen content coding. | S. Liu (chair), J. Boyce, A. Filippov, Y.-C. Sun, J. Xu, H. Yang (vice-chairs) | N |
| High-level parallelism and coded picture regions (AHG12) (jvet@lists.rwth-aachen.de) - Study wavefront processing including the relationship with tiles and low delay characteristics. - Study flexible loop filter control and tile size restriction, including identifying implications on coding tools and implementation. - Study flexible tile partitioning (e.g. more flexible than HEVC and tile boundaries not spanning a full picture). - Study support of independently coded picture regions, including easy rewriting of such regions into a conforming sub-bitstream. - Prepare software and configurations for the test model to facilitate parallel processing tests. - Study the coding efficiency impact of parallel processing and coded picture regions. | S. Deshpande (chair), M. M. Hannuksela, R. Sjöberg, R. Skupin, W. Wan, Y.-K. Wang S. Wenger (vice-chairs) | N |
| Tool reporting procedure (AHG13) (jvet@lists.rwth-aachen.de) - Prepare output document JVET-O2005, which describes the methodology of tool-off testing and a list of tools to be tested by identified testers. - Provide configurations files, bitstreams, and results of tool-on/tool-off testing. - Use the tool usage counts and memory bandwidth usage to study the decoder complexity of features in on/off testing. - Prepare a report with results of the tests. | W.-J. Chien, J. Boyce (co-chairs), W. Chen, Y.-W. Chen, R. Chernyak, K. Choi, R. Hashimoto, Y.-W. Huang, H. Jang, R.-L. Liao, S. Liu (vice-chairs) | N |
| Operation modes for low latency support (AHG14) (jvet@lists.rwth-aachen.de) - Define relevant test conditions for the study of low latency modes - Study and propose low-latency performance assessment criteria/metrics - Update the implementation in the VTM model for supporting GRA. - Study a parallel framework for GRA assessment | J.-M. Thiesse (chair), S. Deshpande, A. Duenas, Hendry, K. Kazui, R. Sjöberg, A. Tourapis (vice-chairs) | N |
| Quantization control (AHG15) (jvet@lists.rwth-aachen.de) - Identify methods for quantization step size control for luma and chroma, including spatially and frequency-adaptive approaches - Develop methods for evaluating quantization step size control operation - Study the association between transforms and quantization matrices - Develop testing conditions for evaluating QP signalling improvements including rate control and perceptual optimization strategies as appropriate - Evaluate the performance of the current VVC QP design using the adaptive quantization control techniques currently available in the VTM | R. Chernyak (chair), E. François, C. Helmrich, S. McCarthy, A. Segall (vice-chairs) | N |
| Implementation studies (AHG16) (jvet@lists.rwth-aachen.de) - Study draft and proposed coding tools to identify implementation issues relating to decoder pipelines, decoder throughput, and other aspects of implementation difficulty. - Solicit hardware analysis of complex tools. - Provide feedback on potential solutions to address identified issues. | M. Zhou (chair), J. An, E. Chai, K. Choi, S. Sethuraman, T. Hsieh, X. Xiu (vice-chairs) | N |
| High-level syntax (AHG17) (jvet@lists.rwth-aachen.de) - Study NAL unit header, sequence parameter set, picture parameter set, adaptation parameter set, and tile group header syntax designs - Study reference picture buffering and list construction - Study random access signalling and random access approaches, including approaches with reference pictures provided by external means - Assist in software development and text drafting for the high-level syntax in the VVC design. - Study syntax approaches for interoperability point signalling - Study selection of constraint flags to be included in the VTM and their impact on syntax, semantics, and decoding process | R. Sjöberg, J. Boyce (co-chairs), S. Deshpande, M. M. Hannuksela, R. Skupin, A. Tourapis, Y.-K. Wang, W. Wan, S. Wenger (vice-chairs) | N |
| Lossless and near-lossless coding tools (AHG18) (jvet@lists.rwth-aachen.de) - Study coding tools for lossless and near-lossless coding, including transform skip, BDPCM, and other potential technologies - Consider the interaction between coding tools and other processing such as loop filtering and LMCS for lossless and near-lossless coding - Develop proposals for lossless and near-lossless coding for chroma and non-YCbCr colour space content - Consider throughput bottlenecks for lossless and near-lossless coding at high resolutions and frame rates | T. Nguyen and T.-C. Ma (co-chairs), M. Ikeda, S. Iwamura, H. Jang, X. Zhao (vice-chairs) | N |

# Output documents

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production. Where applicable, dates of planned finalization and corresponding parent-body document numbers are also noted.

It was reminded that in cases where the JVET document is also made available as MPEG output document, a separate version under the MPEG document header should be generated. This version should be sent to GJS and JRO for upload.

JVET-O2000 Meeting Report of the 15th JVET Meeting [G. J. Sullivan, J.-R. Ohm] (2019-xx-xx, near next meeting)

Initial versions of the meeting notes (d0 … d9) were made available on a daily basis during the meeting.

JVET-O2001 Versatile Video Coding (Draft 6) [B. Bross, J. Chen, S. Liu] [WG 11 CD 23090-3, N18692] (2019-07-31)

(Initial version planned to be made available by 2019-07-18.)

See the list of elements under section 11.7, as agreed by the Fri. 12 July plenary.

JVET-O2002 Algorithm description for Versatile Video Coding and Test Model 6 (VTM 6) [J. Chen, Y. Ye, S. Kim] [WG 11 N 18693] (2019-09-09)

(Initial version planned to be made available by 2019-08-15.)

See the list of elements under section 0, as agreed by the Wed. 27 March plenary.

Remains valid – not updated: JVET-N1003 Guidelines for VVC reference software development [K. Sühring] (2019-04-01)

Remains valid – not updated: JVET-M1004 Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 9) [Y. Ye, J. Boyce] (2019-02-15)

JVET-O2005 Methodology and reporting template for coding tool testing [W.-J. Chien and J. Boyce] (2019-07-31)

Remains valid – not updated: JVET-M1006 Methodology and reporting template for neural network coding tool testing [Y. Li, S. Liu, K. Kawamura] (2019-02-01)

This output was produced to capture aspects specific to enable study of neural network techniques.

JVET-O2007 Supplemental enhancement information messages for coded video bitstreams [J. Boyce, G. J. Sullivan, Y.-K. Wang] [WG 11 CD 23002-7, N18699] (2019-07-31)

(.)

See the list of elements under section 11.8, as agreed by the Wed. 27 March plenary.

Remains valid – not updated: JVET-N1010 JVET common test conditions and software reference configurations for SDR video [F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring] (2019-04-12)

(Old document revised for editorial refinement of headings.)

JVET-O2011 JVET common test conditions and evaluation procedures for HDR/WCG video [A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy] (2019-07-31)

Remains valid – not updated: JVET-L1012 JVET common test conditions and evaluation procedures for 360° video [P. Hanhart, J. Boyce, K. Choi, J.-L. Lin] (2018-10-26)

JVET-O2021 Description of Core Experiment 1 (CE 1): Reference picture resampling filters [D. Luo, V. Seregin, W. Wan]

Regarding 0.5 second switching point, it was agreed to round down the switching point frame number to be a multiple of 4 to align with the GOP structure in LDB. In terms of the number of pictures tested, it will be set equal to 5-second-worth of test material without rounding.

Because the dimensions of coded pictures (full-resolution and downsampled) must be multiple of 8, it was agreed that the CE coordinators and participants will fill out the detailed plan for what lower resolution to code for each of the different source resolutions and different resampling ratios in the CE description document.

For PSNR1 caluclation, averaging PSNR values of different picture sizes may not be desirable. One way to address this is to accumulate the MSEs of all pictures and only convert to PSNR at the end of coding. It was agreed that the CE coordinators and participants will fill out the detailed plan for PSNR1 caluclation in the CE description document.

JVET-O2022 Description of Core Experiment 2 (CE2): Gradual decoding refresh [K. Kazui, J.-M. Thiesse, Hendry, K. Kawamura]

Reviewed in closing plenary 1245.

Will test normative change of intra boundary, normative change of MC boundary extrapolation, and wavefront GDR.

JVET-O2023 Description of Core Experiment 3 (CE3): Intra prediction and mode coding [G. Van der Auwera, L. Li, A. Filippov]

JVET-O2024 Description of Core Experiment 4 (CE4): Inter prediction [C.-C. Chen, H. Yang, X. Xiu]

JVET-O2025 Description of Core Experiment 5 (CE5): Loop filtering [C.-Y. Chen, A. Norkin]

Includes ALF, SAO, bilateral, HDTF, and deblocking.

Includes considering removal of SAO (or merging filters).

CE5-1.3 will include testing the proposed methods with DMVR off.

CE5-3.3 will include testing removal of SAO.

JVET-O2026 Description of Core Experiment 6 (CE6): Transforms and transform signalling [H. E. Egilmez, M. Salehifar, X. Zhao]

CE tests that retrain the basis functions will provide the training software, training data, and version information of the VTM software used to conduct the training.

The CE test condition will include some non-CTC sequences.

JVET-O2027 Description of Core Experiment 7 (CE7): Quantization and coefficient coding [H. Schwarz, M. Coban, C. Auyeung]

Bug fix to ensure that bin-to-bit ratio is accurate will be included in VTM6.0.

JVET-O2028 Description of Core Experiment 8 (CE8): 4:4:4 Screen Content Coding Tools [X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu]

It was noted that CTC for 4:4:4 content has not been decided yet.

In the context of CE8, it was agreed that palette mode should be turned on in the anchors.

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (ordinarily starting meetings on the Tuesday of the first week and closing it on the Wednesday of the second week of the SG 16 meeting – a total of 9 meeting days), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (ordinarily starting meetings on the Wednesday prior to such meetings and closing it at lunchtime on the last day of the WG 11 meeting – a total of 9.5 meeting days).

In cases where an exceptionally high workload is expected for a meeting, an earlier starting date may be defined. For the upcoming meeting in October 2019, the SG16 parent-level meeting dates had been shifted after the JVET and MPEG meeting plans were established, which has caused an additional adjustment in the JVET dates.

Some specific future meeting plans (to be confirmed) were established as follows:

- Wed. 8 – Fri. 17 January 2020, 17th meeting under WG 11 auspices in Brussels, BE.

- Wed. 15 – Fri. 24 April 2020, 18th meeting under WG 11 auspices in Alpbach, AT.

- Tue. 23 June – Wed. 1 July 2020, 19th meeting under ITU-T auspices in Geneva, CH.

- Wed. 7 – Fri. 16 October 2020, 20th meeting under WG 11 auspices in Rennes, FR.

The agreed document deadline for the 16th JVET meeting was planned to be XXday XX January 2020. CE proposal documents are due one week ahead of that date. Plans for scheduling of agenda items within that meeting remained TBA.

ITU was thanked for the excellent hosting of the 14th meeting of the JVET.

EBU, HHI, ITU-T and Sharp Labs of America were thanked for providing equipment used for subjective viewing during the 14th JVET meeting. Philippe Hanhart, Andrew Segall and Mathias Wien were thanked for preparing and conducting the subjective test efforts related to CE1, CE5 and CE11, and Vittorio Baroncini was thanked for his advice. Roger Miles was thanked for support in providing the displays from EBU; Kenneth Andersson, Johannes Sauer and Vadim Seregin were thanked for carefully transporting these. The experts who participated in the role as test subjects were also thanked.

The 16th JVET meeting was closed at approximately XXXX hours on Friday 11 Oct. 2019.

# Annex A to JVET report:
List of documents

# Annex B to JVET report:
List of meeting participants

The participants of the sixteenth meeting of the JVET, according to an attendance sheet circulated during the meeting sessions (approximately XXX people in total), were as follows: