| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0968-v4 |
| --- | --- |

| Title: | Report of BoG on high-level syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | BoG report |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Futurewei, San Diego, CA, USA | Email: | +1-908-803-3888 ye-kui.wang@futurewei.com |
| Source: | BoG chair |  |  |

_____________________________

# Abstract

The HLS BoG met on the following days and time slots:

- 3 Oct. Thursday 10:30 to 21:40

- 4 Oct. Friday 09:00 to 21:35

- 5 Oct. Friday 09:00 to 21:30

In the list of recommendations below, the name in each instance of "SW/name" is suggested to be the person who would be responsible for coordinating the software implementation (if needed) of the recommendation if confirmed by the JVET. Note that there is no need for identifying a person for spec text coordination for HLS adoptions.

Note that decisions (e.g., on subpicture topics) that were made earlier at JVET plenary or track meetings are not listed below. There were also some purely editorial suggestions that were noted in the HLS BoG meeting minutes to be delegated to the editors, and those are not listed here, either.

The BoG recommends the following adoptions:

- Reserve nuh_layer_id values 56 to 63, inclusive, in VVCv1 [JVET-P0362][SW/R. Sjöberg]

- Rearrange/cleanup the NAL unit types as follows [JVET-P0363][SW/M. Pettersson]:

  - Switch position of RADL and RASL in the table to align with the order in HEVC.

  - Add VPS_NUT, which is missing, and order the parameter set NUTs in hierarchical order as in HEVC, in the order of DPS_NUT, VPS_NUT, SPS_NUT, PPS_NUT, APS_NUT.

  - Group reserved non-IRAP VCL NUTs within one code range instead of two.

  - Allocate 5 codes instead of 6 for reserved non-IRAP VCL NUTs and 3 codes instead of 2 for reserved IRAP VCL NUTs.

- Change the PTL syntax and semantics as follows [JVET-P0217][SW/J. Samuelsson]:

  - Move general constraints and general_level_idc before sub-profiles.

  - Allow signalling of zero or more sub-profiles.

  - Add inference of sub-layer level.

- Allow more than one PTL syntax structure in the DPS, with the following changes: reserve 5 bits instead of 1 bit, and use u(4) for the number of PTL syntax structures. [JVET-P0478, SW/R. Skupin]

- Add the following RPL-related constraints to ensure that the inter prediction relationship among IRAP, RASL, RADL, trailing, and GDR pictures etc. function as expected [JVET-P0978][SW/M. Pettersson]:

- When the current picture is a CRA picture, there shall be no entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes, in output order or decoding order, any preceding IRAP picture in decoding order (when present).

- When the current picture is a trailing picture, there shall be no picture referred to by an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that was generated by the decoding process for generating unavailable reference pictures for the IRAP picture associated with the current picture.

- When the current picture is a trailing picture that follows in both decoding order and output order one or more leading pictures associated with the same IRAP picture, if any, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that was generated by the decoding process for generating unavailable reference pictures for the IRAP picture associated with the current picture.

- When the current picture is a recovery point picture or a picture that follows the recovery point picture in output order, there shall be no entry in RefPicList[ 0 ] or RefPicList[ 1 ] that contains a picture that was generated by the decoding process for generating unavailable reference pictures for the GDR picture of the recovery point picture.

- When the current picture is a trailing picture, there shall be no picture referred to by an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes the associated IRAP picture in output order or decoding order.

- When the current picture is a trailing picture that follows in both decoding order and output order one or more leading pictures associated with the same IRAP picture, if any, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes the associated IRAP picture in output order or decoding order

- When the current picture is a RADL picture, there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that is any of the following:

  - A RASL picture

  - A picture that was generated by the decoding process for generating unavailable reference pictures

  - A picture that precedes the associated IRAP picture in decoding order

And replace the requirement on exact match of GDR recovery point picture to a NOTE (exact wording delegated to editors).

- Constrain that, when vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, the value of inter_layer_ref_pics_present_flag shall be equal to 0. [JVET-P0182, JVET-P0221][SW/S. Deshpande]

- Adopt the following improvement and bug fix: [JVET-P0221, JVET-P0589][SW/B. Choi]

  - Update the value range of ilrp_idc[ listIdx ][ rplsIdx ][ i ] to be in the range of 0 to the NumDependentLayers[ GeneralLayerIdx[ nuh_layer_id ] ] − 1, inclusive.

  - Fix the equation for derivation of DirectDependentLayerIdx, by changing "i < " to "i  <=  ", and "j = i" to "j = i − 1".

- Remove the following constraints: [JVET-P0122][SW/Hendry]

- For intra slices and slices in an IRAP picture, slice_alf_aps_id_luma[ i ] shall not refer to an ALF APS associated with other pictures rather than the picture containing the intra slices or the IRAP picture.

- For intra slices and slices in an IRAP picture, slice_alf_aps_id_chroma shall not refer to an ALF APS associated with other pictures rather than the picture containing the intra slices or the IRAP picture.

- Add an SEI message that contains only a flag self_contained_cvs_flag in its syntax. The flag equal to 1 indicates that parameter sets are "self-contained" in a manner that enables random access from any CVSS picture without fetching parameter sets from AUs before the CVS (this piece is the same as in HEVC), and additionally, enables sub-layer up-switching at STSA pictures without fetching parameter sets from AUs before the switching point (this piece is not in HEVC). [JVET-P0359][SW/R. Sjöberg]

- Adopt the following clarifications: [JVET-P0184][SW/S. Deshpande]

  - Clarify the process for generating unavailable reference pictures such that it is clear that the process only applies to intra-layer reference pictures.

  - Fix the use of the sps_max_dec_pic_buffering_minus1 syntax element without using an index now that it is an array/list.

- Adopt the following clean-up changes: [JVET-P0243, JVET-P0244, JVET-P0429][SW/W. Wan]

  - Signal a single bit depth to be used for both luma and chroma.

  - Use u(4) for SPS IDs in the SPS syntax as well as in other places for referring to SPS.

  - Code chroma_format_idc with u(3).

  - Code log2_max_pic_order_cnt_lsb_minus4 with u(4).

  - Specify the value range of 1.4	min_qp_prime_ts_minus4 to be 0 to 48, inclusive.

  - Specify the value range of log2_min_luma_coding_block_size_minus2 to be 0 to log2_ctu_size_minus5 + 3, inclusive.

- Add the following constraint flags into the general_constraint_info( ) syntax structure: [JVET-P0366][SW/M. Pettersson]

  - no_trail_constraint_flag

  - no_stsa_constraint_flag

  - no_rasl_constraint_flag

  - no_radl_constraint_flag

  - no_idr_constraint_flag

  - no_cra_constraint_flag

  - no_gdr_constraint_flag

  - no_aps_constraint_flag

- Add the following constraints to ensure that the referencing of ALF parameters from slice header to ALF APS works as expected: [JVET-P0438][SW/J. Chen]

  - It is requirement of bitstream conformance that the values of alf_luma_filter_signal_flag and alf_chroma_filter_signal_flag shall not both be equal to 0.

  - The value of alf_luma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_luma[ i ] shall be equal to 1.

  - The value of alf_chroma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_chroma shall be equal to 1.

- Allow an APS NAL unit to be after the last VCL NAL unit of the associated picture, by defining two NUTs for APSs, prefix APS NUT and suffix APS NUT. [JVET-P0588, JVET-P0452][SW/V. Seregin]

- Adopt the following clean-up changes: [JVET-P0181, JVET-P0183][SW/S. Deshpande]

  - Require that bp_nal_hrd_parameters_present_flag and bp_vcl_hrd_parameters_present_flag cannot be both equal to 0, and clean-up the syntax of the BP SEI message accordingly.

  - Add inference rules for du_cpb_removal_delay_increment_length_minus1 and dpb_output_delay_du_length_minus1 if needed.

  - Add a flag to the BP SEI message syntax to specify whether the initial CPB buffering delay values are signalled for each temporal sub-layer or only for the highest temporal sub-layer.

  - Conditionally signal cpb_removal_delay_delta_enabled_flag[ i ] for i in the range of TemporalId to pt_max_sub_layers_minus1−1, inclusive, based on cpb_removal_delay_deltas_present_flag.

  - Specify the length of cpb_removal_delay_delta_idx[ i ] to be Ceil( Log2( num_cpb_removal_delay_deltas_minus1+1) ) bits.

- Adopt the following changes related to the BP, PT, and DUI SEI messages: [JVET-P0202, JVET-P0189, JVET-P0203][SW/ V. Drugeon]

  - Add a loop to the PT and the DUI SEI messages for signalling of CPB timing parameters for sub-layers in DU-based HRD operations, and remove pt_max_sub_layers_minus1 and dui_max_sub_layers_minus1 and use the corresponding syntax element in the BP SEI message.

  - Repeat decoding_unit_hrd_params_present_flag in the BP SEI message and move decoding_unit_cbp_params_in_pic_timing_sei_flag to the BP SEI message, and use these syntax elements in the BP SEI message in syntax conditions in the PT and DUI SEI messages as needed. [JVET-P0202, JVET-P0189][SW/ V. Drugeon]

- Add the following HEVC and AVC SEI messages to ITU-T H.SEI | ISO/IEC 23002-7. [JVET-P0337][SW/M. Sean]

  - User data registered by Recommendation ITU-T T.35 SEI message

  - Film grain characteristics SEI message

  - Frame packing arrangement SEI message

  - Mastering display colour volume SEI message

  - Content light level information SEI message

  - Alternative transfer characteristics information SEI message

  - Ambient viewing environment SEI message

  - Content colour volume SEI message

- Add the following HEVC and AVC SEI messages to ITU-T H.SEI | ISO/IEC 23002-7. [JVET-P0462][SW/R. Skupin]

  - Equirectangular Projection SEI message

  - Sphere rotation SEI message

  - Region-wise packing SEI message

  - Omnidirectional viewport SEI message

- Adopt a new SEI message that provides picture rate information, as proposed in JVET-P0610. Into ITU-T H.SEI | ISO/IEC 23002-7. [JVET-P0610][SW/M. Sychev]

- Adopt a new SEI message that provides shutter interval information, as proposed in JVET-P0338, but add the number of sub-layers to avoid parsing dependency on SPS. Into ITU-T H.SEI | ISO/IEC 23002-7. [JVET-P0338][SW/M. Sean]

- Adopt a new generalized cubemap projection SEI message, as proposed in JVET-P0597, but remove two lines of syntax for the byte alignment. into ITU-T H.SEI | ISO/IEC 23002-7. [JVET-P0597][SW/Y.-H. Lee]

- Adopt a new SEI message that indicates subpicture level information, with the syntax as proposed in JVET-P0984, with the following changes: [JVET-P0984][SW/R. Skupin]

  - Change the reserved 4 bits to reserved 5 bits for byte alignment.

  - Change "shall have" in NOTEs to "has".

  - Add the SPS ID.

  - Correct the wording of where the CPB size information is signalled.

- Adopt a new SEI message that provides information on SAR dynamics, with the syntax as proposed in JVET-P0450 option 2, but drop the cancel flag and the persistence flag. The persistency scope is the set of AUs starting from the current until (exclusive) the next AU associated with the same SEI message. [JVET-P0450][SW/R. Skupin]

- Fix a bug on dependency between subpictures, by fixing some equations in the clause of the derivation process for constructed affine control point motion vector merging candidates and in the clause of the luma integer sample fetching process. [JVET-P0378, JVET-P0572][SW/K. Zhang]

- Adopt the following clean-up changes: [JVET-P0228][SW/B. Choi]

  - Add "&& !vps_all_independent_layers_flag" to the syntax condition for vps_output_layers_mode, and add the inference rule "When not present, the value of vps_output_layers_mode is inferred to be equal to 1."

  - Replace "pic_output_flag" in the following sentence with "PictureOutputFlag": vps_output_layer_mode is equal to 0 and the current access unit contains a picture that has pic_output_flag equal to 1, has nuh_layer_id nuhLid greater than that of the current picture, and belongs to an output layer (i.e., OutputLayerFlag[ GeneralLayerIdx[ nuhLid ] ] is equal to 1)."

- Remove pps_temporal_mvp_enabled_idc, and add a constraint like the following: When no reference picture with the same resolution as the one of the current picture is available, it is a requirement of bitstream conformance that ph_temporal_mvp_enabled_flag shall be equal to 0. [JVET-P0206][SW/T. Nishi]

# Review of contributions

The HLS BoG meeting minutes are included in the attached document, with change marks relative to JVET-P_Notes_d2.