| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0983-v1 |
| --- | --- |

| Title: | Test results on removing sps_sbt_max_size_64_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yin Zhao #360, Jiangshu Road, Hangzhou 310011, China | Tel: Email: | +86-571-28163645 yin.zhao@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

- Abstract

The sps_sbt_max_size_64_flag was designed to provide functionality similar to log2_transform_skip_max_size_minus2 for configuring encoder complexity. This contribution reports coding results of two tests of removing sps_sbt_max_size_64_flag with MaxSbtSize always being set equal to 64 in CTC. In test 1, an encoder only method is used to disallow SBT for CUs with one side greater than 32 for class C, D. and E, which tests the coding loss from encoder only method that achieves the functionality of sps_sbt_max_size_64_flag = 0. In test 2, the encoder only method is not used, which shows the increase of encoding time as well as the coding gain if always trying SBT for CUs with one side of 64 pixels.

Test 1 results in 0.01% and -0.02% PSNR-Y-BD-rates in RA configuration for class C and D, respectively, and 0.01%, 0.02% and 0.13% PSNR-Y-BD-rates in LDB configuration for class C, D and E, respectively. The maximum loss is 0.29% for KristenAndSara sequence in LDB.

Test 2 results in -0.04% and -0.08% PSNR-Y-BD-rates in RA configuration for class C and D, respectively, and -0.03%, -0.01%, -0.11% PSNR-Y-BD-rates in LDB configuration for class C, D and E, respectively. The encoding time increase is around 2% and 3% for RA and LDB.

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |  |
|  | Tool off SBT |  | Test 1 |  | Test 2 |  |
|  | Y | EncT | Y | EncT | Y | EncT |
| Class C | 0.40% | 95% | 0.01% | 100% | -0.04% | 102% |
| Class D | 0.41% | 95% | -0.02% | 100% | -0.08% | 102% |
| Class E |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |  |
|  | Tool off SBT |  | Test 1 |  | Test 2 |  |
|  | Y | EncT | Y | EncT | Y | EncT |
| Class C | 0.60% | 92% | 0.01% | 100% | -0.03% | 103% |
| Class D | 0.72% | 92% | 0.02% | 100% | -0.01% | 102% |
| Class E | 0.52% | 96% | 0.13% | 100% | -0.11% | 103% |

Test 1: the encoder only restriction of MaxSBTSize=32

In VVC6, when sps_sbt_max_size_64_flag is equal to 0, SBT is not allowed for CUs with width or height greater than 32, and cu_sbt_flag is not signaled for these CUs.

The encoder only method is not to try SBT in RDO if one side of a CU is greater than 32, and to signal a cu_sbt_flag as 0 for this CU. Compared to having sps_sbt_max_size_64_flag, this encoder only method wastes one flag for large size CU with inter prediction residual, and the waste is not trivial in some sequences being coded with higher portion of large CUs with inter residual.

Coding results of the encoder only method compared to VTM6.0 for class C, D, E are provided in Table 1.

Table 1 Coding performance of the encoder only method (Test 1) on class C, D and E

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| BasketballDrill | 0.02% | 0.11% | -0.07% | 100% | 100% |
| BQMall | 0.00% | -0.39% | -0.32% | 100% | 100% |
| PartyScene | 0.00% | 0.02% | 0.03% | 100% | 100% |
| RaceHorses | 0.01% | -0.06% | -0.18% | 100% | 101% |
| BasketballPass | -0.04% | -0.02% | 0.02% | 100% | 100% |
| BQSquare | -0.05% | -0.07% | 0.01% | 100% | 99% |
| BlowingBubbles | -0.01% | -0.09% | -0.05% | 100% | 100% |
| RaceHorses | 0.00% | -0.01% | 0.00% | 100% | 99% |
| FourPeople |  |  |  |  |  |
| Johnny |  |  |  |  |  |
| KristenAndSara |  |  |  |  |  |
| Class C | 0.01% | -0.08% | -0.14% | 100% | 100% |
| Class D | -0.02% | -0.05% | 0.00% | 100% | 99% |
| Class E |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| BasketballDrill | 0.00% | 0.32% | 0.53% | 100% | 100% |
| BQMall | 0.03% | 0.18% | 0.23% | 100% | 100% |
| PartyScene | 0.01% | 0.09% | 0.16% | 100% | 99% |
| RaceHorses | 0.00% | 0.00% | 0.00% | 100% | 100% |
| BasketballPass | 0.13% | 0.30% | -0.01% | 99% | 100% |
| BQSquare | -0.07% | 1.35% | 0.86% | 101% | 101% |
| BlowingBubbles | -0.05% | -0.58% | 0.02% | 100% | 100% |
| RaceHorses | 0.07% | -0.12% | 0.23% | 100% | 99% |
| FourPeople | -0.07% | -0.18% | -0.24% | 100% | 101% |
| Johnny | 0.17% | 0.34% | 0.64% | 100% | 101% |
| KristenAndSara | 0.29% | 0.34% | 0.00% | 100% | 99% |
| Class C | 0.01% | 0.15% | 0.23% | 100% | 100% |
| Class D | 0.02% | 0.24% | 0.28% | 100% | 100% |
| Class E | 0.13% | 0.17% | 0.14% | 100% | 100% |

Some coding loss is observed on class E in LDB, and the maximum loss is 0.29% in KristenAndSara. A smaller loss is also observed on Johnny (which is similar to KristenAndSara in content).

Test 2: without the encoder only method

Coding results of simply setting maxSbtSize = 64 compared to VTM6.0 for class C, D, E are provided in Table 2.

Table 2 Coding performance of setting maxSbtSize = 64 (Test 2) on class C, D and E.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| BasketballDrill | 0.01% | 0.12% | 0.14% | 101% | 100% |
| BQMall | -0.10% | -0.01% | -0.19% | 102% | 100% |
| PartyScene | -0.03% | -0.02% | 0.04% | 102% | 100% |
| RaceHorses | -0.04% | -0.01% | 0.28% | 102% | 101% |
| BasketballPass | -0.06% | 0.01% | -0.13% | 101% | 100% |
| BQSquare | -0.10% | -0.18% | 0.09% | 103% | 99% |
| BlowingBubbles | -0.11% | -0.21% | -0.22% | 102% | 101% |
| RaceHorses | -0.04% | 0.04% | -0.10% | 101% | 100% |
| FourPeople |  |  |  |  |  |
| Johnny |  |  |  |  |  |
| KristenAndSara |  |  |  |  |  |
| Class C | -0.04% | 0.02% | 0.07% | 102% | 100% |
| Class D | -0.08% | -0.08% | -0.09% | 102% | 100% |
| Class E |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| BasketballDrill | -0.14% | -0.04% | 0.19% | 103% | 99% |
| BQMall | -0.05% | 0.05% | -0.01% | 103% | 100% |
| PartyScene | 0.00% | 0.22% | -0.14% | 102% | 97% |
| RaceHorses | 0.07% | -0.03% | -0.49% | 103% | 100% |
| BasketballPass | 0.05% | 0.41% | 0.05% | 102% | 100% |
| BQSquare | -0.09% | 0.04% | 1.40% | 103% | 99% |
| BlowingBubbles | 0.00% | -0.22% | -0.25% | 103% | 99% |
| RaceHorses | 0.01% | -0.06% | 0.30% | 101% | 98% |
| FourPeople | -0.19% | 0.05% | 0.55% | 103% | 100% |
| Johnny | -0.23% | 0.43% | 1.71% | 102% | 100% |
| KristenAndSara | 0.10% | -0.15% | -1.12% | 103% | 98% |
| Class C | -0.03% | 0.05% | -0.11% | 103% | 99% |
| Class D | -0.01% | 0.04% | 0.38% | 102% | 99% |
| Class E | -0.11% | 0.11% | 0.38% | 103% | 99% |

Compared to the coding gain of MaxSbtSize=32 (from tool-off test in AHG13 on SBT), allowing SBT for larger blocks does not achieve a good trade-off for these small resolution sequences and increases around 2% and 3% encoding time.

Table 3 Coding performance of SBT off on class C, D and E.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| BasketballDrill | 0.23% | 0.12% | 0.00% | 96% | 99% |
| BQMall | 0.57% | -0.14% | -0.14% | 96% | 100% |
| PartyScene | 0.37% | 0.15% | 0.17% | 95% | 100% |
| RaceHorses | 0.44% | 0.06% | 0.33% | 95% | 100% |
| BasketballPass | 0.49% | -0.45% | 0.40% | 96% | 99% |
| BQSquare | 0.35% | 0.19% | 0.32% | 96% | 100% |
| BlowingBubbles | 0.37% | -0.13% | 0.03% | 95% | 99% |
| RaceHorses | 0.45% | -0.09% | 0.03% | 93% | 99% |
| FourPeople |  |  |  |  |  |
| Johnny |  |  |  |  |  |
| KristenAndSara |  |  |  |  |  |
| Class C | 0.40% | 0.05% | 0.09% | 95% | 100% |
| Class D | 0.41% | -0.12% | 0.20% | 95% | 99% |
| Class E |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| BasketballDrill | 0.23% | -0.24% | 0.56% | 94% | 98% |
| BQMall | 0.93% | 0.49% | 0.14% | 92% | 98% |
| PartyScene | 0.67% | 0.91% | 0.83% | 91% | 99% |
| RaceHorses | 0.58% | -0.23% | -0.40% | 92% | 98% |
| BasketballPass | 0.56% | 0.19% | 0.70% | 93% | 99% |
| BQSquare | 0.97% | 0.73% | 1.35% | 93% | 98% |
| BlowingBubbles | 0.75% | 0.26% | 0.15% | 90% | 97% |
| RaceHorses | 0.60% | -0.18% | -0.14% | 91% | 99% |
| FourPeople | 0.57% | -0.77% | -0.73% | 95% | 100% |
| Johnny | 0.41% | 0.43% | 1.34% | 97% | 100% |
| KristenAndSara | 0.59% | 0.26% | -0.13% | 97% | 100% |
| Class C | 0.60% | 0.23% | 0.28% | 92% | 98% |
| Class D | 0.72% | 0.25% | 0.52% | 92% | 98% |
| Class E | 0.52% | -0.03% | 0.16% | 96% | 100% |

Conclusions

The coding gain of SBT on larger-sized CUs (e.g., 64x64, 64x32, 64x16 etc) is marginal for small resolution sequences. The proponent of SBT still considers the sps_sbt_max_size_64_flag useful for configuring encoder complexity. The proponent would agree to clean up this sps flag from VVC spec if other experts consider that the sps flag imposes a burden on conformance testing.

Patent rights declaration(s)

Huawei Technologies Co., Ltd does not have current or pending patent rights relating to the technology described in this contribution.