| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P1000 |
| --- | --- |

| Title: | Proposed text and encoder for transform shift removal in transform skip mode (harmonization of JVET-P0272, JVET-P0515 and JVET-P0538) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Li Zhang, Weijia Zhu, Kai Zhang, Hongbin Liu, Zhiping Deng (Bytedance Inc.) Takeshi Tsukuba, Masaru Ikeda, Yoichi Yagasaki, Teruhiko Suzuki (Sony Corporation) Hong-Jheng Jhu, Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Xianglin Wang (Kwai Inc.) | Email: | xujizheng@bytedance.com |
| Source: | Bytedance Inc. Sony Corporation Kwai Inc. |  |  |

_____________________________

# Abstract

It is proposed to remove transform shift in transform skip mode in JVET-P0272, JVET-P0515 and JVET-P0538. This contribution presents suggested text changes and an encoder for consideration.

# Introduction

During the discussing of JVET-P0538, it was asserted that JVET-P0272, JVVET-P0515 and JVET-P0538 all propose to remove the transform shift in transform skip mode. The decoders of these three proposals are essentially the same. It was requested to provide suggested text changes and an encoder.

# Suggested encoder

During the discussion, it was mentioned that to remove transform shift in transform skip mode during forward transform stage at the encoder could make the software easier to understand. In JVET-P0272, the encoder is also modify to remove the corresponding transforms shift and show BD-rate gain even in the CTC. The results have been verified. The CTC performance is summarized in the following table. It is suggested to use the software from JVET-P0272. The sofware patch is attached with this proposal.

Table I. Performance of the suggested encoder.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 96% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 96% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 99% |
| 4:2:0 TGM | 0.00% | 0.00% | 0.00% | 99% | 99% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 116% |
| Class F | 0.00% | 0.00% | 0.02% | 100% | 100% |
| 4:2:0 TGM | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-6.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 104% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 96% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class F | 0.00% | -0.03% | -0.05% | 100% | 97% |
| 4:2:0 TGM | -0.04% | -0.07% | -0.07% | 100% | 100% |

# Suggested text changes

The text changes, marked in yellow, are based on JVET-P0113-v1_DraftText.

8.7.2 Scaling and transformation process

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable cIdx specifying the colour component of the current block,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height.

Output of this process is the (nTbW)x(nTbH) array of residual samples resSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variables bitDepth, bdShift and tsShift are derived as follows:

bitDepth = ( cIdx  = =  0 ) ? BitDepthY : BitDepthC		(8-942)

bdShift = Max( 20 − bitDepth, 0 )		(8-943)

tsShift = 5 + ( ( Log2( nTbW ) + Log2( nTbH ) ) / 2 )		(8-944)

The variable codedCIdx is derived as follows:

- If cIdx is equal to 0 or TuCResMode[ xTbY ][ yTbY ] is equal to 0, codedCIdx is set equal to cIdx.

- Otherwise, if TuCResMode[ xTbY ][ yTbY ] is equal to 1 or 2, codedCIdx is set equal to 1.

- Otherwise, codedCIdx is set equal to 2.

The variable cSign is set equal to ( 1 − 2 * slice_joint_cbcr_sign_flag ).

The (nTbW)x(nTbH) array of residual samples resSamples is derived as follows:

  - The scaling process for transform coefficients as specified in clause 8.7.3 is invoked with the transform block location ( xTbY, yTbY ), the transform block width nTbW and the transform block height nTbH, the colour component variable cIdx being set equal to codedCIdx and the bit depth of the current colour component bitDepth as inputs, and the output is an (nTbW)x(nTbH) array of scaled transform coefficients d.

  - The (nTbW)x(nTbH) array of residual samples r is derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ] is equal to 1 and cIdx is equal to 0, the residual sample array values r[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

r[ x ][ y ] = d[ x ][ y ]  <<  tsShift	(8-945)

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ] is equal to 0 or and cIdx is not equal to 0), tThe transformation process for scaled transform coefficients as specified in clause 8.7.4.1 is invoked with the transform block location ( xTbY, yTbY ), the transform block width nTbW and the transform block height nTbH, the colour component variable cIdx and the (nTbW)x(nTbH) array of scaled transform coefficients d as inputs, and the output is an (nTbW)x(nTbH) array of residual samples r.

  - The intermediate residual samples res [ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

    - If transform_skip_flag[ xTbY ][ yTbY ] is equal to 1 and cIdx is equal to 0, the following applies:

res [ x ][ y ] = d[ x ][ y ]

    - Otherwise (transform_skip_flag[ xTbY ][ yTbY ] is equal to 0 or and cIdx is not equal to 0), the following applies:

res [ x ][ y ] = ( r[ x ][ y ] + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift	(8-946)

  - The residual samples resSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- If cIdx is equal to codedCIdx, the following applies:

resSamples[ x ][ y ] = res[ x ][ y ]	(8-947)

- Otherwise, if TuCResMode[ xTbY ][ yTbY ] is equal to 2, the following applies:

resSamples[ x ][ y ] = cSign * res[ x ][ y ]	(8-948)

- Otherwise, the following applies:

resSamples[ x ][ y ] = ( cSign * res[ x ][ y ] ) >> 1	(8-949)

8.7.3 Scaling process for transform coefficients

…

The variables  bdShift, rectNorm and bdOffset are derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ] is equal to 1 and cIdx is equal to 0, the following applies:

bdShift = 10

- Otherwise, the following applies:

bdShift = bitDepth + ( ( rectNonTsFlag  ?  1  :  0 ) +                                    (8-956)
	( Log2( nTbW ) + Log2( nTbH ) ) / 2 ) − 5 + slice_dep_quant_enabled_flag

bdOffset = ( 1 << bdShift ) >> 1		(8-957)

…

# Recommendations

It is recommended to adopt the proposed text into the next version of VVC draft and the proposed code changes into the next version of VTM.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).