| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P1002 |
| --- | --- |

| Title: | Non-CE5: Chroma QP derivation fix for deblocking filter (Combination of JVET-P0105 and JVET-P0539) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Li Zhang, Weijia Zhu, Kai Zhang, Hongbin Liu | Email: | xujizheng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In VTM-6.0, chroma QP derivation does not take into account that a given chroma block (Cb or Cr)  may use joint Cb-Cr (JCCR) coding tool and therefore the appropriate mapping conversion from Luma QP to Chroma QP is not performed. The current contribution proposes to derive the Chroma QP as follows: In the first step the QP mapping from Luma to Chroma is performed for each neighboring block of a given edge separately wherein, if the given block uses joint cb-cr tool, then the appropriate mapping conversion is invoked. Once the mapped chroma QP for each block is derived, then the average of the Chroma QP for both the blocks is computed and this value is further used in the Chroma deblocking process.

For VTM-6.0, under CTC conditions, there is no BD-Rate change observed as the Chroma QP mapping tables are currently the same for all three chroma components (Cb, Cr and Joint Cb-Cr).

# Introduction

Unlike HEVC Chroma deblocking which is performed when one of the blocks sharing the given edge is Intra, the current VVC deblocking [1] also has a longer tap Chroma deblocking which is decided to be chosen when some of the conditions (which internally rely on the QP of the two blocks sharing the edge) are evaluated to be true. These decisions are separately performed for the both the Cb and Cr components.

The Chroma QP derivation in VTM-6.0 [1] is performed by taking the average luma QP (QPY) of both the chroma blocks (P and Q) adjacent to the edge and then the averaged luma value is mapped to Chroma QP value by using the ChromaQpTable.

The ChromaQpTable is a two dimensional table where the first dimension is the componentIdx (Cb, Cr or CbCr) and the second dimension is the QP index.  The appropriate syntax for the Chroma QP derivation from the spec text [1] is as shown below:

The variables QpQ and QpP are set equal to the QpY values of the coding units which include the coding blocks containing the sample q0,0 and p0,0, respectively.

The variable QpC is derived as follows:

qPi = Clip3( 0, 63, ( ( QpQ + QpP + 1 )  >>  1 ) + cQpPicOffset )	(8-1132)

QpC = ChromaQpTable[ cIdx − 1 ][ qPi ]

However the componentIdx (cIdx) does not take into account the fact that the given chroma block may use joint Cb-Cr (JCCR) coding tool with JCCR mode equal to 2and therefore the cIdx value should be 3 for such blocks instead of 1 (Cb) or 2 (Cr). Therefore this may result in in-correct Chroma QP derivation for chroma deblocking.

Moreover, before adoption of the flexible chroma table design in VTM-6.0, chroma QP mapping on the average of luma QP approximates the average of chroma QP mapping effectively. However, because the chroma QP tables can be flexibly signalled, there might be large discrepancy between chroma QP mapping on the average of luma QP approximates the average of chroma QP mapping.

In some cases, the difference between the average of mapping and mapping of the average can be much large. For example, in the case shown in Figure 1, where a chroma QP table consists of two flat lines, the difference between the two methods can be as large as 37. In such a case, the current design can lead to a unstable situation: when QP on one side is decreased from 36 to 35, the QP for deblocking decision will be changed from 63 to -12, i.e. from a maximum deblocking to totally no deblocking.

Although in practice, maybe no one uses such an extreme chroma QP table by purpose, the above example demonstrates that the current design is not robust enough.

Figure 1. An example chroma QP table applied to chroma deblocking.

# Proposed Solutions

Solution 1:

The proposed solution suggests to derive the chroma QP as follows: As a first step, the Chroma QP mapping is performed for each neighboring block (P or Q) individually. This step also takes into account if the given block has a componentIdx Cb or Cr or joint Cb-Cr. As shown in Fig. 2, it may be the case that block P may use Joint Cb-Cr and block Q is just a normal Cb or Cr block (or) vice versa (or) both blocks P and Q could both use joint Cb-Cr tool.  Therefore the correct componentIdx is used when the mapping from Luma Qp to Chroma Qp is performed.

P (uses JCCR)

Q (Cb or Cr)

P (Cb or Cr)

Q (uses JCCR)

P (uses JCCR)

Q (uses JCCR)

(a)

(b)

(c)

Figure 2. Block P and/or Block Q might be coded with Joint Cb-Cr mode

In a second step, the mapped Chroma QP values of both blocks P and Q are averaged and the resulting value is further used as a final Chroma QP value for the Chroma deblocking process.

Solution 2:

The other solution is to align the chroma QP at the deblocking stage to that at the dequantization stage so that deblocking can be applied with the dequantization QP consistently.

The specification text changes are given in the next section.

# Specification text changes

Solution 1:

Specification text changes are provides on top of [1]. The text additions are highlighted using Green color and strikethroughs are highlighted using yellow color.

8.8.3.6.3 Decision process for chroma block edges

This process is only invoked when ChromaArrayType is not equal to 0.

Inputs to this process are:

- a chroma picture sample array recPicture,

- a chroma location ( xCb, yCb ) specifying the top-left sample of the current chroma coding block relative to the top-left chroma sample of the current picture,

- a chroma location ( xBl, yBl ) specifying the top-left sample of the current chroma block relative to the top-left sample of the current chroma coding block,

- a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered,

- a variable cIdx specifying the colour component index,

- a variable cQpPicOffset specifying the picture-level chroma quantization parameter offset,

- a variable bS specifying the boundary filtering strength,

- a variable maxFilterLengthCbCr.

Outputs of this process are

- the modified variable maxFilterLengthCbCr,

- the variable tC.

The variable maxK is derived as follows:

- If edgeType is equal to EDGE_VER, the following applies:

maxK = ( SubHeightC = = 1 )  ?  3  :  1	(8-1124)

- Otherwise (edgeType is equal to EDGE_HOR), the following applies:

maxK = ( SubWidthC = = 1 )  ?  3  :  1	(8-1125)

The values pi and qi with i = 0.. maxFilterLengthCbCr and k = 0..maxK are derived as follows:

- If edgeType is equal to EDGE_VER, the following applies::

qi,k = recPicture[ xCb + xBl + i ][ yCb + yBl + k ]	(8-1126)

pi,k = recPicture[ xCb + xBl − i − 1 ][ yCb + yBl + k ]	(8-1127)

subSampleC = SubHeightC	(8-1128)

variable QpQ is derived by invoking the chorma QP derivation process as specified in clause 8.8.3.6.10 with chroma location (xCb + xBl + i , yCb + yBl + k ]) and cIdx as inputs

variable QpP is derived by invoking the chorma QP derivation process as specified in clause 8.8.3.6.10 with chroma location (xCb + xBl − i − 1 , yCb + yBl + k ]) and cIdx as inputs

- Otherwise (edgeType is equal to EDGE_HOR), the following applies:

qi,k = recPicture[ xCb + xBl + k ][ yCb + yBl + i ]	(8-1129)

pi,k = recPicture[ xCb + xBl + k ][ yCb + yBl − i − 1 ]	(8-1130)

subSampleC = SubWidthC	(8-1131)

variable QpQ is derived by invoking the chorma QP derivation process as specified in clause 8.8.3.6.10 with chroma location (xCb + xBl + k , yCb + yBl + i ]) and cIdx as inputs

variable QpP is derived by invoking the chorma QP derivation process as specified in clause 8.8.3.6.10 with chroma location (xCb + xBl + k , yCb + yBl − i − 1 ]) and cIdx as inputs

The variables QpQ and QpP are set equal to the QpY values of the coding units which include the coding blocks containing the sample q0,0 and p0,0, respectively.

The variable QpC is derived as follows:

qPi = ( QpQ + QpP + 1 )  >>  1 )

qPi = Clip3( 0, 63, ( ( QpQ + QpP + 1 )  >>  1 ) + cQpPicOffset )	(8-1132)

QpC = ChromaQpTable[ cIdx − 1 ][ qPi ]	(8-1133)

NOTE – The variable cQpPicOffset provides an adjustment for the value of pps_cb_qp_offset or pps_cr_qp_offset, according to whether the filtered chroma component is the Cb or Cr component. However, to avoid the need to vary the amount of the adjustment within the picture, the filtering process does not include an adjustment for the value of slice_cb_qp_offset or slice_cr_qp_offset nor (when cu_chroma_qp_offset_enabled_flag is equal to 1) for the value of CuQpOffsetCb, CuQpOffsetCr, or CuQpOffsetCbCr.

8.8.3.6.10 Quantization parameter derivation process for chroma coding blocks

This process is only invoked when ChromaArrayType is not equal to 0.

Inputs to this process are:

- A chroma coding block containing a given chroma sample location (xCb, yCb )

- a variable cIdx specifying the colour component index of the given chroma coding block,

Output of this process is a quanization parmater qP of the coding block contiaing the sample (xCb, yCb )

- if TuCResMode[ xCb ][ yCb ] is equal to 2, the following applies:

qP = QpCbCr		(8-952)

cQpPicOffset is set equal to pps_joint_cbcr_qp_offset

- Otherwise, if cIdx is equal to 1, the following applies:

qP = QpCb		(8-953)

- Otherwise (cIdx is equal to 2), the following applies:

qP = QpCr		(8-954)

qP  = Clip3( 0, 63,  qP + cQpPicOffset )

NOTE – The variable cQpPicOffset provides an adjustment for the value of pps_cb_qp_offset or pps_cr_qp_offset, according to whether the filtered chroma component is the Cb or Cr component. However, to avoid the need to vary the amount of the adjustment within the picture, the filtering process does not include an adjustment for the value of slice_cb_qp_offset or slice_cr_qp_offset nor (when cu_chroma_qp_offset_enabled_flag is equal to 1) for the value of CuQpOffsetCb, CuQpOffsetCr, or CuQpOffsetCbCr.

NOTE – The variables QpCbCr , QpCb , QpCr  are derived as in section 8.7.1(Derivation process for quantization parameters)

Solution 2:

8.8.3.6.3 Decision process for chroma block edges

…

The variables QpQ and QpP are set equal to the QpY values of the coding units which include the coding blocks containing the sample q0,0 and p0,0, respectively.

The variable QpC is derived as follows:

qPi = Clip3( 0, 63, ( ( QpQ + QpP + 1 )  >>  1 ) + cQpPicOffset )	(8-1132)

QpC = ChromaQpTable[ cIdx − 1 ][ qPi ]			  		           (8-1133)

The variables QpQ and QpP are set equal to Qp′CbCr − QpBdOffsetC when TuCResMode[ xCb ][ yCb ] is equal to 2, Qp′Cb − QpBdOffsetC when cIdx is equal to 1; Qp′Cr − QpBdOffsetC when values of the coding units which include the coding blocks containing the sample q0,0 and p0,0, respectively.

QpC = ( QpQ + QpP + 1 )  >>  1

…

# Simulation Results

The modified Chroma QP derivation is implemented on top of VTM-6.0 [1] and the simulation is performed under common test conditions as described in JVET-N1010 [2]. There is no BD-Rate change observed, as in current CTC test conditions, the Chroma QP mapping table information is the same for all three different componentIdx (Cb, Cr and Joint Cb-Cr). Moreover there is no additional computational complexity observed.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM 6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 98% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM 6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 96% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 96% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 98% |

# Conclusion

The current contribution proposes a modified method of deriving the Chroma QP for Chroma deblocking. The proposed method checks if the given Chroma block uses joint Cb-Cr coding tool and then uses the respective mapping table to map the Luma QP to Chroma QP.  Under CTC conditions, there is no BD-Rate impact observed.

It is suggested to adopt the modified Chroma QP derivation process to correctly derive the Chroma QP during the deblocking process considering the fact that a given chroma block may use joint cb-cr tool and the mapping information may be different to that of Cb or Cr component.

Specification text changes and the software patch on top of VTM-6.0 are attached along with the contribution.

# References

- Versatile Video Coding (Draft 5), Document JVET-O2001-vEI, B. Bross, J. Chen, S. Liu

- “JVET common test conditions and software reference configurations”, JVET-N1010

- Non-CE5: Modified Chroma QP derivation for deblocking filter, JVET-P0105

- Non-CE5: A Chroma deblocking clean-up, JVET-P0539

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).