| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P1006-v2 |
| --- | --- |

| Title: | AHG17: Text for picture header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wade Wan Tim Hellman Brian Heng Hendry Ye-Kui Wang Jianle Chen Miska M. Hannuksela Sachin Deshpande Jonatan Samuelsson Jean-Marc Thiesse Jie Chen Ru-Ling Liao Jiancong Luo Yan Ye | Email: | firstname.lastname@broadcom.com fhendry@futurwei.com ye-kui.wang@futurewei.com jianle.chen@futurewei.com firstname.lastname@nokia.com sdeshpande@sharplabs.com jean-marc.thiesse@vitec.com jiechen.cj@alibaba-inc.com |
| Source: | Broadcom Inc, Futurewei Technologies Inc, Nokia, Sharp Labs of America, Inc, VITEC, Alibaba Group |  |  |

_____________________________

# Abstract

This contribution proposes the spec text for picture header.

It is asserted that the implemented text for signalling mechanism for subpicture Id mapping has difference compared to the guideline noted in the in the meeting notes. This should be discussed and clarified.

In v2 of this contribution the following are updated:

- Signalling of subpicture Id mapping. With the update, the design is aligned with the agreed intent as noted in the meeting notes.

- Signalling of partition constraints related syntax elements in picture header which was agreed in the coding tool discussion.

- Update the signalling of LMCS syntax elements in picture header. With the update, LMCS signalling is moved from slice header to picture header (i.e., signalled in picture header without override mechanism in slice header)

# Introduction

Picture header was agreed to be adopted into the VVC spec. The following notes describe details of what the property of picture header and what should be included in the picture header.

----------- snippet from meeting notes JVET-P_Notes_d6.docx -----------

- Do we want to have Picture Header?

Proposing yes. JVET-P0095, JVET-P0120, JVET-P0239

Why? JVET-P0092 & JVET-P0120: AUD is AU specific. An AU may contain multiple pictures.

JVET-P0120: Order of AUD is supposed to the first NAL unit in an AU. If AUD preceeds SPS & PPS, some picture-level syntax elements in SH cannot be moved to AUD. Having PH is asserted would solve this problem.

Decision: Yes, after considering the quantity of syntax elements that would go there, we should have a picture header.

- If yes, what to signal in PH?

  - Move some signalling from PPS to PH

JVET-P0095: Subpicture Id mapping, if it is not signalled in the SPS and not signaled in the PPS, to avoid needing to send a PPS for this

Decision: Allow the mapping in the PH when it is not in the SPS. If present, it overrides what is in the PPS. If not in the PPS and not in the SPS, then is shall be in the PH. As detailed below:

SPS syntax:

fancy_weird_stuff_allowed_flag

if( fancy_weird_stuff_allowed_flag ) {

sending_details_in_sps_flag /* infer 0 if not present */

if ( sending_details_in_sps_flag )

Details

}

PPS syntax:

sending_details_in_pps_flag

if (sending_details_in_pps_flag)

Details

constraint: If fancy_weird_stuff_allowed_flag is 0 or sending_details_in_sps_flag is 1, sending_details_in_pps_flag shall be equal to 0.

PH syntax:

if ( fancy_weird_stuff_allowed_flag && !sending_details_in_sps_flag ) {

sending_details_in_ph_flag

if(sending_details_in_ph_flag)

Details

}

  - Signal syntax element both in PPS and PH. Present in PH if not present in PPS

JVET-P0095: Tile/Brick/Rect Slice signalling

Deferred to after considering GDR needs.

  - Move some signalling from SH to PH

Preliminary decision:

    - JVET-P0095, P0120: no_output_of_prior_pics_flag

    - JVET-P0120, P0239: slice_pic_parameter_set_id, non_reference_picture_flag, colour_plane_id, slice_pic_order_cnt_lsb, recovery_poc_cnt, pic_output_flag, slice_temporal_mvp_enabled_flag.

Comment: Think more about slice_pic_order_cnt_lsb as a loss detection mechanism

    - JVET-P0239: slice_lmcs_aps_id and slice_scaling_list_aps_id

    - JVET-P0239 move some syntax elements that are currently not constrained to be the same for all slices of a picture, but asserted having no / minimal benefit and coding loss to transmitting them in every slice header as their anticipated usage would change at the picture level. 18 such syntax elements are identified in the contribution. (In the CTC, about half of these don't even change across different pictures.)

It was commented that this could affect the ability to merge slices from different encoders. However, others commented that encoding for such applications would already be customized to this use case (and possibly only coming from one common encoder or encoders from only one company).

      - six_minus_max_num_merge_cand

      - five_minus_max_num_subblock_merge_cand

      - slice_fpel_mmvd_enabled_flag

      - slice_disable_bdof_dmvr_flag

      - max_num_merge_cand_minus_max_num_triangle_cand

      - slice_six_minus_max_num_ibc_merge_cand

      - partition_constraints_override_flag

      - slice_log2_diff_min_qt_min_cb_luma

      - slice_max_mtt_hierarchy_depth_luma

      - slice_log2_diff_max_bt_min_qt_luma

      - slice_log2_diff_max_tt_min_qt_luma

      - slice_log2_diff_min_qt_min_cb_chroma (only needed for I slices)

      - slice_max_mtt_hierarchy_depth_chroma (only needed for I slices)

      - slice_log2_diff_max_bt_min_qt_chroma (only needed for I slices)

      - slice_log2_diff_max_tt_min_qt_chroma (only needed for I slices)

      - mvd_l1_zero_flag (only needed for B slices)

      - dep_quant_enabled_flag

      - sign_data_hiding_enabled_flag

  - Signal some syntax element that are not constrained to be the same for all slices of a picture both in PH and SH. Present in SH if not present in PH

    - JVET-P0120: RPL, joint cb/cr sign flag, SAO, ALF, LMCS, Scaling list

Preliminary decision:

joint cb/cr sign flag should be in the PH and never be in the SH

RPL, SAO, ALF, LMCS, deblocking (4 syntax elements in July output doc) should possible to put in SH (as override if also in higher layer), also possible in PH and in SPS (for each of these five categories, if it's in the PH, then it's not in the SH)

Scaling list control should never be in the SH

- If yes, what is property of PH?

  - JVET-P0095, JVET-P0120, JVET-P0239: Mandating one PH one picture

Decision:

PH comes before the VCL NAL units for the picture; the SPS and PPS must precede it. APSs do not need to precede it but may. SEI messages for the picture follow it.

Do not allow PH to be "out of band" (and it should be in Type I category of HRD).

Allow repetitions; not more PHs than slices; not after the last VCL NAL unit.

  - JVET-P0120: Specifies layer Id, temporal Id, order of PH in access unit, etc.

Decision: Layer Id and temporal Id are those of the picture.

  - Is it VCL or non-VCL?

Decision: Non-VCL

    - JVET-P0095, JVET-P0120: PH is a non-VCL NAL unit.

    - JVET-P0239: PH is a VCL NAL unit.

Decision: Put it in the SPS; if present there, it cannot change. If not present there, it becomes a gated presence in the PPS or PH. J.-M. Thiesse volunteered the text work (and software for handling the syntax).

- CE 2 GDR – section 5.2

  - Boundary signalling in SPS or picture header

---------------------------------------------------------------------------------------

# Patent rights declaration(s)

Broadcom, Futurewei Technologies, Nokia, Sharp Labs of America, Inc, VITEC, and Alibaba Group Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).