| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P1012-v1 |
| --- | --- |

| Title: | AHG17: Improvement for removal of bricks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Woong Lim Gun Bang | Email: | woong.lim@etri.re.kr, gbang@etri.re.kr |
| Source: | ETRI |  |  |

_____________________________

- Abstract

In this meeting, the contribution JVET-P0240 was proposed. The proposal is to remove brick concept and represent rectangular slice using tile with allowing a tile to be partitioned into multiple rectangular slices. However, there is no need to signal tile_idx_delta syntax element for the last rectangular slice in case there are two or more rectangular slices in one tile. In this contribution, small fix for the last rectangular slice is proposed.

Introduction

In the scanning process, the rectangular slices are constructed as follows.

tileIdx = 0

for( i=0; i <= num_slices_in_pic_minus1; i++ ) {

NumCtuInSlice[ i ] = 0

}

for( i=0; i <= num_slices_in_pic_minus1; i++ ) {

tileX = tileIdx % NumTileColumns
		tileY = tileIdx / NumTileColumns

if( i = = num_slices_in_pic_minus1 ) {

slice_width_in_tiles_minus1[ i ] = NumTileColumns – 1 – tileX

slice_height_in_tiles_minus1[ i ] = NumTileRows – 1 – tileY

num_slices_in_tile_minus1[ i ] = 0

}

if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 ) {

ctbY = tileRowBd[ tileY ]

numSlicesInTileMinus1 = num_slices_in_tile_minus1[ i ]
			for( j = 0; j < numSlicesInTileMinus1; j++ ) {

AddCtbsToSlice( i, tileColBd[ tileX ], tileColBd[ tileX + 1 ],

ctbY, ctbY + slice_height_in_ctu_minus1[ i ] + 1 )

ctbY += slice_height_in_ctu_minus1[ i ] + 1

i++
			}

AddCtbsToSlice( i, tileColBd[ tileX ], tileColBd[ tileX + 1 ],

ctbY, tileRowBd[ tileY + 1] )

}

else {

for( j = 0; j <= slice_height_in_tiles_minus1[ i ]; j++ ) {
				for( k = 0; k <= slice_width_in_tiles_minus1[ i ]; k++ ) {

AddCtbsToSlice( i, tileColBd[ tileX + k ], tileColBd[ tileX + k + 1 ],

tileRowBd[ tileY + j ], tileRowBd[ tileY + j + 1 ] )

}

}

}

tileIdx += tile_idx_delta[ i ]

}

…

The highlighted (as green) parts are related to signal tile_idx_delta for each rectangulare slice. However, if there are two or more rectangular slices in one tile, tile_idx_delta for each rectangular slice (except for the last rectangular slice) can be derived as 0. And it doesn’t need to signal tile_idx_delta for the last one.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] == 0 && slice_height_in_tiles_minus1[ i ] == 0 ) { |  |
| num_slices_in_tile_minus1[ i ] | ue(v) |
| numSlicesInTileMinus1 = num_slices_in_tile_minus1[ i ] |  |
| for( j = 0; j < numSlicesInTileMinus1; j++ ) |  |
| slice_height_in_ctu_minus1[ i++ ] | ue(v) |
| } |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |
| … |  |
| } |  |

Proposal

The simple fix in syntax structure is shown below.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] == 0 && slice_height_in_tiles_minus1[ i ] == 0 ) { |  |
| num_slices_in_tile_minus1[ i ] | ue(v) |
| numSlicesInTileMinus1 = num_slices_in_tile_minus1[ i ] |  |
| for( j = 0; j < numSlicesInTileMinus1; j++ ) |  |
| slice_height_in_ctu_minus1[ i++ ] | ue(v) |
| } |  |
| if(i < num_slices_in_pic_minus1) |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |
| … |  |
| } |  |

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).