| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P1033-v2 |
| --- | --- |

| Title: | BoG report on CE5 loop filtering related contributions |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrey Norkin C.-Y. Chen | Email: | anorkin@netflix.com chingyeh.chen@mediatek.com |
| Source: | BoG coordinators |  |  |

_____________________________

# Summary

The BoG reviewed proposals related to the loop filter topics (CE5-related). This report includes the recommendations of the BoG for adopting or further studying of proposals.

The BoG met during the following times:

- Tuesday Oct. 8: 9:00-13:00, 14:30-21:00

- Wednesday Oct. 9: 11:00-13:30, 15:00-20:10

- Thursday Oct. 10: 9:00-12:30, 18:20-19:50 (Revisit of P0552 and related)

# Recommendations

Deblocking filter

It is recommended to adopt proposal P0043 (the specification text). The software attached to the proposal needs to be fixed.

Study proposal 1 of P0161 (and similar proposals) in the CE. This would be either or two test cases (depending if proponents of P0586 decided to join the other test or have a separate test).

Test in the CE of P0441 (and improvements are expected).

Adopt method 1 of P0571

Adopt solution 2 of P1001

ALF

Recommendation: adopt method 1 of P0158, applying ALF VB to all CTU in a picture, including the bottom CTU row.

Adopt JVET-P0162

Adopt method 2 in 0551, which is also method 4 in 0157 and first aspect of 0053. The software is from P0551.

Recommendation:  It was agreed to use the check for the unavailable samples as in 0552 (and same as in 0492).

It was agreed that the repetitive padding is used for all the top samples, when there are unavailable samples. This is similar to what was proposed in 0156 and 0552 except that the mirrored padding is replaced by the repetitive padding according to the discussion earlier. This means a top horizontal line in the CTU is used to pad top samples. This applies to the top-left unavailable case, the bottom-right case would use symmetrical operations.

Recommendation: adopt method 2 of P0554 (specification changes).

Recommendation: BoG recommends fixing the specification to reflect the desired behaviour (using the correct samples for padding).

Recommendation: it was agreed to adopt the v5 of the document P0505.

CCALF

Recommendation: test P0106 in the CE. Since the shape is highly asymmetric, a test on other chroma locations is required.

Recommendation: study P0173 in the CE

Recommendation: Test C6-2 of JVET-P0251 in the CE.

Recommendation: test P0376 in the CE

Recommendation: test method 3 of P0557 in the CE.

Recommendation: test method 5 of P0558 the CE.

Recommendation: Test the results of P0666 with limiting the coefficient range to [-8, 8]. The proponents are advised to discuss offline what coefficient bit depth (range) to test in the CE.

JVET-P1008

Recommendation of the first day: Adopt one of the proposed two options when more results are available. Further checking specification before the discussion in the track is encouraged.

On Thursday discussion, one participant recommended testing all ccALF proposals in the CE before the adoption.

# Contributions by category

## Deblocking Filter

| JVET number | Title | Tag |
| --- | --- | --- |
| JVET-P0043 | AHG16/Non-CE5: A cleanup for de-blocking in the affine and TPM mode |  |
| JVET-P0086 | AHG16/Non-CE5: Deblocking boundary strength fix for Affine and TPM |  |
| JVET-P0087 | Non-CE5: Deblocking boundary strength modification for triangle and affine mode |  |
| JVET-P0105 | Non-CE5: Modified Chroma QP derivation for deblocking filter |  |
| JVET-P0109 | AHG18/Non-CE5: Boundary strength fix for coding units using BCW |  |
| JVET-P0159 | CE5-related: DMVR deblocking by inheriting neighbouring boundary strength values |  |
| JVET-P0160 | CE5-related: Unification of deblocking processes for transform block and prediction block boundaries |  |
| JVET-P0161 | CE5-related: Deblocking considering prediction weights in BCW and TPM |  |
| JVET-P0195 | Non-CE5: Unified strong deblocking filter for luma and chroma |  |
| JVET-P0262 | AHG16/Non-CE5: Deblocking boundary design cleanup for affine and TPM mode |  |
| JVET-P0269 | AHG18 / non-CE5: Deblocking for TPM and BCW |  |
| JVET-P0306 | Non CE5: DMVR internal edge deblocking without using refined MVs |  |
| JVET-P0386 | Non-CE8/Non-CE5: Deblocking for IBC blocks |  |
| JVET-P0411 | Non-CE5: Fixes for long luma deblocking filter decision |  |
| JVET-P0423 | Non-CE5: Modified boundary strength derivation |  |
| JVET-P0441 | Non-CE5: On deblocking on affine internal prediction sub-block edges |  |
| JVET-P0534 | AhG16/Non-CE5: On deblocking at ALF virtual boundaries |  |
| JVET-P0539 | Non-CE5: A chroma deblocking clean-up |  |
| JVET-P0547 | Non-CE5: Deblocking filter at PROF sub-block boundary |  |
| JVET-P0570 | AHG10/Non-CE5: Performance of encoder-side deblocking optimization in VTM-6.0 |  |
| JVET-P0571 | CE5/CE8: Deblocking Filter for BDPCM coded block |  |
| JVET-P0586 | [AHG18][Non-CE5] Proposed cleanup of deblocking filter process |  |
| JVET-P0602 | [AHG18][non-CE5] Boundary strength derivation for CUs with TPM |  |
| JVET-P0611 | Non-CE5/AHG-11: Boundary strength harmonization for BDPCM, TS, Palette and IBC |  |
| JVET-P0613 | Non-CE5: On the average calculation for luma adaptive deblocking filter |  |
| JVET-P0688 | Non-CE5: Intra deblocking coefficients for weak filters |  |
| JVET-P0740 | CE5-related: Combination of JVET-P0086 and JVET-P0161 for TPM deblocking |  |
| JVET-P1001 | Non-CE5: Chroma QP derivation fix for deblocking filter (Combination of JVET-P0105 and JVET-P0539) |  |
| JVET-P1002 | Non-CE5: Chroma QP derivation fix for deblocking filter (Combination of JVET-P0105 and JVET-P0539) |  |

JVET-P0043 AHG16/Non-CE5: A cleanup for de-blocking in the affine and TPM mode [B. Heng, M. Zhou (Broadcom)]

This proposal is related to P0086, P0160.

It is asserted that the current de-blocking design is not consistent for affine mode and TPM mode in some of corner cases. In the affine mode, two 4x4 subblock edges can be treated differently even if they have the exactly same conditions (i.e. having no non-zero coefficients and the same amount of large enough MV differences that can trigger de-blocking along the edge); in the TPM mode, an inner TU edge with no non-zero coefficients can still be de-blocked if the edge is located along the diagonal TPM boundary and the triangle block MV difference is large enough to trigger de-blocking. In order to resolve this inconsistency, it is proposed to separate the de-blocking filtering decision for a TU edge and from that of an MV edge by making the following modifications: 1) using CBFs to make decision only for TU edges; 2) using MV differences to make decisions only for MV edges and 3) if an edge is both a TU edge and an MV edge, de-blocking is on if either 1) or 2) is true.

It was mentioned that there is an inconsistency. There are more proposals in the same category that need to be reviewed. It was also mentioned that the text is correct but the code may be wrong.

Recommendation: it is recommended to adopt Broadcom’s proposal (the specification text). The software needs to be fixed (by this meeting). It is also similar to Peking University proposal 1. P0262 is a similar proposal.

JVET-P0656 Crosscheck of JVET-P0043 (AHG16/Non-CE5: A cleanup for de-blocking in the affine and TPM mode) [J. Xu (Bytedance)]

JVET-P0086 AHG16/Non-CE5: Deblocking boundary strength fix for Affine and TPM [A. M. Kotra, S. Esenlik, H. Gao, B. Wang, E. Alshina (Huawei)]

This is a different solution to the issue than was proposed in P0043. It was commented that the proposed solution needs to access the TMP mode although the end result would be the same. It was also mentioned that Broadcom’s solution is more generic and future proof.

JVET-P0844 Crosscheck of JVET-P0086 (AHG16/Non-CE5: Deblocking boundary strength fix for Affine and TPM) [C.-M. Tsai (MediaTek)]

JVET-P0087 Non-CE5: Deblocking boundary strength modification for triangle and affine mode [K. Misra, F. Bossen, A. Segall (Sharp Labs of America)]

The proposed solution is to skip motion based deblocking on internal edges of a triangle mode CU and of the affine transform edges. It was also mentioned that the proposals needs to know the TPM mode of the CU. Similar to P0262.

JVET-P0845 Crosscheck of JVET-P0087 (Non-CE5: Deblocking boundary strength modification for triangle and affine mode) [C.-M. Tsai (MediaTek)]

JVET-P0262 AHG16/Non-CE5: Deblocking boundary design cleanup for affine and TPM mode [X. W. Meng (PKU)]

The proponent did not think it was necessary to present the proposal since solution 1 is identical to Broadcom’s proposal and Solution 2 is identical to Sharp’s proposal. The proponent prefers solution 1.

JVET-P0105 Non-CE5: Modified Chroma QP derivation for deblocking filter [A. M. Kotra, S. Esenlik, B. Wang, H. Gao, E. Alshina (Huawei)]

There is a joint proposal with P0539 that resulted in proposals P1001 and P1002.

P0539 does not need to be reviewed.

JVET-P0846 Crosscheck of JVET-P0105 (Non-CE5: Modified Chroma QP derivation for deblocking filter) [C.-M. Tsai (MediaTek)]

JVET-P0109 AHG18/Non-CE5: Boundary strength fix for coding units using BCW [A. M. Kotra, S. Esenlik, B. Wang, H. Gao, E. Alshina (Huawei)]

Same as P0160 Method 1. No presentation necessary.

JVET-P0848 Crosscheck of JVET-P0109 (AHG18/Non-CE5: Boundary strength fix for coding units using BCW) [C.-M. Tsai (MediaTek)]

JVET-P0159 CE5-related: DMVR deblocking by inheriting neighbouring boundary strength values [C.-M. Tsai, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

The proposal suggests to fix artifacts caused by DMVR by:

- Method 1. Fixed DVRM deblocking grind to 16x16. Copy the BS from horizontal and vertical boundaries.

- Method 2 is method 1 + additional constraint of MVs (see the proposal).

It was commented that the result of the proposal can be a quality degradation. It was commented that a proposal that used the actual motion vectors that .

Recommendation: No action.

JVET-P0948 Crosscheck of JVET-P0159 (CE5-related: DMVR deblocking by inheriting neighbouring boundary strength values) [A. M. Kotra (Huawei)]

JVET-P0160 CE5-related: Unification of deblocking processes for transform block and prediction block boundaries [C.-M. Tsai, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

This proposal also addresses the same issue as P0043

.

Method 1. Set deblocking on internal TU boundaries to 8x8 grid.

Method 2. To perform the bS calculation for motion edges on 8x8 grid (similar to Huawei’s and Sharps).

It was mentioned that proposal does not need access to TPM mode. The proponent claims that the proposal also fixed inconsistencies on the CU boundaries.

It was mentioned that the method 1 could cause some blocks on the SBT not been deblocking.

JVET-P0792 Crosscheck of JVET-P0160 (CE5-related: Unification of deblocking processes for transform block and prediction block boundaries) [Y. Han, N. Hu (Qualcomm)]

JVET-P0161 CE5-related: Deblocking considering prediction weights in BCW and TPM [C.-M. Tsai, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

The proposals suggests two changes:

Proposal 1: Set bS of the block edge for luma to 1 if the two adjacent blocks are coded with different BCW weight indices.

This proposals claims subjective benefits. It was mentioned that this should be confirmed by the subjective test. It was also mentioned that HEVC did not have this check and no problems have been observed.

P0109 from Huawei is exactly the same.

Method 2 from P0269 is also exactly the same.

P0586 aspect 2 is also similar (not exactly). The proponent thinks that there were problem with weighted prediction in HEVC deblocking.

Recommendation: Study proposal 1 (and similar proposals) in the CE. This would be either or two test cases (depending if proponents of P0586 decided to join the other test or have a separate test).

Proposal 2: Set bS of the current edge to 1 is any of the adjacent blocks are coded in the TPM mode.

In this case, it is suggested to change the behaviour of the deblocking along the TPM boundary. This seems more a proposals than a bugfix. Some improvements (on a still image) are claimed.

P0269 is similar but different (the deblocking is only applied to the corner block and not along the whole boundary).

No action on  proposal 2.

JVET-P0927 Crosscheck report of JVET-P0161 [K. Misra (Sharp Labs of America)]

JVET-P0195 Non-CE5: Unified strong deblocking filter for luma and chroma [Y. Ahn, D. Sim (Digital Insights)]

It is proposed to used the same filter for luma and chroma.

Method 1: Use luma filter for both.

Method 2: use chroma filter for both.

The proponent prefers method 1 to be adopted.

It was mentioned that the complexity savings are not significant, and it might be dangerous to change the deblocking filter at this stage.

No action is suggested.

JVET-P0760 Cross-check of JVET-P0195 (Non-CE5: Unified strong deblocking filter for luma and chroma) [S.-C. Lim, H. Lee, J. Lee, J. Kang (ETRI)]

JVET-P0247 Non-CE5: Intra deblocking coefficients for weak filters [L.-H. Xu, J.-Q. Zhu, K. Kazui (Fujitsu)]

This contribution was withdrawn.

JVET-P0950 Crosscheck of JVET-P0262 (AHG16/Non-CE5: Deblocking boundary design cleanup for affine and TPM mode) [H. Liu (Bytedance)]

JVET-P0269 AHG18 / non-CE5: Deblocking for TPM and BCW

Method 1 is similar to P0161 method 2 with a difference that deblocking is only applied to

P0602 is exactly the same as method 1 here.

Recommendation: Test method 1 in the CE.

Method 2 is similar to P0161 method 1.

JVET-P0862 Crosscheck report of JVET-P0269 (AHG18 / non-CE5: Deblocking for TPM and BCW) [X. Xu (Tencent)]

JVET-P0306 Non CE5: DMVR internal edge deblocking without using refined MVs [J. R. Arumugam, S. Kotecha, S. Ramamurthy (Ittiam)]

Proponent said the contribution did not need to be presented.

JVET-P0743 Crosscheck of JVET-P0306 (Non CE5: DMVR internal edge deblocking without using refined MVs) [T. Chujoh (Sharp)]

JVET-P0386 Non-CE8/Non-CE5: Deblocking for IBC blocks [H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang (Huawei)]

It is claimed that currently, in VVC, wrong Bs decisions may be taken. The alleged problem is that the real MVs in IBC may be pointing to the same area after the modulo operation, and the deblocking would not be needed but still applied.

It was noticed that the proposal may not solve the problem when the blocks in the virtual buffer are discontinuous.

It was proposed to study other proposals related to IBC first before making a decision on the current proposal.

It was also mentioned that after the modulo operation, different MVs may become similar, and the deblocking would not be applied, although it should.

It was mentioned that the current spec allows to either use or not use the deblocking by choosing the motion vectors.

Revisit after offline discussion.

P0611 is a related proposal in the part related to IBC.

JVET-P0864 Crosscheck report of JVET-P0386 (Non-CE8/Non-CE5: Deblocking for IBC blocks) [X. Xu (Tencent)]

JVET-P0411 Non-CE5: Fixes for long luma deblocking filter decision [K. Andersson, J. Enhorn (Ericsson)]

It is claimed that sometimes filtering for long luma deblocking case is applied, although it should not.

It is proposed to modify the deblocking decisions (to add the gradient check for samples … ). It is also proposed to change the thresholds

Two solutions have been proposed.

First proposal is to add a gradient check for the calculation of the deblocking filter in the case when maxFilterLength==7.

Proposal 2 is to change the threshold for dqp for a factor of 4.

It was commented that it could be possible to send more threshold on the deblocking level.

Recommendation: study in the CE.

JVET-P0423 Non-CE5: Modified boundary strength derivation [W. Zhu, L. Zhang, J. Xu (Bytedance)]

It is proposed to replace the conditions for the inter prediction motion derivation for the MV with the following expression.

(Abs(MVP[0].x - MVQ[0].x) > Th || Abs(MVP[0].y - MVQ[0].y) > Th

|| Abs(MVP[1].x - MVQ[1].x) > Th) || Abs(MVP[1].y - MVQ[1].y) > Th)

Th is half integer pixel.

It was commented that the proposed condition allows many corner cases compared to the current deblocking, which was thoroughly tested in HEVC.

No action.

JVET-P0635 Crosscheck of JVET-P0423 (Non-CE5: Modified boundary strength derivation) [T.-H. Li, C.-Y. Teng (Foxconn)]

JVET-P0441 Non-CE5: On deblocking on affine internal prediction sub-block edges [G. Li, X. Li, X. Xu, S. Liu (Tencent)]

It is proposed to disable affine sub-PU boundary deblocking when PROF is enabled, to make the MV delta based deblocking behavior consistent. When the edge is an affine internal subblock edge which is not aligned with CU boundary or TU boundary, the deblocking is disabled by skipping the MV difference based deblocking strength derivation. Some minor gains are reported.

It was commented that the suggested change is consistent with the PROF tool, which is targeted to remove the discontinuities between the subblock boundaries. The check for the transform boundaries is still performed.

It was agreed that the change is theoretically sound but assumes that PROF works well. It was commented that it should be tested before.

Recommendation: test in the CE (and improvements are expected).

JVET-P0944 Crosscheck of JVET-P0441 (Non-CE5: On deblocking on affine internal prediction sub-block edges) [K. Unno (KDDI)]

JVET-P0534 AhG16/Non-CE5: On deblocking at ALF virtual boundaries [Z. Deng, L. Zhang, K. Zhang, H. Liu, H.-C. Chuang (Bytedance)]

It is claimed that processing in CTU pipeline is not consistent.

It is proposed to disable deblocking on the 4x4 deblocking grid that coincides with the ALF virtual boundary.

It was commented from the floor that there is no problem with the current design.

No action.

JVET-P0727 Crosscheck of JVET-P0534 (AhG16/Non-CE5: On deblocking at ALF virtual boundaries) [Y.-W. Chen (Kwai Inc.)]

JVET-P0539 Non-CE5: A chroma deblocking clean-up [J. Xu, L. Zhang, W. Zhu, K. Zhang, H. Liu (Bytedance)]

This was discussed in combination with P1001 and P1002. The clean-up aspect of the contribution may need to be presented pending on decision on solution 2 of P1001.

Revisit.

JVET-P0547 Non-CE5: Deblocking filter at PROF sub-block boundary [K. Unno, K. Kawamura, S. Naito (KDDI)]

The proposal is similar in spirit to P0441. The proponent prefers the proposal in P0441 and supports it.

JVET-P0877 Crosscheck of JVET-P0547 "Non-CE5: Deblocking filter at PROF sub-block boundary" [G. Li (Tencent)]

JVET-P0570 AHG10/Non-CE5: Performance of encoder-side deblocking optimization in VTM-6.0 [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

This proposal is informational.

The encoder-only deblocking optimization is tested on top of VTM-6.0. The average Y/U/V BD-rates and encoding time for the common test conditions for the filter on top of VTM-6.0 are reported to be −0.77%/−1.53%/−1.56%/105% for all intra, −0.92%/−1.94%/−1.93%/103% for random access and −0.78%/−0.90%/−0.88%/102%  for low delay B.

JVET-P0571 CE5/CE8: Deblocking Filter for BDPCM coded block [H. Jang, J. Nam, S. Kim, J. Lim (LGE)]

The proposal suggests that there is a mismatch between the spec and the software regarding the behaviour for the BDPCM (independent derivation of Bs for BDPCM mode between the components in the spec, and dependent in the software).

Method 1. Align software with the specification.

Method 2. It is proposed to bypass the deblocking on the BDPCM side in the same way how it is done for Palette.

It was mentioned that the focus should be on 4:2:0.

This proposal is related to P0386.

Recommendation: adopt method 1 (align the software to the specification)

JVET-P0851 Crosscheck of JVET-P0571 (CE5/CE8: Deblocking filter for BDPCM coded block) [C.-M. Tsai (MediaTek)]

JVET-P0851 Crosscheck of JVET-P0571 (CE5/CE8: Deblocking filter for BDPCM coded block) [C.-M. Tsai (MediaTek)]

JVET-P0586 [AHG18][Non-CE5] Proposed cleanup of deblocking filter process [S. Iwamura, S. Nemoto, A. Ichigaya (NHK), K. Andersson, R. Yu, J. Enhorn (Ericsson)]

Aspect 1 of the proposal has been presented.

If at least one of the blocks have non-zero coefficient and not TS, then BS is set equal to 1. See the discussion below

//-------------------------------------

Current deblocking behaviour:

- If both blocks are BDPCM, Bs = 0; if one side is BDPCM, it is treated as intra, Bs = 2.

- If one blocks is palette, we do skip on the palette side.

- TS is treated as a normal (transform) block.

Three related proposals

- P0586. If at least one of the blocks have non-zero coefficient and not TS, then BS is set equal to 1.

- P0571 Method 2. It is proposed to bypass the deblocking on the BDPCM side in the same way how it is done for Palette. The claim is mostly consistency between palette and BDPCM modes.

- P0611. If one of the blocks is BDPCM, TS, or Palette, then BS = 0.

It was mentioned that BDPCM and TS do not apply transforms in a similar way, so deblocking should be applied in a similar way. It was mentioned that it would be desirable to treat three modes similarly.

It was mentioned that BDPCM is not allowed in CTC on classes E-A and is only used in the screen content.

Recommendation: further study outside the CE, possibly in the context of former AHG11.

//-------------------------------------

Aspect 2 has been considered in the previous discussion on …

Aspect 3.

When VTM uses scaling list, the QP values are still derived without taking it into account that may results in wrong tc and beta values. An offset is derived based on the 6*log2(m/16), where m is the representative value of the scaling list (in this case the DC). The logarithm is approximated with the LUT of size 23 bytes.

It was commented that the proposal makes sense in general but some evidence needs to determine if the action is needed.

Recommendation: Further study (possibly in former AHG15). In particular, it needs to be decided what quantization matrices need to be used.

JVET-P0700 Crosscheck of JVET-P0586 Aspect #1 ([AHG18][Non-CE5] Proposed cleanup of deblocking filter process) [Z. Deng (Bytedance)]

JVET-P0783 Crosscheck of JVET-P0586 (Aspect 3 of [AHG18][Non-CE5] Proposed cleanup of deblocking filter process) [K. Kondo, M. Ikeda (Sony)]

JVET-P0815 Crosscheck of JVET-P0586 (Non-CE5: Proposed cleanup of deblocking filter process) [K. Kazui (Fujitsu)]

JVET-P0602 [AHG18][non-CE5] Boundary strength derivation for CUs with TPM [S. Iwamura, S. Nemoto, A. Ichigaya (NHK)]

This proposal was discussed in P0269.

JVET-P0611 Non-CE5/AHG-11: Boundary strength harmonization for BDPCM, TS, Palette and IBC [B. Ray, G. Van der Auwera, M. Karczewicz (Qualcomm)]

The IBC aspect of this proposal is related to P0386.

Three tools do not use deblocking TS, BDPCM, and Palette, and IBC has is often used for screen content. Proposal 1. It is proposed to set Bs to zero is either of the adjacent blocks are TS, BDPCM or Palette.

Proposal 2. It is proposed to set Bs to zero is either of the adjacent blocks are TS, BDPCM, Palette, and IBC.

Some objective gains are reported, esp. for the TGM test conditions. One of the participants supports Proposal 2.

It was mentioned that IBC can be used for All Intra case on natural content (up to -1.3% BD-rate gain).

It was mentioned that one of the aspect in 586 is related to the transform skip

P0571 is a similar contribution (for BDPCM).

It was decided that there is not enough evidence to disable deblocking for IBC.

JVET-P0981 Cross-check of JVET-P0611 (Non-CE5/AHG-11: Boundary strength harmonization for BDPCM, TS, Palette and IBC) [K. Andersson (Ericsson)]

JVET-P0613 Non-CE5: On the average calculation for luma adaptive deblocking filter [B. Ray, D. Rusanovskyy, G. Van der Auwera, M. Karczewicz (Qualcomm)]

The contribution proposes to modify the average luma calculation value for luma average dependent deblocking filter, by utilizing the information of maximum filter length from side P and side Q. The results for SDR-CTC and HDR-CTC conditions are reported below.

In the current VTM, the nearest samples to the block boundary are used for the average luma calculation. It is proposed to use the average luma calculation based on the maximum filter size for the block.

In the current VVC, the calculations for the average is

LL= ( ( p0,0 + p0,3 + q0,0 + q0,3 ) >> 2 ) / ( 1 << iBitDepth ).

It is proposed to replace it with

LL= ( ( p0,0 + pmaxP,0 + p0,3 + pmaxP,3 + q0,0 + qmaxQ,0 + q0,3 + qmaxQ,3) >> 3 ) / ( 1 << iBitDepth ),

A proponent of the initial average luma deblocking filter has expressed support.

We optimal parameters for the average luma DF for the proposal to show benefit of the proposal.

Recommendation: Further study.

JVET-P0614 Non-CE5: On the gridsize for virtual boundaries.

In the current VTM, virtual boundary is only defined on the 8x8 grid (the edges on a VB are not deblocked). It is requested to allow specifying VB on a 4x4 grid for more flexibility.

It was commented that more flexibility is not needed since the problem can be solved with padding and the cropping window.

Recommendation: No action

JVET-P0863 Crosscheck of JVET-P0613: Non-CE5: On the average calculation for luma adaptive deblocking filter [K. Naser (InterDigital)]

JVET-P0688 Non-CE5: Intra deblocking coefficients for weak filters [L. Xu, J. Zhu, K. Kazui (Fujitsu)] [late]

New weights for the intra deblocking filter have been proposed, obtained by the training process (on an training) by minimizing reconstruction error.

It was mentioned by the proponent that sometimes the proposal results in a weaker deblocking filtering than the current deblocking.

The proponents have not looked at the frequency response of the filters.

It was also claimed that the gain can be partially attributed to the higher dynamic range of the filter coefficients.

It was mentioned that the further studies are not likely to results in an adoption.

Recommendation: No action.

JVET-P0837 Crosscheck of JVET-0688 on Intra deblocking coefficients for weak filters [S. Iwamura (NHK)]

JVET-P0740 CE5-related: Combination of JVET-P0086 and JVET-P0161 for TPM deblocking [C.-M. Tsai, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek), A. M. Kotra, S. Esenlik, H. Gao, B. Wang, E. Alshina (Huawei)] [late] [miss]

Discussed when presenting P0085 and P0161.

JVET-P0918 Crosscheck of JVET-P0740 (CE5-related: Combination of JVET-P0086 and JVET-P0161 on deblocking boundary strength fix for TPM and affine mode) [H. Jang (LGE)]

JVET-P1001 Non-CE5: Chroma QP derivation fix for deblocking filter (Combination of JVET-P0105 and JVET-P0539) [A. M. Kotra, S. Esenlik, B. Wang, H. Gao, E. Alshina (Huawei)] [late]

It is claimed that there is a problem that table for Joint CbCr mode not used in the deblocking although used in transform coefficient scaling.

Two solutions are proposed. Solution 1 uses only the picture level QP offset.

Solution 2 uses slice level and block level QP offsets.

The proponent thinks that Solution 1 is more in the spirit of the VTM behavior, while the Solution 2 better takes into account the transform coefficient scaling.

There are two aspects of this proposal: solution 1 fixes the chroma mapping problem, whereas solution 2 also fixes the aspect of QP offsets.

It was noted that there is a note in the specification that suggests that this problem was not fixed (HEVC), however, none of the participants remember why.

Some participants mentioned that fundamentally you would like to solve this problem. Some participants expressed concerns regarding complexity.

Recommendation: adopt solution 2 (if there are no complexity concerns raised by the time of review in the track).

JVET-P1002 Non-CE5: Chroma QP derivation fix for deblocking filter (Combination of JVET-P0105 and JVET-P0539) [J. Xu, L. Zhang, W. Zhu, K. Zhang, H. Liu (Bytedance Inc.)] [late]

Same as P1001.

JVET-P0246 AHG17/Non-CE5: on loop filter processing for subpicture treated as a picture

See for the disposition of the HLS aspects of this contribution.

TBP for the low-level signal processing aspects.

The contribution has two aspects:

  - Aspect 1: signal “loop_filter_across_subpic_enabled_flag[i]” depending on “subpic_treated_as_pic_flag[ i ]”

  - Aspect 2: modifying pixels across subpicture that has subpic_treated_as_pic_flag is equal to 1 should be disallowed. ( for bottom / right subpicture boundary)

It was commented that the HLS AHG agreed that Method 1 is not needed.

It was suggested that this should be discussed in the HLS group. It was also mentioned that this proposal can have some low-level impact.

## Adaptive Loop Filtering ()

| JVET number | Title | Tag |
| --- | --- | --- |
| JVET-P0053 | AHG16/Non-CE5: A cleanup for the ALF sample padding |  |
| JVET-P0121 | On adaptive loop filter |  |
| JVET-P0156 | CE5-related: ALF padding process when raster scan slices are used |  |
| JVET-P0157 | CE5-related: Align ALF padding processes at picture and subpicture boundaries |  |
| JVET-P0158 | CE5-related: Align ALF virtual boundary processes in the bottom CTU rows of one picture and one subpicture |  |
| JVET-P0162 | CE5-related: Simplified ALF syntax with removal of alf_ctb_use_first_aps_flag |  |
| JVET-P0164 | CE5-related: Simplified ALF syntax regarding to alf_luma_coeff_signalled_flag and alf_luma_coeff_flag |  |
| JVET-P0179 | Non-CE5: ALF as a post-filter |  |
| JVET-P0180 | Non-CE5: Cleanup of ALF syntax elements |  |
| JVET-P0192 | CE5-related: Reducing the number of luma filters in ALF |  |
| JVET-P0246 | AHG17/Non-CE5: on loop filter processing for subpicture treated as a picture |  |
| JVET-P0332 | AHG17/Non-CE5: On parsability of alternative ALF Chroma filter signaling | was reviewed |
| JVET-P0452 | Non-CE5/AHG17: Low-delay ALF syntax |  |
| JVET-P0551 | AHG16/Non-CE5: On ALF boundary padding |  |
| JVET-P0552 | AHG12/Non-CE5: Extending slice boundary processing for adaptive loop filter for raster scanned slices |  |
| JVET-P0553 | Non-CE5: Using truncated binary codes for ALF filter indices |  |
| JVET-P0554 | Non-CE5: Clean up of coefficient coding of adaptive loop filter |  |
| JVET-P0614 | Non-CE5: On the gridsize for virtual boundaries |  |
| JVET-P0665 | Non-CE5: Spec fix for ALF filter and transpose index calculation |  |

JVET-P0053 AHG16/Non-CE5: A cleanup for the ALF sample padding [M. Zhou (Broadcom)]

In the current ALF design, the padding is applied to

- Picture boundaries

- 360 video virtual boundaries (VBs) that are not aligned with the CTU boundaries (in this case, the de-blocking and SAO are disabled along those 360 virtual boundaries when the ALF sample padding takes place).

While the ALF virtual boundary mirrored padding is applied to

- ALF virtual boundaries (located at 4 luma lines above the bottom CTU boundaries, horizontal only)

- Sub-picture/slice/tile/brick boundaries, and 360 video virtual boundaries that are aligned with the CTU boundaries (in this case, the de-blocking and SAO are disabled along those boundaries when the ALF sample padding takes place).

I it is proposed to apply the ALF mirrored padding to the ALF virtual boundaries only, and apply the repetitive padding to all the other boundaries.

Some support has been expressed by the participants for this repetitive padding except for the ALF virtual boundaries

To simplify the current design, it is further recommended to remove the 360 video virtual boundaries that are not CTU boundary aligned.

This was not agreed on.

Related contribution are P0157 and P551

JVET-P0121 On adaptive loop filter [J. Chen, Hendry (Futurewei)]

Contributions 0121, 0156, 0492, 0552 are related.

It was identified that the ALF sampling padding process in VTM6 does not work properly at slice boundaries in case of raster-scan slice mode. This contribution provides two options to fix the identified problem as follows:

Option 1: Apply a constraint such that raster scan slice mode shall be disallowed when ALF across brick boundaries is enabled but ALF across slice boundaries is disabled. The following constraint is proposed:

It is a requirement of bitstream conformance that when the value of loop_filter_across_bricks_enabled_flag is equal to 1 and loop_filter_across_slice_enabled_flag is equal to 0, the value of rect_slice_flag shall be equal to 1.

Option 2: Checking process is added to handle the raster-scan slice mode during ALF boundary position derivation process. For this process, in addition to check whether the top, bottom, left and right coding tree blocks (CTBs) are from the same slice, the top-left and bottom-right CTBs are also checked

Support was expressed for Method 1. It was also mentioned by a participants that Method 1 is not preferable.

It was mentioned that only top padding or left padding is needed in Method 2.

JVET-P0156 CE5-related: ALF padding process when raster scan slices are used [C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek), A. M. Kotra, S. Esenlik, H. Gao, B. Wang, E. Alshina (Huawei)]

Contributions 0121, 0156, 0492, 0552 are related.

To solve the issue with padding samples availability in the raster scan order, two additional checks are added into the condition of enabling ALF padding process, when loop filter cannot cross slice boundaries (loop_filter_across_slices_enabled_flag is equal to 0). If top-left neighboring samples are unavailable and left neighbouring samples are available, top neighbouring samples are also treated as unavailable. If bottom-right neighboring samples are unavailable and right neighbouring samples are available, bottom neighbouring samples are treated as unavailable. Next, the current ALF padding process is applied to handle the unavailable neighbouring samples.

It was mentioned that the conditions on samples checks in this proposal might be tighter (checking a sum of the sample offsets rather each of them individually).

JVET-P0157 CE5-related: Align ALF padding processes at picture and subpicture boundaries [C.-Y. Lai, O. Chubach, L. Chen, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

In VVC Draft 6, two kinds of padding methods are used in adaptive loop filter (ALF) to generate unavailable neighbouring samples. One is the repetitive padding for picture boundaries, and the other is the ALF virtual boundary (VB) padding for slice/brick/subpicture boundaries. When a picture consists of independently decodable subpictures and the subpictures may be extracted as a picture, a mismatch may occur for the boundary padding if the subpicture boundaries are not the picture boundaries. Four methods are proposed in this contribution.

1. If picture boundaries are aligned with coding tree unit (CTU) boundaries, applying the ALF-VB padding to the picture boundaries; otherwise applying the repetitive padding to the picture boundaries.

2. If subpic_treated_as_pic_flag[ i ] is equal to 1 and loop_filter_across_subpic_enabled_flag[ i ] is equal to 0, applying the repetitive padding to the subpicture boundary; otherwise, if loop_filter_across_subpic_enabled_flag[ i ] is equal to 0, applying the ALF-VB padding to the subpicture boundary.

3. Applying the repetitive padding to all subpicture boundaries.

4. Applying the repetitive padding to all slice/brick/360/subpicture/picture boundaries.

The proponents prefer Method 1 and Method 4.

Method 4 is aligned with 0053.

It was claimed that mirrored padding has subjective quality benefit compared to the repetitive boundary.

JVET-P0158 CE5-related: Align ALF virtual boundary processes in the bottom CTU rows of one picture and one subpicture [C.-Y. Lai, O. Chubach, L. Chen, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

In VVC Draft 6, to remove the line buffers required in adaptive loop filter (ALF) hardware decoding, the ALF virtual boundary (VB) process is applied to each coding tree unit (CTU) row in a picture except for the bottom CTU row. When a picture consists of independently decodable subpictures and the subpictures may be extracted as a picture, the mismatch may occur for the bottom CTU row handling if the subpicture bottom boundary is not the picture bottom boundary. Two methods are proposed in the contribution to solve this issue.

1. Applying the ALF VB process to all CTU rows in a picture, including the bottom CTU row

2. Disabling the ALF VB process for subpicture bottom CTU rows if the subpicture can be independently decodable (subpic_treated_as_pic_flag[ i ] = 1 and loop_filter_across_subpic_enabled_flag[i]=0 )

Recommendation: adopt method 1, applying ALF VB to all CTU in a picture, including the bottom CTU row.

JVET-P0162 CE5-related: Simplified ALF syntax with removal of alf_ctb_use_first_aps_flag [Y.-L. Hsiao, C.-C. Chen, O. Chubach, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

The contribution proposes to remove the alf_ctb_use_first_aps_flag that indicates whether a luma CTB uses the filter set from the first adaptation parameter set (APS), is removed, and related parsing processes are simplified. Results are summarized as follows.

Proposed method
VTM6.0-AI: {Y-BD-rate=0.00%, EncT=100%, DecT=100%}
VTM6.0-RA: {Y-BD-rate=0.01%, EncT=100%, DecT=100%}
VTM6.0-LB: {Y-BD-rate=-0.03%, EncT=100%, DecT=99%}

It was commented that it is possible that the alf_ctb_use_first_aps_flag shortcut is not used in the current CTC bitstream. It was agreed that the SE alf_ctb_use_first_aps_flag should be removed if it the current encoder frequently puts the ALF syntax in the first position in the APS list in the current picture because that would indicate that the SE is not helpful.

Karsten volunteered to help with this check. Revisited: Food Market and BlowingBubbles were studied and it was reported that from the inspection of the bitstreams it looks like that the most recent SPS is often in a correct position and the syntax element alf_ctb_use_first_aps_flag is 1 roughly half of the time. From that, it can be concluded that the current syntax does not help much.

It was mentioned that the current syntax helped previously (at Ljuabljana meeting) but not previously

0180 method 2 is identical to this proposal except the binarization part.

Recommentation: It was agreed to adopt the proposal.

JVET-P0748 Cross-check of JVET-P0162 (CE5-related: Simplified ALF syntax with removal of alf_ctb_use_first_aps_flag) [K. Andersson (Ericsson)]

JVET-P0164 CE5-related: Simplified ALF syntax regarding to alf_luma_coeff_signalled_flag and alf_luma_coeff_flag [C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, O. Chubach, Y.-W. Huang, S.-M. Lei (MediaTek)]

In VTM6.0 adaptive loop filter (ALF), alf_luma_coeff_signalled_flag is used to indicate whether a zero filter exists in the filter set. If alf_luma_coeff_signalled_flag is true, an alf_luma_coeff_flag is signalled for each filter in the filter set to indicate whether the filter is a zero filter. In this contribution, two methods to simplify the related ALF syntax are proposed. In Method 1, alf_luma_coeff_signalled_flag is signalled before alf_luma_num_filters_signalled_minus1 with removal of alf_luma_coeff_flag. In Method 2, alf_luma_coeff_signalled_flag and alf_luma_coeff_flag are both removed. It is asserted that both methods simplify the related specification text. BD-rates are reported as follows.

Method 1:
AI: 0.00% (Y), 0.00% (Cb), 0.00% (Cr)
RA: 0.00% (Y), 0.00% (Cb), 0.00% (Cr)
LB: -0.01% (Y), 0.08% (Cb), 0.12% (Cr)

Method 2:
AI: 0.00% (Y), 0.00% (Cb), 0.00% (Cr)
RA: 0.00% (Y), 0.01% (Cb), 0.00% (Cr)
LB: -0.03% (Y), -0.03% (Cb), -0.08% (Cr)

Proponents prefer method 1.

It was mentioned that the proposed syntax in method 1would force the encoder to merge the zero-valued ALF filters.

Method 2 would require to signal coefficients for all filters.

Recommendation: it was agreed to adopt Method 2 (and SW from this proposal).

JVET-P0946 Crosscheck of JVET-P0164 (CE5-related: Simplified ALF syntax regarding to alf_luma_coeff_signalled_flag and alf_luma_coeff_flag) [A. M. Kotra (Huawei)]

JVET-P0179 Non-CE5: ALF as a post-filter [H.-B. Teo, H.-W. Sun, C.-S. Lim (Panasonic)]

The contribution proposes the use of ALF as a post-filter. A flag is signalled which switches ALF to function as either in-loop filter or post-filter. The operation and signalling of ALF itself is not changed.

Two methods for signalling ALF as a post-filter are proposed: in SPS, in SEI.

It was mentioned that the current loop ALF can do sharpening but the sharpening can decrease the coding efficiency.

It was mentioned that there is a decoder hint SEI message in HEVC that can be used instead.

It was mentioned that using ALF as a post-filter normatively may need applying ALF “on the fly” when outputting the picture. It was mentioned that using ALF as a post-filter in SPS is outside the scope of the standard.

It was mentioned that there is no need to rush with this SEI.

Further study recommended.

JVET-P0180 Non-CE5: Cleanup of ALF syntax elements [H. Liu, L. Zhang, K. Zhang, N. Zhang, Y. Wang, Y. Wang (Bytedance)]

Method 2 of the proposal is identical to P0162 except the binarization part.

Method 1 of the proposal is identical to P0164 method 2.

JVET-P0672 Crosscheck of JVET-P0180 (Non-CE5: Cleanup of ALF syntax elements) [C.-Y. Lai (MediaTek)]

JVET-P0192 CE5-related: Reducing the number of luma filters in ALF [O. Chubach, C.-Y. Chen, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)]

In VTM6.0 adaptive loop filter (ALF), the number of luma filters is in the range of 0 to 25, inclusive, and after the previous Gothenburg meeting, the number of chroma filters is increased from 1 to 8. As a result, the memory required for storing all ALF coefficients is considereably increased. In this contribution, it is proposed to reduce the maximum number of luma filters signalled for ALF from 25 to 16. It is asserted that proposed method allows to reduce the required on-chip memory, while keeping flexibility of 25 initial luma filter classes, since there is no constraint on the ways of combining 25 luma filter classes into 16 different merged classes.

BD-rates are reported as follows.
AI: 0.00% (Y), 0.00% (Cb), 0.00% (Cr)
RA: 0.00% (Y), 0.01% (Cb), -0.01% (Cr)
LB: 0.00% (Y), 0.03% (Cb), 0.04% (Cr)

The number of ALF classes in the proposal is still 25. It was commented that some classes need to be merged to use the same filter.

The proponents claim that there are about 2% on non-CTC of the APS that use more than 16 filters.

It was mentioned that the syntax does not need to be 16-minus.

It was mentioned that the number of filters can be a level constraint.

It was also mentioned that it is not clear if a memory is a problem. It was mentioned that we already reduced the memory four times at the last meeting.

There were concerns raised against the proposed change. It was mentioned that some participants would prefer to keep the same number of filters as the number of classes.

Recommendation: no action.

JVET-P0767 Crosscheck of JVET-P0192 (CE5-related: Reducing the number of luma filters in ALF) [Y.-W. Chen (Kwai)]

JVET-P0332 AHG17/Non-CE5: On parsability of alternative ALF Chroma filter signaling [J. Taquet, P. Onno, C. Gisquet, G. Laroche (Canon)]

This was reviewed. The results are in the HLS BoG.

JVET-P0801 Crosscheck of JVET-P0332: AHG17/Non-CE5: On parsability of alternative ALF Chroma filter signaling [S. Esenlik (Huawei)]

JVET-P0452 Non-CE5/AHG17: Low-delay ALF syntax [K. Sühring, R. Skupin, Y. Sanchez, T. Schierl (HHI)]

According to the contribution, in low-delay coding, ALF processing currently cannot be used because the ALF filter parameters of a picture are estimated after the picture is completely encoded. The ALF APS still needs to be inserted into the bitstream before the picture and the per-CTU data interspersed with the coded CTUs in the slice payload. This causes unnecessary processing delays in writing the bitstream, because the decoder also runs the ALF at the end of the decoding process of a picture. This contribution proposes to move ALF related data (ALF APS, ALF per-CTU data) towards the end of the picture, or behind the picture.

In the HLS BoG, it was suggested to use a VCL NAL unit instead of a new APS type for coding CTU-related ALF information. This variant is added as option 2c in version 2 of this contribution. Additionally an enable-flag is proposed in the SPS and a missing initialization indication for CABAC is added in options 2a and 2c.

It was commented that the ALF parameters from the previous picture can be used for as ALF parameters of the current picture. It was mentioned that this low-delay ALF was tested in Ljubljana with 0.5 to 0.7% BD-rate loss. It was also mentioned that the decoder would need ALF data for the decoding.

It was commented that the proposal 1 was already agreed on in the HLS BoG.

There was no agreement to recommend proposals 2a-2c.

JVET-P0492 Non-CE5: Fixes of ALF sample padding [H. Liu, L. Zhang, K. Zhang, Y. Wang, J. Xu, Y. Wang (Bytedance)]

Contributions 0121, 0156, 0492, 0552 are related.

It is proposed to use the top left sample of the current slice to pad the unavailable samples (for the top left case). The check is done similar to 552. The proposals checks all four corners (similar to 0492) although it was mentioned that only top left and bottom right are needed.

JVET-P0551 AHG16/Non-CE5: On ALF boundary padding [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

Similar topic as to 0053 and 0157.

In VVC draft 6, there are two ways to handle unavailable pixels beyond boundaries for adaptive loop filter (ALF).

- Repetitive padding: for picture boundaries and 360 virtual boundaries that are not on CTU boundaries, pixels are padded towards outside of the boundaries. Then after padding, Laplacian calculation and filtering process are performed without changes.

- Mirrored padding: for the boundaries (virtual boundary for line buffer reduction, sub-picture boundaries and 360 virtual boundaries on CTU boundaries), the samples located beyond the line buffer boundary are excluded in ALF process, where Laplacian is re-scaled and truncated symmetrical filtering is applied.

In JVET-N0654, it was proposed to unify the padding methods, such that only one padding method is applied to all type of boundaries. In version 2, results are updated.

Three possible unification and simplification by reusing existed methods are applied:

- apply mirrored padding to 360 virtual boundaries and picture boundaries

- apply repetitive padding for all boundaries excluding virtual boundaries for ALF line buffer reduction

- apply repetitive padding for all boundaries

Method 2 is the same as method 4 in 0157 and first proposal in 0053.

Recommendation: Adopt method 2 in 0551, which is also method 4 in 0157 and first aspect of 0053. The software is from P0551. Clarification: on Thursday, Oct. 10, at 18:20.

For cases of filtering with repetitive padding, the classification padding should be the same as currently in the cases of repetitive padding (e.g. as at the picture boundary). ALF virtual boundary classification should not be changed.

JVET-P0552 AHG12/Non-CE5: Extending slice boundary processing for adaptive loop filter for raster scanned slices [N. Hu, V. Seregin, M. Coban, A. K. Ramasubramonian, M. Karczewicz (Qualcomm)]

Contributions 0121, 0156, 0492, 0552 are related.

To handle unsupported cases in VVC draft 6, it is proposed to convert such cases to a case, which is already supported, such as to treat the above CTU boundary or the left CTU boundary as a slice boundary even some samples belong to the same slice.

The problem is converted into the case when the top CTU is in a different slice such that mirrored padding is applied along the top CTU boundary.

It was agreed to add the green text to the specification as a fix:

- The variables ac[ x ][ y ], sumH[ x ][ y ], sumV[ x ][ y ], sumD0[ x ][ y ], sumD1[ x ][ y ] and sumOfHV[ x ][ y ] with x, y = 0..( CtbSizeY − 1 ) >> 2 are derived as follows:

The variables clipLeftPos, clipRightPos, clipTopPos and clipBottomPos are derived by invoking the ALF boundary position derivation process as specified in clause 8.8.5.5 with ( xCtb, yCtb ), ( x << 2, y << 2 ),

Proposals 0156 and 0552 uses top padding for the top-left block when top-left sample(s) are unavailable. 0492 uses a repetitive padding from the top-left corner sample.

The definition of a sample being unavailable uses a sum of the top and left offset in 0552 and 0492, and two separate checks (left and right) in 0156. The condition in 0552 is tighter.

Since the repetitive padding was recommended (see discussion on padding), proposals 0156, 0552 would use the repetitive padding instead.

Recommendation:  It was agreed to use the check for the unavailable samples as in 0552 (and same as in 0492).

It was agreed that the repetitive padding is used for all the top samples, when there are unavailable samples. This is similar to what was proposed in 0156 and 0552 except that the mirrored padding is replaced by the repetitive padding according to the discussion earlier. This means a top horizontal line in the CTU is used to pad top samples. This applies to the top-left unavailable case, the bottom-right case would use symmetrical operations.

Revisit on Thursday, Oct. 10, at 18:20.

It was mentioned that current VTM would use sample (0,0) to fill unavailable samples to the top-left (as in 0492) for the top-left corner of the proposal.

It was mentioned that even the current design may not be good.

It was proposed by Minhua (Broadcom) to use horizontal repetitive padding for filling unavailable samples. No text or software are available though.

Support has been expressed for the proposal from Broadcom.

Support has also been expressed for using the top left sample to fill the unavailable samples 0492. It was commented that 0492 would create a special case.

Recommended: Adopt padding as in Broadcom verbal proposal that uses horizontal repetitive padding. Bytedance volunteered to provide specification text and the software.

Clarification: the unavailable samples are determined as in P0552 / P0492.

JVET-P0553 Non-CE5: Using truncated binary codes for ALF filter indices [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

In VVC draft 6, an alf_ctb_filter_alt_idx is signalled for a chroma coding tree block (CTB) to indicate the index of the chroma filter. A unary code is used to signal an alf_ctb_filter_alt_idx and one context is shared for all bins. In this contribution, it is proposed to remove the context and replace the unary codes by truncated binary codes to unify with the signaling method of the filter set index for a luma CTB. Compared to VTM-6.0 with common test conditions, similar coding efficiency and running times are observed.

- Context removal: each bin of the unary code is signalled with bypass mode

- Unification with luma CTB: replace unary code by truncated binary code

There is slight loss on the chroma BD-rate and no problem is solved.

No action.

JVET-P0953 Crosscheck of JVET-P0553 (Non-CE5: Using truncated binary codes for ALF filter indices) [H. Liu (Bytedance)]

JVET-P0554 Non-CE5: Clean up of coefficient coding of adaptive loop filter [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

In VVC draft 6, the absolute value of a chroma adaptive loop filter coefficient is constrained to be in the range of 27 to 27 − 1, inclusive. While a chroma filter coefficient is constrained to be in the range of −27 − 1 to 27 − 1, inclusive. Following these two constraints, the chroma coefficients cannot be equal to −27, however this value is within 8 bits, and secondly −27 – 1 is outside of 8 bits range. The contribution proposes some options to clean up the conflict.

Three methods were proposed to fix the problem. It was agreed that method 2 is the simplest one to fix the problem, and only text changes are needed.

Recommendation: adopt method 2 (specification changes).

JVET-P0614 Non-CE5: On the gridsize for virtual boundaries [B. Ray, G. Van der Auwera, M. Karczewicz (Qualcomm)]

Was reviewed in the first day.

JVET-P0665 Non-CE5: Spec fix for ALF filter and transpose index calculation [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)] [late]

It is claimed that the current specification does not reflect correctly the processing for the ALF virtual boundary. The whole CTU is filled with samples at once. It is claimed that in order to do the ALF virtual boundary padding correctly, the processing should be done on the 4x4 block basis. The software implementation is correct.

Recommendation: BoG recommends fixing the specification to reflect the desired behaviour (using the correct samples for padding).

JVET-P0505 Non-CE5: On non-linear ALF clipping values [Y. Du, X. Zhao, X. Li, S. Liu (Tencent)]

In VTM-6.0, the clipping values used in non-linear ALF are derived for luma and chroma components with separate equations, which is not aligned with VVC Draft 6. In this contribution, a bug-fix to VTM-6.0 SW is first proposed to align with VVC Draft 6, and a method is proposed to derive ALF clipping values which are proportional to 2bitDepth. It is reported that, when InternalBitDepth is set equal to 8, average -0.01%, -0.06% and -0.10% luma BD-rates are obtained under AI, RA and LDB configurations, respectively, and average -0.19%, -0.31% and -0.40% luma BD-rates are obtained for TGM sequences under AI, RA and LDB configurations, respectively.

Bug-fix to VTM-6.0

It is reported that VTM-6.0 is not aligned with VVC Draft 6 on ALF clipping values (https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/551#ticket). Per decision from 15th JVET meeting, luma and chroma should use one single table of ALF clipping values. In this contribution, a bug-fix to VTM-6.0 is proposed to align with VVC Draft 6, and results are provided in Table 1 and 2 in Section 3.

Proposed method

In order to derive the clipping values which are proportional to 2bitDepth, the following one equation is proposed to derive ALF clipping values for both luma and chroma:

AlfClip=round2B-*n  for n∈0..N-1, 			(2-1)

where AlfClip is clipping value, B refers to internal bit depth or the bit depth of input samples for ALF,  is a pre-defined constant value equal to 2.35.

It was mentioned that there is indeed a mismatch between the specification and the software.

It was mentioned that the decision from the last meeting was to have a single table for luma and chroma.

There was a support for fixing the bug and using the proposed table. It was also mentioned that it is not clear whether the results would still be valid for other bit depths.

It is agreed that the decision from the last meeting for using the same table for luma and chroma clipping values has not been implemented in VTM6.0.

When implementing a single table in VTM6.0, there is no loss with IBD of 10. However, with IBD = 8, there is a loss of 0.6% - 0.8% in chroma.

It was claimed that the proposed formula may have been trained whereas the original formula has been derived on certain logic.

It is agreed that there is a problem with the current specification solution that needs to be solved.

It was mentioned that the decision at the last meeting was only on the 10-bit version.

It seems like the difference with the current VTM is small.

Recommendation: it was agreed to adopt the v5 of the document.

JVET-P0708 Crosscheck of JVET-P0505 (Non-CE5: On non-linear ALF clipping values) [H. Yang (InterDigital)]

## Cross-component ALF ()

| JVET number | Title | Tag |
| --- | --- | --- |
| JVET-P0106 | AHG16/CE5-Related: Simplifications for Cross Component Adaptive Loop Filter |  |
| JVET-P0165 | CE5-related: Simplified CCALF |  |
| JVET-P0173 | AHG16/Non-CE5: Cross component ALF simplification |  |
| JVET-P0178 | CE5-related: Unification of CCALF and ALF |  |
| JVET-P0251 | CE5-related: Simplified CCALF |  |
| JVET-P0290 | CE5-related: Clipping of intermediate value in CC-ALF |  |
| JVET-P0330 | Non-CE5: CC-ALF filtering simplification |  |
| JVET-P0331 | Non-CE5: CC-ALF design simplification |  |
| JVET-P0372 | CE5-related: Joint chroma cross-component adaptive loop filtering |  |
| JVET-P0468 | CE5-Related: Reducing multiplier count in CC-ALF |  |
| JVET-P0470 | Non-CE5: Simplification of Cross Component Adaptive Loop Filter |  |
| JVET-P0543 | CE5-related: Cleanup on cross-component adaptive loop filter |  |
| JVET-P0555 | CE5-related: Dynamic range reduction for coefficients of cross component adaptive loop filter |  |
| JVET-P0556 | CE5-related: Temporal buffer removal for cross component adaptive loop filter |  |
| JVET-P0557 | CE5-related: Multiplication removal for cross component adaptive loop filter |  |
| JVET-P0558 | CE5-related: Reduced filter shape for cross component adaptive loop filter |  |
| JVET-P0666 | Non-CE5: Combination of JVET-P0470 and JVET-P0557 on simplification of CC-ALF |  |
| JVET-P0739 | CE5-related: Combination of CCALF methods in JVET-P0165, JVET-P0556, and JVET-P0557 |  |

JVET-P0106 AHG16/CE5-Related: Simplifications for Cross Component Adaptive Loop Filter [A. M. Kotra, S. Esenlik, B. Wang, H. Gao, E. Alshina (Huawei)]

As shown in Fig.2, the filter shape is modified to have only 8 coefficients. The new filer shape also has a claimed advantage that the line buffer is reduced by one line when compared to the original filter shape.

Figure. 2 Modified CCALF filter shape

The results against the anchor are 0.65%, 0.66% Chroma losses on RA compared to CE5-2.1.

Recommendation: test in the CE. Since the shape is highly asymmetric, a test on other chroma locations is required.

JVET-P0847 Crosscheck of JVET-P0106 (AHG16/CE5-related: Simplifications for cross component adaptive loop filter) [C.-M. Tsai (MediaTek)]

JVET-P0165 CE5-related: Simplified CCALF [O. Chubach, C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)]

No need to review.

JVET-P0923 Crosscheck report of JVET-P0165 [K. Misra (Sharp Labs of America)]

JVET-P0173 AHG16/Non-CE5: Cross component ALF simplification [J. Li, C. S. Lim (Panasonic)]

This contribution proposes following simplification to cross-component ALF (CCALF):

  - Reduce number of signaled CCALF coefficients. Number of coefficents equals to 6, 7, 8 are tested separately.

  - Add a clipping process to refinement value of CCALF to reduce on-chip memory and also allows more flexibility to control subjective quality.

Coding impact of a is as below(with VTM6.0 as anchor):

Test a1 (CE5-2.1+ 6 coefficients without gain shift): 		All Intra:  0.10%, -6.60%, -4.83%

RA:	   0.16%, -9.00%, -8.22%

LDB:     0.13%, -11.98%, -8.24%

Test a2 (CE5-2.1+ 7 coefficients without gain shift): 		All Intra:  0.10%, -6.84%, -5.18%

RA:	   0.17%, -9.64%, -9.10%

LDB:     0.14%, -13.72%, -9.80%

Test a3 (CE5-2.1+ 8 coefficients without gain shift): 		All Intra:  0.11%, -7.13%, -5.42%

RA:	   0.16%, -9.75%, -9.40%

LDB:     0.16%, -13.90%, -10.05%

Coding impact of b is as below(with VTM6.0 as anchor):

Test b (CE5-2.1+ with clipping process): 				All Intra:  0.13%, -7.47%, -5.92%

RA:	   0.16%, -10.57%, -9.93%

LDB:     0.12%, -15.72%, -11.72%

Additional information(with VTM6.0 as anchor):

Test a1 (CE5-2.1+ 6 coefficients with gain shift): 		All Intra:  -1.02%, 1.92%, 3.89%

RA:	   -1.18%, 2.19%, 2.79%

LDB:     -0.55%, 0.57%, 5.31%

Test a2 (CE5-2.1+ 7 coefficients with gain shift): 		All Intra:  -1.02%, 1.68%, 3.51%

RA:	   -1.17%, 1.42%, 1.79%

LDB:     -0.50%, -1.32%, 3.53%

Test a3 (CE5-2.1+ 8 coefficients with gain shift): 		All Intra:  -1.01%, 1.32%, 3.24%

RA:	   -1.16%, 1.31%, 1.49%

LDB:     -0.55%, -1.32%, 3.12%

Two modifications to the CCALF are proposed.

- reduced filter shapes (6, 7, 8 coefficients). The proponent prefers the variant with 6 coefficients to save ccALF complexity by 57% and reduce the need for handling the virtual boundary handling for 4:2:0.

- use clipping process to reduce the chip memory to 31 percent.

The proponent mentions that in general, ccALF subjectively has some loss at high QP and gain at low QP. It was mentioned that if we have a CE on this topic, we can test this proposal in the CE.

Recommendation: Study in the CE if the CE exists.

JVET-P0852 Crosscheck of JVET-P0173 (AHG16/Non-CE5: Cross component ALF simplification) [K. Fan (??)]

JVET-P0178 CE5-related: Unification of CCALF and ALF [C.-W. Kuo, J. Li, C.S. Lim (Panasonic)]

No need to review

JVET-P0717 Crosscheck of JVET-P0178 (CE5-related: Unification of CCALF and ALF) [H.-J. Jhu (Kwai Inc.)]

JVET-P0251 CE5-related: Simplified CCALF [Y. Zhao, H. Yang (Huawei)]

This contribution simplifies the CCALF filter of CE5-2.1 from 14 filter coefficients to 6. It is reported that the 6-coefficient CCALF (C6-1) results in 0.11% / -6.80% / -5.34% BD-rates in AI configuration, and 0.17% / -9.29% / -8.70% BD-rates in RA configuration.

In version 2, a flag is coded in APS to indicate whether C6-1 or C6-2 filter is used for each of Cb and Cr. It is reported that this 2-type 6-coefficient CCALF (C6-3) results in 0.11% / -7.02% / -5.58% BD-rates in AI configuration, and 0.18% / -9.96% / -9.48% BD-rates in RA configuration.

The proponent suggests testing C6-2, which is a subset of the 8 coefficient filter and C6-3, which is a switching between C6-1 and C6-2. Switching is done per frame. The choice of the filter in C6-2 is done by the RDO. The proponent did not try to switch at the CTU level in CE6-2.

Recommendation: Test C6-2 in the CE.

JVET-P0849 Crosscheck of JVET-P0251 (CE5-related: Simplified CCALF with 6 filter coefficients) [C.-M. Tsai (MediaTek)]

JVET-P0892 Cross-check of JVET-P0251 (Simplified CCALF with 6 filter coefficients) [P. Onno (Canon)]

JVET-P0290 CE5-related: Clipping of intermediate value in CC-ALF [Y. Yasugi, T. Ikai (Sharp)]

No need to review

JVET-P0668 Crosscheck of JVET-P0290 (CE5-related: Clipping of intermediate value in CC-ALF) [A. Nalci (Qualcomm)]

JVET-P0330 Non-CE5: CC-ALF filtering simplification [J. Taquet, P. Onno, C. Gisquet, G. Laroche (Canon)]

No need to review

JVET-P0962 Crosscheck report of JVET-P0330 [K. Misra (Sharp Labs of America)]

JVET-P0331 Non-CE5: CC-ALF design simplification [J. Taquet, P. Onno, C. Gisquet, G. Laroche (Canon)]

No need to review

JVET-P0963 Crosscheck report of JVET-P0331 [K. Misra (Sharp Labs of America)]

JVET-P0372 CE5-related: Joint chroma cross-component adaptive loop filtering [H. Yang, Y. He, H. Li (InterDigital)]

Cross-Component Adaptive Loop Filter (CCALF) applies a linear diamond shaped filter on luma samples and uses the output signal to refine each chroma component. In the proposal, to reduce the complexity of CCALF, we propose joint chroma CCALF (JC-CCALF), where only one set of CCALF filter coefficients will be trained and used to generate one filtered output as the refinement signal, which will be added directly to the Cb component, and be properly weighted and then added to the Cr component. It reduces the complexity of existing CCALF roughly by half. Our JC-CCALF is implemented based on CCALF in CE5-2.1 (i.e. In-loop CCALF with chroma gain shifted to luma). Simulation result shows that compared to VTM-6.0, the proposed method yields RA: −0.99%/+0.62%/−0.22% Y/Cb/Cr Y BD-rate change, while the original CCALF yields −1.16%/+0.24%/−0.59%. Further changing the used ALF filter from the original 18-tap 14 unique coefficients filter to a 3x4 diamond shaped 8-tap filter yields RA: −0.99%/+1.61%/+0.48% Y/Cb/Cr BD-rate change.

The proponents have also tried the 8-tap filter to further reduce complexity.

The proponents would like to test an aspect of the using one filter for both Cb and Cr component.

Recommendation: test in the CE

JVET-P0704	Crosscheck of JVET-P0372 on joint chroma cross-component adaptive loop filtering [X. Li (Tencent)]

JVET-P0468 CE5-Related: Reducing multiplier count in CC-ALF [K. Misra, F. Bossen, A. Segall (Sharp Labs of America)]

No need to review

JVET-P0470 Non-CE5: Simplification of Cross Component Adaptive Loop Filter [X. Li, X. Zhao, L. Zhao, Y. Du, S. Liu (Tencent)]

There is a joint proposal 0666.

The bit-depth of multiplier is reduced to 4-bit by 8-bit with following changes.

- CC-ALF coefficients are constrained to 4 bits, i.e., in the range of (-8, 7), inclusive.

- Before feeding into CC-ALF, luma reconstructed samples are first shifted to 8-bit dynamic range with rounding.

In addition, as CC-ALF filter coefficients are within 4 bits, 4-bit fixed length coding is used instead of 3rd order exp-Golomb binarization uek(v). In fact, for 4-bit values, 4-bit fixed length coding is not only simpler, but also more efficient than 3rd order exp-Golomb binarization.

- Over VTM-6, BD rate changes are 0.12%, -7.35%, -5.84% for AI, 0.18%, -10.42%, -9.83% for RA, 0.13%, -15.70%, -11.91% for LDB.

- Over CE5-2.1, BD rate changes are -0.01%, 0.26%, 0.20% for AI, 0.02%, 0.30%, 0.28% for RA, -0.04%, -0.08%, -0.19% for LDB.

Two participants mentioned that there is a concern with constraining the input to 8 bits.

No action.

JVET-P0738 Crosscheck report of JVET-P0470 [K. Misra (Sharp Labs of America)]

JVET-P0543 CE5-related: Cleanup on cross-component adaptive loop filter [J. Nam, J. Choi, J. Lim, S. Kim (LGE)]

No need to review

JVET-P0893 Cross-check of JVET-P0543 (Cleanup on cross-component adaptive loop filter) [P. Onno (Canon)]

JVET-P0555 CE5-related: Dynamic range reduction for coefficients of cross component adaptive loop filter [N. Hu, J. Dong, V. Seregin, M. Karczewicz (Qualcomm)]

No need to review

JVET-P0850 Crosscheck of JVET-P0555 (CE5-related: Dynamic range reduction for coefficients of cross component adaptive loop filter) [C.-M. Tsai (MediaTek)]

JVET-P0556 CE5-related: Temporal buffer removal for cross component adaptive loop filter [N. Hu, J. Dong, V. Seregin, M. Karczewicz (Qualcomm)]

No need to review

JVET-P0941 Cross-check of JVET-P0556 (CE5-related: Temporal buffer removal for cross component adaptive loop filter) [S.-C. Lim, H. Lee, J. Lee, J. Kang (ETRI)	]

JVET-P0557 CE5-related: Multiplication removal for cross component adaptive loop filter [N. Hu, J. Dong, V. Seregin, M. Karczewicz (Qualcomm)]

In JVET-P0080, a cross component adaptive loop filter (CC-ALF) is tested to refine chroma components by using luma samples. Each filter has 14 filter coefficients and 18 taps. Every coefficient has an 8-bit dynamic range and is signalled with an exponential-Golomb code. To reduce the complexity of multiplication, in this proposal, multiplications are replaced by bit shifting. Under common test conditions, compared to CE5-2.1, similar overall coding efficiency and running time are observed.

## Method 1

CC-ALF filter coefficients are restricted to have only the following values: {-128, -64, -32, -16, -8, -4, -2, -1, 0, 1, 2, 4, 8, 16, 32, 64, 128}, which are signalled as {-8, -7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 7, 8} respectively before binarization in order to reduce the signalling overhead. Third order exponential-Golomb codes are used to signal the absolute values of the coefficients. Another sign bit is signalled for non-zero coefficients.

## Method 2

Method 1, where EG3 is replaced with EG1.

## Method 3

Filter coefficients are restricted to have values: {-64, -32, -16, -8, -4, -2, -1, 0, 1, 2, 4, 8, 16, 32, 64}, which are signalled as {-7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 7} respectively before binarization in order to reduce the signalling overhead. Fixed length codes of 3 bits are used to signal the absolute values of the coefficients. Another sign bit is signalled for non-zero coefficients.

## Method 4

Filter coefficients are restricted to have values: {-4, -2, -1, 0, 1, 2, 4}, which are signalled as {-3, -2, -1, 0, 1, 2, 3} respectively before binarization in order to reduce the signalling overhead. Fixed length codes of 2 bits are used to signal the absolute values of the coefficients. Another sign bit is signalled for non-zero coefficients.

Recommendation: test method 3 in the CE.

JVET-P0703 Crosscheck of JVET-P0557 on Multiplication removal for cross component adaptive loop filter [X. Li (Tencent)]

JVET-P0967 Crosscheck report of JVET-P0557 Method 2 [K. Misra (Sharp Labs of America)]

JVET-P0558 CE5-related: Reduced filter shape for cross component adaptive loop filter [N. Hu, J. Dong, V. Seregin, M. Karczewicz (Qualcomm)]

In JVET-P0080, a cross component adaptive loop filter (CC-ALF) is tested to refine chroma components by using luma sample values. Each filter has 14 filter coefficients and 18 taps as the figure below. However, this increases the complexity, especially in decoder, by introducing additional 12 8-bit multipliers, 1 9-bit multiplier and 1 10-bit multiplier per chroma pixel. In version 2, results are updated.

|  |  | f0 |  |  |
| --- | --- | --- | --- | --- |
|  | f1 | f2 | f3 |  |
| f4 | f5 | f6 | f7 | f4 |
| f4 | f8 | f9 | f10 | f4 |
|  | f11 | f12 | f13 |  |
|  |  | f0 |  |  |

It is proposed to use a reduced symmetrical filter shape with 13 coefficients and 13 taps is proposed.

|  |  | f0 |  |  |
| --- | --- | --- | --- | --- |
|  | f1 | f2 | f3 |  |
| f4 | f5 | f6 | f7 | f8 |
|  | f9 | f10 | f11 |  |
|  |  | f12 |  |  |

It was mentioned by a proponent that asymmetric filter may not work as well for other chroma subsampling types but that has not been tested. It was also mentioned that the asymmetric filter shape in the joint proposal 1008 can be reduced to the proposed symmetric filter shape.

There are 5 different variants of the proposal.

Proposal 5 is the combination of the filter shape with multiplication.

Recommend: test method 5 in the CE.

JVET-P0674 Crosscheck of JVET-P0558 (CE5-related: Reduced filter shape for cross component adaptive loop filter) [C.-Y. Lai (MediaTek)]

JVET-P0666 Non-CE5: Combination of JVET-P0470 and JVET-P0557 on simplification of CC-ALF [X. Li., X. Zhao, L. Zhao, X. Du, S. Liu (Tencent), N. Hu, J. Dong, V. Seregin, M. Karczewicz (Qualcomm)] [late]

This proposal provides a combination of JVET-P0470 and JVET-P0557 on simplification of cross-component ALF.

1. Using shifts instead of multiplications, limiting the coefficients range to [-8, 8].

Another aspect is limiting the input to 12 bits irrespective to the bit depth.

With the proposed method, the dynamic range of filter coefficients are constrained within 4-bit including 1-bit sign and shift instead of multiplication is used in filtering process so that 16-bit operation is kept for up to 12-bit internal bit-depth. Coding performance is reported as follows

- Over VTM-6, BD rate changes are 0.11%, -7.27%, -5.83% for AI, 0.17%, -10.38%, -9.86% for RA, 0.14%，-15.63%，-11.80% for LDB.

- Over CE5-2.1, BD rate changes are -0.02%, 0.35%, 0.21% for AI, 0.01%, 0.36%, 0.23% for RA, -0.03%， 0.04%， -0.05% for LDB.

Recommendation: Test the results with limiting the coefficient range to [-8, 8]. The proponents are advised to discuss offline what coefficient bit depth (range) to test in the CE.

JVET-P0709 Crosscheck of JVET-P0666 (Non-CE5: Combination of JVET-P0470 and JVET-P0557 on simplification of CC-ALF) [H. Yang (InterDigital)]

JVET-P0739 CE5-related: Combination of CCALF methods in JVET-P0165, JVET-P0556, and JVET-P0557 [O. Chubach, C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek), N. Hu, J. Dong, V. Seregin, M. Karczewicz (Qualcomm)] [late] [miss]

No need to review

JVET-P0949 Crosscheck of JVET-P0739 (CE5-related: Combination of CCALF methods in JVET-P0165, JVET-P0556, and JVET-P0557) [H. Liu (Bytedance)]

JVET-P1008 CE5-related: On the design of CC-ALF [K. Misra, F. Bossen, A. Segall (Sharp Labs of America), N. Hu, J. Dong, V. Seregin, M. Karczewicz (Qualcomm), P. Onno, C. Gisquet, G. Laroche (Canon), J. Li, C.S. Lim, C.-W. Kuo (Panasonic), J. Nam, J. Choi, J. Lim, S. Kim (LGE), O. Chubach, C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution proposes a design for the Cross Component Adaptive Loop Filter (CC-ALF). CC-ALF operates as part of the adaptive loop filter process and makes use of luma sample values to refine each chroma component. The tool is controlled by information in the bit-stream, and this information includes both (a) filter coefficients for each chroma component and (b) CTU level selection of the filter.  The filter coefficients are signalled in the APS, and the appropriate APS is referenced in the slice header. A maximum of four filters are allowed per chroma component, and each filter is a 3x4 diamond shaped filter containing 8 unique coefficients. Filter coefficient dynamic range is limited to 6-bit signed, i.e. [-32, 31]. Filtering along virtual boundary make use of symmetrical line selection to align with regular ALF filtering. The coding performance of the design is evaluated using the Common Test Conditions (CTC) as compared to VTM-6.0, and the YUV BD-Rate is reported to be -1.58%, -2.11%, -2.38% and -2.33% for the AI, RA, LDB, and LDP test conditions, where the BD-Rate is calculated using the combined YUV metric of AhG13. To limit the per-pixel multiplier to 16 (current ALF is 15), a restriction is incorporated that enables only one of either chroma ALF or CC-ALF for a chroma component of a CTU. The coding performance of the design is evaluated using the Common Test Conditions (CTC) as compared to VTM-6.0, and the YUV BD-Rate is reported to be -1.30%, X.XX%, X.XX% and X.XX% for the AI, RA, LDB, and LDP test conditions, where the BD-Rate is calculated using the combined YUV metric of AhG13.

Changes to CE5-2:

Proposed simplifications:

- Complexity reduction

  - Reduce the number of multiplies in the filter operation

  - Limit dynamic range of coefficient to 6-bits

  - Allow for sharing of multipliers with chroma ALF

- Alignment with ALF

  - Limit filter selection to signaling at a CTU level

  - Removal of the temporal layer coefficient buffers

  - Use of symmetric line selection at virtual boundary

Additionally, as the simplifications reduce coding efficiency, the authors also propose to use up to 4 filters per chroma component

- Disallow the use of both CC-ALF and chroma ALF in a CTB

- For each chroma CTB

  - Signal CC-ALF on/off flag

  - If CC-ALF enabled, chroma ALF is disabled

  - Otherwise, signal chroma ALF on/off flag

Benefit: Number of multiplications per pixel are reduced from 19 to 16. (15 in current ALF)

6 bits per tap are used

There was support from a non-proponent.

It was commented that it may be useful to put an additional flag to switch off the tool. It was mentioned that there is a slice level flag in the proposal.

Two options are presented:

Option 1 is where Chroma ALF and ccALF are simultaneously allowed

Option 2 is where they are not.

Results for Option 1 on RA are Y=0.17%, Cb = -11.53%, Cr -10.99%.

Results for Option 2 are incomplete.

The text is available. It was commented that someone should check the text.

It was agreed that the BoG recommends one of the two proposed options for adoption. Two non-proponents strongly prefer Option 1 to Option 2 since they do not see complexity reduction from option 2. One of the proponents prefers Option 2.

Recommendation: Adopt one of the proposed two options when more results are available. Further checking specification before the discussion in the track is encouraged.

On the third day of the BoG, one participant recommended testing all ccALF proposals in the CE before the adoption.

## Miscellaneous

| JVET number | Title | Tag |
| --- | --- | --- |
| JVET-P0163 | CE5-related: SAO encoder-only improvements |  |
| JVET-P0255 | CE5-related: Bilateral filter with SAO band offset |  |
| JVET-P0299 | CE5-related: Interaction between Bilateral Filter and Cross-Component Adaptive Loop Filter |  |
| JVET-P0333 | On SAO performance |  |
| JVET-P0504 | CE5-related: Harmonization of Hadamard filter, SAO and ALF |  |
| JVET-P0505 | Non-CE5: On non-linear ALF clipping values |  |
| JVET-P0570 | AHG10/Non-CE5: Performance of encoder-side deblocking optimization in VTM-6.0 |  |

JVET-P0163 CE5-related: SAO encoder-only improvements [C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, O. Chubach, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0820 Cross-check of JVET-P0163 (CE5-related: SAO encoder-only improvements) [K. Choi, W. Choi, K. P. Choi (Samsung)]

JVET-P0255 CE5-related: Bilateral filter with SAO band offset [K. Abe, T. Toma (Panasonic)]

This proposal removes SAO edge offset but keeps SAO band offset on top of bilateral filter.

Figure2 shows the diagram of this proposal. It is similar to CE5-3.1 but SAO EO is removed. The filtered samples are generated using the sum of both output offsets same as CE5-3.1. But SaoOffset is derived only from SAO BO processing. It is noted, that BIF is applied to Luma only, so the filtered samples of chroma are generated using only SAO BO.

IOUT=Clip3(IIN+BifOffset+SaoOffset)

Figure2: The diagram of proposed method

According to the above modification, the semantics of sao_type_idx_luma and sao_type_idx_chroma are changed as shown in Table2. Originally, these syntaxes have three values (none/SAO BO/SAO EO). But, in this proposal, they have only two values (none/SAO BO). Wherein, bif_ctu_flag will be switched on/off regardless of sao_type.

The objective results are similar to 5-3.1. There is some simplification due to removal of SAO edge offset.

No action is recommended.

JVET-P0868 Crosscheck of JVET-P0255: CE5-related: Bilateral filter with SAO band offset [J. Ström (Ericsson)]

JVET-P0299 CE5-related: Interaction between Bilateral Filter and Cross-Component Adaptive Loop Filter [C. Hollmann, J. Ström (Ericsson)]

The proponents ran a combination of the bilateral filter and ccALF. The results are almost perfectly additive gains of the bilateral and ccALF.

Random access has a performance on AI, RA, LDB

| -0.27% | -7.40% | -5.85% |
| --- | --- | --- |
| -0.27% | -10.40% | -9.80% |
| -0.16% | -15.40% | -11.31% |

No action since there is no action on BIF this mmeting

JVET-P0710 Cross check report of JVET-P0299 [K. Misra (Sharp Labs of America)]

JVET-P0333 On SAO performance [P. Onno, G. Laroche (Canon)]

This contribution reports the performance of SAO when the Adaptive Loop Filter is disabled. The test is performed under the Common Testing Conditions of VTM 6.0, where the anchor is the VTM 6.0 without ALF and SAO. The average Y/Cb/Cr BDR gains of the VTM 6.0 with SAO over the VTM-6.0 without both SAO and ALF are -0.37%/-0.83%/-1.24% for the AI configuration, -0.89%/-1.33%/-1.25% for the RA configuration, -1.06%/-2.67%/-4.25% for the Low Delay B configuration, and -2.74%/-4.08%/-5.43% for the Low Delay P configuration.

It was mentioned that these results have been obtained using a greedy version of the SAO encoder algorithm that is not the same as the one used in the CTC (slightly better gains).

JVET-P0504 CE5-related: Harmonization of Hadamard filter, SAO and ALF [V. Stepin, S. Ikonin, A. Karabutov (Huawei)]

This contribution reports results of the combination Hadamard transform domain filter HTDF (JVET-M0468) and SAO in-loop filter when ALF is turned on and off.

The simulations over VTM-6.0 show following results:

- -0.40%/-0.41/-0.20% of the luma BD-rate change with 107%/103%/102% of encoding time and 105%/101%/101% of decoding time for AI/RA/LDB configurations correspondingly when HTDF is on, SAO is on, ALF is on (sequential filter connection is used )

- 0.40%/-0.36/-0.09% of the luma BD-rate change with 107%/103%/102% of encoding time and 103%/101%/100% of decoding time for AI/RA/LDB configurations correspondingly when HTDF is on, SAO is off, ALF is on (sequential filter connection is used)

The simulations over VTM-6.0 with SAO off and ALF off show following results:

- -0.32%/-0.79%/-0.95% of the luma BD-rate change with 100%/100%/100% of encoding time and 104%/102%/102% of decoding time for AI/RA/LDB configurations correspondingly ,when, SAO is on, HTDF is off, ALF is off (sequential filter connection is used)

- -0.93%/-1.35%/-1.34% of the luma BD-rate change with 108%/104%/102% of encoding time and 107%/103%/102% of decoding time for AI/RA/LDB configurations correspondingly ,when SAO is off, HTDF is on, ALF is off (sequential filter connection is used)

- -1.02%/-1.64%/-1.57% of the luma BD-rate change with 108%/103%/102% of encoding time and 109%/104%/104% of decoding time for AI/RA/LDB configurations correspondingly ,when  SAO is on, HTDF is on, ALF is off (sequential filter connection is used)

- -0.99%/-1.50%/-1.48% of the luma BD-rate change with 109%/104%/102% of encoding time and 111%/107%/106% of decoding time for AI/RA/LDB configurations correspondingly ,when  SAO is on, HTDF is on, ALF is off (parallel SAO and HTDF filters connection is used, as in CE5-3.2)

Based on test result it is proposed to disable SAO when De-blocking, HTDF and ALF filter are used for in-loop filtering and enable SAO and HTDF filter when ALF filter is off.

Most of the results have been studied in the context of CE5, except the sequential application of bilateral and SAO.

One participant has mentioned that similar behaviour is seen for the bilateral filter, including the sequential application of the bilateral filter and SAO.

The proponent wants to draw attention to the fact that HDTF can be beneficial for the low-delay case.

No action was recommended.

JVET-P0412 CE5-related: On further modification of Hadamard filter from CE5-3.2 [S. Ikonin, V. Stepin, A. Karabutov, S. Nikolaeva (Huawei)]

This contribution reports results of modifications of test CE5-3.2 on combination of Hadamard and SAO filter related to encoder in-loop RDO decision fix, bugfix of SAO and Hadamard filter averaging stage, and fix of accessing uninitialized memory at encoder side at the block padding stage. The following results were obtained:

- -0.44%/-0.42%/-0.17%/-0.22% of the luma BD-rate change with 108%/104%/103%/104% of encoding time and 106%/104%/102%/102% of decoding time for AI/RA/LDB/LDP configurations correspondingly over VTM6.0 (corresponding to CE5-3.2.1 test)

- -0.67%/-0.72%/-0.53%/-0.69% of the luma BD-rate change with 108%/104%/103%/105% of encoding time and 108%/105%/104%/104% of decoding time for AI/RA/LDB/LDP configurations correspondingly over VTM6.0 with ALF disabled (corresponding to CE5-3.2.1-noALF test).

In addition, the effect of applying Hadamard filter to chroma components is reported.