| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P2025-v4 |
| --- | --- |

| Title: | Description of Core Experiment 5 (CE5): Cross Component Adaptative Loop Filtering |  |  |
| --- | --- | --- | --- |
| Status: | Output Document to JVET |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen Andrew Segall | Tel: Email: | chingyeh.chen@mediatek.com asegall@sharplabs.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

The goal of this Core Experiment (CE) is to improve and/or enhance cross component loop filtering (CC-ALF) techniques.  These CC-ALF techniques will be evaluated according to both subjective and objective metrics.

# Test conditions and evaluation criteria

As decided in the 16th JVET meeting, proposals in the CE will be evaluated relative to a “CE Anchor”.  The software for this anchor will correspond to VTM-7.0 combined with JVET-P1008 (Option 1), and this anchor will be provided as part of the core experiment process.  Furthermore, an evaluation of the CE Anchor compared to VTM-7.0 will also be performed.

Testing will be organized into sub-tests based on design aspects of the proposal.  The sub-tests will include:

- Alternative filter shapes and footprints

- Complexity reduction of the filtering process

Additionally, combinations of proposals in the different sub-tests will also be identified and studied.  Evaluation of the proposals will be performed with respect to subjective quality, objective quality and complexity.

# Test descriptions

| Test | Description | Source | Proponent(s) | Cross-checker(s) |
| --- | --- | --- | --- | --- |
| Common Base | Cross-component Adaptive Loop Filter as in JVET-P1008 (Option 1) | JVET-P1008 (Option 1) | K. Misra (Sharp) N. Hu (Qualcomm) P. Onno (Cannon) J. Li (Panasonic) J. Nam (LGE) O. Chubach (MediaTek) | H. Yang (InterDigital) |
| CE5-1 (Alternative Filter Shapes) |  |  |  |  |
| CE5-1.1 | 5x4 filter shape with 8 coefficients | JVET-P0106 | A. Kotra (Huawei) | K. Misra (Sharp) |
| CE5-1.2 | 3x2 filter shape with 6 coefficients | JVET-P0173 | J. Li (Panasonic) | N. Hu (Qualcomm) |
| CE5-1.3 | 3x4 filter shape with 6 coefficients | JVET-P0251 | Y. Zhao (Huawei) | H. Yang (InterDigital) |
| CE5-1.4 | Joint Cb/Cr CC-ALF with one additional weight and CTU level selection | JVET-P0372 | H. Yang (InterDigital) | Y. Zhao (Huawei) |
| CE5-1.5 | Joint Cb/Cr CC-ALF with one additional weight and block level selection | JVET-P0372 | H. Yang (InterDigital) | A. Kotra (Huawei) |
| CE5-2 (Complexity reduction of the filtering process) |  |  |  |  |
| CE5-2.1 | Multiplication removal with coefficient range in [-64, 64] | JVET-P0557 | N. Hu (Qualcomm) | J. Li (Panasonic) |
| CE5-2.2 | Multiplication removal with coefficient range in [-8, 8] | JVET-P0666 | N. Hu (Qualcomm) G. Li (Tencent) | P. Onno (Cannon) |
| CE5-3 (Combination Tests) |  |  |  |  |
| CE5-3.1 | CE5-2.1 + 5x5 filter shape with 13 coefficients | JVET-P0558 | N. Hu (Qualcomm) | K. Misra (Sharp) |
| CE5-3.2 | CE5-1.4 + CE5-2.1 Joint Cb/Cr CC-ALF with one additional weight and CTU-level selection combined with multiplication removal with coefficient range in [-64,64] | Proposed during CE editing period | H. Yang (InterDigital) N. Hu (Qualcomm) | G. Li (Tencent) |

# Detailed description of tests

## CE Anchor (JVET-P1008)

The anchor for the CE test is provided in JVET-P1008 (Option 1).  This method provides a CC-ALF design that has the following properties:

- Filter shape is a 3x4 diamond with 8 unique coefficients

- Filter coefficient dynamic range is limited to 6-bit signed

- Filter selection to performed at the CTU level with support for a maximum of 4 filters

- Symmetric line selection is used at virtual boundary (to better harmonize with ALF)

- Temporal layer coefficient buffers are not used (to better harmonize with the APS concept)

- Residual output is clipped to -2BitDepthC-1 to 2BitDepthC-1-1, inclusive.

An overview of the CC-ALF process is shown in Figure C-1.  As can be seen in the Figure, CC-ALF uses a linear filter to filter luma sample values and generate a residual correction for the chroma channels from the co-located filtered output.  An illustration of the filter shape is provided in Figure C-2.

Fig. C-1: Placement of CC-ALF with respect to other loop filters

Fig. C-2: Filter support in CE Anchor

Draft text for the anchor is also available in JVET-P1008.

## CE5-1: Alternative Filter Shapes

### CE5-1.1: Simplifications for Cross Component Adaptive Loop Filter (JVET-P0106)

The CC-ALF filter shape is modified to have only 8 coefficients as shown in the figure below. This modified filter shape is proposed to be tested with sequences containing different chroma type.

### CE5-1.2: Cross component ALF simplification (JVET-P0173)

To further reduce number of operations and remove virtual boundary handling of CCALF, 6 tap filter shape is proposed as below:

| f0 | f1 | f2 |
| --- | --- | --- |
| f3 | f4 | f5 |

### CE5-1.3: Simplified CCALF with 6 filter coefficients (JVET-P0251)

To further reduce the complexity and align number of multiplications of CC-ALF with chroma ALF, a six-tap filter was proposed, as shown in Fig. 5-1.3-1. The six taps are a sub-set of the eight taps of JVET-P1008, and each tap is associated with a unique filter coefficient.

Fig. 5-1.3-1 Six-tap CC-ALF.

### CE5-1.4: Joint chroma cross-component adaptive loop filtering (JVET-P0372)

Joint Chroma Cross-Component Adaptive Loop Filter (JC-CCALF) uses only one set of CCALF filter coefficients trained at the encoder to generate one filtered output as the refinement signal, which will be added directly to the Cb component, and be properly weighted and then added to the Cr component. In this test, filters are indicated at the CTU-level.

SAO Luma

ALF Luma

SAO Cb

SAO Cr

Joint chroma

CC ALF

ALF Cb

Y

Cb

Cr

w

ALF Cr

Fig. 5-1.4-1. JC-CCALF workflow.

### CE5-1.5: Joint chroma cross-component adaptive loop filtering (JVET-P0372)

Joint Chroma Cross-Component Adaptive Loop Filter (JC-CCALF) uses only one set of CCALF filter coefficients trained at the encoder to generate one filtered output as the refinement signal, which will be added directly to the Cb component, and be properly weighted and then added to the Cr component. In this test, filters are indicated with a block size, which is signaled per slice.

The supported such chroma block sizes range from the minimum chroma CTU size to the current chroma CTU size. The minimum chroma CTU size is the minimum between the smallest possible width and height of a chroma CTU, i.e. Min( 32 / SubWidthC, 32 / SubHeightC ), while the current chroma CTU size is the minimum between the width and height of the current chroma CTU, i.e. Min( CtbWidthC, CtbHeightC ). For example, if CTU size is set to the maximal 128x128, the JC-CCALF chroma block size of a slice will be one from 32x32, 64x64 and 128x128 for 4:4:4 video, or one from 16x16, 32x32 and 64x64 for 4:2:0 and 4:2:2 video.

## CE5-2: Complexity reduction in filtering process

### CE5-2.1: Multiplication removal for cross component adaptive loop filter (JVET-P0557)

Filter coefficients are restricted to have values: {-64, -32, -16, -8, -4, -2, -1, 0, 1, 2, 4, 8, 16, 32, 64}, which are signalled as {-7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 7} respectively before binarization in order to reduce the signalling overhead. Fixed length codes of 3 bits are used to signal the absolute values of the coefficients. Another sign bit is signalled for non-zero coefficients.

### CE5-2.2: Multiplication removal for cross component adaptive loop filter (JVET-P0666)

Filter coefficients are restricted to have values: {-8, -4, -2, -1, 0, 1, 2, 4, 8}. A bit is first signalled to indicate whether it is a zero coefficient. If not, a fixed length code of 2 bits is used to signal the absolute value of the coefficient followed by a sign bit.

## CE5-3: Combination Tests

### CE5-3.1: Combination of CE5-2.1 and 5x5 diamond filter used in chroma ALF (JVET-P0558)

Alternative filter shape with multiplication removal for CCALF

|  |  | f0 |  |  |
| --- | --- | --- | --- | --- |
|  | f1 | f2 | f3 |  |
| f4 | f5 | f6 | f7 | f8 |
|  | f9 | f10 | f11 |  |
|  |  | f12 |  |  |

The 5x5 diamond filter shape will be used for CCALF on top of the tests in CE5-2.1. Filter coefficients are restricted to have values: {-64, -32, -16, -8, -4, -2, -1, 0, 1, 2, 4, 8, 16, 32, 64}, which are signalled as {-7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 7} respectively before binarization in order to reduce the signalling overhead. Fixed length codes of 3 bits are used to signal the absolute values of the coefficients. Another sign bit is signalled for non-zero coefficients.

### CE5-3.2: Combination of CE5-1.4 and CE5-2.1

### Proposed during the CE editing period, the Joint Cb/Cr CC-ALF with one additional weight and CTU-level selection method in CE5-1.4 is combined the multiplication removal with coefficient range in [-64,64] method in CE5-2.1.

# Expected Gains

- Coding efficiency improvement

- Subjective picture quality preservation

# Evaluation Criteria

The CE Anchor will be compared against VTM-7.0, while all other tests will be compared against the CE Anchor.  These comparisons will consist of the following:

- BD-rate

- Subjective performance

- Complexity performance

More information about each of these comparisons is provided in the sub-sections below.

## BD-rate Comparison

BD-rate information shall be provided according to the JVET SDR Common Test Conditions [1].  For proposals to be tested with alternative chroma types, additional BD-rate information shall also be provided according to the JVET HDR Common Test Conditions [2].  The HDR information shall only be for Class H1, as it contains the alternative filter types.  The HDR information is required for CE5-1.1 and optional for the other tests.

## Subjective performance evaluation

Expert viewing at the Brussels meeting may be performed for all proposals in the CE.  The main purpose of the subjective evaluation is to determine if proposals provide degraded subjective performance.  The following sequences and QP points will be used in the subjective tests:

| Num | Name | Resolution | Fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| SDR_01 | FoodMarket | 3840x2160p | 60fps | 600 | RA | 42, 37, 32 |
| SDR_02 | Campfire | 3840x2160p | 30fps | 300 | RA | 42, 37, 32 |
| SDR_03 | Red Kayak* | 1920x1080p | 30fps | 300 | RA | 42, 37, 32 |
| SDR_04 | Cactus | 1920x1080p | 50fps | 500 | LDB | 42, 37, 32 |
| SDR_05 | KristenAndSara | 1280x720p | 60fps | 600 | LDP | 42, 37, 32 |

* First 300 frames of the RedKayak sequence shall be used.

Please note that the length of each sequence is 10 seconds, which means that the versions of some sequences encoded for the subjective test are longer than those used in the JVET SDR Common Test Conditions [1].

Bit-streams to be used for the subjective test and md5sums of the decoded sequence results shall be uploaded to the JVET ftp folder /ce/JVET-P2025_loop_filters according to the schedule in Section 6.  Proponents should also bring the decoded sequences to the Brussels meeting.  Anchors will be provided by volunteers.

## Complexity performance evaluation

Complexity information shall also be provided for each proposal, including:

- Decoding time versus the CE Anchor.  Decoding time shall be measured on the same computer without writing output pictures to disk

- Encoding time versus the CE Anchor.  Encoding times measured using cluster times are acceptable.

- Number of additional multiplications, shift, and addition per pixel for the tested CC-ALF process.

# Supplemental Information

Supplemental testing of sub-sets of the features in the tools above may be identified during the CE process.  If those results are available and cross-checked prior to T4 in the Time-line, the results shall be included in the CE report.

# Time-line and Responsibilities

T1: 2019-Nov-1:				   			Final CE description uploaded

T2: VTM7.0 release + 1 week:				CE Anchor is released

T2: VTM7.0 release + 3 weeks:	   			Proponents provide software and cross-checking begins

T3: 2019-Dec-17: 							Final version of proponent software is provided

T4: 2019-Dec-24: 							CE contribution documents including specification text and complete test results are uploaded to the JVET document repository. Sequences for the subjective test are uploaded to the FTP repository. (Regular document deadline – 1 week)

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010.

- A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-P2011.