| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0014-v1 |
| --- | --- |

| Title: | JVET AHG report: Lossless and near-lossless coding tools (AHG14) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | T. Nguyen and T.-C. Ma (co-chairs), M. Ikeda, H. Jang, X. Zhao (vice-chairs) | Email: | tung.nguyen@hhi.fraunhofer.de tsung-chuanma@kwai.com masaru.ikeda@sony.com hm.jang@lge.com xinzzhao@tencent.com |
| Source: | AHG14 on lossless and near-lossless coding tools |  |  |

_____________________________

# Abstract

This document reports the activity of AHG 14 on lossless and near-lossless coding tools between the 16th JVET meeting in Geneva and the 17th Meeting in Brussels.

# Introduction

The JVET established the AHG 14 at the Geneva meeting with the following mandates:

- Study coding tools for lossless and near-lossless coding, including transform skip, BDPCM, and other potential technologies

- Consider the interaction between coding tools and other processing such as loop filtering and LMCS for lossless and near-lossless coding

- Develop proposals for lossless and near-lossless coding for chroma and non-YCbCr colour space content

- Consider throughput bottlenecks for lossless and near-lossless coding at high resolutions and frame rates

# Activities

Discussions related to AHG14 used the JVET email reflector (jvet@lists.rwth-aachen.de), and the AHG chairs sent a kick-off message on 28th October 2019. No emails have been exchanged related to the AHG. The AHG chairs provided a software implementation (encoder only) that enables lossless capability in VTM-7.0. The performance of VTM-7.0 are as follows.

The results for HEVC RExt relative to HEVC Main/Main10 are as follows using HM-16.20 and CE3 test conditions.

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | HM-16.20 | HM-16.20 Rext |  | HM-16.20 | HM-16.20 Rext |  |
| Class A1 | 2,2 | 2,3 | -4,50% | 2,3 | 2,4 | -3,88% |
| Class A2 | 1,7 | 1,8 | -5,88% | 1,8 | 1,9 | -4,52% |
| Class B | 2,2 | 2,3 | -5,06% | 2,3 | 2,4 | -2,59% |
| Class C | 1,9 | 2,0 | -5,42% | 2,5 | 2,5 | -2,22% |
| Class D | 1,9 | 2,1 | -7,85% | 2,8 | 2,9 | -2,56% |
| Class E | 2,7 | 3,0 | -8,22% |  |  |  |
| Class F | 4,5 | 5,2 | -12,17% | 26,6 | 30,6 | -8,54% |
| TGM | 6,1 | 8,1 | -22,91% | 74,4 | 99,5 | -20,65% |
| Overall | 2,1 | 2,3 | -5,71% | 2,3 | 2,3 | -3,14% |
| Enc Time[%] | 95% |  |  | 105% |  |  |
| Dec Time[%] | 93% |  |  | 90% |  |  |

The results for VVC VTM-7.0 relative to HEVC Main/Main10 are as follows.

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | HM-16.20 | VTM-7.0 |  | HM-16.20 | VTM-7.0 |  |
| Class A1 | 2,2 | 2,2 | -0,24% | 2,3 | 2,2 | 1,49% |
| Class A2 | 1,7 | 1,6 | 5,96% | 1,8 | 1,7 | 5,45% |
| Class B | 2,2 | 2,2 | -0,30% | 2,3 | 2,3 | 0,11% |
| Class C | 1,9 | 1,9 | -0,36% | 2,5 | 2,4 | 1,55% |
| Class D | 1,9 | 1,9 | -0,82% | 2,8 | 2,8 | 1,20% |
| Class E | 2,7 | 2,8 | -2,18% |  |  |  |
| Class F | 4,5 | 5,3 | -13,28% | 26,6 | 33,7 | -10,55% |
| TGM | 6,1 | 11,8 | -44,31% | 74,4 | 107,1 | -30,87% |
| Overall | 2,1 | 2,1 | 0,43% | 2,3 | 2,2 | 1,84% |
| Enc Time[%] | 3133% |  |  | 1339% |  |  |
| Dec Time[%] | 172% |  |  | 136% |  |  |

The results for VVC VTM-7.0 relative to HEVC RExt are as follows.

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | HM-16.20 Rext | VTM-7.0 |  | HM-16.20 Rext | VTM-7.0 |  |
| Class A1 | 2,3 | 2,2 | 4,47% | 2,4 | 2,2 | 5,59% |
| Class A2 | 1,8 | 1,6 | 12,66% | 1,9 | 1,7 | 10,50% |
| Class B | 2,3 | 2,2 | 5,06% | 2,4 | 2,3 | 2,81% |
| Class C | 2,0 | 1,9 | 5,37% | 2,5 | 2,4 | 3,86% |
| Class D | 2,1 | 1,9 | 7,65% | 2,9 | 2,8 | 3,86% |
| Class E | 3,0 | 2,8 | 6,59% |  |  |  |
| Class F | 5,2 | 5,3 | -1,51% | 30,6 | 33,7 | -2,52% |
| TGM | 8,1 | 11,8 | -28,31% | 99,5 | 107,1 | -12,06% |
| Overall | 2,3 | 2,1 | 6,55% | 2,3 | 2,2 | 5,18% |
| Enc Time[%] | 3285% |  |  | 1270% |  |  |
| Dec Time[%] | 185% |  |  | 151% |  |  |

# Related contributions

The related input contributions can be discriminated into three groups:

- AHG14 related: high throughput and lossless operation mode

- CE3 related: low level design proposals further imporiving the lossless performance

- CE3 input documents: input documents for the tests conducted in CE3

## AHG14 related

JVET-Q0149: AHG14: Report of CABAC skip mode results on VTM-7.0

JVET-Q0387: AHG14: Throughput and coding efficiency report of JVET-P0300 on VTM-7.0

JVET-Q0141: Non-CE1/Non-CE3: Deblocking filter control for lossless blocks

## CE3 related

JVET-Q0056: CE3-related: Level prediction for transform skip samples

JVET-Q0107: CE3-related: Flag-free Lossless Coding in VVC

JVET-Q0108: CE3-related: On signaling overhead for low-level lossless flags in CE3-2.5

JVET-Q0109: CE3-related: On Last Position Signaling for Lossless Coding

JVET-Q0137: CE3-related: Rice Parameter Derivation with Unified Lookup Table

JVET-Q0139: CE3-related: Lookup Table Free Rice Parameter Derivation

JVET-Q0143: Non-CE3: Issue of Level mapping in transform skip residual

JVET-Q0144: Non-CE3: Contexts in Chroma transform skip residual coding

JVET-Q0145: CE3-related: Unified rice parameter derivation of transfrom skip residual

JVET-Q0186: CE3-related: TB-level residual coding selection for lossless coding

JVET-Q0187: CE3-related: Rice parameter derivation in residual coding

JVET-Q0263: CE3-related: Bit-exact simplification of Rice parameter selection for regular residual coding

JVET-Q0264: CE3-related: Computation reduction in the derivation of Rice parameter for abs_remainder[ ] in regular residual coding

JVET-Q0269: CE3-related: On transform skip residual coding

JVET-Q0294: CE3-related: CTU level local lossless coding of VVC

JVET-Q0323: Non-CE3: Encoder optimization for chroma BDPCM

JVET-Q0347: CE3-related: Fixed rice parameters in transform residual coding

JVET-Q0439: CE3 Related: Modified Rice Parameter derivation

JVET-Q0460: Non-CE3: Inter BDPCM for lossless video coding

JVET-Q0462: CE3-related: Modified transform skip residual coding for lossless coding

JVET-Q0463: CE3-related: Inter BDPCM with RRC for lossless video coding

JVET-Q0489: CE3-related: Modified TS residual coding

JVET-Q0490: CE3-related: Simplification of rice parameter derivation

JVET-Q0498: Non-CE3: TB based Rice parameter selection for transform skip residual coding

JVET-Q0561: CE3-related: Transquant Bypass Mode for Lossless Coding

## CE3 input documents

JVET-Q0068: CE3-1.3: Using RRC for lossless coding without state transition

JVET-Q0069: CE3-1.4: On residual scanning order for lossless coding

JVET-Q0070: CE3-2.5: Residual coding selection signaling for lossless coding

JVET-Q0071: CE3-2.10: Luma BDPCM for lossless coding with RRC and residual rotation

JVET-Q0072: CE3-2.11: Luma and Chroma BDPCM for lossless coding with RRC and residual rotation

JVET-Q0080: CE3-1.5: Rice parameter derivation of transform skip residual coding

JVET-Q0081: CE3-2.6: Luma BDPCM and IBC with TSRC for lossless coding

JVET-Q0082: CE3-2.7: Luma and chroma BDPCM and IBC with TSRC for lossless coding

JVET-Q0086: CE3-2.1 and CE3-2.2: Luma/Chroma BDPCM for Lossless Coding with Transform Skip Residual Coding

JVET-Q0088: CE3-2.3: Luma BDPCM for Lossless Coding with Regular Residual Coding

JVET-Q0089: CE3-2.4: Luma and Chroma BDPCM for Lossless Coding with Regular Residual Coding

JVET-Q0092: CE3: Modified Rice Parameter Derivation for Residual Coding (CE3-1.1, CE3-1.2, CE3-2.8 and CE3-2.9)

# Recommendations

The AHG recommends:

- To review all related contributions

- To continue the investigation on lossless and near-lossless performance of VVC

  - for high throughput

  - for chroma 4:4:4 chroma format