| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0016-v1 |
| --- | --- |

| Title: | JVET AHG report: Implementation studies (AHG16) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | M. Zhou, J. An, E. Chai, K. Choi, S. Sethuraman, T. Hsieh, X. Xiu | Email: | minhua.zhou@broadcom.com jicheng.ajc@alibaba-inc.com eric.chai@ublnx.com kiho14.choi@samsung.com sriram.sethuraman@ittiam.com thsieh@qti.qualcomm.com xiaoyuxiu@kwai.com |
| Source: | AHG16 |  |  |

_____________________________

# Abstract

This document summarizes the activity of AHG16: Implementation studies, between the 16th JVET meeting in Geneva, CH (1–11 October 2019) and the 17th JVET meeting in Brussels, BE (7–17 January 2020).

The AHG was established with the following three mandates:

- Study draft and proposed coding tools to identify implementation issues relating to decoder pipelines, decoder throughput, and other aspects of implementation difficulty.

- Solicit hardware analysis of complex tools.

- Provide feedback on potential solutions to address identified issues.

# Activities

Email discussions were held on the main JVET email reflector (jvet@lists.rwth-aachen.de) with an [AHG16] indication on message headers. A summary of the AHG activities is provided as follows:

## Topics discussed on the email reflector

- Entry point offset signalling

- In the current design the signalling of entry point offsets is optional for tiles.

- It was commented that without entry point offsets decoder implementation of raster scan order processing is impossible for single slice/multi-tile streams.

- It was suggested that if the signaling of entry point offset in ES is optional the system level spec definition of entry point offsets be made mandatory.

- It was commented without mandating tiles parallel decoding is not guaranteed even if the entry point offsets are made mandatory, unless the parsing and pixel processing are fully decoupled (in which case, however, decoder implementation of raster scan order processing is possible without entry point offsets).

- It was further commented that even if entry point offsets are made mandatory the parallel parsing/decoding may still be not be guaranteed because of bit-rate unbalancing among tiles.

## Feedback provided on tools being tested in CEs

- CE4 GEO (Geometric partitions)

  - The JVET-P0884/ JVET-P0885 (a combination of JVET-P0107/JVET-P0264/JVET-P0304) version of GEO removes the original GEO elements of partial transform and deriving the motion mask from the blending mask, which addresses the majority of implementation concerns.

  - The newly proposed slope-based GEO (JVET-P0107/JVET-P0264) removes the multiplications from the blending mask/motion mask derivation and provides a second option of on-the-fly derivation. This simplifies the design and provides more implementation flexibility. There is no big difference between those two options in terms of implementation complexity. Compared to other designs such as the one tested in CE4-1.14, this kind of simplification does not seem to be critical if it hurts coding efficiency (visual quality).

  - The newly proposed idea of deriving the blending/motion mask of different sizes by cropping directly from the pre-stored masks of the largest sizes would be the simplest because only (offsetX, offsetY) needs to be computed at CU level. However, this is not practical for implementation as it is too costly due to the large table size (still needs about ~300 KB for 80 modes according to JVET-P0304). Not sure whether giving up on this would help improve coding efficiency and/or simplify the design.

  - It is desirable to reduce the number of GEO partitions and supported PU sizes. In particular, PU sizes of 64x128/128x64/128x128 should be disabled.  JVET-P0663 and JVET-P0107 have some good ideas that are worth trying out. Unlike the angular intra prediction in which a subset of the modes supported by a less capable encoder still provides decent gain (the angular intra prediction has increased from 9 modes in AVC to 33 modes in HEVC, and to 65 modes in VVC), the GEO design of JVET-P0884/JVET-P0885 does not seem to be quality scalable on the encoder side. Using the JVET-P0884 code and supporting the first 16 modes out of 82 modes on the encoder side led to 0.19% loss in RA as opposed to -0.22% RA gain if all the 82 modes would be supported. It would be more practical to support a reasonable amount of partitions and PU sizes. Otherwise, the GEO becomes just an off-line coding tool, but imposes unnecessary burden on the testing and verification of decoder designs. As a comparison, the wedge mode has 16 partitions and supports 9 PU sizes in an existing standard.

  - The last round of subjective testing is disappointing in many senses. Nevertheless, it might be worth testing it again to make sure that the proposed technology at least does not hurt visual quality.

- CE5 Cross-Component ALF (CCALF)

  - Recommended adding the combination tests of the CE anchor with the joint Cb/Cr CC-ALF and the multiplication removal, to see whether further reduction of buffering and multiplication costs is possible without compromising coding efficiency.

- CE3 Lossless mode

  - In terms of signalling, lots of tests seem to assume that a picture or a slice is either entirely coded in lossy or in lossless mode, but the real use case might be that only portion of a picture/slice is coded in lossless mode.

    - Examples include: TSRC/RRC switch in CE3-1.3, CE3-2.3/2.4, CE3-2.5.3, residual rotation in CE3-1.4 and lossless IBC blocks using RRC in CE3-2.6/2.7.

    - Signalling using high-level flags to control those switches may not be sufficient in mixed lossy/lossless coding environment.

  - Also, the high-level signalling used in CE3-2.5.1/CE3-2.5.2 (TU/CU level TSRC/RRC switch) may not be that relevant.

## Additional comments posted on the email reflector

- With the adoption of JVET-P1001, the actual chroma QPs (instead of QPs derived from luma QPs) are now used for chroma de-blocking, including those for not-coded TUs. However, in the current design the chroma QP derivation for not-coded TUs is not defined. The spec is broken in this regard and needs to be fixed.

- In the current spec text alf_chroma_num_alt_filters_minus1 is signalled using ue(v) but no maximum value for this element is defined, which leads to unbounded ALF APS size.  In the adopted test of JVET-O0090 variant CE5-4.2 and in VTM7.0 the maximum number of alternative chroma ALF filter sets is set to 8. (Note: the spec text is since fixed)

- The adoption of JVET-P1026 has opened up the possibility of signaling non-zero MTS coefficients outside the top-left 16x16 TU region. The current design of checking last_sig_coeff_pos <= (15, 15) does not guarantee at syntax level that all coefficients outside 16x16 are zero. The temporary fix is to impose a bitstream restriction, but a much more preferred solution is to prevent it at syntax level.

- At the last meeting the chroma transform skip was adopted to support lossless mode, but the combination of LFNST and chroma TS was not disabled in the syntax table. Due to the adoption of JVET-P0365 which can disable scaling lists for LFNST, a decoder does not know what kind of inverse quantization and inverse transform to apply before the lfnst_idx is decoded, which is signaled at the end of CU after luma/chroma transform coefficients. Allowing the combination of chroma TS and LFNST doubles transform coefficient buffering and processing latency.

## Memory bandwidth study for VTM7.0

Broadcom conducted a memory bandwidth study by running both the VTM7.0 and VTM6.0 for CTC with a commercial motion compensation cache model integrated. The summary results of the random access configuration are provided in the Table 1 (for informational purposes).

For class F, the peak memory bandwidth consumption (see MBW_diff column) has roughly 2% increases for non-obvious reasons.

|  | Random Access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |  |  |  |
|  | Y | U | V | TCM_diff | ABW_diff | MBW_diff | EncT | DecT |
| Class A1 | 0.00% | -3.16% | -2.57% | -0.17% | -0.14% | -0.64% | 97% | 96% |
| Class A2 | 0.26% | -1.39% | -1.61% | 0.22% | 0.28% | 0.94% | 102% | 99% |
| Class B | 0.00% | -2.65% | -2.09% | 0.20% | 0.21% | 0.40% | 95% | 92% |
| Class C | 0.08% | -2.90% | -2.66% | 0.22% | 0.20% | -0.24% | 98% | 96% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.07% | -2.56% | -2.24% | 0.12% | 0.14% | 0.12% | 98% | 95% |
| Class D | 0.17% | -1.76% | -1.81% | 0.36% | 0.39% | -1.05% | 92% | 104% |
| Class F (optional) | 0.51% | -1.59% | -1.24% | 1.77% | 2.40% | 1.85% | 101% | 103% |

Table 1. Memory bandwidth comparison (VTM7.0 vs. VTM6.0, RA)

Where

- TCM_diff : Total cache misses (over all the frames coded), percentage difference relative to VTM6.0.

- ABW_diff: Average memory bandwidth (over all the frames coded), percentage difference relative to VTM6.0.

- MBW_diff : Worst case (Max) memory bandwidth (among all the frames coded), percentage difference relative to VTM6.0.

MBW_diff is the most important measure, which shows the worst case memory bandwidth consumption difference of VTM7.0 relative to VTM6.0.

## Bin to bit ratio study for VTM7.0

JVET-Q0102 reports that the bin to bit ratio remains unchanged from VTM6.0 to VTM7.0. As far as the low-QP (QP =2, 7, 12, 17) AI configuration is concerned, the weighted and un-weighted bin to bit ratio of the VTM7.0 is roughly 18% and 10% higher than that of HM16.19, respectively.

In the weighted bin to bit ratio, a bypass bin is counted at 0.25 context coded bins; in the unweight bin to bit ratio, a bypass bin and a context coded bin carry an equal weight (1:1).

# Related contributions

The following contributions are identified for the AHG.

## CABAC bin to bit ratio (2)

- JVET-Q0102, “AHG16: A study of bin to bit ratio for VTM7.0”, M. Zhou (Broadcom)

- JVET-Q0436, “CABAC zero word threshold”, 	A. Browne, K. Sharman, S. Keating (Sony)

## Entry point offset signalling (2)

- JVET-Q0151, “AHG12: On entry point offset signalling”, M. Coban, Y.-J. Chang, V. Seregin, A. K. Ramasubramonian, M. Karczewicz (Qualcomm)

- JVET-Q0205, “AHG12: On the presence of entry point signaling”, Hendry, S. Kim, S. Lee (LGE)

## Chroma QP offset signalling (2)

- JVET-Q0267, “AHG16: On Propagation of Chroma CU QP Offsets”, B. Heng, M. Zhou, W. Wan (Broadcom)

- JVET-Q0476, “On chroma QP offsets for zero-CBF leading chroma coding blocks”, A. K. Ramasubramonian, B. Ray, G. Van der Auwera, M. Karczewicz (Qualcomm)

## Maximum number of Chroma ALF filter sets (1)

- JVET-Q0378, “Non-CE5: On the number of ALF Chroma filters”, P. Onno, G. Laroche, N. Ouedraogo (Canon)

## Prevention of non-zero MTS coefficients in “zero-out” region (5)

- JVET-Q0055, “On MTS index signaling”, J. Lainema (Nokia)

- JVET-Q0057, “Coefficient group based restriction on MTS signaling”, M. Coban, M. Karczewicz, H.E. Egilmez, V. Seregin (Qualcomm)

- JVET-Q0136, “Alignment of MTS index signalling condition with MTS zero-out”, M. Koo, M. Salehifar, J. Lim, S. Kim (LGE)

- JVET-Q0430, “AHG16: Syntax based MTS zero out”, F. Le LÃ©annec, K. Naser, F. Galpin, P. Delagrange (InterDigital)

- JVET-Q0448, “MTS dependent coefficient subblock scanning for zero-out”, S.  De-Luxán-Hernández,T. Nguyen,B. Bross,H. Schwarz,D. Marpe,T. Wiegand (HHI)

## Combination of LFNST with Chroma transform skip (9)

- JVET-Q0099, “On Interaction of LFNST and Transform Skip”, T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)

- JVET-Q0103, “LFNST Signaling for Chroma based on Chroma Transform Skip Flags”, H.E. Egilmez, A. Nalci, M. Coban, V. Seregin, M. Karczewicz (Qualcomm)

- JVET-Q0106, “AHG16: Combination of LFNST with transform skip”, B. Heng, T. Hellman, M. Zhou, W. Wan (Broadcom)

- JVET-Q0126, “Initializations and propagation of Chroma QP Offset”, K. Kawamura, S. Naito (KDDI)

- JVET-Q0138, “Separate transform skip checking of Luma and Chroma for LFNST index signaling”, M. Koo, M. Salehifar, J. Lim, S. Kim (LGE)

- JVET-Q0193, “LFNST signalling cleanup with TS checking”, M.-S. Chiang, C.-W. Hsu, C.-M. Tsai, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)

- JVET-Q0328, “On LFNST signalling and transform skip”, C. Rosewarne, J. Gan (Canon)

- JVET-Q0499, “On LFNST signaling with Transform-skip mode”, T.-C. Ma, X. Xiu, Y.-W. Chen, H.-J. Jhu, X. Wang (Kwai Inc.)

- JVET-Q0529, “On LFNST index and MTS index signaling”, Z.-Y. Lin, M.-S. Chiang, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)

## Parallel merge and merge estimation region (3)

- JVET-Q0185, “AHG16: On merge estimation region for VVC”,	Y.-L. Hsiao, C.-C. Chen, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek), H. Huang, W.-J. Chien, T. Hsieh, V. Seregin, C.-C. Chen, K. Reuze, M. Karczewicz (Qualcomm)

- JVET-Q0297, “AHG16: Merge estimation region with constrain in HMVP update”, H. Huang, W.-J. Chien, T. Hsieh, V. Seregin, C.-C. Chen, K. Reuze, M. Karczewicz (Qualcomm)

- JVET-Q0356, “AHG16: Parallel Merge Estimation for VVC”,	S. Esenlik, H. Gao, B. Wang, A. M. Kotra, E. Alshina (Huawei), Y.-L. Hsiao, C.-C. Chen, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)

## VVC decoder implementation (2)

- JVET-Q0211, “AHG16: VVC software decoder and performance analysis”, S. Gudumasu, S. Bandyopadhyay, A. Srivastava, Y. He, Y. He (InterDigital)

- JVET-Q0386, “AHG16: Feature-rich implementation of VVC real-time decoding and playback on ARM based mobile clients”,	J.Arumugam, S.Kotecha, S.Ramamurthy (Ittiam)

## Other AHG16-related contributions (4)

- JVET-Q0495, “AHG16: Simplified clip ranges for NL-ALF”, Y.-W. Chen, X. Xiu, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)

- JVET-Q0496, “AHG16: On motion shift derivation of SbTMVP	“, Y.-W. Chen, X. Xiu, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)

- JVET-Q0500, “AHG16: On derivation of CCLM predictors”, Y.-W. Chen, X. Xiu, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)

- JVET-Q0513, “AHG16: Clipping residual samples for JCCR”, X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai)

# Recommendations

The AHG recommends reviewing the input contributions.