| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0023-v3 |
| --- | --- |

| Title: | CE3: Summary Report on Lossless Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | T. Nguyen T.-C. Ma A. Nalci | Tel: Email: | tung.nguyen@hhi.fraunhofer.de tsung-chuanma@kwai.com analci@qti.qualcomm.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Abstract

The CE report summarizes the test results and crosscheck reports for CE3 on lossless coding. The CE includes two sub-CEs on the following topics:

- CE3-1:	Regular and TS residual coding (RRC, TSRC) for lossless coding, and modifications to RRC and TSRC for lossless and lossy operation modes

- CE3-2:	Enabling BDPCM and high-level techniques for lossless coding, and the combination of BDPCM with different RRC/TSRC techniques

Changes in v2:

- add excel sheets for the test and crosscheck results

- incorporate crosscheck results into the document

- list the according input proposals for each test

Changes in v3:

- add RExt results as test candidate

# Introduction

The purpose of this core experiment is to explore the coding efficiency and complexity impact of proposed algorithms for lossless coding and their impact on, if any, lossy operation modes.

# Core Experiment Results

## CE3-1:	Regular and TS residual coding (RRC, TSRC) for lossless coding, and modifications to RRC and TSRC for lossless and lossy operation modes

|  | Tester | Tool | Cross checker |
| --- | --- | --- | --- |
| CE3-1.1* | H. Wang (Qualcomm) | JVET-P0559: Modified RRC and TSRC for lossless - modification to RRC as discussed in JVET-P0559-m1 - modification to rice parameter as discussed in JVET-P0559-m1, without the normalization - modification to TSRC as discussed in JVET-P0559-m2 - similar modification to TSRC as 3 without normalization | T. Nguyen (HHI) ST 1 & 2 M. Sarwer (Alibaba) ST 2 & 3 |
| CE3-1.2* | H. Wang (Qualcomm) | JVET-P1028: Modified TSRC for lossless | Z.-Y. Lin (MediaTek) |
| CE3-1.3 | T.-C. Ma (Kwai) Z.-Y. Lin (MediaTek) A. Nalci (Qualcomm) M. Sarwer (Alibaba) | JVET-P0148/JVET-P0258: Using RRC for lossless coding without state transition | T. Tsukuba (Sony) |
| CE3-1.4 | T. -C. Ma (Kwai) | JVET-P0528: On residual scanning order for lossless coding | J. Choi (LGE) |
| CE3-1.5* | M. Sarwer (Alibaba) | JVET-P0463/JVET-P0072: Rice parameter derivation of TSRC for lossless | T.-C Ma (Kwai) |

Remarks (TN): CE3-1.3 uploaded two different configurations, whereas the CE3 description only consists of a single test. The performance difference between the two tests is the same for natural content. The only difference is in screen content where it is decided on picture level whether or not TSRC is used, depending on an existing algorithm in the encoder (hashThreshold, value increase from 20 to 40).

### Lossless

| Overall | All Intra (AI) |  |  |  |  |  | Random Access (RA) |  |  |  |  |  | Low Delay B (LB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CR-A | CR-T | BRS | Enc | Dec | CR-A | CR-T | BRS | Enc | Dec | CR-A | CR-T | BRS | Enc | Dec |
| RExt | 2.1 | 2.3 | -6.04% | 3% | 54% | 2.2 | 2.3 | -4.82% | 8% | 66% | 2.6 | 2.7 | -4.24% | 9% | 70% |
| CE3-1.1a | 2.1 | 2.3 | -5.75% | 96% | 104% | 2.2 | 2.4 | -5.88% | 101% | 101% | 2.6 | 2.7 | -4.98% | 97% | 105% |
| CE3-1.1b | 2.1 | 2.2 | -5.38% | 93% | 105% | 2.2 | 2.3 | -5.64% | 98% | 100% | 2.6 | 2.7 | -4.98% | 97% | 105% |
| CE3-1.1c | 2.1 | 2.2 | -5.09% | 104% | 105% | 2.2 | 2.3 | -3.49% | 105% | 104% | 2.6 | 2.6 | -2.48% | 103% | 105% |
| CE3-1.1d | 2.1 | 2.2 | -4.51% | 101% | 104% | 2.2 | 2.3 | -3.01% | 103% | 104% | 2.6 | 2.6 | -2.31% | 102% | 106% |
| CE3-1.2 | 2.1 | 2.2 | -4.31% | 93% | 103% | 2.2 | 2.3 | -2.94% | 100% | 101% | 2.6 | 2.6 | -2.29% | 100% | 103% |
| CE3-1.3 | 2.1 | 2.2 | -5.23% | 93% | 102% | 2.2 | 2.3 | -5.58% | 97% | 97% | 2.6 | 2.7 | -4.97% | 98% | 106% |
| CE3-1.4 | 2.1 | 2.3 | -5.63% | 98% | 105% | 2.2 | 2.4 | -5.72% | 97% | 100% | 2.6 | 2.7 | -5.09% | 100% | 110% |
| CE3-1.5 | 2.1 | 2.2 | -2.52% | 98% | 104% | 2.2 | 2.2 | -0.82% | 99% | 107% | 2.6 | 2.6 | -0.91% | 101% | 108% |
| Class F | All Intra (AI) |  |  |  |  |  | Random Access (RA) |  |  |  |  |  | Low Delay B (LB) |  |  |  |  |  |
|  | CR-A | CR-T | BRS | Enc | Dec | CR-A | CR-T | BRS | Enc | Dec | CR-A | CR-T | BRS | Enc | Dec |
| CE3-1.1a | 5.3 | 5.4 | -1.72% |  |  | 33.7 | 34.0 | -2.64% |  |  | 50.7 | 50.4 | -2.27% |  |  |
| CE3-1.1b | 5.3 | 5.3 | -1.53% |  |  | 33.7 | 33.9 | -2.50% |  |  | 50.7 | 50.2 | -2.19% |  |  |
| CE3-1.1c | 5.3 | 5.6 | -4.21% |  |  | 33.7 | 35.0 | -3.30% |  |  | 50.7 | 52.4 | -3.06% |  |  |
| CE3-1.1d | 5.3 | 5.5 | -3.94% |  |  | 33.7 | 34.9 | -3.14% |  |  | 50.7 | 52.3 | -2.94% |  |  |
| CE3-1.2 | 5.3 | 5.5 | -3.67% |  |  | 33.7 | 34.8 | -2.97% |  |  | 50.7 | 52.1 | -2.83% |  |  |
| CE3-1.3 | 5.3 | 5.3 | -1.33% |  |  | 33.7 | 33.8 | -2.35% |  |  | 50.7 | 50.8 | -2.36% |  |  |
| CE3-1.4 | 5.3 | 5.4 | -1.86% |  |  | 33.7 | 33.8 | -2.45% |  |  | 50.7 | 50.8 | -2.40% |  |  |
| CE7-1.5 | 5.3 | 5.5 | -2.67% |  |  | 33.7 | 34.5 | -2.38% |  |  | 50.7 | 51.8 | -2.28% |  |  |
| TGM | All Intra (AI) |  |  |  |  |  | Random Access (RA) |  |  |  |  |  | Low Delay B (LB) |  |  |  |  |  |
|  | CR-A | CR-T | BRS | Enc | Dec | CR-A | CR-T | BRS | Enc | Dec | CR-A | CR-T | BRS | Enc | Dec |
| CE3-1.1a | 11.8 | 11.8 | -0.17% |  |  | 107.1 | 107.3 | -0.46% |  |  | 124.9 | 122.9 | 0.68% |  |  |
| CE3-1.1b | 11.8 | 11.6 | 1.10% |  |  | 107.1 | 106.4 | 0.36% |  |  | 124.9 | 122.9 | 0.97% |  |  |
| CE3-1.1c | 11.8 | 12.5 | -5.09% |  |  | 107.1 | 116.7 | -6.75% |  |  | 124.9 | 137.0 | -6.89% |  |  |
| CE3-1.1d | 11.8 | 12.4 | -4.53% |  |  | 107.1 | 116.2 | -6.37% |  |  | 124.9 | 136.6 | -6.63% |  |  |
| CE3-1.2 | 11.8 | 12.3 | -3.79% |  |  | 107.1 | 115.7 | -5.96% |  |  | 124.9 | 136.0 | -6.16% |  |  |
| CE3-1.3 | 11.8 | 11.8 | 0.00% |  |  | 107.1 | 107.1 | 0.00% |  |  | 124.9 | 124.9 | 0.00% |  |  |
| CE3-1.4 | 11.8 | 11.8 | 0.00% |  |  | 107.1 | 107.1 | 0.00% |  |  | 124.9 | 124.9 | 0.00% |  |  |
| CE3-1.5 | 11.8 | 12.3 | -3.40% |  |  | 107.1 | 114.8 | -5.36% |  |  | 124.9 | 135.0 | -5.67% |  |  |

## Low QP

| Overall |  | All Intra (AI) |  |  |  |  |  |  |  |  |  | Random Access (RA) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Low Delay B (LB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | Cb |  | Cr |  | Enc |  | Dec | Y |  |  | Cb |  |  |  | Cr |  |  | Enc |  |  | Dec |  | Y |  |  | Cb |  | Cr |  |  | Enc |  |  | Dec |  |
| CE3-1.1a | -0.10% |  | -0.11% |  | -0.11% |  | 102% |  | 100% | 0.01% |  |  | -0.01% |  |  |  | -0.01% |  |  | 101% |  |  | 100% |  | 0.01% |  |  | -0.02% |  | 0.01% |  |  | 101% |  |  | 102% |  |
| CE3-1.1b | -0.03% |  | -0.04% |  | -0.05% |  | 100% |  | 99% | -0.01% |  |  | -0.01% |  |  |  | -0.02% |  |  | 100% |  |  | 100% |  | 0.00% |  |  | 0.00% |  | 0.00% |  |  | 101% |  |  | 100% |  |
| CE3-1.1c | -0.08% |  | -0.10% |  | -0.13% |  | 100% |  | 100% | -0.03% |  |  | -0.02% |  |  |  | -0.03% |  |  | 100% |  |  | 100% |  | -0.03% |  |  | -0.01% |  | -0.03% |  |  | 100% |  |  | 100% |  |
| CE3-1.1d | -0.07% |  | -0.10% |  | -0.12% |  | 100% |  | 100% | -0.02% |  |  | -0.03% |  |  |  | -0.04% |  |  | 100% |  |  | 100% |  | -0.02% |  |  | -0.01% |  | -0.02% |  |  | 101% |  |  | 101% |  |
| CE3-1.2 | -0.07% |  | -0.10% |  | -0.12% |  | 100% |  | 101% | -0.02% |  |  | -0.03% |  |  |  | -0.04% |  |  | 100% |  |  | 100% |  | -0.03% |  |  | -0.01% |  | -0.02% |  |  | 100% |  |  | 100% |  |
| CE3-1.5 | -0.06% |  | -0.10% |  | -0.12% |  | 99% |  | 99% | -0.02% |  |  | -0.03% |  |  |  | -0.04% |  |  | 99% |  |  | 100% |  | -0.02% |  |  | -0.02% |  | -0.02% |  |  | 100% |  |  | 101% |  |
| Class F |  | All Intra (AI) |  |  |  |  |  |  |  |  |  | Random Access (RA) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Low Delay B (LB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  | Cb |  | Cr |  | Enc |  | Dec |  | Y |  |  | Cb |  |  | Cr |  |  | Enc |  |  | Dec |  | Y |  |  | Cb |  | Cr |  |  | Enc |  |  | Dec |  |
| CE3-1.1a | -0.14% |  | -0.19% |  | -0.19% |  | 102% |  | 99% |  | -0.16% |  |  | 0.00% |  |  | 0.07% |  |  | 102% |  |  | 101% |  | -0.04% |  |  | -0.01% |  | -0.13% |  |  | 101% |  |  | 100% |  |
| CE3-1.1b | -0.02% |  | -0.05% |  | -0.04% |  | 102% |  | 101% |  | -0.07% |  |  | -0.04% |  |  | -0.08% |  |  | 102% |  |  | 103% |  | 0.04% |  |  | 0.03% |  | -0.05% |  |  | 100% |  |  | 101% |  |
| CE3-1.1c | -1.55% |  | -0.98% |  | -0.98% |  | 103% |  | 101% |  | -1.93% |  |  | -0.81% |  |  | -0.80% |  |  | 102% |  |  | 103% |  | -0.64% |  |  | -0.63% |  | -0.75% |  |  | 100% |  |  | 100% |  |
| CE3-1.1d | -1.44% |  | -0.84% |  | -0.83% |  | 102% |  | 100% |  | -1.92% |  |  | -0.92% |  |  | -0.95% |  |  | 102% |  |  | 102% |  | -0.61% |  |  | -0.51% |  | -0.72% |  |  | 100% |  |  | 98% |  |
| CE3-1.2 | -1.34% |  | -0.81% |  | -0.76% |  | 102% |  | 99% |  | -1.76% |  |  | -0.77% |  |  | -0.78% |  |  | 102% |  |  | 103% |  | -0.56% |  |  | -0.56% |  | -0.68% |  |  | 100% |  |  | 99% |  |
| CE3-1.5 | -1.18% |  | -0.77% |  | -0.73% |  | 100% |  | 100% |  | -1.52% |  |  | -0.70% |  |  | -0.80% |  |  | 100% |  |  | 98% |  | -0.53% |  |  | -0.46% |  | -0.65% |  |  | 100% |  |  | 101% |  |
| TGM |  | All Intra (AI) |  |  |  |  |  |  |  |  |  | Random Access (RA) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Low Delay B (LB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  | Cb |  | Cr |  | Enc |  | Dec | Y |  |  | Cb |  |  | Cr |  |  | Enc |  |  | Dec |  | Y |  |  | Cb |  | Cr |  |  | Enc |  |  | Dec |  |  |
| CE3-1.1a | -0.24% |  | -0.29% |  | -0.28% |  | 102% |  | 103% | 0.37% |  |  | -0.17% |  |  | -0.15% |  |  | 102% |  |  | 100% |  | 0.08% |  |  | -0.18% |  | -0.22% |  |  | 101% |  |  | 99% |  |  |
| CE3-1.1b | -0.01% |  | -0.08% |  | -0.07% |  | 101% |  | 101% | 0.19% |  |  | -0.12% |  |  | -0.10% |  |  | 101% |  |  | 99% |  | -0.10% |  |  | -0.17% |  | -0.24% |  |  | 101% |  |  | 100% |  |  |
| CE3-1.1c | -1.83% |  | -1.64% |  | -1.67% |  | 103% |  | 100% | -2.43% |  |  | -1.51% |  |  | -1.48% |  |  | 101% |  |  | 98% |  | -1.51% |  |  | -1.14% |  | -1.17% |  |  | 103% |  |  | 101% |  |  |
| CE3-1.1d | -1.54% |  | -1.39% |  | -1.40% |  | 103% |  | 99% | -2.30% |  |  | -1.45% |  |  | -1.40% |  |  | 102% |  |  | 100% |  | -1.38% |  |  | -0.93% |  | -0.93% |  |  | 103% |  |  | 104% |  |  |
| CE3-1.2 | -1.35% |  | -1.21% |  | -1.22% |  | 102% |  | 99% | -1.94% |  |  | -1.13% |  |  | -1.15% |  |  | 101% |  |  | 99% |  | -1.18% |  |  | -0.89% |  | -0.87% |  |  | 102% |  |  | 101% |  |  |
| CE3-1.5 | -1.35% |  | -1.22% |  | -1.20% |  | 101% |  | 97% | -1.85% |  |  | -1.01% |  |  | -1.03% |  |  | 101% |  |  | 100% |  | -1.20% |  |  | -0.95% |  | -0.94% |  |  | 101% |  |  | 101% |  |  |

### CTC

| Overall | All Intra (AI) |  |  |  |  |  |  |  |  |  | Random Access (RA) |  |  |  |  |  |  |  |  | Low Delay B (LB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | Cb |  | Cr |  | Enc |  | Dec | Y |  |  | Cb |  | Cr |  | Enc |  | Dec | Y |  | Cb | Cr |  | Enc |  | Dec |  |
| CE3-1.1a | 0.00% |  | -0.01% |  | 0.01% |  | 101% |  | 100% | -0.01% |  |  | -0.02% |  | 0.07% |  | 101% |  | 100% | 0.00% |  | -0.01% | -0.17% |  | 101% |  | 98% |  |
| CE3-1.1b | 0.00% |  | 0.00% |  | 0.00% |  | 100% |  | 102% | 0.00% |  |  | -0.01% |  | 0.00% |  | 100% |  | 100% | 0.00% |  | 0.00% | 0.00% |  | 101% |  | 100% |  |
| CE3-1.1c | -0.01% |  | 0.02% |  | -0.02% |  | 100% |  | 100% | -0.01% |  |  | -0.04% |  | 0.04% |  | 100% |  | 100% | -0.02% |  | -0.06% | -0.25% |  | 100% |  | 100% |  |
| CE3-1.1d | -0.01% |  | 0.00% |  | -0.01% |  | 101% |  | 102% | -0.01% |  |  | -0.06% |  | 0.01% |  | 100% |  | 100% | -0.02% |  | -0.08% | -0.24% |  | 100% |  | 99% |  |
| CE3-1.2 | -0.01% |  | 0.00% |  | -0.01% |  | 100% |  | 100% | -0.01% |  |  | -0.07% |  | 0.01% |  | 100% |  | 100% | -0.02% |  | -0.08% | -0.24% |  | 100% |  | 100% |  |
| CE3-1.5 | -0.01% |  | 0.00% |  | -0.02% |  | 96% |  | 97% | 0.00% |  |  | -0.01% |  | 0.02% |  | 98% |  | 98% | -0.03% |  | -0.03% | -0.12% |  | 97% |  | 96% |  |
| Class F | All Intra (AI) |  |  |  |  |  |  |  |  |  | Random Access (RA) |  |  |  |  |  |  |  |  | Low Delay B (LB) |  |  |  |  |  |  |  |  |  |
|  | Y | Cb |  | Cr |  | Enc |  | Dec |  |  | Y | Cb |  | Cr |  | Enc |  | Dec |  | Y | Cb |  |  | Cr |  | Enc |  | Dec |
| CE3-1.1a | -0.03% |  | -0.01% |  | 0.12% |  | 101% |  | 100% | 0.01% |  |  | 0.03% |  | -0.10% |  | 101% |  | 100% | -0.01% |  | -0.54% | 0.18% |  | 100% |  | 100% |  |
| CE3-1.1b | 0.00% |  | 0.00% |  | 0.00% |  | 102% |  | 101% | 0.00% |  |  | 0.01% |  | 0.01% |  | 101% |  | 100% | 0.00% |  | 0.02% | 0.01% |  | 101% |  | 100% |  |
| CE3-1.1c | -0.14% |  | -0.21% |  | -0.08% |  | 102% |  | 101% | -0.15% |  |  | -0.09% |  | -0.09% |  | 101% |  | 100% | -0.14% |  | -0.28% | 0.53% |  | 100% |  | 98% |  |
| CE3-1.1d | -0.18% |  | -0.21% |  | -0.06% |  | 102% |  | 102% | -0.18% |  |  | -0.02% |  | -0.12% |  | 101% |  | 100% | -0.14% |  | -0.27% | 0.46% |  | 100% |  | 98% |  |
| CE3-1.2 | -0.18% |  | -0.21% |  | -0.06% |  | 102% |  | 100% | -0.18% |  |  | -0.01% |  | -0.12% |  | 101% |  | 100% | -0.14% |  | -0.30% | 0.44% |  | 100% |  | 101% |  |
| CE3-1.5 | -0.15% |  | -0.19% |  | 0.02% |  | 101% |  | 102% | -0.08% |  |  | 0.00% |  | -0.04% |  | 101% |  | 102% | -0.07% |  | -0.28% | 0.05% |  | 102% |  | 103% |  |
| TGM | All Intra (AI) |  |  |  |  |  |  |  |  |  | Random Access (RA) |  |  |  |  |  |  |  |  | Low Delay B (LB) |  |  |  |  |  |  |  |  |  |
|  | Y |  | Cb |  | Cr |  | Enc |  | Dec | Y |  |  | Cb |  | Cr |  | Enc |  | Dec | Y |  | Cb | Cr |  | Enc |  | Dec |  |
| CE3-1.1a | -0.03% |  | -0.01% |  | -0.02% |  | 102% |  | 104% | 0.04% |  |  | 0.01% |  | 0.04% |  | 101% |  | 100% | -0.03% |  | -0.14% | -0.07% |  | 101% |  | 103% |  |
| CE3-1.1b | 0.00% |  | 0.00% |  | 0.00% |  | 102% |  | 104% | 0.00% |  |  | 0.00% |  | 0.00% |  | 101% |  | 100% | 0.00% |  | -0.01% | 0.00% |  | 101% |  | 100% |  |
| CE3-1.1c | -0.39% |  | -0.31% |  | -0.33% |  | 102% |  | 103% | -0.22% |  |  | -0.23% |  | -0.19% |  | 101% |  | 100% | -0.18% |  | -0.28% | -0.30% |  | 102% |  | 107% |  |
| CE3-1.1d | -0.40% |  | -0.33% |  | -0.37% |  | 102% |  | 104% | -0.27% |  |  | -0.21% |  | -0.18% |  | 101% |  | 101% | -0.26% |  | -0.27% | -0.18% |  | 101% |  | 101% |  |
| CE3-1.2 | -0.40% |  | -0.33% |  | -0.37% |  | 101% |  | 102% | -0.27% |  |  | -0.21% |  | -0.18% |  | 101% |  | 100% | -0.26% |  | -0.27% | -0.18% |  | 101% |  | 98% |  |
| CE3-1.5 | -0.37% |  | -0.27% |  | -0.30% |  | 101% |  | 102% | -0.17% |  |  | -0.20% |  | -0.17% |  | 101% |  | 102% | -0.25% |  | -0.24% | -0.30% |  | 102% |  | 104% |  |

## CE3-2:	Enabling BDPCM and high-level techniques for lossless coding. and the combination of BDPCM with different RRC/TSRC techniques

|  | Tester | Tool | Cross checker |
| --- | --- | --- | --- |
| CE3-2.1 | A. Nalci (Qualcomm) J. Choi (LGE) | JVET-P0082: Enabling Luma BDPCM for lossless coding (using TSRC) | T. Tsukuba (Sony) |
| CE3-2.2 | A. Nalci (Qualcomm) Z.-Y. Lin (MediaTek) J. Choi (LGE) | JVET-P0082: Luma and Chroma BDPCM for lossless coding (using TSRC) | T. Tsukuba (Sony) |
| CE3-2.3 | A. Nalci (Qualcomm) Z.-Y. Lin (MediaTek) M. Sarwer (Alibaba) H. Jang (LGE) T. Tsukuba (Sony) M. G. Sarwer (Alibaba) | JVET-P0082/JVET-P0463/JVET-P0525/JVET-P0574/ JVET-P0584: Luma BDPCM for lossless coding with RRC (no changes to RRC). | T. Nguyen (HHI) |
| CE3-2.4 | A. Nalci (Qualcomm) Z.-Y. Lin (MediaTek) T. Tsukuba (Sony) T.-C. Ma (Kwai) | JVET-P0082/JVET-P0525/JVET-P0584: Luma and Chroma BDPCM for lossless coding with RRC (no changes to RRC) | T. Poirier (InterDigital) |
| CE3-2.5* | T.-C. Ma (Kwai) Z.-Y. Lin (MediaTek) A. Nalci (Qualcomm) | JVET-P0525/JVET-P0082: Residual coding selection signaling for lossless coding - CU level signaling - TB level signaling | M. Sarwer (Alibaba) |
| CE3-2.6 | J. Choi (LGE) | JVET-P0466: Luma BDPCM and IBC with TSRC for lossless coding | T. Nguyen (HHI) |
| CE3-2.7 | J. Choi (LGE) A. Nalci (Qualcomm) Z.-Y. Lin (MediaTek) | JVET-P0466: Luma and chroma BDPCM and IBC with TSRC for lossless coding | T.-C. Ma (Kwai) |
| CE3-2.8* | H. Wang (Qualcomm) | Modified RRC* and TSRC* from CE3-1.1 + BDPCM luma/chroma (4 subtests in total) | X. Wang (Kwai) ST 1 & 2 M. Sarwer (Alibaba) ST 3 & 4 |
| CE3-2.9* | H. Wang (Qualcomm) | Modified TSRC* from CE3-1.2 + BDPCM luma/chroma | Z.-Y. Lin (MediaTek) |
| CE3-2.10 | T.C. Ma (Kwai) A. Nalci (Qualcomm) | Luma BDPCM for lossless coding with RRC and residual rotation | H. Wang (Qualcomm) |
| CE3-2.11 | T.C. Ma (Kwai) A. Nalci (Qualcomm) | Luma and Chroma BDPCM for lossless coding with RRC and residual rotation | H. Wang (Qualcomm) |

| Overall | All Intra (AI) |  |  |  |  |  |  |  |  |  |  |  | Random Access (RA) |  |  |  |  |  |  |  |  |  |  |  |  | Low Delay B (LB) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CR-A |  | CR-T |  | BRS |  |  | Enc |  |  | Dec |  | CR-A |  | CR-T |  |  | BRS |  |  | Enc |  |  | Dec |  | CR-A |  | CR-T |  |  | BRS |  | Enc |  | Dec |  |
| RExt | 2.1 |  | 2.3 |  | -6.04% |  |  | 3% |  |  | 54% |  | 2.2 |  | 2.3 |  |  | -4.82% |  |  | 8% |  |  | 66% |  | 2.6 |  | 2.7 |  |  | -4.24% |  | 9% |  | 70% |  |
| CE3-2.1 | 2.1 |  | 2.2 |  | -1.85% |  |  | 110% |  |  | 94% |  | 2.2 |  | 2.2 |  |  | -0.47% |  |  | 104% |  |  | 98% |  | 2.6 |  | 2.6 |  |  | -0.73% |  | 105% |  | 100% |  |
| CE3-2.2 | 2.1 |  | 2.2 |  | -2.73% |  |  | 115% |  |  | 88% |  | 2.2 |  | 2.2 |  |  | -0.76% |  |  | 106% |  |  | 98% |  | 2.6 |  | 2.6 |  |  | -1.01% |  | 105% |  | 97% |  |
| CE3-2.3 | 2.1 |  | 2.3 |  | -6.76% |  |  | 103% |  |  | 97% |  | 2.2 |  | 2.4 |  |  | -5.96% |  |  | 103% |  |  | 99% |  | 2.6 |  | 2.7 |  |  | -5.54% |  | 101% |  | 108% |  |
| CE3-2.4 | 2.1 |  | 2.3 |  | -7.70% |  |  | 106% |  |  | 92% |  | 2.2 |  | 2.4 |  |  | -6.21% |  |  | 104% |  |  | 97% |  | 2.6 |  | 2.7 |  |  | -5.84% |  | 102% |  | 104% |  |
| CE3-2.5a | 2.1 |  | 2.2 |  | -5.22% |  |  | 96% |  |  | 104% |  | 2.2 |  | 2.3 |  |  | -5.58% |  |  | 99% |  |  | 98% |  | 2.6 |  | 2.7 |  |  | -4.96% |  | 99% |  | 105% |  |
| CE3-2.5b | 2.1 |  | 2.2 |  | -5.21% |  |  | 95% |  |  | 102% |  | 2.2 |  | 2.3 |  |  | -5.57% |  |  | 99% |  |  | 96% |  | 2.6 |  | 2.7 |  |  | -4.95% |  | 100% |  | 106% |  |
| CE3-2.5c | 2.1 |  | 2.2 |  | -5.22% |  |  | 94% |  |  | 104% |  | 2.2 |  | 2.3 |  |  | -5.58% |  |  | 100% |  |  | 100% |  | 2.6 |  | 2.7 |  |  | -4.96% |  | 98% |  | 107% |  |
| CE3-2.6 | 2.1 |  | 2.3 |  | -6.25% |  |  | 103% |  |  | 97% |  | 2.2 |  | 2.4 |  |  | -5.85% |  |  | 102% |  |  | 100% |  | 2.6 |  | 2.7 |  |  | -5.29% |  | 100% |  | 100% |  |
| CE3-2.7 | 2.1 |  | 2.3 |  | -6.75% |  |  | 104% |  |  | 94% |  | 2.2 |  | 2.4 |  |  | -5.95% |  |  | 105% |  |  | 97% |  | 2.6 |  | 2.7 |  |  | -5.42% |  | 102% |  | 98% |  |
| CE3-2.8a-1 | 2.1 |  | 2.3 |  | -7.26% |  |  | 107% |  |  | 97% |  | 2.2 |  | 2.4 |  |  | -6.26% |  |  | 105% |  |  | 101% |  | 2.6 |  | 2.7 |  |  | -5.56% |  | 102% |  | 107% |  |
| CE3-2.8a-2 | 2.1 |  | 2.3 |  | -8.21% |  |  | 110% |  |  | 93% |  | 2.2 |  | 2.4 |  |  | -6.52% |  |  | 107% |  |  | 98% |  | 2.6 |  | 2.7 |  |  | -5.87% |  | 104% |  | 103% |  |
| CE3-2.8b-1 | 2.1 |  | 2.3 |  | -6.91% |  |  | 104% |  |  | 96% |  | 2.2 |  | 2.4 |  |  | -6.02% |  |  | 102% |  |  | 99% |  | 2.6 |  | 2.7 |  |  | -5.55% |  | 101% |  | 107% |  |
| CE3-2.8b-2 | 2.1 |  | 2.3 |  | -7.85% |  |  | 107% |  |  | 92% |  | 2.2 |  | 2.4 |  |  | -6.27% |  |  | 103% |  |  | 97% |  | 2.6 |  | 2.7 |  |  | -5.85% |  | 103% |  | 103% |  |
| CE3-2.8c-1 | 2.1 |  | 2.3 |  | -6.25% |  |  | 114% |  |  | 98% |  | 2.2 |  | 2.3 |  |  | -3.80% |  |  | 109% |  |  | 103% |  | 2.6 |  | 2.6 |  |  | -3.03% |  | 109% |  | 105% |  |
| CE3-2.8c-2 | 2.1 |  | 2.3 |  | -6.80% |  |  | 120% |  |  | 95% |  | 2.2 |  | 2.3 |  |  | -3.96% |  |  | 111% |  |  | 101% |  | 2.6 |  | 2.7 |  |  | -3.18% |  | 109% |  | 102% |  |
| CE3-2.8d-1 | 2.1 |  | 2.3 |  | -5.68% |  |  | 111% |  |  | 100% |  | 2.2 |  | 2.3 |  |  | -3.32% |  |  | 106% |  |  | 101% |  | 2.6 |  | 2.6 |  |  | -2.86% |  | 106% |  | 104% |  |
| CE3-2.8d-2 | 2.1 |  | 2.3 |  | -6.25% |  |  | 115% |  |  | 95% |  | 2.2 |  | 2.3 |  |  | -3.49% |  |  | 108% |  |  | 100% |  | 2.6 |  | 2.6 |  |  | -3.01% |  | 107% |  | 102% |  |
| CE3-2.9-1 | 2.1 |  | 2.3 |  | -5.53% |  |  | 109% |  |  | 98% |  | 2.2 |  | 2.3 |  |  | -3.26% |  |  | 104% |  |  | 100% |  | 2.6 |  | 2.6 |  |  | -2.84% |  | 104% |  | 103% |  |
| CE3-2.9-2 | 2.1 |  | 2.3 |  | -6.10% |  |  | 113% |  |  | 94% |  | 2.2 |  | 2.3 |  |  | -3.43% |  |  | 106% |  |  | 99% |  | 2.6 |  | 2.6 |  |  | -2.99% |  | 105% |  | 100% |  |
| CE3-2.10 | 2.1 |  | 2.3 |  | -7.01% |  |  | 105% |  |  | 96% |  | 2.2 |  | 2.4 |  |  | -6.06% |  |  | 102% |  |  | 99% |  | 2.6 |  | 2.7 |  |  | -5.62% |  | 104% |  | 108% |  |
| CE3-2.11 | 2.1 |  | 2.3 |  | -6.89% |  |  | 116% |  |  | 94% |  | 2.2 |  | 2.4 |  |  | -6.03% |  |  | 106% |  |  | 99% |  | 2.6 |  | 2.7 |  |  | -5.49% |  | 104% |  | 104% |  |
| Class F |  | All Intra (AI) |  |  |  |  |  |  |  |  |  |  |  |  |  | Random Access (RA) |  |  |  |  |  |  |  |  |  |  |  |  | Low Delay B (LB) |  |  |  |  |  |  |  |  |  |  |  |
|  | CR-A |  | CR-T |  |  | BRS |  |  | Enc |  |  | Dec | CR-A |  |  | CR-T |  |  | BRS |  |  | Enc |  |  | Dec | CR-A |  |  | CR-T |  |  | BRS |  | Enc |  | Dec |
| CE3-2.1 | 5.3 |  | 5.6 |  |  | -3.87% |  |  |  |  |  |  | 33.7 |  |  | 34.6 |  |  | -1.59% |  |  |  |  |  |  | 50.7 |  |  | 51.6 |  |  | -1.31% |  |  |  |  |
| CE3-2.2 | 5.3 |  | 5.7 |  |  | -5.42% |  |  |  |  |  |  | 33.7 |  |  | 34.9 |  |  | -2.24% |  |  |  |  |  |  | 50.7 |  |  | 51.9 |  |  | -1.85% |  |  |  |  |
| CE3-2.3 | 5.3 |  | 5.7 |  |  | -5.74% |  |  |  |  |  |  | 33.7 |  |  | 34.9 |  |  | -4.37% |  |  |  |  |  |  | 50.7 |  |  | 51.2 |  |  | -3.70% |  |  |  |  |
| CE3-2.4 | 5.3 |  | 5.8 |  |  | -7.60% |  |  |  |  |  |  | 33.7 |  |  | 35.3 |  |  | -5.22% |  |  |  |  |  |  | 50.7 |  |  | 51.6 |  |  | -4.33% |  |  |  |  |
| CE3-2.5a | 5.3 |  | 5.4 |  |  | -2.43% |  |  |  |  |  |  | 33.7 |  |  | 34.6 |  |  | -3.49% |  |  |  |  |  |  | 50.7 |  |  | 51.7 |  |  | -3.40% |  |  |  |  |
| CE3-2.5b | 5.3 |  | 5.4 |  |  | -2.43% |  |  |  |  |  |  | 33.7 |  |  | 34.6 |  |  | -3.45% |  |  |  |  |  |  | 50.7 |  |  | 51.5 |  |  | -3.20% |  |  |  |  |
| CE3-2.5c | 5.3 |  | 5.4 |  |  | -2.49% |  |  |  |  |  |  | 33.7 |  |  | 34.7 |  |  | -3.64% |  |  |  |  |  |  | 50.7 |  |  | 51.8 |  |  | -3.50% |  |  |  |  |
| CE3-2.6 | 5.3 |  | 5.6 |  |  | -5.40% |  |  |  |  |  |  | 33.7 |  |  | 34.7 |  |  | -4.20% |  |  |  |  |  |  | 50.7 |  |  | 51.1 |  |  | -3.60% |  |  |  |  |
| CE3-2.7 | 5.3 |  | 5.8 |  |  | -7.15% |  |  |  |  |  |  | 33.7 |  |  | 35.2 |  |  | -5.00% |  |  |  |  |  |  | 50.7 |  |  | 51.5 |  |  | -4.16% |  |  |  |  |
| CE3-2.8a-1 | 5.3 |  | 5.7 |  |  | -6.07% |  |  |  |  |  |  | 33.7 |  |  | 35.0 |  |  | -4.58% |  |  |  |  |  |  | 50.7 |  |  | 51.4 |  |  | -3.82% |  |  |  |  |
| CE3-2.8a-2 | 5.3 |  | 5.8 |  |  | -7.95% |  |  |  |  |  |  | 33.7 |  |  | 35.5 |  |  | -5.43% |  |  |  |  |  |  | 50.7 |  |  | 51.8 |  |  | -4.46% |  |  |  |  |
| CE3-2.8b-1 | 5.3 |  | 5.7 |  |  | -5.84% |  |  |  |  |  |  | 33.7 |  |  | 34.9 |  |  | -4.44% |  |  |  |  |  |  | 50.7 |  |  | 51.3 |  |  | -3.76% |  |  |  |  |
| CE3-2.8b-2 | 5.3 |  | 5.8 |  |  | -7.71% |  |  |  |  |  |  | 33.7 |  |  | 35.4 |  |  | -5.30% |  |  |  |  |  |  | 50.7 |  |  | 51.7 |  |  | -4.37% |  |  |  |  |
| CE3-2.8c-1 | 5.3 |  | 5.8 |  |  | -7.43% |  |  |  |  |  |  | 33.7 |  |  | 35.7 |  |  | -4.62% |  |  |  |  |  |  | 50.7 |  |  | 53.1 |  |  | -4.26% |  |  |  |  |
| CE3-2.8c-2 | 5.3 |  | 5.9 |  |  | -8.68% |  |  |  |  |  |  | 33.7 |  |  | 36.0 |  |  | -5.16% |  |  |  |  |  |  | 50.7 |  |  | 53.4 |  |  | -4.61% |  |  |  |  |
| CE3-2.8d-1 | 5.3 |  | 5.8 |  |  | -7.19% |  |  |  |  |  |  | 33.7 |  |  | 35.6 |  |  | -4.47% |  |  |  |  |  |  | 50.7 |  |  | 53.0 |  |  | -4.13% |  |  |  |  |
| CE3-2.8d-2 | 5.3 |  | 5.9 |  |  | -8.43% |  |  |  |  |  |  | 33.7 |  |  | 35.9 |  |  | -5.01% |  |  |  |  |  |  | 50.7 |  |  | 53.1 |  |  | -4.41% |  |  |  |  |
| CE3-2.9-1 | 5.3 |  | 5.8 |  |  | -6.98% |  |  |  |  |  |  | 33.7 |  |  | 35.5 |  |  | -4.34% |  |  |  |  |  |  | 50.7 |  |  | 52.7 |  |  | -3.96% |  |  |  |  |
| CE3-2.9-2 | 5.3 |  | 5.9 |  |  | -8.24% |  |  |  |  |  |  | 33.7 |  |  | 35.8 |  |  | -4.89% |  |  |  |  |  |  | 50.7 |  |  | 53.0 |  |  | -4.27% |  |  |  |  |
| CE3-2.10 | 5.3 |  | 5.7 |  |  | -6.28% |  |  |  |  |  |  | 33.7 |  |  | 35.0 |  |  | -4.56% |  |  |  |  |  |  | 50.7 |  |  | 51.3 |  |  | -3.86% |  |  |  |  |
| CE3-2.11 | 5.3 |  | 5.8 |  |  | -7.26% |  |  |  |  |  |  | 33.7 |  |  | 35.1 |  |  | -4.97% |  |  |  |  |  |  | 50.7 |  |  | 51.5 |  |  | -4.19% |  |  |  |  |
| TGM |  | All Intra (AI) |  |  |  |  |  |  |  |  |  |  |  |  |  | Random Access (RA) |  |  |  |  |  |  |  |  |  |  |  |  | Low Delay B (LB) |  |  |  |  |  |  |  |  |  |  |  |
|  | CR-A |  | CR-T |  | BRS |  |  | Enc |  |  | Dec |  | CR-A |  | CR-T |  |  | BRS |  |  | Enc |  |  | Dec |  | CR-A |  | CR-T |  |  | BRS |  | Enc |  | Dec |  |
| CE3-2.1 | 11.8 |  | 12.1 |  | -2.21% |  |  |  |  |  |  |  | 107.1 |  | 108.1 |  |  | -0.96% |  |  |  |  |  |  |  | 124.9 |  | 126.2 |  |  | -0.81% |  |  |  |  |  |
| CE3-2.2 | 11.8 |  | 12.5 |  | -4.99% |  |  |  |  |  |  |  | 107.1 |  | 109.4 |  |  | -2.03% |  |  |  |  |  |  |  | 124.9 |  | 127.2 |  |  | -1.63% |  |  |  |  |  |
| CE3-2.3 | 11.8 |  | 12.0 |  | -1.59% |  |  |  |  |  |  |  | 107.1 |  | 107.6 |  |  | -0.71% |  |  |  |  |  |  |  | 124.9 |  | 124.0 |  |  | 0.16% |  |  |  |  |  |
| CE3-2.4 | 11.8 |  | 12.4 |  | -4.78% |  |  |  |  |  |  |  | 107.1 |  | 109.1 |  |  | -1.90% |  |  |  |  |  |  |  | 124.9 |  | 123.8 |  |  | -0.05% |  |  |  |  |  |
| CE3-2.5a | 11.8 |  | 12.2 |  | -2.93% |  |  |  |  |  |  |  | 107.1 |  | 110.8 |  |  | -3.31% |  |  |  |  |  |  |  | 124.9 |  | 129.0 |  |  | -3.22% |  |  |  |  |  |
| CE3-2.5b | 11.8 |  | 12.2 |  | -3.26% |  |  |  |  |  |  |  | 107.1 |  | 111.1 |  |  | -3.37% |  |  |  |  |  |  |  | 124.9 |  | 129.9 |  |  | -3.43% |  |  |  |  |  |
| CE3-2.5c | 11.8 |  | 12.2 |  | -3.35% |  |  |  |  |  |  |  | 107.1 |  | 111.4 |  |  | -3.79% |  |  |  |  |  |  |  | 124.9 |  | 130.3 |  |  | -3.89% |  |  |  |  |  |
| CE3-2.6 | 11.8 |  | 12.1 |  | -2.21% |  |  |  |  |  |  |  | 107.1 |  | 108.2 |  |  | -1.37% |  |  |  |  |  |  |  | 124.9 |  | 124.0 |  |  | -0.30% |  |  |  |  |  |
| CE3-2.7 | 11.8 |  | 12.6 |  | -6.03% |  |  |  |  |  |  |  | 107.1 |  | 109.9 |  |  | -2.79% |  |  |  |  |  |  |  | 124.9 |  | 126.0 |  |  | -1.68% |  |  |  |  |  |
| CE3-2.8a-1 | 11.8 |  | 12.2 |  | -3.16% |  |  |  |  |  |  |  | 107.1 |  | 108.8 |  |  | -1.74% |  |  |  |  |  |  |  | 124.9 |  | 125.0 |  |  | -0.64% |  |  |  |  |  |
| CE3-2.8a-2 | 11.8 |  | 12.6 |  | -6.18% |  |  |  |  |  |  |  | 107.1 |  | 110.2 |  |  | -2.86% |  |  |  |  |  |  |  | 124.9 |  | 125.3 |  |  | -1.20% |  |  |  |  |  |
| CE3-2.8b-1 | 11.8 |  | 12.0 |  | -1.95% |  |  |  |  |  |  |  | 107.1 |  | 107.9 |  |  | -0.92% |  |  |  |  |  |  |  | 124.9 |  | 124.2 |  |  | -0.02% |  |  |  |  |  |
| CE3-2.8b-2 | 11.8 |  | 12.5 |  | -5.10% |  |  |  |  |  |  |  | 107.1 |  | 109.3 |  |  | -2.10% |  |  |  |  |  |  |  | 124.9 |  | 124.3 |  |  | -0.51% |  |  |  |  |  |
| CE3-2.8c-1 | 11.8 |  | 12.7 |  | -6.86% |  |  |  |  |  |  |  | 107.1 |  | 117.7 |  |  | -7.56% |  |  |  |  |  |  |  | 124.9 |  | 138.0 |  |  | -7.57% |  |  |  |  |  |
| CE3-2.8c-2 | 11.8 |  | 13.1 |  | -8.97% |  |  |  |  |  |  |  | 107.1 |  | 118.8 |  |  | -8.37% |  |  |  |  |  |  |  | 124.9 |  | 137.7 |  |  | -7.77% |  |  |  |  |  |
| CE3-2.8d-1 | 11.8 |  | 12.7 |  | -6.32% |  |  |  |  |  |  |  | 107.1 |  | 117.2 |  |  | -7.18% |  |  |  |  |  |  |  | 124.9 |  | 137.9 |  |  | -7.32% |  |  |  |  |  |
| CE3-2.8d-2 | 11.8 |  | 13.0 |  | -8.46% |  |  |  |  |  |  |  | 107.1 |  | 118.3 |  |  | -8.00% |  |  |  |  |  |  |  | 124.9 |  | 137.2 |  |  | -7.39% |  |  |  |  |  |
| CE3-2.9-1 | 11.8 |  | 12.6 |  | -5.72% |  |  |  |  |  |  |  | 107.1 |  | 116.7 |  |  | -6.83% |  |  |  |  |  |  |  | 124.9 |  | 137.0 |  |  | -6.91% |  |  |  |  |  |
| CE3-2.9-2 | 11.8 |  | 12.9 |  | -7.90% |  |  |  |  |  |  |  | 107.1 |  | 117.9 |  |  | -7.66% |  |  |  |  |  |  |  | 124.9 |  | 136.7 |  |  | -7.08% |  |  |  |  |  |
| CE3-2.10 | 11.8 |  | 12.0 |  | -1.78% |  |  |  |  |  |  |  | 107.1 |  | 107.7 |  |  | -0.73% |  |  |  |  |  |  |  | 124.9 |  | 123.8 |  |  | -0.48% |  |  |  |  |  |
| CE3-2.11 | 11.8 |  | 12.6 |  | -5.73% |  |  |  |  |  |  |  | 107.1 |  | 109.7 |  |  | -2.62% |  |  |  |  |  |  |  | 124.9 |  | 124.8 |  |  | -0.94% |  |  |  |  |  |

# Crosscheck Reports

The following table summarizes the crosscheck reports.

| CE3-1: Regular and TS residual coding (RRC, TSRC) for lossless coding, and modifications to RRC and TSRC for lossless and lossy operation modes |  |
| --- | --- |
| test (crosschecker) | crosscheck report |
| CE3-1.1a/b (T. Nguyen) | results: Results for lossless and CTC are the same, for low QP differs, probably due to uninitialized variables in the TCQ path. code verification: The provided source code was inspected. The description covers the provided software implementation. draft text verification: further comments: |
| CE3-1.1c/d (M. Sarwer) | results: Partial results matched with the proponent results. code verification: Consistent with CE description. draft text verification: further comments: |
| CE3-1.2 (Z.-Y. Lin) | results: matched code verification: same as their description draft text verification: same as the software further comments: |
| CE3-1.3 (T. Tsukuba) | results: Cross-check results completely match proponent’s. code verification: Slice-header level TSRC enabling flag is integrated. TSRC and RRC is switched based on HashRatio. Picture-header level TSRC enabling flag is integrated. TSRC and RRC is switched based on HashRatio. draft text verification: further comments: Remarks (TN): After source code inspection, it is concluded that the software includes further changes in checkDisFracMmvd()adaptively decide to select RRC/TSRC, which was not really clear from the test description. |
| CE3-1.4 (J. Choi) | results: cross checker tested the software provided by Kwai in the following git repository. https://vcgit.hhi.fraunhofer.de/jvet-p-ce3/VVCSoftware_VTM/tree/CE3-1.4 The cross checker ran all required tests under VTM-7.0 lossless configuration and confirmed that there is no problem in the reported test results. code verification: The cross checker carefully checked the provided source code and confirmed that the implementation matches the input document. draft text verification: further comments: |
| CE3-1.5 (T.-C Ma) | results: Cross-check results completely match proponent’s results. code verification: Use selection rice parameter in P0072. draft text verification: The cross checker confirmed that source code matches spec and description. further comments: |

| CE3-2: Enabling BDPCM and high-level techniques for lossless coding, and the combination of BDPCM with different RRC/TSRC techniques |  |
| --- | --- |
| test (crosschecker) | crosscheck report |
| CE3-2.1 (T. Tsukuba) | results: Cross-check results completely match proponent’s. code verification: No code changes. BDPCM for luma is enabled by encoder option as in CE description. draft text verification: further comments: |
| CE3-2.2 (T. Tsukuba) | results: Cross-check results completely match proponent’s. code verification: BDPCM for luma and chroma are enabled, and integration matches CE description. draft text verification: further comments: |
| CE3-2.3 (T. Nguyen) | results: Results match with the results provided by the proponent. code verification: Source code was inspected. It matches the description provided by the proponent. draft text verification: Not checked, it is expected that the changes within the WD is manageable. further comments: Only the first test (SPS flag) was crosschecked. |
| CE3-2.4 (T. Poirier) | results: test results confirmed code verification: verified, match the description, enable chroma BDPCM and a high-level switch to use RRC for lossless draft text verification: further comments: |
| CE3-2.5 (M. Sarwer) | results: Partial results matched with the proponent results. code verification: Consistent with CE description. In addition to CU/TB level RRC/TSRC, another high level flag (in PPS) is also implemented. draft text verification: further comments: |
| CE3-2.6 (T. Nguyen) | results: Results match with the results provided by the proponent. code verification: Only Luma BDPCM and IBC use TSRC, otherwise RRC. draft text verification: Not checked, it is expected that the changes within the WD is manageable further comments: |
| CE3-2.7 (T.-C. Ma) | results: Cross-check result completely match proponent’s one code verification: Only BDPCM and IBC are applied with TSRC, other modes are applied with RRC. draft text verification: further comments: |
| CE3-2.8a/b (X. Wang) | results: Our (Kwai Inc.) finished sequences do match proponent’s results. Few sequences are still on-going. Will update later. code verification: CE3-1.1 with Luma BDPCM and chroma BDPCM. draft text verification: further comments: |
| CE3-2.8c/d (M. Sarwer) | results: Partial results matched with the proponent results. code verification: Consistent with CE description. draft text verification: further comments: |
| CE3-2.9 (Z.-Y. Lin) | results: matched code verification: same as their description draft text verification: not provided ( not necessary since it’s a combined test) further comments: |
| CE3-2.10 (H. Wang) | results: Qualcomm’s results do match proponent’s results. code verification: Implementation was studied and was matched with the descriptions. draft text verification: further comments: |
| CE3-2.11 (H. Wang) | results: Qualcomm’s results do match proponent’s results. code verification: Implementation was studied and was matched with the descriptions. draft text verification: further comments: |

# Participants List

| Affiliation | Person, Email address |
| --- | --- |
| Fraunhofer HHI | Tung Nguyen tung.nguyen@hhi.fraunhofer.de Benjamin Bross benjamin.bross@hhi.fraunhofer.de |
| Qualcomm | Alican Nalci analci@qti.qualcomm.com Yung-Hsuan Chao yunghsua@qti.qualcomm.com Hongtao Wang hongtaow@qti.qualcomm.com Muhammed Coban mcoban@qti.qualcomm.com |
| MediaTek | Tzu-Der Chuang peter.chuang@mediatek.com Zhi-Yi Lin Zhi-Yi.Lin@mediatek.com |
| Kwai | Tsung-Chuan Ma tsung-chuanma@kwai.com Yi-Wen Chen yiwenchen@kwai.com Xiaoyu Xiu xiaoyuxiu@kwai.com Hong-jheng Jhu jhuhong-jheng@kwai.com Xianglin Wang xianglinwang@kwai.com |
| LG | Jungah Choi jungah.choi@lge.com Hyeongmun Jang hm.jang@lge.com Jin Heo jin78.heo@lge.com Jangwon Choi jangwon84.choi@lge.com Sunmi Yoo sunmi.yoo@lge.com Jaehyun Lim jaehyun.lim@lge.com Seung Hwan Kim seunghwan3.kim@lge.com Mehdi Salehifar mehdi.salehifar@lge.com |
| Alibaba | Yan Ye yan.ye@alibaba-inc.com Ruling Liao ruling.lrl@alibaba-inc.com Jiancong Luo jiancong.luo@alibaba-inc.com Mohammed Sarwer m.sarwer@alibaba-inc.com |
| Sony | Takeshi Tsukuba takeshi.tsukuba@sony.com Masaru Ikeda masaru.ikeda@sony.com Yoichi Yagasaki yoichi.yagasaki@sony.com |

# List of Input Contributions

JVET-Q0092	CE3: Modified Rice Parameter Derivation for Residual Coding (CE3-1.1, CE3-1.2, CE3-2.8 and CE3-2.9)

JVET-Q0068	CE3-1.3: Using RRC for lossless coding without state transition

JVET-Q0069	CE3-1.4: On residual scanning order for lossless coding

JVET-Q0080	CE3-1.5: Rice parameter derivation of transform skip residual coding

JVET-Q0086	CE3-2.1 and CE3-2.2: Luma/Chroma BDPCM for Lossless Coding with Transform Skip Residual Coding

JVET-Q0088	CE3-2.3: Luma BDPCM for Lossless Coding with Regular Residual Coding

JVET-Q0089	CE3-2.4: Luma and Chroma BDPCM for Lossless Coding with Regular Residual Coding

JVET-Q0070	CE3-2.5: Residual coding selection signaling for lossless coding

JVET-Q0081	CE3-2.6: Luma BDPCM and IBC with TSRC for lossless coding

JVET-Q0082	CE3-2.7: Luma and chroma BDPCM and IBC with TSRC for lossless coding

JVET-Q0071	CE3-2.10: Luma BDPCM for lossless coding with RRC and residual rotation

# JVET-Q0072	CE3-2.11: Luma and Chroma BDPCM for lossless coding with RRC and residual rotation

# Test Conditions

## Software, reference configurations, and test conditions

All tools have to be integrated into the VTM-7.0 software (outcome of the Geneva meeting). Results have to be reported for the VTM-7.0 software and the common test conditions for lossless, which has BDPCM disabled for both camera captured and screen content sequences. The lossless mode can be enabled at the encoder by using the provided configuration file “lossless.cfg” additionally to the CTC configuration files.

- Lossless: AI, RA, and LD configurations with 100 frames only (i.e., by using the encoder option “--FramesToBeEncoded=100”).

All tools are tested against the VTM-7.0 anchor (same set of tools, except the tool to be tested).

For tests targeting modifications to RRC or TSRC, results for two ranges of QP values are mandatory, using the CTC configurations (HashME, IBC, and luma BDPCM enabled for screen content sequences):

- Standard QPs (37, 32, 27, 22): AI, RA, and LD configurations as specified in CTC (all frames);

- Low QPs (17, 12, 7, 2): AI, RA, and LD configurations with 100 frames only (i.e., by using the encoder option “--FramesToBeEncoded=100”).

For all categories, additional results for screen content sequences (class TGM with YUV 4:2:0 format) are mandatory, using the same configuration for class F CTC (HashME, IBC, and luma BDPCM enabled).

## Coding Performance Measurements

The measurement relies on the compression ratio relative to the uncompressed raw data. A bit-rate saving or overhead is then calculated by comparing the compression ratio of the test relative to the VTM-7.0 anchor.

For the lossy test conditions, the impact on bitrate/PSNR using provided data (4 points, BD-rate) is measured. Proponents are encouraged to provide supplementary results.

## Complexity Considerations

To provide an indicative assessment of complexity, the following numbers should be provided:

- Encoding time (compared to the reference configurations);

- Decoding time (compared to the reference configuration).

In addition, proponents are encouraged to provide the following (but not limited to) information for complexity and entropy throughput analysis:

- Number of context models

- Number of context-coded and bypass bins in average case and the worst case

- Table size of scanning paths

- Analysis of the frequency of context switches

- Analysis of the dependency between two consecutive bins

- Analysis of entropy parsing dependency (whether bits to bins conversion depends on complex reconstruction and/or pixel processing process)

- Analysis of entropy de-binarization dependency (whether bins to symbols conversion depends on complex reconstruction and/or pixel processing process)

# Description of the Tests

## Category A: CE3-1.x

### CE3-1.1 from JVET-P0559: Modified RRC and TSRC residual coding for lossless (Qualcomm)

The purpose of test CE3-1.1 is to test bit-rate savings for lossless coding based on RRC and TSRC modifications as detailed in JVET-P0559. There are a total of 4 sub-tests in this category which are detailed as follows: Test (a) involves a modification to RRC as discussed in JVET-P0559-m1, where the rice parameter is derived based on the base level and the normalized sum of absolute levels of neighboring coefficients in local template. Test (b) involves a modification to rice parameter as discussed in JVET-P0559-m1, where rice parameter is derived based only on the base level and the local template without the normalization of Test (a). Test (c) involves a modification to TSRC as discussed in JVET-P0559-m2 according to new TS coding of VTM-7.0. The rice parameter derivation is based on the base level and normalized sum of absolute levels of neighboring coefficients in the local template as discussed in JVET-P0559-m2. Finally, Test (d) involves a similar modification to TSRC as in Test (c) without normalization of the sum of absolute levels in the local template.

### CE3-1.2 from JVET-P1028: Modified TSRC residual coding for lossless (Qualcomm)

In this test, the Rice parameter derivation process of TSRC is modified as described in JVET-P1028. This involves using the CE7-1.2d-alternative TS coding as detailed in JVET-P0079 which is modified according to VTM-7.0 e.g. CCB count check method of P0072, and level mapping removal in last coding pass of P0298 along with 1.75 bin/coefficient CCB limit.

### CE3-1.3 from JVET-P0148/JVET-P0258: Using RRC for lossless coding without state transition (MediaTek/Qualcomm/Kwai/Alibaba)

In this test, two subtests are proposed to enable 1-bit high level syntax to select RRC/TSRC since the cu_transquant_bypass_mode is not used in CE and the CE mandate. In the first test, one PPS flag is added to indicate which residual coding scheme is selected. In the second test, one syntax in picture header is added to indicate which residual coding scheme is selected.

### CE3-1.4 from JVET-P0528: On residual scanning order for lossless coding (Kwai)

The residual samples from intra prediction located at the bottom-right area of the block tend to have larger amplitude than those located at the top-left area, because of the distance between the reconstructed pixels that are used for prediction and the predicted pixels. When regular residual coding is applied for lossless residual coding, rotating the residual blocks (i.e. flipping the blocks along both horizontal and vertical direction) would place the statistically large amplitude residue at the top left area of the TU, which is more suitable for regular coefficient coding. In this CE, it is proposed to test the rotation of residual blocks if RRC is applied for lossless residual.

### CE3-1.5 from JVET-P0463/JVET-P0072: Rice parameter derivation of TSRC for lossless (Alibaba)

In this test, the Rice parameter derivation process of TSRC is modified. The rice parameter derivation process and look-up table for Rice parameter selection are same as CE7-1.3b* described in JVET-P0072.

## Category B: CE3-2.x

### CE3-2.1 from JVET-P0082: Enabling Luma BDPCM for lossless coding using TSRC (Qualcomm)

In this test it is proposed to enable luma BDPCM for lossless coding for both natural sequences and screen content. TSRC is used for lossless coding. The test will show whether BDPCM is beneficial for lossless coding if TSRC is used.

### CE3-2.2 from JVET-P0082/JVET-P0147: Luma and Chroma BDPCM for lossless using TSRC (Qualcomm, MediaTek)

In this test it is proposed to enable BDPCM for both luma and chroma for lossless coding for both natural sequences and screen content. TSRC will be used for lossless coding. The test will show whether BDPCM is beneficial for lossless coding if TSRC is used for lossless coding.

### CE3-2.3 from JVET-P0082/JVET-P0147/JVET-P0574/JVET-P0584: Luma BDPCM for lossless using RRC (Qualcomm, MediaTek, Alibaba, Sony, Kwai)

In this test it is proposed to enable luma BDPCM for lossless for both natural sequences and screen content. The core experiment will tests the benefit of using Luma BDPCM with RRC for lossless coding. Selecting RRC can be done using a high-level (e.g. SPS) flag that will force/select RRC for lossless instead of TSRC. In this case such a flag could be defined as follows:

- sps_transform_residual_coding_flag equal to 1 specifies that if the transform_skip_flag is equal to 1 to the current block at the location (x0,y0), then the current block at the location (x0, y0) uses transform residual coding.

- sps_transform_residual_coding_flag equal to 0 specifies that if the transform_skip_flag is equal to 1 to the current luma block at the location (x0,y0) , then the current block at the location (x0, y0) uses transform-skip residual coding.

In this example, the flag is signaled at SPS level, so all blocks use either RRC/TSRC depending on the flag value. A similar flag could be defined at PPS etc. However, since the core test is to determine whether RRC is useful for lossless coding, a simple macro can also be used to always select RRC for this test. This would provide the same result.

### CE3-2.4 from JVET-P0082/JVET-P0147/JVT-P0584: Luma and Chroma BDPCM for lossless using RRC (Qualcomm, MediaTek, Alibaba, Sony, Kwai)

In this test it is proposed to enable luma and chroma BDPCM mode for lossless coding for both natural sequences and screen content. RRC will be used in this test as defined in CE3-2.3.

### CE3-2.5 from JVET-P0525:

During this (Geneva) meeting, since the current TSRC still performs significantly worse for camera content but better for screen content, it is proposed to test the performance of signaling 1-bit syntax to select which coding scheme (i.e., RRC or TSRC) is applied under lossless coding. Three sub-tests are defined in this CE test, they are:

- CU level signaling to select RRC/TSRC

- TB level signaling to select RRC/TSRC

- Implicitly select RRC/TSRC by transform_skip_flag

### CE3-2.6 from JVET-P0466: Luma BDPCM and IBC with TSRC for lossless coding (LGE)

In this test, it is proposed to enable luma BDPCM mode for lossless coding for both natural sequences and screen content. In the proposed method, luma BDPCM and IBC modes are coded by TSRC, and others are coded by RRC.

### CE3-2.7 from JVET-P0466: Luma and Chroma BDPCM and IBC with TSRC for lossless coding (LGE)

In this test, it is proposed to enable luma and chroma BDPCM mode for lossless coding for both natural sequences and screen content. In the proposed method, BDPCM and IBC modes are coded by TSRC, and others are coded by RRC.

### CE3-2.8: Luma and chroma BDPCM with the modified RRC and TSRC methods of CE3.1-1 (Qualcomm)

This combined test will investigate the modified RRC and TSRC methods from CE3-1.1 and BDPCM for luma and chroma from CE3-2.2 and CE3-2.4 (4 subtests).

### CE3-2.9: Luma and chroma BDPCM with the modified TSRC of CE3-1.2 (Qualcomm)

This combined test will investigate the modified TSRC method from CE3-1.2 and BDPCM for luma and chroma from CE3-2.2.

### CE3-2.10: Luma BDPCM for lossless coding with RRC and residual rotation

This combined test will be on top of CE3-2.3 and will enable residual rotation if RRC is applied.

### CE3-2.11: Luma and Chroma BDPCM for lossless coding with RRC and residual rotation

This combined test will be on top of CE3-2.7 and will enable residual rotation if RRC is applied.