| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0024-v2 |
| --- | --- |

| Title: | CE4: Summary report on inter prediction with geometric partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen Ru-Ling Liao Xiaoyu Xiu Haitao Yang | Tel: Email: | chunchic@qti.qualcomm.com ruling.lrl@alibaba-inc.com xiaoyuxiu@kwai.com haitao.yang@huawei.com |
| Source: | CE4 coordinators |  |  |

_____________________________

- Abstract

This contribution provides a summary report of Core Experiment 4 on inter prediction with geometric partitioning.

The technique proposed in JVET-P0884/P0885 was suggested to be the common base for this core experiment. All other techniques to be tested is implemented on top of the common base.

For performance evaluation, the common base is requested to be compared with both VTM-7.0 and VTM-7.0 with TPM off. All other tests is requested to be compared with the common base.

Test specification

| Test # | Description | Source |
| --- | --- | --- |
| Common base | Geometric partitioning for merge mode as in JVET-P0884&JVET-P0885 | JVET-Q0079 |
| 4-1 | Common base simplification by reducing the number of partition mode to e.g. 64 | JVET-Q0059 |
| 4-2.1 | Block size restriction of GEO, disable GEO for block size greater than 64×64 | JVET-Q0077 |
| 4-2.2 | Block size restriction of GEO, disable GEO for block size greater than 32×32 | JVET-Q0077 |
| 4-3.1 | Disallow SBT for GEO mode | JVET-Q0062 |
| 4-3.2 | Disallow SBT for GEO mode when displacement is 0 | JVET-Q0062 |
| 4-4 | Adaptive blending for screen content | JVET-Q0060 |
| 4-5 | GEO with a single mode for 4xN and Nx4 CU | JVET-Q0091 |
| 4-6 | Combination of geometric partitioning and CIIP | JVET-Q0078 |
| Combined test 1* | CE4-1+CE4-2.1+CE4-3.1 | JVET-Q0061 |
| Combined test 2* | CE4-1+CE4-2.1+CE4-3.2 | JVET-Q0061 |

* It is requested by the proponents and agreed on CE reflector to include the results of combined tests of test 4-1, 4-2 and 4-3 in CE4 report for providing more information for the consideration of JVET experts.

Simulation results

The common base is compared against two anchors, VTM-7.0 and VTM-7.0 with TPM off. The two sets of results are shown in the table below.

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Anchor | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| VTM-7.0 | -0.28% | -0.35% | -0.34% | 103% | 100% | -0.70% | -0.67% | -0.77% | 103% | 100% |
| VTM-7.0 w/o TPM | -0.66% | -0.98% | -1.01% | 104% | 99% | -1.61% | -1.93% | -1.98% | 106% | 98% |

All other tests are compared against the common base.

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4-1 | 0.00% | -0.01% | 0.03% | 99% | 100% | 0.00% | 0.10% | 0.10% | 99% | 99% |
| 4-2.1* | -0.01% | -0.01% | -0.03% | 99% | 99% | 0.06% | -0.06% | 0.06% | 100% | 98% |
| 4-2.2* | 0.00% | -0.03% | -0.03% | 99% | 99% | 0.22% | 0.10% | 0.24% | 99% | 99% |
| 4-3.1 | 0.04% | 0.08% | 0.04% | 96% | 95% | 0.11% | -0.04% | -0.05% | 95% | 98% |
| 4-3.2 | -0.01% | 0.01% | 0.00% | 97% | 95% | 0.07% | 0.05% | 0.19% | 96% | 97% |
| 4-4 | / | / | / | / | / | / | / | / | / |  |
| 4-5 | -0.05% | -0.02% | -0.09% | 100% | 97% | -0.03% | -0.10% | 0.13% | 100% | 95% |
| 4-6 | -0.05% | 0.00% | 0.01% | 102% | 101% | -0.08% | -0.21% | -0.19% | 102% | 103% |
| Combined test 1 | 0.03% | 0.01% | 0.02% | 98% | 98% | 0.12% | 0.07% | 0.39% | 98% | 96% |
| Combined test 2 | -0.01% | 0.04% | 0.06% | 98% | 98% | 0.06% | 0.15% | 0.24% | 98% | 97% |

* It is pointed out in the CE4 reflector that the number of SATD and RDO checkings are increased to 60 and 8.

Results for Class F and Class SCC is shown for CE4-4 since the test applies to screen content only. The performance compared against the common base, VTM-7.0, and VTM-7.0 without TPM are shown separately, since the data reflecting the behaviour of the common base on screen content are not provided elsewhere in CE4 report.

|  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Anchor | Sequence | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| VTM-7.0 | Class F | -0.51% | -0.51% | -0.46% | 100% | 99% | -0.70% | -1.03% | -0.27% | 102% | 98% |
|  | TGM420 | -2.50% | -2.19% | -2.12% | 99% | 99% | -3.26% | -2.95% | -2.94% | 101% | 99% |
| VTM-7.0 | Class F | -0.76% | -0.92% | -0.85% | 102% | 100% | -1.14% | -1.10% | -1.43% | 105% | 99% |
| w/o TPM | TGM420 | -2.54% | -2.33% | -2.27% | 101% | 99% | -3.33% | -3.08% | -3.02% | 103% | 99% |
| Common | Class F | -0.19% | -0.12% | -0.13% | 100% | 100% | -0.20% | -0.12% | 0.03% | 100% | 101% |
| base | TGM420 | -2.03% | -1.52% | -1.46% | 99% | 99% | -2.82% | -2.19% | -2.10% | 99% | 100% |

Crosscheck report

| Test# | Crosschecker | SW match CE description (Y/N) | Text match SW (Y/N) | Simulation results (Y/N) | EncT/DecT match (Y/N) |
| --- | --- | --- | --- | --- | --- |
| Common base | Yu-Ling Hsiao (MediaTek) | Y | Y | Y | Y |
| 4-1 | Yu-Ling Hsiao (MediaTek) | Y | Y | Y | Y |
| 4-2.1 | Ru-Ling Liao (Alibaba) | Y | Y | Y | Y |
| 4-2.2 | Ru-Ling Liao (Alibaba) | Y | Y | Y | Y |
| 4-3.1 | Ru-Ling Liao (Alibaba) | Y | Y | Y | Y |
| 4-3.2 | Ru-Ling Liao (Alibaba) | Y | Y | Y | Y |
| 4-4 | Yu-Ling Hsiao (MediaTek) | Y | Y | Y | Y |
| 4-5 | G. Kulupana (BBC) | Y | Y | Y | Y |
| 4-6 | K. Panusopone (Nokia) | Y | Y | Y | Y |
| Combined test 1 | Xiaoyu Xiu (Kwai) | Y | Y | Y | Y |
| Combined test 2 | Xiaoyu Xiu (Kwai) | Y | Y | Y | Y |

SW match CE description: Y SW implementation match CE description, N mismatch.

Text match SW: Y text match SW, N mismatch.

Simulation results: Y substantial results (all but class A for RA, all but class B for LDB) obtained and match results from tester, N mismatch or no substantial results.

EncT/DecT match: Y encoder and decoder run time match those from tester, N mismatch.

Detailed description of tests

- Common base in JVET-Q0079 (Huawei, Alibaba, Qualcomm, RWTH Aachen)

VVC-7.0 TPM                      Generalized as GEO

………

Figure: Examples for GEO modes

The common base of geometric partitioning is defined as the method descripted in JVET-P0884&JVET-P0885. The total number of geometric partitioning mode is 82, which is composed with 24 slope based angle and 4 distance for each angle. For angles larger than or equal to slope -1, the distance 0 is redundant and is removed. Besides, the horizontal and vertical angles with distance 0 is removed since it is overlapped with binary tree split. Therefore, the total number of geometric partitioning mode is 24×4 − 12 – 2 = 82. Note that the VTM-7.0 TPM is harmonized into the design of the common base as the two diagonal partitioning modes.

Figure: Angles distribution of GEO partitions in the common base

The GEO mode is applied to a merge block whose width and height are larger than or equal to 8. When a block coded using GEO mode, an index is signaled to indicate which one of 82 partitioning modes is used to split the block into two partitions. Each partition is inter predicted with its own motion vector. After predicting each of the partition, the sample values along the partitioning edge are adjusted using a blending processing with weights. This is the prediction signal for the whole block, and transform and quantization process will be applied to the whole block as in other prediction modes.

Figure. An example of blending mask for GEO

The weights for luma samples used in the blending process are calculated as follows:

weightIdx=x+offsetX≪1+1*DisdisplacementX+
y+offsetY≪1+1*DisdisplacementY-rho                                (1)

weightIdxAbs=Clip30, 26, absweightIdx                                                  (2)

sampleWeight=weightIdx≤0 ?GeoFilterweightIdxAbs :8-GeoFilter[weightIdxAbs]         (3)

Dis[.] is a look up table with 24 entries and the possible out put values are {0, 1, 2, 4}. GeoFilter[.] is a look up table with 27 entries. The variables rho, offsetX and offsetY are pre-computed based on:

rho=(Dis[displacementX]<<8)+(Dis[displacementY]<<8)                                 (4)

offsetX=(256-W)≫1φ % 12 = 6 or (φ % 12 ≠ 0 and H ≥ W)256-W≫1+φ<12 ?ρ×W≫3 :-(ρ×W≫3)otherwise

offsetY=256-H≫1+φ<12 ?ρ×H≫3 :-(ρ×H≫3)φ % 12 = 6 or (φ % 12 ≠ 0 and H ≥ W)256-H≫1otherwise

The φ and ρ represent angle and distance which are derived from a look up table using the signaled index.

The weights for chroma samples are subsampled from the luma weights. The motion storage masks are derived using same method of weights derivation independently.

- Test 4-1 in JVET-Q0059 (Huawei, Alibaba, Qualcomm RWTH Aachen)

(a) 82 partition modes in the common base    (b) 64 partition modes in test 4-1

Figure: Angles distribution of GEO partitions

In test 4-1, the number of GEO partitions of the common base is reduced from 82 modes to 64 modes. i.e. 20 angles and 4 distances. Comparing to the common base, the angles {5, 7, 17, 19} are removed since in nature video sequences the motion objects are mostly vertical layout and the near horizontal split modes are less frequently used. Besides, the horizontal angles {0, 12} and vertical angles {6, 18} with distance index 0 are removed since these partitioning are overlapped with ternary tree splitting. The total number of geometric partitioning mode can be calculated as 20×3 + 10×3  4  2 = 64.

- Test 4-2.1 & 4-2.2 in JVET-Q0077 (Bytedance)

In test 4-2.1, GEO mode is disabled for a block which width or height is greater than 64.

In test 4-2.2, GEO mode is disabled for a block which width or height is greater than 32.

- Test 4-3.1 & 4-3.2 in JVET-Q0062 (Qualcomm, Hikvision)

In test 4-3.1, SBT is disabled for GEO mode.

In test 4-3.2, SBT is disabled for GEO mode when displacement is 0.

- Test 4-4 in JVET-Q0060 (Huawei, RWTH Aachen)

In test 4-4 the blending process is turned off for SCC in order to avoid over smoothing the sharp edges of screen contents. A SPS-level control flag and a picture header blending-off flag is introduced to control the blending process on and off.

- Test 4-5 in JVET-Q0091 (Nokia)

In test 4-5, the GEO of common base is extended to support 4xN and Nx4 CU. For these CU dimensions, GEO skips wedge_partition_idx syntax, and allows one partitioning angle (either horizontal or vertical) and one distance. For 4xN CU, GEO employs vertical angle blending weights. For Nx4 CU, GEO employs horizontal angle blending weights. Motion vector prediction and blending operation of GEO with a single mode for 4xN and Nx4 CU are aligned with the GEO of common base.

- Test 4-6 in JVET-Q0078 (BBC)

An intra predictor and an inter predictor are computed for a block. The inter predictor and the intra predictor are combined using the geometric partitioning scheme as in the common base. The intra predictor is generated as Planar mode and is used in one of the two GEO partitions, and the inter predictor is used in the other GEO partition. The blending scheme as used in GEO is used to blend the two GEO partitions.

There are four candidate GEO partition modes available and the syntax geo_ciip_mode is signalled for indication. The set of the four candidates is adaptively selected based on whether the above and the left neighbouring CU is intra coded or not, using a pre-computed LUT.

Non-CE4/CE4-related contributions

Geometric partitioning (28)

| JVET number | Title | Tag |
| --- | --- | --- |
| JVET-Q0061 | CE4-related: Combined Test of CE4-1, CE4-2 and CE4-3 |  |
| JVET-Q0123 | CE4-related: Modifications of GEO |  |
| JVET-Q0127 | Non-CE4: On merge list generation for geometric partitioning |  |
| JVET-Q0131 | Non-CE4: GEO with mode reduction |  |
| JVET-Q0132 | Non-CE4: Separate syntax for GEO angle and distance |  |
| JVET-Q0160 | CE4-related: CE4-1 spec text with suggested fixes |  |
| JVET-Q0168 | CE4-related: On GEO cleanups |  |
| JVET-Q0188 | CE4-related: Simplification on geometric partitioning mode by replacing motion index calculation with subsampled weight information |  |
| JVET-Q0189 | CE4-related: Reducing GEO modes |  |
| JVET-Q0268 | CE4-related: GEO with 32 modes |  |
| JVET-Q0307 | CE4-related: Block-dimension based GEO mode selection |  |
| JVET-Q0309 | CE4-related: Further constraints on block shapes for GEO |  |
| JVET-Q0312 | CE4-related: On simplification for GEO weight derivation |  |
| JVET-Q0338 | CE4-related: Harmonized conditions for CIIP and GEO |  |
| JVET-Q0339 | CE4-related: Adjustment of the distance on the GEO mode |  |
| JVET-Q0348 | CE4-related: Displacement Restriction on Geometric Inter Prediction |  |
| JVET-Q0365 | CE4-Related: On-the-fly Weighting Index to Sample Blending Weight Conversion without Table Look-Up in GEO |  |
| JVET-Q0388 | CE4-related: GEO memory reduction in weighting masks by fixed shifting |  |
| JVET-Q0429 | CE4-related: Geometric partitioning signaling |  |
| JVET-Q0437 | Non-CE4: Disabling MTS for Geo |  |
| JVET-Q0440 | Non-CE4: Disabling TrSkip for Geo |  |
| JVET-Q0456 | Non-CE4: reduced LUT for GEO blending weights generation |  |
| JVET-Q0458 | CE4-Related: On-the-fly Weighting Index to Sample Blending Weight Conversion without Table Look-Up on Top of Combine Test in Q0061 |  |
| JVET-Q0459 | CE4-Related: Look-Up Table Free Weighting Index to Sample Blending Weight Calculation By Using Piecewise Constant Function in GEO |  |
| JVET-Q0466 | CE4-Related: On-the-fly Weighting Calculation and MV Storage Derivation in GEO |  |
| JVET-Q0507 | CE4-related: Quality scalable GEO |  |
| JVET-Q0508 | CE4-related: Combined test of JVET-Q0123 and JVET-Q0268 |  |
| JVET-Q0526 | CE4-related: GEO support for 4xN and Nx4 CU |  |

Others (15)

| JVET number | Title | Tag |
| --- | --- | --- |
| JVET-Q0105 | nonCE4: BCW SIF Derivation for Pairwise Candidate | Pairwise |
| JVET-Q0306 | Non-CE4: Removal of MVD scaling process for 4x8/8x4 blocks in MMVD | MMVD |
| JVET-Q0313 | Non-CE4: Constraints on block size for ATMVP | SbTMVP |
| JVET-Q0315 | Non-CE4: Triangular prediction mode with motion vector difference | TPM |
| JVET-Q0337 | Non-CE4: On weight values of the chroma-component for TPM mode | TPM |
| JVET-Q0340 | Non-CE4: Cleanup of the MMVD offset derivation | MMVD |
| JVET-Q0341 | Non-CE4: Range of the motion vector | MV coding |
| JVET-Q0364 | CE4.4-related: Combination of GEO or TPM with Weighted-Prediction | TPM |
| JVET-Q0389 | Non-CE4: SbTMVP harmonization | SbTMVP |
| JVET-Q0390 | Non-CE4: SbTMVP simplification | SbTMVP |
| JVET-Q0422 | CE4 related: cleanup for signaling maximum number of triangle merge candidates | TPM |
| JVET-Q0522 | Non-CE4: Early termination of DMVR and study of its pixel coverage | DMVR |
| JVET-Q0524 | Non-CE4: DC balancing in DMVR | DMVR |
| JVET-Q0525 | Non-CE4: Support large rotation and flipping in affine and PROF | Affine |
| JVET-Q0601 | Non-CE4: Extension of JVET-P0325 | Merge |

Recommendations

Discuss CE4 tests and review all Non-CE4/CE4-related contributions.