| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0025-v2 |
| --- | --- |

| Title: | CE5: Summary report on cross component adaptive loop filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen Andrew Segall | Tel: Email: | chingyeh.chen@mediatek.com asegall@sharplabs.com |
| Source: | CE5 coordinators |  |  |

_____________________________

# Abstract

This contribution provides a summary report of Core Experiment 5 on cross component adaptive loop filtering.  The goal of this experiment is to improve and/or enhance cross component loop filtering techniques and evaluate those techniques according to both subjective and objective metrics.

The technique proposed in JVET-P1008 (Option 1) was agreed to be the common base for this core experiment. All other techniques were to be implemented on top of the common base.

For performance evaluation, the common base is compared with VTM-7. All other tests are requested to be compared with the common base.

# Test specification

| Test # | Description | Source |
| --- | --- | --- |
| Common base | Cross-component Adaptive Loop Filter as in JVET-P1008 (Option 1) | JVET-Q0058 |
| CE5-1 (Alternative Filter Shapes) |  |  |
| CE5-1.1 | 5x4 filter shape with 8 coefficients | JVET-Q0085 |
| CE5-1.2 | 3x2 filter shape with 6 coefficients | JVET-Q0097 |
| CE5-1.3 | 3x4 filter shape with 6 coefficients | JVET-Q0093 |
| CE5-1.4 | Joint Cb/Cr CC-ALF with one additional weight and CTU level selection | JVET-Q0229 |
| CE5-1.5 | Joint Cb/Cr CC-ALF with one additional weight and block level selection | Withdrawn |
| CE5-2 (Complexity reduction of the filtering process) |  |  |
| CE5-2.1 | Multiplication removal with coefficient range in [-64, 64] | JVET-Q0073 |
| CE5-2.2 | Multiplication removal with coefficient range in [-8, 8] | JVET-Q0095 |
| CE5-3 (Combination Tests) |  |  |
| CE5-3.1 | CE5-2.1 + 5x5 filter shape with 13 coefficients | JVET-Q0073 |
| CE5-3.2 | CE5-1.4 + CE5-2.1 Joint Cb/Cr CC-ALF with one additional weight and CTU-level selection combined with multiplication removal with coefficient range in [-64,64] | JVET-Q0229 |
| CE5-Supplemental |  |  |
| S1 | Performance of CE5 common base with 8-bit dynamic range | JVET-Q0074 |

# Simulation results

## All Intra and Random Access

The common base is compared against VTM-7.0 and results are shown in the table below.

|  | All Intra Main 10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Anchor | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| VTM-7.0 | 0.17% | -8.31% | -7.06% | 99% | 101% | 0.14% | -10.14% | -9.82% | 102% | 101% |

All other tests are compared against the common base.

|  | All Intra Main 10 |  |  |  |  | Random Access B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE5-1 (Alternative Filter Shapes) |  |  |  |  |  |  |  |  |  |  |
| CE5-1.1 | 0.00% | 0.13% | 0.01% | 100% | 98% | 0.01% | -0.43% | -0.35% | 100% | 100% |
| CE5-1.2 | -0.02% | 0.69% | 0.88% | 100% | 100% | -0.02% | 0.93% | 1.08% | 100% | 98% |
| CE5-1.3 | -0.02% | 0.39% | 0.26% | 99% | 98% | -0.01% | 0.22% | 0.31% | 99% | 99% |
| CE5-1.4 | -0.06% | 2.15% | 2.03% | 100% | 99% | 0.01% | 1.34% | 1.34% | 100% | 99% |
| CE5-2 (Complexity reduction of the filtering process) |  |  |  |  |  |  |  |  |  |  |
| CE5-2.1 | -0.03% | 0.53% | 0.36% | 100% | 100% | 0.01% | 0.08% | 0.06% | 100% | 100% |
| CE5-2.2 | -0.03% | 0.74% | 0.60% | 100% | 100% | 0.00% | 0.35% | 0.45% | 100% | 100% |
| CE5-3 (Combination Tests) |  |  |  |  |  |  |  |  |  |  |
| CE5-3.1 | 0.01% | -0.20% | -0.61% | 100% | 101% | 0.01% | -0.55% | -0.71% | 100% | 101% |
| CE5.3.2 | -0.07% | 2.59% | 2.28% | 99% | 97% | 0.01% | 1.44% | 1.48% | 99% | 97% |
| CE5-Supplemental |  |  |  |  |  |  |  |  |  |  |
| S1 | 0.02% | -0.08% | 0.12% | 100% | 100% | 0.00% | 0.05% | 0.13% | 100% | 101% |

## Low Delay

The common base is compared against VTM-7.0 and results are shown in the table below.

|  | Low Delay B Main 10 |  |  |  |  | Low Delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Anchor | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| VTM-7.0 | 0.12% | -13.00% | -9.48% | 99% | 98% | 0.13% | -12.89% | -9.29% | 98% | 100% |

All other tests are compared against the common base.

|  | Low Delay B Main 10 |  |  |  |  | Low Delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE5-1 (Alternative Filter Shapes) |  |  |  |  |  |  |  |  |  |  |
| CE5-1.1 | -0.03% | -0.50% | -0.81% | 101% | 101% | -0.01% | -0.70% | -0.93% | 101% | 101% |
| CE5-1.2 | -0.01% | 1.95% | 1.54% | 101% | 102% | - | - | - | - | - |
| CE5-1.3 | 0.01% | 0.22% | 0.23% | 100% | 99% | - | - | - | - | - |
| CE5-1.4 | -0.01% | 0.25% | -0.01% | 100% | 100% | -0.01% | 0.12% | -0.03% | - | - |
| CE5-2 (Complexity reduction of the filtering process) |  |  |  |  |  |  |  |  |  |  |
| CE5-2.1 | -0.01% | -0.24% | -0.13% | 100% | 101% | 0.02% | -0.17% | -0.31% | 100% | 99% |
| CE5-2.2 | 0.02% | -0.08% | -0.10% | 100% | 99% | 0.00% | -0.13% | -0.25% | 100% | 99% |
| CE5-3 (Combination Tests) |  |  |  |  |  |  |  |  |  |  |
| CE5-3.1 | -0.01% | -1.52% | -1.82% | 100% | 101% | 0.01% | -1.52% | -1.73% | 99% | 99% |
| CE5.3.2 | -0.01% | 0.16% | -0.08% | 99% | 97% | - | - | - | - | - |
| CE5-Supplemental |  |  |  |  |  |  |  |  |  |  |
| S1 | -0.02% | 0.31% | 0.70% | 100% | 100% | 0.01% | 0.33% | 0.56% | 100% | 99% |

## Summary YUV

The common base is compared against VTM-7.0 and results are shown in the table below.

|  | YUV Metric |  |  |  | EncT |  | DecT |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Anchor | AI | RA | LDB | LDP | Min | Max | Min | Max |
| VTM-7.0 | -1.40% | -1.88% | -2.15% | -2.11% | 98% | 102% | 98% | 101% |

All other tests are compared against the common base.

|  | YUV Metric |  |  |  | EncT |  | DecT |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | AI | RA | LDB | LDP | Min | Max | Min | Max |
| CE5-1 (Alternative Filter Shapes) |  |  |  |  |  |  |  |  |
| CE5-1.1 | 0.01% | -0.07% | -0.16% | -0.17% | 100% | 101% | 98% | 101% |
| CE5-1.2 | 0.14% | 0.19% | 0.34% | - | - | - | - | - |
| CE5-1.3 | 0.05% | 0.05% | 0.05% | - | 99% | 100% | 98% | 99% |
| CE5-1.4 | 0.37% | 0.28% | 0.02% | - | - | - | - | - |
| CE5-2 (Complexity reduction of the filtering process) |  |  |  |  |  |  |  |  |
| CE5-2.1 | 0.07% | 0.02% | -0.05% | -0.03% | 100% | 100% | 99% | 101% |
| CE5-2.2 | 0.11% | 0.08% | 0.00% | -0.04% | 100% | 100% | 99% | 100% |
| CE5-3 (Combination Tests) |  |  |  |  |  |  |  |  |
| CE5-3.1 | -0.07% | -0.12% | -0.34% | -0.32% | 99% | 100% | 99% | 101% |
| CE5.3.2 | 0.43% | 0.30% | 0.00% | - | - | - | - | - |
| CE5-Supplemental |  |  |  |  |  |  |  |  |
| S1 | 0.02% | 0.02% | 0.09% | 0.10% | 100% | 100% | 99% | 101% |

## HDR - All Intra

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
| Anchor | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| CE5-Anchor | -6.81% | 0.24% | 0.23% | -11.26% | -20.69% | 0.23% | -12.41% | -21.31% | 100% | 102% |
|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
| Anchor | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| CE5-1 (Alternative Filter Shapes) |  |  |  |  |  |  |  |  |  |  |
| CE5-1.1 | 0.36% | 0.00% | 0.00% | 0.65% | 1.27% | 0.00% | 0.60% | 1.42% | 100% | 99% |

## HDR – Random Access

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
| Anchor | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| CE5-Anchor | -7.92% | 0.55% | 0.53% | -11.85% | -25.58% | 0.53% | -12.79% | -25.26% | 101% | 102% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
| Anchor | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| CE5-1 (Alternative Filter Shapes) |  |  |  |  |  |  |  |  |  |  |
| CE5-1.1 | 0.54% | -0.04% | -0.03% | 0.02% | 1.48% | -0.04% | -0.23% | 1.24% | 99.57% | 98.50% |

## HDR – YUV Summary

|  | wYUV Metric |  | YUV Metric |  | EncT |  | DecT |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Anchor | AI | RA | AI | RA | Min | Max | Min | Max |
| VTM-7.0 | -3.01% | -3.32% | -3.19% | -3.38% | 100% | 101% | 102% | 102% |
|  | wYUV Metric |  | YUV Metric |  | EncT |  | DecT |  |
| Test # | AI | RA | AI | RA | Min | Max | Min | Max |
| CE5-1 (Alternative Filter Shapes) |  |  |  |  |  |  |  |  |
| CE5-1.1 | 0.19% | 0.12% | 0.20% | 0.07% | 100% | 100% | 98% | 99% |

# Crosscheck reports

| Test# | Cross-checker | SW match CE description (Y/N) | Text match SW (Y/N) | Simulation results (Y/N) | EncT/DecT match (Y/N) | Additional Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Common base | H. Yang (InterDigital) |  |  | Y | Y |  |
| CE5-1.1 | K. Misra (Sharp) |  |  | Y | Y |  |
| CE5-1.2 | N. Hu (Qualcomm) |  |  | Y | N |  |
| CE5-1.3 | - |  |  | Y | Y |  |
| CE5-1.4 | Y. Zhao (Huawei) |  |  | Y | Y |  |
| CE5-2.1 | J. Li (Panasonic) |  |  | Y | Y |  |
| CE5-2.2 | P. Onno (Cannon) |  |  | Y | Y |  |
| CE5-3.1 | K. Misra (Sharp) |  |  | Y | Y |  |
| CE5-3.2 | G. Li (Tencent) |  |  | Y | Y |  |

# Detailed description of tests

## CE Anchor (JVET-P1008)

The anchor for the CE test is provided in JVET-P1008 (Option 1).  This method provides a CC-ALF design that has the following properties:

- Filter shape is a 3x4 diamond with 8 unique coefficients

- Filter coefficient dynamic range is limited to 6-bit signed

- Filter selection to performed at the CTU level with support for a maximum of 4 filters

- Symmetric line selection is used at virtual boundary (to better harmonize with ALF)

- Temporal layer coefficient buffers are not used (to better harmonize with the APS concept)

- Residual output is clipped to -2BitDepthC-1 to 2BitDepthC-1-1, inclusive.

An overview of the CC-ALF process is shown in Figure C-1.  As can be seen in the Figure, CC-ALF uses a linear filter to filter luma sample values and generate a residual correction for the chroma channels from the co-located filtered output.  An illustration of the filter shape is provided in Figure C-2.

Fig. C-1: Placement of CC-ALF with respect to other loop filters

Fig. C-2: Filter support in CE Anchor

## CE5-1: Alternative Filter Shapes

### CE5-1.1: Simplifications for Cross Component Adaptive Loop Filter (JVET-P0106)

The CC-ALF filter shape is modified to have only 8 coefficients as shown in the figure below. This modified filter shape is proposed to be tested with sequences containing different chroma type.

### CE5-1.2: Cross component ALF simplification (JVET-P0173)

To further reduce number of operations and remove virtual boundary handling of CCALF, 6 tap filter shape is proposed as below:

| f0 | f1 | f2 |
| --- | --- | --- |
| f3 | f4 | f5 |

### CE5-1.3: Simplified CCALF with 6 filter coefficients (JVET-P0251)

To further reduce the complexity and align number of multiplications of CC-ALF with chroma ALF, a six-tap filter was proposed, as shown in Fig. 5-1.3-1. The six taps are a sub-set of the eight taps of JVET-P1008, and each tap is associated with a unique filter coefficient.

Fig. 5-1.3-1 Six-tap CC-ALF.

### CE5-1.4: Joint chroma cross-component adaptive loop filtering (JVET-P0372)

Joint Chroma Cross-Component Adaptive Loop Filter (JC-CCALF) uses only one set of CCALF filter coefficients trained at the encoder to generate one filtered output as the refinement signal, which will be added directly to the Cb component, and be properly weighted and then added to the Cr component. In this test, filters are indicated at the CTU-level.

SAO Luma

ALF Luma

SAO Cb

SAO Cr

Joint chroma

CC ALF

ALF Cb

Y

Cb

Cr

w

ALF Cr

Fig. 5-1.4-1. JC-CCALF workflow.

### CE5-1.5: Joint chroma cross-component adaptive loop filtering (JVET-P0372)

Joint Chroma Cross-Component Adaptive Loop Filter (JC-CCALF) uses only one set of CCALF filter coefficients trained at the encoder to generate one filtered output as the refinement signal, which will be added directly to the Cb component, and be properly weighted and then added to the Cr component. In this test, filters are indicated with a block size, which is signaled per slice.

The supported such chroma block sizes range from the minimum chroma CTU size to the current chroma CTU size. The minimum chroma CTU size is the minimum between the smallest possible width and height of a chroma CTU, i.e. Min( 32 / SubWidthC, 32 / SubHeightC ), while the current chroma CTU size is the minimum between the width and height of the current chroma CTU, i.e. Min( CtbWidthC, CtbHeightC ). For example, if CTU size is set to the maximal 128x128, the JC-CCALF chroma block size of a slice will be one from 32x32, 64x64 and 128x128 for 4:4:4 video, or one from 16x16, 32x32 and 64x64 for 4:2:0 and 4:2:2 video.

## CE5-2: Complexity reduction in filtering process

### CE5-2.1: Multiplication removal for cross component adaptive loop filter (JVET-P0557)

Filter coefficients are restricted to have values: {-64, -32, -16, -8, -4, -2, -1, 0, 1, 2, 4, 8, 16, 32, 64}, which are signalled as {-7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 7} respectively before binarization in order to reduce the signalling overhead. Fixed length codes of 3 bits are used to signal the absolute values of the coefficients. Another sign bit is signalled for non-zero coefficients.

### CE5-2.2: Multiplication removal for cross component adaptive loop filter (JVET-P0666)

Filter coefficients are restricted to have values: {-8, -4, -2, -1, 0, 1, 2, 4, 8}. A bit is first signalled to indicate whether it is a zero coefficient. If not, a fixed length code of 2 bits is used to signal the absolute value of the coefficient followed by a sign bit.

## CE5-3: Combination Tests

### CE5-3.1: Combination of CE5-2.1 and 5x5 diamond filter used in chroma ALF (JVET-P0558)

Alternative filter shape with multiplication removal for CCALF

|  |  | f0 |  |  |
| --- | --- | --- | --- | --- |
|  | f1 | f2 | f3 |  |
| f4 | f5 | f6 | f7 | f8 |
|  | f9 | f10 | f11 |  |
|  |  | f12 |  |  |

The 5x5 diamond filter shape will be used for CCALF on top of the tests in CE5-2.1. Filter coefficients are restricted to have values: {-64, -32, -16, -8, -4, -2, -1, 0, 1, 2, 4, 8, 16, 32, 64}, which are signalled as {-7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 7} respectively before binarization in order to reduce the signalling overhead. Fixed length codes of 3 bits are used to signal the absolute values of the coefficients. Another sign bit is signalled for non-zero coefficients.

### CE5-3.2: Combination of CE5-1.4 and CE5-2.1

### Proposed during the CE editing period, the Joint Cb/Cr CC-ALF with one additional weight and CTU-level selection method in CE5-1.4 is combined the multiplication removal with coefficient range in [-64,64] method in CE5-2.1.

# Non-CE5/CE5-related contributions

## CC-ALF (x)

| JVET Number | Description | Tag |
| --- | --- | --- |
| JVET-Q0124 | CE5-related: JC-CCALF with Power of 2 Weight Values |  |
| JVET-Q0125 | CE5-related: JC-CCALF with Alternative Filter Weight |  |
| JVET-Q0165 | CE5-related: On the CC-ALF filtering process |  |
| JVET-Q0190 | CE5-related: On CC-ALF modifications related to coefficients and signalling |  |
| JVET-Q0250 | CE5-related: Removing number of filters for CC-ALF in slice and picture header |  |
| JVET-Q0251 | CE5-related: Unified cross component adaptive loop filter |  |
| JVET-Q0253 | CE5-related: High level syntax modifications for CCALF |  |
| JVET-Q0296 | Non-CE5: Adaptive precision for CCALF coefficients |  |
| JVET-Q0301 | CE5-related: CCALF filter for 4:2:2 and 4:4:4 color format |  |
| JVET-Q0304 | Non-CE5: Non-Linear Cross Component Adaptive Loop Filter |  |
| JVET-Q0310 | Non-CE5: Suggested text for CC-ALF padding process with raster scan slices |  |
| JVET-Q0311 | Non-CE5: On CC-ALF padding for ALF virtual boundaries |  |
| JVET-Q0326 | CE5-related: On CC-ALF slice and picture header syntax |  |
| JVET-Q0382 | CE5-related: On high level syntax of CC-ALF |  |
| JVET-Q0467 | CE5-related: Simplified CCALF |  |

## Others (x)

| JVET Number | Description | Tag |
| --- | --- | --- |
| JVET-Q0167 | Non-CE5: Multiplication simplification for ALF and CC-ALF |  |
| JVET-Q0319 | Non-CE5: On SEI for ALF |  |
| JVET-Q0378 | Non-CE5: On the number of ALF Chroma filters. |  |

# Recommendations

Discuss CE5 tests and review all Non-CE5/CE5-related contributions.