| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0041-v1 |
| --- | --- |

| Title: | AHG2: Editorial input on VVC draft text |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposed draft |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Bytedance, San Diego, CA, USA Benjamin Bross HHI, Berlin, Germany Virginie Drugeon Panasonic, Langen, Germany | Email: | yekui.wang@bytedance.com benjamin.bross@hhi.fraunhofer.de virginie.drugeon@eu.panasonic.com |
| Source: | AHG2 |  |  |

_____________________________

# Abstract

This document provides integrations of a number of bug fixes reported in the bug tracking system as well as some other editorial changes made by the authors, based on the latest JVET output draft VVC text in JVET-P2001-vE.

It is suggested to use the attached spec text as the basis for integration of adoptions of the 17th JVET meeting. And proponents are encouraged to use this text as the basis for their proposed text changes.

List of logged changes:

- Fixed the bugs #672, #700, #711, #715, #716, #720, #729, #737, #739.

- Fixed subpicture figure together with accompanying text (#740).

- Removed the editing note comment in the semantics of tile_idx_delta[ i ], which was concluded not needed after a study (input: Hendry).

- Fixed quite some format and style consistency issues in syntax tables, equations, etc. as well as some bugs in those places.

- Made some fixes in the semantics of DPS and VPS.

- Fixed the semantics of sps_loop_filter_across_virtual_boundaries_disabled_present_flag and ph_loop_filter_across_virtual_boundaries_disabled_present_flag, and changed the syntax element names to sps_virtual_boundaries_present_flag and ph_virtual_boundaries_present_flag.

- Fixed the following issues:

  - The definition of the sub-bitstream extraction process still referred to a target LayerId: "A specified process by which NAL units in a bitstream that do not belong to a target set, determined by a target OLS index and a target highest TemporalId and a target LayerId, are removed from the bitstream, …". However, the process in clause C.6 only mentions a target OLS index and a target highest TemporalId in the inputs, but no target LayerId.

  - In the VPS syntax: "ols_hrd_parameters( firstSubLayer, hrd_max_temporal_id[ i ] )". hrd_max_temporal_id is not defined anywhere. It should be "hrd_max_tid" instead.

  - In the VPS semantics, when deriving LayerUsedAsRefLayerFlag, it was initialised as follows: "LayerUsedAsRefLayerFlag[ j ] = 0." But j is not defined at this point, since it gets defined in the following loop.

  - Fixed a bug in Equation 40 by changing "layerIncludedFlag[ i ][ j ] = 1" to "layerIncludedFlag[ i ][ k ] = 1" and then changed the variable name to layerIncludedInOlsFlag[ ][ ] to be more intuitive.

  - In the semantics of dpb_size_only_flag, only the value 1 was specified, twice, and in a contradicting way. The second sentence should be: "dpb_size_only_flag[ i ] equal to 0 specifies that…".

  - In the semantics of num_ols_hrd_params_minus1: "When TotalNumOlss is greater than 1, the value of num_ols_hrd_params_minus1 is inferred to be equal to 0". However, num_ols_hrd_params_minus1 is signalled when TotalNumOlss is greater than 1. It should be "When TotalNumOlss is equal to 1, the value of num_ols_hrd_params_minus1 is inferred to be equal to 0".

  - In the semantics of hrd_max_tid, there should be an inference for this syntax element when the value of vps_max_sublayers_minus1 is greater than 0, but vps_all_layers_same_num_sublayers_flag is 1, similarly as in the semantics of ptl_max_temporal_id[ i ] and dpb_max_temporal_id[ i ].

  - In the semantics of sps_ptl_dpb_hrd_params_present_flag: "The value of sps_ptl_dpb_hrd_params_present_flag shall be equal to vps_independent_layer_flag[ nuh_layer_id ]." Everywhere else vps_independent_layer_flag is indexed by the layer index rather than the layer identifier. It should be the following instead: "The value of sps_ptl_dpb_hrd_params_present_flag shall be equal to vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ]."

  - Annex A still mentioned bit_depth_luma_minus8 and bit_depth_chroma_minus8, although these two syntax elements have been replaced by bit_depth_minus8.

  - Annex A still mentioned num_tile_columns_minus1 and num_tile_rows_minus1, although these syntax elements do not exist anymore.

  - The list NestingLayerId[ ] and the variable NestingNumLayers specified in the semantics of the scalable nesting SEI message are not used in other places, thus two variable names were changed to be local variables, i.e., nestingLayerId[ ] and nestingNumLayers.