| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0042 |
| --- | --- |

| Title: | AHG9: On Video Usability Information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Virginie Drugeon | Tel: Email: | [+49-6103-766240 virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

The VUI syntax structure contains essential information for proper display of the video. The current specification text contains several comments on the VUI syntax and semantics. Suggested fixes for three of these comments are proposed in the present document. Specifically, it is proposed to

- Move the overscan information to immediately follow the aspect ratio information;

- Infer that the chroma sample location type is unspecified when not present;

- Use the two flags general_progressive_source_flag and general_interlaced_source_flag as condition to switch between signalling of chroma_sample_loc_type_frame and chroma_sample_loc_type_top_field/chroma_sample_loc_type_bottom_field.

# Introduction

The information contained in the VUI (Video Usability Information) syntax structure is not required for decoding the luma and chroma samples of the video signal. However, some of the information is essential for proper rendering of the signal on a display and is becoming even more essential in today’s world, where video can be HDR or SDR, use the SD, HD or UHD colour primaries, use the BT. 709 or the BT. 2020 chroma samples location with respect to luma samples location. Given how essential this information is to proper rendering of the video content, some application standards, such as DVB in ETSI TS 101 154 [1] mandates the presence of the VUI syntax structure and the presence and value of some of its syntax element for previous video coding standards (AVC and HEVC).

There are several comments within the specification text of the current VUI semantics for VVC. In particular, the present document intends to answer the three comments below:

- [Ed. (GJS) Since overscan information is most closely related to aspect ratio information, it would probably make more sense for these two flags to immediately follow the aspect ratio syntax rather than be located after other additional syntax elements.]

- [Ed. (GJS): In general, there should be some allowance for VUI parameters to be determined by external means when they are not present in the bitstream. This may not be a problem for some parameters that have inferred values that indicate that they are unspecified, but others do not seem to have a similar provision, such as the chroma sample location indicators. For example, it is not possible to specify the use of Rec. ITU-R BT.2020 or Rec. ITU-R BT.2100 video by external means, because these semantics say that if the VUI is not present, the chroma_sample_loc_type_frame is inferred to be 0, which is contrary to what those standards specify for 4:2:0 chroma, as mentioned below.]

- [Ed. (GJS): I don't know how the ability to remove distinct per-field chroma types was removed for the case with field_seq_flag is equal to 0, but this removes the capability to signal distinct chroma sample types for the two fields of a frame, which seems strange, as it appears to force all sequences to be coded as fields if the chroma sample type is different for the two fields. Ordinarily, we would not force particular methods of coding in order to enable metadata indications.]
[Ed.: Further study encouraged on whether to align with HEVC and AVC, by simply signalling chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field, regardless of the value of field_seq_flag.]

# Overscan information (comment #1)

It is suggested to move the overscan information to immediately follow the aspect ratio syntax, as suggested in the comment.

# Inference when chroma sample indicators are not present (comment #2)

The author of the present document agrees with the comment. It has created issues in the past with HEVC whereby some content providers did not signal the chroma location of content using location type 0. Since this is bad practice, it is suggested to make the chroma location unspecified when this information is not present.

The following modifications to the specification text are suggested:

When chroma_sample_loc_type_top_field, and chroma_sample_loc_type_bottom_field and chroma_sample_loc_type_frame are not present, the values of chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field are inferred to be equal tochroma_sample_loc_type_frame are inferred to be unspecified.

# Chroma sample location (comment #3)

Signalling chroma sample location for progressive content has become essential now that recommendations BT. 2020 and BT. 2100 use the type 2 location, instead of the type 0 location used for previous formats. Unfortunately, in bitstreams conforming to previous video coding standards, this information tended to be forgotten. One reason may have been that the name of the fields (chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field) tended to give the impression that this was an information relevant for interlaced content only. In addition, it is unclear how the display should react to different values of these two fields for progressive content.

The author of the present document therefore believes that having only one chroma location type field for progressive content would greatly clarify the situation. However, the author of this document also agrees that field_seq_flag is the wrong information to determine if two chroma location type fields or only one are necessary, since interlaced content can be encoded both as fields or frames. It is therefore suggested to use the two flags general_progressive_source_flag and general_interlaced_source_flag instead. These two flags are currently signalled within the general_constraint_info syntax structure. Since these two flags convey information about the encoded video signal rather than the bitstream (contrary to all the other flags from the general_constraint_info syntax structure), it is suggested to move these flags to the VUI instead, and use them to switch between signalling of chroma_sample_loc_type_frame and chroma_sample_loc_type_top_field/chroma_sample_loc_type_bottom_field.

It is suggested to modify the VUI semantics as follows:

general_progressive_source_flag and general_interlaced_source_flag are interpreted as follows:

–	If general_progressive_source_flag is equal to 1 and general_interlaced_source_flag is equal to 0, the source scan type of the pictures should be interpreted as progressive only.

–	Otherwise, if general_progressive_source_flag is equal to 0 and general_interlaced_source_flag is equal to 1, the source scan type of the pictures should be interpreted as interlaced only.

–	Otherwise, if general_progressive_source_flag is equal to 0 and general_interlaced_source_flag is equal to 0, the source scan type of the pictures should be interpreted as unknown or unspecified.

- Otherwise (general_progressive_source_flag is equal to 1 and general_interlaced_source_flag is equal to 1), the source scan type of the pictures should be interpreted as unknown or unspecified.

[…]

chroma_sample_loc_type_frame, chroma_sample_loc_type_top_field, and chroma_sample_loc_type_bottom_field specify the location of chroma samples as follows:

–	If field_seq_flag is equal to 0 general_progressive_source_flag is equal to 1 and general_interlaced_source_flag is equal to 0 and chroma_format_idc is equal to 1 (4:2:0 chroma format), chroma_sample_loc_type_frame specifies the location of chroma samples for both fields of each frame of the CLVS as shown in Figure 1.

–	Otherwise, if field_seq_flag is equal to 1 and chroma_format_idc is equal to 1 (4:2:0 chroma format), chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field specify the location of chroma samples for each top field and bottom field of the CLVS, respectively, as shown in Figure 1.

–	Otherwise (chroma_format_idc is not equal to 1), the values of the syntax elements chroma_sample_loc_type, chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field shall be ignored. When chroma_format_idc is equal to 2 (4:2:2 chroma format) or 3 (4:4:4 chroma format), the location of chroma samples is specified in clause 6.2 of Rec. ITU-T H.VVC | ISO/IEC 23090-3. When chroma_format_idc is equal to 0, there is no chroma sample array.

[…]

When chroma_format_idc is equal to 1 (4:2:0 chroma format) and the decoded video content is intended for interpretation according to Rec. ITU-R BT.2020 or Rec. ITU-R BT.2100, chroma_loc_info_present_flag should be equal to 1, general_progressive_source_flag should be equal to 1, general_interlaced_source_flag should be equal to 0 and chroma_sample_loc_type_frame, chroma_sample_loc_type_top_field, and chroma_sample_loc_type_bottom_field (as applicable) should be equal to 2.

# Proposed VUI syntax

The VUI syntax after addressing the comments as suggested above would be as follows:

| vui_parameters( ) { | Descriptor |
| --- | --- |
| aspect_ratio_info_present_flag | u(1) |
| if( aspect_ratio_info_present_flag ) { |  |
| aspect_ratio_constant_flag | u(1) |
| aspect_ratio_idc | u(8) |
| if( aspect_ratio_idc = = 255 ) { |  |
| sar_width | u(16) |
| sar_height | u(16) |
| } |  |
| } |  |
| overscan_info_present_flag | u(1) |
| if( overscan_info_present_flag ) |  |
| overscan_appropriate_flag | u(1) |
| colour_description_present_flag | u(1) |
| if( colour_description_present_flag ) { |  |
| colour_primaries | u(8) |
| transfer_characteristics | u(8) |
| matrix_coeffs | u(8) |
| full_range_flag | u(1) |
| } |  |
| general_progressive_source_flag | u(1) |
| general_interlaced_source_flag | u(1) |
| chroma_loc_info_present_flag | u(1) |
| if( chroma_loc_info_present_flag ) { |  |
| if(general_progressive_source_flag && !general_interlaced_source_flag !field_seq_flag ) |  |
| chroma_sample_loc_type_frame | ue(v) |
| else { |  |
| chroma_sample_loc_type_top_field | ue(v) |
| chroma_sample_loc_type_bottom_field | ue(v) |
| } |  |
| } |  |
| overscan_info_present_flag | u(1) |
| if( overscan_info_present_flag ) |  |
| overscan_appropriate_flag | u(1) |
| } |  |

# References

- ETSI TS 101 154 “Digital Video Broadcasting (DVB); Specification for the use of Video and Audio Coding in Broadcast and Broadband Applications” https://www.etsi.org/deliver/etsi_ts/101100_101199/101154/02.06.01_60/ts_101154v020601p.pdf

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).