| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0043 |
| --- | --- |

| Title: | AHG9: Constraint about usage of reference picture resampling and subpictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takahiro Nishi Kiyofumi Abe Virginie Drugeon | Tel: Email: | +49-6103-766240 nishi.takahiro@jp.panasonic.com abe.kiyo@jp.panasonic.com virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

While reference picture resampling and subpictures are not meant to be enabled together in a bitstream, the current VVC draft does not clearly mention any such constraint in the SPS semantics, which may lead to inconsistent behaviour when both ref_pic_resampling_enabled_flag and subpics_present_flag are set to 1. It is therefore suggested to include a constraint to prevent such a signalling.

# Problem Statement

The current VVC draft text supports both reference picture resampling and subpictures, but not together (i.e. when RPR is enabled, subpictures are disabled).

At the last meeting, a flag was introduced to signal the usage of RPR at the SPS level: ref_pic_resampling_enabled_flag. Unfortunately, no constraint was added to the value of subpics_present_flag, so that both flags could theoretically be set to 1 in an SPS. Should that happen, RPR would still be technically disabled because of the following constraints in the semantics of the PPS: “When subpics_present_flag is equal to 1 or ref_pic_resampling_enabled_flag equal to 0, the value of pic_width_in_luma_samples shall be equal to pic_width_max_in_luma_samples. When subpics_present_flag is equal to 1 or ref_pic_resampling_enabled_flag equal to 0, the value of pic_height_in_luma_samples shall be equal to pic_height_max_in_luma_samples.” Unfortunately, setting ref_pic_resampling_enabled_flag to 1 even when RPR is not used means that scaling window offsets may be signalled (scaling_window_flag equal to 1), which would lead to inconsistent behaviour if subpictures are also used.

# Additional constraint in semantics

In order to clarify within the SPS signalling that subpictures and RPR cannot be used together, it is suggested to add either of the following constraints in the SPS semantics:

When ref_pic_resampling_enabled_flag is equal to 1, the value of subpics_present_flag shall be equal to 0.

or

When subpics_present_flag is equal to 1, the value of ref_pic_resampling_enabled_flag shall be equal to 0.

Together with above, the following changes in the PPS semantics are suggested:

pic_width_in_luma_samples specifies the width of each decoded picture referring to the PPS in units of luma samples. pic_width_in_luma_samples shall not be equal to 0, shall be an integer multiple of Max( 8, MinCbSizeY ), and shall be less than or equal to pic_width_max_in_luma_samples.

When subpics_present_flag is equal to 1 or ref_pic_resampling_enabled_flag equal to 0, the value of pic_width_in_luma_samples shall be equal to pic_width_max_in_luma_samples.

pic_height_in_luma_samples specifies the height of each decoded picture referring to the PPS in units of luma samples. pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of Max( 8, MinCbSizeY ), and shall be less than or equal to pic_height_max_in_luma_samples.

When subpics_present_flag is equal to 1 or ref_pic_resampling_enabled_flag equal to 0, the value of pic_height_in_luma_samples shall be equal to pic_height_max_in_luma_samples.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).