| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0045 |
| --- | --- |

| Title: | AHG9: On DPS identifier |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Virginie Drugeon | Tel: Email: | +49-6103-766240 virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

It is suggested to remove the possibility to have DPS identifiers, since the DPS is not allowed to change within a whole bitstream and has no influence on the decoding process.

Two proposals are made to identify if an SPS refers to a DPS:

- If a DPS NAL unit is present within a bitstream or a DPS is available by other means, the SPS refers to the information signalled in that DPS.

OR

- Replace the sps_decoding_parameter_set_id by an sps_decoding_parameter_set_flag to signal if the SPS refers to any DPS.

# Introduction

The DPS (Decoding Parameter Set) was adopted to express a worst-case capability for the whole bitstream. As such, it is never allowed to change within a bitstream. The DPS contains information to signal properties about the bitstream only and has no influence on the actual decoding process or even the interpretation of the data within the bitstream. It can therefore be safely ignored by a decoder that does not require this information.

The DPS is currently referred in the same way as all the other parameter sets specified in VVC. However, since the Decoding Parameter Set is never allowed to change within a bitstream and only signals worst-case capabilities that have no influence on the decoding process, it is rather confusing to be able to signal many different DPS identifiers (dps_decoding_parameter_set_id), although only one is allowed to be referred per bitstream.

It is therefore suggested to remove this confusion and simplify the specification by removing the dps_decoding_parameter_set_id. Two mutually exclusive different strategies are proposed to identify if an SPS refers to a DPS or not within a bitstream:

- If a DPS NAL unit is present within a bitstream or a DPS is available by other means, the SPS refers to the information signalled in that DPS NAL unit.

OR

- Replace the sps_decoding_parameter_set_id by an sps_decoding_paremeter_set_flag to signal if the SPS refers to any DPS.

# Proposal 1

It is suggested to completely remove both the dps_decoding_parameter_set_id and the sps_decoding_parameter_set_id. If a DPS NAL unit is present within a bitstream or a DPS is available by other means, then the corresponding DPS is referred to by the SPS and the information that it carries is not allowed to change within the bitstream. Otherwise, no DPS is referred to by this bitstream.

# Proposal 2

The dps_decoding_parameter_set_id is removed and the sps_decoding_parameter_set_id is replaced by a flag: sps_decoding_parameter_set_flag. When this flag is equal to 0, no DPS is referred (same as in the current VVC draft). When this flag is equal to 1, a DPS must be available and is referred.

# Conclusion

It is suggested to remove the possibility to have DPS identifiers, since it falsely gives the impression that the DPS has the same status as other parameter sets, in the sense that a bitstream may carry different sets of DPS information. However, this is not the case, since the DPS is not allowed to change within a whole bitstream and has no influence on the decoding process.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).