| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0046 |
| --- | --- |

| Title: | AHG9: On signalling of PTL/HRD parameters for single layer OLSs and DPB parameters for independent layers |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takahiro Nishi Kiyofumi Abe Virginie Drugeon | Tel: Email: | +49-6103-766240 nishi.takahiro@jp.panasonic.com abe.kiyo@jp.panasonic.com virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

In the current draft specification, PTL parameters and HRD parameters are not signalled in the VPS for OLSs which have only one layer since the PTL and HRD parameters signalled in the corresponding SPS are used instead. Similarly, DPB parameters from the VPS are not used for independent layers. In order to know the characteristics of the whole multi-layer bitstream by parsing only the VPS, it is proposed to signal, in the VPS, identical PTL/DPB/HRD parameters as in the corresponding SPS for single layer OLS and independent layers.

# Introduction

In the current draft specification, PTL (Profile, Tier, Level) parameters and HRD (Hypothetical Reference Decoder) parameters are not signalled in the VPS for OLSs which have only one layer. Instead the PTL and HRD parameters signalled in the corresponding SPS are used. Similarly, the DPB (Decoded Picture Buffer) parameters from the VPS are not used for independent layers. Therefore, tracing parameter set ID from PH to SPS is required to know the characteristics of the whole multi-layer bitstream. The authors of the present contribution believe that there is benefit in being able to identify the PTL characteristics and the DPB and HRD parameters for a whole multi-layer bitstream using only the VPS.

The present document proposes to signal, in the VPS, identical PTL/DPB/HRD parameters as in the corresponding SPS for single layer OLS and independent layers, as well as the corresponding indices of those syntax structures.

# Proposed changes

Proposed changes over the draft specification are presented below.

7.3.2.2	Video parameter set syntax

| video_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| … |  |  |
| if( vps_num_ptls > 1 ) |  |  |
| for( i = 0; i < TotalNumOlss; i++ ) |  |  |
| if( NumLayersInOls[ i ] > 1 && vps_num_ptls > 1 ) |  |  |
| ols_ptl_idx[ i ] | u(8) |  |
| … |  |
| if( vps_num_dpb_params > 1 ) |  |  |
| for( i = 0; i <= vps_max_layers_minus1 && vps_num_dpb_params > 1; i++ ) { |  |  |
| if( !vps_independent_layer_flag[ i ] ) |  |  |
| layer_output_dpb_params_idx[ i ] | ue(v) |  |
| if( LayerUsedAsRefLayerFlag[ i ] && !same_dpb_size_output_or_nonoutput_flag ) |  |  |
| layer_nonoutput_dpb_params_idx[ i ] | ue(v) |  |
| } |  |
| vps_general_hrd_params_present_flag | u(1) |  |
| if( vps_general_hrd_params_present_flag ) { |  |  |
| … |  |  |
| for( i = 0; i <= num_ols_hrd_params_minus1; i++ ) { |  |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| hrd_max_tid[ i ] | u(3) |  |
| firstSubLayer = vps_sublayer_cpb_params_present_flag ? 0 : hrd_max_tid[ i ] |  |  |
| ols_hrd_parameters( firstSubLayer, hrd_max_tid[ i ]hrd_max_temporal_id[ i ] ) |  |  |
| } |  |  |
| if( num_ols_hrd_params_minus1 > 0 ) |  |  |
| for( i = 0 i = 1; i < TotalNumOlss; i++ ) |  |  |
| ols_hrd_idx[ i ] | ue(v) |  |
| } |  |  |
| … |  |  |
| } |  |  |

7.4.3.2	Video parameter set RBSP semantics

ols_ptl_idx[ i ] specifies the index, to the list of profile_tier_level( ) syntax structures in the VPS, of the profile_tier_level( ) syntax structure that applies to the i-th OLS. When present, the value of ols_ptl_idx[ i ] shall be in the range of 0 to vps_num_ptls − 1, inclusive.

When NumLayersInOls[ i ] is equal to 1, the profile_tier_level( ) syntax structure that applies to the i-th OLS is present in both the VPS and the SPS referred to by the layer in the i-th OLS, and it is a requirement of bitstream conformance that the profile_tier_level( ) syntax structure signalled in the VPS and SPS for the i-th OLS shall be identical.

layer_output_dpb_params_idx[ i ] specifies the index, to the list of dpb_parameters( ) syntax structures in the VPS, of the dpb_parameters( ) syntax structure that applies to the i-th layer when it is an output layer in an OLS. When present, the value of layer_output_dpb_params_idx[ i ] shall be in the range of 0 to vps_num_dpb_params − 1, inclusive.

When vps_num_dpb_params is equal to 1, the value of layer_output_dpb_params_idx[ i ] is inferred to be equal to 0.

If vps_independent_layer_flag[ i ] is equal to 1, the dpb_parameters( ) syntax structure that applies to the i-th layer when it is an output layer is the dpb_parameters( ) syntax structure is present in both the VPS and the SPS referred to by the layer, and it is a requirement of bitstream conformance that the dpb_parameters( ) syntax structure signalled in the VPS and SPS for the i-th layer shall be identical.

Otherwise (vps_independent_layer_flag[ i ] is equal to 0), the following applies:

- When vps_num_dpb_params is equal to 1, the value of layer_output_dpb_params_idx[ i ] is inferred to be equal to 0.

- It is a requirement of bitstream conformance that the value of layer_output_dpb_params_idx[ i ] shall be such that dpb_size_only_flag[ layer_output_dpb_params_idx[ i ] ] is equal to 0.

ols_hrd_idx[ i ] specifies the index of the ols_hrd_parameters( ) syntax structure that applies to the i-th OLS. The value of ols_hrd_idx[[ i ] shall be in the range of 0 to num_ols_hrd_params_minus1, inclusive. When not present, the value of ols_hrd_idx[[ i ] is inferred to be equal to 0. When NumLayersInOls[ i ] is equal to 1, the general_hrd_parameters( ) syntax structure and the ols_hrd_parameters( ) syntax structure that apply to the i-th OLS are present in both the VPS and the SPS referred to by the layer in the i-th OLS, and it is a requirement of bitstream conformance that the general_hrd_parameters( ) syntax structure and the ols_hrd_parameters( ) syntax structure signalled in the VPS and SPS for the i-th OLS shall be identical.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).