| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0047 |
| --- | --- |

| Title: | AHG9: Unified signalling of PTL and HRD parameters in VPS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takahiro Nishi Kiyofumi Abe Virginie Drugeon | Tel: Email: | +49-6103-766240 nishi.takahiro@jp.panasonic.com abe.kiyo@jp.panasonic.com virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

Unified signalling of profile_tier_level( ) and ols_hrd_parameters( ) syntax structures in the VPS is proposed. PTL and HRD share the following syntax elements in the VPS: 1) number of syntax structures, 2) index of the syntax structure that each OLS applies and 3) max temporal ID.

# Introduction

In the current draft specification, several profile_tier_level( ) syntax structures and several ols_hrd_parameters( ) syntax structures are independently signalled in the VPS. These syntax structures are then assigned to individual OLSs (output layer sets). For each OLS containing more than one layer, an ols_ptl_idx signals which profile_tier_level( ) syntax structure applies to that OLS and an ols_hrd_idx signals which ols_hrd_parameters( ) syntax structure applies to that OLS. The benefit of having a different number of profile_tier_level( ) syntax structures and ols_hrd_parameters( ) syntax structures is currently unclear for practical use cases, so that it is possible to simplify the signalling by unification.

The present document proposes to unify the signalling of profile_tier_level( ) syntax structures and ols_hrd_parameters( ) syntax structures in the VPS. PTL and HRD syntax structures share the following syntax elements: 1) number of syntax structures ( vps_num_ptls and num_ols_hrd_params_minus1 ), 2) index of the syntax structure that each OLS applies ( ols_ptl _idx[ ] and ols_hrd_idx[ ] ) and 3) max temporal ID ( ptl_hrd_max_temporal_id[ ] and hrd_max_tid[ ] ) as described in the following section.

In most practical use cases, different PTL and HRD syntax structures would be required for each OLS. It is therefore asserted that the proposed unified signalling does not harm the flexibility of the current signalling for practical use cases, while simplifying the syntax of the VPS.

In addition, the proposed changes solve a redundancy of the current signalling, whereby an ols_hrd_idx specifying an index to an ols_hrd_parameters( ) syntax structure of the VPS is signalled for OLSs containing only one layer, although Annex C specifies that in such a case, the ols_hrd_parameters( ) syntax structure signalled in the SPS is used instead. Unification with the signalling of the profile_tier_level( ) syntax structure removes this redundancy that currently does not exist for the PTL syntax structure.

# Proposed changes

Proposed changes over the draft specification are presented below.

7.3.2.2	Video parameter set syntax

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| vps_num_ptls_hrds | u(8) |
| for( i = 0; i < vps_num_ptls_hrds; i++ ) { |  |
| if( i > 0 ) |  |
| pt_present_flag[ i ] | u(1) |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |
| ptl_hrd_max_temporal_id[ i ] | u(3) |
| } |  |
| while( !byte_aligned( ) ) |  |
| vps_ptl_byte_alignment_zero_bit /* equal to 0 */ | u(1) |
| for( i = 0; i < vps_num_ptls_hrds; i++ ) |  |
| profile_tier_level( pt_present_flag[ i ], ptl_hrd_max_temporal_id[ i ] ) |  |
| if( vps_num_ptls_hrds > 1 ) |  |
| for( i = 01; i < TotalNumOlss; i++ ) |  |
| if( NumLayersInOls[ i ] > 1 && vps_num_ptls > 1 ) |  |
| ols_ptl_hrd_idx[ i ] | u(8) |
| … |  |
| vps_general_hrd_params_present_flag | u(1) |
| if( vps_general_hrd_params_present_flag ) { |  |
| general_hrd_parameters( ) |  |
| if( vps_max_sublayers_minus1 > 0 ) |  |
| vps_sublayer_cpb_params_present_flag | u(1) |
| if( TotalNumOlss > 1 ) |  |
| num_ols_hrd_params_minus1 | ue(v) |
| for( i = 0; i < vps_num_ptls_hrds <= num_ols_hrd_params_minus1; i++ ) { |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |
| hrd_max_tid[ i ] | u(3) |
| firstSubLayer = vps_sublayer_cpb_params_present_flag ? 0 : ptl_hrd_max_temporal_id[ i ] hrd_max_tid[ i ] |  |
| ols_hrd_parameters( firstSubLayer, ptl_hrd_max_temporal_id[ i ] ) |  |
| } |  |
| if( num_ols_hrd_params_minus1 > 0 ) |  |
| for( i = 1; i < TotalNumOlss; i++ ) |  |
| ols_hrd_idx[ i ] | ue(v) |
| } |  |
| … |  |
| } |  |

7.4.3.2	Video parameter set RBSP semantics

vps_num_ptls_hrds specifies the number of profile_tier_level( ) and ols_hrd_parameters( ) syntax structures in the VPS.

ptl_hrd_max_temporal_id[ i ] specifies the common TemporalId of the highest sublayer representation for which the level information is present in the i-th profile_tier_level( ) syntax structure and for which the HRD parameters are contained in the i-th ols_hrd_parameters( ) syntax structure in the VPS. The value of ptl_hrd_max_temporal_id[ i ] shall be in the range of 0 to vps_max_sublayers_minus1, inclusive. When vps_max_sublayers_minus1 is equal to 0, the value of ptl_max_temporal_id[ i ] is inferred to be equal to 0. When vps_max_sublayers_minus1 is greater than 0 and or vps_all_layers_same_num_sublayers_flag is equal to 1, the value of ptl_hrd_max_temporal_id[ i ] is inferred to be equal to vps_max_sublayers_minus1.

ols_ptl_hrd_idx[ i ] specifies the index, to the list of profile_tier_level( ) and ols_hrd_parameters( ) syntax structures in the VPS, of the profile_tier_level( ) and ols_hrd_parameters( ) syntax structure that applies to the i-th OLS. When present, the value of ols_ptl_hrd_idx[ i ] shall be in the range of 0 to vps_num_ptls_hrds − 1, inclusive.

When NumLayersInOls[ i ] is equal to 1, the profile_tier_level( ) syntax structure, the general_hrd_parameters( ) syntax structure and the ols_hrd_parameters( ) syntax structure that applies to the i-th OLS areis present in the SPS referred to by the layer in the i-th OLS.

vps_sublayer_cpb_params_present_flag equal to 1 specifies that the i-th ols_hrd_parameters( ) syntax structure in the VPS contains HRD parameters for the sublayer representations with TemporalId in the range of 0 to ptl_hrd_max_temporal_idhrd_max_tid[ i ], inclusive. vps_sublayer_cpb_params_present_flag equal to 0 specifies that the i-th ols_hrd_parameters( ) syntax structure in the VPS contains HRD parameters for the sublayer representation with TemporalId equal to ptl_hrd_max_temporal_idhrd_max_tid[ i ] only. When vps_max_sublayers_minus1 is equal to 0, the value of vps_sublayer_cpb_params_present_flag is inferred to be equal to 0.

When vps_sublayer_cpb_params_present_flag is equal to 0, the HRD parameters for the sublayer representations with TemporalId in the range of 0 to ptl_hrd_max_temporal_idhrd_max_tid[ i ] − 1, inclusive, are inferred to be the same as that for the sublayer representation with TemporalId equal to ptl_hrd_max_temporal_idhrd_max_tid[ i ]. These include the HRD parameters starting from the fixed_pic_rate_general_flag[ i ] syntax element till the sublayer_hrd_parameters( i ) syntax structure immediately under the condition "if( general_vcl_hrd_params_present_flag )" in the ols_hrd_parameters syntax structure.

num_ols_hrd_params_minus1 plus 1 specifies the number of ols_hrd_parameters( ) syntax structures present in the general_hrd_parameters( ) syntax structure. The value of num_ols_hrd_params_minus1 shall be in the range of 0 to 63, inclusive. When TotalNumOlss is greater than 1, the value of num_ols_hrd_params_minus1 is inferred to be equal to 0.

hrd_max_tid[ i ] specifies the TemporalId of the highest sublayer representation for which the HRD parameters are contained in the i-th ols_hrd_parameters( ) syntax structure. The value of hrd_max_tid[ i ] shall be in the range of 0 to vps_max_sublayers_minus1, inclusive. When vps_max_sublayers_minus1 is equal to 0, the value of hrd_max_tid[ i ] is inferred to be equal to 0.

ols_hrd_idx[ i ] specifies the index of the ols_hrd_parameters( ) syntax structure that applies to the i-th OLS. The value of ols_hrd_idx[[ i ] shall be in the range of 0 to num_ols_hrd_params_minus1, inclusive. When not present, the value of ols_hrd_idx[[ i ] is inferred to be equal to 0.

C.1	General

- The general_hrd_parameters( ) syntax structure, the ols_hrd_parameters( ) syntax structure, and the sublayer_hrd_parameters( ) syntax structure applicable to BitstreamToDecode are selected as follows:

- If NumLayersInOls[ TargetOlsIdx ] is equal to 1, the general_hrd_parameters( ) syntax structure and the ols_hrd_parameters( ) syntax structure in the SPS referenced by the layer in BitstreamToDecode (or provided through an external means not specified in this Specification) are selected. Otherwise, the general_hrd_parameters( ) syntax structure and the ols_ptl_hrd_idx[ TargetOlsIdx ]-th ols_hrd_parameters( ) syntax structure in the VPS (or provided through an external means not specified in this Specification) are selected.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).