| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0049 |
| --- | --- |

| Title: | Bit Stream Feature Analyzer (BSFA) for Coding Tool Statistics based on VTM-7.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christian Herglotz Matthias Kränzler André Kaup | Tel: Email: | +49 9131 8527101 christian.herglotz@fau.de |
| Source: | Multimedia Communications and Signal Processing, Friedrich-Alexander University Erlangen-Nürnberg (FAU) |  |  |

_____________________________

# Abstract

This contribution presents an analyzer add-on for the VTM-7.0 decoder. For any given VTM-7.0 encoded bit stream, the analyzer counts the occurrence of certain coding tools and coding modes. Examples are tools like intra prediction, transformations, residual coefficients, and in-loop filtering operations on slice level, block level, or pel level. The authors report that the tool is lightweight in terms of complexity and that the results of the analyzer can be used for further interpretation and assessment of coding tools.

# Introduction

During the last JVET meeting, an analyzer was proposed that is capable of counting the frequency of occurrence of a large variety of different coding modes and coding tools. For this meeting, the analyzer was updated to VTM-7.0 and extended with several new features.

Summarizing, the tool provides the following information at a significantly smaller complexity than the already available DTRACE approach:

- Frequency of occurrence of coding tools like inter / intra prediction blocks, transformations, refinement, coefficient coding, and in-loop filtering.

- Evaluation on block-level.

- Overall sequence-wise statistics.

Furthermore, it is claimed that the following benefits can be drawn:

- Information on the use of newly introduced coding modes.

- Information on the use of established coding modes, which might not or barely be used anymore.

- Investigation of the influence of external parameters (e.g., QP, encoder configuration, tools on/off).

- Exploitation for energy modeling, cf. JVET-Q0052 [8] and [3].

- Exploitation for decoding-energy optimal encoding [4].

We propose to include the analyzer software into the VTM-reference software.

# Bit Stream Analysis in Decoding

To implement the analyzer, the decoder of VTM-7.0 was extended with additional functionality. The main extension is a structure called featureCounter, which includes integer variables that count the occurrence of features. During decoding, after parsing and interpretation of the bit stream information, the variables are incremented when a certain feature occurs. After finishing the overall decoding process, the counted numbers are written to an ASCII text file, which can be read using software like Matlab or Python.

Version VTM-7.0 of the analyzer provides information for all features listed in Table 1. The features comprise parsing, prediction, transformation, block size, and loop filter information. For further clarification, we distinguish features by a reference unit. Namely, features are counted on slice level (slice level features), CTB level (CTB features), block level (block features), on pel level (pel features), or on boundary level (boundary features). Block level features are counted depending on the block dimensions (e.g., CU dimensions) as explained in Section 2.1.

In the current implementation, feature counting is enabled by setting a command line flag

--BSFA="targetTextFile.m",

which causes the decoder to write an ASCII-text file in the source folder of the bit stream, which is directly Matlab-readable.

## Block-Wise Feature Counting

Most of the features are counted block-wise, which reflects the multi-type tree coding structure implemented in VTM. Allowed block dimensions in counting include a single pel (size 1x1) up to CTU size of 128x128 pels. The proposed analyzer considers all possible square and rectangular block sizes by using a matrix structure, where each entry corresponds to a single block size. The matrix has dimensions 8x8, which corresponds to block dimensions in powers of two. The resulting matrix in terms of block dimensions is defined as

,

where the first dimension indicates the width and the second dimension the height of the corresponding block.

## Currently Implemented Features

The implemented features are categorized according to the main components of a hybrid decoder, which are the categories GENERAL, PREDICTION, TRANSFORM, PARSING, and IN-LOOP. All features, depending on their category, are listed in Table 1.

Table 1 Features implemented in the VTM analyzer software.

| Category | Index | Feature name | Level |
| --- | --- | --- | --- |
| GENERAL | 1 | ISlice | Slice |
|  | 2 | PSlice | Slice |
|  | 3 | BSlice | Slice |
|  | 4 | QP | Slice |
| PREDICTION | 5 | intraBlock | Block |
|  | 6 | intraPlanarLuma | Block |
|  | 7 | intraDCLuma | Block |
|  | 8 | intraHVDLuma | Block |
|  | 9 | intraAngLuma | Block |
|  | 10 | intraPlanarChroma | Block |
|  | 11 | intraDCChroma | Block |
|  | 12 | intraHVDChroma | Block |
|  | 13 | intraAngChroma | Block |
|  | 14 | intraCrossComp | Block |
|  | 15 | intraMIP | Block |
|  | 16 | intraPDPC | Block |
|  | 17 | intraIBC | Block |
|  | 18 | intraSubPartitionsHor | Block |
|  | 19 | intraSubPartitionsVer | Block |
|  | 20 | interBlocks | Block |
|  | 21 | interInter | Block |
|  | 22 | interMerge | Block |
|  | 23 | interSkip | Block |
|  | 24 | interAffine | Block |
|  | 25 | interAffineInter | Block |
|  | 26 | interAffineMerge | Block |
|  | 27 | interAffineSkip | Block |
|  | 28 | interSubblockMerge | Block |
|  | 29 | interTriangleSplit | Block |
|  | 30 | uniPredPel | Pel |
|  | 31 | biPredPel | Pel |
|  | 32 | fracPelHor | Pel |
|  | 33 | fracPelVer | Pel |
|  | 34 | fracPelBoth | Pel |
|  | 35 | copyPel | Pel |
| TRANSFORM | 36 | Transform | Block |
|  | 37 | TransformDCT2 Hor | Block |
|  | 38 | TransformDCT8 Hor | Block |
|  | 39 | TransformDST7 Hor | Block |
|  | 40 | TransformDCT2 Ver | Block |
|  | 41 | TransformDCT8 Ver | Block |
|  | 42 | TransformDST7 Ver | Block |
|  | 43 | TransformSkip | Block |
|  | 44 | TransformNoCBF | Block |
|  | 45 | TransformLFNST | Block |
| PARSING | 46 | nCoeff | Pel |
|  | 47 | coeffG1 | Pel |
|  | 48 | valueOfCoeff | Log-domain |
| IN-LOOP | 49 | Bs0 | Boundary |
|  | 50 | Bs1 | Boundary |
|  | 51 | SAOLumaEO | CTB |
|  | 52 | SAOLumaBO | CTB |
|  | 53 | SAOChromaEO | CTB |
|  | 54 | SAOChromaBO | CTB |
|  | 55 | ALFLuma | CTB |
|  | 56 | ALFChroma | CTB |

Most of the features are described in detail in [7]. New features are intra block copy (IBC), which is relevant for screen content coding, and interAffineSkip, which was not separated from interAffineMerge before.

It shall be noted that the implemented list of features is not exhaustive in describing the overall decoding process. Further features considering coding tools like decoder-side motion vector derivation, bi-direction optical flow, or others could also be implemented.

## Extension with New Features

To include new features for counting, the following steps must be undertaken:

- Add a new feature to the structure featureStruct in TypeDef.h.

- Set the initial value to zero.

- Increment counter after parsing (mostly in the function decompressCTU() in DecCu.cpp).

- Make a new output fprintf() in the if clause if (usePrunedFeatureSet == 0) in DecApp.cpp.

Afterwards, the newly introduced feature is printed to the output file and can be further used for analysis.

## Complexity

To assess the additional complexity of the analysis, we encoded 26 sequences from the common test conditions for standard dynamic range (SDR) video with the four QPs 22, 27, 32, and 37 and the three encoder configurations intra, lowdelay, and randomaccess. The set is the same as used in [5]. Afterwards, we decoded the resulting 276 bit streams with both the conventional VTM-7.0 decoder and the decoder including the analyzer add-on. Processing was performed on an Intel i7-8700 CPU. On average, the decoder complexity increased by 4.77% where in general, the complexity increase is larger for intra-only coded sequences (5.54% on average). The exact values are given in the table below.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  | 105.40% |
| Class A2 |  |  |  |  | 105.89% |
| Class B |  |  |  |  | 105.50% |
| Class C |  |  |  |  | 105.91% |
| Class E |  |  |  |  | 104.73% |
| Overall |  |  |  |  | 105.54% |
| Class D |  |  |  |  | 106.57% |
| Class F |  |  |  |  | 104.63% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  | - |
| Class A2 |  |  |  |  | - |
| Class B |  |  |  |  | 104.36% |
| Class C |  |  |  |  | 104.81% |
| Class E |  |  |  |  | 103.97% |
| Overall |  |  |  |  | 104.65% |
| Class D |  |  |  |  | 105.24% |
| Class F |  |  |  |  | 104.78% |

|  |  |  | Random access Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  | 104.48% |
| Class A2 |  |  |  |  | 104.19% |
| Class B |  |  |  |  | 103.63% |
| Class C |  |  |  |  | 103.96% |
| Class E |  |  |  |  | - |
| Overall |  |  |  |  | 104.13% |
| Class D |  |  |  |  | 105.29% |
| Class F |  |  |  |  | 103.44% |

# Applications

This sections presents two showcase applications for the resulting statistics.

## Occurrence of Block Sizes

With the help of the obtained data, overall statistics on coding tools can be calculated. An example is the occurrence of block sizes. We encode the BQTerrace sequence with QPs 22 and 37 and evaluate the overall share of block sizes for intra- and inter-predicted blocks. The resulting diagrams are split into four figures.  Figure 1 shows the overall share of intra blocks at QP 22.

Figure 1 Share of intra-coded blocks for BQTerrace, QP 22.

The height of each bar indicates the relative share of the corresponding block size shown on the x-axis and the y-axis. The largest block in the middle shows that more than 1% of all pixels are intra-coded with a block size of 8x8 pixels. The other block sizes occur much less frequently.

The corresponding bar diagram for a QP of 37 is depicted in Figure 2.

Figure 2 Share of intra-coded blocks for BQTerrace, QP 37.

We can see that due to the larger QP, the block sizes are larger in general. In this case, the block size of 32x32 occurs for approximately 0.5% of all pixels. In general, the share of intra-coded pixels is smaller than for QP 22 because the share of inter-coded pixels is larger.

In terms of inter coding, the share for QP 22 is shown in Figure 3.

Figure 3: Share of inter-coded blocks for BQTerrace, QP 22.

We can see that block sizes 8x8, 16x16, and 32x32 all reach a similar share of 13% to 15%. Furthermore, it is obvious that square block sizes are chosen more often than rectangular block sizes.

Finally, Figure 4 shows the share of block sizes for QP37. We can see that more than 60% of the pixels can be found in block sizes of 128x128 pixels. This indicates that for high-QP applications, it may be beneficial to increase the maximum block size for RD-optimal compression.

Figure 4 Share of inter-coded blocks for BQTerrace, QP 37.

## Exploitation for Energy Modeling

The counted feature numbers can be used for accurate energy modeling as presented in [8].

# Concluding Remarks

This contribution introduced an analyzer add-on for the VTM-7.0 decoder software. The software is reported to be lightweight in complexity. It enables to obtain detailed statistics on the use of compression tools such as prediction modes, transformation, coefficient coding, CU block sizes, and in-loop filtering. For further assessment and in-depths investigations of coding tools, it is proposed to include the analyzer into the VTM-reference software.

The source code is freely available for download and further use at https://gitlab.lms.tf.fau.de/LMS/vtm-analyzer .

# Patent rights declaration(s)

The Friedrich-Alexander University Erlangen-Nürnberg (FAU) does not have any current or pending patent rights relating to the technology described in this contribution.

# References

[1] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019.

[2] J. Sauer, J. Schneider, M. Bläser, M. Wien, “Reference software extension for coding block statistics”, JVET-K0149, July 2018.

[3] C. Herglotz, D. Springer, M. Reichenbach, B. Stabernack, and A. Kaup, „Modeling the energy consumption of the HEVC decoding process”, IEEE Transactions on Circuits and Systems for Video Technology, 28:217–229, Jan 2018.

[4] C. Herglotz, A. Heindel, and A. Kaup. Decoding energy distortion optimization for video coding. IEEE Transactions on Circuits and Systems for Video Technology, 29:171-182, Jan. 2019.

[5] M. Kränzler, C. Herglotz, A. Kaup, “Decoding Energy Assessment of VTM-6.0”, JVET-P0084, October 2019.

[6] C. Herglotz, M. Kränzler, A. Kaup, “Decoding-Energy-Rate-Distortion Optimization”, JVET-K0108, July 2018.

[7] C. Herglotz, M. Kränzler, A. Kaup, “Bit Stream Analyzer for Coding Tool Statistics”, JVET-P0085, October 2019.

[8] M. Kränzler, C. Herglotz, A. Kaup, “Modeling of the Decoding Energy for VTM-7.0 with a Bit Stream Feature-Based Model”, JVET-Q0052, January 2020.