| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0050 |
| --- | --- |

| Title: | AHG13: Decoding Time and Energy Assessment of VTM-7.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Matthias Kränzler Christian Herglotz André Kaup | Tel: Email: | +49 9131 85 27105 matthias.kraenzler@fau.de christian.herglotz@fau.de andre.kaup@fau.de |
| Source: | Multimedia Communications and Signal Processing, Friedrich-Alexander Universität Erlangen-Nürnberg (FAU) |  |  |

_____________________________

# Abstract

This contribution evaluates the decoding energy demand of VTM-7.0 in relation to HM-16.20. The authors compare both video codecs in terms of decoding energy and time. The evaluation reveals that the decoding energy demand of the VTM-7.0 decoder has a significant increase compared to HM-16.20, which depends on the coding configuration. For randomaccess, it is reported that the decoding energy is increased by over 80% with SIMD optimizations. Furthermore, the authors investigate the influence of SIMD optimizations on decoding time and energy. For randomaccess, it is reported that disabling SIMD increases the energy demand even further by approximately 65%.

# Introduction

The energy efficiency of the VTM decoder was already evaluated on the VTM-6.0 decoder [1] at the last JVET meeting. For this meeting, the VTM-7.0 decoder is further analyzed.

This document has the following contributions:

- Analysis of VTM-7.0 decoder in terms of decoding energy and time.

- Analysis of the SIMD optimization influence on decoding energy and time.

- With respect to the HM decoder, the energy increase of VTM is disproportionately high in comparison with the decoding time increase.

# Energy Measurement & Test Set

According to the descriptions in JVET-P0084 [1], decoding energies are measured with RAPL on an Intel i7-8700 CPU. For the decoding of HEVC bit streams, we use the HM-16.20 [2] reference software. For VVC bit streams, the VTM-7.0 [3] reference software is used. In order to have a certain accuracy for the measured decoding energy, multiple measurements are performed and evaluated with statistical tests (cf. JVET-P0084). For evaluation, the bit streams are encoded as described in the CTC for SDR content [4] with the reference software HM-16.20 and VTM-7.0.

# Evaluation of HM-16.20 and VTM-7.0

This section compares the complexity of both video codecs with respect to decoding time and decoding energy. Therefore, we use the Bjøntegaard-Delta-Decoding-Energy (BDDE) [5] to describe the additional energy demand of the decoder and the Bjøntegaard-Delta-Decoding-Time (BDDT) for the additional decoding time, respectively. Furthermore, the Bjøntegaard-Delta-Rate (BDR) is calculated to evaluate the improved compression performance.

In Table 1, the results of the VTM-7.0 decoder are shown compared to the HM-16.20 decoder depending on the encoder configuration. For all-intra coding, the decoding energy demand is increased by 67.13% for an equal objective visual quality and decoding time by 62.03%, respectively. For lowdelayB, decoding energy is increased by 77.76% and the decoding time by 67.66%. The decoding energy demand has the highest increase for sequences of Class D with 134.35%, which is more than a doubling. For randomaccess configuration, the BDDE is 82.56% and BDDT is 71.05%. The results of Table 1 reveal that the increase in decoding energy demand is disproportionately high in comparison with the decoding time.

Table 1: Evaluation of BDR, BDDE, and BDDT for the comparison of the different coding configurations with HM-16.20 and VTM-7.0

|  |  | All Intra Over HM-16.20 |  |
| --- | --- | --- | --- |
|  | YUV-BDR [%] | BDDT [%] | BDDE [%] |
| Class A1 | -27.77 | 51.11 | 55.51 |
| Class A2 | -24.19 | 63.66 | 67.10 |
| Class B | -20.76 | 64.31 | 67.95 |
| Class C | -21.77 | 70.87 | 75.75 |
| Class D | -17.40 | 71.84 | 81.33 |
| Class E | -25.31 | 50.88 | 55.70 |
| Class F | -38.76 | 55.89 | 30.62 |
| Mean | -24.90 | 62.03 | 67.13 |

|  |  | Lowdelay B Over HM-16.20 |  |
| --- | --- | --- | --- |
|  | YUV-BDR [%] | BDDT [%] | BDDE [%] |
| Class A1 | - | - | - |
| Class A2 | - | - | - |
| Class B | -29.89 | 52.66 | 58.77 |
| Class C | -27.60 | 71.53 | 71.53 |
| Class D | -24.56 | 114.94 | 134.35 |
| Class E | -31.78 | 51.12 | 59.55 |
| Class F | -41.68 | 47.67 | 54.58 |
| Mean | -31.01 | 67.66 | 77.76 |

|  |  | Randomaccess over HM-16.20 |  |
| --- | --- | --- | --- |
|  | YUV-BDR [%] | BDDT [%] | BDDE [%] |
| Class A1 | -37.65 | 60.22 | 70.31 |
| Class A2 | -40.11 | 72.08 | 83.59 |
| Class B | -34.01 | 59.96 | 70.53 |
| Class C | -28.89 | 73.58 | 84.79 |
| Class D | -26.49 | 96.17 | 111.65 |
| Class E | - | - | - |
| Class F | -41.09 | 64.61 | 74.71 |
| Mean | -34.31 | 71.05 | 82.56 |

# Evaluation of SIMD-Optimization on VTM Software

This section evaluates the impact of SIMD optimizations in the VTM-7.0 decoder on energy and time. Therefore, we measure the decoding time and energy demand with and without SIMD optimization, which are switched off by the preprocessor-flag (SIMD_ENABLE 0). In Table 2, the VTM-7.0 decoder with SIMD optimization is used as an anchor. In general, the usage of SIMD improves energy- and time-efficiency of the VTM-decoder significantly. The time demand without SIMD is increased by a higher degree than the energy demand. For all-intra coding, the decoding energy demand is increased by 28.78% and decoding time by 38.37%. For lowdelayB, it is increased by 46.01% and 56.60%, respectively. Randomaccess has the highest increase of the decoding energy demand of 65.04% and with 79.15% for the decoding time. Consequently, without the usage of SIMD there is a higher difference between the VTM- and HM-decoder in terms of decoding time and energy.

Table 2: Evaluation of BDDE, and BDDT for the evaluation of SIMD optimization

|  | All Intra Over VTM-7.0 with SIMD |  |
| --- | --- | --- |
|  | Δ Dec. Energy[%] | Δ Dec. Time [%] |
| Class A1 | 46.10 | 57.23 |
| Class A2 | 34.80 | 44.06 |
| Class B | 30.61 | 39.92 |
| Class C | 18.31 | 26.94 |
| Class D | 13.69 | 23.48 |
| Class E | 36.16 | 45.74 |
| Class F | 29.04 | 38.79 |
| Mean | 28.78 | 38.37 |

|  | Lowdelay B Over VTM-7.0 with SIMD |  |
| --- | --- | --- |
|  | Δ Dec. Energy [%] | Δ Dec. Time [%] |
| Class A1 | - | - |
| Class A2 | - | - |
| Class B | 65.64 | 78.05 |
| Class C | 48.35 | 59.42 |
| Class D | 34.22 | 45.53 |
| Class E | 38.42 | 46.58 |
| Class F | 36.72 | 45.57 |
| Mean | 46.01 | 56.60 |
|  | Randomaccess Over VTM-7.0 with SIMD |  |
|  | Δ Dec. Energy [%] | Δ Dec. Time [%] |
| Class A1 | 84.00 | 99.95 |
| Class A2 | 86.56 | 104.22 |
| Class B | 73.45 | 87.32 |
| Class C | 62.48 | 75.23 |
| Class D | 53.54 | 68.13 |
| Class E | - | - |
| Class F | 38.22 | 49.49 |
| Mean | 65.04 | 79.15 |

# Conclusion

This contribution showed that the VTM decoder has a significant increase in decoding energy demand. Furthermore, it could be shown that the additional decoding energy demand depends on the class and coding configuration of the bit stream. For randomaccess, the decoding time is increased by 71.05% and the decoding energy by 82.56%. The analysis of SIMD revealed that the optimization significantly improves the energy demand and time to decode a bit stream. The highest reduction of the decoding energy and time demand can be achieved by the randomaccess configuration with 37.95% and 42.75%, respectively.

# References

[1] M. Kränzler, C. Herglotz, A. Kaup, “Decoding Energy Assessment of VTM-6.0”, JVET-P0084, October 2019.

[2] Joint Collaborative Team on Video Coding (JCT-VC). HEVC test model reference software (HM-16.20). https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.20/, 2019. Accessed 2019-07-08.

[3] Joint Video Exploration Team (JVET). VVC test model reference software (VTM-7.0). https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-7.0, 2019. Accessed 2019-11-22.

[4] F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010-v1, Mar. 2019.

[5] C. Herglotz, A. Heindel, and A. Kaup. Decoding-Energy-Rate-Distortion Optimization for Video Coding. IEEE Transactions on Circuits and Systems for Video Technology, Vol. 29(1), 2019.

# Patent rights declaration

The Friedrich-Alexander-Universität Erlangen-Nürnberg (FAU) does not have any current or pending patent rights relating to the technology described in this contribution.