| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0052 |
| --- | --- |

| Title: | Modeling of the Decoding Energy for VTM-7.0 with a Bit Stream Feature-Based Model |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Informative |  |  |
| Author(s) or Contact(s): | Matthias Kränzler Christian Herglotz André Kaup | Tel: Email: | +49 9131 85 27105 matthias.kraenzler@fau.de christian.herglotz@fau.de andre.kaup@fau.de |
| Source: | Multimedia Communications and Signal Processing, Friedrich-Alexander Universität Erlangen-Nürnberg (FAU) |  |  |

_____________________________

# Abstract

This contribution presents a bit stream feature-based model for the VTM-7.0 decoder energy, which is called feature-based versatile (FV) model. Mean estimation errors from the proposed model are reported to yield 1.35%, which is claimed to be comparable to a state-of-the-art model for decoding. It is claimed that this model can help in the characterization of the processing energy of coding tools and that it can be exploited during encoding for the generation of decoding-energy-saving bit streams.

# Introduction

In previous research, we showed that it is possible to model the decoding energy demand of different video codecs (e.g. HEVC [1] or AVC [2]) with high accuracy. Furthermore, it was shown that bit stream feature-based models can be utilized to optimize the decoding energy demand [3]. This contribution proposes a new bit stream feature-based model for VVC-coded bit streams. The newly introduced model estimates the decoding energy demand with a mean estimation error of 1.35%. The bit stream model can be used to determine tools that have an increased energy demand by investigating the specific energy coefficients of the model.

# Energy Estimation with Bit Stream Feature-Based Models

The decoding energy demand is estimated with a bit stream feature-based model by

| Edec=∀ini∙ei , | (2.1) |
| --- | --- |

where i is the index of a feature, ni the number of occurrences of the feature within the bit stream, and ei the specific energy coefficient of the feature [1]. For example, Islice is a feature that counts the number of I-slices within the bit stream. For modeling of the VTM-decoder, we introduce the feature-based versatile (FV) model. All 217 features of the FV model are shown in Table 1. A description of each feature can be found in detail in [4]. For the extraction of features for a specific bit stream, the bit stream analyzer software proposed in [5] is required, which is freely available for download at https://gitlab.lms.tf.fau.de/LMS/vtm-analyzer.

All features are categorized into five different groups that represent the main components of a hybrid video codec, which are General, Intra, Inter, Transform, and In-Loop. Furthermore, all features are counted on different levels, which are Slice, Pel, CTB, Log-Domain, Block, and BlockPel. The first three levels count the number of slices, pels, or coding tree blocks (CTB). The feature val is counted in the Log-Domain, which shall represent bits that are needed for the transmission of the residual coefficients [6]. The remaining features are counted on Blockpel level that we obtain as follows. Using the bit stream analyzer from [4], we obtain feature numbers on block level. The block-wise counting allows block sizes that have the following dimensions

,

where the width is given by the first dimension and the height by the second dimension of the block [4]. However, using block-wise counting, the energy model would have 791 features, which is too large for the given training set. To reduce the number of features, we merge all block-size features having the same number of pixels per block to another counting level, which is Blockpel. The levels of Blockpel are given in the following vector

| 2 | 4 | 8 | 16 | 32 | 64 | 128 | 256 | 512 | 1024 | 2048 | 4096 | 8192 | 16384 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

,

where each level corresponds to the luma pels within the block. All levels are integer values to the power of 2. For the merging of features, we sum up all features from Block-wise counting that have the same number of pels. For example, the dimensions 1x8, 2x4, 4x2, and 8x1 from block level are merged to the dimension 8 on Blockpel level. Through merging, the number of features is reduced from 791 features to 217 features.

Table 1: Bit Stream Features of the bit stream feature-based versatile (FV) model for VTM.

| Category | Index | Feature Name | Level |
| --- | --- | --- | --- |
| General | 1 | EO | - |
|  | 2 | ISlice | Slice |
|  | 3 | PSlice | Slice |
|  | 4 | BSlice | Slice |
| Intra | 5-18 | intraBlocks | Blockpel |
|  | 19-32 | intraSubPartitions | Blockpel |
|  | 33-46 | intraPDPC | Blockpel |
|  | 47-60 | intraMIP | Blockpel |
|  | 61-74 | intraIBC | Blockpel |
| Inter | 75-88 | interInter | Blockpel |
|  | 89-102 | interSkip | Blockpel |
|  | 103-116 | interMerge | Blockpel |
|  | 117-130 | affine | Blockpel |
|  | 131-144 | triangleSplit | Blockpel |
|  | 145 | uni | Pel |
|  | 146 | bi | Pel |
|  | 147 | fracPelHor | Pel |
|  | 148 | fracPelVer | Pel |
|  | 149 | fracPelBoth | Pel |
|  | 150 | copyPel | Pel |
| Transform | 151-164 | transformBlocks | Blockpel |
|  | 165-178 | transformSkipBlocks | Blockpel |
|  | 179-192 | transformNoCbfBlocks | Blockpel |
|  | 193-206 | transformLFNST | Blockpel |
|  | 207 | coeff | Pel |
|  | 208 | coeffG1 | Pel |
|  | 209 | val | Log-Domain |
| In-Loop | 210 | BS0 | CTB |
|  | 211 | BS1 | CTB |
|  | 212 | SAO Y BO | CTB |
|  | 213 | SAO Y EO | CTB |
|  | 214 | SAO C BO | CTB |
|  | 215 | SAO C EO | CTB |
|  | 216 | ALF Y | CTB |
|  | 217 | ALF C | CTB |

# Evaluation

The modeling of the decoding energies is evaluated on the setups and measurements of JVET-Q2 [7]. These measurements are carried out with the HM-16.20, FFmpeg-4.2, and VTM-7.0 decoder. The bit stream setups are encoded according to the CTC recommendations of JVET-N1010 [8]. The CTC setups have 276 bit streams in this contribution for each video codec. For HEVC-coded bit streams, we use the state-of-the-art feature-based universal (FU) model, which has 100 features [1]. Despite the fact that the FU model is only applicable for HEVC, it is used as reference to compare results.

In this contribution, we evaluate the accuracy of the estimation by

| ε =1Nn=0N-1Edec,n-Edec,nEdec,n , | (3.1) |
| --- | --- |

where  ε is the mean absolute estimation error, N the number of bit streams in the setup, Edec,n the measured decoding energy of the bit stream n, and Edec,n the estimated decoding energy. For the training of the specific energy coefficients, we use least-square curve fitting with a trust-region reflective algorithm [9]. For the validation of the trained specific energies, we use k-fold cross validation.

In Table 2, the evaluation of  ε is given for each decoder. For the FU model with 1-fold cross validation, the estimation error is 0.95% for the HM-16.20 decoder and 2.60% for the FFmpeg decoder. With 10-fold cross-validation, the estimation error is 1.76% and 5.11%, respectively. For the VTM-decoder, the FV model has an estimation error of 1.35% with 1-fold cross validation and 4.26% with 10-fold cross validation. These results have sufficient accuracy for the modeling of the decoding energy. The higher estimation error of the VTM model can be explained by the fact that the model has 217 features and 276 bit streams for training. For a setup with a higher number of bit streams, we additionally use all 564 bit streams of the tool testing from JVET-Q0053 [10], which corresponds to the setup in the last row of Table 1. For this setup with 840 bit streams, the 1-fold cross validation has an estimation error of 1.71% and the 10-fold cross validation of 2.31%, respectively. We assume that the estimation accuracy will be further improved by the introduction of new features (e.g. DMVR, BDOF).

Table 2: Evaluation of  ε for the HM, VTM, and FFmpeg decoder for the corresponding bit stream feature-based model with different k-fold cross validations.

|  |  |  |  |  | FU Model |  | FV Model |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Decoder | Setup |  | #Bit Streams |  | k = 1 | k = 10 | k = 1 | k = 10 |
| HM-16.20 |  | CTC |  | 276 | 0.95% | 1.76% | - | - |
| FFmpeg |  | CTC |  | 276 | 2.60% | 5.11% | - | - |
| VTM-7.0 |  | CTC |  | 276 | - | - | 1.35% | 4.26% |
| VTM-7.0 |  | CTC+ToolTest |  | 840 | - | - | 1.71% | 2.31% |

# Conclusion

This contribution introduced a new bit stream feature-based model for the VTM-7.0 decoder, which uses the bit stream feature analyzer (BFSA) software for the extraction of the features. The bit stream model achieves an estimation accuracy of 1.35%. The model can be used for in-depth energy analysis or decoding-energy optimized encoding.

# References

[1] M. Kränzler, C. Herglotz, and A. Kaup, “Extending Video Decoding Energy Models for 360° and HDR Video Formats in HEVC”, in Proc. Picture Coding Symposium (PCS), Ningbo, CN, Nov. 2019.

[2] C. Herglotz, Y. Wen, B. Dai, M. Kränzler, and A. Kaup. “A Bitstream Feature Based Model for Video Decoding Energy Estimation”, in Proc. Picture Coding Symposium (PCS), Nuremberg, Germany, Dec. 2016.

[3] C. Herglotz, A. Heindel, and A. Kaup. Decoding energy distortion optimization for video coding. IEEE Transactions on Circuits and Systems for Video Technology, 29:171-182, Jan. 2019.

[4] C. Herglotz, M. Kränzler, and A. Kaup, “Bit Stream Analyzer for Coding Tool Statistics”, JVET-P0085, Oct. 2019.

[5] C. Herglotz, M. Kränzler, and A. Kaup, “Bit Stream Feature Analyzer (BSFA) for Coding Tool Statistics Based on VTM-7.0”, JVET-Q0049, Jan. 2020.

[6] C. Herglotz, A. Heindel, and A. Kaup. Decoding energy distortion optimization for video coding. IEEE Transactions on Circuits and Systems for Video Technology, 29:171-182, Jan. 2019.

[7] M. Kränzler, C. Herglotz, and A. Kaup, “AHG13: Decoding Time and Energy Assessment of VTM-7.0”, JVET-Q0050, Jan. 2020.

[8] F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010-v1, Mar. 2019.

[9] T. F. Coleman and Y. Li. An interior trust region approach for nonlinear minimization subject to bounds. SIAM J. Optim., vol. 6, no. 2, pp. 418–445, 1996.

[10] M. Kränzler, C. Herglotz, and A. Kaup, “AHG13: Analysis of the Energy Demand and Time Complexity of Several Coding Tools in VTM-7.0”, JVET-Q0053, Jan. 2020.

# Patent rights declaration

The Friedrich-Alexander-Universität Erlangen-Nürnberg (FAU) does not have any current or pending patent rights relating to the technology described in this contribution.