| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0053 |
| --- | --- |

| Title: | AHG13: Analysis of the Energy Demand and Time Complexity of Several Coding Tools in VTM-7.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Matthias Kränzler Christian Herglotz André Kaup | Tel: Email: | +49 9131 85 27105 matthias.kraenzler@fau.de christian.herglotz@fau.de andre.kaup@fau.de |
| Source: | Multimedia Communications and Signal Processing, Friedrich-Alexander Universität Erlangen-Nürnberg (FAU) |  |  |

_____________________________

# Abstract

This contribution analyzes the influence of 9 coding tools on the decoding time and energy demand compared to the VTM-7.0 reference software. The analysis compares 2 tools with the All-Intra (AI) configuration and 7 tools with the Randomaccess (RA) configuration. It is reported that the overall decoding energy demand can both be increased and reduced by disabling a specific coding tool. For ALF, the decoding energy demand is reduced by approximately 7% with an increase of 4.36% in bit rate if the tool is switched off. For AMVR, the decoding energy demand is increased by 2.28% with a bit rate increase of 1.29%.

# Introduction

At the last JVET-Meeting it was suggested to analyze the influence of a single coding tool on the decoding energy demand. Therefore, this contribution takes a deeper look at some coding tools in order to show the capabilities of a tool analysis.

It is claimed that the following insights and benefits can be drawn from the tool analysis:

- Analysis of the influence on the decoding energy and time demand of a specific coding tool.

- Optimization of energy and time demand by finding optimal configuration settings.

# Tool Analysis – Basics

In this section, the measurement set, the used metrics, and evaluated tools will be explained. For the measurement of the decoding time and energy, we decode the bit streams on an Intel i7-8700 CPU with RAPL according to the descriptions in JVET-P0084 [1]. Furthermore, all bit streams are encoded with the VTM-7.0 reference software [2].

For the evaluation, we use three different metrics, which are Bjøntegaard-Delta-Rate (BDR), Bjøntegaard-Delta-Decoding-Energy (BDDE) [3], and Bjøntegaard-Delta-Decoding-Time (BDDT). For each metric, the bit streams that are coded with a tool switched-off are compared to the VTM-7.0 reference software with the tool switched-on. Therefore, a positive value of BDDE corresponds to a higher energy demand if the tool is switched-off, which implies that the usage of the specific tool improves the energy-efficiency of the video codec. The same applies for the other two metrics.

In this contribution, we evaluated 9 different tools that are currently used by the VTM-7.0 video codec. For each tool, we used either All-Intra (AI) or Randomaccess (RA) configuration. In Table 1, all tools are given with the corresponding configuration [4]. A detailed description of each coding tool can be found in [5].

Table 1: List of all evaluated tools and corresponding configurations

| Tool | Configuration |
| --- | --- |
| ISP – Intra sub-partitioning | AI |
| MIP – Matrix based intra prediction | AI |
| ALF – Adaptive loop filter | RA |
| AMVR – Adaptive motion vector resolution | RA |
| BDOF – Bi-directional optical flow | RA |
| DMVR – Decoder-side motion vector refinement | RA |
| LFNST – Low frequency non-separable transform | RA |
| MTS – Multiple transform set | RA |
| TPM – Triangular partition | RA |

# Evaluation of Tools

Figure 1: Evaluation of three coding tools when switched-off in terms of BDR- and BDDE-performance for each coded sequence.

In general, tools with a positive value of BDDE improve the energy-efficiency of the video codec and tools with a negative BDDE value have a negative impact on the energy-efficiency, respectively. Therefore, the desirable solution would be a tool with a high BDR and BDDE value, if the tool is switched-off. In Figure 1, the evaluation of ALF, AMVR, and BDOF is shown. It can be determined that ALF has the highest values for BDR and lowest values for BDDE compared to the other tools. On average, the YUV-BDR is 4.36% and the BDDE is -6.99%, which indicates that ALF has a high improvement of the RD-efficiency at the cost of high computational costs. For BDOF, the values of BDR are lower compared to ALF and higher for BDDE, which means that the usage of ALF leads to an increased energy demand at the decoder. An improvement of the energy-efficiency is achieved for the tool AMVR, which can be determined by the positive values of BDDE that is 2.28% on average. Simultaneously, the RD-efficiency is improved by 1.29%. By analyzing the bit streams with [6], it can be determined that the usage of AMVR leads to fewer motion vectors that refer to a fractional pel position, which is a possible explanation for the improved energy-efficiency.

In Table 2, the evaluation of BDR, BDDE, and BDDT is given for each tested tool in detail. The bit streams are compared to the corresponding anchor bit streams, which are encoded according to JVET-N1010 [7]. Furthermore, the tools in Table 2 are sorted by the value of BDDE. For all coding tools, the RD-efficiency is improved, but the energy-efficiency is only improved for 5 tools, which are MIP, AMVR, LFNST, MTS, and TPM. For all tool tests, it can be determined that the influence on the decoding time demand is similar to the decoding energy.

Table 2: Evaluation of BDR, BDDE, and BDDT for switched off coding tools compared to VTM-7.0

|  | All Intra Over VTM-7.0 |  |  |
| --- | --- | --- | --- |
|  | YUV-BDR [%] | BDDT [%] | BDDE [%] |
| MIP | 0.51 | 1.16 | 0.80 |
| ISP | 0.54 | -1.34 | -1.82 |

|  | Randomaccess Over VTM-7.0 |  |  |
| --- | --- | --- | --- |
|  | YUV-BDR [%] | BDDT [%] | BDDE [%] |
| AMVR | 1.29 | 2.84 | 2.28 |
| TPM | 0.39 | 2.32 | 2.02 |
| LFNST | 0.73 | 2.09 | 1.63 |
| MTS | 0.61 | 1.69 | 1.38 |
| BDOF | 0.80 | -0.81 | -1.30 |
| DMVR | 0.73 | -0.59 | -1.44 |
| ALF | 4.36 | -7.08 | -6.99 |

# Conclusion

This contribution showed that an analysis of tools is necessary to determine which tools improve the decoding energy and time demand. The analysis revealed that the usage of four tools decreased the energy-efficiency of the VTM-decoder and for five tools the energy-efficiency is improved. For ALF, the decoding energy demand is increased by 6.99% and for AMVR, the decoding energy demand is reduced by 2.28%.

# References

[1] M. Kränzler, C. Herglotz, A. Kaup, “Decoding Energy Assessment of VTM-6.0”, JVET-P0084, Oct. 2019.

[2] Joint Video Exploration Team (JVET). VVC test model reference software (VTM-7.0). https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-7.0, 2019. Accessed 2019-11-22.

[3] C. Herglotz, A. Heindel, and A. Kaup. Decoding-Energy-Rate-Distortion Optimization for Video Coding. IEEE Transactions on Circuits and Systems for Video Technology, Vol. 29(1), 2019.

[4] W.-J. Chien, J. Boyce, W. Chen, Y.W. Chen, R. Chernyak, K. Choi, R. Hashimoto, Y.-W. Huang, H. Jang, R.-L. Liao, S. Liu, “JVET-AHG report: Tool reporting procedure (AHG13)”, JVET-Q0013, Jan. 2020.

[5] J. Chen, Y. Ye, and S. H. Kim, “Algorithm description for Versatile Video Coding and Test Model 5 (VTM 5)”, JVET-N1002-v2, Jun.2019.

[6] C. Herglotz, M. Kränzler, A. Kaup, “Bit Stream Feature Analyzer (BSFA) for Coding Tool Statistics Based on VTM-7.0”, JVET-Q0049, Jan. 2020.

[7] F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010-v1, Mar. 2019.

# Patent rights declaration

The Friedrich-Alexander-Universität Erlangen-Nürnberg (FAU) does not have any current or pending patent rights relating to the technology described in this contribution.