| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0054 |
| --- | --- |

| Title: | CE1-1.1 to CE1-1.3: Fixes for long luma deblocking filter decision |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, J. Enhorn | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution reports results for three CE1 sub-tests related to fixes for the long luma deblocking decisions:

- CE1-1.2 adds an additional check on samples furthest away from the block boundary including one sample which not is checked at all by VVC when max filter length is equal to 7 on that side of the block boundary. This is asserted to avoid filtering natural structure that VVC currently ignores.

- CE1-1.3 reduces a threshold that relates to gradient checks close to the block boundary. This is asserted to avoid filtering natural structure close to the block boundary.

- CE1-1.1 is a combination of both CE1-1.2 and CE1-1.3. This is asserted to improve visual quality on both SDR and HDR content.

The BD rate impact for luma for SDR:

- CE1-1.1: AI -0.01%, RA -0.07%, LDB -0.12%, LDP -0.09%

- CE1-1.2: AI -0.01%, RA -0.03%, LDB -0.12%, LDP -0.04%

- CE1-1.3: AI -0.01%, RA -0.05%, LDB -0.06%, LDP -0.02%

The BD rate impact for HDR:

- CE1-1.1:

  - AI  DE:   0.00% PSNRL: -0.04% wPSNRY: -0.03%, PSNRY: -0.02%

  - RA DE: -0.07% PSNRL: -0.22% wPSNRY: -0.11%, PSNRY: -0.06%

- CE1-1.2:

  - AI  DE: -0.01% PSNRL: -0.03% wPSNRY: -0.02%, PSNRY: -0.01%

  - RA DE:  0.00% PSNRL: -0.13% wPSNRY: -0.07%, PSNRY: -0.05%

- CE1-1.3:

  - AI  DE:  0.00% PSNRL: -0.03% wPSNRY: -0.03%, PSNRY: -0.02%

  - RA DE:  0.01% PSNRL: -0.17% wPSNRY: -0.09%, PSNRY: -0.06%

Similar encoding and decoding time as the anchor.

Updated text provided separately for CE1-1.1 and also fixed some typos in text.

# Introduction

The long luma deblocking filter in VVC draft [1] is very good in supressing blocking artifacts. However, in some cases it can happen that wrong long luma deblocking filter decisions are made due to not checking all samples before deblocking. In some other cases natural edges are filtered due to too high beta threshold for long luma deblocking filter decision check of samples close to the boundary. Section 2 proposes additional gradient calculations to also check edges around p6/q6, e.g. around 7:th sample from boundary in block P and block Q respectively, which currently not is checked, to avoid applying the long filter in cases of natural edges around q6/p6. Section 3 propose to reduce the beta threshold for samples p0 to p5 and q0 to q5 to avoid applying the long filter in case of natural edges somewhere between first and 6:th sample in block P or somewhere between first and 6:th sample in block Q. Results are shown in section 4 and conclusions in section 5. The fixes were first described in JVET-P0411 [2] and in CE1 description [3] and the suggested changes here are identical to those with respect to software but the specification text have been corrected such that strong and weak deblocking decisions not are modified.

# Fix to check samples p6/q6 used when filtering 7 samples

Consider a line of samples across the boundary between block P and Q, with 8 samples in block P and 8 samples in block Q as follows:

Samples:       p7 p6 p5 p4 p3 p2 p1 p0 | q0 q1 q2 q3 q4 q5 q6 q7

In VVC draft 7 [1], when maxFilterLengtP is equal to 7 sample p6 is never checked and when maxFilterLengthQ is equal to 7 sample q6 is never checked, although those samples can be filtered and thus a line orthogonal to the vertical or the horizontal boundary going through p6 and q6 can never be discovered by the long deblocking decisions and can therefore be filtered by the long deblocking filter.

Same can happen for a wider line orthogonal to the vertical or the horizontal boundary with peak in sample p5 or q5 where p4, p5, p6, q4, q5 and q6 are larger or smaller than neighboring samples as shown below which neither can be detected by the long deblocking decisions and can therefore be filtered by the long deblocking filter:

p7 p6 p5 p4 p3 p2 p1 p0 | q0 q1 q2 q3 q4 q5 q6 q7

10 20 30 20 10 10 10 10 | 10 10 10 10 20 30 20 10  (example values with peak in p5 or q5)

To combat this issue additional ramp insensitive gradient calculations are added to the long luma filter decision when maxFilterLengthP or maxFilterLengthQ is equal to 7 as high-lighted in yellow on top of VVC draft 7[1]:

8.8.3.6.1	Decision process for luma block edges

..

8.	When sidePisLargeBlk or sideQisLargeBlk is greater than 0, the following applies:

..

c. The variables sp0L and sp3L are derived as follows:

- If maxFilterLengthP is equal to 7, the following applies:

sp0L = sp0 + Abs( p7,0 – p6,0 – p5,0  + p4,0)	(1281)

sp3L = sp3 + Abs( p7,3 – p6,3 – p5,3 +  p4,3)		(1282)

- Otherwise, the following applies:

sp0L = sp0		(1281)

sp3L = sp3		(1282)

d. The variables sq0L and sq3L are derived as follows:

- If maxFilterLengthQ is equal to 7, the following applies:

sq0L = sq0 + Abs( q4,0 – q5,0 – q6,0 + q7,0 )	(1281)

sq3L = sq3 + Abs( q4,3 – q5,3 – q6,3 + q7,3 )		(1282)

- Otherwise, the following applies:

sq0L = sq0		(1281)

sq3L = sq3		(1282)

..

When dL is less than β, the following ordered steps apply:

ii.	The variable sp is set equal to sp0L, the variable sq is set equal to sq0L and the variable spq is set equal to spq0.

..

vi.	The variable sp is set equal to sp3L, the variable sq is set equal to sq3L and the variable spq is set equal to spq3.

## Complexity analysis

Additional calculation for the case of block size orthogonal to the block boundary of 32 or larger. Worst case for the change is when other side has a block size orthogonal to the block boundary of 8. For that case two absolute value and 8 additions are added for each 4 sample segment along the block boundary compared to VVC. Considering the per sample operation for 32x8 region the increase per sample is 10*2segments /(32*8) =0.078125 per sample.

The current long filter decision for that case has for each 4 sample segment to make calculations used for both long and strong deblocking filter but also calculations specific for long filter decision and then also calculations specific for strong filter decision in case long filter not is selected. In total about 2 segments*(28+32+12)/ (32*8)=2*72/(32*8)=180/(32*8) = 0.56 operations per sample.

The worst case for decision calculations is not modified since that happens when all blocks are 8x8 where the number of operations per sample for decisions are about 2 segments * (28+12)/(8*8)=2*40/64=1.25 operations per sample.

Calculations in general for both long and strong deblocking filter (10 abs + 4 shifts + 14 adds = 28 op):

dp0 = Abs( p2,0 − 2 * p1,0 + p0,0 )		(1266)

dp3 = Abs( p2,3 − 2 * p1,3 + p0,3 )		(1267)

dq0 = Abs( q2,0 − 2 * q1,0 + q0,0 )		(1268)

dq3 = Abs( q2,3 − 2 * q1,3 + q0,3 )		(1269)

sp0 = Abs( p3,0 − p0,0 )		(1270)

sq0 = Abs( q0,0 − q3,0 )		(1271)

spq0 = Abs( p0,0 − q0,0 )		(1272)

sp3 = Abs( p3,3 − p0,3 )		(1273)

sq3 = Abs( q0,3 − q3,3 )		(1274)

spq3 = Abs( p0,3 − q0,3 )		(1275)

Additional calculations for long filter of one side (4 abs + 4 shifts + 17 adds + 7cmp=32 op):

dp0L = ( dp0 + Abs( p5,0 − 2 * p4,0 + p3,0 ) + 1 ) >> 1	(1276)

dp3L = ( dp3 + Abs( p5,3 − 2 * p4,3 + p3,3 ) + 1 ) >> 1	                                                        	   (1277)

When dL is less than β

dpq0L = dp0L + dq0L		(1285)

dpq3L = dp3L + dq3L		(1286)

dL = dpq0L + dpq3L		(1287)

2x :

sp = ( sp + Abs( p3 − p0 ) + 1 )  >>  1	(1346)

- dpq is less than ( β  >>  2 ),

- sp + sq is less than sThr,

- spq is less than ( 5 * tC + 1 )  >>  1.

Additional calculations for strong filter (5 adds + 7 cmp = 12op)

dpq0 = dp0 + dq0		(1296)

dpq3 = dp3 + dq3		(1297)

dp = dp0 + dp3		(1298)

dq = dq0 + dq3		(1299)

d = dpq0 + dpq3		(1300)

When d is less than β

2x:

- dpq is less than ( β  >>  2 ),

- sp + sq is less than sThr,

- spq is less than ( 5 * tC + 1 )  >>  1.

# Fix for beta threshold for samples p0 to p5 and q0 to q5

In VVC draft 7 [1], samples p0 to p5 and q0 to q5 are checked by a beta threshold with same level as used for the shorter filters which makes it possible for the longer luma filters to filter natural edges close to the boundary.

To combat this issue the dpq threshold for the long luma filter decision related to samples closer to the boundary in section 8.8.3.6.5 is reduced as high-lighted in yellow on top of VVC draft 7 [1]:

..

The variable sThr is derived as follows:

- If sidePisLargeBlk is equal to 1 or sideQisLargeBlk is equal to 1, the following applies:

sThr1 = 3 * β  >>  5	(8-1348)

sThr2 =  β  >>  4		(8-1348b)

- Otherwise, the following applies:

sThr1 = β  >>  3	(8-1349)

sThr2 = β  >>  2	(8-1349b)

The variable dSam is specified as follows:

- If all of the following conditions are true, dSam is set equal to 1:

- dpq is less than ( β  >>  2 )sThr2,

- sp + sq is less than sThr1,

- spq is less than ( 5 * tC + 1 )  >>  1.

- Otherwise, dSam is set equal to 0.

## Complexity analysis

No increase of complexity.

# Results

## Subjective impact

The additional decision improves most for AlohaWave, EBUKidsSoccer, DayStreet (HLG), FourPeople and Cactus. The threshold change improves most for Hurdles (PQ) and Starting (PQ). The combination of both changes performs best.

DayStreet has been viewed on LG OLED display in native HLG mode by compressing with HEVC and indication HLG format. Hurdles and Starting have been viewed on SIM2. Other sequences have been viewed on LG OLED.

## Objective performance

### SDR

The objective performance for the proposal is shown below in comparison to VTM-7.0 [4] using CTC [5]. The objective results show a small gain. Encoding time is measured on simulations run on a cluster with machines with same capability. Decoding time is measured by running test and anchor on same machine without yuv output. Encoding and decoding time is similar as for the anchor.

#### CE1-1.1 vs VTM-7.0

|  | All intra Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,05% | 0,00% | 0,02% | 96% | 102% |
| Class A2 | -0,01% | 0,00% | 0,01% | 96% | 99% |
| Class B | -0,02% | 0,00% | 0,00% | 96% | 99% |
| Class C | 0,00% | 0,00% | 0,00% | 98% | 99% |
| Class E | 0,00% | 0,00% | 0,00% | 95% | 100% |
| Overall | -0,01% | 0,00% | 0,00% | 96% | 100% |
| Class D | 0,00% | 0,00% | 0,00% | 97% | 102% |
| Class F | -0,01% | 0,00% | 0,01% | 98% | 99% |

|  | Random access Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,09% | -0,04% | -0,02% | 100% | 101% |
| Class A2 | -0,10% | 0,08% | 0,01% | 99% | 99% |
| Class B | -0,08% | 0,09% | 0,05% | 100% | 99% |
| Class C | -0,03% | -0,08% | -0,05% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0,07% | 0,02% | 0,00% | 99% | 100% |
| Class D | -0,03% | -0,12% | -0,02% | 99% | 102% |
| Class F | -0,05% | -0,02% | 0,00% | 98% | 99% |

|  | Low-delay B Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,14% | -0,30% | -0,22% | 96% | 101% |
| Class C | -0,09% | -0,37% | -0,21% | 98% | 97% |
| Class E | -0,14% | 0,11% | 0,05% | 98% | 100% |
| Overall | -0,12% | -0,22% | -0,15% | 97% | 99% |
| Class D | -0,05% | 0,46% | -0,36% | 97% | 98% |
| Class F | -0,12% | -0,56% | 0,00% | 95% | 102% |

|  | Low-delay P Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,10% | 0,18% | -0,36% | 99% | 99% |
| Class C | -0,03% | -0,11% | -0,03% | 99% | 99% |
| Class E | -0,14% | 0,23% | 0,17% | 98% | 100% |
| Overall | -0,09% | 0,09% | -0,12% | 99% | 99% |
| Class D | -0,04% | -0,23% | 0,45% | 100% | 100% |
| Class F | -0,10% | -0,13% | 0,13% | 98% | 100% |

#### CE1-1.2 vs VTM-7.0

|  | All intra Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,03% | 0,00% | 0,01% | 96% | 100% |
| Class A2 | -0,01% | 0,01% | 0,01% | 96% | 100% |
| Class B | -0,01% | 0,00% | 0,00% | 96% | 98% |
| Class C | 0,00% | 0,00% | 0,00% | 93% | 102% |
| Class E | 0,00% | 0,00% | 0,00% | 98% | 99% |
| Overall | -0,01% | 0,00% | 0,00% | 96% | 100% |
| Class D | 0,00% | 0,00% | 0,00% | 98% | 100% |
| Class F | 0,00% | 0,00% | 0,00% | 98% | 98% |

|  | Random access Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,06% | 0,00% | 0,01% | 100% | 95% |
| Class A2 | -0,07% | 0,03% | 0,06% | 99% | 100% |
| Class B | -0,03% | 0,05% | 0,06% | 99% | 101% |
| Class C | 0,01% | -0,06% | -0,02% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0,03% | 0,01% | 0,03% | 99% | 99% |
| Class D | -0,02% | -0,07% | 0,02% | 99% | 101% |
| Class F | -0,05% | -0,02% | -0,02% | 106% | 100% |

|  | Low-delay B Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,08% | -0,43% | -0,08% | 96% | 99% |
| Class C | -0,07% | -0,04% | -0,14% | 98% | 98% |
| Class E | -0,24% | 0,35% | 0,04% | 98% | 100% |
| Overall | -0,12% | -0,10% | -0,07% | 98% | 99% |
| Class D | -0,02% | 0,40% | 0,24% | 97% | 101% |
| Class F | -0,05% | -0,21% | 0,37% | 97% | 106% |

|  | Low-delay P Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,03% | 0,20% | -0,25% | 99% | 99% |
| Class C | -0,06% | 0,00% | -0,05% | 99% | 100% |
| Class E | -0,03% | -0,59% | -0,73% | 98% | 100% |
| Overall | -0,04% | -0,06% | -0,30% | 99% | 100% |
| Class D | 0,05% | 0,06% | 0,71% | 100% | 103% |
| Class F | -0,14% | -0,52% | 0,19% | 98% | 100% |

#### CE1-1.3 vs VTM-7.0

|  | All intra Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,05% | 0,00% | 0,01% | 98% | 101% |
| Class A2 | -0,01% | 0,01% | 0,00% | 96% | 101% |
| Class B | -0,01% | 0,00% | 0,00% | 98% | 100% |
| Class C | 0,00% | 0,00% | 0,00% | 95% | 99% |
| Class E | 0,00% | 0,00% | 0,00% | 95% | 104% |
| Overall | -0,01% | 0,00% | 0,00% | 97% | 101% |
| Class D | 0,00% | 0,01% | 0,01% | 94% | 101% |
| Class F | -0,01% | 0,00% | 0,01% | 97% | 99% |

|  | Random access Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,07% | -0,01% | -0,06% | 100% | 102% |
| Class A2 | -0,08% | 0,02% | -0,03% | 100% | 98% |
| Class B | -0,05% | 0,13% | 0,06% | 98% | 100% |
| Class C | -0,01% | -0,05% | -0,08% | 98% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0,05% | 0,03% | -0,02% | 99% | 100% |
| Class D | -0,01% | -0,04% | -0,06% | 98% | 101% |
| Class F | -0,02% | 0,02% | -0,02% | 100% | 98% |

|  | Low-delay B Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,08% | -0,23% | -0,16% | 98% | 100% |
| Class C | -0,01% | -0,31% | -0,17% | 99% | 99% |
| Class E | -0,07% | 0,40% | -0,20% | 101% | 100% |
| Overall | -0,06% | -0,10% | -0,17% | 99% | 100% |
| Class D | -0,07% | 0,23% | 0,22% | 100% | 102% |
| Class F | -0,11% | -0,59% | 0,19% | 99% | 100% |

|  | Low-delay P Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,05% | 0,13% | -0,10% | 98% | 101% |
| Class C | -0,03% | -0,07% | -0,06% | 97% | 98% |
| Class E | 0,02% | 0,15% | -0,20% | 96% | 101% |
| Overall | -0,02% | 0,07% | -0,11% | 97% | 100% |
| Class D | -0,05% | 0,29% | 0,85% | 94% | 102% |
| Class F | -0,04% | -0,20% | 0,22% | 98% | 102% |

### HDR

HDR metrics was calculated using HDRTools v0.19.1 linked in to VTM-7.0 according to HDR CTC [6]. Metrics are similar as for the anchor or slightly better. Encoding and decoding simulations was run on machines with similar capabilities but are not fully accurate.

#### CE1-1.1 vs VTM-7.0

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0,00% | -0,04% | -0,03% | 0,00% | 0,00% | -0,02% | 0,00% | -0,01% | 100% | 95% |
| Class H2 |  |  |  |  |  | -0,02% | 0,01% | -0,01% | 97% | 101% |
| Overall | 0,00% | -0,04% | -0,03% | 0,00% | 0,00% | -0,02% | 0,00% | -0,01% | 99% | 97% |

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0,07% | -0,22% | -0,11% | 0,06% | 0,06% | -0,07% | 0,01% | 0,08% | 99% | 99% |
| Class H2 |  |  |  |  |  | -0,06% | 0,03% | -0,03% | 101% | 97% |
| Overall | -0,07% | -0,22% | -0,11% | 0,06% | 0,06% | -0,06% | 0,02% | 0,04% | 100% | 98% |

#### CE1-1.2 vs VTM-7.0

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0,01% | -0,03% | -0,02% | 0,00% | 0,00% | -0,01% | 0,00% | 0,00% | 100% | 96% |
| Class H2 |  |  |  |  |  | -0,01% | 0,00% | -0,01% | 97% | 101% |
| Overall | -0,01% | -0,03% | -0,02% | 0,00% | 0,00% | -0,01% | 0,00% | 0,00% | 99% | 98% |

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0,00% | -0,13% | -0,07% | 0,04% | 0,11% | -0,05% | 0,02% | 0,11% | 98% | 96% |
| Class H2 |  |  |  |  |  | -0,06% | 0,06% | 0,07% | 101% | 97% |
| Overall | 0,00% | -0,13% | -0,07% | 0,04% | 0,11% | -0,05% | 0,03% | 0,10% | 100% | 96% |

#### CE1-1.3 vs VTM-7.0

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0,00% | -0,03% | -0,03% | 0,00% | 0,00% | -0,02% | 0,00% | -0,01% | 100% | 94% |
| Class H2 |  |  |  |  |  | -0,01% | 0,01% | -0,01% | 97% | 102% |
| Overall | 0,00% | -0,03% | -0,03% | 0,00% | 0,00% | -0,02% | 0,00% | -0,01% | 99% | 97% |

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0,01% | -0,17% | -0,09% | 0,05% | 0,21% | -0,06% | 0,01% | 0,19% | 99% | 96% |
| Class H2 |  |  |  |  |  | -0,05% | -0,02% | 0,03% | 101% | 95% |
| Overall | 0,01% | -0,17% | -0,09% | 0,05% | 0,21% | -0,06% | 0,00% | 0,13% | 100% | 96% |

# Conclusions

It is proposed to adopt CE1-1.1 to VVC draft and VTM reference software since the proponents asserts that the visual quality of that subtest is best.

# References

- B. Bross, J. Chen, and S. Liu, "Versatile Video Coding (Draft 7)", Document of Joint Video Experts Team, JVET-P2001.

- K. Andersson, J. Enhorn, “Non-CE5: Fixes for long luma deblocking filter decision”, JVET-P0411

- A. Norkin, K. Andersson, “Description of Core Experiment 1 (CE1): Deblocking filtering”, JVET-P2021

- VTM-7.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations”, JVET-N1010

- A. Segall, E. François, W. Husak, S. Iwamura and D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-P2011

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).