| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0058 |
| --- | --- |

| Title: | CE5 Common Base: Cross Component Adaptive Loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Misra1, F. Bossen2, A. Segall1, P. Cowan1, N. Hu3, J. Dong3, V. Seregin3, M. Karczewicz3, P. Onno4, C. Gisquet4, G. Laroche4, J. Li5, C.S. Lim5, C.-W. Kuo5, J. Nam6, J. Choi6, J. Lim6, and S. Kim6, O. Chubach7, C.-Y. Lai7, C.-Y. Chen7, T.-D. Chuang7, Y.-W. Huang7, S.-M. Lei7 | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com |
| Source: | 1Sharp Labs of America 2Sharp Electronics of Canada, 3Qualcomm Inc., 4Canon, 5Panasonic, 6LGE, 7MediaTek |  |  |

_____________________________

# Abstract

This contribution provides specification text for the cross-component adaptive loop filer (CC-ALF) that is used as the CE5 anchor.  This anchor was proposed in the Geneva meeting and made available in JVET-P1008. The goal of this contribution is to provide text that is integrated into VVC 7 draft text.

Additionally, results are provided that compare the performance of the CE5 anchor with VTM 7.0.  Here, it is shown that CC-ALF provides a coding efficiency improvement of 1.4%, 1.9%, 2.2% and 2.1% using the AHG13 YUV metric for the All Intra, Random Access, Low Delay and Low Delay P configurations, respectively.  Furthermore, it is shown that the CC-ALF anchor provides a coding efficiency improvement for the HDR common test conditions of 3.0% and 3.4% for the All Intra and Random Access configurations, respectively.

# Background

The goal of the cross-component adaptive loop filter (CC-ALF) is to use luma sample values to refine each chroma component.  This is achieved by applying an adaptive, linear filter to the luma channel and then using the output of this filtering operation for the chroma refinement.  Fig. 1a provides a system level diagram of the CC-ALF process with respect to the other loop filters.

Filtering in CC-ALF is accomplished by applying a linear, diamond shaped filter (Fig 1b) to the luma channel for each chroma component, which is expressed as

ΔIix,y=x0,y0∈SiI0xC+x0,yC+y0cix0,y0,
where
x,y is chroma component i location being refined
xC,yC is the luma location based on x,y
Si is filter support in luma for chroma component i
cix0,y0 represents the filter coefficients

| (a) | (b) |
| --- | --- |

Fig. 1: (a) Placement of CC-ALF with respect to other loop filters (b) Diamond shaped filter

The CC-ALF process is further described in JVET-P1008 [1].  A summary of characteristics of the solution include:

- Filter coefficients are transmitted in the APS and have 6-bit dynamic range.

- An APS may be referenced in the slice header.

- CC-ALF filter selection is controlled at CTU-level

- Boundary padding for the horizontal virtual boundaries makes use of same memory access pattern as regular luma ALF.

The syntax and semantics for CC-ALF are provided in the attached word document.

# Experimental Results

The proposed changes were implemented in VTM7 [4] and evaluated for the CTC [5][6].  A summary of the results is provided below.  As can be seen from the summary, the proposed approach provides coding efficiency improvements in the chroma channel for All Intra, Random Access, and Low Delay configurations.  Detailed results can be found in the attached Excel files.

## Standard Dynamic Range (SDR):

## High Dynamic Range (HDR):

YUV* AHG13 weighting 8:1:1

# Conclusion

This contribution provides draft text of the Cross-Component Adaptive Loop Filter (CC-ALF) anchor used in the Core Experiment process.  The tool operates as part of the adaptive loop filter and uses samples from the luma channel to further refine the chroma channel.

Results show the coding gain of the tool to be 1.4%, 1.9%, 2.2% and 2.1% using the AHG13 YUV metric for the All Intra, Random Access, Low Delay and Low Delay P configurations, respectively.  Furthermore, the tool is shown to provide a coding efficiency improvement for the HDR common test conditions of 3.0% and 3.4% for the All Intra and Random Access configurations, respectively.

It is recommended to adopt CC-ALF into the VVC draft specification.

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] K. Misra, F. Bossen, A. Segall, N. Hu, J. Dong, V. Seregin, M. Karczewicz, P. Onno, C. Gisquet, G. Laroche, J. Li, C.S. Lim, C.-W. Kuo, J. Nam, J. Choi, J. Lim, S. Kim, O. Chubach, C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, Y.-W. Huang, S.-M. Lei, "CE5-related: On the design of CC-ALF, " JVET-P1008, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 16th Meeting: Geneva, CH, 1–11 October 2019

[2] K. Misra, F. Bossen, A. Segall, "CE5-2.1, CE5-2.2: Cross Component Adaptive Loop Filter," JVET-P0080, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 16th Meeting: Geneva, CH, 1–11 October 2019

[3] K. Misra, F. Bossen, A. Segall, "Cross-Component Adaptive Loop Filter for chroma," JVET-O0636, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 15th Meeting: Gothenburg, SE, 3–12 July 2019

[4] VVC reference software (VTM) version 7.0, available at: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

[5] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, "JVET common test conditions and software reference configurations for SDR video," JVET-N1010, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 14th Meeting: Geneva, CH, 19–27 Mar. 2019

[6] A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video,” JVET-P2011, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 16th Meeting: Geneva, CH, 1–11 October 2019