| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0059_v1 |
| --- | --- |

| Title: | CE4-1: Geometric Inter Prediction with 64 Modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Gao, Semih Esenlik, Elena Alshina, Anand Meher Kotra, Biao Wang (Huawei), Max Bläser, Johannes Sauer (RWTH Aachen) | Tel: Email: | han.gao@huawei.com semih.esenlik@huawei.com elena.alshina@huawei.com blaeser@ient.rwth-aachen.de |
| Source: | Huawei Technologies Co., LTD, RWTH Aachen University |  |  |

_____________________________

# Abstract

A 64 modes geometric inter prediction is proposed. Near horizontal splits (angle {5, 7, 17, 19}) and distance 2 of horizontal and vertical angles are removed from the CE4 common bases (82 modes GEO). Simulation results shows:

Almost no coding performance changes compared with CE4 common base

0.28%/0.36%/0.30% RA and 0.70%/0.57%/0.67% LDB coding gain compared with VTM7.0

0.66%/0.99%/0.98% RA and 1.61%/1.83%/1.89% LDB coding gain compared with VTM7.0 TPM off case

# Introduction

Geometric inter prediction is firstly proposed as a merge mode extension in [1] of 15th JVET Meeting in Gothenburg, SE. Later, several aspect of geometric inter prediction was studied in the CE4 [2] of 16th JVET Meeting in Geneva, CH. It agreed that the combined test proposal of [3], [4] are used as common base of CE4 [5] of 16th JVET Meeting in Brussels, BE.

In the common base of geometric inter prediction (termed as GEO), there are in total 82 partitioning modes. The 82 modes are composed with 24 slope based angles, and 4 distance for each angles (horizontal and vertical angles and angles larger than or equal to slop -1 without distance 0). Since the horizontal and vertical angles with distance 0 are overlapped with binary tree split, it removed from the geo split modes. The angles and its indices are illustrated in Figure 1. The number of geometric split modes = 12x4 + 12x3 – 2 = 82.

Figure 1: Angles distribution of CE4 common base (82 modes)

# Proposed Method

It’s desirable to further remove redundant geo split modes. We propose a 64 modes geometric inter prediction in [6] and tested in CE4-1.

Since in nature video sequences the motion objects are mostly vertical layout, the near horizontal split modes are less frequently used. In the proposed 64 modes GEO, we removed the angle {5, 7, 17, 19}. Figure 2 shows the angle distribution of 64 modes GEO.

Figure 1: Angles distribution of proposed 64 modes GEO

Furthermore, In the 82 modes GEO, the distances with distance index 2 of horizontal angles {0, 12} and vertical angles {6, 18} are overlapped with ternary tree split boundaries. They are also removed in the proposed 64 modes GEO.

The total modes of the proposed method can be calculated as 10*4 + 10*3 – 2 – 4 = 64 modes

In addition, since the geo split modes are signaled by using truncated binary, a 64 modes will be signalled most efficient with 6 bits TB.

# Simulation Results

As the test condition descripted in [4], the proposed method is simulated under CTC condition and compared with CE4 common base, VTM7.0 and VTM7.0 TpmOff in following Tables

Table 1, CE4-1 compared with CE4 common based

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4-CommonBase |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.11% | 0.00% | 99% | 99% |
| Class A2 | 0.02% | 0.07% | 0.00% | 99% | 100% |
| Class B | -0.01% | -0.01% | 0.08% | 99% | 100% |
| Class C | -0.02% | 0.01% | 0.02% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.03% | 99% | 100% |
| Class D | 0.02% | 0.02% | 0.09% | 100% | 100% |
| Class F (optional) | -0.01% | 0.04% | -0.01% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over CE4-CommonBase |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.10% | -0.06% | 99% | 99% |
| Class C | 0.01% | 0.12% | 0.04% | 99% | 100% |
| Class E | -0.04% | 0.08% | 0.45% | 99% | 99% |
| Overall | 0.00% | 0.10% | 0.10% | 99% | 99% |
| Class D | 0.00% | -0.34% | 0.47% | 99% | 100% |
| Class F (optional) | -0.08% | 0.25% | 0.40% | 100% | 99% |

Table 2, CE4-1 compared with VTM7.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.20% | -0.18% | 102% | 100% |
| Class A2 | -0.20% | -0.12% | -0.02% | 102% | 100% |
| Class B | -0.15% | -0.12% | -0.09% | 102% | 100% |
| Class C | -0.61% | -0.98% | -0.88% | 102% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.28% | -0.36% | -0.30% | 102% | 100% |
| Class D | -0.41% | -0.64% | -0.53% | 103% | 100% |
| Class F (optional) | -0.33% | -0.34% | -0.34% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.34% | -0.40% | -0.66% | 103% | 99% |
| Class C | -0.78% | -1.13% | -0.85% | 103% | 100% |
| Class E | -1.21% | -0.11% | -0.44% | 100% | 100% |
| Overall | -0.70% | -0.57% | -0.67% | 102% | 99% |
| Class D | -0.68% | -0.79% | -0.58% | 103% | 99% |
| Class F (optional) | -0.59% | -0.66% | 0.09% | 102% | 99% |

Table 3, CE4-1 compared with VTM7.0 TPM off case

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0TpmOff |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.35% | -0.60% | -0.61% | 103% | 100% |
| Class A2 | -0.50% | -0.73% | -0.60% | 103% | 100% |
| Class B | -0.41% | -0.54% | -0.56% | 104% | 99% |
| Class C | -1.32% | -2.05% | -2.05% | 104% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.66% | -0.99% | -0.98% | 104% | 99% |
| Class D | -0.79% | -1.35% | -1.05% | 104% | 99% |
| Class F (optional) | -0.57% | -0.75% | -0.73% | 101% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM7.0TpmOff |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.95% | -1.14% | -1.56% | 106% | 99% |
| Class C | -1.96% | -2.70% | -2.52% | 106% | 97% |
| Class E | -2.23% | -1.82% | -1.60% | 103% | 98% |
| Overall | -1.61% | -1.83% | -1.89% | 105% | 98% |
| Class D | -1.59% | -2.54% | -1.88% | 106% | 98% |
| Class F (optional) | -1.02% | -0.72% | -1.07% | 104% | 99% |

# Conclusion

The proposed 64 modes geometric inter prediction mode further reduced the redundant modes in the CE4 common based. Simulation results show almost no coding performance changes compared with CE4-common base. It’s proposed to be adopted in the next version of VVC draft.

# Reference

- S. Esenlik, H. Gao, A. Filippov, V. Rufitskiy, A. M. Kotra, B. Wang, E. Alshina, M. Bläser, J. Sauer  “Non-CE4: Geometrical partitioning for inter blocks”, JVET-O0489, Gothenburg, SE, 3–12 July 2019

- C.-C. Chen, H. Yang, X. Xiu, “CE4: Summary report on inter prediction”, JVET-P0024, Geneva, CH, 1–11 October 2019

- H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang, M. Blaser, J. Sauer, “Description of Core Experiment 4 (CE4): Inter prediction with geometric partitioning”, JVET-P0884, Geneva, CH, 1–11 October 2019

- K. Reuzé, C.-C Chen, H. Huang, W.-J Chien, V. Seregin, M. Karczewicz, R.-L Liao, J. Chen, Y. Ye, J. Luo, M. Blaser, J. Sauer, “Simplified GEO without multiplication and minimum blending mask storage (harmonization of JVET-P0107, JVET-P0264 and JVET-P0304)”, JVET-P0885, Geneva, CH, 1–11 October 2019

- C.-C. Chen, R.-L. Liao, X. Xiu, H. Yang, “Description of Core Experiment 4 (CE4): Inter prediction with geometric partitioning”, JVET-P2024, Geneva, CH, 1–11 October 2019

- H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang, M. Bläser, J. Sauer, “CE4-Related: Geometric Merge Mode (GEO) Simplifications”, JVET-P0107, Geneva, CH, 1–11 October 2019

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

RWTH Aachen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, would be prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form), or make them available free of charge (per box 1 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form). We are however aware that also other entities may have current or pending patent rights relating to the technology described in this contribution.