| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0062 |
| --- | --- |

| Title: | CE4-3: Constrain SBT for GEO mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Ted Hsieh, Vadim Seregin, Kevin Reuzé, Chun-Chi Chen, Hilmi Enes Egilmez, Wei-Jung Chien, Marta Karczewicz (Qualcomm) L. Xu, X. Cao, Y. Sun, F. Chen, L. Wang (Hikvision) | Email: | hanhuang@qti.qualcomm.com xuliying5@hikvision.com |
| Source: | Qualcomm Inc. Hikvision Corporation |  |  |

_____________________________

# Abstract

In this contribution, the test results of CE4-3.1: Disallow SBT for GEO mode and CE4-3.2: Disallow SBT for GEO mode when displacement is 0 are reported.

Compared to the CE4 common-base software, CE4-3.1 provides 0.04%, 0.08%, 0.04% average BD-rate in RA configuration and 0.11%, -0.04%, -0.05% average BD-rate in LDB configuration. CE4-3.2 provides -0.01%, 0.01%, 0.00% average BD-rate in RA configuration and 0.07%, 0.05%, 0.19% average BD-rate in LDB configuration.

# Introduction

Compares to VTM-7.0, the geometric partition mode (GEO) common base provides -0.28%, -0.35%, -0.34% average BD-rate in RA configuration and -0.70%, -0.67%, -0.77% average BD-rate in LDB configuration.

In this contribution, the test results of two sub CEs, CE4-3.1 and CE4-3.2, on top of GEO common base are reported. In CE4-3.1, the SBT is disabled for GEO mode, it provides 0.04%, 0.08%, 0.04% average BD-rate in RA configuration and 0.11%, -0.04%, -0.05% average BD-rate in LDB configuration. In CE4-3.2, the SBT is disabled in GEO mode if the displacement is 0, it provides -0.01%, 0.01%, 0.00% average BD-rate in RA configuration and 0.07%, 0.05%, 0.19% average BD-rate in LDB configuration.

Text change for CE4-3.1:

| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_sbt_enabled_flag && !ciip_flag[ x0 ][ y0 ] && ! MergeGeoFlag[ x0 ][ y0 ] && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY) { |  |
| --- | --- |
| allowSbtVerH = cbWidth >= 8 |  |
| allowSbtVerQ = cbWidth >= 16 |  |
| allowSbtHorH = cbHeight >= 8 |  |
| allowSbtHorQ = cbHeight >= 16 |  |
| if( allowSbtVerH ¦ ¦ allowSbtHorH ) |  |
| cu_sbt_flag | ae(v) |

Text change for CE4-3.2:

| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_sbt_enabled_flag && !ciip_flag[ x0 ][ y0 ] && (! MergeGeoFlag[ x0 ][ y0 ] ¦¦ distanceIdx != 0) && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY) { |  |
| --- | --- |
| allowSbtVerH = cbWidth >= 8 |  |
| allowSbtVerQ = cbWidth >= 16 |  |
| allowSbtHorH = cbHeight >= 8 |  |
| allowSbtHorQ = cbHeight >= 16 |  |
| if( allowSbtVerH ¦ ¦ allowSbtHorH ) |  |
| cu_sbt_flag | ae(v) |

# Test Results

The test results of CE4-3.1 are shown in Table 2-1.

Table 2-1 Test results of CE4-3.1

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over GEO common base |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | 0.03% | 97% | 95% |
| Class A2 | 0.05% | 0.09% | -0.04% | 97% | 97% |
| Class B | 0.04% | 0.08% | 0.00% | 96% | 95% |
| Class C | 0.06% | 0.14% | 0.15% | 95% | 94% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.08% | 0.04% | 96% | 95% |
| Class D | 0.09% | -0.03% | 0.00% | 84% | 97% |
| Class F | 0.07% | 0.05% | 0.05% | 96% | 95% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over GEO common base |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | -0.11% | 0.01% | 96% | 97% |
| Class C | 0.11% | 0.14% | 0.03% | 94% | 99% |
| Class E | 0.18% | -0.15% | -0.24% | 97% | 99% |
| Overall | 0.11% | -0.04% | -0.05% | 95% | 98% |
| Class D | 0.09% | -0.45% | 0.11% | 86% | 104% |
| Class F | 0.11% | 0.45% | 0.17% | 97% | 96% |

The test results of CE4-3.2 are shown in Table 2-2.

Table 2-2 Test results of CE4-3.2

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over GEO common base |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.07% | -0.06% | 97% | 95% |
| Class A2 | -0.02% | 0.05% | -0.06% | 98% | 97% |
| Class B | 0.00% | -0.01% | 0.04% | 96% | 95% |
| Class C | -0.01% | 0.06% | 0.04% | 96% | 94% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.01% | 0.00% | 97% | 95% |
| Class D | 0.02% | -0.10% | 0.05% | 85% | 99% |
| Class F | 0.00% | -0.01% | 0.06% | 96% | 95% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over GEO common base |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.04% | 0.33% | 97% | 97% |
| Class C | 0.06% | 0.19% | 0.15% | 96% | 98% |
| Class E | 0.21% | -0.12% | 0.01% | 96% | 97% |
| Overall | 0.07% | 0.05% | 0.19% | 96% | 97% |
| Class D | 0.05% | -0.10% | 0.47% | 87% | 104% |
| Class F | -0.10% | 0.27% | -0.35% | 97% | 93% |

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).