| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0063-v1 |
| --- | --- |

| Title: | CE1-3.1: Deblocking for blocks with different BCW weights |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei, | Email: | {chia-ming.tsai, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
|  | Anand Meher Kotra, Semih Esenlik Biao Wang, Han Gao, Elena Alshina, |  | Anand.meher.kotra @huawei.com |
|  | Xuewei Meng, Shanshe Wang, Siwei Ma, Xiaozhen Zheng |  | xwmeng@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | MediaTek Inc., Huawei Technologies Co., Ltd, Peking University, SZ DJI Technology Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports the results of CE1-3.1. In the proposed method, the boundary strength (bS) value of an edge for luma is set to 1 if the two adjacent blocks of the edge use bi-prediction and have different indices of bi-prediction with coding unit level weights (BCW). It is asserted that the proposed methods can improve subjective quality. Objective results of the proposed method are reported as follows.

VTM7.0-RA: {Y-BD-rate=0.02%, EncT=100%, DecT=100%}
VTM7.0-LB: {Y-BD-rate=0.03%, EncT=100%, DecT=99%}

# Introduction

The bS derivation in deblocking process of VTM7.0 [1] does not take into account that a potential discontinuity may arise when two blocks of an edge use different BCW weights.

# Proposed method

The proposed method is based on the aspect of JVET-P0109 [2], JVET-P0161 [3], and JVET-P0269 [4]. When deriving the bS value of an edge, bS for luma is set to 1 if the two sides of the edge are both bi-prediction coded but with different BCW indexes.

# Experimental results

The proposed methods are implemented on top of VTM7.0 [1], and the simulation was performed under the common test condition (CTC) [5]. Table 1 shows the objective results of the proposed method.

Table 1. Objective results of the proposed method

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.01% | 100% | 100% |
| Class A2 | -0.01% | -0.01% | -0.03% | 100% | 100% |
| Class B | 0.02% | 0.01% | 0.04% | 100% | 100% |
| Class C | 0.03% | -0.01% | 0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.00% | 0.02% | 100% | 100% |
| Class D | 0.00% | -0.03% | 0.08% | 100% | 100% |
| Class F | 0.00% | 0.01% | 0.00% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.29% | -0.19% | 100% | 98% |
| Class C | 0.03% | -0.20% | 0.06% | 101% | 100% |
| Class E | 0.05% | 0.15% | -0.64% | 100% | 100% |
| Overall | 0.03% | -0.15% | -0.22% | 100% | 99% |
| Class D | -0.04% | 0.07% | -0.41% | 100% | 101% |
| Class F | -0.06% | -0.41% | 0.25% | 100% | 100% |

# Subjective results

To illustrate the benefit of proposed method, two examples are shown in the following.

| (a) | (b) | (c) |
| --- | --- | --- |
| Figure 1. BasketballDrill test sequence, LDB, QP 37, POC #208, ALF switch off, (a) VTM7.0; (b) VTM7.0 + proposed method; (c) blocks marked in green are coded with nonzero BCW index. |  |  |

| (a) | (b) | (c) |
| --- | --- | --- |
| Figure 2. RedKayak test sequence, RA, QP 37, POC #86, ALF switch off, (a) VTM7.0; (b) VTM7.0 + proposed method; (c) blocks marked in green are coded with nonzero BCW index. |  |  |

# Specification text changes

8.8.3.5	Derivation process of boundary filtering strength

……

For xDi with i = 0..xN and yDj with j = 0..yN, the following applies:

- If edgeFlags[ xDi ][ yDj ] is equal to 0, the variable bS[ xDi ][ yDj ] is set equal to 0.

- Otherwise, the following applies:

- The sample values p0 and q0 are derived as follows:

    - If edgeType is equal to EDGE_VER, p0 is set equal to recPicture[ xCb + xDi − 1 ][ yCb + yDj ] and q0 is set equal to recPicture[ xCb + xDi ][ yCb + yDj ].

    - Otherwise (edgeType is equal to EDGE_HOR), p0 is set equal to recPicture[ xCb + xDi ][ yCb + yDj − 1 ] and q0 is set equal to recPicture[ xCb + xDi ][ yCb + yDj ].

- The variable bS[ xDi ][ yDj ] is derived as follows:

    - If cIdx is equal to 0 and both samples p0 and q0 are in a coding block with intra_bdpcm_luma_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if cIdx is greater than 0 and both samples p0 and q0 are in a coding block with intra_bdpcm_chroma_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if the sample p0 or q0 is in the coding block of a coding unit coded with intra prediction mode, bS[ xDi ][ yDj ] is set equal to 2.

    - Otherwise, if the block edge is also a transform block edge and the sample p0 or q0 is in a coding block with ciip_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 2.

    - Otherwise, if the block edge is also a transform block edge and the sample p0 or q0 is in a transform block which contains one or more non-zero transform coefficient levels, bS[ xDi ][ yDj ] is set equal to 1.

    - Otherwise, if the prediction mode of the coding subblock containing the sample p0 is different from the prediction mode of the coding subblock containing the sample q0 (i.e. one of the coding subblock is coded in IBC prediction mode and the other is coded in inter prediction mode), bS[ xDi ][ yDj ] is set equal to 1.

    - Otherwise, if cIdx is equal to 0, edgeFlags[ xDi ][ yDj ] is equal to 2, and one or more of the following conditions are true, bS[ xDi ][ yDj ] is set equal to 1:

      - The coding subblock containing the sample p0 and the coding subblock containing the sample q0 are both coded in IBC prediction mode, and the absolute difference between the horizontal or vertical component of the block vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in units of 1/16 luma samples.

      - For the prediction of the coding subblock containing the sample p0 different reference pictures or a different number of motion vectors are used than for the prediction of the coding subblock containing the sample q0.

NOTE 1 – The determination of whether the reference pictures used for the two coding sublocks are the same or different is based only on which pictures are referenced, without regard to whether a prediction is formed using an index into reference picture list 0 or an index into reference picture list 1, and also without regard to whether the index position within a reference picture list is different.

NOTE 2 – The number of motion vectors that are used for the prediction of a coding subblock with top-left sample covering ( xSb, ySb ), is equal to PredFlagL0[ xSb ][ ySb ] + PredFlagL1[ xSb ][ ySb ].

      - One motion vector is used to predict the coding subblock containing the sample p0 and one motion vector is used to predict the coding subblock containing the sample q0, and the absolute difference between the horizontal or vertical component of the motion vectors used is greater than or equal to 8 in units of 1/16 luma samples.

      - Two motion vectors and two different reference pictures are used to predict the coding subblock containing the sample p0, two motion vectors for the same two reference pictures are used to predict the coding subblock containing the sample q0 and the absolute difference between the horizontal or vertical component of the two motion vectors used in the prediction of the two coding subblocks for the same reference picture is greater than or equal to 8 in units of 1/16 luma samples.

      - Two motion vectors for the same reference picture are used to predict the coding subblock containing the sample p0, two motion vectors for the same reference picture are used to predict the coding subblock containing the sample q0 and both of the following conditions are true:

        - The absolute difference between the horizontal or vertical component of list 0 motion vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in 1/16 luma samples, or the absolute difference between the horizontal or vertical component of the list 1 motion vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in units of 1/16 luma samples.

        - The absolute difference between the horizontal or vertical component of list 0 motion vector used in the prediction of the coding subblock containing the sample p0 and the list 1 motion vector used in the prediction of the coding subblock containing the sample q0 is greater than or equal to 8 in units of 1/16 luma samples, or the absolute difference between the horizontal or vertical component of the list 1 motion vector used in the prediction of the coding subblock containing the sample p0 and list 0 motion vector used in the prediction of the coding subblock containing the sample q0 is greater than or equal to 8 in units of 1/16 luma samples.

      - If bcw_idx of coding block containing the sample p0 is different from the bcw_idx of the coding block containing the sample q0

    - Otherwise, the variable bS[ xDi ][ yDj ] is set equal to 0.

# Conclusion

This contribution presents the results of considering BCW indexes in deblocking. It is reported that BD-rate and runtime changes are negligible, and examples of subjective quality improvements can be found.

# References

- VTM7.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0

- A. M. Kotra, S. Esenlik, H. Gao, B. Wang, and E. Alshina, “AHG18/Non-CE5: Boundary strength fix for coding units using BCW,” Document of Joint Video Experts Team, JVET-P0109.

- C.-M. Tsai, C.-W. Hsu, Y.-W. Huang, and S.-M. Lei, “CE5-related: Deblocking considering prediction weights in BCW and TPM,” Document of Joint Video Experts Team, JVET-P0161.

- X. Meng, X. Zheng, S. Wang, and S. Ma, “AHG18/non-CE5: Deblocking for TPM and BCW,” Document of Joint Video Experts Team, JVET-P0269.

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).