| - Joint Video Experts Team (JVET) - of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 - 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0064 |
| --- | --- |

| Title: | CE2-3.1: Simplification of palette predictor update for small CUs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yung-Hsuan Chao, Ted Hsieh, Wei-Jung Chien, Vadim Seregin, Marta Karczewicz | Tel: Email: | +1-858-8452163 yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

- CE2-3.1a

- CE2-3.1b

- CE2-3.1c

- CE2-3.1d

7.4.10.6	Palette coding semantics

In the following semantics, the array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. The array indices xC, yC specify the location ( xC, yC ) of the sample relative to the top-left luma sample of the picture, when treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA; and relative to the top-left chroma sample of the picture, when treeType is equal to DUAL_TREE_CHROMA. The array index startComp specifies the first colour component of the current palette table. startComp equal to 0 indicates the Y component; startComp equal to 1 indicates the Cb component; startComp equal to 2 indicates the Cr component. numComps specifies the number of colour components in the current palette table.

The predictor palette consists of palette entries from previous coding units that are used to predict the entries in the current palette.

PredictorPaletteSize[ startComp ] specifies the size of the predictor palette for the first colour component of the current palette table startComp. PredictorPaletteSize[ startComp ] is derived as specified in clause 8.4.5.3.

PalettePredictorEntryReuseFlags[ i ] equal to 1 specifies that the i-th entry in the predictor palette is reused in the current palette. PalettePredictorEntryReuseFlags[ i ] equal to 0 specifies that the i-th entry in the predictor palette is not an entry in the current palette. All elements of the array PalettePredictorEntryReuseFlags[ i ] are initialized to 0.

palette_predictor_run is used to determine the number of zeros that precede a non-zero entry in the array PalettePredictorEntryReuseFlags.

It is a requirement of bitstream conformance that the value of palette_predictor_run shall be in the range of 0 to ( PredictorPaletteSize[ startComp ] − predictorEntryIdx ), inclusive, where predictorEntryIdx corresponds to the current position in the array PalettePredictorEntryReuseFlags. The variable NumPredictedPaletteEntries specifies the number of entries in the current palette that are reused from the predictor palette. The value of NumPredictedPaletteEntries shall be in the range of 0 to 31, inclusive.

num_signalled_palette_entries specifies the number of entries in the current palette that are explicitly signalled for the first colour component of the current palette table startComp.

When num_signalled_palette_entries is not present, it is inferred to be equal to 0.

The variable CurrentPaletteSize[ startComp ] specifies the size of the current palette for the first colour component of the current palette table startComp and is derived as follows:

CurrentPaletteSize[ startComp ] = NumPredictedPaletteEntries + num_signalled_palette_entries	(177)

The value of CurrentPaletteSize[ startComp ] shall be in the range of 0 to 31, inclusive.

new_palette_entries[ cIdx ][ i ] specifies the value for the i-th signalled palette entry for the colour component cIdx.

The variable PredictorPaletteEntries[ cIdx ][ i ] specifies the i-th element in the predictor palette for the colour component cIdx.

The variable CurrentPaletteEntries[ cIdx ][ i ] specifies the i-th element in the current palette for the colour component cIdx and is derived as follows:

numPredictedPaletteEntries = 0

numPredictorentriesUsed = ( cbWidth * cbHeight < PredictorPaletteSize[ startComp ])? cbWidth * cbHeight : PredictorPaletteSize[ startComp ]

numPredictorentriesUsed = ( cbWidth * cbHeight / 2 < PredictorPaletteSize[ startComp ])? ( cbWidth * cbHeight / 2 ): PredictorPaletteSize[ startComp ]
for( i = 0; i < numPredictorentriesUsed numPredictorentriesUsed; i++ )
	if( PalettePredictorEntryReuseFlags[ i ] ) {
		for( cIdx =startComp; cIdx < ( startComp + numComps ); cIdx++ )
			CurrentPaletteEntries[ cIdx ][ numPredictedPaletteEntries ] = PredictorPaletteEntries[ cIdx ][ i ]
		numPredictedPaletteEntries++
		}

for( cIdx = startComp; cIdx < (startComp + numComps); cIdx++)	(178)
	for( i = 0; i < num_signalled_palette_entries[startComp]; i++ )
		CurrentPaletteEntries[ cIdx ][ numPredictedPaletteEntries + i ] = new_palette_entries[ cIdx ][ i ]

palette_escape_val_present_flag equal to 1 specifies that the current coding unit contains at least one escape coded sample. palette_escape_val_present_flag equal to 0 specifies that there are no escape coded samples in the current coding unit. When not present, the value of palette_escape_val_present_flag is inferred to be equal to 1.

The variable MaxPaletteIndex specifies the maximum possible value for a palette index for the current coding unit. The value of MaxPaletteIndex is set equal to CurrentPaletteSize[ startComp ] − 1 + palette_escape_val_present_flag.

palette_idx_idc is an indication of an index to the palette table, CurrentPaletteEntries. The value of palette_idx_idc shall be in the range of 0 to MaxPaletteIndex, inclusive, for the first index in the block and in the range of 0 to ( MaxPaletteIndex − 1 ), inclusive, for the remaining indices in the block.

When palette_idx_idc is not present, it is inferred to be equal to 0.

palette_transpose_flag equal to 1 specifies that vertical traverse scan is applied for scanning the indices for samples in the current coding unit. palette_transpose_flag equal to 0 specifies that horizontal traverse scan is applied for scanning the indices for samples in the current coding unit. When not present, the value of palette_transpose_flag is inferred to be equal to 0.

The array TraverseScanOrder specifies the scan order array for palette coding. TraverseScanOrder is assigned the horizontal scan order HorTravScanOrder if palette_transpose_flag is equal to 0 and TraverseScanOrder is assigned the vertical scan order VerTravScanOrder if if palette_transpose_flag is equal to 1.

run_copy_flag equal to 1 specifies that the palette run type is the same as the run type at the previously scanned position and palette run index is the same as the index at the previous scanned position if copy_above_palette_indices_flag is equal to 0. Otherwise, run_copy_flag equal to 0 specifies that the palette run type is different from the run type at the previously scanned position.

copy_above_palette_indices_flag equal to 1 specifies that the palette index is equal to the palette index at the same location in the row above if horizontal traverse scan is used or the same location in the left column if vertical traverse scan is used. copy_above_palette_indices_flag equal to 0 specifies that an indication of the palette index of the sample is coded in the bitstream or inferred.

The variable CopyAboveIndicesFlag[ xC ][ yC ] equal to 1 specifies that the palette index is copied from the palette index in the row above (horizontal scan) or left column (vertical scan). CopyAboveIndicesFlag[ xC ][ yC ] equal to 0 specifies that the palette index is explicitly coded in the bitstream or inferred.

The variable PaletteIndexMap[ xC ][ yC ] specifies a palette index, which is an index to the array represented by CurrentPaletteEntries. The value of PaletteIndexMap[ xC ][ yC ] shall be in the range of 0 to MaxPaletteIndex, inclusive.

The variable adjustedRefPaletteIndex is derived as follows:

adjustedRefPaletteIndex = MaxPaletteIndex + 1
if( PaletteScanPos > 0 ) {
	xcPrev = x0 + TraverseScanOrder[ log2CbWidth ][ log2bHeight ][ PaletteScanPos − 1 ][ 0 ]
	ycPrev = y0 + TraverseScanOrder[ log2CbWidth ][ log2bHeight ][ PaletteScanPos − 1 ][ 1 ]
	if( CopyAboveIndicesFlag[ xcPrev ][ ycPrev ]  = =  0 ) {
		adjustedRefPaletteIndex = PaletteIndexMap[ xcPrev ][ ycPrev ] {	(179)
	}
	else {
	 	if( !palette_transpose_flag )
			adjustedRefPaletteIndex = PaletteIndexMap[ xC ][ yC − 1 ]
		else
			adjustedRefPaletteIndex = PaletteIndexMap[ xC − 1 ][ yC ]
	}
}

When CopyAboveIndicesFlag[ xC ][ yC ] is equal to 0, the variable CurrPaletteIndex is derived as follows:

if( CurrPaletteIndex  >=  adjustedRefPaletteIndex )
	CurrPaletteIndex++		(180)

palette_escape_val specifies the quantized escape coded sample value for a component.

The variable PaletteEscapeVal[ cIdx ][ xC ][ yC ] specifies the escape value of a sample for which PaletteIndexMap[ xC ][ yC ] is equal to MaxPaletteIndex and palette_escape_val_present_flag is equal to 1. The array index cIdx specifies the colour component.

It is a requirement of bitstream conformance that PaletteEscapeVal[ cIdx ][ xC ][ yC ] shall be in the range of 0 to (1  <<  ( BitDepth + 1 ) ) − 1, inclusive, for cIdx equal to 0, and in the range of 0 to (1  <<  ( BitDepth + 1 ) ) − 1, inclusive, for cIdx not equal to 0.

8.4.5.3	Decoding process for palette mode

Inputs to this process are:

- a location ( xCbComp, yCbComp ) specifying the top-left sample of the current coding block relative to the top-left sample of the current picture,

- a variable treeType specifying whether a single or a dual tree is used and if a dual tree is used, it specifies whether the current tree corresponds to the luma or chroma components,

- a variable cIdx specifying the colour component of the current block,

- two variables nCbW and nCbH specifying the width and height of the current coding block, respectively.

Output of this process is an array recSamples[ x ][ y ], with x = 0.. nCbW − 1, y = 0.. nCbH − 1 specifying reconstructed sample values for the block.

Depending on the value of treeType, the variables startComp and numComps are derived as follows:

- If treeType is equal to SINGLE_TREE:

startComp = 0		(444)

numComps = 3		(445)

- Otherwise, treeType is equal to DUAL_TREE_LUMA:

startComp = 0		(446)

numComps = 1		(447)

- Otherwise, treeType is equal to DUAL_TREE_CHROMA:

startComp = 1		(448)

numComps = 2		(449)

Depending on the value of cIdx, the variables nSubWidth and nSubHeight are derived as follows:

- If cIdx is greater than 0 and startComp is equal to 0, nSubWidth is set to SubWidthC and nSubHeight is set to SubHeightC.

- Otherwise, nSubWidth is set to 1 and nSubHeight is set to 1.

The ( nCbW x nCbH ) block of the reconstructed sample array recSamples at location ( xCbComp, yCbComp ) is represented by recSamples[ x ][ y ] with x = 0..nCTbW − 1 and y = 0..nCbH − 1, and the value of recSamples[ x ][ y ] for each x in the range of 0 to nCbW − 1, inclusive, and each y in the range of 0 to nCbH − 1, inclusive, is derived as follows:

- The variables xL, yL, xCbL, and yCbL are derived as follows:

xL = x * nSubWidth		(450)

yL = y * nSubHeight		(451)

xCbL = xCbComp * nSubWidth		(452)

yCbL = yCbComp * nSubHeight		(453)

- The variable bIsEscapeSample is derived as follows:

- If PaletteIndexMap[ xCbL + xL ][ yCbL + yL ] is equal to MaxPaletteIndex and palette_escape_val_present_flag is equal to 1, bIsEscapeSample is set equal to 1.

- Otherwise, bIsEscapeSample is set equal to 0.

- If bIsEscapeSample is equal to 0, the following applies:

recSamples[ x ][ y ] = CurrentPaletteEntries[ cIdx ][ PaletteIndexMap[ xCbL + xL ][ yCbL + yL ] ]	(454)

- Otherwise (bIsEscapeSample is equal to 1), the following ordered steps apply:

- The quantization parameter qP is derived as follows:

- If cIdx is equal to 0,

qP = Max( QpPrimeTsMin, Qp′Y )		(455)

- Otherwise, if cIdx is equal to 1,

qP = Max( QpPrimeTsMin, Qp′Cb )		(456)

- Otherwise (cIdx is equal to 2),

qP = Max( QpPrimeTsMin, Qp′Cr )		(457)

- The list levelScale[ ] is specified as levelScale[ k ] = { 40, 45, 51, 57, 64, 72 } with k = 0..5.

- The following applies:

tmpVal = ( PaletteEscapeVal[ cIdx ][ xCbL + xL ][ yCbL + yL ] *
			levelScale[ qP%6 ] )  <<  ( qP / 6 ) + 32 )  >>  6	(458)

recSamples[ x ][ y ] = Clip3( 0, ( 1  <<  BitDepth ) − 1, tmpVal )	(459)

When one of the following conditions is true:

- cIdx is equal to 0 and numComps is equal to 1 and cbWidth * cbHeight >= 64 and cbWidth * cbHeight > 64;

- cIdx is equal to 2 and cbWidth * cbHeight >= 64 and cbWidth * cbHeight > 64;

the value PredictorPaletteSize[ startComp ] and the array PredictorPaletteEntries are derived or modified as follows:

for( i = 0; i < CurrentPaletteSize[ startComp ]; i++ )
	for( compIdx = startComp; compIdx < (startComp + numComps); compIdx++ )
		newPredictorPaletteEntries[ compIdx ][ i ] = CurrentPaletteEntries[ compIdx ][ i ]
newPredictorPaletteSize = CurrentPaletteSize[ startComp ]
for( i = 0; i < PredictorPaletteSize[ startComp ]  &&  newPredictorPaletteSize < 63; i++ )
	if( !PalettePredictorEntryReuseFlags[ i ] ) {
		for( compIdx = startComp; compIdx < (startComp + numComps); compIdx++ )	(460)
			newPredictorPaletteEntries[ compIdx ][ newPredictorPaletteSize ] =
				PredictorPaletteEntries[ compIdx ][ i ]
		newPredictorPaletteSize++
	}
for( compIdx = startComp; compIdx < ( startComp + numComps ); compIdx++ )
	for( i = 0; i < newPredictorPaletteSize; i++ )
		PredictorPaletteEntries[ compIdx ][ i ] = newPredictorPaletteEntries[ compIdx ][ i ]
PredictorPaletteSize[ startComp ] = newPredictorPaletteSize

It is a requirement of bitstream conformance that the value of PredictorPaletteSize[ startComp ] shall be in the range of 0 to 63, inclusive.

# Reference

- Y.-H. Chao, T. Hsieh, W.-J. Chien, V. Seregin, and M. Karczewicz, “Non-CE8: Simplification of palette predictor update for small CUs”, JVET-P0475, 16th meeting, Geneva, CH, October 2019.

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 2 (CE2): Palette Mode Coding”, JVET-P2022, 16th meeting, Geneva, CH, October 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).