| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0064 |
| --- | --- |

| Title: | CE2-3.1: Simplification of palette predictor update for small CUs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yung-Hsuan Chao, Ted Hsieh, Wei-Jung Chien, Vadim Seregin, Marta Karczewicz | Tel: Email: | +1-858-8452163 yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In this proposal, two methods are proposed to simplify the palette predictor update for small coding in order to solve the issue of pipeline latency:

In the first method, the palette predictor elements allowed to be used in predicting palette table are restricted to the first M elements. In test CE2-3.1a, the utilized palette predictors are restricted to the first M = (CU width) x (CU height) elemetns. In test CE2-3.1b, the utilized palette predictors are restricted to the first M = 12 (CU width) x (CU height) elements. The results on YUV4:4:4 sequences show minor loss compared to VTM7.0:

Dual tree OFF:

CE2-3.1a:  0.04% AI, 0.03% RA, 0.00% LB

CE2-3.1b:  0.22% AI, 0.18% RA, 0.04% LB

Dual tree ON:

CE2-3.1a:  -0.01% AI, 0.01% RA, -0.02% LB

CE2-3.1b:  -0.01% AI, 0.05% RA, -0.03% LB

In the second method, palette predictor update is bypassed for CU size (CU width) x (CU height) < N. In test CE2-3.1c, the update of palette predictor is bypassed for coding units of size < 8x8.  In test CE2-3.1d, the update of palette predictor is bypassed for coding units of size ≤ 8x8. The results on YUV4:4:4 sequences show minor loss compared to VTM7.0:

Dual tree OFF:

CE2-3.1c:  0.07% AI, 0.06% RA, 0.04% LB

CE2-3.1d:  0.22% AI, 0.17% RA, 0.08% LB

Dual tree ON:

CE2-3.1c:  0.03% AI, 0.07% RA, 0.06% LB

CE2-3.1d:  0.09% AI, 0.13% RA, 0.01% LB

# Introduction

In VTM6.0, If the palette mode is utilized, the pixels values in the CU are represented by a small set of representative colour values. The set is referred to as the palette table. For coding of the palette table, a palette predictor is maintained. For each entry in the palette predictor, a reuse flag is signaled to indicate whether it is part of the current palette. After encoding a palette coded CU, the palette predictor will be updated using the current palette (consists of reused palette predictors and newly signaled palette colors), and entries from the previous palette predictor which are not reused in the current palette. The unused elements from the previous palette predictor will be added after the current palette sequencitally until the maximum predictor size allowed (63 in VTM7.0) is reached. The process is called palette stuffing. Figure 1 and 2 illustrate the palette table derivation and the update of palette predictor.

The palette stuffing, however, may takes multiple cycles to complete because the encoder/decoder needs to check sequentially whether each entry in the predictor is used to predict the palette table. After checking predictor entries, fill in the updated color entries will also be a sequential process. Since the maximum palette predictor size currently set in VTM6.0 is 63 and the predictors have to be updated before encoding/decoding the next palette coded coding units, for small blocks such as 4x4, 4x8, 8x4, and 8x8, the palette predictor update process can be the bottleneck and cause latency in the pipeline.

Figure 1: Palette table derivation from palette predictor (entries marked as blue) and newly signaled colours

Figure 2: Palette predictor update with current palette table and un-used palette predictor entries (the ordering is maintained).

# Patent rights declaration(s)

## Method 1

In the first method proposed (CE2-3.1a and CE2-3.1b), the palette predictor elements allowed to be used in predicting palette table is restricted to the first M elements: M=WxH and the first M=WxH/2 elements (W and H stands for width and height of the CU). Figure 3 illustrates the process.

Figure 3: Illustration of palette stuffing when the prediction entries used for prediction is restricted to the first M elements

As a result, the number of predictor entries encoder/decoder needs to check (whether the element is used for prediction or not) for palette stuffing is reduced to M. For predictor entries after the M’th position (marked as grey in the figure), since the entries are not used for prediction, the elements can be directly copied until the maximum palette size is reached.

## Method 2

In the second method proposed (CE2-3.1c and CE2-3.1d), palette predictor update is bypassed for CUs of size < 8x8 and CUs of size ≤ 8x8, namely, the palette predictor will stay the same as the previous palette predictor.

# Simulation Results

The proposed method is evaluated according to the CE2 description [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. The IBC, HashME, BDPCM (=2), PLT are turned on for all Classes. The results with DualITree OFF are shown in Table 1-4.

## Dual Tree OFF

Method1:

Table 1: Results of CE2-3.1a, DualITree = 0 for standard QP (22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 PLT=1 DT=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.08% | 0.09% | 0.12% | 101% | 100% |
| TGM 720p | 0.04% | 0.00% | 0.05% | 100% | 101% |
| Animation | -0.02% | -0.01% | -0.04% | 100% | 103% |
| Mixed content | 0.08% | 0.01% | 0.00% | 100% | 102% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 99% | 96% |
| Overall | 0.04% | 0.02% | 0.04% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM7.0 PLT=1 DT=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.06% | 0.06% | 0.05% | 100% | 99% |
| TGM 720p | 0.02% | 0.09% | 0.16% | 100% | 103% |
| Animation | 0.02% | 0.06% | 0.02% | 100% | 99% |
| Mixed content | 0.02% | 0.07% | 0.11% | 100% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 97% |
| Overall | 0.03% | 0.06% | 0.08% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM7.0 PLT=1 DT=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.07% | 0.13% | 0.12% | 99% | 101% |
| TGM 720p | -0.11% | 0.13% | 0.26% | 100% | 98% |
| Animation | -0.03% | 0.02% | -0.06% | 99% | 99% |
| Mixed content | 0.10% | 0.12% | 0.38% | 100% | 99% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 98% |
| Overall | 0.00% | 0.09% | 0.15% | 100% | 99% |

Table 2: Results of CE2-3.1b, DualITree = 0 for standard QP (22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0rc1 PLT=0 DT=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.47% | 0.48% | 0.48% | 102% | 100% |
| TGM 720p | 0.13% | 0.14% | 0.20% | 100% | 99% |
| Animation | 0.04% | 0.02% | 0.01% | 101% | 103% |
| Mixed content | 0.33% | 0.25% | 0.21% | 101% | 101% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 98% |
| Overall | 0.22% | 0.20% | 0.21% | 101% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM7.0rc1 PLT=0 DT=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.33% | 0.33% | 0.30% | 101% | 100% |
| TGM 720p | 0.17% | 0.16% | 0.26% | 100% | 100% |
| Animation | 0.03% | 0.02% | -0.01% | 101% | 100% |
| Mixed content | 0.28% | 0.29% | 0.15% | 101% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 98% |
| Overall | 0.18% | 0.18% | 0.17% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM7.0rc1 PLT=0 DT=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.09% | 0.14% | 0.09% | 99% | 99% |
| TGM 720p | -0.03% | 0.06% | -0.02% | 101% | 102% |
| Animation | -0.02% | -0.01% | -0.06% | 99% | 100% |
| Mixed content | 0.16% | 0.21% | 0.35% | 101% | 98% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Overall | 0.04% | 0.09% | 0.07% | 100% | 100% |

Method2:

Table 3: Results of CE2-3.1c, DualITree = 0 for standard QP (22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0rc1 PLT=0 DT=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.13% | 0.19% | 0.16% | 101% | 102% |
| TGM 720p | 0.03% | 0.12% | 0.11% | 100% | 99% |
| Animation | 0.10% | 0.20% | 0.18% | 100% | 102% |
| Mixed content | 0.06% | 0.03% | 0.06% | 100% | 102% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Overall | 0.07% | 0.12% | 0.11% | 100% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM7.0rc1 PLT=0 DT=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.12% | 0.17% | 0.13% | 100% | 100% |
| TGM 720p | 0.03% | 0.09% | 0.18% | 100% | 99% |
| Animation | 0.09% | 0.21% | 0.30% | 101% | 99% |
| Mixed content | 0.00% | 0.01% | 0.00% | 100% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 97% |
| Overall | 0.06% | 0.11% | 0.13% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM7.0rc1 PLT=0 DT=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.06% | 0.15% | 0.13% | 100% | 102% |
| TGM 720p | -0.11% | -0.08% | -0.16% | 99% | 103% |
| Animation | 0.12% | 0.47% | 0.52% | 100% | 101% |
| Mixed content | 0.14% | 0.29% | 0.46% | 100% | 98% |
| Camera-Captured content | 0.00% | 0.02% | 0.03% | 101% | 98% |
| Overall | 0.04% | 0.16% | 0.18% | 100% | 101% |

Table 4: Results of CE2-3.1d, DualITree = 0 for standard QP (22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0rc1 PLT=0 DT=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.41% | 0.55% | 0.47% | 101% | 99% |
| TGM 720p | 0.22% | 0.34% | 0.42% | 100% | 99% |
| Animation | 0.21% | 0.35% | 0.38% | 100% | 100% |
| Mixed content | 0.13% | 0.23% | 0.20% | 100% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 96% |
| Overall | 0.22% | 0.33% | 0.33% | 100% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM7.0rc1 PLT=0 DT=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.38% | 0.49% | 0.40% | 101% | 100% |
| TGM 720p | 0.07% | 0.15% | 0.30% | 100% | 99% |
| Animation | 0.10% | 0.14% | 0.19% | 101% | 99% |
| Mixed content | 0.22% | 0.27% | 0.24% | 101% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 98% |
| Overall | 0.17% | 0.24% | 0.26% | 101% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM7.0rc1 PLT=0 DT=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.11% | 0.29% | 0.28% | 100% | 102% |
| TGM 720p | -0.03% | 0.12% | 0.17% | 98% | 98% |
| Animation | 0.12% | 0.64% | 0.50% | 99% | 101% |
| Mixed content | 0.21% | 0.35% | 0.44% | 100% | 99% |
| Camera-Captured content | 0.00% | 0.02% | 0.03% | 101% | 99% |
| Overall | 0.08% | 0.29% | 0.29% | 100% | 100% |

## Dual Tree ON

The results with DualITree ON are shown in Table 5-8.

Method1:

Table 5: Results of CE2-3.1a, DualITree = 1 for standard QP (22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 DT=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.04% | -0.01% | -0.01% | 100% | 101% |
| TGM 720p | 0.02% | -0.01% | 0.05% | 99% | 102% |
| Animation | 0.01% | 0.01% | 0.00% | 100% | 102% |
| Mixed content | -0.02% | 0.01% | 0.01% | 100% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 107% |
| Overall | -0.01% | 0.00% | 0.01% | 100% | 102% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM7.0 DT=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.01% | 0.00% | 0.02% | 100% | 100% |
| TGM 720p | -0.01% | -0.01% | 0.05% | 100% | 98% |
| Animation | 0.00% | 0.03% | 0.00% | 100% | 99% |
| Mixed content | 0.07% | 0.11% | 0.14% | 100% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Overall | 0.01% | 0.03% | 0.05% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM7.0 DT=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.03% | -0.01% | 0.00% | 100% | 100% |
| TGM 720p | -0.05% | -0.22% | -0.03% | 101% | 100% |
| Animation | -0.02% | 0.04% | -0.07% | 99% | 98% |
| Mixed content | 0.00% | -0.10% | 0.01% | 101% | 102% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Overall | -0.02% | -0.07% | -0.02% | 100% | 100% |

Table 6: Results of CE2-3.1b, DualITree = 1 for standard QP (22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 DT=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.06% | -0.01% | -0.02% | 100% | 102% |
| TGM 720p | 0.04% | -0.04% | -0.03% | 100% | 102% |
| Animation | -0.01% | 0.03% | 0.02% | 101% | 102% |
| Mixed content | 0.01% | 0.01% | -0.01% | 100% | 102% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 104% |
| Overall | -0.01% | -0.01% | -0.01% | 100% | 102% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM7.0 DT=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.07% | 0.06% | 0.10% | 101% | 101% |
| TGM 720p | 0.09% | 0.07% | 0.08% | 101% | 99% |
| Animation | -0.05% | 0.00% | -0.02% | 100% | 99% |
| Mixed content | 0.09% | 0.10% | 0.22% | 101% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Overall | 0.05% | 0.05% | 0.08% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM7.0 DT=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.02% | 0.09% | 0.13% | 100% | 97% |
| TGM 720p | -0.02% | 0.02% | -0.11% | 100% | 100% |
| Animation | -0.09% | -0.09% | -0.10% | 99% | 99% |
| Mixed content | -0.05% | -0.10% | 0.09% | 101% | 101% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | -0.03% | -0.01% | 0.00% | 100% | 99% |

Method2:

Table 7: Results of CE2-3.1c, DualITree = 1 for standard QP (22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 DT=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.05% | 0.04% | 0.04% | 100% | 105% |
| TGM 720p | 0.04% | -0.01% | 0.07% | 100% | 101% |
| Animation | 0.02% | 0.06% | 0.07% | 100% | 100% |
| Mixed content | 0.02% | 0.05% | 0.02% | 100% | 99% |
| Camera-Captured content | 0.00% | -0.02% | -0.01% | 100% | 104% |
| Overall | 0.03% | 0.03% | 0.05% | 100% | 102% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM7.0 DT=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.12% | 0.14% | 0.13% | 100% | 101% |
| TGM 720p | 0.07% | 0.08% | 0.09% | 100% | 102% |
| Animation | 0.05% | 0.07% | 0.06% | 100% | 101% |
| Mixed content | 0.09% | 0.12% | 0.22% | 99% | 100% |
| Camera-Captured content | -0.01% | -0.01% | 0.03% | 101% | 101% |
| Overall | 0.07% | 0.09% | 0.11% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM7.0 DT=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.06% | 0.09% | 0.07% | 99% | 98% |
| TGM 720p | 0.13% | -0.12% | -0.01% | 101% | 102% |
| Animation | 0.03% | 0.03% | -0.05% | 100% | 99% |
| Mixed content | 0.04% | -0.07% | 0.33% | 100% | 98% |
| Camera-Captured content | 0.03% | -0.01% | 0.00% | 100% | 109% |
| Overall | 0.06% | -0.01% | 0.07% | 100% | 101% |

Table 8: Results of CE2-3.1d, DualITree = 1 for standard QP (22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 DT=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.15% | 0.09% | 0.09% | 100% | 102% |
| TGM 720p | 0.14% | -0.04% | 0.10% | 100% | 100% |
| Animation | 0.05% | 0.11% | 0.14% | 101% | 98% |
| Mixed content | 0.06% | 0.08% | 0.06% | 100% | 101% |
| Camera-Captured content | 0.00% | -0.02% | -0.01% | 101% | 101% |
| Overall | 0.09% | 0.04% | 0.08% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM7.0 DT=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.24% | 0.30% | 0.24% | 101% | 101% |
| TGM 720p | 0.19% | 0.13% | 0.22% | 100% | 101% |
| Animation | 0.01% | 0.02% | 0.09% | 100% | 99% |
| Mixed content | 0.13% | 0.13% | 0.16% | 102% | 101% |
| Camera-Captured content | -0.01% | 0.00% | 0.00% | 101% | 102% |
| Overall | 0.13% | 0.13% | 0.16% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM7.0 DT=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.16% | 0.30% | 0.22% | 99% | 98% |
| TGM 720p | -0.10% | -0.04% | 0.20% | 99% | 100% |
| Animation | -0.05% | 0.11% | 0.05% | 99% | 101% |
| Mixed content | 0.01% | 0.09% | 0.26% | 101% | 98% |
| Camera-Captured content | 0.01% | -0.03% | 0.02% | 100% | 103% |
| Overall | 0.01% | 0.10% | 0.17% | 99% | 100% |

# Reference

- Y.-H. Chao, T. Hsieh, W.-J. Chien, V. Seregin, and M. Karczewicz, “Non-CE8: Simplification of palette predictor update for small CUs”, JVET-P0475, 16th meeting, Geneva, CH, October 2019.

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 2 (CE2): Palette Mode Coding”, JVET-P2022, 16th meeting, Geneva, CH, October 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).