| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0065 |
| --- | --- |

| Title: | Level restrictions on maximum tile width for line buffer reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaru Ikeda Yoichi Yagasaki Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81 50 3750 2782 Masaru.Ikeda@sony.com Yoichi.Yagasaki@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This document proposes to have level constraints on the maximum tile size to reduce line buffer. It is proposed that the constraint on maximum tile width is introduced and the value is set depending on maximum picture size. It can provide hardware decoder implementations with the reduction of the amount of line buffer that is required for in-loop filters.

Introduction

In hardware, the decoding processes are normally conducted in CTU raster scan order. In that case, the line buffers are required to store the samples at the CTU boundaries since those samples are filtered in next CTU line where all samples for loop filtering become available. The higher the resolution of the video becomes, the larger a line buffer size becomes, and then the total line buffer size becomes larger. In HEVC, the value of pic_width_in_luma_samples shall be less than or equal to Sqrt( MaxLumaPs * 8 ). The value of maximum pic_width_in_luma_samples is shown per level in Table 1. It is depending on maximum luma picture size (= Sqrt( MaxLumaPs * 8 )). The corresponding picture size is panoramic like 4,222 x 527, 8,444 x 1,055 and 16,888 x 2,111 for level 4.x, 5.x and 6.x, respectively. The decoder needs to have large buffer to support such picture sizes using single tile.

Table 1: General tier and level limits

| Level | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) |  | Max slice segments per picture MaxSliceSegmentsPerPicture | Max # of tile rows MaxTileRows | Max # of tile columns MaxTileCols | Max pic width in luma samples = Sqrt( MaxLumaPs * 8 ) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier |  |  |  |  |
| 4 | 2 228 224 | 12 000 | 30 000 | 75 | 5 | 5 | 4 222 |
| 4.1 | 2 228 224 | 20 000 | 50 000 | 75 | 5 | 5 | 4 222 |
| 5 | 8 912 896 | 25 000 | 100 000 | 200 | 11 | 10 | 8 444 |
| 5.1 | 8 912 896 | 40 000 | 160 000 | 200 | 11 | 10 | 8 444 |
| 5.2 | 8 912 896 | 60 000 | 240 000 | 200 | 11 | 10 | 8 444 |
| 6 | 35 651 584 | 60 000 | 240 000 | 600 | 22 | 20 | 16 888 |
| 6.1 | 35 651 584 | 120 000 | 480 000 | 600 | 22 | 20 | 16 888 |
| 6.2 | 35 651 584 | 240 000 | 800 000 | 600 | 22 | 20 | 16 888 |

Proposed constraints

This contribution document proposes to introduce the same constraint on maximum picture width, Max_pic_width_in_luma_samples, as for HEVC, and, in addition, another constraint on maximum tile width, Max_tile_width_in_luma_samples. The value of max tile width is dependent on MaxLumaPs, which is similar with the constraints on pic_width_in_luma_samples, but the weight is smaller (2 instead of 8). With this proposal, the decoder can save the line buffer size. And it can decode the panoramic picture sizes, 4,222 x 527, 8,444 x 1,055 and 16,888 x 2,111 for 4.x, 5.x and 6.x, respectively, with multiple tiles which are supported in HEVC. Looking at single tile, the size is 2,111 x 1,055, 4,222 x 2,111 and 8,444 x 4,222 for level 4.x, 5.x and 6.x, respectively. Single tile can support typical picture size (2Kx1K, 4Kx2K and 8Kx4K) for each level, thus this constraint doesn’t give us so much impact in most of use cases.

Table 2: Proposed general tier and level limits

| Level | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) |  | Max slice segments per picture MaxSliceSegmentsPerPicture | Max # of tile rows MaxTileRows | Max # of tile columns MaxTileCols | Max tile width in luma samples = Sqrt( MaxLumaPs * 2) | Pic width in luma samples = Sqrt( MaxLumaPs * 8 ) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier |  |  |  |  |  |
| 4 | 2 228 224 | 12 000 | 30 000 | 75 | 5 | 5 | 2 111 | 4 222 |
| 4.1 | 2 228 224 | 20 000 | 50 000 | 75 | 5 | 5 | 2 111 | 4 222 |
| 5 | 8 912 896 | 25 000 | 100 000 | 200 | 11 | 10 | 4 222 | 8 444 |
| 5.1 | 8 912 896 | 40 000 | 160 000 | 200 | 11 | 10 | 4 222 | 8 444 |
| 5.2 | 8 912 896 | 60 000 | 240 000 | 200 | 11 | 10 | 4 222 | 8 444 |
| 6 | 35 651 584 | 60 000 | 240 000 | 600 | 22 | 20 | 8 444 | 16 888 |
| 6.1 | 35 651 584 | 120 000 | 480 000 | 600 | 22 | 20 | 8 444 | 16 888 |
| 6.2 | 35 651 584 | 240 000 | 800 000 | 600 | 22 | 20 | 8 444 | 16 888 |

Conclusion

This contribution document proposes to introduce the same constraint on maximum picture width, Max_pic_width_in_luma_samples, as for HEVC, and, in addition, another constraint on maximum tile width, Max_tile_width_in_luma_samples for line buffer reduction in HW decoder side. We recommend considering that those constraints and are included into next version of the specification.

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).