| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0066-v2 |
| --- | --- |

| Title: | CE2-1.2: Fixed-length binarization of palette escape value |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu, Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Xianglin Wang | Email: | {jhuhong-jheng, xiaoyuxiu, yiwenchen, tsung-chuanma, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of Core Experiment (CE) 2-1.2 [1]. In this test, a new binarization method for palette escape coding is tested. The binarization of quantized escape samples is adjusted based on the dynamic range of the quantized values. The fixed-length binarization process is used for escape samples. The proposed method is implemented and evaluated with VTM7[2] by following CE2 test conditions. Results are reported as below:

Dual tree OFF:

Standard QP (22, 27, 32, 37): -0.07% AI, -0.03% RA, -0.04% LB

Low QP (2, 7, 12, 17): -1.2% AI, -0.73% RA, -0.54% LB

Dual tree ON:

Standard QP (22, 27, 32, 37): -0.04% AI, 0.03% RA, 0.02% LB

Low QP (2, 7, 12, 17): -0.7% AI, -0.5% RA, -0.41% LB

The encoding and decoding time are updated in version 2.

# Introduction

In current VVC [3], the palette coding is beneficial when there are limited pixels and the texture is homogeneous in a CU when the coded image is computer generated. If there is a pixel which has different values from other pixels, it is coded not as a regular palette entry, but an escape value. The sample values of each component of the escape value are coded and transmitted after quantized. Each escape sample value is binarized with EG3, which has the bin length from 4 to 18 when the sample value is 0 to (1 << bitdepth) – 1, where the bitdepth in the current VTM is set to 10. Then the binarized symbol is bypass coded. In the worst case, the escape value is coded with 54 bits, whereas it needs only 30 bits when the pixel is coded with FL coding. Therefore, the efficient escape value coding is provided in this proposal.

# Proposal

In this proposal, a new binarization method for palette escape coding is tested. The binarization of quantized escape samples is adjusted based on the dynamic range of the quantized values. The fix-length binarization process is used for escape samples. The length of fix-length binarization process is derived as follows:

length = max (1, bitDepth – (max (QpPrimeTsMin, Qp) - 4)/6).

# Simulation Results

The proposed methods were implemented based on the VTM-7.0 reference software [2] and tested using the test condition and evaluation criteria that are used in CE2 [1] are applied.

Table 1: Results of Test2 for standard QP (22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DualTree = 1 |  |  |  |  | DualTree = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM | -0.09% | -0.07% | -0.06% | 101% | 100% | -0.20% | -0.08% | -0.04% | 100% | 99% |
| TGM | -0.01% | -0.08% | -0.02% | 100% | 100% | -0.04% | -0.03% | -0.01% | 101% | 100% |
| A | -0.03% | -0.01% | 0.03% | 101% | 99% | -0.01% | 0.04% | 0.00% | 100% | 100% |
| M | -0.04% | 0.02% | -0.03% | 100% | 100% | -0.05% | 0.02% | 0.03% | 100% | 100% |
| CC | 0.00% | -0.01% | 0.00% | 100% | 101% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | -0.04% | -0.04% | -0.02% | 100% | 100% | -0.07% | -0.02% | -0.01% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |  |  |  |  |  |
|  | DualTree = 1 |  |  |  |  | DualTree = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM | -0.03% | -0.06% | -0.04% | 100% | 100% | -0.15% | 0.04% | -0.09% | 100% | 100% |
| TGM | 0.01% | -0.01% | 0.09% | 100% | 100% | -0.03% | 0.05% | -0.02% | 100% | 100% |
| A | 0.05% | 0.03% | 0.11% | 99% | 100% | 0.09% | 0.05% | 0.07% | 100% | 100% |
| M | 0.14% | 0.29% | 0.28% | 100% | 100% | 0.01% | 0.09% | 0.01% | 99% | 100% |
| CC | 0.01% | -0.03% | 0.02% | 100% | 101% | 0.00% | 0.00% | -0.01% | 101% | 100% |
| Overall | 0.03% | 0.04% | 0.09% | 100% | 100% | -0.03% | 0.05% | -0.02% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | DualTree = 1 |  |  |  |  | DualTree = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM | -0.04% | -0.07% | -0.05% | 101% | 100% | -0.10% | -0.01% | 0.00% | 100% | 100% |
| TGM | 0.09% | 0.02% | 0.21% | 100% | 100% | -0.15% | -0.03% | 0.14% | 100% | 100% |
| A | 0.08% | 0.11% | -0.03% | 100% | 101% | 0.04% | 0.15% | 0.00% | 100% | 100% |
| M | -0.05% | -0.08% | 0.27% | 101% | 100% | 0.10% | 0.42% | 0.98% | 99% | 100% |
| CC | 0.02% | -0.04% | -0.02% | 100% | 101% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.02% | -0.01% | 0.08% | 100% | 100% | -0.04% | 0.10% | 0.22% | 100% | 100% |

Table 2: Results of Test2 for low QP (2, 7, 12, 17)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DualTree = 1 |  |  |  |  | DualTree = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM | -1.40% | -1.26% | -1.31% | 101% | 100% | -2.58% | -3.03% | -2.93% | 99% | 101% |
| TGM | -0.46% | -0.42% | -0.44% | 100% | 100% | -0.91% | -0.79% | -1.13% | 101% | 99% |
| A | -0.31% | -0.18% | -0.17% | 100% | 100% | -0.37% | -0.63% | -0.69% | 100% | 101% |
| M | -0.94% | -1.08% | -1.13% | 100% | 99% | -1.39% | -1.74% | -1.71% | 100% | 100% |
| CC | 0.00% | 0.00% | -0.02% | 101% | 99% | 0.00% | 0.01% | 0.00% | 101% | 99% |
| Overall | -0.70% | -0.66% | -0.68% | 100% | 100% | -1.20% | -1.40% | -1.47% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |  |  |  |  |  |
|  | DualTree = 1 |  |  |  |  | DualTree = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM | -1.23% | -1.23% | -1.25% | 100% | 100% | -1.80% | -1.95% | -1.84% | 100% | 100% |
| TGM | -0.30% | -0.19% | -0.34% | 100% | 100% | -0.42% | -0.32% | -0.57% | 100% | 100% |
| A | -0.24% | -0.34% | -0.33% | 100% | 100% | -0.29% | -0.36% | -0.40% | 100% | 100% |
| M | -0.39% | 0.30% | 0.24% | 100% | 100% | -0.63% | -0.55% | -0.49% | 99% | 101% |
| CC | -0.01% | -0.01% | 0.03% | 100% | 101% | -0.01% | -0.01% | 0.00% | 100% | 101% |
| Overall | -0.50% | -0.36% | -0.41% | 100% | 100% | -0.73% | -0.74% | -0.77% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | DualTree = 1 |  |  |  |  | DualTree = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM | -0.92% | -1.01% | -0.99% | 100% | 100% | -1.29% | -1.53% | -1.45% | 100% | 101% |
| TGM | -0.23% | -0.14% | -0.20% | 100% | 101% | -0.24% | -0.37% | -0.40% | 100% | 100% |
| A | -0.39% | -0.40% | -0.43% | 100% | 99% | -0.47% | -0.46% | -0.51% | 100% | 100% |
| M | -0.24% | 0.16% | 0.10% | 100% | 100% | -0.40% | -0.34% | -0.40% | 100% | 100% |
| CC | 0.00% | 0.01% | 0.01% | 100% | 100% | 0.01% | -0.01% | 0.01% | 99% | 101% |
| Overall | -0.41% | -0.33% | -0.36% | 100% | 100% | -0.54% | -0.63% | -0.64% | 100% | 100% |

# References

- X. Xu, Y.-H. Chao, Y.-C. Sun and J. Xu, “ Description of Core Experiment 2 (CE2): Palette Mode Coding”, JVET-O2022, 16th meeting, Geneva, CH, 1–11 October 2019

- VTM-7.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0.

- B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001-vE, the 16th JVET meeting, Geneva, CH, 1–11 October 2019

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).