| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0073 |
| --- | --- |

| Title: | CE5-2.1, CE5-3.1: Multiplication removal for cross component adaptive loop filter and 5x5 filter shape |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Jie Dong, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In JVET-P1008, a cross component adaptive loop filter (CC-ALF) is tested to refine chroma components by using luma samples. The shape of a filter is 3x4. Each filter has 8 filter coefficients and 8 taps. Every coefficient has a 6-bit dynamic range and is signalled with a 6-bit code. To reduce the complexity of multiplication, in CE5-2.1, multiplications are replaced by bit shifting. On top of CE5-2.1, in CE5-3.1, filter shape is replaced with 5x5 diamond shape, which is used for chroma adaptive loop filter. The tests evaluate the performance of the proposed approaches by using common test condition (CTC).

The average BD rate for the tests versus CE5 anchor is as follows:

| Results over CE5 anchor (SDR) | All Intra |  |  | Random Access |  |  | Low Delay B |  |  | Low Delay P |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE Test | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| CE5-2.1 (bit shifting) | -0.03% | 0.53% | 0.36% | 0.01% | 0.08% | 0.06% | -0.01% | -0.24% | -0.13% | 0.02% | -0.17% | -0.31% |
| CE5-3.1 (bit shifting + 5x5 filter shape) | 0.01% | -0.20% | -0.61% | 0.01% | -0.55% | -0.71% | -0.01% | -1.52% | -1.82% | 0.01% | -1.52% | -1.73% |

In addition, HDR sequences in class H1 with different chroma downsampling format is tested by using HDR CTC. The average BD rate for the tests versus CE5 anchor is as follows:

| Results over CE5 anchor (HDR-B) | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CE Test | Y | U | V | Y | U | V |
| CE5-3.1 (bit shifting + 5x5 filter shape) | 0.01% | -0.24% | -0.51% | 0.08% | -1.81% | -1.87% |

Both CE tests have similar encoding and decoding runtime compared to CE5 anchor.

# Introduction

In JVET-P0080, a cross component adaptive loop filter (CC-ALF) is tested to refine chroma components by using luma sample values. Each filter has 14 filter coefficients and 18 taps. Every coefficient has an 8-bit dynamic range and is signalled with a third order exponential-Golomb code. However, this adds the complexity, especially in decoder, by introducing additional 12 8-bit multiplications, 1 9-bit multiplication and 1 10-bit multiplication per chroma sample.

|  |  | f0 |  |  |
| --- | --- | --- | --- | --- |
|  | f1 | f2 | f3 |  |
| f4 | f5 | f6 | f7 | f4 |
| f4 | f8 | f9 | f10 | f4 |
|  | f11 | f12 | f13 |  |
|  |  | f0 |  |  |

Figure 1. Filter shape in JVET-P0080

In JVET-P1008, a simplified version of CC-ALF is tested, which is also used as CE5 anchor. Each filter has 8 coefficients and 8 taps. Every coefficient has a 6-bit dynamic range and is signalled with a 6-bit code. To filter a chroma sample, the multiplication complexity is reduced to 8 6-bit multiplications.

|  | f0 |  |
| --- | --- | --- |
| f1 | f2 | f3 |
| f4 | f5 | f6 |
|  | f7 |  |

Figure 2. Filter shape in JVET-P1008

To further simplify the computation overhead in JVET-P1008, in CE5-2.1, the multiplications are replaced by bit shifting. Filter coefficients are restricted to have values: {-64, -32, -16, -8, -4, -2, -1, 0, 1, 2, 4, 8, 16, 32, 64}, which are signalled as {-7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 7} respectively before binarization in order to reduce the signalling overhead. Fixed length codes of 3 bits are used to signal the absolute values of the coefficients. Another sign bit is signalled for non-zero coefficients.

On top of CE5-2.1, in CE5-3.1, 5x5 filter shape is used as Figure 3.

|  |  | f0 |  |  |
| --- | --- | --- | --- | --- |
|  | f1 | f2 | f3 |  |
| f4 | f5 | f6 | f7 | f8 |
|  | f9 | f10 | f11 |  |
|  |  | f12 |  |  |

Figure 3. Filter shape in CE5-3.1

# Complexity analysis

Following table describes the difference in complexity when calculating the offset that is added to a chroma sample in CE5-2.1 and CE5-3.1.

|  | multiplication | adder /subtractor | shift | storage of a filter with 8 coefficients |
| --- | --- | --- | --- | --- |
| CE5 anchor | 8*6-bit | 8 | 0 | 8 coeff * 6 bits/coeff = 48 bits |
| CE5-2.1 | 0 | 8 | 8 | 8 coeff * 4 bits/coeff = 32 bits |
| CE5-3.1 | 0 | 13 | 13 | 13 coeff * 4 bits/coeff = 52 bits |

In CE5-2.1 and CE5-3.1, to store a coefficient, 3 bits could be used to store the number of left bit shifting (0-7) and 1 bit could be used to store its sign.

# Experimental results

The proposed methods were implemented on top of CE5 anchor and evaluated under common test condition (CTC) for both SDR [1] and HDR [2] sequences.

## Results of CE5-2.1 (SDR CTC)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0-CE5Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.51% | 0.45% | 100% | 100% |
| Class A2 | 0.00% | 0.21% | 0.81% | 100% | 101% |
| Class B | -0.02% | 0.90% | 0.62% | 100% | 100% |
| Class C | -0.04% | 0.08% | -0.02% | 100% | 99% |
| Class E | -0.05% | 0.85% | -0.11% | 100% | 100% |
| Overall | -0.03% | 0.53% | 0.36% | 100% | 100% |
| Class D | -0.01% | -0.11% | -0.44% | 100% | 101% |
| Class F | -0.04% | 0.31% | 0.35% | 100% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0-CE5Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.13% | 0.23% | 100% | 100% |
| Class A2 | 0.01% | 0.32% | 0.42% | 100% | 101% |
| Class B | 0.02% | 0.18% | -0.08% | 100% | 100% |
| Class C | 0.00% | -0.26% | -0.17% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.08% | 0.06% | 100% | 100% |
| Class D | -0.03% | -0.34% | -0.16% | 100% | 101% |
| Class F | -0.02% | 0.09% | -0.09% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0-CE5Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.14% | 0.09% | 100% | 102% |
| Class C | -0.01% | -0.17% | -0.38% | 100% | 100% |
| Class E | 0.01% | -0.50% | -0.14% | 100% | 101% |
| Overall | -0.01% | -0.24% | -0.13% | 100% | 101% |
| Class D | 0.01% | -0.81% | -0.67% | 100% | 100% |
| Class F | 0.03% | -0.14% | 0.40% | 101% | 101% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-7.0-CE5Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.13% | -0.20% | 100% | 97% |
| Class C | 0.03% | -0.39% | -0.10% | 100% | 100% |
| Class E | 0.03% | -0.38% | -0.76% | 100% | 101% |
| Overall | 0.02% | -0.17% | -0.31% | 100% | 99% |
| Class D | 0.10% | -2.05% | -1.25% | 100% | 101% |
| Class F | -0.03% | -0.26% | 0.01% | 100% | 102% |

## Results of CE5-3.1 (SDR CTC)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0-CE5Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.15% | -0.54% | 100% | 101% |
| Class A2 | 0.01% | -0.77% | 0.01% | 100% | 98% |
| Class B | 0.00% | 0.12% | -0.57% | 100% | 100% |
| Class C | 0.01% | -0.59% | -0.73% | 100% | 103% |
| Class E | 0.03% | -0.02% | -1.18% | 101% | 101% |
| Overall | 0.01% | -0.20% | -0.61% | 100% | 101% |
| Class D | 0.02% | -0.67% | -0.76% | 100% | 100% |
| Class F | 0.00% | -0.51% | -0.41% | 100% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0-CE5Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.10% | -0.24% | 100% | 100% |
| Class A2 | 0.01% | -1.13% | -0.68% | 99% | 100% |
| Class B | 0.01% | -0.55% | -1.14% | 99% | 100% |
| Class C | 0.00% | -0.45% | -0.56% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.55% | -0.71% | 100% | 101% |
| Class D | 0.00% | -0.47% | -0.92% | 100% | 101% |
| Class F | 0.01% | -0.65% | -0.60% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0-CE5Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -1.60% | -1.32% | 99% | 102% |
| Class C | 0.02% | -1.54% | -1.29% | 100% | 101% |
| Class E | -0.07% | -1.34% | -3.35% | 100% | 99% |
| Overall | -0.01% | -1.52% | -1.82% | 100% | 101% |
| Class D | -0.03% | -1.23% | -1.40% | 100% | 101% |
| Class F | 0.04% | -1.72% | -0.53% | 100% | 99% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-7.0-CE5Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -1.71% | -1.74% | 100% | 98% |
| Class C | 0.00% | -1.80% | -1.21% | 99% | 99% |
| Class E | 0.00% | -0.82% | -2.42% | 99% | 99% |
| Overall | 0.01% | -1.52% | -1.73% | 99% | 99% |
| Class D | 0.06% | -2.40% | -2.08% | 100% | 101% |
| Class F | 0.02% | -1.92% | -1.25% | 100% | 101% |

## Results of CE5-3.1 (HDR-B CTC)

|  |  | All Intra BD-rate Over VTM7.0-CE5Anchor |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| HDR-B | Market | 0.41% | 0.00% | 0.00% | 0.18% | -0.07% | 0.00% | 0.30% | -0.31% |
|  | SunRise | -0.32% | 0.00% | 0.00% | -0.36% | -1.95% | 0.00% | -0.37% | -3.29% |
|  | ShowGirl2 | -0.64% | 0.03% | 0.03% | -2.13% | -0.71% | 0.03% | -1.90% | -0.34% |
|  | BalloonFestival | 0.12% | 0.00% | 0.00% | 0.37% | 0.61% | 0.00% | -0.28% | 0.69% |
|  | Hurdles | -0.03% | -0.01% | 0.00% | 0.64% | -0.51% | 0.00% | 0.61% | -0.28% |
|  | Starting | 0.54% | 0.00% | 0.00% | 0.20% | 0.22% | 0.00% | 0.23% | 0.41% |
|  | Cosmos1 | 0.02% | 0.01% | 0.01% | -0.13% | -0.62% | 0.01% | -0.27% | -0.46% |
|  | Average HDR-B | 0.01% | 0.00% | 0.01% | -0.18% | -0.43% | 0.01% | -0.24% | -0.51% |
|  | Enc Time[%] | 100% |  |  |  |  |  |  |  |
|  | Dec Time[%] | 101% |  |  |  |  |  |  |  |

|  |  | Radom Access BD-rate Over VTM7.0-CE5Anchor |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| HDR-B | Market | -0.41% | 0.18% | 0.18% | -5.13% | -5.78% | 0.17% | -4.36% | -3.80% |
|  | SunRise | -0.26% | 0.18% | 0.19% | -1.51% | -3.10% | 0.20% | -1.36% | -3.37% |
|  | ShowGirl2 | -0.99% | 0.01% | 0.00% | -2.75% | -1.22% | -0.02% | -2.88% | -1.32% |
|  | BalloonFestival | -0.62% | 0.02% | 0.10% | -0.88% | -2.34% | 0.10% | -1.62% | -1.97% |
|  | Hurdles | -0.27% | 0.01% | 0.00% | 0.01% | 0.68% | 0.00% | -0.07% | 0.67% |
|  | Starting | -0.52% | 0.19% | 0.19% | -1.24% | -0.90% | 0.18% | -0.71% | -0.97% |
|  | Cosmos1 | -0.11% | 0.02% | -0.03% | -1.13% | -2.77% | -0.05% | -1.63% | -2.31% |
|  | Average HDR-B | -0.45% | 0.09% | 0.09% | -1.80% | -2.20% | 0.08% | -1.81% | -1.87% |
|  | Enc Time[%] | 100% |  |  |  |  |  |  |  |
|  | Dec Time[%] | 101% |  |  |  |  |  |  |  |

# Reference

[1] JVET-N1010, “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring

[2] JVET-P2011, “JVET common test conditions and evaluation procedures for HDR/WCG video”, A. Segall, E. Francois, W. Husak, S. Iwamura, D. Rusanovskyy

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).