| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0078 |
| --- | --- |

| Title: | CE4-6: Combination of geometric partitioning and CIIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Saverio Blasi, Gosala Kulupana 201 Wood Lane, The Lighthouse W12 7TQ, London, UK | Email: | saverio.blasi@bbc.co.uk gosala.kulupana@bbc.co.uk |
| Source: | British Broadcasting Corporation |  |  |

_____________________________

# Abstract

This contribution presents the results of test CE4-6 where the usage of geometrical partitions is proposed in combination with Combined Inter-Intra Prediction (CIIP) as an extension to JVET-P0071. The test is performed on top of the CE4 common base described in JVET-P0884&JVET-P0885. In this test, intra and inter predictions are combined following the partitioning scheme used in the Geometrical partition mode. The intra predictor is used in one of the two Geometrical partitions, and the merge inter-predictor is used in the other Geometrical partition. Four possible Geo CIIP modes are considered with different splits and/or usage of inter or intra prediction in the two partitions.  Average BD-rates of -0.32% (Y), -0.35% (U) and -0.33% (V) are reported for RA with 104% encoding time and 101% decoding time with respect to VTM 7.0. Higher gains are reported for Class C, where an average -0.70% luma bd-rate is obtained. For LD, the reported BD-rates are -0.79% (Y), -0.88% (U) and -0.95% (V) with 105% and 102% encoding and decoding times, respectively.

# Introduction

The current VVC draft specification makes use of Combined Intra-Inter Prediction (CIIP), which can be used in merge-predicted inter-blocks. In CIIP, an intra-predictor is combined with an inter-predictor based on weights that depend on the neighbours of the current block. In JVET-P0071 [1], it was proposed to allow usage of the triangular partitions resulting from usage of the TPM mode, in combination with Combined Inter-Intra Prediction (CIIP). Based on the discussions in the 16th JVET meeting, the technique proposed in JVET-P0884/P0885 was suggested to be the common base for this core experiment, whereby the common base of tests in CE4 was defined as the method described in JVET-P0884&JVET-P0885 [2], with the modifications that the TPM mode is turned off, and the TPM angle are added to the list of possible angles considered in the Geo mode. Following these modifications, test CE4-6 extends the method described in JVET-P0071 whereby instead of using some of the TPM modes in combination with CIIP, some of the Geo modes are used in combination with CIIP instead. As in conventional CIIP, an intra- and inter-predicted predictions are combined to form a prediction for the current block. The combination between the two predictors is performed following the partitioning scheme used in the Geo mode. The intra predictor is used in one of the two Geo partitions, and the merge inter-predictor is used in the other Geo partition.

Four possible Geo CIIP modes are considered with different splits and/or usage of inter or intra prediction in the two partitions. Usage of the specific Geo CIIP mode is explicitly signalled in the bitstream as an alternative to conventional CIIP.

# Proposed method

JVET- P0071 proposes to use the same intra and inter predictors, as well as the same information extracted from the top neighbouring CU (N0) and the left neighbouring CU (N1) used by CIIP for the inference of the weights to use. In particular, the inter-predictor is performed by means of conventional merge inter prediction, whereas the intra-prediction is performed using the Planar mode, as in conventional CIIP. Moreover, information on whether N0 and N1 are intra-coded or not is used. This information, in combination with the height and width of the current PU, is used to extract one look-up table containing four possible Geo modes that can be used to predict the current block in the CIIP mode. This is a simple extension of the mechanism used in JVET-P0071 to extract a list of possible TPM modes to use depending on the N0 and N1 modes.

While adapting JVET-P0071 to work with Geo modes instead of the original TMP modes, it was noticed that the TPM splits work differently than the Geo angles with respect to the shape of the block. In particular, the TPM splits as defined in the original TPM mode take into account the shape of the block: for instance, a split equal to 0 would always correspond to a triangular partition whereby the edge between the two partitions would connect the top-left corner to the bottom-left corner of the block, independently on the shape of the block. This means that a different angle would be used to perform this split for a square block than for a rectangular block. On the contrary, the modes defined in JVET-P0884&JVET-P0885 consider absolute angles and offsets, independently on the block shape. A given mode would make use of an absolute angle, and that angle is used to perform the split into Geo partitions regardless of the block shape. For that reason, in this test, an adaptation of the mechanism used in JVET-P0071 is used whereby the shape of the block is taken into account when extracting the possible modes to use on a given PU depending on its neighbours.  In particular, an index sizeRatio is defined as:

sizeRatio = (width == height) ? 0 : (width > height? 1 : 2);

Then, different Geo modes are considered depending on the neighbours N0 and N1, for each value of sizeRatio. In particular, if both neighbours N0 and N1 are intra-coded, or if neither of them is intra-coded, then:

GeoModes[sizeRatio] = { { 1, 0, 34, 30}, {1, 0, 34, 30}, {1, 0, 38, 42 } }

Else if only N0 is intra-coded then:

GeoModes[sizeRatio] = { { 34, 38, 74, 73}, {30, 70, 34, 72}, {38, 42, 77, 76 } }

Else (if only N1 is intra-coded) then:

GeoModes[sizeRatio] = { { 2, 1, 0, 7}, {2, 1, 18, 0}, {2, 1, 0, 46 } }

An index referred to as ciipGeoMode is then parsed from the bitstream to detect which of the four Geo modes to use in the current CIIP block. When combining the two predictors, the same weighting scheme employed within the Geo mode, including the smoothing filter used to smooth out the separation between the two partitions. As in JVET-P0071, no additional modes are added to the RD search.

# Performance results

The performance of test CE4-1.9 is summarised in Table 1 and Table 2. The tests were performed using the JVET common test conditions [3] with parallel encoding/decoding used for RA. The software modifications were made on top of VTM 6.0 [4], compiled with gcc 4.8.2. All experiments were run using AVX.

Table 1 - Performance of test CE4-6 for RA.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.12% | -0.26% | -0.11% | 104% | 100% |
| Class A2 | -0.23% | -0.10% | -0.02% | 104% | 101% |
| Class B | -0.20% | -0.11% | -0.14% | 104% | 102% |
| Class C | -0.70% | -0.92% | -0.96% | 104% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.32% | -0.35% | -0.33% | 104% | 101% |
| Class D | -0.43% | -0.70% | -0.61% | 104% | 102% |
| Class F | -0.33% | -0.34% | -0.29% | 102% | 100% |

Table 2 - Performance of test CE4-6 for LD.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.47% | -0.60% | -0.57% | 105% | 101% |
| Class C | -0.93% | -1.47% | -1.33% | 106% | 104% |
| Class E | -1.13% | -0.56% | -1.08% | 104% | 102% |
| Overall | -0.79% | -0.88% | -0.95% | 105% | 102% |
| Class D | -0.77% | -1.05% | -0.81% | 106% | 98% |
| Class F | -0.63% | -0.80% | -0.44% | 103% | 98% |

Additional results against the CE4 common base are Additional results against the CE4 common base are provided below:

Table 3 - Performance of test CE4-6 for RA vs common base.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.02% | -0.17% | 0.07% | 103% | 101% |
| Class A2 | -0.01% | 0.09% | 0.00% | 101% | 100% |
| Class B | -0.05% | 0.00% | 0.04% | 101% | 101% |
| Class C | -0.12% | 0.07% | -0.06% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | 0.00% | 0.01% | 102% | 101% |
| Class D | 0.00% | -0.04% | 0.01% | 101% | 99% |
| Class F | -0.02% | 0.04% | 0.04% | 101% | 101% |

Table 4 - Performance of test CE4-6 for LD vs common base.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.12% | -0.10% | 0.02% | 101% | 101% |
| Class C | -0.13% | -0.22% | -0.45% | 102% | 106% |
| Class E | 0.04% | -0.37% | -0.20% | 103% | 104% |
| Overall | -0.08% | -0.21% | -0.19% | 102% | 103% |
| Class D | -0.08% | -0.60% | 0.23% | 102% | 104% |
| Class F | -0.12% | 0.11% | -0.14% | 101% | 99% |

# References

- S. Blasi, A. Seixas Dias, G. Kulupana, “CE4-1.9: CIIP with triangular partitions”, JVET-P0071, Geneva, October 2019.

- C. Chen, R. Liao, X. Xiu, and H. Yang, “Description of Core Experiment 4 (CE4): Inter prediction with geometric partitioning”, JVET-P2024, Geneva, October 2019

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, March 2019.

- VTM-7.0 software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.git, November 2019

# Patent rights declaration(s)

The British Broadcasting Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).