| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0079-v1 |
| --- | --- |

| Title: | CE4 Common Base: Geometric inter prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Gao1, Semih Esenlik, Elena Alshina, Anand Meher Kotra, Biao Wang (Huawei) Kevin Reuzé2, Chun-Chi Chen, Han Huang, Wei-Jung Chien, Vadim Seregin, Marta Karczewicz (Qualcomm) Ru-Ling Liao3, Jie Chen, Yan Ye, Jiancong Luo (Alibaba) Max Bläser4, Johannes Sauer (RWTH Aachen) | Tel: Email: | 1han.gao@huawei.com 2kreuz@qti.qualcomm.com 3ruling.lrl@alibaba-inc.com 4blaeser@ient.rwth-aachen.de |
| Source: | Huawei Technologies Co., LTD, Qualcomm Inc, Alibaba Group, RWTH Aachen University |  |  |

_____________________________

- Abstract

The contribution reports the simulation results of CE4 common base for GEO mode. It is reported that, compared to VTM-7.0 with TPM off, the overall performance impact for {Y, U, V} is {-0.66%, -0.98%, -1.01%} for RA and {-1.61%, -1.93%, -1.98%} for LB. Compared to VTM-7.0, the overall performance impact for {Y, U, V} is {-0.28%, -0.35%, -0.34%} for RA and {-0.70%, -0.67%, -0.77%} for LB.

Introduction

In the JVET-P meeting, a simplified GEO mode is proposed in JVET-P0884/JVET-P0885, and is suggested to be the common base for CE4 core experiment. In this common base, the GEO mode is applied to a merge block whose width and height are both larger than or equal to 8. When a block coded using GEO mode, an index is signaled to indicate which one of 82 partitioning modes is used to split the block into two partitions. Each partition is inter predicted with its own motion vector. After predicting each of the partition, the sample values along the partitioning edge are adjusted using a blending processing with weights. This is the prediction signal for the whole block, and transform and quantization process will be applied to the whole block as in other prediction modes.

The weights for luma samples used in the blending process are calculated as follows:

weightIdx = (((x + offsetX)<<1) + 1)*Dis[displacementX] + (((y+ offsetY )<<1) + 1))*Dis[displacementY] − rho.																	(1)

weightIdxAbs = Clip3(0, 26, abs(weightIdx)).							(2)

sampleWeight = weightIdx <= 0 ? GeoFilter[weightIdxAbs] : 8 − GeoFilter[weightIdxAbs]	(3)

Dis[.] is a look up table with 24 entries and the possible out put values are {0, 1, 2, 4}. GeoFilter[.] is a look up table with 27 entries. The variables rho, offsetX and offsetY are pre-computed based on:

rho = (Dis[displacementX]<< 8) + (Dis[displacementY] << 8)

offsetX=(256-W)≫1φ % 12 = 6 or (φ % 12 ≠ 0 and H ≥ W)256-W≫1+φ<12 ?ρ×W≫3 :-(ρ×W≫3)otherwise

offsetY=256-H≫1+φ<12 ?ρ×H≫3 :-(ρ×H≫3)φ % 12 = 6 or (φ % 12 ≠ 0 and H ≥ W)256-H≫1otherwise

The φ and ρ represent angle and distance which are derived from a look up table using the signaled index.

The weights for chroma samples are subsampled from the luma weights. The motion storage masks are derived using same method of weights derivation independently. More details description for this common base can be found in JVET-P0884/JVET-P0885.

Simulation results

The common base is implemented on top of VTM-7.0. The simulation results under CTC are shown in Table 1 and Table 2.

Table 1. Results of GEO common base relative to VTM-7.0 with TPM off

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 with TPM off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.35% | -0.49% | -0.61% | 104% | 100% |
| Class A2 | -0.52% | -0.80% | -0.60% | 104% | 100% |
| Class B | -0.41% | -0.53% | -0.65% | 104% | 99% |
| Class C | -1.29% | -2.06% | -2.07% | 104% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.66% | -0.98% | -1.01% | 104% | 99% |
| Class D | -0.81% | -1.37% | -1.14% | 105% | 99% |
| Class F | -0.56% | -0.79% | -0.72% | 101% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 with TPM off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.97% | -1.24% | -1.50% | 107% | 99% |
| Class C | -1.97% | -2.82% | -2.56% | 107% | 97% |
| Class E | -2.19% | -1.90% | -2.02% | 104% | 98% |
| Overall | -1.61% | -1.93% | -1.98% | 106% | 98% |
| Class D | -1.59% | -2.20% | -2.33% | 107% | 98% |
| Class F | -0.94% | -0.98% | -1.46% | 104% | 100% |

Table 2. Results of GEO common base relative to VTM-7.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.09% | -0.18% | 103% | 101% |
| Class A2 | -0.22% | -0.19% | -0.02% | 103% | 100% |
| Class B | -0.14% | -0.10% | -0.17% | 103% | 101% |
| Class C | -0.59% | -0.99% | -0.90% | 103% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.28% | -0.35% | -0.34% | 103% | 100% |
| Class D | -0.43% | -0.67% | -0.62% | 103% | 100% |
| Class F | -0.32% | -0.38% | -0.33% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.36% | -0.51% | -0.60% | 104% | 99% |
| Class C | -0.79% | -1.25% | -0.89% | 104% | 100% |
| Class E | -1.17% | -0.19% | -0.88% | 101% | 101% |
| Overall | -0.70% | -0.67% | -0.77% | 103% | 100% |
| Class D | -0.68% | -0.44% | -1.04% | 103% | 99% |
| Class F | -0.51% | -0.90% | -0.30% | 102% | 100% |

Conclusion

The geometric inter prediction mode provides 0.28% and 0.70% BD rate reduction for random access and low delay B conditions, respectively. The encoding time increases by 3% and no impact on decoding time. Considering the coding performance, we recommend to adopt this contribution in VVC.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

RWTH Aachen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, would be prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form), or make them available free of charge (per box 1 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form). We are however aware that also other entities may have current or pending patent rights relating to the technology described in this contribution.