| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0082 |
| --- | --- |

| Title: | CE3-2.7: Luma and chroma BDPCM and IBC with TSRC for lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungah Choi, Hyeongmun Jang, Sunmi Yoo, Jin Heo, Jaehyun Lim, Seung Hwan Kim 19 Yangjaedaero-11gil, Seocho-gu, Seoul, 06772, South Korea Alican Nalci, Hongtao Wang, Muhammed Coban, Marta Karczewicz 5775 Morehouse Dr., San Diego, CA, 92121, USA Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +82-2-6912-6561 jungah.choi@lge.com analci@qti.qualcomm.com {zhi-yi.lin, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | LG Electronics Inc., Qualcomm Inc., MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution reports the results of CE3-2.7. In CE3-2.7, it is proposed to enable luma and chroma BDPCM for lossless coding for both camera-captured and screen content sequences. In addition, it is suggested that luma BDPCM, chroma BDPCM, and IBC modes are coded by transform skip residual coding, and others are coded by regular residual coding. The following coding performances are reported:

- Overall: -6.75% (AI), -5.95% (RA), -5.42% (LD)

- Class F: -7.15% (AI), -5.00% (RA), -4.16% (LD)

- TGM: -6.03% (AI), -2.79% (RA), -1.68% (LD)

Proposed method

In VTM-7.0, transform skip residual coding (TSRC) is used for lossless coding and luma BDPCM is not used for all test sequences. However, it is necessary to study the performance of TSRC and RRC in lossless coding. VVC has another residual coding method: regular residual coding (RRC). It is known that RRC is efficient for camera-captured sequences and TSRC is efficient for screen content sequences.

In CE3-2.7, it is proposed to use both residual coding methods: TSRC and RRC, and the residual coding method is determined by the prediction mode. With luma and chroma BDPCM enabled in all sequences, if the prediction mode is luma/chroma BDPCM or IBC, TSRC is selected. For other modes, RRC is selected.

The following proposed text modifications are based on JVET-P2001-vE. The added texts are highlighted in yellow. Highlighted in green is a flag to indicate RRC is the default residual coding method for lossless coding. In CE 3, various ways to indicate the default residual coding method are investigated. The flag could be signaled in SPS/PPS/CU/TU level, and either way is acceptable.

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_joint_cbcr_enabled_flag && ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && ( tu_cbf_cb[ xC ][ yC ] ¦ ¦ tu_cbf_cr[ xC ][ yC ] ) ) ¦ ¦ ( tu_cbf_cb[ xC ][ yC ] && tu_cbf_cr[ xC ][ yC ] ) ) && chromaAvailable ) |  |
| tu_joint_cbcr_residual_flag[ xC ][ yC ] | ae(v) |
| if( tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 0 ] && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && !cu_sbt_flag ) |  |
| transform_skip_flag[ x0 ][ y0 ][ 0 ] | ae(v) |
| if( (!transform_skip_flag[ x0 ][ y0 ][ 0 ] && !sps_use_regular_coding_flag) ¦¦ (!BdpcmFlag[x0][y0][0] && !pred_mode_ibc_flag[x0][y0] &&sps_use_regular_coding_flag)) |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| else |  |
| residual_ts_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| } |  |
| if( tu_cbf_cb[ xC ][ yC ] && treeType != DUAL_TREE_LUMA ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 1 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag ) |  |
| transform_skip_flag[ xC ][ yC ][ 1 ] | ae(v) |
| if( !transform_skip_flag[ xC ][ yC ][ 1 ] && !sps_use_regular_coding_flag) ¦¦ (!BdpcmFlag[x0][y0][1] && sps_use_regular_coding_flag)) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| else |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| } |  |
| if( tu_cbf_cr[ xC ][ yC ] && treeType != DUAL_TREE_LUMA && !( tu_cbf_cb[ xC ][ yC ] && tu_joint_cbcr_residual_flag[ xC ][ yC ] ) ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 2 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag ) |  |
| transform_skip_flag[ xC ][ yC ][ 2 ] | ae(v) |
| if( !transform_skip_flag[ xC ][ yC ][ 1 ] && !sps_use_regular_coding_flag) ¦¦ (!BdpcmFlag[x0][y0][2] && sps_use_regular_coding_flag)) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| else |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |
| } |  |

Experimental results

Table 1 summarizes the performance of CE3-2.7. The anchor is VTM-7.0 with the lossless configuration (lossless.cfg). For the anchor, BDPCM is disabled for both camera-captured and screen content sequences. For the proposed method, we additionally used following settings: -- ForceRegularCoding=1 –BDPCM=2 –qp 0. Detailed test condition is described in the CE3 description [1].

Table 1: Coding performance of CE3-2.7 versus VTM-7.0

References

T. Nguyen, T.C. Ma, A. Nalci, “Description of Core Experiment 3 (CE 3): Lossless Coding,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-P2023, Geneva, CH, 1–11 Oct. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).