| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0084-v1 |
| --- | --- |

| Title: | CE1-2: Deblocking for TPM (Triangular Partition Mode) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xuewei Meng1, Shanshe Wang1, Siwei Ma1, Shunsuke Iwamura2, Shimpei Nemoto2, Atsuro Ichigaya2, Xiaozhen Zheng3 Peking University1, NHK2, SZ DJI Technology Co., Ltd.3 | Tel: Email: | xwmeng@pku.edu.cn iwamura.s-gc@nhk.or.jp xiaozhen.zheng@dji.com |
| Source: | Peking University, NHK , SZ DJI Technology Co., Ltd. |  |  |

_____________________________

- Abstract

In this contribution, CE1-2 test to apply deblocking filter on TPM CU boundaries is reported. This contribution addresses the potential blocking artifacts of current deblocking design for TPM. The potential discontinuity of block boundary is mainly caused by the discrepancy of the weights in TPM mode. To resolve this problem, this contribution proposes to conduct deblocking filtering on these overlooked boundaries between TPM blending area and another CU in current VVC. BD-rate are summarized as follows,

Anchor: VTM7.0

Test: Anchor + deblocking for TPM

RA:   0.00% (Y), 0.00% (Cb), 0.02% (Cr)
LDB:  -0.01% (Y), 0.04% (Cb), -0.33% (Cr)

Introduction

The triangle partition merge (TPM) mode is a newly adopted tool in VVC. TPM conducts motion compensation based on triangular PU and then conducts blending process on the diagonal area, as shown in Figure1.

Figure 1.  Blending process of TPM

The following example (as shown in Figure) will be used to elaborate the potential discontinuity of block boundary caused by the discrepancy of the weights in TPM.

As shown in Figure2, the motion vectors of a CU coded in triangle partition mode are stored in 4x4 units. Depending on the position of each 4x4 unit, either uni-prediction or bi-prediction motion vectors are stored. Denote Mv1 and Mv2 as uni-prediction motion vectors for partition 1 and partition 2, respectively. If a 4x4 unit is located in the non-weighted area shown in the example of Figure2, either Mv1 or Mv2 is stored for that 4x4 unit. Otherwise, if the 4x4 unit is located in the weighted area, a bi-prediction motion vector is stored. The bi-prediction motion vector is derived from Mv1 and Mv2 according to the following process:

- If Mv1 and Mv2 are from different reference picture lists (one from L0 and the other from L1), then Mv1 and Mv2 are simply combined to form the bi-prediction motion vector.

- Otherwise, if Mv1 and Mv2 are from the same list, and without loss of generality, assume they are both from L0. In this case, instead of bi-prediction motion, only uni-prediction motion Mv2 is stored.

Because of the particularMV storage method and spatially varying weighted average process for diagonal area, some potential blocking artifacts may occur.

Figure 2.  Motion vector storage of TPM

As shown in Figure 3(a), block P is coded by TPM (L0: MVp1, L1: MVp2), and block Q is coded by full-block bi-prediction (L0: MVq1, L1: MVq2). When MVp1 and MVp2 are identical or mostly equal to MVq1 and MVq2, respectively, deblocking is not triggered if both blocks are skipped. However, the prediction of block P is generated by spatially varying weighted average and that of the block Q is generated by equal-weighted average. This misalignment may cause discontinuity of the block boundary which is overlooked in the current VTM design.

Similar as Figure3(a), Figure3(b) shows the example where block P is coded by TPM (L0: MVp) and block Q is coded by full-block bi-prediction (L0: MVq). When MVp is identical or mostly equal to MVq, deblocking is not triggered if both blocks are skipped. However, the prediction of block P is generated by spatially varying weighted average and that of the block Q is uni-predicted. This may cause discontinuity.

- (b)

Figure 3.  Deblocking filter example in TPM

Proposed Method

To resolve these problems mentioned above, the proposed method is as follows,

Deblocking for TPM

The spec changes are shown bellow to elaborate the proposed method.

      - Two motion vectors for the same reference picture are used to predict the coding subblock containing the sample p0, two motion vectors for the same reference picture are used to predict the coding subblock containing the sample q0 and both of the following conditions are true:

        - The absolute difference between the horizontal or vertical component of list 0 motion vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in 1/16 luma samples, or the absolute difference between the horizontal or vertical component of the list 1 motion vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in units of 1/16 luma samples.

        - The absolute difference between the horizontal or vertical component of list 0 motion vector used in the prediction of the coding subblock containing the sample p0 and the list 1 motion vector used in the prediction of the coding subblock containing the sample q0 is greater than or equal to 8 in units of 1/16 luma samples, or the absolute difference between the horizontal or vertical component of the list 1 motion vector used in the prediction of the coding subblock containing the sample p0 and list 0 motion vector used in the prediction of the coding subblock containing the sample q0 is greater than or equal to 8 in units of 1/16 luma samples.

        - The coding subblock containing the sample p0 is triangle mode, and the coding subblock containing the sample p0 is different from that of the sample q0, and sample is in the blending area of corresponding coding subblock.

        - The coding subblock containing the sample q0 is triangle mode, and the coding subblock containing the sample p0 is different from that of the sample q0, and sample is in the blending area of corresponding coding subblock.

    - Otherwise, the variable bS[ xDi ][ yDj ] is set equal to 0.

Simulation results

The experiments were performed based on VTM-7.0 software [2] with VTM configuration files under JVET common test condition [3]. And, the experiments were conducted under Ubuntu Linux. The encoding time is unreliable.

The experimental results of the proposed method compared to VTM-7.0 anchor are summarized in Table 1. The average BD-rates of 0.00%, and -0.01% are observed for RA, and LDB cases, respectively.

Table 1. Experimental result of the proposed method

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.03% | 100% | 100% |
| Class A2 | -0.01% | 0.01% | 0.00% | 105% | 100% |
| Class B | 0.00% | 0.01% | -0.01% | 99% | 100% |
| Class C | 0.00% | 0.00% | 0.07% | 98% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.02% | 100% | 100% |
| Class D | 0.02% | -0.03% | -0.04% | 100% | 100% |
| Class F | 0.00% | 0.02% | -0.04% | 98% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.15% | -0.18% | 100% | 100% |
| Class C | -0.01% | -0.15% | -0.21% | 98% | 100% |
| Class E | -0.03% | 0.62% | -0.75% | 101% | 100% |
| Overall | -0.01% | 0.04% | -0.33% | 100% | 100% |
| Class D | -0.06% | -0.01% | -0.37% | 101% | 101% |
| Class F | 0.01% | -0.45% | 0.27% | 100% | 100% |

References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-P2001, Oct. 2019

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Mar. 2019

- X.W. Meng (PKU), X. Zheng (DJI), S.S. Wang, S.W. Ma (PKU), “AHG18 / non-CE5: Deblocking for TPM and BCW”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-P0269, Oct. 2019

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).