| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0085-v1 |
| --- | --- |

| Title: | CE5-1.1: CCALF with 5x4 filter shape and 8 filter coefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra Semih Esenlik Biao Wang Han Gao Elena Alshina | Tel: Email: | anand.meher.kotra@huawei.com |
| Source: | Huawei Technologies Dusseldorf GmbH |  |  |

_____________________________

# Abstract

This contribution reports the results of CE5-1.1 wherein a filter shape of 5x4 with 8 coefficients is proposed for the cross component adaptive loop filter (CCALF) tool.

The objective results, over CE5 Anchor for CTC configuration are as follows.

YUV*: AHG13 weighting 8:1:1

| Config. | Y | U | V | YUV* | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| AI | 0.00% | 0.13% | 0.01% | 0.01% | 100% | 98% |
| RA | 0.01% | -0.43% | -0.35% | -0.07% | 100% | 100% |
| LDB | -0.01% | -0.50% | -0.81% | -0.16% | 101% | 101% |
| LDP | -0.01% | -0.70% | -0.93% | -0.17% | 101% | 101% |

# Introduction

The best filter shape for Cross component adaptive loop filter (CCALF) in terms of trade-off between subjective/objective quality and computational complexity is current being investigated in CE5[1].

# Proposed Filter shape

CE 5-1.1 uses a 5 x 4 filter shape with 8 filter coefficients as shown in the figure below

# Complexity evaluation

When compared to the filter shape proposed in the CE5 Anchor (3 x 4 diamond), the proposed filter shape uses 4 additional filter taps. Therefore the complexity increase is reportedly minor (which is only 4 more additions).

# Simulation Results

The test is implemented on top of CE5 Anchor [4] and the simulation is performed under common test conditions as described in JVET-N1010 [2].

## Results with CTC

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE5 Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.12% | -0.61% | 100% | 99% |
| Class A2 | 0.00% | -0.46% | -0.36% | 100% | 99% |
| Class B | 0.00% | 0.22% | 0.29% | 99% | 100% |
| Class C | -0.01% | 0.24% | 0.23% | 99% | 95% |
| Class E | -0.01% | 0.67% | 0.25% | 100% | 96% |
| Overall | 0.00% | 0.13% | 0.01% | 100% | 98% |
| Class D | 0.00% | 0.35% | -0.06% | 99% | 95% |
| Class F | 0.00% | 0.22% | 0.09% | 99% | 95% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over CE5 Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.78% | -0.87% | 100% | 100% |
| Class A2 | -0.01% | -0.43% | -0.06% | 100% | 99% |
| Class B | 0.01% | -0.45% | -0.42% | 100% | 101% |
| Class C | -0.03% | -0.14% | -0.09% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.43% | -0.35% | 100% | 100% |
| Class D | -0.05% | 0.27% | -0.23% | 100% | 99% |
| Class F | 0.00% | 0.07% | -0.13% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over CE5 Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.73% | -0.59% | 100% | 100% |
| Class C | -0.06% | -0.79% | -0.70% | 101% | 103% |
| Class E | -0.05% | 0.28% | -1.34% | 101% | 103% |
| Overall | -0.03% | -0.50% | -0.81% | 101% | 101% |
| Class D | -0.04% | -0.65% | -1.39% | 100% | 100% |
| Class F | 0.10% | -0.65% | -0.36% | 101% | 101% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over CE5 Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.62% | -0.79% | 100% | 100% |
| Class C | -0.03% | -1.40% | -0.86% | 101% | 103% |
| Class E | -0.01% | 0.09% | -1.26% | 100% | 102% |
| Overall | -0.01% | -0.70% | -0.93% | 101% | 101% |
| Class D | 0.04% | -1.35% | -2.31% | 100% | 101% |
| Class F | -0.03% | -1.39% | -0.66% | 101% | 102% |

# Conclusion

CE5-1.1 proposes a 5 x 4 filter shape with 8 filter coefficients. When compared to the CE5 Anchor [4], YUV BD-Rate gain of 0.1%, 0.2%, 0.2% are observed for RA, LDB and LDP configurations respectively.

The specification text changes are attached along with the contribution.

Based on the subjective evaluation results, it is recommended to adopt the proposed filter shape for CCALF.

# References

- Versatile Video Coding (Draft 7), Document JVET-P2001-vE, B. Bross, J. Chen, S. Liu,Y.K. Wang

- “JVET common test conditions and software reference configurations”, JVET-N1010

- Description of Core Experiment 5 (CE5): Cross Component Adaptive Loop Filtering, JVET-P2025

- CE5 Common Base: Cross Component Adaptive Loop Filter, JVET-Q0058

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).