| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0086-v1 |
| --- | --- |

| Title: | CE3-2.1 and CE3-2.2: Luma/Chroma BDPCM for Lossless Coding with Transform Skip Residual Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alican Nalci, Hongtao Wang, Muhammed Coban, Marta Karczewicz 5775 Morehouse Dr., San Diego, CA, 92121, USA Jungah Choi, Hyeongmun Jang, Jin Heo, Sunmi Yoo, Jaehyun Lim, Seung Hwan Kim 19 Yangjaedaero-11gil, Seocho-gu, Seoul, 06772, South Korea Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +1 (858) 845-6730 analci@qti.qualcomm.com jungah.choi@lge.com {zhi-yi.lin, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | Qualcomm Inc., LG Electronics Inc., MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of CE3-2.1 and CE3-2.2. In CE 3-2.1, luma BDPCM is enabled in config file for lossless coding, and in CE3-2.2 both luma and chroma BDPCM are enabled for lossless coding. In both tests transform skip residual coding is used as the coefficient coding method (as in current VVC design). There are no changes to the code or spec text.

For CE3-2.1, the simulation results show overall bit-rate savings of -1.85% AI, -0.47% RA, and -0.73% LDB with Class F: -3.87% AI, -1.59% RA, and -1.31% LDB and Class TGM: -2.21% AI, -0.96 % RA, and -0.81% LDB.

For CE3-2.2, the simulation results show overall bit-rate savings of -2.73% AI, -0.76% RA, and -1.01% LDB with Class F: -5.42% AI, -2.24% RA, and -1.85% LDB and Class TGM: -4.99% AI, -2.03% RA, and -1.63% LDB.

# Introduction

In VTM-7.0, lossless coding is possible via already existing syntax with TS enabled both for luma and chroma [1]. In the current CE3 anchor for lossless coding, BDPCM is disabled both for natural and screen content sequences. In CE3-2.1 it is proposed to enable luma BDPCM for lossless coding with transform skip residual coding (TSRC) and in CE3-2.2 it is proposed to enable both luma and chroma BDPCM for lossless coding with TSRC [2].

# Results

In the results presented below, VTM-7.0 is used as the anchor with “lossless.cfg” config file on top of the CTC configurations. BDPCM is disabled for both camera captured and screen content sequences in the anchor. In the test BDPCM config is set to 1 for all test sequences including natural and screen content for CE3-2.1. In CE3-2.2 BDPCM is set in the config file as 2. As outlined in CE3 description, the experiments were run for 100 frames only for AI, RA and LDB configurations.

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | bit-rate savings |
|  | Anchor | TS LumaBD |  | Anchor | TS LumaBD |  | Anchor | TS LumaBD |  |
| Class A1 | 2.2 | 2.3 | -0.54% | 2.2 | 2.3 | -0.39% |  |  |  |
| Class A2 | 1.6 | 1.6 | -1.45% | 1.7 | 1.7 | -0.36% |  |  |  |
| Class B | 2.2 | 2.2 | -1.88% | 2.3 | 2.4 | -0.54% | 2.3 | 2.4 | -0.59% |
| Class C | 1.9 | 2.0 | -1.89% | 2.4 | 2.4 | -0.51% | 2.4 | 2.4 | -0.54% |
| Class D | 1.9 | 2.0 | -3.29% | 2.8 | 2.8 | -0.58% | 2.7 | 2.8 | -0.45% |
| Class E | 2.8 | 2.9 | -3.44% |  |  |  | 3.1 | 3.2 | -1.24% |
| Class F | 5.3 | 5.6 | -3.87% | 33.7 | 34.6 | -1.59% | 50.7 | 51.6 | -1.31% |
| TGM | 11.8 | 12.1 | -2.21% | 107.1 | 108.1 | -0.96% | 124.9 | 126.2 | -0.81% |
| Overall | 2.1 | 2.2 | -1.85% | 2.2 | 2.2 | -0.47% | 2.6 | 2.6 | -0.73% |
| Enc Time[%] | 110% |  |  | 104% |  |  | 105% |  |  |
| Dec Time[%] | 94% |  |  | 98% |  |  | 100% |  |  |

Table 1. CE3-2.1 results for enabling Luma BDPCM in lossless coding.

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | bit-rate savings |
|  | Anchor | TS ChromaBD |  | Anchor | TS ChromaBD |  | Anchor | TS ChromaBD |  |
| Class A1 | 2.2 | 2.3 | -1.32% | 2.2 | 2.3 | -0.96% |  |  |  |
| Class A2 | 1.6 | 1.6 | -1.97% | 1.7 | 1.7 | -0.48% |  |  |  |
| Class B | 2.2 | 2.2 | -2.68% | 2.3 | 2.4 | -0.82% | 2.3 | 2.4 | -0.83% |
| Class C | 1.9 | 2.0 | -2.66% | 2.4 | 2.4 | -0.72% | 2.4 | 2.4 | -0.72% |
| Class D | 1.9 | 2.0 | -4.71% | 2.8 | 2.8 | -0.87% | 2.7 | 2.8 | -0.65% |
| Class E | 2.8 | 2.9 | -5.06% |  |  |  | 3.1 | 3.2 | -1.72% |
| Class F | 5.3 | 5.7 | -5.42% | 33.7 | 34.9 | -2.24% | 50.7 | 51.9 | -1.85% |
| TGM | 11.8 | 12.5 | -4.99% | 107.1 | 109.4 | -2.03% | 124.9 | 127.2 | -1.63% |
| Overall | 2.1 | 2.2 | -2.73% | 2.2 | 2.2 | -0.76% | 2.6 | 2.6 | -1.01% |
| Enc Time[%] | 115% |  |  | 106% |  |  | 105% |  |  |
| Dec Time[%] | 88% |  |  | 98% |  |  | 97% |  |  |

Table 2. CE3-2.2 results for enabling Luma and Chroma BDPCM in lossless coding.

# Recommendation

It is recommended to enable BDPCM for lossless coding.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 7)”, JVET-P2001, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 16th meeting, Geneva, CH, 1-11 Oct. 2019.

[2]  A. Nalci, H. Wang, H. E. Egilmez, M. Coban, M. Karczewicz “AHG18: BDPCM for Lossless,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-P0082, 16th Meeting: Geneva, CH, 1–11 October 2019.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).