| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0088-v1 |
| --- | --- |

| Title: | CE3-2.3: Luma BDPCM for Lossless Coding using Regular Residual Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alican Nalci, Hongtao Wang, Muhammed Coban, Marta Karczewicz 5775 Morehouse Dr., San Diego, CA, 92121, USA Takeshi Tsukuba, Masaru Ikeda, Yoichi Yagasaki, Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan Mohammed Golam Sarwer, Ruling Liao, Yan Ye, Jiancong Luo 525 Almanor Ave, Sunnyvale, CA Tsung-Chuan Ma, Hong-Jheng Jhu, Xiaoyu Xiu, Yi-Wen Chen, Xianglin Wang 8910 University Center Dr., Suite 400, San Diego, CA 92122, USA Hyeongmun Jang, Jungah Choi, Seung-hwan Kim, Jaehyun Lim 19, Yangjae-daero 11-gil, Seocho-gu, Seoul, 06772, Korea | Tel: Email: | +1 (858) 845-6730 analci@qti.qualcomm.com takeshi.tsukuba@sony.com {zhi-yi.lin, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com m.sarwer@alibaba-inc.com tsung-chuanma@kwai.com hm.jang@lge.com |
| Source: | Qualcomm Inc., Sony, MediaTek Inc., Alibaba Group, Kwai Inc. LG Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of CE3-2.3 in which luma BDPCM is enabled for lossless coding. In aspect #1, a high-level flag (e.g. SPS/PPS) is used to switch the residual coding method between regular residual coding (RRC) and transform skip residual coding (TSRC). If the flag value is 1 then RRC is always used with luma BDPCM enabled. In the second aspect, a slice-level flag with an encoder change is based from CE3-1.3. This flag switches between TSRC and RRC based on the encoder decision on IBC usage.

For aspect #1, the simulation results show overall bit-rate savings of -6.76% AI, -5.96% RA, and -5.54% LDB with Class F: -5.74% AI, -4.37% RA, and -3.70% LDB and Class TGM: -1.59% AI, -0.71 % RA, and 0.16% LDB.

For aspect #2, the simulation results show overall bit-rate savings of -6.76% AI, -5.96% RA, and -5.54% LDB with Class F: -5.23% AI, -3.99% RA, and -3.73% LDB and Class TGM: -2.21% AI, -0.96 % RA, and -0.81% LDB.

# Introduction

In VTM-7.0, lossless coding is possible via already existing syntax with TS enabled both for luma and chroma [1]. This form of lossless coding can only use transform skip residual coding (TSRC) and is unable to switch between TSRC and regular residual coding (RRC).

In the 16th JVET Geneva meeting, it was observed that RRC had more coding-gains as compared to doing TSRC [2]. CE3 was established to investigate bit-rate savings with both residual coding methods for lossless coding. In this document (aspect #1) it is proposed to enable luma BDPCM for lossless coding when RRC is used instead of TSRC. This switching to RRC is controlled by the high-level flag (e.g. an SPS/PPS or a slice level flag). In aspect #2, a slice-level syntax with an encoder change is used from CE3-1.3. The decision to encode a frame with RRC and TSRC is done for each frame separately where the encoder uses IBC usage for each frame.

# Results

In the results presented below, VTM-7.0 is used as the anchor with “lossless.cfg” config file on top of the CTC configurations. BDPCM is disabled for both camera captured and screen content sequences in the anchor. In the test BDPCM config is set to 1 for all test sequences including natural and screen content. Moreover, additional config option “ForceRegularCoding=1” is used in the tests. As outlined in CE3 description, the experiments were run for 100 frames only for AI, RA and LDB configurations.

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | bit-rate savings |
|  | Anchor | RRC Luma BDPCM (#1) |  | Anchor | RRC Luma BDPCM (#1) |  | Anchor | RRC Luma BDPCM (#1) |  |
| Class A1 | 2.2 | 2.4 | -5.81% | 2.2 | 2.4 | -6.51% |  |  |  |
| Class A2 | 1.6 | 1.8 | -9.99% | 1.7 | 1.9 | -8.40% |  |  |  |
| Class B | 2.2 | 2.3 | -5.58% | 2.3 | 2.5 | -4.30% | 2.3 | 2.4 | -4.33% |
| Class C | 1.9 | 2.1 | -6.41% | 2.4 | 2.6 | -5.79% | 2.4 | 2.6 | -5.74% |
| Class D | 1.9 | 2.1 | -7.50% | 2.8 | 2.9 | -5.77% | 2.7 | 2.9 | -5.63% |
| Class E | 2.8 | 3.0 | -6.93% |  |  |  | 3.1 | 3.4 | -7.29% |
| Class F | 5.3 | 5.7 | -5.74% | 33.7 | 34.9 | -4.37% | 50.7 | 51.2 | -3.70% |
| TGM | 11.8 | 12.0 | -1.59% | 107.1 | 107.6 | -0.71% | 124.9 | 124.0 | 0.16% |
| Overall | 2.1 | 2.3 | -6.76% | 2.2 | 2.4 | -5.96% | 2.6 | 2.7 | -5.54% |
| Enc Time[%] | 103% |  |  | 103% |  |  | 101% |  |  |
| Dec Time[%] | 97% |  |  | 99% |  |  | 108% |  |  |

Table 1. CE3-2.3 results for enabling Luma BDPCM in lossless coding. A high-level flag (e.g. SPS/PPS) is used to always do Regular Residual Coding (RRC) instead of TSRC.

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | bit-rate savings |
|  | Anchor | RRC Luma BDPCM (#2) |  | Anchor | RRC Luma BDPCM (#2) |  | Anchor | RRC Luma BDPCM (#2) |  |
| Class A1 | 2.2 | 2.4 | -5.81% | 2.2 | 2.4 | -6.51% |  |  |  |
| Class A2 | 1.6 | 1.8 | -9.99% | 1.7 | 1.9 | -8.40% |  |  |  |
| Class B | 2.2 | 2.3 | -5.58% | 2.3 | 2.5 | -4.30% | 2.3 | 2.4 | -4.33% |
| Class C | 1.9 | 2.1 | -6.41% | 2.4 | 2.6 | -5.79% | 2.4 | 2.6 | -5.74% |
| Class D | 1.9 | 2.1 | -7.50% | 2.8 | 2.9 | -5.77% | 2.7 | 2.9 | -5.63% |
| Class E | 2.8 | 3.0 | -6.93% |  |  |  | 3.1 | 3.4 | -7.29% |
| Class F | 5.3 | 5.6 | -5.23% | 33.7 | 34.6 | -3.99% | 50.7 | 51.7 | -3.73% |
| TGM | 11.8 | 12.1 | -2.21% | 107.1 | 108.1 | -0.96% | 124.9 | 126.2 | -0.81% |
| Overall | 2.1 | 2.3 | -6.76% | 2.2 | 2.4 | -5.96% | 2.6 | 2.7 | -5.54% |
| Enc Time[%] | 104% |  |  | 103% |  |  | 102% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  | 108% |  |  |

Table 2. CE3-2.3 results for enabling Luma BDPCM in lossless coding. A slice-level flag and encoder decision (from CE3-1.3) is used switch between regular residual coding (RRC) instead of TSRC on a slice-level basis.

# Recommendation

It is recommended to enable BDPCM with high-level regular coefficient coding switch for lossless coding.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 7)”, JVET-P2001, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 16th meeting, Geneva, CH, 1-11 Oct. 2019.

[2]  A. Nalci, H. Wang, H. E. Egilmez, M. Coban, M. Karczewicz “AHG18: BDPCM for Lossless,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-P0082, 16th Meeting: Geneva, CH, 1–11 October 2019.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).