| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0091 |
| --- | --- |

| Title: | CE4-5: Single mode partition for 4xN and Nx4 CU |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krit Panusopone Seungwook Hong Limin Wang | Tel: Email: | +1 858-674-6883 krit.panusopone@nokia.com seungwook.hong@nokia.com limin.2.wang@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

Geometric merge mode (GEO) is proposed in 15th Gothenburg JVET meeting and is used as a common base for CE4 tests. CE4-5 extends GEO support to 4xN and Nx4 CU. For 4xN and Nx4 GEO, only 1 mode partition is allowed. Using VTM-7.0 as a reference, simulation results show that CE4-5 have luma BD-rate of approximately -0.33% and -0.73% for RA and LB configurations, respectively.

# Introduction

CE4 investigates inter prediction with geometric partitioning (GEO) based on the technique proposed in JVET-P0884/P0885 [1],[2]. In CE4 [3], GEO is implemented on top of VTM-7.0 as a common base for CE4 experiments. CE4 common base allows 82 mode partition for CU width and CU height larger than or equal to 8 and smaller than 128.

JVET-P0449 [4] proposed to allow 1 mode partition for 4xN and Nx4 CU using TPM. CE4-5 extends the GEO of common base to support 4xN and Nx4 CU. Similar to JVET-P0449, GEO skips merge_geo_partition_idx syntax and allows one partitioning angle (either horizontal or vertical) and one distance. For 4xN CU, GEO employs vertical angle blending weights. For Nx4 CU, GEO employs horizontal angle blending weights. Motion vector prediction and blending operation of GEO with a single mode for 4xN and Nx4 CU are aligned with the GEO of common base.

# Proposed Solution

Like GEO in CE4 common base, a 4xN and Nx4 CU using GEO has 2 partitions and each partition has its own uni-prediction temporal prediction MV. However, mode partitioning of 4xN and Nx4 CU using GEO is determined based on CU dimension. Specifically, a CU with 4-pixel wide is split horizontally while a CU with 4-pixel high is split vertically. The split happens in the middle of the CU so that two split rectangular partitions have the same dimension.

Coding efficiency of GEO for 4xN and Nx4 can be improved with blending process. CE4-5 uses blending weights of [7/8 5/8 3/8 1/8] for luma block and [5/8 1/8] for chroma block.

Syntax to support GEO extension to 4xN and Nx4 in CE4-5 is slightly different because GEO is now allowed for 4xN and Nx4 CU, but its partition signaling for these CU dimensions is not needed. The related syntax is shown in the following merge data syntax table.

| merge_data( x0, y0, cbWidth, cbHeight, chType ) { | Descriptor |
| --- | --- |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumIbcMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( ( cbWidth * cbHeight ) >= 64 && ( (sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && cbWidth < 128 && cbHeight < 128 ) ¦ ¦ ( sps_geo_enabled_flag && MaxNumGeoMergeCand > 1 && slice_type = = B ) ) ) |  |
| regular_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( regular_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( sps_mmvd_enabled_flag ) |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| mmvd_cand_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_ciip_enabled_flag && sps_geo_enabled_flag && MaxNumGeoMergeCand > 1 && slice_type = = B && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| if( !ciip_flag[ x0 ][ y0 ] && MaxNumGeoMergeCand > 1 ) { |  |
| if ( cbWidth > 4 && cbHeight > 4 ) |  |
| merge_geo_partition_idx[ x0 ][ y0 ] | ae(v) |
| merge_geo_idx0[ x0 ][ y0 ] | ae(v) |
| if( MaxNumGeoMergeCand > 2 ) |  |
| merge_geo_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

MV derived for 4xN and Nx4 GEO mode is stored for later use as neighbor MV (AMVP, TMVP). In the current VVC design, MV is stored at 4x4 block granularity. For CE4-5, each 4x4 block stores MV of GEO partition that it belongs to. 4x4 blocks along horizontal/vertical split (blending block) that have their final predictors come from blending of two rectangular partitions, should store MV from the both rectangular partitions.

# Experimental Results

CE4-5 is implemented on top of VTM-7.0 common base. Experimental results are shown in Tables 1 and 2 against the VTM-7.0 under RA and LB CTC [5], respectively. Additionally, experimental results are shown in Tables 3 and 4 against the CE4 common base under RA and LB CTC, respectively.

Table 1. Simulation result of CE4-5, compared to VTM-7.0, for RA

|  |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.17% | -0.31% | 103% | 100% |
| Class A2 | -0.25% | -0.15% | -0.13% | 103% | 100% |
| Class B | -0.19% | -0.20% | -0.22% | 103% | 99% |
| Class C | -0.68% | -0.92% | -0.99% | 104% | 95% |
| Class E |  |  |  |  |  |
| Overall | -0.33% | -0.38% | -0.43% | 103% | 98% |
| Class D | -0.54% | -0.86% | -0.87% | 104% | 96% |
| Class F | -0.35% | -0.43% | -0.46% | 101% | 96% |

Table 2. Simulation result of CE4-5, compared to VTM-7.0, for LB

|  |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.42% | -0.73% | -0.66% | 105% | 96% |
| Class C | -0.87% | -1.35% | -0.98% | 106% | 94% |
| Class E | -1.07% | -0.05% | -0.13% | 101% | 94% |
| Overall | -0.73% | -0.77% | -0.63% | 104% | 95% |
| Class D | -0.88% | -0.99% | -0.85% | 105% | 96% |
| Class F | -0.55% | -1.03% | -0.26% | 103% | 96% |

Table 3. Simulation result of CE4-5, compared to CE4 common base, for RA

|  |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE4 common base |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.07% | -0.13% | 101% | 99% |
| Class A2 | -0.03% | 0.03% | -0.11% | 101% | 99% |
| Class B | -0.04% | -0.10% | -0.05% | 99% | 97% |
| Class C | -0.10% | 0.07% | -0.10% | 101% | 94% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | -0.02% | -0.09% | 100% | 97% |
| Class D | -0.11% | -0.19% | -0.26% | 100% | 93% |
| Class F | -0.03% | -0.05% | -0.13% | 99% | 95% |

Table 4. Simulation result of CE4-5, compared to CE4 common base, for LB

|  |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE4 common base |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | -0.23% | -0.06% | 100% | 96% |
| Class C | -0.08% | -0.11% | -0.09% | 100% | 95% |
| Class E | 0.10% | 0.14% | 0.76% | 99% | 95% |
| Overall | -0.03% | -0.10% | 0.13% | 100% | 95% |
| Class D | -0.20% | -0.52% | 0.20% | 100% | 95% |
| Class F | -0.04% | -0.12% | 0.04% | 99% | 97% |

# Conclusions

This contribution presents simulation results of CE4-5. In CE4-5, CU dimension is used to determine split direction when GEO mode is used for 4xN and Nx4 CU to keep complexity low. The results show a BD-rate performance of -0.05% and -0.03% for CE4-5, compared to CE4 common base under RA and LB coding condition, respectively, with similar encoding and decoding complexity.

# References

- H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang, M. Blaser, J. Sauer, “Simplified GEO without multiplication and minimum blending mask storage (harmonization of JVET-P0107, JVE-P0264, and JVET-P0304)”, JVET-P0884, Oct. 2019.

- K. Reuze, C.-C. Chen, H. Huang, W.-J. Chien, V. Seregin, M. Karczewicz, R.-L. Liao, J. Chen, Y. Ye, J. Luo, M. Blaser, J. Sauer, “Simplified GEO without multiplication and minimum blending mask storage (harmonization of JVET-P0107, JVE-P0264, and JVET-P0304)”, JVET-P0885, Oct. 2019.

- C.-C. Chen, R.-L. Liao, X. Xiu, H. Yang, “Description of Core Experiment 4 (CE4): Inter prediction with geometric partitioning”, JVET-P2024, Oct. 2019.

- K. Panusopone, S. Hong and L. Wang, “CE4 related: Thin partition mode”, JVET-P0449, Oct. 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Mar. 2019.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).