| sJoint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0092 |
| --- | --- |

| Title: | CE3: Modified Rice Parameter Derivation for Residual Coding (CE3-1.1, CE3-1.2, CE3-2.8 and CE3-2.9) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongtao Wang, Marta Karczewicz, Alican Nalci, Yung-Hsuan Chao, Muhammed Coban | Tel: Email: | +1-858-658-4535 hongtaow@qti.qualcomm.com martak@qti.qualcomm.com analci@qti.qualcomm.com yunghsua@qti.qualcomm.com mcoban@qti.qualcomm.com |
| Source: | Qualcomm Incorporated. |  |  |

_____________________________

# Abstract

This document reports the test results of CE3-1.1, CE3-1.2, CE3-2.8, CE3-2.9, in which several modifications to rice parameter calculation for regular transform residual coding (RRC) and transform-skip residual coding (TSRC) are proposed. The methods are summarized as follows:

- CE3-1.1-subtest 1&2. The maximum value of rice parameters used for regular transform residual coding is increased to 5. The value of the sum of the neighboring absolute coefficient values (localSumAbs) used to calculate rice parameter is normalized (in subtest 1 only) based on the number of existing neighbors.

- CE3-1.1-subtest 3&4. The maximum value of rice parameter used for transform-skip residual coding is increased to 5. The sum of absolute values of neighboring coefficients (localSumAbs) within a local template is used to derive the rice parameter. The value of localSumAbs is normalized (in subtest 3 only) based on the number of existing neighbors.

- CE3-1.2. A modified rice parameter derivation based on neighboring values is proposed, the maximum possible value of rice parameter used in the method is 3.

- Methods used in CE3-2.8 and CE3-2.9 are the same as CE3-1.1 and CE3-1.2, respectively. BDPCM is enabled in the lossless tests (Luma/Luma+Chroma).

The performance achieved by the proposed methods are summarized as follows:

CE3-1.1-subtest 1

|  | Lossless (BDPCM=0) | Lossy |
| --- | --- | --- |
| Natural | -5.75%(AI), -5.88%(RA), -4.98%(LB) | 0.00% (AI), -0.01% (RA), 0.00% (LB). |
| ClassF | -1.72%(AI), -2.64%(RA), -2.27%(LB) | -0.03% (AI), 0.01% (RA), -0.01%(LB). |
| ClassTGM | -0.17%(AI), -0.46%(RA), 0.68%(LB) | -0.03% (AI), 0.04% (RA), -0.03%(LB). |

CE3-1.1-subtest 3

|  | Lossless (BDPCM=0) | Lossy |
| --- | --- | --- |
| Natural | -5.09%(AI), -3.49%(RA), -2.48%(LB) | -0.01% (AI), -0.01% (RA), -0.02% (LB). |
| ClassF | -4.21%(AI), -3.30%(RA), -3.06%(LB) | -0.14% (AI), -0.15% (RA), -0.14% (LB). |
| ClassTGM | -5.09%(AI), -6.75%(RA), -6.89%(LB) | -0.39% (AI), -0.22% (RA), -0.18% (LB). |

To show the performance improvement introduced by the changes to RRC, the performance of CE3-2.8-subtest1 vs. CE3-2.3 (the current RRC is used for lossless coding) is summarized as follows:

|  | Lossless (BDPCM=1) |  |
| --- | --- | --- |
| Natural | -0.55%(AI), -0.33%(RA), -0.02%(LB) |  |
| ClassF | -0.35%(AI), -0.22%(RA), -0.12%(LB) |  |
| ClassTGM | -1.59%(AI), -1.04%(RA), -0.80%(LB) |  |

# CE Description

## CE3-1.1

### CE3-1.1-subtest 1

In CE3-1.1-subtest1, rice parameter for RRC is calculated based on the sum of neighboring absolute coefficient values (locSumAbs) and the base level (baseLevel) of the current coefficient:

if baseLevel == 0

cRiceParam=(locSumAbs > 256)? 5: ((locSumAbs > 128)? 4: riceParTable[max(min(31, locSumAbs), 0)])

else

cRiceParam=(locSumAbs > 256)? 4: riceParTable[max(min(31, locSumAbs– 5*baseLevel), 0)],

where the table riceParTable[] is the same as the one defined in VTM7.0 for transform coefficient coding. The value of locSumAbs is calculated as in VTM 7.0 using local template of up to 5 neighbouring coefficients. It is proposed to normalize the value of locSumAbs using the number of coefficients available in this template (noPos):

locSumAbs = (noPos == 0 || noPos == 5) ? locSumAbs: ((5*locSumAbs) >> (noPos >> 1)).

### CE3-1.1-subtest 2

In CE3-1.1-subtest2, rice parameters are derived in the same way as that in CE3-1.1-subtest1 except that the normalization of locSumAbs is not performed.

### CE3-1.1-subtest 3

In CE3-1.1-subtest3, rice parameter for TSRC is derived based on the sum of neighboring (top and left) absolute coefficient values (locSumAbs) and the base level (baseLevel) of the current coefficient.

if offset >= 0

cRiceParam = (locSumAbs > 128)? 5: ((locSumAbs > 64)? 4: riceParTable[min(max(locSumAbs + offset, 0), 31)])

else

cRiceParam = (locSumAbs > 128)? 4: riceParTable[min(max(locSumAbs + offset, 0), 31)],

where

offset = baseLevel == 0 ? 0 : (baseLevel == 2 ? -4 : -8),

and

riceParTable[]={0, 0, 0, 0, 1, 1, 1, 1, 1, 1, 1, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 3, 3, 3, 3, 3, 3, 3, 3, 3, 3}.

locSumAbs is calculated as a sum of absolute values of the neighbouring (left and top) coefficients levels. The number of coefficients available (noPos) when calculating locSumAbs is used to modify locSumAbs:

locSumAbs = (noPos = = 1)? locSumAbs<<1: locSumAbs.

### CE3-1.1-subtest 4

In CE3-1.1-subtest4, rice parameters are derived in the same way was as that in CE3-1.1-subtest3 except that the normalization of locSumAbs is not performed.

## CE3-1.2 Modified TSRC for lossless coding

In this test, rice parameter for TSRC is derived based on the sum of neighboring absolute coefficient values (locSumAbs) and the base level (baseLevel) of the current coefficient.

cRiceParam = riceParTable[min(max(locSumAbs + offset, 0), 31)]),

where

offset = baseLevel == 0 ? 0 : (baseLevel == 2 ? -4 : -8),

and

riceParTable[]={0, 0, 0, 0, 1, 1, 1, 1, 1, 1, 1, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 3, 3, 3, 3, 3, 3, 3, 3, 3, 3}.

locSumAbs is calculated as a sum of absolute values of the neighbouring (left and top) coefficients levels.

## CE3-2.8 and CE3-2.9

In CE3-2.8 (subtest 1, 2, 3 and 4) and CE3-2.9, the proposed RRC or TSRC methods are the same as CE3-1.1 (subtest 1, 2, 3 and 4) and CE3-1.2, respectively. The BDPCM is enabled in the lossless test (for Luma / Luma + Chroma).

The spec text change needed by CE3-2.8 and CE3-2.9 to support chroma BDPCM for YUV420 on top of the corresponding tests in CE3-1.1 and CE3-1.2 is shown as follows:

| … |  |
| --- | --- |
| sps_sao_enabled_flag | u(1) |
| sps_alf_enabled_flag | u(1) |
| sps_transform_skip_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag && chroma_format_idc = = 3 ) |  |
| sps_bdpcm_enabled_flag | u(1) |
| if( sps_bdpcm_enabled_flag ) |  |
| sps_bdpcm_chroma_enabled_flag | u(1) |
| sps_ref_wraparound_enabled_flag | u(1) |
| … |  |

# Simulation Results

The proposed methods are implemented on top of VTM7.0 and evaluated for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations.

To obtain the lossless results the configuration file ‘lossless.cfg’ is used and the first 100 frames of each sequence are encoded. BDPCM is disabled in lossless anchor and disabled/enabled depending on the configuration to be tested. IBC and HashME are turned on only for class F and class TGM.

Test results of lossy coding (standard QPs (22, 27, 32, 37) and low QPs (2,7,12,17)) are also presented for CE3-1.1 and for CE3-1.2. In the lossy tests, IBC, HashME, and BDPCM are turned on only for class F and class TGM. For low QP, the first 100 frames of each sequence are tested.

## CE3-1.1

### CE3-1.1 subtest 1: Extended rice parameters for RRC with normalization

In this section, simulation results of CE3-1.1, subtest 1: extended rice parameters for RRC are presented. In this method, the maximum values of rice parameter used for RRC is extended to 5 and the value of locSumAbs is normalized based on the number of existing neighbors.

In lossless coding, RRC is configured to be used for residual coding (--ForceRegularCoding=1). While in lossy coding, the modified RRC is used for regular transform residual coding and the original TSRC is used for transform-skip residual coding.

The lossless, standard QPs lossy and low QPs lossy are presented in Table 1, Table 2 and Table 3, respectively.

Table 1 Lossless results of CE3-1.1-subtest 1

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -5.98% | -6.69% |  |
| Class A2 | -10.46% | -9.01% |  |
| Class B | -4.33% | -3.90% | -3.91% |
| Class C | -5.34% | -5.39% | -5.30% |
| Class D | -5.14% | -5.30% | -5.26% |
| Class E | -3.74% |  | -6.35% |
| Class F | -1.72% | -2.64% | -2.27% |
| TGM | -0.17% | -0.46% | 0.68% |
| Overall | -5.75% | -5.88% | -4.98% |
| Enc Time[%] | 96% | 101% | 97% |
| Dec Time[%] | 104% | 101% | 105% |

Table 2 Lossy results of CE3-1.1-subtest1, starndard QPs (22,27,32,37)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.06% | 0.07% | 100% | 99% |
| Class A2 | 0.00% | 0.02% | 0.00% | 101% | 98% |
| Class B | 0.00% | 0.00% | -0.02% | 102% | 100% |
| Class C | 0.00% | 0.04% | 0.03% | 102% | 100% |
| Class E | 0.00% | -0.06% | -0.02% | 101% | 104% |
| Overall | 0.00% | -0.01% | 0.01% | 101% | 100% |
| Class D | -0.01% | -0.04% | -0.05% | 102% | 101% |
| Class F | -0.03% | -0.01% | 0.12% | 101% | 100% |
| TGM | -0.03% | -0.01% | -0.02% | 102% | 104% |
|  | Random Access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.07% | 0.01% | 101% | 101% |
| Class A2 | -0.02% | -0.08% | -0.07% | 101% | 100% |
| Class B | 0.01% | 0.03% | 0.22% | 101% | 100% |
| Class C | 0.00% | 0.00% | 0.03% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.02% | 0.07% | 101% | 100% |
| Class D | -0.04% | -0.24% | 0.09% | 101% | 99% |
| Class F | 0.01% | 0.03% | -0.10% | 101% | 100% |
| TGM | 0.04% | 0.01% | 0.04% | 101% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.13% | -0.14% | 101% | 98% |
| Class C | 0.01% | -0.30% | -0.26% | 100% | 97% |
| Class E | -0.05% | 0.57% | -0.12% | 102% | 101% |
| Overall | 0.00% | -0.01% | -0.17% | 101% | 98% |
| Class D | -0.05% | 0.45% | -0.65% | 101% | 99% |
| Class F | -0.01% | -0.54% | 0.18% | 100% | 100% |
| TGM | -0.03% | -0.14% | -0.07% | 101% | 103% |

Table 3 Lossy results of CE3-1.1-subtest1, low QPs (2,7,12,17)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.00% | 0.02% | 101% | 98% |
| Class A2 | 0.03% | -0.04% | -0.04% | 102% | 99% |
| Class B | -0.06% | -0.05% | -0.06% | 103% | 103% |
| Class C | -0.31% | -0.29% | -0.30% | 103% | 99% |
| Class E | -0.12% | -0.12% | -0.16% | 101% | 99% |
| Overall | -0.10% | -0.11% | -0.11% | 102% | 100% |
| Class D | -0.43% | -0.43% | -0.45% | 104% | 90% |
| Class F | -0.14% | -0.19% | -0.19% | 102% | 99% |
| TGM | -0.24% | -0.29% | -0.28% | 102% | 103% |
|  | Random Access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.01% | 0.02% | 101% | 99% |
| Class A2 | 0.06% | 0.00% | 0.00% | 101% | 101% |
| Class B | 0.02% | 0.01% | 0.01% | 101% | 101% |
| Class C | -0.04% | -0.05% | -0.08% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.01% | -0.01% | 101% | 100% |
| Class D | -0.08% | -0.08% | -0.13% | 101% | 100% |
| Class F | -0.16% | 0.00% | 0.07% | 102% | 101% |
| TGM | 0.37% | -0.17% | -0.15% | 102% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.04% | 0.04% | 101% | 99% |
| Class C | 0.01% | 0.03% | -0.02% | 101% | 102% |
| Class E | -0.01% | -0.01% | 0.02% | 103% | 106% |
| Overall | 0.01% | 0.02% | 0.01% | 101% | 102% |
| Class D | 0.00% | 0.07% | -0.11% | 101% | 100% |
| Class F | -0.04% | -0.01% | -0.13% | 101% | 100% |
| TGM | 0.08% | -0.18% | -0.22% | 101% | 99% |

### CE3-1.1-subtest 2: Extended rice parameters for RRC without normalization

In this section, simulation results of CE3-1.1-subtest 2: extended rice parameters for RRC without normalization are presented. In this method, the possible values of rice parameters used for RRC are extended up to 5, and no normalization is performed on the local sum of absolute values.

In lossless coding, RRC is configured to be used for residual coding (--ForceRegularCoding=1). While in lossy coding, the modified RRC is used for regular transform residual coding and the original TSRC is used for transform-skip residual coding.

The lossless, standard QPs lossy and low QPs lossy are presented in Table 4, Table 5 and Table 6, respectively.

Table 4 Lossless results of CE3-1. 1-subtest2

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -5.55% | -6.32% |  |
| Class A2 | -9.30% | -8.28% |  |
| Class B | -4.09% | -3.85% | -3.86% |
| Class C | -5.06% | -5.37% | -5.30% |
| Class D | -4.80% | -5.26% | -5.25% |
| Class E | -3.85% |  | -6.41% |
| Class F | -1.53% | -2.50% | -2.19% |
| TGM | 1.10% | 0.36% | 0.97% |
| Overall | -5.38% | -5.64% | -4.98% |
| Enc Time[%] | 93% | 98% | 97% |
| Dec Time[%] | 105% | 100% | 105% |

Table 5 Lossy results of CE3-1. 1-subtest2, starndard QPs (22,27,32,37)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | 0.01% | 100% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 105% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 102% | 101% |
| TGM | 0.00% | 0.00% | 0.00% | 102% | 104% |
|  | Random Access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | -0.01% | 101% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | -0.01% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.01% | 0.00% | 100% | 101% |
| Class F | 0.00% | 0.01% | 0.01% | 101% | 100% |
| TGM | 0.00% | 0.00% | 0.00% | 101% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class F | 0.00% | 0.02% | 0.01% | 101% | 100% |
| TGM | 0.00% | -0.01% | 0.00% | 101% | 100% |

Table 6 Lossy results of CE3-1. 1-subtest2, low QPs (2,7,12,17)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.01% | 100% | 96% |
| Class A2 | -0.02% | -0.06% | -0.07% | 101% | 100% |
| Class B | -0.03% | -0.02% | -0.03% | 100% | 102% |
| Class C | -0.06% | -0.08% | -0.07% | 100% | 98% |
| Class E | -0.04% | -0.03% | -0.08% | 100% | 99% |
| Overall | -0.03% | -0.04% | -0.05% | 100% | 99% |
| Class D | -0.08% | -0.12% | -0.11% | 100% | 88% |
| Class F | -0.02% | -0.05% | -0.04% | 102% | 101% |
| TGM | -0.01% | -0.08% | -0.07% | 101% | 101% |
|  | Random Access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | 0.00% | 100% | 100% |
| Class A2 | -0.01% | -0.01% | -0.01% | 99% | 100% |
| Class B | 0.00% | -0.01% | 0.00% | 100% | 99% |
| Class C | -0.02% | -0.02% | -0.05% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | -0.02% | 100% | 100% |
| Class D | -0.02% | 0.06% | 0.03% | 100% | 99% |
| Class F | -0.07% | -0.04% | -0.08% | 102% | 103% |
| TGM | 0.19% | -0.12% | -0.10% | 101% | 99% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.01% | -0.01% | 101% | 99% |
| Class C | 0.01% | 0.01% | -0.02% | 100% | 102% |
| Class E | -0.01% | 0.03% | 0.03% | 101% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class D | -0.02% | -0.02% | 0.00% | 100% | 100% |
| Class F | 0.04% | 0.03% | -0.05% | 100% | 101% |
| TGM | -0.10% | -0.17% | -0.24% | 101% | 100% |

### CE3-1.1-subtest 3: Extended rice parameters for TSRC with normalization

In this section, simulation results of CE3-1.1-subtest 3: extended rice parameters for TSRC are presented. In this method, the maximum value of rice parameters used for TSRC is extended to 5 and the value of locSumAbs is normalized based on the number of existing neighbors.

The lossless, standard QPs lossy and low QPs lossy are presented in Table 7, Table 8 and Table 9, respectively.

Table 7 Lossless results of CE3-1. 1-subtest3

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -4.45% | -4.39% |  |
| Class A2 | -8.73% | -6.58% |  |
| Class B | -3.72% | -1.77% | -1.77% |
| Class C | -4.93% | -2.64% | -2.50% |
| Class D | -5.54% | -2.80% | -2.66% |
| Class E | -4.60% |  | -3.64% |
| Class F | -4.21% | -3.30% | -3.06% |
| TGM | -5.09% | -6.75% | -6.89% |
| Overall | -5.09% | -3.49% | -2.48% |
| Enc Time[%] | 104% | 105% | 103% |
| Dec Time[%] | 105% | 104% | 105% |

Table 8 Lossy results of CE3-1. 1-subtest3, standard QPs (22,27,32,37)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | 0.05% | 99% | 99% |
| Class A2 | 0.00% | 0.00% | -0.01% | 101% | 98% |
| Class B | -0.01% | 0.06% | -0.06% | 101% | 101% |
| Class C | -0.03% | 0.02% | 0.00% | 100% | 100% |
| Class E | 0.01% | -0.03% | -0.07% | 101% | 101% |
| Overall | -0.01% | 0.02% | -0.02% | 100% | 100% |
| Class D | -0.06% | 0.06% | -0.06% | 100% | 100% |
| Class F | -0.14% | -0.21% | -0.08% | 102% | 101% |
| TGM | -0.39% | -0.31% | -0.33% | 102% | 103% |
|  | Random Access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.01% | 100% | 100% |
| Class A2 | -0.05% | -0.04% | 0.03% | 101% | 100% |
| Class B | 0.03% | -0.03% | 0.17% | 100% | 101% |
| Class C | -0.02% | -0.09% | -0.08% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.04% | 0.04% | 100% | 100% |
| Class D | -0.02% | -0.34% | -0.04% | 100% | 99% |
| Class F | -0.15% | -0.09% | -0.09% | 101% | 100% |
| TGM | -0.22% | -0.23% | -0.19% | 101% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.18% | -0.29% | 100% | 101% |
| Class C | 0.00% | -0.23% | -0.12% | 99% | 98% |
| Class E | -0.05% | 0.34% | -0.35% | 102% | 102% |
| Overall | -0.02% | -0.06% | -0.25% | 100% | 100% |
| Class D | 0.01% | 0.48% | 0.28% | 101% | 99% |
| Class F | -0.14% | -0.28% | 0.53% | 100% | 98% |
| TGM | -0.18% | -0.28% | -0.30% | 102% | 107% |

Table 9 Lossy results of CE3-1. 1-subtest3, low QPs (2,7,12,17)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.03% | -0.03% | 100% | 96% |
| Class A2 | 0.00% | -0.02% | -0.02% | 100% | 99% |
| Class B | -0.04% | -0.04% | -0.08% | 100% | 102% |
| Class C | -0.14% | -0.15% | -0.15% | 100% | 101% |
| Class E | -0.17% | -0.29% | -0.38% | 100% | 100% |
| Overall | -0.08% | -0.10% | -0.13% | 100% | 100% |
| Class D | -0.16% | -0.20% | -0.22% | 100% | 90% |
| Class F | -1.55% | -0.98% | -0.98% | 103% | 101% |
| TGM | -1.83% | -1.64% | -1.67% | 103% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.02% | -0.02% | 100% | 101% |
| Class A2 | 0.00% | -0.01% | 0.00% | 100% | 101% |
| Class B | -0.01% | -0.02% | -0.04% | 100% | 97% |
| Class C | -0.06% | -0.01% | -0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.02% | -0.03% | 100% | 99% |
| Class D | -0.12% | -0.11% | -0.21% | 100% | 100% |
| Class F | -1.93% | -0.81% | -0.80% | 102% | 103% |
| TGM | -2.43% | -1.51% | -1.48% | 101% | 98% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.01% | 0.00% | 100% | 99% |
| Class C | -0.03% | -0.01% | -0.01% | 100% | 101% |
| Class E | -0.05% | 0.00% | -0.12% | 101% | 101% |
| Overall | -0.03% | -0.01% | -0.03% | 100% | 100% |
| Class D | -0.08% | -0.03% | -0.13% | 101% | 100% |
| Class F | -0.64% | -0.63% | -0.75% | 100% | 100% |
| TGM | -1.51% | -1.14% | -1.17% | 103% | 101% |

### CE3-1.1-subtest4: Extended rice parameters for TSRC without normalization

In this section, simulation results of CE3-1.1-subtest 4: extended rice parameters without normalization for TSRC are presented. In this method, the maximum value of rice parameters used for TSRC is extended to 5 and no normalization is performed on the local sum of absolute neighbor values.

The lossless, standard QPs lossy and low QPs lossy are presented in Table 10, Table 11 and Table 12, respectively.

Table 10 Lossless results of CE3-1.1-subtest4

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -3.72% | -3.71% |  |
| Class A2 | -7.46% | -5.58% |  |
| Class B | -3.27% | -1.46% | -1.46% |
| Class C | -4.47% | -2.48% | -2.36% |
| Class D | -5.00% | -2.62% | -2.49% |
| Class E | -4.49% |  | -3.66% |
| Class F | -3.94% | -3.14% | -2.94% |
| TGM | -4.53% | -6.37% | -6.63% |
| Overall | -4.51% | -3.01% | -2.31% |
| Enc Time[%] | 101% | 103% | 102% |
| Dec Time[%] | 104% | 104% | 106% |

Table 11 Lossy results of CE3-1.1-subtest4, standard QPs (22,27,32,37)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | 0.04% | 101% | 101% |
| Class A2 | 0.00% | 0.01% | -0.02% | 101% | 98% |
| Class B | 0.00% | 0.03% | -0.05% | 101% | 100% |
| Class C | -0.03% | 0.01% | 0.06% | 101% | 105% |
| Class E | 0.00% | -0.03% | -0.07% | 101% | 106% |
| Overall | -0.01% | 0.00% | -0.01% | 101% | 102% |
| Class D | -0.06% | 0.01% | -0.11% | 100% | 99% |
| Class F | -0.18% | -0.21% | -0.06% | 102% | 102% |
| TGM | -0.40% | -0.33% | -0.37% | 102% | 104% |
|  | Random Access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.03% | 101% | 100% |
| Class A2 | -0.05% | -0.05% | 0.03% | 100% | 100% |
| Class B | 0.02% | -0.06% | 0.07% | 100% | 100% |
| Class C | -0.01% | -0.13% | -0.11% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.06% | 0.01% | 100% | 100% |
| Class D | -0.02% | -0.07% | -0.05% | 100% | 100% |
| Class F | -0.18% | -0.02% | -0.12% | 101% | 100% |
| TGM | -0.27% | -0.21% | -0.18% | 101% | 101% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.30% | -0.10% | 100% | 98% |
| Class C | 0.01% | -0.18% | -0.23% | 100% | 99% |
| Class E | -0.06% | 0.40% | -0.47% | 101% | 100% |
| Overall | -0.02% | -0.08% | -0.24% | 100% | 99% |
| Class D | -0.02% | 0.82% | -0.74% | 100% | 97% |
| Class F | -0.14% | -0.27% | 0.46% | 100% | 98% |
| TGM | -0.26% | -0.27% | -0.18% | 101% | 101% |

Table 12 Lossy results of CE3-1.1-subtest4, low QPs (2,7,12,17)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.03% | -0.04% | 100% | 98% |
| Class A2 | 0.00% | -0.02% | -0.02% | 101% | 101% |
| Class B | -0.04% | -0.05% | -0.09% | 100% | 103% |
| Class C | -0.12% | -0.13% | -0.12% | 100% | 101% |
| Class E | -0.18% | -0.30% | -0.37% | 100% | 99% |
| Overall | -0.07% | -0.10% | -0.12% | 100% | 100% |
| Class D | -0.12% | -0.18% | -0.15% | 101% | 90% |
| Class F | -1.44% | -0.84% | -0.83% | 102% | 100% |
| TGM | -1.54% | -1.39% | -1.40% | 103% | 99% |
|  | Random Access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.03% | -0.03% | 100% | 102% |
| Class A2 | 0.00% | 0.00% | -0.01% | 100% | 99% |
| Class B | -0.01% | -0.04% | -0.06% | 100% | 99% |
| Class C | -0.04% | -0.06% | -0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.03% | -0.04% | 100% | 100% |
| Class D | -0.10% | -0.11% | -0.08% | 100% | 100% |
| Class F | -1.92% | -0.92% | -0.95% | 102% | 102% |
| TGM | -2.30% | -1.45% | -1.40% | 102% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.01% | -0.02% | 101% | 99% |
| Class C | -0.03% | -0.02% | -0.02% | 100% | 101% |
| Class E | -0.06% | -0.01% | -0.04% | 101% | 103% |
| Overall | -0.02% | -0.01% | -0.02% | 101% | 101% |
| Class D | -0.07% | 0.01% | -0.11% | 100% | 100% |
| Class F | -0.61% | -0.51% | -0.72% | 100% | 98% |
| TGM | -1.38% | -0.93% | -0.93% | 103% | 104% |

## CE3-1.2: Modified rice parameter derivation for TSRC

In this section, simulation results of CE3-1.2 is presented. In this method, rice parameters used for TSRC are derived based on the local sum of absolute values and a look-up table. The maximum value of the rice parameter is 3.

The lossless, standard QPs lossy and low QPs lossy are presented in Table 13, Table 14 and Table 15, respectively.

Table 13 Lossless results of CE3-1.2

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -3.56% | -3.62% |  |
| Class A2 | -6.88% | -5.39% |  |
| Class B | -3.11% | -1.43% | -1.43% |
| Class C | -4.34% | -2.46% | -2.34% |
| Class D | -4.76% | -2.59% | -2.47% |
| Class E | -4.44% |  | -3.66% |
| Class F | -3.67% | -2.97% | -2.83% |
| TGM | -3.79% | -5.96% | -6.16% |
| Overall | -4.31% | -2.94% | -2.29% |
| Enc Time[%] | 99% | 100% | 100% |
| Dec Time[%] | 103% | 101% | 103% |

Table 14 Lossy results of CE3-1.2, starndard QPs (22,27,32,37)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | 0.04% | 100% | 100% |
| Class A2 | 0.00% | 0.01% | -0.02% | 101% | 99% |
| Class B | 0.00% | 0.03% | -0.05% | 100% | 100% |
| Class C | -0.03% | 0.01% | 0.06% | 100% | 102% |
| Class E | 0.00% | -0.03% | -0.07% | 101% | 102% |
| Overall | -0.01% | 0.00% | -0.01% | 100% | 100% |
| Class D | -0.06% | 0.01% | -0.11% | 100% | 98% |
| Class F | -0.18% | -0.21% | -0.06% | 102% | 100% |
| TGM | -0.40% | -0.33% | -0.37% | 101% | 102% |
|  | Random Access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.03% | 100% | 100% |
| Class A2 | -0.05% | -0.05% | 0.03% | 100% | 100% |
| Class B | 0.02% | -0.06% | 0.07% | 100% | 100% |
| Class C | -0.01% | -0.13% | -0.11% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.07% | 0.01% | 100% | 100% |
| Class D | -0.02% | -0.07% | -0.05% | 100% | 100% |
| Class F | -0.18% | -0.01% | -0.12% | 101% | 100% |
| TGM | -0.27% | -0.21% | -0.18% | 101% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.30% | -0.10% | 100% | 100% |
| Class C | 0.01% | -0.18% | -0.23% | 99% | 98% |
| Class E | -0.06% | 0.40% | -0.47% | 101% | 102% |
| Overall | -0.02% | -0.08% | -0.24% | 100% | 100% |
| Class D | -0.02% | 0.82% | -0.74% | 100% | 98% |
| Class F | -0.14% | -0.30% | 0.44% | 100% | 101% |
| TGM | -0.26% | -0.27% | -0.18% | 101% | 98% |

Table 15 Lossy results of CE3-1.2, low QPs (2,7,12,17)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.04% | -0.04% | 99% | 100% |
| Class A2 | 0.00% | -0.02% | -0.02% | 100% | 99% |
| Class B | -0.03% | -0.05% | -0.08% | 100% | 103% |
| Class C | -0.11% | -0.13% | -0.11% | 100% | 102% |
| Class E | -0.17% | -0.30% | -0.38% | 99% | 98% |
| Overall | -0.07% | -0.10% | -0.12% | 100% | 101% |
| Class D | -0.10% | -0.14% | -0.14% | 101% | 93% |
| Class F | -1.34% | -0.81% | -0.76% | 102% | 99% |
| TGM | -1.35% | -1.21% | -1.22% | 102% | 99% |
|  | Random Access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.01% | -0.02% | 100% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | -0.01% | -0.03% | -0.06% | 100% | 99% |
| Class C | -0.04% | -0.06% | -0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.03% | -0.04% | 100% | 100% |
| Class D | -0.10% | -0.08% | -0.10% | 100% | 101% |
| Class F | -1.76% | -0.77% | -0.78% | 102% | 103% |
| TGM | -1.94% | -1.13% | -1.15% | 101% | 99% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.02% | -0.02% | 101% | 99% |
| Class C | -0.03% | -0.01% | 0.00% | 100% | 101% |
| Class E | -0.07% | 0.02% | -0.04% | 100% | 100% |
| Overall | -0.03% | -0.01% | -0.02% | 100% | 100% |
| Class D | -0.05% | -0.05% | -0.18% | 100% | 99% |
| Class F | -0.56% | -0.56% | -0.68% | 100% | 99% |
| TGM | -1.18% | -0.89% | -0.87% | 102% | 101% |

## CE3-2.8

### CE3-2.8-subtest1: Enable BDPCM for CE3-1.1-subtest1

In this section, the method proposed in CE3-1.1-subtest1 is tested with BDPCM enabled for Luma/Luma+Chroma. The maximum value of rice parameter used for RRC is extended up to 5 and the value of locSumAbs is normalized based on the number of existing neighbors.

In lossless coding, RRC is configured to be used for residual coding (--ForceRegularCoding=1).

The simulation results of BDPCM=1 (Luma) and 2 (Luma+Chroma) are presented in Table 16 and Table 17, respectively.

Table 16 Lossless results of CE3-2.8-subtest1, BDPCM=1

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -6.36% | -6.95% |  |
| Class A2 | -11.54% | -9.28% |  |
| Class B | -5.84% | -4.37% | -4.41% |
| Class C | -6.77% | -5.82% | -5.76% |
| Class D | -7.99% | -5.82% | -5.66% |
| Class E | -6.91% |  | -7.23% |
| Class F | -6.07% | -4.58% | -3.82% |
| TGM | -3.16% | -1.74% | -0.64% |
| Overall | -7.26% | -6.26% | -5.56% |
| Enc Time[%] | 107% | 105% | 102% |
| Dec Time[%] | 97% | 101% | 107% |

Table 17 Lossless results of CE3-2.8-subtest1, BDPCM=2

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -7.21% | -7.51% |  |
| Class A2 | -11.87% | -9.33% |  |
| Class B | -6.59% | -4.62% | -4.64% |
| Class C | -7.56% | -6.03% | -5.93% |
| Class D | -9.47% | -6.12% | -5.87% |
| Class E | -9.12% |  | -7.86% |
| Class F | -7.95% | -5.43% | -4.46% |
| TGM | -6.18% | -2.86% | -1.20% |
| Overall | -8.21% | -6.52% | -5.87% |
| Enc Time[%] | 110% | 107% | 104% |
| Dec Time[%] | 93% | 98% | 103% |

### CE3-2.8-subtest2: Enable BDPCM for CE3-1.1-subtest2

In this section, the method proposed in CE3-1.1-subtest2 is tested with BDPCM enabled for Luma/Luma+Chroma. In this method, the maximum value of rice parameters used for RRC is extended to 5. No normalization is performed on locSumAbs.

In lossless coding, RRC is configured to be used for residual coding (--ForceRegularCoding=1).

The simulation results of BDPCM=1 (Luma) and 2 (Luma+Chroma) are presented in Table 18 and Table 19 , respectively.

Table 18 Lossless results of CE3-2.8-subtest2, BDPCM=1

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -5.97% | -6.59% |  |
| Class A2 | -10.48% | -8.57% |  |
| Class B | -5.66% | -4.31% | -4.35% |
| Class C | -6.49% | -5.80% | -5.75% |
| Class D | -7.64% | -5.78% | -5.64% |
| Class E | -6.94% |  | -7.29% |
| Class F | -5.84% | -4.44% | -3.76% |
| TGM | -1.95% | -0.92% | -0.02% |
| Overall | -6.91% | -6.02% | -5.55% |
| Enc Time[%] | 104% | 102% | 101% |
| Dec Time[%] | 96% | 99% | 107% |

Table 19 Lossless results of CE3-2.8-subtest2, BDPCM=2

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -6.76% | -7.11% |  |
| Class A2 | -10.91% | -8.64% |  |
| Class B | -6.38% | -4.55% | -4.56% |
| Class C | -7.26% | -6.01% | -5.91% |
| Class D | -9.09% | -6.08% | -5.85% |
| Class E | -9.10% |  | -7.92% |
| Class F | -7.71% | -5.30% | -4.37% |
| TGM | -5.10% | -2.10% | -0.51% |
| Overall | -7.85% | -6.27% | -5.85% |
| Enc Time[%] | 107% | 103% | 103% |
| Dec Time[%] | 92% | 97% | 103% |

### CE3-2.8-subtest3: Enable BDPCM for CE3-1.1-subtest3

In this section, the method proposed in CE3-1.1-subtest3 is tested with BDPCM enabled for Luma/Luma+Chroma. In this method, the possible values of rice parameters used for TSRC are extended up to 5 and the value of locslSumAbs is normalized based on the number of existing neighbors.

The simulation results of BDPCM=1 (Luma) and 2 (Luma+Chroma) are presented in Table 20 and Table 21, respectively.

Table 20 Lossless results of CE3-2.8-subtest3, BDPCM=1

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -4.70% | -4.60% |  |
| Class A2 | -9.67% | -6.87% |  |
| Class B | -4.79% | -2.13% | -2.16% |
| Class C | -5.96% | -2.99% | -2.91% |
| Class D | -7.76% | -3.23% | -3.00% |
| Class E | -7.16% |  | -4.66% |
| Class F | -7.43% | -4.62% | -4.26% |
| TGM | -6.86% | -7.56% | -7.57% |
| Overall | -6.25% | -3.80% | -3.03% |
| Enc Time[%] | 114% | 109% | 109% |
| Dec Time[%] | 98% | 103% | 105% |

Table 21 Lossless results of CE3-2.8-subtest3, BDPCM=2

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -5.21% | -4.98% |  |
| Class A2 | -9.81% | -6.87% |  |
| Class B | -5.25% | -2.27% | -2.28% |
| Class C | -6.46% | -3.12% | -3.01% |
| Class D | -8.77% | -3.43% | -3.13% |
| Class E | -8.39% |  | -4.91% |
| Class F | -8.68% | -5.16% | -4.61% |
| TGM | -8.97% | -8.37% | -7.77% |
| Overall | -6.80% | -3.96% | -3.18% |
| Enc Time[%] | 120% | 111% | 109% |
| Dec Time[%] | 95% | 101% | 102% |

### CE3-2.8-subtest4: Enable BDPCM for CE3-1.1-subtest4

In this section, the method proposed in CE3-1.1-subtest4 is tested with BDPCM enabled for Luma/Luma+Chroma. In this method, the possible values of rice parameters used for TSRC are extended up to 5. No normalization is performed on localSumAbs.

The simulation results of BDPCM=1 (Luma) and 2 (Luma+Chroma)  are presented in Table 22 and Table 23, respectively.

Table 22 Lossless results of CE3-2.8-subtest4, BDPCM=1

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -3.98% | -3.92% |  |
| Class A2 | -8.43% | -5.88% |  |
| Class B | -4.36% | -1.82% | -1.84% |
| Class C | -5.51% | -2.83% | -2.76% |
| Class D | -7.23% | -3.06% | -2.83% |
| Class E | -7.06% |  | -4.68% |
| Class F | -7.19% | -4.47% | -4.13% |
| TGM | -6.32% | -7.18% | -7.32% |
| Overall | -5.68% | -3.32% | -2.86% |
| Enc Time[%] | 111% | 106% | 106% |
| Dec Time[%] | 100% | 101% | 104% |

Table 23 Lossless results of CE3-2.8-subtest4, BDPCM=2

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -4.49% | -4.32% |  |
| Class A2 | -8.61% | -5.88% |  |
| Class B | -4.84% | -1.97% | -1.97% |
| Class C | -6.02% | -2.97% | -2.86% |
| Class D | -8.26% | -3.26% | -2.96% |
| Class E | -8.28% |  | -4.93% |
| Class F | -8.43% | -5.01% | -4.41% |
| TGM | -8.46% | -8.00% | -7.39% |
| Overall | -6.25% | -3.49% | -3.01% |
| Enc Time[%] | 115% | 108% | 107% |
| Dec Time[%] | 95% | 100% | 102% |

## CE3-2.9

In this section, the method proposed in CE3-1.2 is tested with BDPCM enabled for Luma/Luma+Chroma. The rice parameter used for TSRC is derived based on the local sum of absolute values and a look-up table. The maximum value of rice parameter is 3.

The simulation results of BDPCM=1 (Luma) and 2 (Luma+Chroma) are presented in Table 24 and Table 25, respectively.

Table 24 Lossless results of CE3-2.9, BDPCM=1

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -3.85% | -3.84% |  |
| Class A2 | -7.94% | -5.69% |  |
| Class B | -4.26% | -1.79% | -1.82% |
| Class C | -5.42% | -2.82% | -2.75% |
| Class D | -7.08% | -3.03% | -2.81% |
| Class E | -7.04% |  | -4.68% |
| Class F | -6.98% | -4.34% | -3.96% |
| TGM | -5.72% | -6.83% | -6.91% |
| Overall | -5.53% | -3.26% | -2.84% |
| Enc Time[%] | 109% | 104% | 104% |
| Dec Time[%] | 98% | 100% | 103% |

Table 25 Lossless results of CE3-2.9, BDPCM=2

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -4.37% | -4.25% |  |
| Class A2 | -8.17% | -5.71% |  |
| Class B | -4.74% | -1.94% | -1.95% |
| Class C | -5.93% | -2.95% | -2.85% |
| Class D | -8.11% | -3.24% | -2.96% |
| Class E | -8.27% |  | -4.93% |
| Class F | -8.24% | -4.89% | -4.27% |
| TGM | -7.90% | -7.66% | -7.08% |
| Overall | -6.10% | -3.43% | -2.99% |
| Enc Time[%] | 113% | 106% | 105% |
| Dec Time[%] | 94% | 99% | 100% |

## Supplemental results

In this section, the results of combining test CE3-1.1-subtest 3 (extended rice parameters with normalization), CE3-1.2 (modified rice parameters) and the test 3 of CE3-2.5 are provided. In CE3-2.5 test 3, a CU level flag is signaled to select between TSRC and RRC for transform-skip mode.

The comparison between CE3-2.8-subtest1 and CE3-2.3 is also provided to show the performance changes introduced by the modification RRC.

### Combined CE3-1.1-subtest3 and of CE3-2.5-subtest3

In this section, simulation results of combining CE3-1.1-subtest3 and CE3-2.5 test 3 is presented. The simulation results of BDPCM=0 and 1 are presented in Table 26 and Table 27, respectively.

Table 26 Combined test of CE3-1.1-subtest3 and CE3-2.5 test 3, BDPCM=0

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -5.41% | -6.24% |  |
| Class A2 | -8.85% | #DIV/0! |  |
| Class B | -3.99% | -3.83% | -3.85% |
| Class C | -4.95% | -5.35% | -5.28% |
| Class D | -4.61% | -5.24% | -5.22% |
| Class E | -3.80% |  | -6.41% |
| Class F | -3.27% | -4.18% | -4.03% |
| TGM | -4.91% | -7.08% | -7.11% |
| Overall | -5.22% | #DIV/0! | -4.96% |
| Enc Time[%] | 95% | #NUM! | 100% |
| Dec Time[%] | 105% | #NUM! | 108% |

Table 27 Combined test of CE3-1. 1-subtest3 and CE3-2.5 test 3, BDPCM=1

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -5.80% | -6.50% |  |
| Class A2 | -9.99% | #DIV/0! |  |
| Class B | -5.57% | -4.30% | -4.34% |
| Class C | -6.40% | -5.78% | -5.73% |
| Class D | -7.48% | -5.75% | -5.62% |
| Class E | -6.91% |  | -7.28% |
| Class F | -6.99% | -5.72% | -5.32% |
| TGM | -6.77% | -7.91% | -7.51% |
| Overall | -6.75% | #DIV/0! | -5.54% |
| Enc Time[%] | 105% | #NUM! | 105% |
| Dec Time[%] | 97% | #NUM! | 108% |

### Combined CE3-1.2 and of CE3-2.5-subtest3

In this section, simulation results of combining CE3-1.2 and CE3-2.5 test 3 are presented.

The simulation results of BDPCM=0 and 1 are presented in Table 28 and Table 29, respectively.

Table 28 Combined test of CE3-1.2 and CE3-2.5 test 3, BDPCM=0

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -5.41% | -6.24% |  |
| Class A2 | -8.85% | #DIV/0! |  |
| Class B | -3.99% | -3.83% | -3.85% |
| Class C | -4.95% | -5.35% | -5.28% |
| Class D | -4.61% | -5.24% | -5.22% |
| Class E | -3.80% |  | -6.41% |
| Class F | -3.59% | -4.34% | -4.17% |
| TGM | -5.89% | -7.67% | -7.58% |
| Overall | -5.22% | #DIV/0! | -4.96% |
| Enc Time[%] | 95% | #NUM! | 101% |
| Dec Time[%] | 105% | #NUM! | 107% |

Table 29 Combined test of CE3-1.2 and CE3-2.5 test 3, BDPCM=1

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -5.80% | -6.50% |  |
| Class A2 | -9.99% | #DIV/0! |  |
| Class B | -5.57% | -4.30% | -4.34% |
| Class C | -6.40% | -5.78% | -5.73% |
| Class D | -7.48% | -5.75% | -5.62% |
| Class E | -6.91% |  | -7.28% |
| Class F | -7.23% | -5.84% | -5.45% |
| TGM | -7.64% | -8.46% | -8.41% |
| Overall | -6.75% | #DIV/0! | -5.54% |
| Enc Time[%] | 106% | #NUM! | 104% |
| Dec Time[%] | 97% | #NUM! | 108% |

### CE3-2.8-subtest1 compared to CE3-2.3

To show the performance improvement introduced by the changes to RRC, the comparison of CE3-2.8-subtest1 vs. CE3-2.3 (the current RRC is used for lossless coding) is presented in Table 30.

BDPCM is enabled for luma in both anchor and the test of the comparison.

Table 30 CE3-2.8-subtest1 using CE3-2.3 as anchor

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -0.61% | -0.49% |  |
| Class A2 | -1.73% | -0.97% |  |
| Class B | -0.29% | -0.08% | -0.08% |
| Class C | -0.39% | -0.03% | -0.02% |
| Class D | -0.52% | -0.06% | -0.03% |
| Class E | 0.02% |  | 0.07% |
| Class F | -0.35% | -0.22% | -0.12% |
| TGM | -1.59% | -1.04% | -0.80% |
| Overall | -0.55% | -0.33% | -0.02% |
| Enc Time[%] | 103% | 103% | 102% |
| Dec Time[%] | 100% | 99% | 99% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)