| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0093-v1 |
| --- | --- |

| Title: | CE5-1.3: CCALF with 6 filter coefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yin Zhao Anand Meher Kotra Haitao Yang #360, Jiangshu Road, Hangzhou 310011, China | Tel: Email: | +86-571-28163645 {yin.zhao, anand.meher.kotra, haitao.yang} @huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the results of CE5-1.3, wherein the CCALF filter shape is simplified from 8 taps to 6 taps of a cross shape. It is reported that compared with the CCALF common base, the average coding performance difference is -0.02% / 0.39% / 0.26% in AI, -0.01% / 0.22% / 0.31% in RA, and 0.01% / 0.22% / 0.23% in LDB.

Proposed Method

The CE5-1.3 uses a cross-shape 6-coefficient CCALF filter, as shown in Fig. 1. Therefore, the multiplication per sample of CCALF is reduced by 25%. The other parts are kept the same as the CCALF common base.

Fig. 1. The 6-tap CCALF (the 6 taps are a subset of the 8 taps of the CCALF common base)

Experiment results

The proposed method were implemented on top of CCALF common base (JVET-Q0058), and tested under AI, RA and LDB configuration of the Common Test Conditions (CTC). Detailed results can be found in Table 1.

Table 1 Coding performance of the 6-tap CCALF vs. CCALF common base

Conclusions

It is suggested to adopt the simplified CCALF into VVC.

Draft Text change over CCALF common base

Deleted text are marked in strikethrough. Added text are highlighted in yellow.

| if ( alf_cross_component_cb_filter_signal_flag ) { |  |
| --- | --- |
| alf_cross_component_cb_filters_signalled_minus1 | ue(v) |
| for( k = 0; k < (alf_cross_component_cb_filters_signalled_minus1+1); k++ ) { |  |
| for ( j = 0; j < 86; j++ ) |  |
| alf_cross_component_cb_coeff_plus32[ k ][ j ] | u(6) |
| } |  |
| } |  |
| if ( alf_cross_component_cr_ filter_signal_flag ) { |  |
| alf_cross_component_cr_filters_signalled_minus1 | ue(v) |
| for( k = 0; k < (alf_cross_component_cr_filters_signalled_minus1+1); k++ ) { |  |
| for ( j = 0; j < 86; j++ ) |  |
| alf_cross_component_cr_coeff_plus32[ k ][ j ] | u(6) |
| } |  |
| } |  |

  - For every coding tree unit with luma coding tree block location ( rx, ry ), where rx = 0..PicWidthInCtbsY − 1 and ry = 0..PicHeightInCtbsY − 1, the following applies:

  - When ChromaArrayType is not equal to 0 and alf_ctb_cross_component_cb_idc[ rx ][ ry ] is not equal to 0, the cross component filtering process for block of chroma samples as specified in clause x.x.x.x is invoked with recPictureL set equal to recPictureL, alfPictureC set equal to alfPictureCb, the chroma coding tree block location ( xCtbC, yCtbC ) set equal to ( ( rx << CtbLog2SizeY ) / SubWidthC, ( ry << CtbLog2SizeY  )/ SubHeightC ) ), the luma coding tree block location ( xCtb, yCtb ) set equal to ( rx  <<  CtbLog2SizeY, ry  <<  CtbLog2SizeY ), ccAlfWidth set equal to ( 1 << <<  CtbLog2SizeY ) / SubWidthC, ccAlfHeight set equal to ( 1<< <<  CtbLog2SizeY ) / SubHeightC, and the cross component filter coefficients CcAlfCoeff[ j ] set equal to CcAlfCoeffCb[ slice_cross_component_alf_cb_aps_id ][ alf_ctb_cross_component_cb_idc[ rx ][ ry ] − 1 ][ j ], with j = 0..75, as inputs, and the output is the modified filtered picture ccAlfPictureCb.

  - When ChromaArrayType is not equal to 0 and alf_ctb_cross_component_cr_idc[ rx ][ ry ] is not equal to 0, the cross component filtering process for block of chroma samples as specified in clause x.x.x.x is invoked with recPictureL set equal to recPictureL, alfPictureC set equal to alfPictureCr, the chroma coding tree block location ( xCtbC, yCtbC ) set equal to ( ( rx << CtbLog2SizeY ) / SubWidthC, ( ry << CtbLog2SizeY  )/ SubHeightC ) ), the luma coding tree block location ( xCtb, yCtb ) set equal to ( rx  <<  CtbLog2SizeY, ry  <<  CtbLog2SizeY ), ccAlfWidth set equal to ( 1 << <<  CtbLog2SizeY ) / SubWidthC, ccAlfHeight set equal to ( 1<< <<  CtbLog2SizeY ) / SubHeightC, and the cross component filter coefficients CcAlfCoeff[ j ] set equal to CcAlfCoeffCr[ slice_cross_component_alf_cr_aps_id ][ alf_ctb_cross_component_cr_idc[ rx ][ ry ] − 1 ][ j ], with j = 0..75, as inputs, and the output is the modified filtered picture ccAlfPictureCr.

  - The array of cross component filter coefficients f[ j ] is derived as follows with j = 0..75:

f[ j ] = CcAlfCoeff[ j ]		(8-1287)

  - The variable sum is derived as follows:

sum = 	f[ 0 ] * recPictureL[ hx, vy + yM1 ] +
	f[ 1 ] * recPictureL[ hx+xM1, vy ] +
	f[ 2 ] * recPictureL[ hx, vy ] +
	f[ 3 ] * recPictureL[ hx+xP1, vy ] +
	f[ 4 ] * recPictureL[ hx+xM1, vy+yP1 ] +
	f[ 53 ] * recPictureL[ hx, vy+yP1 ] +
	f[ 64 ] * recPictureL[ hx+xP1, vy+yP1 ] +	
	f[ 75 ] * recPictureL[ hx, vy + yP2 ]

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).