| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0xxx |
| --- | --- |

| Title: | CE1-4: Disable deblocking for affine sub-PU edges when PROF is applied |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li, XiangLi, Xiaozhong Xu, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. | Tel: Email: | +1-650-798-3300 guichunli@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This is a report of the core experiment of CE1-4, disabling deblocking for affine sub-PU edges when PROF is applied. The proposed methods were implemented on top of VTM-7.0, and simulations were conducted with common test conditions. It is reported that compared with VTM-7.0 anchor, the proposed method has overall performance as following.

- ALF on: RA: -0.02%, Enc 100%, Dec 99%; LB -0.04%, Enc 100%, Dec 100%, LP 0.00%, Enc 101%, Dec 100%;

- ALF off: RA: -0.03%, Enc 100%, Dec 100%; LB 0.03%, Enc 100%, Dec 99%, LP -0.03%, Enc 100%, Dec 99%;

# Introduction

It was previously proposed in JVET-P0441 to disable deblocking for affine sub-PU edges when PROF is applied. In current VTM-7.0, deblocking is enabled on sub-block boundaries for sub-block based inter prediction modes, such as affine inter prediction and Sub-block based TMVP (SbTMVP). The deblocking filtering process are applied on a 4x4 grid for CU boundaries and transform subblock boundaries and on an 8x8 grid for prediction subblock boundaries.

Figure 1 - Affine Subblock boundary deblocking

Affine inter prediction has sub-block size of 4x4 luma samples, but MV difference based subblock boundary filtering only applies to sub-block edges aligned with 8x8 grid of luma samples, as shown in the solid lines with orange colour. Other affine subblock edges may be filtered only when they are aligned with a transform block boundary, such as in subblock transform. This causes inconsistency in the subblock deblocking behavior. Even when the neighboring MV delta are the same for subblock edges at 8x8 grid and those at 4x4 grid, only those aligned with 8x8 grids and TU boundaries are filtered.  It is desirable to fix this inconsistency.

Prediction refinement with optical flow (PROF) is applied on subblocks of affine coded CUs. PROF is able to compensate the prediction accuracy loss caused by 4x4 subblock based motion compensation, and it makes the refined prediction output similar to the pixel-based affine motion compensation output. Taking this into consideration, 4x4 sub-block based MV difference checking may lead to wrong deblocking decision.

# Proposed Method

It is proposed to disable affine sub-PU boundary deblocking when PROF is enabled, to make the MV delta based deblocking behavior consistent. When the edge is an affine internal subblock edge which is not aligned with CU boundary or TU boundary, the deblocking is disabled by skipping the MV difference based deblocking strength derivation.

# Experimental Results

The proposed method was implemented based on VTM-7.0. Simulations were conducted following JVET common test conditions. VTM-7.0 was used as anchor.

It was also tested with ALF off on both VTM-7.0 anchor and the proposed test.

Bit streams for subjective tests were also provided and uploaded to the FTP server of CE1.

## Results of disable affine sub-PU boundary deblocking

As shown in Table 3.1, it was observed that the BD-rate change of proposed method is -0.02% on RA, and -0.04% on LB, 0.00% on LP, compared with VTM-7.0 anchor.

Table 3.1 Simulation Results of CTC

|  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.08% | -0.04% | 100% | 99% |
| Class A2 | -0.08% | -0.06% | -0.03% | 100% | 99% |
| Class B | -0.02% | 0.03% | 0.05% | 100% | 99% |
| Class C | 0.00% | -0.04% | 0.04% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.03% | 0.01% | 100% | 99% |
| Class D | 0.01% | 0.03% | 0.07% | 100% | 101% |
| Class F | 0.00% | -0.01% | -0.02% | 100% | 101% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.31% | 0.00% | 100% | 99% |
| Class C | -0.02% | -0.15% | -0.13% | 100% | 102% |
| Class E | -0.10% | 0.38% | -0.19% | 100% | 100% |
| Overall | -0.04% | -0.08% | -0.09% | 100% | 100% |
| Class D | -0.03% | 0.19% | -0.32% | 101% | 101% |
| Class F | 0.02% | -0.19% | -0.05% | 100% | 102% |
|  |  |  | LDP |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.29% | -0.27% | 100% | 99% |
| Class C | 0.01% | -0.08% | -0.18% | 101% | 101% |
| Class E | -0.02% | 0.39% | 0.00% | 101% | 100% |
| Overall | 0.00% | 0.19% | -0.17% | 101% | 100% |
| Class D | 0.04% | -0.16% | 0.53% | 101% | 100% |
| Class F | 0.01% | -0.54% | -0.11% | 101% | 102% |

As shown in Table 3.2, it was observed that the BD-rate change of proposed method is -0.03% on RA, and 0.03% on LB, -0.03% on LP, compared with VTM-7.0 anchor.

Table 3.2 Simulation Results of ALF off

|  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 ALF off |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | -0.06% | 101% | 100% |
| Class A2 | -0.10% | 0.03% | 0.00% | 100% | 99% |
| Class B | 0.00% | 0.01% | 0.00% | 100% | 99% |
| Class C | 0.00% | -0.02% | 0.06% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | 0.00% | 0.01% | 100% | 100% |
| Class D | 0.00% | 0.03% | 0.07% | 100% | 102% |
| Class F | -0.02% | 0.00% | 0.01% | 100% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-7.0 ALF off |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.05% | 0.10% | 100% | 99% |
| Class C | 0.01% | -0.07% | -0.26% | 99% | 100% |
| Class E | 0.13% | -0.01% | -0.37% | 99% | 98% |
| Overall | 0.03% | -0.05% | -0.14% | 100% | 99% |
| Class D | -0.04% | 0.00% | 0.12% | 100% | 98% |
| Class F | 0.03% | -0.11% | 0.24% | 100% | 100% |
|  |  |  | LDP |  |  |
|  |  |  | Over VTM-7.0 ALF off |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.06% | -0.17% | 100% | 98% |
| Class C | -0.04% | 0.02% | -0.02% | 100% | 101% |
| Class E | -0.10% | -0.11% | 0.34% | 100% | 98% |
| Overall | -0.03% | 0.01% | 0.01% | 100% | 99% |
| Class D | 0.02% | -0.13% | 0.71% | 100% | 99% |
| Class F | 0.00% | 0.56% | -0.55% | 99% | 96% |

# Visual quality

Subjective quality was also evaluated, and it was observed that the visual quality of CE1-4 is generally equivalent to VTM-7.0 anchor without degradation. In some cases, there are some more details on the CE1-4 side, such as shown in the screenshots below in Figure 2 and Figure 3.

Figure 2 – Screenshot of Cactus with LB QP 37

Figure 3 – Screenshot of BasketballDrill with LB QP 37

# Conclusion

It was observed that the method in CE1-4 has BD-rate change of -0.02% on RA and -0.04% on LB, and 0.00% on LP compared with VTM-7.0. It was asserted that there’s no degradation on visual quality.

It is proposed to adopt the proposed method to VVC.

# Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-N1010 Marrakech, MA, Mar 2019.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-7.0

- G. Li, X. Li, X. Xu, S. Liu, “Non-CE5: On deblocking on affine internal prediction sub-block edges”, Document of Joint Video Experts Team (JVET), JVET-P0441 Geneva, CH, Oct 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).