| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0096 |
| --- | --- |

| Title: | CE1-3.2: Boundary strength derivation for blocks with BCW |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | S. Iwamura S. Nemoto A. Ichigaya (NHK) K. Andersson R. Yu J. Enhorn (Ericsson) | Tel: Email: | iwamura.s-gc@nhk.or.jp kenneth.r.andersson@ericsson.com |
| Source: | NHK, Ericsson |  |  |

_____________________________

- Abstract

This contribution reports results for the CE1 sub-test for boundary strength derivation for blocks with bi-prediction with CU-level weight (BCW). When deriving the boundary strength, the value of the current edge for luma is set to 1 when the bi-prediction weights of the two adjacent blocks of the current edge are not the same. Note that CE1-3.2 checks coincidence of bi-prediction weights used in the prediction of two adjacent subblocks for the same reference picture, whereas CE1-3.1 checks bcwIdx of two adjacent subblocks.

Summary results of CE1-3.2 over VTM-7.0 are as follows:

RA: 0.02% (Y), -0.02% (Cb), 0.01% (Cr)
 LDB: 0.02% (Y), -0.14% (Cb), -0.04% (Cr)

Introduction

In the derivation process of boundary strength for deblocking filter, a potential discontinuity caused by Bi-prediction with CU-level Weight (BCW) is not considered in the current VVC draft specifications [1]. Specifically, when difference of motion vectors of given adjacent blocks (block P and Q) are below the threshold, and both block P and Q are skipped, BS is set equal to 0 in VTM-7.0. However, when the weights of P and Q are different, the discontinuity between prediction signals of P and Q may appear as shown in Figure 3.

Figure 3. Possible discontinuity for CU boundary with BCW

In JVET-P0586, it was suggested to consider the coincidence of prediction weights as well as motion vector difference and non-zero coefficient existence when determining boundary strength for deblocking filter process [2] and it was decided to include the proposal into the core experiments on deblocking filter [3].

It should be asserted that even when the bcwIdx of block P and Q are the same, if the prediction direction is opposite between P and Q, there may be discontinuity on the block boundary of block P and Q as shown in Figure 4.

Figure 4. Bi-prediction weight coincidence in consideration of prediction direction

Specification changes

The proposed modification of the specification [1] of the BS derivation is as follows (texts yellow highlighted are changed):

If the cIdx is equal to 0 and one or more of the following conditions are true, bS[xDi][yDj] is set equal to 1:

…

      - Two motion vectors and two different reference pictures are used to predict the coding subblock containing the sample p0, two motion vectors for the same two reference pictures are used to predict the coding subblock containing the sample q0 and the absolute difference between the horizontal or vertical component of the two motion vectors used in the prediction of the two coding subblocks for the same reference picture is greater than or equal to 8 in units of 1/16 luma samples, or the bi-prediction weights used in the prediction of the two coding subblocks for the same reference picture are different.

      - Two motion vectors for the same reference picture are used to predict the coding subblock containing the sample p0, two motion vectors for the same reference picture are used to predict the coding subblock containing the sample q0 and both of the following conditions are true:

        - The absolute difference between the horizontal or vertical component of list 0 motion vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in 1/16 luma samples, or the absolute difference between the horizontal or vertical component of the list 1 motion vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in units of 1/16 luma samples, or the bi-prediction weights used in the prediction of the two coding subblocks for list 0 are different.

        - The absolute difference between the horizontal or vertical component of list 0 motion vector used in the prediction of the coding subblock containing the sample p0 and the list 1 motion vector used in the prediction of the coding subblock containing the sample q0 is greater than or equal to 8 in units of 1/16 luma samples, or the absolute difference between the horizontal or vertical component of the list 1 motion vector used in the prediction of the coding subblock containing the sample p0 and list 0 motion vector used in the prediction of the coding subblock containing the sample q0 is greater than or equal to 8 in units of 1/16 luma samples, or the bi-prediction weight used in the prediction of the coding subblock containing p0 for list 0 is not equal to the bi-prediction weight used in the prediction of the coding subblock containing q0 for list 1.

- Otherwise, the variable bS[ xDi ][ yDj ] is set equal to 0.

Simulation results

The proposed boundary strength derivation is implemented on top of VTM-7.0 and coding experiments under JVET common test condituion [3] are conducted. Table 1 and 2 shows the summary results of the sub-test CE1-3.2 over VTM-7.0 anchor with random access and low delay B configurations, respectively.

Table 1. Summary of coding expeimtns of CE1-3.2 with random access configuration over VTM-7.0

|  |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.01% | 100% | 100% |
| Class A2 | 0.01% | -0.03% | -0.04% | 100% | 100% |
| Class B | 0.02% | -0.04% | 0.00% | 100% | 102% |
| Class C | 0.03% | 0.01% | 0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.02% | 0.01% | 100% | 101% |
| Class D | 0.01% | -0.02% | 0.08% | 100% | 101% |
| Class F | 0.00% | -0.02% | -0.03% | 100% | 101% |

Table 2. Summary of coding expeimtns of CE1-3.2 with low delay B configuration over VTM-7.0

|  |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.23% | 0.19% | 100% | 99% |
| Class C | 0.02% | -0.27% | -0.09% | 101% | 102% |
| Class E | 0.01% | 0.19% | -0.34% | 101% | 101% |
| Overall | 0.02% | -0.14% | -0.04% | 100% | 100% |
| Class D | 0.01% | -0.27% | 0.23% | 100% | 101% |
| Class F | -0.05% | -0.13% | 0.22% | 100% | 100% |

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 7)”, Document of Joint Video Experts Team, JVET-P2001.

- S. Iwamura, S. Nemoto, A. Ichigaya, K. Andersson, R. Yu, and J. Enhorn, “[AHG18][Non-CE5] Proposed cleanup of deblocking filter process,” Document of Joint Video Experts Team, JVET-P05863

- A. Norkin, K. Andersson, “Description of Core Experiment 1 (CE1): Deblocking filtering,” Document of Joint Video Experts Team, JVET-P2021.

Patent rights declaration(s)

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).