| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0099-v1 |
| --- | --- |

| Title: | On Interaction of LFNST and Transform skip |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Masaru Ikeda Yoichi Yagasaki Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-2779 Takeshi.Tsukuba@sony.com Masaru.Ikeda@sony.com Yoichi.Yagasaki@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

In WD/SW, lfnst_idx signaling is dependent on usage of transform skip for luma. In case of SingleTree, LFNST for chroma is normatively restricted since lfnst_idx signaling is skipped when transform skip for luma is used. In addition, in case of DualTreeChroma, lfnst_idx can be signaled regardless of usage of transform skip for chroma, which is redundant signaling.

This contribution proposes two variants: proposal#1) omit lfnst_idx signaling when transform skip is used at least for one component, and proposal#2) omit lfnst_idx signaling when transform skip is used for all valid components.

It is reported that proposal#1 gives average BD-rate changes of (0.00 %, -0.01 %, 0.02 %) for AI, and (0.01 %, 0.06%, 0.06%) for RA.

It is reported that proposal#2 gives average BD-rate changes of (0.00%, 0.01%, 0.01%) for AI, and (0.00%, 0.00%, 0.00%) for RA.

Introduction

In the current WD, lfnst_idx signaling is dependent on transform_skip_flag as in Table 1-1. Table 1-2 shows the decision tree on lfnst_idx signaling depending on transform_skip_flag and treeType.

As can be seen, in case of SingleTree, LFNST for chroma is normatively restricted since lfnst_idx signaling is skipped when transform skip for luma is used (highlighted in green). In addition, in case of DualTreeChroma, lfnst_idx can be signaled regardless of usage of transform skip for chroma (highlighted in yellow), which is redundant signaling.

Table 1-1: Syntax table related to lfnst_idx signaling

| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && ( treeType != DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| --- | --- |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |

Table 1-2: Decision tree of lfnst_idx signaling depending on transform_skip_flag and treeType

| treeType | transform_skip_flag[0] | transform_skip_flag[1] | transform_skip_flag[2] | lfnst_idx signaling |
| --- | --- | --- | --- | --- |
| SINGLE_TREE | 1 | 0 | 0 | infer to 0 |
|  | 1 | 0 | 1 | infer to 0 |
|  | 1 | 1 | 0 | infer to 0 |
|  | 1 | 1 | 1 | infer to 0 |
|  | 0 | 0 | 0 | possible |
|  | 0 | 0 | 1 | possible |
|  | 0 | 1 | 0 | possible |
|  | 0 | 1 | 1 | possible |
| DUAL_TREE_LUMA | 0 | - | - | possible |
|  | 1 | - | - | infer to 0 |
| DUAL_TREE_CHROMA | - | 0 | 0 | possible |
|  | - | 0 | 1 | possible |
|  | - | 1 | 0 | possible |
|  | - | 1 | 1 | possible |

Proposal

This contribution proposes two variants: proposal#1) omit lfnst_idx signaling when transform skip is used at least for one component, proposal#2) omit lfnst_idx signaling when transform skip is used for all valid components.

Proposal#1: Omit lfnst_idx signaling when transform skip is used at least for one component

In proposal#1, to align lfnst_idx signaling depending on transform_skip_flag between luma and chroma component, it is proposed to omit lfnst_idx signaling when transform skip is used at least for one component. Table 2-1 shows a changed decision tree of lfnst_idx signaling based on treeType and transform_skip_flag. In other words, LFNST can be used when transform skip is not used for all valid components.

Table 2-2 shows proposed syntax table corresponding to Table 2-1.

Table 2-1: Decision tree of lfnst_idx signaling depending on transform_skip_flag and treeType

| treeType | transform_skip_flag[0] | transform_skip_flag[1] | transform_skip_flag[2] | lfnst_idx signaling |
| --- | --- | --- | --- | --- |
| SINGLE_TREE | 1 | 0 | 0 | infer to 0 |
|  | 1 | 0 | 1 | infer to 0 |
|  | 1 | 1 | 0 | infer to 0 |
|  | 1 | 1 | 1 | infer to 0 |
|  | 0 | 0 | 0 | possible |
|  | 0 | 0 | 1 | infer to 0 |
|  | 0 | 1 | 0 | infer to 0 |
|  | 0 | 1 | 1 | infer to 0 |
| DUAL_TREE_LUMA | 0 | - | - | possible |
|  | 1 | - | - | infer to 0 |
| DUAL_TREE_CHROMA | - | 0 | 0 | possible |
|  | - | 0 | 1 | infer to 0 |
|  | - | 1 | 0 | infer to 0 |
|  | - | 1 | 1 | infer to 0 |

Table 2-2: Proposed Syntax table related to lfnst_idx signaling

| tsUsed = (transform_skip_flag[x0][y0][0] == 1 && tu_cbf_luma[x0][y0]) ¦¦ (transform_skip_flag[xC][yC][1] == 1 && tu_cbf_cb [xC][yC]) ¦¦ (transform_skip_flag[xC][yC][2] == 1 && tu_cbf_cr [xC][yC] ) |  |
| --- | --- |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && (transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 tsUsed==0) && ( treeType != DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |

Proposal#2: omit lfnst_idx signaling when transform skip is used for all valid components

In proposed#1, in case of SingleTree, LFNST for luma is normatively restricted when transform skip for chroma is used. In proposal#2, to relax the transform-skip-based LFNST restriction, it is proposed to omit lfnst_idx signaling when transform skip is used for all valid components.

Table 2-3 shows a changed decision tree of lfnst_idx signaling based on treeType and transform_skip_flag. In other words, LFNST can be used when non-transform-skipped block exist in at least one component.

Table 2-4 shows proposed syntax table corresponding to Table 2-3.

Table 2-3: Decision tree of lfnst_idx signaling depending on transform_skip_flag and treeType

| treeType | transform_skip_flag[0] | transform_skip_flag[1] | transform_skip_flag[2] | lfnst_idx signaling |
| --- | --- | --- | --- | --- |
| SINGLE_TREE | 1 | 0 | 0 | possible |
|  | 1 | 0 | 1 | possible |
|  | 1 | 1 | 0 | possible |
|  | 1 | 1 | 1 | infer to 0 |
|  | 0 | 0 | 0 | possible |
|  | 0 | 0 | 1 | possible |
|  | 0 | 1 | 0 | possible |
|  | 0 | 1 | 1 | possible |
| DUAL_TREE_LUMA | 0 | - | - | possible |
|  | 1 | - | - | infer to 0 |
| DUAL_TREE_CHROMA | - | 0 | 0 | possible |
|  | - | 0 | 1 | possible |
|  | - | 1 | 0 | possible |
|  | - | 1 | 1 | infer to 0 |

Table 2-4 Proposed Syntax table related to lfnst_idx signaling

| nonTsUsed = (treeType != DUAL_TREE_CHROMA && transform_skip_flag[x0][y0][0] == 0 && tu_cbf_luma[x0][y0]) ¦¦ (treeType !=DUAL_TREE_LUMA && transform_skip_flag[xC][yC][1] == 0 && tu_cbf_cb[xC][yC]) ¦¦ (treeType !=DUAL_TREE_LUMA && transfrom_skip_flag[xC][yC][2] == 0 && tu_cbf_cr[xC][yC]) |  |
| --- | --- |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 nonTsUsed == 1 && ( treeType != DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |

Simulation Results

The proposed changes were implemented on top of VTM-7.0 and simulations were conducted following common test condition [2]:

- Test1: VTM-7.0 vs Proposal#1 at normal QP

- Test2: VTM-7.0 vs Proposal#2 at normal QP

Table 3-1 to 3-2 shows the detail results of each Test, respectively.

Table 3-1: Results of Test1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | 0.05% | 101% | 101% |
| Class A2 | -0.01% | -0.01% | 0.01% | 103% | 103% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class C | 0.00% | -0.02% | 0.05% | 102% | 102% |
| Class E | 0.00% | 0.00% | -0.03% | 100% | 101% |
| Overall | 0.00% | -0.01% | 0.02% | 101% | 101% |
| Class D | 0.00% | 0.03% | -0.01% | 100% | 103% |
| Class F | -0.01% | 0.01% | 0.00% | 101% | 102% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.01% | 0.01% | 101% | 101% |
| Class A2 | -0.01% | 0.09% | 0.08% | 102% | 102% |
| Class B | 0.02% | 0.06% | 0.05% | 101% | 101% |
| Class C | 0.02% | 0.07% | 0.10% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.06% | 0.06% | 101% | 101% |
| Class D | -0.04% | -0.24% | -0.07% | 101% | 102% |
| Class F | 0.00% | -0.02% | -0.05% | 101% | 101% |

Table 3-2: Results of Test2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.05% | 98% | 98% |
| Class A2 | 0.00% | 0.00% | 0.01% | 101% | 101% |
| Class B | 0.00% | 0.00% | -0.06% | 99% | 98% |
| Class C | 0.01% | 0.06% | 0.06% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.04% | 101% | 100% |
| Overall | 0.00% | 0.01% | 0.01% | 100% | 99% |
| Class D | 0.00% | -0.02% | 0.09% | 100% | 101% |
| Class F | 0.00% | -0.01% | 0.07% | 101% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.04% | -0.04% | 102% | 102% |
| Class A2 | -0.03% | 0.04% | -0.03% | 102% | 101% |
| Class B | 0.00% | -0.03% | 0.17% | 101% | 99% |
| Class C | 0.02% | 0.00% | 0.00% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.04% | 101% | 101% |
| Class D | -0.03% | -0.07% | 0.07% | 101% | 106% |
| Class F | 0.00% | 0.03% | 0.03% | 101% | 100% |

Conclusion

It is recommended to adopt either of LFNST index signaling into the next VTM/VVC WD.

Reference

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 7), ” JVET-P2001, Oct. 2019

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Apr. 2019

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).